This program demonstrates a technique for playing digitized sound through

the speaker on faster 286's or 386's.  This method employs pulse width

modulation at 16KHz to produce the equivalent of 6-bit resolution.

Basically, the system timer interrupt is taken over and speeded up

to 16KHz, rather than the normal 18.2 Hz.  At each interrupt, the next

sound sample (scaled to a range of 0-72) is used as the duty cycle for

the speaker timer, i.e., the amount of time the output stays high between

interrupts.  Low-pass filtering by the ear is mainly responsible for

reconstruction of the original signal.  This technique is equivalent to

that being employed in "many times" over-sampling systems, where it is

cheaper to use very high speed 1-bit DACs and low pass filter the output,

rather than using costly high-resolution DACs at lower rates.

Sound input to the program is assumed to be in Turtle Beach SampleVision

format, though it would be fairly easy to substitute another source.

The program expects one command line parameter, which is the name of

the SampleVision file.  Rate conversion to 16KHz is done assuming the

nearest 8KHz increment in the sampling range 8KHz to 48KHz.

There is currently a problem which has yet to be satisfactorily

resolved.  Occasionally a high-pitched shrill will be heard over the

sound.  This appears to related to the initialization sequence of the

programmable interval timer.  In general, however, the sound quality

is very intelligible, especially for voice.  Note that unless the

original signal is highly compressed, the low sound level of the pc

speaker will result in very soft output.  In fact, pushing the sample

level beyond clipping may actually help in some cases.  (Remember, this

is meant to be merely functional, not audiophile quality).  Good Luck!