CHAPTER 6    THE 86 INSTRUCTION SET

In this chapter we discuss in detail the instruction set

supported by both the A86 and A386 assemblers.  To use any of the

32-bit registers, the extra segment regsiters FS and GS, or the

instructions marked with a "3", "4", or "5" in the instruction

list, you need my A386 assembler, available only if you register

both A86 and D86.

Effective Addresses

Most memory data accessing in the 86 family is accomplished via

the mechanism of the effective address.  Wherever an effective

address specifier "eb", "ew" "ed", or "ev" appears in the list of

instructions, you may use a wide variety of actual operands in

that instruction.  These include general registers, memory

variables, and a variety of indexed memory quantities.

GENERAL REGISTERS: Wherever an "ew" appears, you can use any of

the 16-bit registers AX,BX,CX,DX,SI,DI,SP, or BP.  Wherever an

"eb" appears, you can use any of the 8-bit registers

AL,BL,CL,DL,AH,BH,CH, or DH.  Whenever an "ed" occurs, you can

(on A386) use any of the 32-bit registers

EAX,EBX,ECX,EDX,ESI,EDI,EBP, or ESP.  For A86, "ev" is the same

as "ew"; for A386, it means you can use either a 16-bit or a

32-bit register.  For example, the "ADD ev,rv" form subsumes the

16-bit register-to-register adds; for example, ADD AX,BX; ADD

SI,BP; ADD SP,AX.  On A386, this form also includes 32-bit

register-to-register adds; e.g., ADD EBX,ESI.  At the machine

level the 16-vs.-32 distinction is made by an "operand override"

opcode byte, 66H, that A386 places before the instruction to

signal a switch from 16-bits to 32-bits.

MEMORY VARIABLES: Wherever an "eb", "ew", "ed", or "ev" appears,

you can use a memory variable of the indicated size: byte, word,

doubleword, or either-word-or-doubleword.  Variables are

typically declared in the DATA segment, using a DD declaration

for a doubleword variable, a DW declaration for a word variable,

or a DB declaration for a byte variable.  For example, you can

declare variables:

DATA_PTR  DW ?

ESC_CHAR  DB ?

Later, you can load or store these variables:

MOV ESC_CHAR,BL    ; store the byte variable ESC_CHAR

MOV DATA_PTR,081   ; initialize DATA_PTR

MOV SI,DATA_PTR    ; load DATA_PTR into SI for use

LODSW              ; fetch the word pointed to by DATA_PTR

Alternatively, you can address specific unnamed memory locations

by enclosing the location value in square brackets; for example,

MOV AL,[02000]     ; load contents of location 02000 into AL

6-2

Note that A86 discerned from context (loading into AL) that a

BYTE at 02000 was intended.  Sometimes this is impossible, and

you must specify byte or word:

INC B[02000]       ; increment the byte at location 02000

MOV W[02000],0     ; set the WORD at location 02000 to zero

INDEXED MEMORY: The 86 supports the use of certain registers as

base pointers and index registers into memory.  BX and BP are the

base registers; SI and DI are the index registers.  You may

combine at most one base register, at most one index register,

and a constant number into a run time pointer that determines the

location of the effective address memory to be used in the

instruction.  These can be given explicitly, by enclosing the

index registers in brackets:

MOV AX,[BX]

MOV CX,W[SI+17]

MOV AX,[BX+SI+5]

MOV AX,[BX][SI]5   ; another way to write the same instr.

Or, indexing can be accomplished by declaring variables in a

based structure (see the STRUC directive in Chapter 9):

STRUC [BP]        ; NOTE: based structures are unique to A86!

BP_SAVE   DW ?  ; BP_SAVE is a word at [BP]

RET_ADDR  DW ?  ; RET_ADDR is a word at [BP+2]

PARM1     DW ?  ; PARM1 is a word at [BP+4]

PARM2     DW ?  ; PARM2 is a word at [BP+6]

ENDS              ; end of structure

INC PARM1         ; equivalent to INC W[BP+4]

Finally, indexing can be done by mixing explicit components with

declared ones:

TABLE DB 4,2,1,3,5

MOV AL,TABLE[BX]      ; load byte number BX of TABLE

The 386 processor also supports indexing using any of the eight

32-bit general registers.  This type of indexing is of limited

use for memory referencing from real-mode programs (most programs

running under DOS), since offsets greater than 64K are disallowed

in real mode (you will get a General Protection Fault if you try

it).  32-bit indexing is, however, useful in conjunction with the

LEA instruction, giving an extremely powerful register arithmetic

instruction.  For example, LEA ECX,[EAX+2*EBX+17000] performs two

additions and a multiplication, all in a single machine

instruction.  Since no memory accessed is actually attempted,

this kind of LEA usage is allowed in real-mode DOS programs.

At the time of this writing, my A386 assembler does not yet

support 32-bit indexing; I hope to add it soon.  If you have A386

on your registered A86+D86 disk, see the A386.DOC file for the

latest implementation news.

6-3

In 32-bit indexing, you may use one or two of any of the 32-bit

general registers.  You may also scale one of the indexing

registers, by multiplying it by 2, 4, or 8.  You may also add or

subtract a constant of any size up to a doubleword capacity to

the indexed quantity.  If you use the same register twice and

scale one of the instances of that register, you get, in effect,

an odd-number scaling (3, 5, or 9) of that register; e.g., A386

will allow LEA EAX,[9*EBX] as an abbreviation for LEA

EAX,[8*EBX+EBX].

Due to coding restrictions, the ESP register can be used only

once within an indexed quantity, and cannot be scaled.

Some more examples of 32-bit indexing are:

XCHG DX,[EAX]

MOV AL,[EAX+EBX]

ADD EBX,[ESI+8*ECX+3391811]

LEA ECX,[4*EBX-7]

Segmentation and Effective Addresses

The 86 family has four segment registers, CS, DS, ES, and SS,

used to address memory.  The 386 and later processors add two

more segment registers FS and GS.  Each segment register points

to 64K bytes of memory within the 1-megabyte memory space of the

86. (The start of the 64K is calculated by multiplying the

segment register value by 16; i.e., by shifting the value left by

one hex digit.)  If your program's code, data and stack areas can

all fit in the same 64K bytes, you can leave all the segment

registers set to the same value.  In that case, you won't have to

think about segment registers: no matter which one is used to

address memory, you'll still get the same 64K.  If your program

needs more than 64K, you must point one or more segment registers

to other parts of the memory space.  In this case, you must take

care that your memory references use the segment registers you

intended.

Each effective address memory access has a default segment

register, to be used if you do not explicitly specify which

segment register you wish.  For most effective addresses, the

default segment register is DS.  The exceptions are those

effective addresses that use the BP register for indexing.  All

BP-indexed memory references have a default of SS.  (This is

because BP is intended to be used for addressing local variables,

stored on the stack.)

6-4

If you wish your memory access to use a different segment

register, you provide a segment override byte before the

instruction containing the effective address operand.  In the A86

language, you code the override by giving the name of the segment

register you wish before the instruction mnemonic.  For example,

suppose you want to load the AL register with the memory byte

pointed to by BX.  If you code MOV AL,[BX], the DS register will

be used to determine which 64K segment BX is pointing to.  If you

want the byte to come from the CS-segment instead, you code CS

MOV AL,[BX].  Be aware that the segment override byte has effect

only upon the single instruction that follows it.  If you have a

sequence of instructions requiring overrides, you must give an

override byte before every instruction in the sequence.  (In that

case, you may wish to consider changing the value of the default

segment register for the duration of the sequence.)

NOTE: This method for providing segment overrides is unique to

the A86 assembler!  The assemblers provided by Intel and IBM

(MS-DOS) attempt to figure out segment allocation for you, and

plug in segment override bytes "behind your back".  In order to

do this, those assemblers require you to inform them which

variables and structures are pointed to by which segment

registers.  That is what the ASSUME directive in those assemblers

is all about.  I wrote Intel's first 86 assembler, ASM86, so I

have been watching the situation since day one.  Over the years,

I have concluded that the ASSUME mechanism creates far, far more

confusion that it solves.  So I scrapped it; and the result is an

assembler with far less red tape.  But if your program needs more

than 64K, you do have to manage those segment registers yourself;

so take care!

Effective Use of Effective Addresses

Remember that all of the common instructions of the 86 family

allow effective addresses as operands.  (The only major functions

that don't are the AL/AX specific ones: multiply, divide, and

input/output).  This means that you don't have to funnel many

numbers through AL or AX just to do something with them.  You can

perform all the common arithmetic, PUSH/POP, and MOVes from any

general register to any general register; from any memory

location (indexed if you like) to any register; and (this is most

often overlooked) from any register TO memory.  The only thing

you can't do in general is memory-to-memory.  Among the more

common operations that inexperienced 86 programmers overlook are:

* setting memory variables to immediate values

* testing memory variables, and comparing them to constants

* preserving memory variables by PUSHing and POPping them

* incrementing and decrementing memory variables

* adding into memory variables

6-5

Encoding of Effective Addresses

This section outlines the number of program opcode bytes

generated by effective-address specifications.  This will let you

make judgments when trying to keep your program as small as

possible.  The precise opcodes generated are explained in the

text files EFF86.DOC in the A86 package, and EFF386.DOC in the

A386 package.

Every instruction with an 16-bit effective address has an encoded

byte, known as the effective address byte, following the

instruction's main opcode.  (For obscure reasons, Intel calls

this byte the ModRM byte.)  If the effective address is a memory

variable, or an indexed memory location with a non-zero constant

offset, then the effective address byte is immediately followed

by the offset amount.  Amounts in the range -128 to +127 are

given by a single signed byte.  Amounts outside that range are

represented by a 2-byte offset.

In the instruction chart given later in this chapter,

effective-address specification opcodes are denoted by a slash /

followed either by the letter "r" or an octal digit.  The meaning

of the r-or-digit is explained in the EFF*.DOC files.  For

example, the instruction DIV CX falls under the DIV eb form in

the instruction chart.  The instruction occupies two bytes: the

main opcode byte 0F6H, followed by a single effective address

byte with no constant offsets involved.  Similarly, the

instruction DIV B[BX] occupies two bytes.  For DIV B[BX+7] you

must add an offset byte for the 7, making a total of three bytes.

For DIV B[BX+1000] you must add a 2-byte offset for the 1000,

making a total of 4 bytes.  For DIV B[02000] (more typically

coded with a symbolic name such as DIV MY_VAR_NAME), the

instruction is also 4 bytes: the main opcode byte, the effective

address byte, and the offset of the memory variable.

An anomalous case is the operand [BP].  The effective-address

byte encoding for this particular operand was usurped by the

simple-variable case.  When A86 sees [BP], it must specify an

8-bit offset whose value is zero.  Thus, the instruction DIV

B[BP] occupies three bytes, not two. This anomaly does not apply

to [BP+SI] or [BP+DI].

In A386, 32-bit indexing is signalled by a special address

override opcode byte (67H) preceding the instruction.  Following

the override byte is the instruction's main opcode, followed by

the effective-address specification.  For a simple memory

variable, the specification consists of a single

effective-address byte followed by the 4-byte offset of the

variable.  For indexing involving a single, non-scaled index

register other than ESP, the specification consists of a single

byte followed by the constant offset component.  For indexing

involving two registers, scaling, or the ESP register, there are

two bytes followed by the constant offset component.  The

constant offset component occpies no space if the the offset is

zero, one byte if between -128 to +127, and 4 bytes otherwise.

There is no provision for a 16-bit-word-sized offset if you are

using 32-bit indexing.

6-6

Note the distinction between the address override byte (67H) and

the operand override byte (66H).  A86 must supply an address

override when the instruction involves a memory operand whose

address has 32 bits.  A86 must supply an operand override when

the data being manipulated has 32 bits.  In general, when a

32-bit register name appears inside the square brackets, that's

an address override; when it appears outside the square brackets,

that's an operand override.  Examples:

MOV DX,[BX]      ; needs neither override in a 16-bit segment

MOV DX,[EBX]     ; needs an address override

MOV EDX,[BX]     ; needs an operand override

MOV EDX,[EBX]    ; needs both overrides

Also note that the generation of these override bytes is handled

automatically by A86 when it scans the operands to an

instruction.  The only exceptions to this are the no-operand

string operations: REP MOVSW, LODSD, SCASB, etc.  For these

instructions, the operand size is signalled by the last letter

(B, W, or D) of the mnemonic; however, the addressing mode is not

signalled by the mnemonic.  If you are in 16-bit mode, as all

simple DOS programs are, you need to precede a string instruction

with an explicit A4 prefix if you wish to use 32-bit addressing

([ESI] and/or [EDI] with count ECX).  If you are assembling to a

32-bit protected-mode segment (when that is implemented) you will

need to use an explicit A2 prefix if you wish to use 16-bit

addressing ([SI] and/or [DI] with count CX).

Here are some examples of instruction size involving 32-bit

indexing in a real-mode segment: DIV B[EBX] requires an address

override byte, the single instruction opcode byte 0F6H, and an

effective address byte: total 3 bytes.  DIV B[EBX+7] adds the

offset byte 07, making the total 4 bytes.  DIV B[EBX+1000] forces

the offset to be 4 bytes, making the total 7 bytes.  DIV

B[EBX+EDI*2] does not require an offset, but the extra index

register expands the effective address specifier to two bytes,

making the total 4 bytes.  Similarly, DIV B[ESP] requires two

effective address bytes (total 4 instruction bytes), because the

ESP register is a special case.  Finally, DIV

ES:D[EBX+EDI*2+1000] requires three overrides (segment override

ES, operand override for the D, and address override for 32-bit

indexing), the main opcode byte, two effective address opcode

bytes, and a 4-byte offset: total 10 bytes.

The [BP] extra-byte anomaly applies, in 32-bit mode, to [EBP] as

well.  In fact, the anomaly also applies when another indexing

register (scaled or not) is added to [EBP].  A386 must generate

an offset byte whose value is 0 when it sees any no-offset forms

involving [EBP].

6-7

The 386 and later processors, when running in protected mode,

allow segments whose default word-size is 32 bits instead of

16-bits.  In such segments, the usage of the operand and address

override bytes is reversed: 32-bit operands do not require the

operand-override byte, and 16-bit operands do.  (8-bit operands

never require an operand-override byte.) 32-bit memory addresses

do not require an address-override byte; 16-bit addresses do.

This mode will be recognized by A386 whenever the USE32 directive

is used; however, at the time of this writing, this feature is

not yet implemented.  All DOS programs, which run in real mode,

have a default of 16 bits.

How to Read the Instruction Set Chart

The following chart summarizes the machine instructions you can

program with A86.  In order to use the chart, you need to learn

the meanings of the specifiers (each given by 2 lower case

letters) that follow most of the instruction mnemonics.  Each

specifier indicates the type of operand (register byte, immediate

word, etc.) that follows the mnemonic to produce the given

opcodes.  The "v" type, for A86, is the same as "w" -- it denotes

a 16-bit word.  On A386, "v" denotes either a word or doubleword,

depending on the presence of an operand override prefix byte.

"c"  means the operand is a code label, pointing to a part of the

program to be jumped to or called.  A86 will also accept a

constant offset in this place (or a constant segment-offset

pair in the case of "cp").  "cb" is a label within about 128

bytes (in either direction) of the current location.  "cv" is

a label within the same code segment as this program; "cp" is

a pair of constants separated by a colon-- the segment value

to the left of the colon, and the offset to the right.  The

offset is always a word in A86; it can be either a word or a

doubleword in A386.  Note that in both the cb and cv cases,

the object code generated is the offset from the location

following the current instruction, not the absolute location

of the label operand.  In some assemblers (most notably for

the Z-80 processor) you have to code this offset explicitly

by putting "$-" before every relative jump operand in your

source code.  You do NOT need to, and should not do so with

A86.

"e"  means the operand is an Effective Address.  The concept of

an Effective Address is central to the 86 machine

architecture, and thus to 86 assembly language programming.

It is described in detail at the start of this chapter.  We

summarize here by saying that an Effective Address is either

a general purpose register, a memory variable, or an indexed

memory quantity.  For example, the instruction "ADD rb,eb"

includes the instructions: ADD AL,BL, and ADD CH,BYTEVAR, and

ADD DL,B[BX+17].

6-8

"i"  means the operand is an immediate constant, provided as part

of the instruction itself.  "ib" is a byte-sized constant;

"iw" is a constant occupying a full 16-bit word.  The operand

can also be a label, defined with a colon.  In that case, the

immediate constant which is the location of the label is

used.  Examples:  "MOV rw,iw" includes the instructions: MOV

AX,17, or MOV SI,VAR_ARRAY, where "VAR_ARRAY:" appears

somewhere in the program, defined with a colon.  NOTE that if

VAR_ARRAY were defined without a colon, e.g., "VAR_ARRAY DW

1,2,3", then "MOV SI,VAR_ARRAY" would be a "MOV rw,ew" NOT a

"MOV rw,iw".  The MOV would move the contents of memory at

VAR_ARRAY (in this case 1) into SI, instead of the location

of the memory. To load the location, you can code "MOV

SI,OFFSET VAR_ARRAY".

"m"  means a memory variable or an indexed memory quantity; i.e.,

any Effective Address EXCEPT a register.

"r"  means the operand is a general purpose register.  The 8 "rb"

registers are AL,BL,CL,DL,AH,BH,CH,DH; the 8 "rw" registers

are AX,BX,CX,DX,SI,DI,BP,SP.

"rv/m" is used in the Bit Test instructions to denote either

a word-or-doubleword register, or an array of bits in memory

that can any length.

NOTE: The following chart gives all instructions for all

processors through the Pentium.  You must take care to use only

the instructions appropriate for the target processor of your

program (the P switch will enforce this for you: see Chapter 3).

If an instruction form does not run on all processors, there is a

letter or digit just before the description field.  "N" means the

instruction runs only on NEC processors (which are rare nowdays).

A digit x means the instruction runs on the x86 or later: 1 for

186, 2 for 286, 3 for 386, 4 for 486, 5 for Pentium.

Instructions with 3 or greater are recognized only by my A386

assembler, received only by those who register both A86 and D86.