177

CHAPTER 17 - INTERRUPTS

Your word processor will work on a Compaq, an IBM, an AST or any

other type of PC compatible computer. Every time it wants to read

a file from disk or write to a printer, it calls a DOS or BIOS

function that takes care of it.{1}  Yet every computer has its

own versions of these subroutines, and they not only are

different, they are in different places in memory. How does the

word processor know where to find them? It uses interrupts.{2}

An interrupt is a glorified subprogram call. The first 1024 bytes

of your computer's memory (that's from 0000:0000 to 0000:03FF)

contain the addresses of all the DOS and BIOS interrupts. Which

address contains the address of which subprogram was decided by

the triumverate of Intel/IBM/Microsoft. We have two different

sets of addresses here, so let's keep them straight. Starting at

memory address 0000 there are 4 byte addresses called interrupt

vectors. There is a different vector at 0000d, at 0004d, at

0008d, at 0012d at 0016d etc.; there are 256 of them in all. Each

of these 256 places can hold the address of a subprogram

somewhere in memory (although not all of them are used).

When you call an interrupt, the 8086 goes to the appropriate

place in low memory (from 0000 to 3FFh) and finds the address of

the subprogram that you want, loads it into CS and IP, and goes

to that subprogram. When that subprogram is done, it goes back to

the next instruction in your program after the interrupt.

How did those addresses get into the first 1024 bytes? The

computer put them there when you started it up. It is one of the

first things that the computer did when you turned it on. These

subprograms do EVERYTHING, and you can't run the computer without

them.

If you want to scroll the screen, you put the appropriate

information in the 8086 registers and use:

int 10h             ; decimal 16 {3}

The 8086 goes to the interrupt 16 address (4*16 = 64), gets the

address of the program that contains the video subprograms, and

____________________

1. BIOS stands for basic input/output services.

2. If you haven't gotten it yet, it's time to get either "DOS

Programmer's Reference" or "The Peter Norton Programmer's Guide

to the IBM PC." I'm serious. They contain the information you

need to make use of this chapter.

3. It is normal to use hex numbers for the interrupts, so if

you read about an interrupt make sure you know if the numbers are

hex or decimal.

______________________

The PC Assembler Tutor - Copyright (C) 1989 Chuck Nelson

The PC Assembler Tutor                                        178

______________________

goes to it. If you want to write to the printer, you put the

appropriate information in the 8086 registers and call:

int 21h             ; decimal 33

The 8086 goes to address 132 (4*33 = 132), gets the address of

the DOS program, puts it in CS and IP, and starts it. If you want

to get input from the keyboard, you put the appropriate

information in the 8086 registers and call:

int  21h            ; decimal 33

That's right, it is the same program as the one that does the

printer. The 8086 goes to 132 (4*33), gets the program address,

and goes to it.

The lowest interrupt is int 0 (address = 4*0 = 0000). The highest

interrupt is int 255 (address = 4*255 = 1020).

This is an intelligent way to handle the situation. As long as

everyone agrees which interrupt contains the address of which

subprogram, our programs will work on any PC compatible. This is

one of the things that is meant by PC compatible.

On my computer, here is a section of these addresses starting

with int 1Eh (30d). High memory is at the top, low memory is at

the bottom.

INT #           DATA       LOCATION

0724h cs       146

36           04A8h ip       144

cs 0724h          142

35        ip 01BDh          140

0724h cs       138

34           01B0h ip       136

cs 019Fh          134

33        ip 05EBh          132

019Fh cs       130

32           05E7h ip       128

cs 0000h          126

31        ip 0000h          124

0070h cs       122

30           0EB8h ip       120

If we call int 30d, the 8086 goes to 120 (4*30), puts 0EB8h into

the IP, and puts 0070h (from the next higher location) into CS.

The next instruction it does will be 0070:0EB8. If we call int

35d, the 8086 goes to 140 (4*35), puts 01BDh in IP, puts 0724h

(from the next higher location) in CS. The next instruction the

8086 does will be 0724:01BD. If we call int 33d, the 8086 goes to

132 (4*33), and puts 05EBh in IP, 019Fh  (from the next higher

location) in CS. The next instruction executed will be

019F:05EBh. The 0000:0000 for int 31d indicates that there is no

int 31d (address 0000:0000 contains data, [the vectors for int

0], not a program). Make sure that you understand how this is

working before you go on.

Chapter 17 - Interrupts                                       179

_______________________

On the PC, information for the interrupts is always passed

through registers, not on the stack. Each interrupt type has a

specific register for each piece of information it needs. We will

do a couple to see how they work.

DISPLAYING A CHARACTER

We can print a character at a time on the monitor, so we'll input

a string, and then print out each character of the string

individually. We will stop the printout when we see the 0 at the

end of the string.

; - - - - - - - - - - START DATA BELOW THIS LINE

buffer  db  80 dup (?)

; - - - - - - - - - - START DATA BELOW THIS LINE

; - - - - - - - - - - START CODE BELOW THIS LINE

call show_regs

outer_loop:

mov  ax, offset buffer

call get_string

mov  si, offset buffer

inner_loop:

mov  al, [si]

cmp  al, 0

je   next_string

mov  ah, 14              ; ah contains function number

mov  bh, 0               ; where in memory

int  10h                 ; 33d

inc  si

jmp  inner_loop

next_string:

call get_continue

jmp  outer_loop

; - - - - - - - - - - START CODE BELOW THIS LINE

The program is simple. It gets a string, then checks each

character for 0 (end of string), before shipping it off to the

screen. I'll explain AH in a second. There are several places

this character can be displayed{4}, so use show_regs to force the

video screen to a certain place in memory, then put BH = 0 to

tell the interrupt that the screen memory is in that place.

Finally, with all the information in place, we do the interrupt.

You will not print a carriage return, only the data, so we use

get_continue to give us a carriage return. It's a little sloppy,

but much easier.

We have lots and lots of subprograms for the disks, printer,

screen, etc. There are only 255 interrupts, so it was decided at

the beginning to make most of the interrupts groups of programs

____________________

4. Cf. one of those two books.

The PC Assembler Tutor                                        180

______________________

instead of a single program. Int 10h (16d) contains about two

dozed different video subprograms. Each program is distinguished

by a specific number in AH. For int 10h (16d):

ah = 2h        Get cursor position

ah = 6h        Scroll window up

ah = Eh        (14d) Write character to screen

For int 21h (33d):

ah = 1h        Keyboard input

ah = 5h        Printer output

ah = 17h       Rename a file

ah = 2Ch       Get the time

As you can see, int 21h is a potpourri of subprograms. We'll do

the same program as above, but with printer output. Everything is

the same except that the inner loop should be changed to look

like this:

; - - - - -

mov  si, offset buffer

inner_loop:

mov  dl, [si]

cmp  dl, 0

je   next_string

mov  ah, 5               ; ah contains function number

int  21h                 ; 33d

inc  si

jmp  inner_loop

; - - - - -

There is practically no change. We use DL instead of AL, have

"int 21h" instead of "int 10h", and change the function number in

AH to 5. Also, BH is not needed.

Leave your printer off at the beginning to see what happens. Turn

your printer on and enter a string of 10 or 20 letters. Probably

nothing happened. Enter another 20 or 30 letters. Nothing again.

Try 50 letters this time. This time it should work. Lots of

printers won't print anything unless (1) they get a carriage

return (which you haven't sent) or (2) they have a backlog of

more than 80 characters. If you ever use the printer interrupt,

you'll have to remember that.

These interrupts which are in your program are called software

interrupts. They are your interface with the peripheral devices

on your machine, doing everything from disk i/o to handling

memory allocation. They do all your housekeeping for you.{5}  If

you are going to work at this level then you should buy one of

those two books. They contain all the software interrupts (and

there are about a hundred of them), tell how they work and which

registers to set for each interrupt. If you don't have access to

____________________

5. But they don't do Windows.

Chapter 17 - Interrupts                                       181

_______________________

these interrupts it's like having your arms cut off - you're

unable to do any i/o at all.

HARDWARE INTERRUPTS

The interrupts that we write in our programs aren't the only

interrupts there are. The hardware uses interrupts to take

temporary control of the computer. On a Macintosh, if you insert

a disk, the program stops and the operating system reads in the

disk directory. A modem that is in use will request time for

doing i/o. Also, if the 8086 detects a zero divide, it will

trigger a special interrupt. You have met int 4 already. It is

the INTO instruction, which will trigger an interrupt if the

overflow flag is set.

Your interrupts are in specific places in your code, but these

machine interrupts can happen at any time. There are two lines

(wires) into the 8086. One is for serious problems that need to

be taken care of NOW, and it has non-maskable interrupts. The

other line is for interrupts that need to be taken care of in a

timely fashion, and they are maskable interrupts.

A non-maskable interrupt (NMI) is when the hardware detects that

it is in deep doo doo. It sends a signal on the NMI line that

says "Hey! I need an interrupt." The 8086 finishes the

instruction it is processing and then IMMEDIATELY gives over

control. The NMI uses the same 1024 bytes in low memory for

interrupt vectors, but has its own interrupt numbers. Normally

this is for very serious errors, so the interrupt program may

decide to abort your program and return to the operating system;

if it makes sense to, it will return to your program where your

program left off.

A maskable interrupt is when a piece of hardware has some work to

do. It sends a signal to the 8086 (on the INTR line), and the

8086 takes care of it when it is ready.

When the 8086 is ready depends on you. In the flags register is

the IEF, the interrupt enable flag. It should always be set to 1

unless you are doing something critical. Basically, the only

things that are critical are interrupts themselves and context

switches. Context switches are done by the operating system in

multitasking environments, so they don't concern you, and you are

not writing interrupts, so they don't concern you. Therefore,

always keep the IEF set. Just for your information, you set the

IEF with:

sti  ; set interrupt flag (interrupts enabled)

and clear it with:

cli  ; clear interrupt flag (interrupts disabled)

Why do interrupt programs clear the interrupt flag? Because you

could have interrupts interrupting other interrupts and wind up

with scads of half finished interrupts lying around. This way,

The PC Assembler Tutor                                        182

______________________

one interrupt finishes before another can take over.

Suppose you are in the middle of the following two instructions

when an interrupt hits:

cmp  al, 7

jne  some_label

If the hardware interrupt takes over after the CMP instruction

but before the JNE instruction, the correctness of the result

will depend on the zero flag staying the same. Can you trust it?

The answer is yes. The first three things an interrupt request

does (in the microcode) are:

push flags     ; push the flags

push old CS    ; push the code segment

push old IP    ; push the instruction pointer

On return, it pops the flags back in place, so they are exactly

the same as just before the interrupt. You will notice that there

are 3 things on the stack instead of the usual 2 in a far call.

Therefore, there is a special RET instruction called IRET (return

from interrupt) which pops not only IP and CS, but the flags as

well. You can only use this instruction in an interrupt because

it assumes that the flags register is right after CS on the

stack.

DEBUGGING

Finally, if you have a debugger installed, the debugger uses two

interrupts, int 1 and int 3. You code int 3 into the program by

placing:

int

in the program with no number.{6}  The interrupt will call the

debugger. The reason for this int with no number is that

afterwards, you can replace it with NOP, since they are both 1

byte instructions. Int 3 is a 2 byte instruction (they are coded

differently, even though they wind up at the same place).

Int 1 is the trap instruction. In the flags register is the trap

flag (TF). If TF is set, the 8086 will interrupt after the next

instruction. That way, the debugger can single step through

sections of code. You put Int 3 to set a breakpoint and then set

TF to single step from there.

The only way to set TF from your own program is:

____________________

6. Though the Microsoft assembler considers this an error. You

must code it as:

int  3

Chapter 17 - Interrupts                                       183

_______________________

pushf

pop  ax

or   ax, 0100h

push ax

popf

but you should let the debugger do it.

Just so you can get a feel for how the debugger system works,

there is a pseudo-debugger called PSEUDDBG.COM in \XTRAFILE. It

is not a debugger. It does nothing but tell you whether you have

generated a breakpoint interrupt (int 3) or a single_step

interrupt (int 1). It is memory resident. That means that once

you have loaded it, it will stay in memory until you shut the

machine off or reset the machine. You load it by executing:

>pseuddbg

It will tell you that it has been loaded and then sit there

waiting for interrupts. When you generate a breakpoint interrupt,

PSEUDDBG will allow you to set the trap flag or just continue.

When you generate a single step interrupt, PSEUDDBG will allow

you to clear TF or to continue single stepping. All you need to

try it out is a simple program:

; - - - - - ENTER CODE BELOW THIS LINE

outer_loop:

call get_continue

mov  cx, 4

int  3

inner_loop:

mov  ax, 5

add  ax, 3

add  ax, 7

add  ax, 10

loop inner_loop

loop outer_loop

; - - - - - FINISH CODE ABOVE THIS LINE

Each time you start the outer loop, it will generate a breakpoint

interrupt. At this point you can set TF to single step or

continue. If you single step, watch IP. It will be changing,

cycling through the loop till it gets to get_continue. If you

start trapping through get_continue, things will get strange

because get_continue stores the flags. This means that after you

quit trapping, get_continue will POP some flags that have TF set

and it will start trapping again. If this happens, just continue

hitting 0 until you get out of the single step mode.

If you like this method of single stepping through code, there is

a memory resident version of ASMHELP called HELPMEM.COM which

contains show_regs and allows single stepping. It can be used in

conjunction with ASMHELP.OBJ. For details consult APP1.DOC in the

appendix.

The PC Assembler Tutor                                        184

______________________

SUMMARY

Software interrupts send the 8086 to the first 1024 bytes of

memory where it finds the address of the DOS or BIOS subprogram

that you want to go to. The correspondence between the interrupt

number and the location of the address is:

address location = 4 * interrupt number

The address is stored in the next 4 bytes; first IP, then CS.

The first 5 interrupts are:

int 0     divide by zero

int 1     trap flag induced single stepping for the debugger

int 2     non_maskable_interrupt (NMI)

int 3     1 byte interrupt for the debugger

int 4     interrupt on overflow

There are two types of hardware interrupts. NMI (non maskable

interrupt) interrupts are for serious problems that need to be

taken care of IMMEDIATELY. They have priority and take over right

after the current instruction is finished.

Maskable interrupts are hardware interrupts that should be taken

care of in a timely fashion. If IEF (the interrupt enable flag)

is set the maskable interrupts will go through. If IEF is

cleared, the maskable interrupts must wait until it is set again.

Set the IEF with:

sti  ; set interrupt flag

and clear it with:

cli  ; clear interrupt flag

The IEF should always stay set unless there is a specific reason

for not allowing interrupts to happen.

If you are writing an interrupt routine, then instead of RET you

use IRET (return from interrupt) which not only pops off IP and

CS, but pops the flags register as well.