Command Line Format:

[d:][path]POST[option 1 value(s) ... option n value(s)]

Upper or lower case is OK except for Font names.  There are 72

points per inch.  The default page origin is located at the lower

left corner with positive X to the right and positive Y upwards.

The output filename must be different from the input filename.

POST will space to the next column position if it finds a tab

character.  Column positions can be defined as preset distances

from the left margin.  POST will skip to top of form if it finds

a form feed character. POST will ignore control characters other

than line feed, form feed and tab.

OPTIONS:         LETTER:    VALUE(S):                          DEFAULTS:

Left Margin         L       Left margin in points              L 54

Top Print Line      T       Top print position in points       T 720

Bottom Print        B       Bottom print limit in points       B 54

Column Tab          C       Column tab spacing in points       C 56

Typeface Size       P       Typeface size in points            P 12

Leading             D       Vertical line spacing in points    D 12

Typeface Font       F       Selected font for printing         F Courier

Scaling             S       X and Y scaling multipliers        S 1.0 1.0

Page Origin         X       X and Y page translation points    X 0 0

Page Orientation    R       Page rotation angle in degrees     R 0

Paper Tray Number   N       Must be a valid paper tray number  N 1