NWOS2.DOC           07/10/91

Copyright 1991, Compaq Computer Coprporation.

NOVELL NETWARE REQUESTOR FOR OS/2 INSTALLATION INSTRUCTIONS

CONTENTS

1. INTRODUCTION

2. SYSTEM REQUIREMENTS

3. INSTALLATION

4. NETWIRE

1. INTRODUCTION

The device driver (CPQTOKNW.SYS) V1.04 allows a system using

MS OS/2 to attach to networks using Novell NetWare.  The

following files are needed to support NetWare operation on

OS/2:

CPQTOKNW.SYS - OS/2 Device Driver

OS2NET.CFG - Sample Configuration File

In addition to these files, additional device drivers

available from Novell are required to configure an MS OS/2

system for use with Novell NetWare.  Refer to your Novell

NetWare Requester for OS/2 documentation or contact an

Authorized Novell NetWare Dealer for more information on the

NetWare Requester for OS/2.

2. SYSTEM REQUIREMENTS

This device driver requires:

*    Any COMPAQ EISA computer with the COMPAQ 32-bit DualSpeed

Token-Ring controller installed. The system must have

been configured using the COMPAQ EISA Configuration

Utility.

*    MS OS/2

*    Novell NetWare Requester for OS/2

3. INSTALLATION

In the following procedure, refer to your Novell NetWare

OS/2 Requester documentation.

1.   Follow the Novell OS/2 Requester installation

instructions to copy device drivers and support files

provided by Novell to your system.  Proceed to the step

where you are instructed to edit the CONFIG.SYS file to

choose the network interface board.

2.   Instead of removing "rem" from the beginning of an

existing network interface driver line, add the following

line in the same section of the CONFIG.SYS file:

DEVICE=C:\directory\CPQTOKNW.SYS

NOTE:  Substitute for directory the name of the subdirectory

into which you installed the other Novell NetWare Requester

drivers in step 1.

3.   Copy the CPQTOKNW.SYS file to the subdirectory you

specified during the Novell Requester installation.

(This is the same subdirectory you added to the

CONFIG.SYS file.)

4.   You may create a section in the Novell Requester network

configuration file, NET.CFG, to control several aspects

of the CPQTOKNW.SYS driver operation.  These parameters

and the NET.CFG file itself are completely optional.  The

configuration options used in the NET.CFG file for the

CPQTOKNW.SYS device driver are as follows:

Frame frame_type    This keyword is generally required

when protocols other than IPX, such

as TCP/IP, are loaded. The variable

frame_type can be TOKEN-RING or

TOKEN-RING_SNAP.

Node Address x      Overwrites the controller Burn-In

Address.  The variable x must be 12

hexadecimal digits.  Default is the

Burn-In Address.

No Earlyrelease     This keyword is for the token ring

controller running at 16 megabits per

second.  No parameters are necessary.

The default is early token release

at 16 Mbps.

Protocol IPX E0     This keyword specifies which protocol

is used with the driver. The example

above is Novell's IPX and its

protocol identification, defined by

Novell, is E0 (hex). This is also the

default for the driver.

Source Routing      This keyword allows the MS OS/2

workstation to access a server that

is not on the same physical network

through source routing bridges.  No

parameters are necessary.  Default

is no source routing.

Send Retries 3      This keyword configures the MLID to

attempt 3 retries on transmit

packets.  Default is 0 retries.

Refer to the sample NET.CFG file and your Novell NetWare

documentation for more information on these and other parameters

that may be set in the NET.CFG file.

4. NETWIRE

The NetWare OS/2 Requestor driver files for the COMPAQ 32-Bit

DualSpeed Token Ring Controller reside on Compuserve NetWire

under NOVA forum in library 11 (or library 17 for new uploads)

in the CPQTR2.ZIP file.