=============================================================================

Drivers for Digital Equipment Personal Computer Bus Adapter

DEPCA tm

DEPCA LC,    Digital Equipment Corporation Part # DE100-AA

DEPCA Turbo, Digital Equipment Corporation Part # DE200-AA

=============================================================================

DEPCA tm Highlights

o    High Performance Ethernet Adapter for IBM PC/AT tm and EISA tm

compatible.

o    Supports Ethernet v2.0 and IEEE 802.3 Standards.

o    Compatible with PC LANs that comply with NDIS tm standard.

o    Supports Multi-vendor PC LAN Software.

o    Supports DEC LanWORKS.

o    Requires only one PC slot.

===========================================================================

October 1990

===========================================================================

===========================================================================

Disclaimer:

===========================================================================

Although Digital Equipment Corporation will attempt to supply current

and accurate information relating to its file formats, algorithms, and the

subject programs, the possibility of error can not be eliminated.  Digital

therefore expressly disclaims any warranty that the information contained

in the associated materials relating to the subject programs and/or the

format of the files created or accessed by the subject program and/or the

algorithms used by the subject programs, or any matter, is current, correct

or accurate as delivered.  Any risk of damage due to any possible

inaccurate information is assumed by the user of the information.

Furthermore, the information relating to the subject programs and/or the

file formats created or accessed by the subject programs and/or the

algorithms used by the subject programs is subject to change without

notice.

The subject program is provided "as is" without warranty or any kind,

either  expressed or implied.  Should the subject program be defective,

Digital will assume no cost on any necessary servicing, repair, or

correction.  In the event of a loss, Digital is not responsible for

damages, including any lost profits, software, or other incidental or

consequential damages arising out of the use or inability to use the

subject program.

===========================================================================

Table of Contents

===========================================================================

Page

------

1.   Contents...................................................1

2.   General Information........................................2

3.   DOS Workstation Shell......................................3

4.   OS/2 Requester Driver......................................4

5.   Netware 2.1x Lan Driver....................................5

6.   Netware 386 3.0 Lan Driver.................................6

7.   Problems / Updates.........................................7

8.   Trademarks.................................................8

===========================================================================

1. Contents

===========================================================================

The self-extracting zip file contains the following files:

DEPCAII.DOC    - This document.

SDEPCAII.OBJ   - DOS Workstation Shell.

SDEPCAII.LAN   - Lantab file for Dos Workstation Shell.

DEPCAII.SYS    - OS/2 Requester Driver.

NET.CFG        - Sample Config file for OS/2 Requester.

ADEPCAII.OBJ   - Lan Drivers for Netware v2.1x

BDEPCAII.OBJ

CDEPCAII.OBJ

DDEPCAII.OBJ

DEPCAII.LAN    - Lantab file for Netware v2.1x driver.

DEPCA386.LAN   - Lan Driver for Netware 386 v3.0.

===========================================================================

2. General Information.

===========================================================================

The DEPCA drivers have been tested with the DECPA LC & DEPCA Turbo.

As of this date no testing has been done on the DEPCA Turbo/TP or DEPCA MC.

We are currently in the process of obtaining Novell certification.  In the

future we plan to upgrade the drivers to support more configurations and

the many new features of Novell Netware 386 v3.1 and Novell Requester for

OS/2 v1.2.

This document assumes that you have a fair knowledge of Novell

Netware, Netware utilities, and procedures.  There are references to

documents where further clarification can be found.

===========================================================================

3. DOS Workstation Shell.

===========================================================================

The current DOS Workstation shell version is DEC DEPCA II v1.02

(901001).  This shell supports all possible settings of both DEPCA LC and

DEPCA Turbo.  Although there is only one interrupt setting that both cards

have in common, the other interrupts are all supported.

The SDEPCAII.OBJ & SDEPCAII.LAN are need to create a dos workstation

shell.  The following instructions give an overview of how to create a

workstation shell from floppy drive.

Copy these files to a blank disk with the volume label set to:

"LAN_DRV_001" and follow the instructions outlined to generate the DOS

Workstation Shell using shgen.

For further information on installing/configuring the DOS workstation

shell consult the DOS Workstation Installation section in the Novell

Netware Installation Guide.

===========================================================================

4. Novell Requester for OS/2 v1.1

===========================================================================

The current version of the Novell Requester for OS/2 v1.02 (901001).

This driver has been tested the OS/2 v1.1 and Novell Requester for OS/2

v1.1.  At a future date the driver will be updated to support versions v1.2

of both Requester and OS/2.

The Requester for OS/2 Supplement gives important installation

information help on running the install procedures for OS/2 Requester.

After installing the Novell OS/2 Requester files:

o    Copy "DEPCAII.SYS" into the "C:\Netware" directory with the other

drivers.

o    Include "device = C:\NETWARE\DEPCAII.SYS" in the OS/2 config.sys

file.

o    Copy the "net.cfg" file to the boot hard drive root directory.

The net.cfg file contains the default settings to configure the

driver for different interrupts, DMA settings, ethernet media

type, etc.

o    There is further information starting at page 119 of the Netware

Requester for OS/2 to reconfigure the driver.

===========================================================================

5. LAN Driver for Netware v2.1x  (286)

===========================================================================

The current Netware 2.1x LAN Driver is version is DEC DEPCA II v1.02

(901001).  The DEPCA LAN driver has been tested with Novell Netware 2.1x

Advanced and SFT.

In short, this is an overview of the installation.

o    Copy the lan drivers: "adepcaii.obj, bdepcaii.obj, cdepcaii.obj,

ddepcaii.obj" to you \netware\lan_drv.001 directory or to your

lan_drv_.001 disk.

o    Next copy the configuration table, "depca.lan", into the

"\netware\auxgen" directory (or on to your auxgen disk).

o    You are now ready to run "netgen".

o    Follow the installation/configurations procedures outlined in the

Novell SFT/Advance NetWare 286 Installation Manual for running

netgen.

===========================================================================

6. LAN Driver for Netware 386 v3.0

===========================================================================

The current Netware 386 LAN Driver is version is DEC DEPCA II Netware

386 Driver v1.02 (901001).  The DEPCA386.LAN driver has been tested with

Novell Netware 386 v3.0.  Although the driver is upward compatible with the

v3.1 of Netware 386, it does not support all new features of Netware 386

v3.1  At a future date the driver will be updated to support version v3.1

of Netware 386.

From a workstation logged into the server copy "depca386.lan" in to

the "\system" directory.  From the console on the server: load the install

module, select system options, and modify the autoexec.ncf file to

load "depca386.lan" or enter commands on the server console.

A sample/default command line for a DEPCA set at the default settings with

ethernet IEEE 802.3:

: load depca386 frame = ethernet_802.3 port=300 mem=D0000 int=5

: bind ipx to depca386 net=#

For further information on installing/configuring, consult the "File

Server Installation" section of the Novel Netware Installation guide.

===========================================================================

7. Problems / Updates.

===========================================================================

The only short coming that has been found is the inability to create

internal bridges either in the server or externally (router).  This is a

known problem and efforts have begun to correct this problem.  But as of

this date the only work-around is to use another brand ethernet controller

card to complete the bridge.  We have successfully built and tested a

bridge with a Novell NE1000.

We want to get some feed back as to how you intend to use the driver

and what problems, if any, you're having with the driver.  Also we would

like your name and address so that we could, if possible, send notice as to

when updated drivers would be available.  New drivers will be posted on

COMPUSERV NetWire areas as soon as available.

Digital Equipment Corporation

Government System Group

8100 Corporate drive, Suite 100

Landover, Maryland  20785

Attn: Eric B. Betts, Consultant

301-306-7279

===========================================================================

Digital Employees:

There is information available on the DEPCA announcement,

price/performace stats, and marketing literature.  Call or e-mail for

location.

===========================================================================

===========================================================================

8. Copyrights

===========================================================================

IBM PC/AT & OS/2 are a registered trademarks of International Business

Machines Corporation.

DEC, DEPCA, DEPCA LC, DEPCA MC, DEPCA Turbo, DEPCA Turbo/TP, & LanWORKS are

trademarks of Digital Equipment Corporation.

Novell, Netware, Netware 386 3.x, Netware 2.1x, Novell OS/2 Requester v1.x,

SFT, IPX, & NE1000 are registered trademarks of Novell, Inc.

Ethernet is a registered trademark of Digital Equipment Corporation , Intel

Corporation, and Xerox Corporation.

Product names mentioned herein may be trademarks and/or registered

trademarks of their respective companies.