User documentation for the packet driver collection

Russell Nelson

Clarkson University

Potsdam, NY 13676

nelson@clutx.clarkson.edu

Why?

Every PC Ethernet board is different.  The 3c501 was, for

some time, the most popular board, and would have become the

defacto standard but for its poor performance.  By the time 3Com

created a board with acceptable performance, so had many others,

and 3Com lost its lead.  FTP Software, in order to support each

of these boards without losing its collective mind, created the

Packet Driver Spec (PDS).

What?

Some boards use DMA, some use programmed I/O, and some use

shared memory.  The PDS masks the differences between these

boards by defining a software interface to the boards.  This

software interface is a software interrupt in the range between

0x60 (hex) and 0x80.

How?

The packet driver must be installed prior to use.  Since

each packet driver takes about 1800 bytes, this is best done in

your autoexec.bat.  Since the Ethernet boards typically have

jumpers on board, the packet driver must be informed of the

values of these jumpers (auto-configure is possible, but can

disturb other boards).  The first parameter is the software in-

terrupt used to communicate with the packet driver.  And again,

because each board is different, the rest of the parameters will

be different.

Before installing the packet driver, you must choose a software

interrupt number in the range between 0x60 and 0x80.  Some of

these interrupts are used for other purposes, so your first

choice may not work.  See Appendix A for the section of Ralf

Brown's interrupt list between 0x60 and 0x80.

Running a packet driver with no specifications will give a usage

message.  The parameters for each packet driver are documented

below.

3Com 3C501

usage: 3C501 <packet_int_no> <int_no> <io_addr>

The 3c501 requires two additional parameters -- the

hardware interrupt number and the I/O address.  The defaults are

3 and 0x300.

3Com 3C503

usage: 3C503 <packet_int_no> <int_level(2-5)> <io_addr>

<cable_type>

The 3c503 requires three additional parameters -- the

hardware interrupt number, the I/O address, and the cable type.

The 3c503 can be attached to thick or thin Ethernet cables, and

the selection is made in software.  The cable type parameter

should be zero for thin, and one for thick.  The defaults are 2,

0x300, and 0 (thin).  The 3c503 uses shared memory whose address

is set by jumpers, but the software can ask the board what the

address is.

3Com 3c523

usage: 3c523 <packet_int_no> <int_no> <io_addr> <base_addr>

The 3c523 requires three additional parameters -- the

hardware interrupt number, the I/O address, and the memory base

address.  The defaults are 3 and 0x300 and 0xc000.

BICC Data Networks' ISOLAN 4110 ethernet

usage: isolan <packet_int_no> <int_no> <base_addr>

The BICC Isolan requires three additional parameters -- the

hardware interrupt number and the memory base address.  The

defaults are 2 and 0xb800h.

Racal-Interlan (Formerly Interlan) NI5010

usage: NI5010 <packet_int_no> <int_no> <io_addr>

The NI5010 requires two additional parameters -- the

hardware interrupt number and the I/O address.  The defaults are

3 and 0x300.

Racal-Interlan (Formerly Micom-Interlan) NI5210

usage: ni5210 <packet_int_no> <int_no> <io_addr> <base_addr>

The NI5210 requires three additional parameters -- the

hardware interrupt number, the I/O address, and the memory base

address.  The defaults are 2 and 0x360 and 0xd000.  Note that In-

terlan sets the default memory base to 0xa000, which is

brain-damaged, because that area of memory is specifically

reserved for video adapters, and in fact the EGA and VGA use it.

SLIP8250

usage: SLIP8250 packet_int_no [driver_class] [int_no]

[io_addr] [baud_rate] [send_buf_size] [recv_buf_size]

The driver_class should be SLIP, KISS, AX.25, or a

number.

The SLIP8250 is not strictly an Ethernet adapter, however

some software packages (such as KA9Q's NET) support Serial Line

IP (SLIP).  SLIP must be specially supported because it doesn't

use ARP and has no hardware addresses prepended to its packets.

The PDS is not clear on this, but the packet driver does the SLIP

encoding.

The parameters are as follows.  The driver_class is the class

that is returned to a client of the packet driver spec in the

Drive_Info call.  The int_no is the hardware interrupt number,

defaults to 3 (COM1).  The io_addr is the hardware I/O address,

defaults to 0x3f8 (COM1).  The baud_rate defaults to 4800 baud.

The send_buf_size and recv_buf_size default to 3000 each.

Western Digital WD8003E,WD8003ET/A

usage: WD8003E <packet_int_no> <int_level> <io_addr>

<mem_base>

The WD8003E driver is also for the WD8003ET/A, a

MicroChannel Ethernet adapter.  The WD8003E requires three addi-

tional parameters -- the hardware interrupt number, the I/O

address, and the memory base address.  The defaults are 2 and

0x280 and 0xc400.

Who?

See the file support.txt for a listing of all the people who

contributed packet drivers.

Appendix A

Interrupt usage in the range 0x60 through 0x80

60 -- -- reserved for user interrupt

60 -- -- FTP Driver - PC/TCP Packet Driver Specification

60 01 FF FTP Driver - DRIVER INFO

60 02 -- FTP Driver - ACCESS TYPE

60 03 -- FTP Driver - RELEASE TYPE

60 04 -- FTP Driver - SEND PACKET

60 05 -- FTP Driver - TERMINATE DRIVER FOR HANDLE

60 06 -- FTP Driver - GET ADDRESS

60 07 -- FTP Driver - RESET INTERFACE

60 11 -- 10-NET - LOCK AND WAIT

60 12 -- 10-NET - LOCK

60 13 -- 10-NET - UNLOCK

60 20 -- FTP Driver - SET RECEIVE MODE

60 21 -- FTP Driver - GET RECEIVE MODE

60 24 -- FTP Driver - GET STATISTICS

61 -- -- reserved for user interrupt

62 -- -- reserved for user interrupt

63 -- -- reserved for user interrupt

64 -- -- reserved for user interrupt

65 -- -- reserved for user interrupt

66 -- -- reserved for user interrupt

67 -- -- LIM EMS

...

67 DE 00 Virtual Control Program Interface - INSTALLATION CHECK

...

68 01 -- APPC/PC

...

69 -- -- unused

6A -- -- unused

6B -- -- unused

6C -- -- system resume vector (CONVERTIBLE)

6C -- -- DOS 3.2 Realtime Clock update

6D -- -- VGA - internal

6E -- -- unused

6F -- -- Novell NetWare - PCOX API (3270 PC terminal interface)

6F 00 -- 10-NET - LOGIN

...

70 -- -- IRQ8 - AT/XT286/PS50+ - REAL-TIME CLOCK

71 -- -- IRQ9 - AT/XT286/PS50+ - LAN ADAPTER 1

72 -- -- IRQ10 - AT/XT286/PS50+ - RESERVED

73 -- -- IRQ11 - AT/XT286/PS50+ - RESERVED

74 -- -- IRQ12 - PS50+ - MOUSE INTERRUPT

75 -- -- IRQ13 - AT/XT286/PS50+ - 80287 ERROR

76 -- -- IRQ14 - AT/XT286/PS50+ - FIXED DISK

77 -- -- IRQ15 - AT/XT286/PS50+ - RESERVED

78 -- -- not used

79 -- -- not used

7A -- -- Novell NetWare - LOW-LEVEL API

7A -- -- AutoCAD Device Interface

7B -- -- not used

7C -- -- not used

7D -- -- not used

7E -- -- not used

7F -- -- HDILOAD.EXE - 8514/A VIDEO CONTROLLER INTERFACE

7F -- -- HLLAPI (High-Level Language API)

80 -- -- reserved for BASIC