*** FREEWARE ***

Copyright (c) 1993 by Radiodetection Corp. & Russ Bellew

All Rights Reserved.

Program:        DUPLOGIN  version 1.1

Purpose:        Determine if the user is logged on at other stations on

a Novell Netware local area network.

Tested with:    Workstation:  DOS 3.21, 3.3, 5.0

IPX 3.01, 3.04, 3.10

NET3 3.01, NETX 3.10, 3.22, 3.36, 3.31

File Server:  Netware 386 3.10, 3.11

ELS II 2.12

Written:        June 20, 1992,  by Russ Bellew

Typical Application:

On Novell Netware LANs, it's common for some users to request the

ability to login to a file server at more than one station concur-

rently.  However, at day's end, such users (including myself)

frequently forget to log off at their other connections.  This

leaves their files and email messages vulnerable to compromise.

A solution:

DUPLOGIN reminds users when they're logged on elsewhere.  It may

be run at any time.  For instance, by using the system login script

and DOS batch files, I have every user run DUPLOGIN during both the

logging in and logging out processes.

If you execute DUPLOGIN and you're not logged in elsewhere, nothing

will be displayed.

If you execute DUPLOGIN and you *are* logged in elsewhere, you will

be told on which network and station you are logged, as well as the

login time for each connection.

Memory Usage:

DUPLOGIN runs in less than 128 k.  It's *not* a TSR; after running,

it completely removes itself from memory.

Getting more information:

Unfortunately, with the standard DUPLOGIN display, the Network and

Station addresses (in hexadecimal notation, no less!) mean little

to most users.

DUPLOGIN can read a separate text file that contains easy to understand

descriptions of each station at your site. (This file may have any file

name, as long as its name is defined on the command line that you use to

start DUPLOGIN.EXE.)  If you use Fresh Technology's LAN Assist Plus,

your customized copy of its LA.DAT file will work nicely.

To tell DUPLOGIN to read this file, start it with the -F command

line parameter:

DUPLOGIN -Fdrive:path\filename

For example, if my system's LA.DAT file were located

in F:\LANASIST\, I'd run DUPLOGIN with the command line:

DUPLOGIN -FF:\LANASIST\LA.DAT

IMPORTANT:  The user (or his group) must have at least Search

and Read rights to the directory in which your text file is located.

Here's a portion of a typical LAN ASSIST station description file:

1       0A      Acctg - Kim's 16 MHz 386SX VGA, 1st floor

1       0B      Acctg - Viola's 16 MHz 386SX VGA, Tape backup

1       14      Parts Room - 80286 CGA

2       15      Diane's 80286, monochrome, 1st floor

3 80ED000BD1    Parallel port to Ethernet adapter - IQ tech

1 00001B223C88  Russ's monochrome 80286 desktop, 1st floor

Each line contains 3 fields:

Network address (in hex); Node address (in hex); Station Description

Let's examine line 1 of this file:

-Network- ----Node----  --------------------Description-----------------------

1       0A      Acctg - Kim's 16 MHz 386SX VGA, 1st floor

On each line,

The network address field occupies characters 1 through 9;

The node address field occupies characters 11 through 22;

The station description field occupies characters 25 through 79;

Each field may have as many leading or trailing spaces as you want.

The stations may be in any order within the station description file.

A station with a Node Address of 854 may be placed either before or

after a station with a Node Address of 853.  (DUPLOGIN uses a simple

sequential search of the station description file.)

You should flag your station description file as SHAREABLE.

Using DUPLOGIN In Batch Files:

DUPLOGIN's display may be redirected to a DOS device (such as NUL).

DUPLOGIN returns DOS errorlevels as follows:

Errorlevel      Meaning

255        Error:  Netware shell isn't loaded at this

station or no user is logged in at this station.

254        Error:  Filename (following -F command line parameter)

contained wildcard characters (i.e., '?' '*').

1          User is logged in on at least one other station.

(See note below)

0          User isn't logged in at any other stations.

Note: if DUPLOGIN is started with the QTY command line parameter,

then upon exiting it sets the DOS errorlevel equal to the

quantity of other stations at which this user is logged in.

These errorlevels may be used for subsequent branching within your

batch files.  For example,

ECHO OFF

CLS

DUPLOGIN -FF:\LANASIST\LA.DAT CLEAR PAUSE

IF ERRORLEVEL 1 GOTO CONTINUE

:LoadEmailTSR

REM  Load eMail TSR *only* if the user isn't already logged on elsewhere

F:

CD \EMAIL\MAILUSER

EMAILTSR

:CONTINUE

...

CLEAR - You may use the command line parameter CLEAR to clear the screen

before the other logins are displayed.

PAUSE - You may use the command line parameter PAUSE to pause the

display until the user strikes a key.

If you want to pause the display, but wish DUPLOGIN to exit if either

the user presses a key *or* a number of seconds elapse, do this:

Follow the PAUSE command line parameter with the maximum number of seconds

for each of its displays.  For example, if you want DUPLOGIN to exit

after 30 seconds (or if the user presses a key) use the command line:

DUPLOGIN PAUSE 30

(My thought here was to allow all bindery objects - both human and

otherwise - to run DUPLOGIN from the Novell system login script and

get a paused display, but not stall out if there's nobody at the

keyboard.)

NAG - If duplicate logins are found, DUPLOGIN will display the message

"Please remember to log off the other station(s) before leaving."

VERBOSE - Normally, DUPLOGIN displays nothing if it can't find other

stations at which you're logged on.  If, however, you start it with the

command line parameter VERBOSE it will (1) display a message even if

you aren't logged on elsewhere and (2) if you are logged on elsewhere,

it will always display the network and node addresses, as well as the

plain station descriptions from your station description file.  You'd

normally use the VERBOSE command line parameter only when setting up

or debugging DUPLOGIN.

A REEEAAALLY LONG COMMAND LINE (that works, even) -

DUPLOGIN -FF:\LANASIST\LA.DAT CLEAR PAUSE 10 NAG VERBOSE

Novell System Login Script:

In your file server's System Login Script, try a command such as

#DUPLOGIN -FF:\LANASIST\LA.DAT CLEAR PAUSE 60

Technical Stuff:

DUPLOGIN reads the system clock from the file server (not the station).

Leading zeros in network and station addresses are ignored.

Upper- and Lower- case inputs are treated equally.

Command line parameters may be placed in any order on the command line.

The command line parameter HELP tells DUPLOGIN to display a brief help

screen.

You should flag your station description file as SHAREABLE.

DUPLOGIN doesn't check directory user rights.  If it can't find or

read your station description file, maybe you don't have sufficient

rights in that directory.

If your station description file can't be found in the directory

that you've specified, DUPLOGIN will search for it (1) in the current

directory, (2) in DUPLOGIN.EXE's directory (on DOS 3.0 and above),

and (3) along the DOS path.

Once found, DUPLOGIN makes three attempts to open your station

description file in Read Only, Deny Writes mode, with one second

intervals between attempts.  It keeps this file open for no more than

a few seconds.  Such attempts to open and read this file will fail if

somebody else has locked the file (such as when they're editing it).

Command line parameters may be abbreviated, placed in any order on the

command line, and in either upper- or lower- case.  Just the first

letter of each parameter works fine

(e.g., "DUPLOGIN FF:\LANASIST\LA.DAT C P N").

Legal Stuff:

This program is copyright 1993 by Radiodetection Corp. and Russ Bellew.

You may use this program as you wish, without payment of any kind

to its copyright holder.  If you ever need to locate buried utility

lines, contact Radiodetection for the world's best buried utility

locating equipment!

Copyright (c) 1993

Radiodetection Corp.

35 Whitney Road

Mahwah, NJ  07430  USA

tel 201 848-8070

fax 201 848-1303

MCImail: RADIODETECTION

ATTmail: !rdcorp

Author: Russ Bellew, Compuserve 75740,3133

Revisions:

Jul 1, 92  - Fixed network address (when > 0F) reporting error.

Jul 5, 92  - Fixed 12 am to 1 am "0:XX am" time display bug.

Jul 6, 92  - Sped up text file reads by increasing file buffer size to 25k.

Jul 6, 92  - Added rejection of wildcard file names.

Jul 7, 92  - Added file search along DOS path.

Jul 8, 92  - Cleaned up command line param parsing and file initialization.

Jul 11, 92 - Now displays duplicate logins chronologically (oldest

first).  Fixed 12:00 - 12:59 pm time display bug.

Jul 12, 92 - Installed quicksort (was insertion sort).

Jul 15, 92 - Fixed text file open filemode share problem.

Jul 16, 92 - Standardized delay seconds.

Dec 27, 92 - Fixed bug when trying to match station addresses in LA.DAT

that end with 0.

Mar 22, 93 - Released.

< End of duplogin.doc file >