ADDENDUM TO

THE NETWARE INSTALLATION SUPPLEMENT

FOR 3COM ETHERLINK

AND ETHERLINK PLUS (REV 1.00)

You should use this addendum with the NetWare Installation Supplement for

3Com EtherLink and EtherLink Plus (revision 1.00, part #100-000348-001).

The information in this addendum applies only if you are installing EtherLink

II boards (series 3C503) in network stations on a NetWare v2.1x network.

This addendum explains how to set EtherLink II boards.  These boards can

be used in IBM PC, PC XT, PC AT, or compatible computers, and in the IBM

Personal System/2 Model 30 computer.  EtherLink II boards can be used in

workstations, file servers, and bridges; they can be used with thin cable,

thick cable, or a combination of the two.

Before you use this addendum, you should read Chapter 1 of the Netware

Installation Supplement for 3Com EtherLink and EtherLink Plus.  Then follow

the instructions in this addendum to set your EtherLink II boards.  After

you have set your EtherLink II boards, refer to "Inserting EtherLink

Network Interface Boards into Network Stations" in Chapter 2 of the

installation supplement to complete your installation.

NOTE:  If you are installing NetWare ELS v2.1x, you should use the

ELSGEN installation utility, rather than NETGEN, to generate the

operating system.

LABELING ETHERLINK II NETWORK INTERFACE BOARDS

Before you can set your Etherlink II network interface boards, you must

decide which board to install in each workstation.

Refer to the sketch of the network that you made in Chapter 1 of the

installation supplement and to the worksheets that you filled out when you

generated the NetWare operating system or shell for each station.  (The

worksheets came with the main NetWare installation manual.)  Then, using a

string tag or some other form of identification, label each board with the

following information:

o    The specific network station into which the board will be inserted

(for example, "Gloria's workstation by the main door").

o    The kind of station into which the board will be inserted:

workstation, file server, or bridge.

o    The configuration option (this includes the connector type, DMA

channel, interrupt line, base I/O address, and base memory

address).  If the board will be installed in a file server or bridge,

label the board with the configuration option that you chose when

you generated the NetWare operating system.  Boards installed in

the same file server or bridge cannot have the same

configuration option number.  However, boards installed in

different file servers or bridges can.

If the board will be installed in a workstation, label the board

with the configuration option that you chose when you generated

the NetWare shell to be used in that particular workstation.

MAKING SETTINGS ON ETHERLINK II BOARDS

Configuration option settings for EtherLink II boards consist of the

following:

o    Connector type

o    DMA channel

o    Interrupt line

o    Base I/O address

o    Base memory (RAM) address

Connector type, DMA channel, and interrupt line settings are determined by

the configuration option you selected in the NETGEN (or ELSGEN), SHGEN,

or BRGEN utility.  You do not have to make these settings on the board.

Base I/O address and base memory address settings are made by mounting

shorting plugs on jumper blocks.

If you have configured the operating system (for file servers or bridges) or

the shell (for workstations) with an invalid option, your EtherLink II boards

will not work.  You must reconfigure the operating system or shell by

running NETGEN (or ELSGEN), SHGEN, or BRGEN and selecting a valid

option.

NOTE:  The Remote Reset feature allows workstations to boot from a

network (file server) disk.  NetWare v2.1x does not currently support

this feature for the EtherLink II (series 3C503) board.

The following sections explain each of the configuration option settings.

CONNECTOR TYPE

EtherLink II boards have both a DIX connector and a BNC connector on the

metal mounting bracket.  Use the DIX connector to connect to thick cable.

Use the BNC connector to connect to thin cable.

Options 0 through 15 are valid only with thin cable; options 16 through 31

are valid only with thick cable.

DMA CHANNEL

EtherLink II boards allow information to be processed either through DMA

(direct memory access) or RAM.  If you selected DMA, you must set the

base memory address to DISABLE.

INTERRUPT LINE

The following configuration option table shows the interrupt line (IRQ) used

with each configuration option.

Base      Base                Cable

Option    IRQ       I/O       Memory    DMA       Connector

0         3         300h      C8000     none      BNC

1         2         310h      CC000     none      BNC

2         4         330h      D8000     none      BNC

3         5         350h      DC000     none      BNC

4         3         250h      Disabled  1         BNC

5         2         280h      Disabled  3         BNC

6         4         2A0h      C8000     none      BNC

7         5         2E0h      CC000     none      BNC

8         3         300h      D8000     none      BNC

9         2         310h      DC000     none      BNC

10        4         330h      Disabled  1         BNC

11        5         350h      Disabled  3         BNC

12        3         250h      C8000     none      BNC

13        2         280h      CC000     none      BNC

14        4         2A0h      D8000     none      BNC

15        5         2E0h      DC000     none      BNC

16        3         300h      C8000     none      DIX

17        2         310h      CC000     none      DIX

18        4         330h      D8000     none      DIX

19        5         350h      DC000     none      DIX

20        3         250h      Disabled  1         DIX

21        2         280h      Disabled  3         DIX

22        4         2A0h      C8000     none      DIX

23        5         2E0h      CC000     none      DIX

24        3         300h      D8000     none      DIX

25        2         310h      DC000     none      DIX

26        4         330h      Disabled  1         DIX

27        5         350h      Disabled  3         DIX

28        3         250h      C8000     none      DIX

29        2         280h      CC000     none      DIX

30        4         2A0h      D8000     none      DIX

31        5         2E0h      DC000     none      DIX

BASE I/O ADDRESS AND BASE MEMORY ADDRESS

To set the base I/O address and base memory address on Etherlink II boards,

complete the following steps for each board:

1)   Identify the configuration option you should use by checking the tag

attached to the board.

2)   Find the appropriate configuration option in the table located above.

3)   Set the base memory address and base I/O address by mounting

shorting plugs on the appropriate jumper blocks.

COPYING ETHERLINK II DRIVER FILES FOR NETWARE VERSION 2.1x

If you are installing a NetWare v2.1x network containing EtherLink II boards, you

will need to copy the 3C503.LAN driver file from the driver update diskette to the

AUXGEN diskette you received with your NetWare shipment before you run the

following NetWare v2.1x installation utilities:

o    NETGEN, used to generate the network operating system

o    BRGEN, used to generate a NetWare bridge (if applicable)

o    SHGEN, used to generate your workstation shells

1)   Boot a computer with DOS.

2)   Insert the diskette containing the new 3C503 drivers into drive A.

3)   Use the DOS LABEL command to create a label "LAN_DRV_108" on the driver

update diskette.

4)   Copy the EtherLink II 3C503.LAN file from the driver update diskette to the

AUXGEN diskette.

WHERE TO GO FROM HERE

When you have finished setting your EtherLink II boards, refer to "Inserting

EtherLink Network Interface Boards into Network Stations" in Chapter 2 of

the installation supplement.