INSTRUCTIONS FOR LOADING AND USING THE KODIAK DOS ODI DRIVER

============================================================

1.   Installation

To install an ODI driver, 4 loadable modules need to be loaded.

A typical AUTOEXEC.BAT for an ODI workstation follows:

LSL

K16OD

IPXODI

NETX

F:

LOGIN

2.   Configuration

In addition, there is a control file named NET.CFG that is used to

set the base address, interrupt line and frame type for the Kodiak

card. A typical NET.CFG for an ODI workstation follows:

Link driver K16OD

Int  9

Port 200

Frame     ETHERNET_II

The following frame types are supported by the ODI driver:

ETHERNET_802.2

ETHERNET_802.3

ETHERNET_II

ETHERNET_SNAP

NOTE:     In the NET.CFG file, the syntax is critical. The first

line referring to the Link Driver must start in column

one. Also, the words must be separated by a space and not

a TAB or other control character. The other parameter

lines may not start in column one. The first character

must be a TAB and the words must be separated by a TAB

and not a space or other control character.