Installing NDIS Drivers for Banyan VINES DOS-based Workstations

===============================================================

Before the installation, refer to the VINES Network Installation

Guide.

In order to create a VINES workstation boot diskette perform the

following steps:

1.   Obtain a new 3.5" or 5.25" diskette.

2.   Run the DOS format utility on the diskette and specify

the /s option. This will create a bootable DOS diskette.

You must use Version 3.01 of DOS or a later version.

3.   Logon to the VINES server from a workstation.

4.   Place the diskette formatted in step 2 in the A: drive of

the VINES workstation.

5.   Run the VINES utility PCCOPY at the workstation.

6.   From the menu, select "PC CONFIG".

7.   From the menu, select "NDIS Ethernet".

8.   From the menu, select drive A:.

9.   Enter the REDIRECTOR menu and select REDIR3.EXE or

REDIRALL.EXE. The VINES software is now copied to your

new boot disk.

10.  Change to drive A: by typing "A:".

11.  Start the configuration utility by typing "PCCONFIG".

12.  Select INTERRUPT LEVEL to set the interrupt. The Kodiak

cards are set to IRQ 3 at the factory. To change the card

setting, see the hardware installation manual.

13.  Set the BASE ADDRESS. The Kodiak cards are set to IO

address 300 at the factory. To change the card setting,

see the installation manual.

14.  Save the settings by pressing F10.

15.  Return to the main menu by pressing ESC.

16.  Save the configuration by pressing F10.

17.  The CONFIG.SYS on the boot diskette will need to be

modified so that the drivers will be installed on

boot-up. Add the following two lines to the CONFIG.SYS

file:

DEVICE=PROTMAN.DOS

DEVICE=KOD16.DOS

- OR -

DEVICE=PROTMAN.DOS

DEVICE=KOD8.DOS

18.  The drivers from the Kodiak~8 or Kodiak~16 distribution

disk now need to be copied on to the boot disk. To create

a directory for the drivers enter the following DOS

command:

MKDIR A:\LANMAN

19.  Copy all files from the \MSLANMAN.DOS\DRIVERS\ETHERNET

directory on the distribution diskette to the A:\LANMAN

directory on the VINES boot diskette.

Two files will be copied. For the Kodiak~8 the two files

are KOD8.DOS and PROTOCOL.INI. For the Kodiak~16 the two

files are KOD16.DOS and PROTOCOL.INI.

20.  Check the PROTOCOL.INI file to ensure that it matches the

hardware configuration selected in steps 12 and 13.

EXAMPLE 1. PROTOCOL.INI for Kodiak~8

; protocol.ini for Kodiak~8 Ethernet adapter card

[KOD8_NIF]

DRIVERNAME = KOD8$

INTERRUPT = 3

IOADDRESS = 0x300

EXAMPLE 2. PROTOCOL.INI for Kodiak~16

; protocol.ini for Kodiak~16 Ethernet adapter card

[KOD16_NIF]

DRIVERNAME = KOD16$

INTERRUPT = 3

IOADDRESS = 0x300

21.  You now can connect to the VINES network by re-starting

the new workstation with the new boot diskette in drive

A:.