TIARA Computer Systems, Inc.

(C) Copyright 1990.  All Rights Reserved.

---------------------------------------------------------------------------

NETBIOS/pc Version 1.1 Installation

---------------------------------------------------------------------------

This product is intended to provide a simple mechanism for the evaluation

of this NetBIOS emulator in conjunction with NetBIOS-based network

operating systems and applications.

IMPORTANT:  This NetBIOS is provided for evaluation purposes only and is

therefore limited to one hour of execution time.

An unlimited version of the this NetBIOS product may be purchased from:

Micro Computer Systems, Inc.

2300 VALLEY VIEW LANE

SUITE 800

Irving, Texas 75062, USA

(214) 659-1514

A NetBIOS based operating system, called TRIAC/pc, is included to aid in

evaluation of this product.

To simplify the installation, the product is delivered in a self-expanding

utility file.  Simply copy the "NETBIOS2.EXE" file to a designated

directory and execute the file by typing: NETBIOS2<return>

This will extract the following modules:

Filename       Description

------------   --------------------------------------------------

LNETBIOS.EXE   NetBIOS module

DLMGR.EXE      Data Link Manager

PXNS.EXE       XNS Protocol stack

DEVTE.EXE      Tiara Ethernet Driver

MCSNET.EXE     "2A" NetBIOS interface

NET.EXE        Command line parser

NETNAME.EXE    Naming Utility

UTIL.EXE       General Utility

NETGO.BAT      Executes NetBIOS with "2A" NetBIOS interface

NETB.BAT       Executes NetBIOS without "2A" NetBIOS interface

An extended manual for this NetBIOS emulator (file name:  NBREAD.ME) can be

downloaded from Tiara's BBS:

Telephone #:  1-415-966-8533       Baud Rate:  300/1200/2400

Word Size:  8 bits

Parity   :  None

Stop Bits:  1 bit.

___________________________________________________________________________

Document:  NETBIOS.DOC      November 27, 1990                        Page 1

TIARA Computer Systems, Inc.

(C) Copyright 1990.  All Rights Reserved.

---------------------------------------------------------------------------

NETBIOS/pc Version 1.1 Installation

---------------------------------------------------------------------------

Trademark information:

NetBIOS is a trademark of International Business Machines

Corporation.

Ethernet is a registered trademark of Xerox Corp.

TRIAC/pc and NETBIOS/pc are trademarks of Micro Computer Systems,

Inc.

Network-OS is a trademark of CBIS, Inc.

IBM is a registered trademark of International Business Machines

Corporation.

LANtastic NOS is a trademark of Artisoft, Inc.

10Net is a trademark of DCA Communications, Inc.

Modifications to the startup batch files:

This package includes two run-time batch files which are used to load

and start the NetBIOS emulator with or without support for software

interrupt 2Ah:

NETGO.BAT which executes NetBIOS with interrupt 2Ah support, and

NETB.BAT which executes NetBIOS without interrupt 2Ah support.

If the TIARA LanCard/E adapter is jumpered for an I/O address other

than 0300h, or an interupt request level other than 3, use any ASCII

text editor (such as EDLIN) to modify the line in the NETGO.BAT or

NETB.BAT file which reads "DEVTE" as follows:

DEVTE /IN:i /PN:ppp

Where:    "i" should be replaced with the desired

interrupt request level.

"ppp" should be replaced with the desired I/O

address.

For Example, if the LanCard/E adapter is using I/O

address 320h and interrupt request level 4, the new

line would read:

DEVTE /IN:4 /PN:320

___________________________________________________________________________

Document:  NETBIOS.DOC      November 27, 1990                        Page 2

TIARA Computer Systems, Inc.

(C) Copyright 1990.  All Rights Reserved.

---------------------------------------------------------------------------

NETBIOS/pc Version 1.1 Installation

---------------------------------------------------------------------------

Executing NETBIOS with Interrupt 2Ah support:

These instructions apply to network operating systems that

require a NetBIOS interface which supports software interrupt

vector 2Ah; such as MCS's TRIAC/pc and Artisoft's LANtastic NOS.

1.   For Artisoft's LANtastic NOS, using any ASCII text editor (such

as EDLIN), remove the line in the NETGO.BAT file which reads:

NET NAME NAME1

2.   Execute the NetBIOS emulator by entering the command:

NETGO<return>

Executing NETBIOS with Interrupt 2Ah support:

These instructions apply to network operating systems that DO NOT

require a NetBIOS interface which supports software interrupt

vector 2Ah; such as IBM PC LAN, DCA 10Net, and CBIS Network-OS.

1.   Execute the NetBIOS emulator by entering the command:

NETB<return>

___________________________________________________________________________

Document:  NETBIOS.DOC      November 27, 1990                        Page 3