MPRPER.EXE; MPR Performance

23SEP94

NOVELL PRODUCT and VERSION:

NetWare MultiProtocol Router 2.0

NetWare MultiProtocol Router 2.1

NetWare MultiProtocol Router 2.11

NetWare MultiProtocol Router PLUS 2.1

NetWare MultiProtocol Router PLUS 2.11

-----------------------------------------------------------------

DISCLAIMER

THE ORIGIN OF THIS INFORMATION MAY BE INTERNAL OR EXTERNAL TO NOVELL.

NOVELL MAKES EVERY EFFORT WITHIN ITS MEANS TO VERIFY THIS INFORMATION.

HOWEVER, THE INFORMATION PROVIDED IN THIS DOCUMENT IS FOR YOUR INFORMATION

ONLY.  NOVELL MAKES NO EXPLICIT OR IMPLIED CLAIMS TO THE VALIDITY OF THIS

INFORMATION.

-----------------------------------------------------------------

NOTE: The tables in the document will not make sense unless you use the

following parameters.

- Left Justification

- Courier 10

- 5 character margin on each side.

Performance Benchmarks

Product Description

The NetWare MultiProtocol Router and NetWare MultiProtocol Router Plus

products are software based routers and source route bridges that run on

80386, 80486, and Pentium-based Pcs.  These routers enable you to build

global internetworks using the familiar NetWare" operating system and

standard PC technology.  These router products have the same routing

capabilities as the previous release (concurrent routing of IPX, IP,

AppleTalk, and OSI over the same interface).  Both router products also

have a new capability to concurrently bridge (over source route bridged

token ring networks) any protocol that is not configured to be routed,

including IPX, IP, IBM, SNA, and IBM NetBIOS. In addition, NetWare

MultiProtocol Router Plus provides wide area routing and source route

bridging over synchronous PPP and X.25 networks.  The NetWare MultiProtocol

Router products enable you to build bridge/router configurations that

minimize cost, maximize performance, and optimize applications.

Performance Considerations

In evaluating the performance of the NetWare router software, you should

consider a number of factors; performance is more than just a frame per

second (fps) measurement.  Although Novell's performance recommendations

include fps rates, this guide also provides helpful hints on optimizing a

router configuration to meet your internetworking requirements for

maximizing connectivity and throughput.

The factors that affect router performance include the following:

Data frame sizes and data throughput requirements

Client and server configuration

RAM, hard disk, and controller requirements

PC bus type and speed

CPU type

Network interface boards

Data Frame Size and Data Throughput Requirements

Most performance benchmarks provide measurements for 64-byte frames.

However, most network traffic has a frame size of at least 512 bytes, and

frame sizes can reach up to 1,518 bytes (Ethernet) or 17,900 bytes (16 Mbps

token ring).  To determine which frame sizes you transmit on your networks,

use the following guidelines:

1.  Look at the frame size transmitted by your applications. Setups and

acknowledgments are usually 64-byte frames, whereas data frames are usually

512-byte frames or larger.

2.  Look at the protocols you are using. IPX, for example, has 560-byte

data frames and AppleTalk has a maximum frame length of 603 bytes.

3.  Look at the LAN types you are transmitting across. Ethernet, token

ring, ARCnet, LocalTalk, and FDDI all have different maximum frame length

requirements.

Table 1

LAN Types and Frame Sizes

LAN Type               Frame Size

--------               ----------

Ethernet               1,518 bytes

4 Mbps token ring      4,472 bytes

16 Mbps token ring    17,900 bytes

ARCnet                 4,202 bytes

LocalTalk                603 bytes

FDDI                   4,500 bytes

4.  Frame size has a direct impact on the data throughput of your network.

Data throughput is the amount of traffic you can transmit among interfaces

before dropping frames. In general, smaller frames require more frames per

second and more overhead per amount of data transfer, resulting in lower

data throughput. Larger frames require fewer frames per second and lower

overhead per amount of data transfer, resulting in higher data throughput.

5.  Frame size not only affects throughput, it also plays a role in the

type of system and LAN boards you choose. For example, if your network is

dominated by smaller frames, you need a PC and network interface boards

that can handle a large number of interrupts.

6.  Frame size and throughput requirements also affect the number of

interfaces that you should have on your router. When using an Intel-based

system (with a given performance) as a router connecting networks that use

mostly small frames, it should have fewer interfaces than if it is used as

a router connecting networks that use larger frames.

Client and Server Configuration

Two tunable factors affect throughput: the size of the packet being used

and the number of packets that can be transmitted without receiving an

acknowledgment.

For IPX, two protocols and related software allow two parameters to be

tuned. The packet burst protocol allows a burst of data of up to 64

kilobytes to be sent with only a single acknowledgment required. The large

internet packet protocol allows the Maximum Transmission Unit (MTU) for the

path to be negotiated to something larger than the default 560 bytes. These

utilities, used separately or together, increase the efficiency of the data

transfer between IPX servers and workstations across the router.

For IP, certain implementations allow the tuning of the MTU/MRU (Maximum

Receive Unit), and the TCP window size. For example, if your MTU is 512

bytes and your TCP window size is 4096 bytes, you can have 8 packets of 512

bytes outstanding.

RAM, Hard Disk, and Controller Requirements

The amount of RAM required in the PC router depends on your connectivity

needs. For routers with many interfaces and higher throughput, you should

have at least 16 megabytes of RAM. This allows you to set your maximum

receive buffers to 2000, preventing frame loss due to buffer overruns.

Router with fewer interfaces and lower throughput, require only 8 megabytes

of RAM.

In evaluating hard disks and controllers, verify that the disks and LAN

boards are compatible. This is especially important if you are installing

the NetWare router on a NetWare v3.11 server. With controllers, make sure

that they can handle the high throughput requirements of frames that use

burst mode.

PC Bus

You have three PC bus choices for your router: Industry Standard

Architecture (ISA), Enhanced Industry Standard Architecture (EISA), and

Micro Channel Architecture (MCA). Table 2 shows the instantaneous transfer

rates for a burst of words at the bus speed of 8 Mhz (using the bus clock,

BCLK). For higher performance routing and source route bridging, Novell

recommends an EISA or MCA bus.

Table 2

PC Bus Types and Transfer Rates

Bus Type      Peak Transfer Rate

ISA 8-bit      1 MBps (8 Mbps)

ISA 16-bit      2 MBps (16 Mbps)

EISA 32-bit burst   33 MBps (264 Mbps)

MCA 32-bit burst   40 MBps (320 Mbps)

CPU

The router runs on 80386, 80486, or Pentium based PCs.  Common clock speeds

for these processors are 16, 20, 25, 33, and 40 MHz for 80386 systems, 20,

25, 33, 50, and 66 MHz for 80486 systems, and 60 and 66 MHz for Pentium

PCs. Novell recommends that you choose a CPU that meets the routing speed

requirements for your network configuration.  A general rule of thumb is

the faster the CPU, the higher the routing performance.

Table 3 shows the speed rating of each PC platform.  The speed rating is

provided by the NetWare v3.11 server using the speed command.  The speed

rating ranks a bridge/router performance with respect to CPU types, clock

speeds, memory, and cache. The higher the rating, the faster the system.

Table 3

Speed Ratings for 80386 and 80486 Systems

PC Platform      Speed Rating*1

-----------      -------------

80386/16 MHz       121

80386/25 MHz       242

80386/33 MHz       322

80486/25 MHz       686

80486/33 MHz       915

80486/50 MHz      1372

80486/66 MHz      1830

*1 The speed rating is the number of the last pattern returned from the

CPU. The greater the number, the faster the system.

Network Interface Boards

There are two types of boards: bus-mastered boards and slave boards.

Bus-mastered boards have an on-board processor that places a frame in a PC

buffer without intervention from the CPU. This type of board reduces

processing cycles and increases performance.

Slave boards need a PC's CPU to remove a frame from the network interface

board and put it into the PC buffer. The use of slave boards in a

bridge/router can adversely affect performance.

Classifying Router Configurations

Router configuration has been classified based on the number of interfaces

and throughput. Larger sites have a concentration of departmental networks

and provide connectivity to remote offices. These routers typically have

many local and remote interfaces and higher throughput needs, based on the

types of LANs used (Ethernet, token ring 16/4, or FDDI) and the number of

users (see Table\x114 and Table\x115). Smaller sites include regional

offices which typically have fewer departmental LANs and wide area

connections (see Table\x116).  Some smaller and remote sites only require

two LAN connections or one LAN and one WAN connection (see Table\x117).

Note that the following classifications show the minimum CPUs required.

More powerful PCs can be used, but the slight increase in performance may

not justify the increased cost. However, if you plan to upgrade your

network or servers at a later date, you may want to take into account the

increased router requirements associated with your migration path.

Router Configurations for Larger Sites

Table 4

Many Interfaces and Higher Throughput Configuration

Host Bus   MCA or EISA with 6 or more available slots

Host CPU   486/DX2 66 MHz

LAN Connections   Allows up to 6 bus master LAN boards

WAN Connections   Allows up to 16 WAN ports with each port supporting

speeds greater than 1 Mbps

Table 5

Fewer Interfaces and Higher Throughput Configuration

Host Bus   MCA or EISA with 4 or more available slots

Host CPU   486/DX 33, 50 MHz

486/DX2 50, 66 MHz

LAN Connections Allows up to 4 EISA slave boards

WAN Connections   Allows up to 12 WAN ports with each port supporting

speeds less than 512 Kbps*1

*1 You can use T1 lines with any of these router classifications. Just

build a router using less then the maximum WAN connections listed for each

class of router.

Router Configurations for Smaller Sites

---------------------------------------

Table 6

Many Interfaces and Lower Throughput Configuration

Host Bus   ISA with 6 or more available slots

Host CPU   386/33 MHz

LAN Connections   Allows up to 4 ISA boards

WAN Connections   Allows up to 8 WAN ports with each port supporting

speeds less than 1 Mbps

Table 7

Fewer Interfaces and Lower Throughput Configuration

Host Bus   ISA with at least 2 available slots (after installation of

video or disk controller)

Host CPU   386/20 MHz

LAN Connections   Allows at least 2 LAN boards

WAN Connections   Allows up to 4 WAN ports with each port supporting

speeds less than or equal to 56 Kbps

EVALUATING PERFORMANCE BENCHMARKS

Types of Benchmarks

Performance measurements for bridges and routers are based on the number of

frames per second (fps) forwarded. This measurement tells network managers

how many frames a bridge or router can process and transmit per second.

Standard performance testing uses a minimum frame size (64 bytes). Actual

networks use a combination of small and large frames. For example, a

64-byte frame is used with data requests and terminal emulation, while a

512-byte frame is used with data and file transfers.

Selection Criteria - A User's Perspective

The fps is not the most useful measurement for network managers. Novell

recommends evaluating the data throughput and bandwidth utilization of a

bridge or router, because these measurements more accurately predict the

rate of data flow on your network.

Throughput

The maximum rate at which none of the offered frames are dropped by the

device. Because the loss of even one frame in a data stream can cause

significant delays while waiting for the higher level protocols to time

out, it is useful to know the actual maximum data rate that the device can

support. Table 8 and Table 9 show some maximum theoretical throughput

rates.

Table 8

Theoretical Throughput (Packets per Second) with an Ethernet LAN as the

Source

Frame  Packet  pps       pps       pps       pps       pps

Size   Size    from      from      from      from      from

IPX                    Ethernet  Ethernet  Ethernet  Ethernet  Ethernet

Routing                to        to        to        to        to

Ethernet  Ethernet  Ethernet  64Kbps    64Kbps

(10 Mbps) 64KBPS    1.544Mbps 2.048Mbps X.25

PPP       PPP       PPP

---------------------------------------------------------------------

IPX        64      46  14,881    148       3,574     4,741     145

Routing   516     498   2,332     16         381       506      16

1,516   1,498     814      5         128       170       5

----------------------------------------------------------------------

IP         64      46  14,881    148       3,574     4,741     145

Routing   516     498   2,332     16         381       506      16

1,516   1,498     814      5         128       170       5

--------------------------------------------------------------------

AppleTalk  64      38  14,881    174       4,196      5,565    170

Routing   512     490   2,350     16         368        514     16

===================================================================

Table 9

Theoretical Throughput (Packets per Second) with a Token Ring LAN as the

Source

Frame  Packet  pps        pps     pps    pps     pps

Size    Size   from       from    from   from    from

Token-     Token-  Token- Token-  Token-

Ring       Ring    Ring   Ring    Ring

to         to      to     to      to

Ethernet   64KBPS  1.544  2.048   64Kbps

(10 Mbps)  PPP     Mbps   Mbps    X.25

ppp    ppp

-------------------------------------------------------------

IPX         64      38   22,883       174  4,196  5,565   170

Routing    516     490    3,708        16    388    514    16

4,100   4,074      485         2     47     63     2

-------------------------------------------------------------

IP          64      43   22,883       157  3,784  5,020   154

Routing    516     495    3,708         6    384    509    16

4,100   4,079      485         2     47     63     2

-------------------------------------------------------------

Apple       64      38   22,383       174  4,196  5,565   170

Talk       512     490    3,736        16    388    514    16

Routing

-------------------------------------------------------------

Source      64      46   22,883       148  3,574  4,741   145

Route      516     498    3,708        16    381    506    16

Bridging 4,100   4,082      485         2     47     63     2

IPX

-------------------------------------------------------------

Bandwidth Utilization

Bandwidth utilization is the percentage of channel capacity consumed by a

bridge or router. The total bandwidth of a LAN or WAN determines how much

data can pass through a network. In theory, the maximum packet rate of a 10

Mbps Ethernet LAN using 64 byte packets is 14,881 pps

(10,000,000/672=14,881). A token ring LAN using 63 byte packets has either

a 7,246 pps rate at 4 Mbps or a 28,985 pps rate at 16 Mbps. The following

are the bandwidth limitations for WANs:

- X.25 services are generally available at 19.2 kilobits per second

(Kbps). In Western Europe and the United States, X.25 is available at 64

Kbps.

- T1 bandwidth limitation is 1.544 Mbps and E1 bandwidth limitation is

2.048 Mbps.

In a typical Ethernet environment, the average bandwidth utilization is 5

to 20 percent. Heavy loaded networks operate at 30 to 40 percent

utilization, and sometimes short bursts of data can consume up to 80

percent of the available bandwidth. Routers that forward packets with

utilization rates of 20 percent or more provide adequate performance for

most LANs. For wide area connections, bandwidth utilization rates greater

than ten percent of an Ethernet can saturate a T1/E1 link. Therefore,

remote routers can have lower performance without degrading user response

time.

TEST RESULTS

Table 10 through Table 19 show the bandwidth utilization, data throughput,

and fps measurements of various router and source route bridge

configurations.  Note that for large frame sizes, NetWare MultiProtocol

Router and NetWare MultiProtocol Router Plus achieve more than 90 percent

bandwidth utilization for Ethernet and token ring networks. Lower

performance numbers are the result of the system configuration and driver

performance, not the software.

You can configure the bridge/router using a high-performance system and LAN

boards. One of the more interesting results is the use of a 386SX computer

with two NE2000 boards.  This configuration has a 54 percent Ethernet

bandwidth consumption.  Most LANs run at rates far below this number. This

configuration enables customers to use a low-cost system and interface

board for many LAN-to-LAN bridge/routers.

Testing Methodology

The performance test results were compiled by LANQuest Labs, an independent

test lab. Frame generation and measurement equipment include the following:

Bytex ETS1000 test set

LANQuest Labs' FrameThrower

Wandel & Goltermann DA-30

Hewlett-Packard 4972A

Hewlett-Packard Network Advisor

Measurements were taken using a half-duplex (or source/sink) test

configuration. For the 1-to-1 test configuration, one interface board was

attached to a LAN segment that supplied the source of the data flow and the

other interface board was attach to a different LAN that received the data.

Similarly, for the 2-to-2 test configuration, two interface boards were

attached to source LANs and two interface boards were attached to receiving

LANs. Likewise, for the 3-to-3 test configuration, three interface boards

were attached to the source and sink LANs.

The measurements are the average of three iterations of each test (30

seconds for each iteration). Measurements were recorded at a sustained fps

rate, which is the number of frames received by the router equal to the

number of frames routed out of the router without any frame loss. On a

2-to-2 configuration or 3-to-3 configuration, this fps number is a per

interface throughput. Data throughput is calculated using the following

formula:

(fps x frame size) x (8 bits/byte)

---------------

1000

Table 10

Benchmark Summary for IPX Routing with Ethernet (per interface)

Interface  Ports  Frame  Data     FPS    % of

Board             Size   Thruput         BandWidth

Kbps*1          Utilization

-----------------------------------------------------------------

Epson    3Com       1-1       64  3,424    6,689  34

486/50   Etherlink           516  8,736    2,116  87

50 MHz   16 ISA*2          1,516  9,352      771  94

---------------------------------------------------------

IBM      3Com       1-1       64  2,203    4,303  22

8595     Etherlink           516  8,549    2,071  85

MC32              1,516  9,384      774  94

MCA*2

---------------------------------------------------------

NetWorth 3Com       1-1       64  3,200    6,251  32

NAE      EtherLink           516  8,464    2,051  85

486/33   16TP              1,516  9,448      779  94

ISA*2

----------------------------------------------------------

Compaq   Mylex      1-1       64  5,072    9,905   51

486/50   EISA*2*4          1,516  8,832      727   88

----------------------------------------------------------

Epson    NE2000     1-1       64  1,120    2,187   11

320sx    *2*3                516  4,248    1,029   42

20 MHz                     1,516  5,400      445   54

---------------------------------------------------------

Compaq   NE3200     1-1       64  1,120    2,187   11

486      *2*3                516  4,248    1,029   42

33 MHz                     1,516  5,400      445   54

----------------------------------------------

2-2       64  1,572    3,070   16

516  7,372    1,755   74

1,516  8,840      728   88

*1 Calculated value.

*2 These results were obtained using NetWare MultiProtocol Router v2.0.

*3 These results were compiled by Novell, Inc.

*4 These results were compiled using the benchmark test from Scott Bradner

at Harvard University (fall, 1992).

==========================================================

Table 11

Benchmark Summary for IPX Routing with Token Ring 16 (per interface)

Interface  Ports  Frame   Data     FPS     % of

Board             Size    Thruput          BandWidth

Kbps*1                   Utilization

----------------------------------------------------------

Compaq    Madge     1-1       64   10,002   19,535  63

Deskpro   SMART              516   15,108    3,660  94

486       16/4 EISA        4,100   15,774      480  98

66 MHz    Ringnode*2

-----------------------------------------------------------

IBM PS/2  Madge     1-1       64    3,931    7,678  25

Model 95  SMART              516   14,120    3,420  88

XP50      16/4             4,100   15,774      480  98

50 Mhz    (MC32)*2          -------------------------------

2-2       64    2,160    4,218  13

516    6,784    1,644  42

4,100   15,680      478  98

--------------------------------

3-3       64    1,392    2,720   9

516    4,554    1,103  28

4,100   13,667      417  85

-------------------------------------------------------

Compaq   Olicom     1-1       64    9,950   19,434  62

Deskpro   EISA*2             516   15,136    3,667  95

66 Mhz                     4,100   15,912      485  99

------------------------------

2-2       64    4,156    8,118  26

516   14,456    3,502  90

4,100   15,808      482  99

-----------------------------------------------------------

IBM      Olicom     1-1       64    2,184    4,266  14

PS/2     MCA*2               516    8,072    1,955  50

Model 95                   4,100   10,040      306  63

33 MHz                    ------------------------------

Olicom              2-2      *64      984    1,922   6

516    4,248    1,029  27

4,100    5,052      154  32

-------------------------------------------------------

Everex   Thomas     1-1       64    1,778    3,473  11

486      Conrad              516    9,536    2,310  60

33MHZ   TC4045*2           4,100   15,680      478  98

------------------------------

2-2       64    1,764    3,445  11

516    8,264    2,001  52

4,100   10,808      329  68

----------------------------------------------------------

Compaq   Proteon    1-1       64      888    1,739   6

Deskpro   P1392*2            516    4,816    1,167  30

386/20e                    4,100   10,920      333  68

20 MHz                ------------------------------------

2-2       64    1,764    3,445  11

516    8,264    2,001  52

4,100   10,808      329  68

----------------------------------------------------------

NetWorth  Madge     1-1       64    2,134    4,169   6

NAE        SMART             516    5,638    1,300  34

486/33     16/4 AT         4,100    8,696      265  54

---------------------------------------------------------

AST       Eagle     1-1       64    1,560    3,050   9

Power    NTR2000             516    4,672    1,131  29

Premium   *2               4,100   10,368      316  65

486/33

---------------------------------------

IBM       1-1       64      949   1,854    5

Token Ring         516    2,728     660   17

16/4*2           4,100    3,840     106   22

*1 Calculated value.

*2 These results were obtained using NetWare MultiProtocol Router v2.0.

==========================================================

Table 12

Benchmark Summary for IP Routing with Ethernet (per interface)

Interface  Ports  Frame   Data     FPS      % of

Board             Size    Thruput           BandWidth

Kbps*1                    Utilization

----------------------------------------------------------

Epson    3Com       1-1       64   1,562    3,051     16

486/50   EtherLink           516   8,776    2,126     88

50 MHz   16 TP             1,516   9,496      783     95

ISA*2

-------------------------------------------------------

IBM      3Com       1-1       64   2,204    4,306     22

8595     EtherLink           516   8,528    2,066     85

3Com     MC32              1,516   9,384      774     94

MCA)*2

-------------------------------------------------------

NetWorth 3Com       1-1       64   2,889    5,644     29

NAE      EtherLink           516   8,466    2,020     83

486/33   16 TP             1,516   9,447      790     96

(ISA)*2

----------------------------------------------------------

Compaq   Mylex *2*3 1-1       64   4,640    9,062     46

486/50   EISA              1,518   8,632      711     86

*1 Calculated value.

*2 These results were obtained using NetWare MultiProtocol Router v2.0.

*3 These notes were compiled using the benchmark test from Scott Bradner at

Harvard University.

==========================================================

Table 13

Summary for IP Routing with TokenRing (per interface)

Interface   Ports  Frame  Data     FPS     % of       Media

Board              Size   Thruput          BandWidth  Type

Kbps*1           Utilizat

==========================================================

Compaq    Madge        1-1      64   9,439   18,436  59         TR 16

Deskpro   SMART                516  15,096    3,657  94         TR 16

66MHz     16/4 EISA          4,100  15,580      475  97         TR 16

Ringnode*2

----------------------------------------------------------

AST       Eagle        1-1      64   1,568    3,066  10

Power     NTR2000              516   6,080    1,473  38

Premium   *2                 4,100   7,640      233  48

486/33

IBM Token    1-1      64     945    1,847   6

Ring 16/4*2          516   2,776      672  17

4,100   3,840      106  22

----------------------------------------------------------

IBM       Madge        1-1      64   3,932    7,681  25

PS/2      SMART                516  14,120    3,420  88

Model     16/4               4,100  15,774      480  98

95/XP50   MC32*2      --------------------------------------

2-2      64   2,004    3,914  13

516   6,436    1,559  40

4,100  15,416      470  96

--------------------------------------

3-3      64   1,317    2,575   8

516   4,389    1,063  27

4,100  15,472      471  97

----------------------------------------------------------

IBM PS/2  Olicom       1-1      64   2,072    4,044  13

Model 95  MCA *2               516   8,048    1,950  50

33 MHz                       4,100  10,072      307  63

--------------------------------------

2-2      64     964    1,883   6

516   4,020      974  25

4,100   5,004      153  31

----------------------------------------------------------

Everex    Thomas       1-1      64   1,776    3,473  11

486       Conrad               516   9,560    2,315  60

33 MHz    TC4045*2           4,100  15,680      478  98

--------------------------------------

2-2      64   1,704    3,326  11

516   8,048    1,950  50

4,100  10,856      331  68

----------------------------------------------------------

Compaq    Proteon      1-1      64     752    1,466   5

Desk Pro  P1392*2              516   4,544    1,100  28

386/20e                      4,100  10,824      330  68

20 MHz               ---------------------------------------------

2-2      64     236      461   1

516   1,120      271   7

4,100   2,132       65  13

----------------------------------------------------------------

NetWorth  Madge        1-1      64   2,128    4,160  13

NAE486/33 SMART                516   5,384    1,305  34

16/4 AT            4,100   8,656      264  54

Bridgenode*2

------------------------------------------------------------------

Compaq    Olicom       1-1      64   9,624   18,790   60

Deskpro M EISA *2              516  14,896    3,608   93

66 MHz                       4,100  15,875      484   99

----------------------------------------------

2-2      64   4,032    7,871   25

516  14,144    3,426   88

4,100  15,876      484   99

-------------------------------------------------------------------

*1 Calculated value.

*2 These results were obtained using NetWare MultiProtocol Router v2.0.

======================================================================

Table 14

Benchmark Summary for AppleTalk Routing with Ethernet (per interface)

Interface  Ports  Frame  Data     FPS     % of

Board             Size   Thruput          BandWidth

Kbps*1           Utililization

--------------------------------------------------------------------

Compaq   NE3200     1-1     64    1,568    3,060   16

486      *2, *3            516    7,592    1,839   76

33 MHz              -------------------------------------------------

2-2     64    1,124    2,191   11

516    6,640    1,609   66

--------------------------------------------------------------------

NetWorth  3Com      1-1     64    1,608    3,139   16

NAE      EtherLink         516    6,520    1,580   65

486/33   16 TP *2

--------------------------------------------------------------------

Epson   3Com        1-1     64    1,982    3,872   20

486/50   EtherLink         516    7,860    1,904   79

3Com      16 ISA *2

--------------------------------------------------------------------

*1 Calculated value.

*2 These results were obtained using NetWare MultiProtocol Router v2.0.

*3 These results were compiled by Novell, Inc.

======================================================================

Table 15

Benchmark Summary for AppleTalk Routing with Token Ring (per interface)

Interface   Ports  Frame  Data     FPS      % of

Board              Size   Thruput           BandWidth

Kbps*1            Utililization

----------------------------------------------------------------

Compaq   Madge       1-1     64     6,088   11,890   38

Deskpro  SMART              516    15,113    3,661   94

486      16/4

66 MHz   EISA *2

Ringnode

--------------------------------------------------------------------

IBM PS/2 Madge       1-1     64     2,896     5,662  18

Model 95 Smart              516    11,272     2,730  70

XP50      16/4         ------------------------------------------

50 MHz   MC32 *2     2-2     64     1,348     2,631   8

516     5,592     1,355  35

------------------------------------------

3-3     64       896     1,748    6

516     3,728       903   23

--------------------------------------------------------------

Compaq   Olicom      1-1     64     5,408    10,560   34

Deskpro   EISA *2           516    15,136     3,667   95

66 MHz               ------------------------------------------

2-2     64     2,448     4,778   15

516    11,144     2,700   70

----------------------------------------------------------------

IBM      Olicom      1-1     64     1,768     3,459   11

PS/2      MCA *2            516     7,624     1,847   48

Model               --------------------------------------------

95                   2-2     64       832     1,623    5

516     4,100       993   26

----------------------------------------------------------------

Everex   Thomas      1-1     64     1,776     3,470   11

486      Conrad             516     9,616     2,329   60

33 MHz   TC4045 *2   --------------------------------------------

2-2     64     1,700     3,321   11

516     6,988     1,693   44

------------------------------------------------------------------

Compaq   Proteon     1-1     64       576     1,119    4

Deskpro   P1392 *2          516     3,416       827   21

386/20e               ------------------------------------------

20 MHz               2-2     64       212       415    1

516     1,152       279    7

-------------------------------------------------------------

*1 Calculated value.

*2 These results were obtained using NetWare MultiProtocol Router v2.0.

======================================================================

Table 16

Benchmark Summary for Mixed Protocols with Ethernet (per interface)

Interface  Ports  Frame  Data     FPS    % of

Board             Size   Thruput         BandWidth

Kbps*1          Utililization

------------------------------------------------------------------

NetWare  3Com       1-1       64  2,088    4,079  21

NAE      EtherLink           516  7,280    1,764  73

486/33   16TP *2           1,516  9,472      781  95

*1 Calculated value.

*2 These results were obtained using NetWare MultiProtocol Router v2.0.

====================================================================

Table 17

Benchmark Summary for Mixed Protocols with Token Ring (16) (per interface)

Interface   Ports  Frame  Data     FPS     % of

Board              Size   Thruput          BandWidth

Kbps*1           Utililization

------------------------------------------------------------------

NetWare  Madge       1-1       64   2,030    3,996  13

NAE      SMART                516   5,254    1,273  34

486/33   16/4 AT            4,100   7,642      233  48

Madge    Bridgenode   ----------------------------------

*2

---------------------------------------------------------

AST      IBM         1-1       64   1,168    2,278   7

Power    Token                516   2,912      705  18

Premium  Ring               4,100   3,576      109  22

486/33   16/4 *2

--------------------------------------------------------

AST      Eagle       1-1       64   1,464    2,864   9

Power    NTR2000              516   5,232    1,268  33

Premium  *2                 4,100   7,512      229  47

486/33

----------------------------------------------------------

Compaq   Olicom      1-1       64   7,504   14,655  47

Deskpro  EISA *2              516  15,104    3,659  94

66 MHz                      4,100  15,808      482  99

------------------------------------

2-2       64   3,200    6,255  20

516  12,792    3,099  80

4,100  15,908      485  99

-----------------------------------------------------------

*1 Calculated value.

*2 These results were obtained using NetWare MultiProtocol Router v2.0.

======================================================================

Table 18

Benchmark Summary for Source Route Bridging (per interface)

Interface       Ports   Frame    Data     FPS    % of

Board                   Size     Thruput         BandWidth

Kbps *1                  Utililization

------------------------------------------------------------------

NetWare  Madge           1-1        64    3,840    7,500  24

NAE      SMART                     516    7,200    1,744  45

486/33   16/4 AT                 4,100    8,696      265  54

Bridgenode *2

-------------------------------------------------------------------

AST      Eagle           1-1        64    1,384    2,707   9

Power    NTR2000                   516    5,200    1,259  32

Premium  *2                      4,100    7,416      226  46

486/33

----------------------------------------------------------------

AST      IBM             1-1         64   1,104    2,153   7

Power    Token Ring                 516   2,880      697  18

Premium  16/4                     4,100   3,544      108  22

486/33   *2

--------------------------------------------------------

*1 Calculated value.

*2 These results were obtained using NetWare MultiProtocol Router v2.1.

======================================================================

Table 19

Benchmark Summary for Source Route Bridging over WAN (per interface)

PPP.  Ports 1-1

Interface     Frame  Data     FPS      % of          Media

Board         Size   Thruput           BandWidth     Type

Kbps *1           Utililization (PPP)

----------------------------------------------------------------------

Compaq   Novell           64     43       100     77           56K

Deskpro  Sync+/          516     53        13     94           56K

50 MHz   Adapter       4,100     56     2(*2)    100           56K

V.35             64    582     1,348     38           T1

& Madge         516  1,336       330     87           T1

16/4 EISA     4,100  1,536     47(*3)   100

Bridgenode

*2

----------------------------------------------------------------------

*1 Calculated value.

*2 These results were obtained using NetWare MultiProtocol Router v2.0.

*3 Rounded to the nearest whole number.

======================================================================

Note: The specifications detailed in this document are subject to change

without notice.

----------------------------------------------------------------

Any trademarks referenced in this document are the property of their

respective owners.  Consult your product manuals for complete trademark

information.

----------------------------------------------------------------