Tips on using Novell ELS level 1

Netware with more than four stations

====================================

By Henry J. Franzoni III

I have been using the Novell ELS netware level 1

software for $595.00 lately, and although there is a limit

of four active users, many more terminals can be added to

the network, they just cannnot be active.  The fifth terminal

to run ANET3.COM gets the message, "A file server could not be

found."  At least one station must be logged out before a new

station can be logged in.

But that's not all...

The station must be cleared from the console screen

on the non-dedicated file server with the command:

CLEAR STATION (and a station number)

AND, the station must be shut off for a moment as well.

So, I devised these batch files to assist in the process.

To use them, certain things must be done first.  In the

system login script, type SET STA="%STATION" and make sure

that this is in upper case, (lower case will not work as

discovered by P.L. Olympia).  Also, I renamed the file

LOGOUT.COM to LGT.COM so that the batch file named

LOGOUT.BAT would be run instead when LOGOUT is typed.

I also SET USER="%LOGIN_NAME" in order to pass the

user name to the DOS environment from the login script.

Additionally, I have as the last command in my system

login script EXIT "N_STA".

The files N_STA.BAT, N_STA1.IN, N_STA2,IN, N_STA3.IN, CR,

and LOGOUT.BAT must be in the Z:\PUBLIC\ directory.

I also use ASK.COM to do a IF ERRORLEVEL branch, but

any of the many interactive batch file programs that set

the errorlevel to different numbers will work as well.

The result of these batch files is that messages are

sent informing the users that a particular station is

logging out as well as informing them that the maximum

number of stations has been reached.  Station number 1,

which is the non-dedicated file server usually, recieves

a slightly different message.  The console screen gets a

message informing the operator of exactly which station

number to clear in order to login a new station.

If the station 1 logs out, no message is sent.

These files may be adapted to your own purposes,

however, if you change the name of N_STA.BAT, you will have

to change The name of all of the *.in files and the fourth

line of LOGOUT.BAT to match.  These batch files are name

sensitive.  Also, if you put a drive specifier in front of

N_STA.BAT, it will not work either.

Every line after SET MYSTA= in N_STA.BAT, ( the 18th line)

can be omitted.  I just put everyone into a menu shell using

the Novell Menu command, but this is not necessary.

I think it would be insult your intelligence if I explained

how these batch files worked.  It should be easy to figure

them out by looking at them.

Good Luck...    Henry J. Franzoni III

822 SE 15th St.

Portland, OR 97214