STANDARD MICROSYSTEMS CORPORATION

Jumper and Configuration Settings

For Non-EliteSeries Western Digital Ethernet LAN Adapters

USER GUIDE

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

June 16, 1993

This user guide provides information on changing jumpers for Western

Digital (WD) Ethernet LAN adapters that are not members of the Elite

adapter family. If you desire information on SMC and WD EliteSeries

Ethernet LAN adapters, please refer to the "JUMPRSMC.DOC" document.

"*" DENOTES THE SETTINGS OF JUMPERS INSTALLED AT THE FACTORY.

== EtherCard PLUS with Boot ROM Socket (WD8003EB) ==

JUMPER

BLOCK    NAME                  FUNCTION

------   ------  ----------------------------------------------------

W1      INIT     If jumper is installed when power is turned on,

280     I/O Base Address is temporarily set to 280.

Setup Program must then be run to store new I/O

Base Address, and jumper must be removed (*).

W3      AUI-    Ethernet type:  use AUI position for thick

BNC     Ethernet (external transceiver); use BNC (*)

position for thin Ethernet (BNC connector).

W9      ROM    Size of optional ROM installed (2 jumpers).

Size                  LEFT JUMPER     RIGHT JUMPER

POSITION         POSITION

16 Kbytes(*)      Top              Top

32 Kbytes       Bottom             Top

64 Kbytes       Bottom           Bottom

Jumpers are ignored if no ROM is installed.

== EtherCard PLUS16 16-Bit Ethernet Adapter (WD8013EBT) ==

JUMPER

BLOCK   NAME                    FUNCTION

------  ------    --------------------------------------------------

W0      Wait      Reduces the required wait states for an 8-bit

States    memory access from four to two.

8-Bit

Access

W0      Wait      Reduces the required wait states for a 16-bit

State     Memory access from one to zero.

16-Bit

Access    Install both the 8- and 16-bit jumpers to enhance

performance on systems with I/O bus speeds of

8MHz or less. Factory setting is both jumpers

removed, which ensures correct operation in

faster bus systems. The jumpers are ignored in

XT-bus systems.

W1      I/O Base  Selects the hex I/O Base Address. Options are

Address   200,  220,  240,  260,  280(*),  2A0,  2C0,

300,  340, and 380

W2      IRQ       Selects the interrupt request (IRQ) level to be

Level      used by the adapter. IRQ10, 11 and 15 are not

available in 8-bit bus systems, such as the IBM

PC and PC XT. IRQ2/9 position selects IRQ2 for

PC- and XT-type systems (8-bit bus), and IRQ9 for

AT-type systems (16-bit bus). Only one jumper may

be installed at a time.

IRQ options are:  2, 3(*), 4, 5, 7, 9, 10, 11, 15

W3      AUI-      Ethernet type:  Use AUI position for thick

BNC       Ethernet (external transceiver): Use BNC(*)

position for thin Ethernet (BNC connector).

W6      ROM      Selects ROM memory space, if a ROM is installed.

Memory    Options:  NONE(*), 16K, 32K, and 64K.

Space

W9      ROM      Size of optional ROM installed (2 jumpers).

Size       Jumpers are ignored when W6 is set to NONE.

ROM SIZE   TOP JUMPER   BOTTOM JUMPER

POSITION      POSITION

16 Kbytes(*)    Left          Left

32 Kbytes       Left          Right

64 Kbytes       Right         Right

W15       ROM      Establishes the base address for the ROM device

Base      (up to 3 jumpers). Jumpers are ignored when W6

Address   is set to NONE.

SELECTED       JUMPERS INSTALLED AT:

ADDRESS    C4000        C8000       D0000

C0000

C4000        X

C8000                     X

D0000                                 X

CC000        X            X

D4000        X                        X

D8000(*)                  X           X

DC000        X            X           X

== EtherCard PLUS (WD8003E) ==

And

== EtherCard PLUS With Boot ROM Socket (WD8003EBT) ==

JUMPER  PIN              FUNCTION SELECTED

BLOCK   NUMBERS  NAME   WHEN JUMPER IS INSTALLED

------  -------   -----  -------------------------------------------

W1       1-2     Zero  Selects zero wait states for optimized

Wait   operation in a 6MHz AT. (Factory setting

is jumper removed, which ensures correct

operation in faster AT-bus systems.) This

jumper is ignored in XT-bus systems.

W1             I/O Base Selects the I/O Base Address

Address (up to 3 jumpers)

3-4*,5-6*,9-10* 280 -- the factory setting

5-6, 9-10       2A0

3-4, 9-10       2C0

9-10            2E0

3-4, 5-6, 7-8   300

5-6, 7-8        320

3-4, 7-8        340

W2       1-2      IRQ7  Interrupt request level.

3-4      IRQ6  Only one jumper may be installed at a

5-6      IRQ5  time.

7-8      IRQ4

9-10*    IRQ3  (Factory setting is IRQ3)

11-12    IRQ2

W3       1-2*     AUI-  Ethernet type: pins all jumpered to use

3-4*     BNC   J1 BNC connector for thin Ethernet cable

5-6*            (* - the factory setting).

7-8*            Jumpers all removed to use J2 AUI

9-10*           connector (thick Ethernet cable).

11-12*

W4       1-2   Ethernet  Jumper removed for thin Ethernet and

Version   IEEE 802.3/Version 2 thick Ethernet

(the factory setting); jumper installed

for thick Ethernet Version 1.

W5       1-2*   Segment Selects thin Ethernet segment length:

Length   jumpered for IEEE 802.3 standard 185m

length (*-the factory setting); jumper

removed for 300m extended length.

== Additional Jumpers On ==

== EtherCard PLUS With ROM Socket Only (WD8003EBT) ==

JUMPER  PIN              FUNCTION SELECTED WHEN

BLOCK   NUMBERS  NAME   INDICATED JUMPERS ARE INSTALLED

------  --------  ------ -------------------------------------------

W6      Open      ROM   C0000 (16K, 32K, 64K ROMs)

1-2       Base   C4000 (16K ROMs)

3-4      Address C8000 (16K, 32K ROMs)

1-2, 3-4         CC000 (16K ROMs)

5-6              D0000 (16K, 32K, 64K ROMs)

1-2, 5-6         D4000 (16K ROMs)

3-4*, 5-6*       D8000 (16K, 32K ROMs)--factory setting

1-2, 3-4, 5-6   DC000 (16K ROMs)

(NOTE:  Do not use address C0000 or

C4000 for Boot ROMs)

W6      7-8*,9-10* ROM   ROM Disabled -- the factory setting

W9      3-4*       Size     "

W10     3-4*           "

W6      Open       ROM   16 Kbytes ROM

W9      3-4        Size     "

W10     3-4            "

W6      7-8        ROM   32 Kbytes ROM

W9      3-4        Size     "

W10     1-2            "

W6      9-10       ROM   64 Kbytes ROM

W9      1-2        Size     "

W10     1-2            "

W7      3-4*       RAM   8 Kbytes RAM -- the factory setting

W8      3-4*       Buffer   "

W11     2-3*       Size     "   (NOTE:  Valid 8 Kbyte RAM base

addresses are C0000,

C2000, C8000, CA000,

D0000, D2000, D8000,

and DA000.)

W7      1-2        RAM   32 Kbytes RAM

W8      1-2        Buffer     "

W11     1-2        Size       "      (NOTE:  Valid 32 Kbyte RAM

base addresses are

C0000, C8000, D0000,

and D8000.)

== EtherCard PLUS TP For Unshielded Twisted Pair Wiring,

LattisNet Compatible (WD8003WT) ==

JUMPER  PIN              FUNCTION SELECTED

BLOCK   NUMBERS  NAME   WHEN JUMPERS ARE INSTALLED

------  ------  -------  -------------------------------------------

W1       1-2      Zero  Selects zero wait states for optimized

Wait   operation in a 6MHz AT. (Factory

State  setting is jumper removed, which

ensures correct operation in faster

AT-bus systems.) This jumper is

ignored in XT-bus systems.

W1             I/O Base Selects the I/O Base Address

Address  (up to 3 jumpers)

3-4*,5-6*,9-10* 280 -- the factory setting

5-6, 9-10       2A0

3-4, 9-10       2C0

9-10            2E0

3-4, 5-6, 7-8   300

5-6, 7-8        320

3-4, 7-8        340

W2       1-2      IRQ7  Selects the Interrupt request level.

3-4      IRQ6  Only one jumper may be installed at a

5-6      IRQ5  time.

7-8      IRQ4

9-10*    IRQ3   -- the factory setting

11-12    IRQ2

W3       1-2*     RJ-45 Ethernet type: pins all jumpered to use

3-4*     -AUI   RJ-45 twisted pair wiring connector

5-6*            (*-the factory setting). Jumpers all

7-8*            removed to use AUI connector.

9-10*

11-12*

W6             ROM Base Selects the ROM base address

Address  (up to 3 jumpers)

Open             C0000 (16K, 32K, 64K ROMs)

1-2              C4000 (16K ROMs)

3-4              C8000 (16K, 32K ROMs)

1-2, 3-4         CC000 (16K ROMs)

5-6              D0000 (16K, 32K, 64K ROMs)

1-2, 5-6         D4000 (16K ROMs)

3-4*, 5-6*       D8000 (16K, 32K ROMs)--factory setting

1-2, 3-4, 5-6    DC000 (16K ROMs)

(NOTE:  Do not use address C0000

or C4000 for Boot ROMs)

W6      7-8*,9-10* ROM   ROM Disabled -- the factory setting

W9      2-3*       Size     "

W10     2-3*               "

W6      Open       ROM   16 Kbytes ROM

W9      2-3        Size     "

W10     2-3                "

W6      7-8        ROM   32 Kbytes ROM

W9      2-3        Size     "

W10     1-2                "

W6      9-10       ROM   64 Kbytes ROM

W9      1-2        Size     "

W10     1-2                "

W7      2-3*      RAM     8 Kbytes RAM -- the factory setting

W8      2-3*      Buffer    "

W11     2-3*      Size      " (NOTE:  Valid 8 Kbyte RAM

base addresses are C0000,

C2000, C8000, CA000,

D0000, D2000, D8000, and

DA000.)

W7      1-2       RAM     32 Kbytes RAM

W8      1-2       Buffer    "

W11     1-2       Size      "       (NOTE:  Valid 32 Kbyte RAM

base addresses are

C0000, C8000, D0000,

and D8000.)

== EtherCard PLUS10T for Unshielded Twisted Pair Wiring - 10BaseT

Draft Standard Compatible (WD8003W) ==

JUMPER

BLOCK   NAME                  FUNCTION

------   ------  ----------------------------------------------------

W1      INIT     If jumper is installed when power is turned on,

280     I/O Base Address is temporarily set to 280.

Setup Program must then be run to store new I/O

Base Address, and jumper must be removed (*).

W9      ROM     Size of optional ROM installed (2 jumpers).

Size                  LEFT JUMPER     RIGHT JUMPER

POSITION         POSITION

16 Kbytes(*)      Top              Top

32 Kbytes       Bottom             Top

64 Kbytes       Bottom           Bottom

Jumpers are ignored if no ROM is installed.

W20 Auto        In the ON position (*), the adapter automatically

Polarity      corrects for incorrect polarity on the receive

Correction  twisted pair. If W20 is installed in the OFF

position, the receive twisted pair polarity must

be correct on the RJ-45 port according to the

10BaseT standard.

W21  Link       The 10BaseT link integrity test between the

Integrity    adapter and a 10BaseT concentrator is performed

with the jumper removed (*). The link integrity

test is not performed if the jumper is installed.

EtherCard PLUS and SuperDisk are trademarks, and SMC and Standard

Microsystems are registered trademarks, of Standard Microsystems

Corporation. Other trademarks mentioned herein belong to their

respective companies.