STANDARD MICROSYSTEMS CORPORATION

Jumper and Configuration Settings

For EliteSeries Ethernet LAN Adapters

USER GUIDE

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

June 16, 1993

This user guide provides information on changing jumpers for SMC

and Western Digital (WD) EliteSeries Ethernet LAN adapters. If

you desire information on WD LAN adapters that are not members

of the EliteSeries family, please refer to the "JUMPRWD.DOC"

document.

"*" DENOTES THE SETTINGS OF JUMPERS INSTALLED AT THE FACTORY.

== EtherCard Elite Ultra (SMC 8216, 8216T AND 8216C) ==

JUMPER

BLOCK    NAME    POSITION                     FUNCTION

------   ------  -----------  ---------------------------------------

W1      CONFIG  SOFT*       If jumper is installed when power is

turned on, the I/O Base Address, IRQ,

and RAM Base Address are set to the

value last stored with the EZSETUP

program. Initially:

-I/O Base Address is set to 340.

-IRQ is set to 10.

-RAM Base Address is set to C800.

280,3,D000  If jumper is installed when power is

turned on:

-I/O Base Address is set to 280.

-IRQ is set to 3.

-RAM Base Address is set to D000.

300,10,CC00 If jumper is installed when power is

turned on:

-I/O Base Address is set to 300.

-IRQ is set to 10.

-RAM Base Address is set to CC00.

280,3,D000  If jumper is installed when power is

D800        turned on:

-I/O Base Address is set to 280.

-IRQ is set to 3.

-RAM Base Address is set to D000.

-ROM Base Address is set to D800.

300,10,CC00 If jumper is installed when power is

D800        turned on:

-I/O Base Address is set to 300.

-IRQ is set to 10.

-RAM Base Address is set to CC00.

-ROM Base Address is set to D800.

== EtherCard PLUS ELITE10T (SMC 8003WC) ==

JUMPER

BLOCK   NAME    POSITION                     FUNCTION

------  ------  -----------  ----------------------------------------

W1     CONFIG  SOFT         If jumper is installed when power is

turned on, the I/O Base Address, IRQ,

and RAM Base Address are set to the

value last stored with the EZSETUP

program. Initially the I/O Base

Address will be 240 with RAM disabled.

280,3,D0000* If jumper is installed when power is

turned on:

-I/O Base Address is set to 280.

-IRQ is set to 3.

-RAM Base Address is set to D0000.

300,5,CA000  If jumper is installed when power is

turned on:

-I/O Base Address is set to 300.

-IRQ is set to 5.

-RAM Base Address is set to CA000.

W2      ROM    SOFT*       If jumper is installed when power is

ADDR                turned on:

-the ROM Base Address is set to the

value last stored with the EZSETUP

program.

D8000        If jumper is installed when power is

turned on:

-ROM Base Address is set to D8000.

== EtherCard PLUS ELITE (SMC 8003EP and WD8003EP) ==

JUMPER

BLOCK   NAME    POSITION                     FUNCTION

------  ------  -----------  ----------------------------------------

W1     CONFIG  SOFT         If jumper is installed when power is

turned on, the I/O Base Address, IRQ,

and RAM Base Address are set to the

value last stored with the EZSETUP

program. Initially the I/O Base

Address will be 240 with RAM disabled.

280,3,D0000* If jumper is installed when power is

turned on:

-I/O Base Address is set to 280.

-IRQ is set to 3.

-RAM Base Address is set to D0000.

300,5,CA000  If jumper is installed when power is

turned on:

-I/O Base Address is set to 300.

-IRQ is set to 5.

-RAM Base Address is set to CA000.

W2      ROM    SOFT*       If jumper is installed when power is

ADDR                turned on:

-the ROM Base Address is set to the

value last stored with the EZSETUP

program.

D8000        If jumper is installed when power is

turned on:

-ROM Base Address is set to D8000.

W3      AUI-                Ethernet type:  use AUI position for

BNC                 thick Ethernet (external transceiver);

use BNC (*) position for thin Ethernet

(BNC connector).

== EtherCard PLUS ELITE16 (SMC 8013EPC and WD8013EP) ==

JUMPER

BLOCK   NAME    POSITION                     FUNCTION

------  ------  -----------  ----------------------------------------

W1     CONFIG  SOFT         If jumper is installed when power is

turned on, the I/O Base Address, IRQ,

and RAM Base Address are set to the

value last stored with the EZSETUP

program. Initially the I/O Base

Address will be 240 with RAM disabled.

280,3,D0000* If jumper is installed when power is

turned on:

-I/O Base Address is set to 280.

-IRQ is set to 3.

-RAM Base Address is set to D0000.

300,10,CC000 If jumper is installed when power is

turned on:

-I/O Base Address is set to 300.

-IRQ is set to 10.

-RAM Base Address is set to CC000.

W2     ROM     SOFT*       If jumper is installed when power is

ADDR                 turned on:

-the ROM Base Address is set to the

value last stored with the EZSETUP

program.

D8000        If jumper is installed when power is

turned on:

-ROM Base Address is set to D8000.

W3     AUI-                 Ethernet type:  use AUI position for

BNC                  thick Ethernet (external transceiver);

use BNC (*) position for thin Ethernet

(BNC connector).

== EtherCard PLUS ELITE16C (SMC 8013EWC and WD8013EW) ==

JUMPER

BLOCK   NAME    POSITION                     FUNCTION

------  ------  -----------  ----------------------------------------

W1     CONFIG  SOFT         If jumper is installed when power is

turned on, the I/O Base Address, IRQ,

and RAM Base Address are set to the

value last stored with the EZSETUP

program. Initially the I/O Base

Address will be 240 with RAM disabled.

280,3,D0000* If jumper is installed when power is

turned on:

-I/O Base Address is set to 280.

-IRQ is set to 3.

-RAM Base Address is set to D0000.

300,10,CC000 If jumper is installed when power is

turned on:

-I/O Base Address is set to 300.

-IRQ is set to 10.

-RAM Base Address is set to CC000.

W2     ROM     SOFT*       If jumper is installed when power is

ADDR                 turned on:

-the ROM Base Address is set to the

value last stored with the EZSETUP

program.

D8000        If jumper is installed when power is

turned on:

-ROM Base Address is set to D8000.

W3     AUI-                 Ethernet type:  use AUI position for

BNC                  thick Ethernet (external transceiver);

use BNC (*) position for thin Ethernet

(BNC connector).

== EtherCard PLUS ELITE16T (SMC 8013WC and WD8013W) ==

JUMPER

BLOCK   NAME    POSITION                     FUNCTION

------  ------  -----------  ----------------------------------------

W1     CONFIG  SOFT         If jumper is installed when power is

turned on, the I/O Base Address, IRQ,

and RAM Base Address are set to the

value last stored with the EZSETUP

program. Initially the I/O Base

Address will be 240 with RAM disabled.

280,3,D0000* If jumper is installed when power is

turned on:

-I/O Base Address is set to 280.

-IRQ is set to 3.

-RAM Base Address is set to D0000.

300,10,CC000 If jumper is installed when power is

turned on:

-I/O Base Address is set to 300.

-IRQ is set to 10.

-RAM Base Address is set to CC000.

W2     ROM     SOFT*       If jumper is installed when power is

ADDR                 turned on:

-the ROM Base Address is set to the

value last stored with the EZSETUP

program.

D8000        If jumper is installed when power is

turned on:

-ROM Base Address is set to D8000.

EtherCard PLUS, and SuperDisk are trademarks, and SMC and Standard

Microsystems are registered trademarks, of Standard Microsystems

Corporation. Other trademarks mentioned herein belong to their

respective companies.