Softpaq Solution:  SP0921

File Name:      MOVEPCI.EXE Version 2.0

Title:  PCI driver problems with Novell Server

_______________________________________________________________________

CATEGORY:  Driver

EFFECTIVE DATE:  10-31-1994

PRODUCTS AFFECTED:  Compaq Deskpro Products with PCI bus

OPERATING SYSTEM:  MS-DOS 6.0 or greater

SYSTEM CONFIGURATION:  Novell Server

DESCRIPTION: MOVEPCI resolves an anomaly between Novell Netware Server and

COMPAQ PCI bus Deskpro products that prevent some PCI network adapters from

functioning properly.

Compaq Deskpro machines that have a PCI bus by default have protected mode 32

bit PCI services located at the end address of memory (4 gigabytes).  This

memory address is not normally mapped by Novell server software.  PCI drivers

that do direct calls to the PCI 32 bit address will not function in this

environment.

MOVEPCI re-locates the PCI services to the F000 block of memory where they are

accessible by the Novell Server software.

This version of MOVEPCI (2.0) replaces the previous version of MOVEPCI (1.0).

Version 1.0 will not function correctly on Compaq's Pentium based products.

SUPERSEDES: MOVEPCI.EXE version 1.0 (SP0881.EXE)

HOW TO USE:

1. If SoftPaq obtained on diskette, copy the SoftPaq file to a temporary

directory on the hard drive and go to that directory:

MD C:\CPQ

COPY A:\SP0921.EXE C:\CPQ

C:

CD \CPQ

If SoftPaq downloaded from BBS, go to the directory where downloaded

to:

C:

CD \CPQ

2. From that drive and directory, run the SoftPaq utility to unbind the

primary files.  Type SP0921.EXE (and press ENTER).  The following files

should be created:

MOVEPCI.DOC MOVEPCI.EXE

3. Copy the files to the directory where you want to keep the files.

COPY MOVEPCI.* \[your choice]

4. MOVEPCI.EXE should be installed in CONFIG.SYS.  It can also be run from the

DOS prompt in order to get information on the following:

1) If MOVEPCI has/has not been run in CONFIG.SYS.  If it has not, MOVEPCI

will report if it can be successfully run in CONFIG.SYS

2) Syntax for installing in CONFIG.SYS

3) Where PCI services will be or have been re-located in the F000 block

of memory

5. MOVEPCI will not change anything when run from the DOS prompt - it will

only report information.

6. Install MOVEPCI at or near the beginning of CONFIG.SYS by adding the

following line:

DEVICE=[drive:][\path(directory)]MOVEPCI.EXE

MOVEPCI *must* be installed before any memory managers like QEMM or 386MAX.

7. If MOVEPCI is used on a Deskpro XL with ROM BIOS dated earlier than

08/01/94 it will function properly but it will not be possible to re-boot

the computer using the Ctrl-Alt-Del key sequence.  The computer will work

properly in every other way.

8. MOVEPCI does not remain resident in memory - it re-locates PCI services and

exits.