Chapter			Hidden Chapter Field Code.  Do Not Delete

6	STATUS REPORTING SERVICESXE "STATUS REPORTING SERVICES"§

This section describes the status reporting services which are available from the Telephony Server API. The section includes descriptions of all the function calls and events that have to do with unsolicited event messages comming from the Telephony Server. Unsolicited event messages can be generated as a result of external telephony activity on the switch/device or activity generated by the users at the physical telephone instrument. These event messages are typically not anticipated by the application and are completely asynchronous in nature and can occur at any given time. For example, an event informing the application of an incoming call to a device (e.g. a telephone station) is an unsolicited, asynchronous event since the call is not initiated by the application and it can arrive at any time.

The status reporting request function defined in this section allows the applications to turn-on or turn-off status event reporting for an associated CSTA device (e.g. a desktop telephone). This function can be used by the application to turn-on/turn-off status reporting for any other stations on the switch where monitoring is required (assuming proper access permissions have been administered at the server).

# Status Reporting Functions XE "Status Reporting Functions "§and Confirmation EventsXE "Status Reporting Confirmation Events"§

This section covers the functions required to establish and request unsolicited event reporting for a specific telephony device or for calls being controlled by the application through the API.  Event reporting is required in order to determine the changes in the state of a call or a connection associated with a device which is of interest to the application.

These events provide the application with crucial information on the state of calls or connections which may be required in order for the application to keep track of calls states within the switch or at a specific device. If the application requires to maintain call state information for a specific device or call within the switch, it must establish a device or call "monitor" to keep track of the real-time state information pertaining to the call or device. Applications should always be "event driven" and react to changes in call or connection state based on events being received from the Telephony Server rather than a specific understanding of the call state model for a specific switch implementation. Following this guideline will simplify the support of applications across multiple implementations of the Telephony Server API defined in this document. The only way to effectively track and receive call or connection state information is through the use of the event monitoring services described in this section of the Telephony Server API Specification.

The cstaMonitorDevice( ), cstaMonitorCall( ), or cstaMonitorCallsViaDevice( ) function must be called in order to initiate event reporting for a specific device or call which is under the control of the application. Event reporting can be provided for a device, a call, or for calls which are associated with a device which is being monitored. There are two different types of event monitors which can be initiated by the application via the use of this function. The monitor type are:

·	Call-type monitorXE "Call-type monitor"§ - call-type monitors will provide monitoring, i.e. event reporting, for unsolicited events pertaining to a specific call from "cradle-to-grave". In other words, events for a specific call will be provided by this type of monitor regardless of how many devices the call may be associated during the life of the call.

The application can then determine the current state of the call based on the events received from the switch. For example, if a call monitor exists for a specific call and the call is transferred or forwarded to other devices, these devices may cease to participate in the call, but event reporting continues even at the new devices participating in the call. Thus, a call-type monitor will provide call state information as a call is routed either by the application or by other external controller (e.g. the end user or other applications controlling the call). The application should also be aware that a call can get assigned a new call identifier (a new call ID) as it is transferred or conferenced within the switch. The new call identifier will be provided in the event report associated with the conference or transfer function being requested by the controller of the call.

·	Device-type monitorXE "Device-type monitor"§ - a device-type monitor will provide the application with call state or connection information pertaining to calls associated with a specific device, i.e. the monitored device. Only those calls associated with the monitored device will be reported within event reports to the application. If a call is transferred, dropped, or forwarded from the device call reporting for that call will discontinue. Event reports for a device will be provided to the application after a monitor is executed and acknowledged by the Telephony Server.

The application can expect to have event reports for calls which arrive after the device monitor is acknowledged or for calls which are still active at the device after the device monitor has been acknowledged. Only those events which are specific to the device being monitored will be provided to the application.

When a monitor is being requested on a CSTA object, the object can be either a call or a device but not both. The application must setup multiple monitors if it wants to monitor multiple devices or calls at the same time. The specific switch implementation of the Telephony Server may have limitations or restrictions on the maximum number of simultaneous monitors which can exists on any given system. The API does not place any such restrictions on the application.

When requesting a device or call monitor, an application can also specify an event filter which is used to only obtain event of a certain type. The event filter can discard any event types which the application is not interested in and only pass those types requested when the event monitor is established. This filter is specified when the monitor is established on a device or a call. The filter can also be changed after the monitor is activated by using the CSTAChangeMonitorFilter( ) function.

Before an event monitor is activated on a device or a call, a ACS Stream and an Event Handling Mechanism must be opened, registered, and initialized before any event status function is called or any event received from the Telephony Server. See "Control Services" for more information on how to open a ACS Stream and register an Event Handling Mechanism.

## cstaMonitorDevice(XE "cstaMonitorDevice("§ )

The Monitor Start service is used to initiate unsolicited event reporting for a  device type monitoring on a device object. The unsolicited event reports will be provided for all endpoints within a CSTA switching sub-domain and optionally for endpoints outside of the CSTA switching sub-domain (implementation specific) which are involved with a monitored device.

Syntax

#include <csta.h>

#include <acs.h>

RetCode_t	cstaMonitorDevice (

ACSHandle_t			acsHandle,

InvokeID_t			invokeID,

DeviceID_t			*deviceID;	CSTAMonitorFilter_t		*monitorFilter,

PrivateData_t			*privateData),

Parameters

acsHandle

This is the value of the unique handle to the opened ACS Stream.

invokeID

A handle provided by the application to be used for matching a specific instance of a function service request with its associated confirmation event.  This parameter is only used when the Invoke ID mechanism is set for Application-generated IDs in the acsOpenStream( ). The parameter is ignored by the ACS Library when the Stream is set for Library-generated invoke IDs.

deviceID

Device ID of the device to be monitored.

monitorFilter

This paramater is used to specify a filter type to be used with the object being monitored. Setting a bit to true in the monitorFilter structure causes the specific event to be filtered out, so the application will never see this event. Initialize the structure to all 0's to receive all types of monitor events. See cstaMonitorDeviceConfEvent for a definition of a monitorFilter structure.

privateData

Private data extension mechanism.  Setting this parameter is optional. If the parameter is not used, the pointer should be set to NULL.

Return Values

This function returns the following values depending on whether the application is using library or application-generated invoke identifiers:

·	Library-generated Identifiers - if the function call completes successfully it will return a positive value, i.e. the invoke identifier. If the call fails a negative error (<0) condition will be returned. For library-generated identifiers the return will never be zero (0).

·	Application-generated Identifiers - if the function call completes successfully it will return a zero (0) value. If the call fails a negative error (<0) condition will be returned. For application-generated identifiers the return will never be positive (>0).

The application should always check the CSTAMonitorStartConfEvent message to insure that the service request has been acknowledged and processed by the Telephony Server and the switch.

The following are possible negative error conditions for this function:

ACSERR_BADHDL

This return value indicates that a bad or unknown acsHandle was provided by the application.

ACSERR_STREAM_FAILED

This return value indicates that a previously active ACS Stream has been abnormally aborted.

Comments

This function is used to start a device monitor on a CSTA device . The confirmation event for this function, i.e. CSTAMonitorConfEvent will provide the application with the CSTA association handle to the monitored device or call, i.e. the Monitor Cross Reference Identifier (monitorCrossRefID) which defines the CSTA association on which the monitor will exist.

## cstaMonitorCall( )XE "cstaMonitorCall( )"§

The Monitor Start service is used to initiate unsolicited event reporting for a call type monitoring on a call object. The unsolicited event reports will be provided for all endpoints within a CSTA switching sub-domain and optionally for endpoints outside of the CSTA switching sub-domain (implementation specific) which are involved with a monitored device.

Syntax

#include <csta.h>

#include <acs.h>

RetCode_t	cstaMonitorCall (

ACSHandle_t			acsHandle,

InvokeID_t			invokeID,

ConnectionID_t			*call,

CSTAMonitorFilter_t		*monitorFilter,

PrivateData_t			*privateData),

Parameters

acsHandle

This is the value of the unique handle to the opened ACS Stream.

invokeID

A handle provided by the application to be used for matching a specific instance of a function service request with its associated confirmation event.  This parameter is only used when the Invoke ID mechanism is set for Application-generated IDs in the acsOpenStream( ). The parameter is ignored by the ACS Library when the Stream is set for Library-generated invoke IDs.

call

Connection  ID of the call to be  monitored.

monitorFilter

This paramater is used to specify a filter type to be used with the object being monitored. Setting a bit to true in the monitorFilter structure causes the specific event to be filtered out, so the application will never see this event. Initialize the structure to all 0's to receive all types of monitor events. See cstaMonitorDeviceConfEvent for a definition of a monitorFilter structure.

privateData

Private data extension mechanism.  Setting this parameter is optional. If the parameter is not used, the pointer should be set to NULL.

Return Values

This function returns the following values depending on whether the application is using library or application-generated invoke identifiers:

·	Library-generated Identifiers - if the function call completes successfully it will return a positive value, i.e. the invoke identifier. If the call fails a negative error (<0) condition will be returned. For library-generated identifiers the return will never be zero (0).

·	Application-generated Identifiers - if the function call completes successfully it will return a zero (0) value. If the call fails a negative error (<0) condition will be returned. For application-generated identifiers the return will never be positive (>0).

The application should always check the CSTAMonitorStartConfEvent message to insure that the service request has been acknowledged and processed by the Telephony Server and the switch.

The following are possible negative error conditions for this function:

ACSERR_BADHDL

This return value indicates that a bad or unknown acsHandle was provided by the application.

ACSERR_STREAM_FAILED

This return value indicates that a previously active ACS Stream has been abnormally aborted.

Comments

This function is used to start a call monitor on a CSTA device . The confirmation event for this function, i.e. CSTAMonitorConfEvent will provide the application with the CSTA association handle to the monitored device or call, i.e. the Monitor Cross Reference Identifier (monitorCrossRefID) which defines the CSTA association on which the monitor will exist.

## cstaMonitorCallsViaDevice( )XE "cstaMonitorCallsViaDevice( )"§

The Monitor Start service is used to initiate unsolicited event reporting for a call type monitoring on a device object. The unsolicited event reports will be provided for all endpoints within a CSTA switching sub-domain and optionally for endpoints outside of the CSTA switching sub-domain (implementation specific) which are involved with a monitored device.

Syntax

#include <csta.h>

#include <acs.h>

RetCode_t	cstaMonitorCallsViaDevice (

ACSHandle_t			acsHandle,

InvokeID_t			invokeID,

DeviceID_t			*deviceID,

CSTAMonitorFilter_t		*monitorFilter,

PrivateData_t			*privateData),

Parameters

acsHandle

This is the value of the unique handle to the opened ACS Stream.

invokeID

A handle provided by the application to be used for matching a specific instance of a function service request with its associated confirmation event.  This parameter is only used when the Invoke ID mechanism is set for Application-generated IDs in the acsOpenStream( ). The parameter is ignored by the ACS Library when the Stream is set for Library-generated invoke IDs.

device

The deviceID of the device for which call monitoring should be started.

monitorFilter

This paramater is used to specify a filter type to be used with the object being monitored. Setting a bit to true in the monitorFilter structure causes the specific event to be filtered out, so the application will never see this event. Initialize the structure to all 0's to receive all types of monitor events. See cstaMonitorDeviceConfEvent for a definition of a monitorFilter structure.

privateData

Private data extension mechanism.  Setting this parameter is optional. If the parameter is not used, the pointer should be set to NULL.

Return Values

This function returns the following values depending on whether the application is using library or application-generated invoke identifiers:

·	Library-generated Identifiers - if the function call completes successfully it will return a positive value, i.e. the invoke identifier. If the call fails a negative error (<0) condition will be returned. For library-generated identifiers the return will never be zero (0).

·	Application-generated Identifiers - if the function call completes successfully it will return a zero (0) value. If the call fails a negative error (<0) condition will be returned. For application-generated identifiers the return will never be positive (>0).

The application should always check the CSTAMonitorStartConfEvent message to insure that the service request has been acknowledged and processed by the Telephony Server and the switch.

The following are possible negative error conditions for this function:

ACSERR_BADHDL

This return value indicates that a bad or unknown acsHandle was provided by the application.

ACSERR_STREAM_FAILED

This return value indicates that a previously active ACS Stream has been abnormally aborted.

Comments

This function is used to start a monitor on a CSTA object (a device or a call). The confirmation event for this function, i.e. CSTACallMonitorStartConfEvent will provide the application with the CSTA association handle to the monitored device or call, i.e. the Monitor Cross Reference Identifier (monitorCrossRefID) which defines the CSTA association on which the monitor will exist. There are two-types of Monitor Service: call-type and device-type.

## CSTAMonitorConfEventXE "CSTAMonitorConfEvent"§

This event is in responce to the cstaMonitorDevice( ), cstaMonitorCall or cstaMonitorCallsViaDevice function and contains the association handle being assigned to the CSTA association being used for status reporting.

Syntax

The following structure shows only the relevant portions of the unions for this message. See  ACS Data Types and  CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{				InvokeID_t	invokeID;

union

{

CSTAMonitorConfEvent_t  monitorStart;

} u;		} cstaConfirmation;

} event;} CSTAEvent_t;

typedef struct CSTAMonitorConfEvent_t {

CSTAMonitorCrossRefID_t		monitorCrossRefID;

CSTAMonitorFilter_t		monitorFilter;

} CSTAMonitorConfEvent_t;

typedef long			CSTAMonitorCrossRefID_t;

typedef unsigned short  CSTACallFilter_t;#define                     CF_CALL_CLEARED 0x8000#define                     CF_CONFERENCED 0x4000#define                     CF_CONNECTION_CLEARED 0x2000#define                     CF_DELIVERED 0x1000#define                     CF_DIVERTED 0x0800#define                     CF_ESTABLISHED 0x0400#define                     CF_FAILED 0x0200#define                     CF_HELD 0x0100#define                     CF_NETWORK_REACHED 0x0080#define                     CF_ORIGINATED 0x0040#define                     CF_QUEUED 0x0020#define                     CF_RETRIEVED 0x0010#define                     CF_SERVICE_INITIATED 0x0008#define                     CF_TRANSFERRED 0x0004typedef unsigned char   CSTAFeatureFilter_t;#define                     FF_CALL_INFORMATION 0x80#define                     FF_DO_NOT_DISTURB 0x40#define                     FF_FORWARDING 0x20#define                     FF_MESSAGE_WAITING 0x10typedef unsigned char   CSTAAgentFilter_t;#define                     AF_LOGGED_ON 0x80#define                     AF_LOGGED_OFF 0x40#define                     AF_NOT_READY 0x20#define                     AF_READY 0x10#define                     AF_WORK_NOT_READY 0x08#define                     AF_WORK_READY 0x04typedef unsigned char   CSTAMaintenanceFilter_t;#define                     MF_BACK_IN_SERVICE 0x80#define                     MF_OUT_OF_SERVICE 0x40typedef struct CSTAMonitorFilter_t {    CSTACallFilter_t		call;    CSTAFeatureFilter_t	feature;    CSTAAgentFilter_t		agent;    CSTAMaintenanceFilter_t	maintenance;    Boolean			private;} CSTAMonitorFilter_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTACONFIRMATION, which  identifies  this message as an CSTA confirmation event.

eventType

This is a tag with the value CSTA_MONITOR_CONF, which  identifies  this message as an CSTAMonitorDeviceConfEvent.

invokeID

This parameter specifies the requested instance of the function or event. It is used to match a specific functions call request with its confirmation events. Unsolicited events will have this parameter set to zero.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which the requested monitor has been established. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

monitorFilter

This paramater is used to specify the filter type which is active on the object being monitored by the application. Possible classes of values are: CALL_FILTER, FEATURE_FILTER, AGENT_FILTER, MAINTENANCE_FILTER, and PRIVATE_FILTER.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This confirmation event should be check by the application to obtain the monitorCrossRefID being assigned by the switch and to insure that the event filter requested has been activated. The events informs the application which filters is active on the given CSTA association.

## cstaMonitorStop( )XE "cstaMonitorStop( )"§

The Monitor Stop Service is used to cancel a previously registered Monitor Start Service on an existing CSTA monitor association, i.e. an active monitorCrossRefID.

Syntax

#include <csta.h>

#include <acs.h>

RetCode_t	cstaMonitorStop (

ACSHandle_t			acsHandle,

InvokeID_t			invokeID,

CSTAMonitorCrossRefID_t	monitorCrossRefID,

PrivateData_t			*privateData),

Parameters

acsHandle

This is the value of the unique handle to the opened ACS Stream.

invokeID

A handle provided by the application to be used for matching a specific instance of a function service request with its associated confirmation event.  This parameter is only used when the Invoke ID mechanism is set for Application-generated IDs in the acsOpenStream( ). The parameter is ignored by the ACS Library when the Stream is set for Library-generated invoke IDs.

monitorCrossRefID

This parameter identifies the original CSTA monitor association for which unsolicited event monitoring is to be canceled. This identifier is provided as a result of a monitor start service request (cstaMonitorStart( )) in a CSTAMonitorStartConfEvent for a call or device monitor within the switching domain.

privateData

Private data extension mechanism.  Setting this parameter is optional. If the parameter is not used, the pointer should be set to NULL.

Return Values

This function returns the following values depending on whether the application is using library or application-generated invoke identifiers:

·	Library-generated Identifiers - if the function call completes successfully it will return a positive value, i.e. the invoke identifier. If the call fails a negative error (<0) condition will be returned. For library-generated identifiers the return will never be zero (0).

·	Application-generated Identifiers - if the function call completes successfully it will return a zero (0) value. If the call fails a negative error (<0) condition will be returned. For application-generated identifiers the return will never be positive (>0).

The application should always check the CSTAMonitorStopConfEvent message to insure that the service request has been acknowledged and processed by the Telephony Server and the switch.

The following are possible negative error conditions for this function:

ACSERR_BADHDL

This return value indicates that a bad or unknown acsHandle was provided by the application.

ACSERR_STREAM_FAILED

This return value indicates that a previously active ACS Stream has been abnormally aborted.

Comments

This function is used to cancel a previously registered monitor association on a CSTA object (a device or a call object). Once a confirmation event is issued for this function, i.e. a CSTAMonitorStopConfEvent; it will terminate the previously active monitoring association and thus end event reporting for the monitored call or device.

## CSTAMonitorStopConfEventXE "CSTAMonitorStopConfEvent"§

This event is in responce to the cstaMonitorStop( ) function and provides the application with a confirmation that the monitor association has been canceled. Once this confirmation event is issued all event reporting for the specific monitoring association will be discontinued.

Syntax

The following structure shows only the relevant portions of the unions for this message. See  ACS Data Types and CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{				InvokeID_t	invokeID;

} cstaConfirmation;

} event;} CSTAEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTACONFIRMATION, which  identifies  this message as an CSTA confirmation event.

eventType

This is a tag with the value CSTA_MONITOR_STOP_CONF, which  identifies  this message as an CSTAMonitorStopConfEvent.

invokeID

This parameter specifies the requested instance of the function or event. It is used to match a specific functions call request with its confirmation events. Unsolicited events will have this parameter set to zero.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This confirmation event indicates a cancelation of a CSTA monitoring association. After this event is issued by the Telephony Server, no further events will be sent to the application on the monitoring association (monitorCrossRefID) which was canceled.

## CSTAChangeMonitorFilter( )XE "CSTAChangeMonitorFilter( )"§

This function is used to request a change in the filter options for CSTA event reporting for a specific CSTA association. It allows the application to specify for which event category the application wishes to receive events.

Syntax

#include <csta.h>

#include <acs.h>

RetCode_t	CSTAChangeMonitorFilter (

ACSHandle_t			acsHandle,

InvokeID_t			invokeID,

CSTAMonitorCrossRefID_t	monitorCrossRefID,

CSTAMonitorFilter_t		*filterlist,

PrivateData_t			*privateData),

Parameters

acsHandle

This is the value of the unique handle to the opened ACS Stream.

invokeID

A handle provided by the application to be used for matching a specific instance of a function service request with its associated confirmation event.  This parameter is only used when the Invoke ID mechanism is set for Application-generated IDs in the acsOpenStream( ). The parameter is ignored by the ACS Library when the Stream is set for Library-generated invoke IDs.

monitorCrossRefID

This parameter identifies the CSTA association (association handle) for which a change in event filtering is required. The association identifier is provided by the server/switch when the association is established.

filterlist

This parameter identifies the filter type being requested. Possible classes of values are CALL_FILTER, FEATURE_FILTER, AGENT_FILTER, MAINTENANCE_FILTER, and PRIVATE_FILTER.  This parameter  also identifies the events to be filtered.

privateData

Private data extension mechanism.  Setting this parameter is optional. If the parameter is not used, the pointer should be set to NULL.

Return Values

This function returns the following values depending on whether the application is using library or application-generated invoke identifiers:

·	Library-generated Identifiers - if the function call completes successfully it will return a positive value, i.e. the invoke identifier. If the call fails a negative error (<0) condition will be returned. For library-generated identifiers the return will never be zero (0).

·	Application-generated Identifiers - if the function call completes successfully it will return a zero (0) value. If the call fails a negative error (<0) condition will be returned. For application-generated identifiers the return will never be positive (>0).

The application should always check the CSTAChangeMonitorFilterConfEvent message to insure that the service request has been acknowledged and processed by the Telephony Server and the switch.

The following are possible negative error conditions for this function:

ACSERR_BADHDL

This return value indicates that a bad or unknown acsHandle was provided by the application.

ACSERR_STREAM_FAILED

This return value indicates that a previously active ACS Stream has been abnormally aborted.

Comments

The cstaEventFilter( ) function is used to inform the API Client Library and the server that only certain types of events are required. All events not requested will be filtered by the server and not provided to the application

## CSTAChangeMonitorFilterConfEventXE "CSTAChangeMonitorFilterConfEvent"§

This event occurs as a result of the cstaEventFilter( ) function and informs the application which event filter was set by the server.

Syntax

The following structure shows only the relevant portions of the unions for this message. See  ACS Data Types and CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{			InvokeID_t	invokeID_t;

union

{

CSTAChangeMonitorFilterConfEvent changeMonitorFilter;

} u;

} cstaConfirmation;

} event;} CSTAEvent_t;

typedef struct CSTAChangeMonitorFilterConfEvent_t

{

CSTAMonitorFitler_t		filterlist;

}

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTACONFIRMATION, which  identifies  this message as an CSTAConfirmation event.

eventType

This is a tag with the value CSTA_CHANGE_MONITOR_FILTER_CONF, which  identifies  this message as an CSTAChangeMonitorFilterConfEvent.

invokeID

This parameter specifies the requested instance of the function or event. It is used to match a specific functions call request with its confirmation events. Unsolicited events will have this parameter set to zero.

filterlist

This parameter identifies the filter type being requested. Possible  classes of values are CALL_FILTER, FEATURE_FILTER, AGENT_FILTER, MAINTENANCE_FILTER, and PRIVATE_FILTER.

This parameter  also identifies the events to be filtered.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This confirmation event should be check by the application to insure that the event filter requested has been activated and which filters are already active on the given CSTA association.

## CSTAMonitorEndedXE "CSTAMonitorEnded"§

This unsolicited indication is sent by the driver/switch to to indicate to the application that the monitor associated with the monitorCrossRefID has been stopped.

Syntax

The following structure shows only the relevant portions of the unions for this message. See Sections ACS Data Types and CSTA Data Types for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{			CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTAMonitorEnded_t  monitorEnded;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct CSTAMonitorEndedEvent_t {

CSTAEventCause_t cause;

} CSTAMonitorEndedEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_MONITOR_ENDED_IND, which  identifies  this message as an CSTAMonitorStopEvent.

monitorCrossRefID,

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

cause

The cause code indicating the reason the monitor was stopped.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event is  provided by the driver/switch when it can no longer provided the requested events associated with the monitorCrossRefId.

# Call Event Reports (Unsolicited)XE "Call Event Reports (Unsolicited)"§

This section covers the unsolicited events which can occur as a result of call activity on the Device or the switch. The events provide the application with call status information which can be used by the application in a variety of manners. These events can also result from a call interacting with switch features that might have been set either by the application or the switch administrator (e.g. call coverage paths).

## cstaCallClearedEventXE "cstaCallClearedEvent"§

This event report indicates when a call is torn down. This can occur when the last device has disconnected from the call or when a call is dissolved by another party to the call - like a conference call being dissolved by the conference controller.

Syntax

The following structure shows only the relevant portions of the unions for this message. See ACS Data Types and CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{			CSTAMonitorCrossRefID_t  monitorCrossRefID;

union

{

CSTACallClearedEvent  callClear;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef enum LocalConnectionState_t {    CS_NULL = 0,

CS_INITIATE = 1,    CS_ALERTING = 2,    CS_CONNECT = 3,    CS_HOLD = 4,    CS_QUEUED = 5,    CS_FAIL = 6} LocalConnectionState_t;typedef enum CSTAEventCause_t {    ACTIVE_MONITOR = 1,    ALTERNATE = 2,    BUSY = 3,    CALL_BACK = 4,    CALL_CANCELLED = 5,    CALL_FORWARD_ALWAYS = 6,    CALL_FORWARD_BUSY = 7,    CALL_FORWARD_NO_ANSWER = 8,    CALL_FORWARD = 9,    CALL_NOT_ANSWERED = 10,    CALL_PICKUP = 11,    CAMP_ON = 12,    DEST_NOT_OBTAINABLE = 13,    DO_NOT_DISTURB = 14,    INCOMPATIBLE_DESTINATION = 15,    INVALID_ACCOUNT_CODE = 16,    KEY_CONFERENCE = 17,    LOCKOUT = 18,    MAINTENANCE = 19,    NETWORK_CONGESTION = 20,    NETWORK_NOT_OBTAINABLE = 21,    NEW_CALL = 22,    NO_AVAILABLE_AGENTS = 23,    OVERRIDE = 24,    PARK = 25,    OVERFLOW = 26,    RECALL = 27,    REDIRECTED = 28,    REORDER_TONE = 29,    RESOURCES_NOT_AVAILABLE = 30,    SILENT_MONITOR = 31,    TRANSFER = 32,    TRUNKS_BUSY = 33,    VOICE_UNIT_INITIATOR = 34} CSTAEventCause_t;

typedef struct

{

ConnectionID_t			clearedCall;

LocalConnectionState_t		localConnectionInfo;

CSTAEventCause_t		cause;

} cstaCallClearedEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_CALL_CLEARED, which  identifies  this message as an CSTACallClearedEvent.

monitorCrossRefID,

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

clearedCall

This parameter identifies the call which has been cleared.

localConnectionInfo

This parameter defines the local connection state of the call after it has been cleared. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event is usually provided after the cstaClearCall( ) function has been called by the application. It can also occur, unsolicited, when another endpoint (device) clears a call and the device being monitored by the API is part of the call cleared by the another endpoint. The event is also generated when the last remaining device has disconnected from the call.

Before	After

Figure 1 - Call Cleared Event Report

## cstaConferencedEventXE "cstaConferencedEvent"§

This event report provides indication that two separate calls have been conferenced (merged) into a single. This occurs without either party being removed from the resulting call.

Syntax

The following structure shows only the relevant portions of the unions for this message. See   Data Types and  CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{			CSTAMonitorCrossRefID_t   monitorCrossRefID;

union

{

CSTAConferencedEvent_t  conferenced;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t			primaryOldCall;

ConnectionID_t			secondaryOldCall;

SubjectDeviceID_t		confController;

SubjectDeviceID_t		addedParty;

ConnectionList_t		conferenceConnections;

LocalConnectionState_t 	localConnectionInfo;

CSTAEventCause_t		cause;

} CSTAConferencedEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_CONFERENCED, which  identifies  this message as an CSTAConferencedEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

primaryOldCall

This parameter identifies the primary known call to be conferenced. This is usually the held call pending the conference.

secondaryOldCall

This parameter identifies the secondary call (e.g. the consultative call) which is to be conferenced. This is usually the active call which is to be conferenced to the held call pending the conference.

confController

This structure identifies the device which is controlling the conference. This is the device which setup the conference. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

addedParty

This parameter identifies the device which is being added to the conference. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

conferenceConnections

This is a list of connections (parties) on the call which resulted from the conference.  The call ID may be different from either the primary or secondary old call (or both).

localConnectionInfo

This parameter defines the local connection state of the call after it has been conferenced. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event provides information regarding a conference after is has been requested by the application using the CSTAConferenceCall( ) function or other endpoints on the switch. The changes in the call states are as follows:

µ §

Before	After

Figure 2 - Conferenced Event Report

## CSTAConnectionClearedEventXE "CSTAConnectionClearedEvent"§

This event report indicates that a device associated with a call disconnects from the call or is dropped from the call. The event does not indicate that a transferring device has left a call through the act of transferring that call.

Syntax

The following structure shows only the relevant portions of the unions for this message. See   Data Types and  CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		  CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTAConnectionClearedEvent_t connectionCleared;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t			droppedConnection;

SubjectDeviceID_t		releasingDevice;

SubjectDeviceID_t		localConnectionInfo;

CSTAEventCause_t		cause;

} CSTAConnectionClearedEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_CONNECTION_CLEARED, which  identifies  this message as an CSTAConnectionClearedEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

droppedConnection

This parameter identifies the Connection which was dropped from the call as a result of  a device dropping from the call.

releasingDevice

This parameter identifies the device which dropped the call.

localConnectionInfo

This parameter defines the local connection state of the call after the connection has been cleared. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event is used to determine which device disconnects from a multiparty call. The device_id identifies the devices which disconnected or was disconnected from the call. The LocalConnectionInfo defines the state of the call at the monitored device after the device has been dropped from the call.

Before	After

Figure 3 - Connection Cleared Event Report

## CSTADeliveredEventXE "CSTADeliveredEvent"§

This event report indicates that a call is alerting (e.g. ringing) at a specific device or that the server has detected that a call is alerting at a specific device.

Syntax

The following structure shows only the relevant portions of the unions for this message. See   Data Types and  CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		  CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTADeliveredEvent_t  delivered;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t			connection;

SubjectDeviceID_t		alertingDevice;

CallingDeviceID_t		callingDevice;

CalledDeviceID_t		calledDevice;

RedirectionDevice_t		lastRedirectionDevice;

LocalConnectionState_t 	localConnectionInfo;

CSTAEventCause_t		cause;

} CSTADeliveredEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_DELIVERED, which  identifies  this message as an CSTADeliveredEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

connection

This parameter identifies the Connection which is alerting

alertingDevice

This parameter indicates which device is alerting. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

callingDevice

This parameter identifies the calling device. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required

calledDevice

This parameter identifies the originally called device. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required

lastRedirectionDevice

This parameter will identify the previously alerted device in cases where the call was redirected or diverted to the alerting device. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

localConnectionInfo

This parameter defines the local connection state of the call after the Connection has alerted. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event provides all the necessary information required when a new call arrives at a device. This will include the calling and called numbers.

Before	After

Figure 4 - Delivered Event Report

## CSTADivertedEventXE "CSTADivertedEvent"§

This event report identifies a call which has been deflected or diverted from a monitored device. The call is no longer present or associated with the device.

Syntax

The following structure shows only the relevant portions of the unions for this message. See Data Types and  CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		   CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTADivertedEvent_t  diverted;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t			connection;

SubjectDeviceID_t		divertingDevice;

CalledDeviceID_t		newDestination;

LocalConnectionState_t		localConnectionInfo;

CSTAEventCause_t		cause;

} CSTADivertedEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_DIVERTED, which  identifies  this message as an CSTADivertedEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

connection

This parameter indicates the Connection which was previously alerting. This can be the intended Connection for the call before it was diverted.

divertingDevice

This parameter indicates the device from which the call was diverted. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

newDestination

This parameter indicates the device to which the call was diverted. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

localConnectionInfo

This parameter defines the local connection state of the device being monitored. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function.

If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event is used to determine information about a call which has been diverted from a monitored device. This includes information on which device the call is being diverted.

Before	After

Figure 5 - Diverted Event Report

## CSTAEstablishedEventXE "CSTAEstablishedEvent"§

This event report identifies a call which has been deflected or diverted from a monitored device. The call is no longer present or associated with the device.

Syntax

The following structure shows only the relevant portions of the unions for this message. See   Data Types and CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		   CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTAEstablishedEvent_t  established;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t			establishedConnection;

SubjectDeviceID_t		answeringDevice;

CallingDeviceID_t		callingDevice;

CalledDeviceID_t		calledDevice;

REdirectionDeviceID_t		lastRedirectionDevice;

LocalConnectionState_t		localConnectionInfo;

CSTAEventCause_t		cause;

} CSTAEstablishedEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_ESTABLISHED, which  identifies  this message as an CSTAEstablishedEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

establishedConnection

This parameter identifies the Connection which joined the call as a result of answering the call.

answeringDevice

This parameter indicates the device which has joined the call, i.e. the answering device. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

callingDevice

This indicates which device made the call, i.e. the calling device. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

calledDevice

This parameter indicates the originally called device. This may not always be the device answering a call as is the case with call forwarding or coverage, i.e. call redirection. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

lastRedirectionDevice

This parameter indicates the previously alerted device in cases where a call is redirected. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

localConnectionInfo

This parameter defines the local connection state of the device for the call which has been established. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event is typically used to determined when a call is answered by an endpoint being called by the application. This includes the calling and called number identification.

Before	After

Figure 6 - Established Event Report

## CSTAFailedEventXE "CSTAFailedEvent"§

This event report indicates that a call cannot be completed. The event applies only to a single Connection.

Syntax

The following structure shows only the relevant portions of the unions for this message. See  Data Types and  CSTA Data Typesin Section 4   for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		   CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTAFailedEvent_t  failed;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t		failedConnection;

SubjectDeviceID_t	failingDevice;

CalledDeviceID_t	calledDevice;

LocalConnectionState_t localConnectionInfo;

CSTAEventCause_t	cause;

} CSTAFailedEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_FAILED, which  identifies  this message as an CSTAFailedEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

failedConnection

This parameter indicates which Connection has failed.

failingDevice

This parameter indicates which device has failed. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

calledDevice

This parameter indicates which device was called when the call failed. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

localConnectionInfo

This parameter defines the local connection state of the call after the Connection has failed. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event occurs anytime a call cannot be completed for any reason (e.g. Stations Busy, Reorder Tone, Trunks Busy, etc...). The cause parameter contains the reason why the call failed.

Before	After

Figure 7 - Failed Event Report

## CSTAHeldEventXE "CSTAHeldEvent"§

This event report indicates that the server has detected that communications on a particular Connection has be interrupted (i.e. put on hold) by one of the devices on the call. This event is usually associated with a call being placed on hold at a device.

Syntax

The following structure shows only the relevant portions of the unions for this message. See  Data Types and CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		    CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTAHeldEvent_t  held;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t			heldConnection;

SubjectDeviceID_t		holdingDevice;

LocalConnectionState_t		localConnectionInfo;

CSTAEventCause_t		cause;

} CSTAHeldEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_HELD, which  identifies  this message as an CSTAHeldEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

heldConnection

This parameter identifies the Connection which was put on hold by thedevice.

holdingDevice

This parameter identifies the device which placed the connection on hold. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

localConnectionInfo

This parameter defines the local connection state of the call after the Connection has been put on hold. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event occurs after a call has been placed on hold at a specific device. This informs the application what device placed the connection on hold.

Before	After

Figure 8 - Held Event Report

## CSTANetworkReachedEventXE "CSTANetworkReachedEvent"§

This event report informs the application that a call has left the switch on an outbound trunk and is being routed through the telephone network.

Syntax

The following structure shows only the relevant portions of the unions for this message. See Data Types and  CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		   CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTANetworkReachedEvent_t  networkReached;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t			connection;

SubjectDeviceID_t		trunkUsed;

CalledDeviceID_t		calledDevice;

LocalConnectionState_t		localConnectionInfo;

CSTAEventCause_t		cause;

} CSTAHeldEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_NETWORK_REACHED, which  identifies  this message as an CSTANetworkReachedEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

connection

This parameter specifies the Connection ID for the outbound connection associated with the trunk and its connection to the network (see figure below).

trunkUsed

This parameter specifies the trunk that was used to establish the Connection with the telephone network. If the device (i.e. the trunk) is not specified, then the parameter will indicate that the device was not known or that it was not required.

calledDevice

This parameter indicates the destination device for the call. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

localConnectionInfo

This parameter defines the local connection state of the call after the Connection has cut-through into the telephone network. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

Once this event occurs the level of call related status information may decrease depending on the type of trunk being used to route the call to it's destination across the telephone network. The amount of call related status information provided by the network will depend on the type of trunk and telephone network being used to complete the call. Call status information may be limited to the disconnect or drop event. This only applies for calls to other network endpoints and not to calls within the switch being controlled by the server.

Switching Sub-domain Boundary

Before	After

Figure 9 - Network Reached Event Report

## CSTAOriginatedEventXE "CSTAOriginatedEvent"§

This event report informs the application that the switch is attempting to establish a call as a result of a completed request from the application.

Syntax

The following structure shows only the relevant portions of the unions for this message. See Data Types and CSTA Data Types in Section 4 for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		   CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTAOriginatedEvent_t  orginated;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t			orginatedConnection;

SubjectDeviceID_t		callingDevice;

CalledDeviceID_t		calledDevice;

LocalConnectionState_t		localConnectionInfo;

CSTAEventCause_t		cause;

} CSTAOrginatedEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_ORGINATED, which  identifies  this message as an CSTAOriginatedEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

originatedConnection

This parameter identifies the Connection where a call has been 	originated.

callingDevice

This parameter identifies the device from which the call has been originated. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

calledDevice

This parameter identifies the device for which the originated call is intended. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

localConnectionInfo

This parameter defines the local connection state of the call after the Connection has been originated. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event indicates that a call is being launched by the switch on behalf of the request from the application. The event only indicates that the switch is attempting to make the call. The application should check for additional events to determine the status of the call as it proceeds either through the switch or out to the telephone network.

Before	After

Figure 10 - Originated Event Report

## CSTAQueuedEventXE "CSTAQueuedEvent"§

This event report indicates that a call has been queued to an ACD Split, a hunt group, or others devices which support call queues. Call can also be queued during network re-routing without specifying a device.

Syntax

The following structure shows only the relevant portions of the unions for this message. See  Data Types and CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		    CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTAQueuedEvent_t  queued;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t			queuedConnection;

SubjectDeviceID_t		queue;

SubjectDeviceID_t		callingDevice;

CalledDeviceID_t		calledDevice;

RedirectionDeviceID_t		lastRedirectionDevice;

int				numberQueued;

LocalConnectionState_t		localConnectionInfo;

CSTAEventCause_t		cause;

} CSTAQueuedEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_QUEUED, which  identifies  this message as an CSTAQueuedEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

queuedConnection

This indicates the Connection was queued to the device.

queue

This parameter specifies the device to which the call has been queued. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

callingDevice

This parameter indicates the device who queued the call. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

calledDevice

This parameter indicates the device which was called (the intended recipient of the call). If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

lastRedirectionDevice

This parameter identifies the last device which redirected the call, if the call has been redirected. If the device is not specified, then the parameter will indicate that the device was not known or that it was not required.

numberQueued

This parameter indicates how many calls are queued to the queuing device.

localConnectionInfo

This parameter defines the local connection state of the call after the call has been queued. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event usually occurs when an application is monitoring a call, a Vector Directory Number (VDN), an ACD Split, or a hunt group. The event also provides information pertaining to the number of calls that have been queued to a device. This information can be useful to applications managing the queue at the device.

Before	After

Figure 11 - Queued Event Report

## CSTARetrieveEventXE "CSTARetrieveEvent"§

This event report identifies a call which was previously on hold and has been retrieved at a device. This is equivalent to taking the call off the hold state and into the active state.

Syntax

The following structure shows only the relevant portions of the unions for this message. See  Data Types and CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{		struct

{		   CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTARetrievedEvent_t  retrieved;

} u;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t			retrievedConnection;

SubjectDeviceID_t		retrivingDevice;

LocalConnectionState_t		localConnectionInfo;

CSTAEventCause_t		cause;

} CSTARetrievedEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_RETRIEVED, which  identifies  this message as an CSTARetrievedEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

retrievedConnection

This parameter specifies the Connection for which the call has been taken off the hold state.

retrievingDevice

This specifies the device which de-activated the call from the hold 	state.

localConnectionInfo

This parameter defines the local connection state of the call after the call has been retrieved from the hold state. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event informs the application that a call is no longer on hold. This can occur if the end-user physically takes the call off the hold state or in response to the cstaRetrieveCall( ) function request.

Before	After

Figure 12 - Retrieved Event Report

## CSTAServiceInitiatedEventXE "CSTAServiceInitiatedEvent"§

This event report indicates to the application that telephony service was requested at a device. This is equivalent to getting dial tone on a standard analog telephone.

Syntax

The following structure shows only the relevant portions of the unions for this message. See Data Types and CSTA Data Types in Section 4  for a complete description of the event structure.

typedef struct

{	ACSHandle_t	acsHandle;	EventClass_t	eventClass;	EventType_t	eventType;

} ACSEventHeader_t;

typedef struct

{

ACSEventHeader_t	eventHeader;

union

{	   struct

{	      CSTAMonitorCrossRefID_t	monitorCrossRefID;

union

{

CSTAServiceInitiatedEvent_t  serviceInitiated;

}u ;

} cstaUnsolicited;

} event;} CSTAEvent_t;

typedef struct

{

ConnectionID_t			initiatedConnection;

LocalConnectionState_t		localConnectionInfo;

CSTAEventCause_t		cause;

} CSTAServiceInitiatedEvent_t;

Parameters

acsHandle

This is the handle for the ACS Stream.

eventClass

This is a tag with the value CSTAUNSOLICITED, which  identifies  this message as an CSTA unsolicited event.

eventType

This is a tag with the value CSTA_SERVICE_INITIATED, which  identifies  this message as an CSTAServiceInitiatedEvent.

monitorCrossRefID

This parameter contains the handle to the CSTA association for which this event is associated. This handle is typically chosen by the switch and should be used by the application as a reference to a specific established association.

initiatedConnection

This parameter indicates the Connection for which service (dial tone) has been established or a feature is invoked. The same Connection identifier will continue to be used if a call is eventually established by the device.

localConnectionInfo

This parameter defines the local connection state of the call after the service has been initiated. This could be null, initiated, alerting, connected, held, queued, or failed.

cause

This parameter contains the cause value which indicates the reason or explanation for the occurrence of this event. The possible events are defined by CSTAEventCause_t.

privateData

If private data accompanied this event, then the private data would be copied to the location pointed to by the privateData pointer in the acsGetEventBlock( ) or acsGetEventPoll( ) function. If the privateData pointer is set to NULL in these functions, then no private data will be delivered to the application.

Comments

This event will not occur every time a call is established or launched from a device. For example, the event will not occur with functional type devices (e.g. ISDN BRI devices) when services is being requested by taking the device off-hook (dial tone state). The event will also not occur when a call is established using the cstaMakeCall( ) function.

Before	After

Figure 13 - Service Initiated Event Report