WESTERN DIGITAL

FOR "PLUS"-FAMILY StarLAN AND ETHERNET ADAPTER BOARDS

Copyright Western Digital Corporation 1988, 1989.

All Rights Reserved.

== CONTENTS ==

".ADF" files used in configuring Western Digital StarLAN and

Ethernet adapters for Micro Channel (*) bus systems.

The .ADF files operate with the "PLUS" (WD8003) family of Western

Digital LAN adapter boards designed for IBM's Micro Channel and

compatible systems:

Ethernet:  EtherCard PLUS For Micro Channel (WD8003ET/A) and (WD8003E/A)

StarLAN:   StarCard PLUS For Micro Channel (WD8003ST/A)

The following files are located in the main (root) directory of

this diskette:

File                       Contents

ADF.DOC           This file and Release notes on the .ADF files

@6FC0.ADF         Configuration file for Ethernet Micro

Channel adapter.

@6FC1.ADF         Configuration file for StarLAN Micro

Channel adapter.

@6FC2.ADF         Configuration file for Ethernet Micro

Channel adapter.

ViaNet is a registered trademark and StarCard PLUS, StarLink PLUS,

EtherCard PLUS, and SuperDisk are trademarks of Western Digital

Corporation.

(*) NetWare is a registered trademark of Novell, Inc.

LANWatch and PC/TCP are trademarks of FTP Software, Inc.

UNIX is a registered trademark of AT&T.

IBM is a registered trademark and Micro Channel is a

trademark of International Business Machines Corporation.

Microsoft and MS-NET are trademarks of Microsoft Corporation.

************************************************************************

;

;

;  This file contains the edit history for the @6fc0.adf file, the

;  Ethernet version of the ADF file.

;

;  Edit History:

;

;   2/19/88  first version(1.0)     By Steve Chen

;   4/6/88   1.1

;            rearrange the choices, so the very first

;            choice is default.

;                                   By Steve Chen

;   4/12/88  1.2

;            change to official id 6FC0

;                                   By Steve Chen

;

;   4/24/89  1.3

;            put under source control,

;            adding Header and Log Keywords.

;            Made default RAM base = d000.

;            Changed previously unconfigured bits to

;            configured to zero via the Fixed Resource

;            command. (Changed bit 2 of pos[3] to 1,

;            this selects 40 Mhz. Changed bit 4 of pos[3]

;            to 0 to select 16K RAM instead of 64K RAM.

;            Changed pos[1] bit 0 to 0.)

;                                   By Mary Barrington

;   6/2/89   1.4

;            Moved these comments to separate file.

;                                   By Mary Barrington

;