Extended AutoLISP and Expanded Memory Managers

Extended AutoLISP included with AutoCAD Release 10 c2a supports the

Quarterdeck/Phar Lap VCPI specification.  VCPI stands for Virtual Control

Program Interface.  VCPI allows an 80386-based expanded memory manager and

a protected mode program to peacefully co-exist under certain

circumstances.  Versions of AutoCAD prior to c2a don't support VCPI.

For AutoCAD users, this means that programs that require expanded memory,

such as device drivers for graphics cards that use display list processing,

can be used concurrently with Extended AutoLISP.

Two 80386 expanded memory managers that will work with Extended AutoLISP

are 386 QEMM.SYS Version 4.2 from Quarterdeck, and 386 To The Max Versions

4.0.3 and 4.0.5 from Qualitas.  Note that 386TTM Version 4.0.4 does NOT

correctly support the VCPI specification.

Configuration is relatively simple.  The rules below seem to work in most

cases; however, we can't promise they'll work in all cases.

Call the expanded memory driver with no command line parameters.  Allow it

to use all extended memory as expanded memory.  For example, QEMM might be

called out in CONFIG.SYS in this manner:

device=c:\qemm.sys

Set the DOS environment variable LISPXMEM to null.  Currently, there

appears to be a conflict between DOS/16M and VCPI when LISPXMEM has a

value.  This is how to set LISPXMEM to null in AUTOEXEC.BAT or in the batch

file you use to run AutoCAD:

set lispxmem=

Set the DOS environment variable ACADLIMEM to some reasonable value so that

AutoCAD doesn't grab all the expanded memory for extended I/O page space,

or set it to NONE if you don't want to use any of the expanded memory for

extended I/O page space:

set acadlimem=<<some_reasonable_value>>

or

set acadlimem=none

VCPI will allocate memory dynamically between the expanded memory driver

and the protected mode program in extended memory.  This involves

communication between the program, the driver, and DOS, and a lot of

fibbing to DOS on the part of VCPI as to just how much and what type of

memory is actually available at any given time.  It all seems to work,

somehow, as long as you keep it simple.

That's the sum total of our knowledge about VCPI.  If you're working with

more complex settings, then congratulations, you're on the leading edge of

technology.

Brad Zehring  Autodesk, Inc. Training Department

May 18, 1989

########################################################################################################