***** PERFECT.DOC ***** (v 2.2)

I.   INTRODUCTION:

PERFECT BOX is a Computer Aided Design (CAD) program for the

simulation of loudspeaker performance.  PERFECT BOX is a very

powerful tool for the amateur or professional speaker builder.

Loudspeaker design is a balance of compromises involving

enclosure size, low frequency cutoff, and transient response.

This program will allow you to model the loudspeaker-enclosure

combination entirely on your PC.  You are assured of the best

(lowest in frequency) bass response in the smallest enclosure.

You can then build the enclosure with full confidence that the

final result will not become the new dog house for Fido, but an

accurate, natural sounding system.

PERFECT BOX does not deal with midrange or tweeter drivers.

It only considers the low frequency driver in a multi-way system

(or the only driver in a full-range system.)  Since enclosure

alignments, such as box size, box type, and box frequency  (now on

refered to as Alignments), effect the woofer response, PERFECT BOX

models systems covering low frequencies from 10 to 200 Hertz.  The

mathematical model will hold for higher frequencies, but I have

optimized the program for the most useful range.

II.  REQUIREMENTS:

PERFECT BOX requires certain parameters that uniquely define

the driver's characteristics.  Some of these parameters have been

coined as the Thiele/Small (or T/S) parameters.  If you are new to

loudspeaker design, it isn't important that you understand what

each value represents, only that you are able to obtain these

values from the driver manufacturer, or measure them yourself

(more accurate.)

PARAMETER                         UNITS

Free air resonance    (Fs)         Hertz

Q Electrical          (Qes)

Q Mechanical          (Qms)

Q Total               (Qts)

Volume Equivalent

Compliance       (Vas)        Ft^3

D.C. Voice Coil

Resistance       (Re)         Ohms

Peak Cone

Displacement     (Xmax)       Inch

Effective Piston

Diameter                      Inch

Thermal Power         (P)          Watts, RMS

NOTE:  Qts=(Qes*Qms)/(Qes+Qms)

PERFECT BOX allows you to store these driver parameters in a

data file ("BOXDATA").  This file can store characteristics for up

to 998 different drivers.  A few sample drivers have been included

so that you can experiment with PERFECT BOX.  These samples

represent "real life" units which are currently available at this

time.

-1-

III. DEFINITIONS:

SYSTEM ORDER:  PERFECT BOX uses a classical high-pass filter

model to simulate a loudspeaker mounted in an enclosure.  The

order of a loudspeaker system represents the same characteristic

roll off as that of an electrical filter.  The roll off rate (or

slope) is:

roll off rate=(6 * system order) dB/octave

or

roll off rate=(20 * system order) dB/decade

PERFECT BOX can simulate both infinite baffle (sealed box),

and vented (ported) systems.  Each of these systems have a natural

order in which they resemble an electrical high pass circuit.  For

infinite baffle, order=2, or that is to say that they are of 2nd

order.  And for vented systems, order=4.  It seems that each kind

of enclosure has pros and cons associated with them.  In general,

both systems, if properly aligned, perform with the same low

frequency response.  In practice, an infinite baffle system is

easier to build, and possibly has better transient response.  The

vented system offers slightly higher efficiency (2.9 dB), and good

cone dampening near the vent frequency (but less below Fb.)  My

preference is for 4th and 6th order systems.

SYSTEM CUTOFF FREQUENCY:  This is the frequency at which the

system amplitude response is exactly 3.0 dB less than at a

mid-band reference frequency (where response is basically flat at

0 dB.)  This is a standard point of interest that can be used to

compare with another system's low frequency response data.  This

point represents the "half-power" amplitude, as compared to the

amplitude at the mid-band reference frequency.

OTHER ORDERS:  As you are problably aware, a Graphic

Equalizer can make a tremendous effect on the way your speaker

system sounds.  The bad news is that an equalizer (EQ) usually

doesn't have a control exactly centered on the proper frequency to

properly extend low frequency cutoff, and thus will typically make

matters worst.

PERFECT BOX can incorporate the effects of either a 1st or

2nd order, ACTIVE EQ into the loudspeaker model.  This mini EQ

consists of a simple active high-pass filter, having a low

turn-over (cutoff) frequency (typically 20-60 Hz), which actually

has a slight boost (peak) in its amplitude response above its

cutoff frequency.  If the amount of boost and cutoff freq. are

suitably chosen, the total system response typically is extended a

half-octave or more, while providing infrasonic (rumble) filtering

to boot!

In 2nd-order EQ's, the boost is controlled by the "D"

parameter (D=1/filter Q, for you filter buffs.)  Typical values of

"D" range from 0.3 to 1.414, giving most to least amount of

amplitude boost.  A "D" of 0.52 gives approximately 6 dB of boost,

the most used value.  The EQ frequency and the "D" factor should

be juggled for the flatest, most extended low frequency response.

-2-

A word of caution:  using large amounts of boost may tax

small power amplifiers (less than 50 Watts,RMS) beyond their

limits.  A properly designed 3, 3.5, 5, or 6th order system will

not over tax your amplifier unless you listen to music either :

1.  at extremely high SPL levels, 2.  with heavy organ pedal

notes, or 3. use other signal modification equipment such as an

extremely low-end boosted Graphic Equalizer (Stupid), or Sub-Sonic

Synthesizer (Harmonizer.)

The EQ can be inserted into the signal path of your system at

either a "Pre-amp out/Amp in" loop, or "tape monitor" connection.

A simple design for a 2nd order EQ is provided with PERFECT BOX:

type EQ2  at the DOS prompt to execute this option.  You must have

a graphics mode in order to run EQ2 because the electrical

skematic is drawn on the screen after all the circuit values are

calculated.  You may execute a screen dump (PrtSc) for a hardcopy

(as with all screens in PERFECT BOX.)

IV.  RUNNING PERFECT BOX:

PROGRAM EXECUTION:  Before you start PERFECT BOX, you must

execute your DOS command GRAPHICS.COM.  This command sets aside

memory for doing graphic dumps of your CRT to your graphics

printer.  I have created a simple batch file called GO.BAT, which

will do this for you.  Of course you must have either GRAPHICS.COM

on the program disk or in the current DOS path.  The actual

program file is BOX2_2.EXE.

1.  Insert the PERFECT BOX program disk into your default

disk drive, and type GO <Return>.  You should see the PERFECT BOX

opening screen.  After about 5 seconds, you may press RETURN to

continue.  Note:  All questions requiring operator input that have

default values will have those values prompted between

parenthesis.  Just press RETURN for the default.

2.  The next question asks which drive the "BOXDATA" file can

be found.  Enter the drive letter or simply press RETURN to denote

the current DOS default drive.

3.  At this point, you need to choose either ;

1.  for Automatic Parameter Entry  or,

2.  for Manual Parameter Entry.

If you choose 1., you can list, reload, edit, or save driver

parameters using a driver record number.  Option 2. will ask you

to enter each driver parameter and box characteristic, one at a

time.  This choice is good if you only want to run a driver

through PERFECT BOX once, with out saving the parameters.  Since

you can store up to 998 different drivers in the data file, I

rarely use this option!

Choose "Automatic", and then choose "LIST current Drivers in

database.  You should see a list of drivers with a record number

for each.  You may choose a particular driver from this list by

entering the record number, and then press RETURN.

-3-

5.  Or, choose "LOAD a Driver by Record Number," and enter

the record number you had memorized in the "LIST" step above.  The

model and manufacturer will now appear.  If this wasn't the driver

that you originally wanted, you can go back by answering "N" to

the question; "Is this the right driver?"  In this case, press

RETURN to choose the default of "Y", and the rest of the

parameters for the driver will appear on the CRT.  Again, you will

be prompted if these are the right parameters or not.  A "N" will

take you back to "Enter Driver record number."

6.  At this point, PERFECT BOX will calculate the "Ideal Box"

volume, the port frequency for that ideal box volume, and the

cutoff frequency (-3dB point.)  The ideal box volume is the

smallest enclosure that provides the flatest, deepest low

frequency response.  The cutoff point will give you a general idea

of how low the driver will perform in any box design (best case.)

The port frequency (box frequency) must be set as calculated to

obtain the cutoff predicted.

7.  Now you must choose the system order.  If the driver Qts

is 0.5 or greater, I would try infinite baffle (order=2) systems

first, otherwise, use order=4, with the ideal box size and

frequency.  Save the other order options for later.  PERFECT BOX

will also recommend an order to you.

8.  Enter the box volume in cubic feet.  I suggest that you

use the ideal box size and box frequency (for 4th order up) as a

starting point.  Then, on the next pass, change one of the box

alignments and check how the response was affected.

9.  PERFECT BOX will now calculate :   1. Amplitude,  2.

Maximum Power, and 3. Maximum Sound Pressure Level, at each

frequency cycle between 10 and 200 Hertz.  Then an Output Menu

will be presented.

Option 1. will plot the classical Bode (pronounced BO DEE; or

Frequency Response) plot.

Option 2, SPL & Power vs. Frequency, will give you an picture

of the system driven at maximum power.  This plot gives the Sound

Pressure Level (in dB) and the input power required to reach the

excursion limit (Xmax) of the driver.

Option 3. will list on the screen, all calculated data,

between the frequency limits of your choice.

Option 4. prints option 3. on your printer with the same

limit choice.

Option 5. produces on your printer, a worksheet which gives

you a handy reference table to use as you change the different

alignments about your system.  Each block can be filled from data

printed on the bode plot option.  I suggest that you do this

option first (only if you have a printer on line.)

Option 6. (or RETURN) will take you to the Change Menu.  At

this point you may choose what parameter to change in an attempt

to optimize the frequency plot, or what ever else you are trying

to acheive.

-4-

After you are satisfied with the current design parameters,

choose "NO CHANGES" in the Change Menu.  PERFECT BOX will now

calculate the port lengths required to tune the box to the

frequency last chosen.  A column for two identical length ports is

given also.  Dual ports are useful when the box volume is large or

the MACH speed is too large for a single port.  The MACH number is

the percent of the speed of sound that will occur at the largest

port output.  Stay away from large (speed> 0.2) port MACH speeds

as they will cause unmusical whistle noises from the port.  Also,

ports that are longer than the box depth, can be curved inside the

enclosure with PVC swept elbows.

V.   DESIGN HINTS:

If the ideal box volume is too large to be realistically

built, PERFECT BOX will allow you to see just how the loudspeaker

performance will suffer, as you down size box volume.  It is this

iterative design process that PERFECT BOX is designed for.  By

changing either box volume, port frequency, or in EQ cases, EQ

frequency and boost, a "perfect" frequency curve can be obtained.

At this point you can tell how good a driver really is because

sometimes another driver will out perform others in half the

enclosure volume!

The port frequency is set by the area and length of the port.

PERFECT BOX will calculate the port for you, but the length is

typically 10-15% off, so add an inch to the calculated value, and

measure it yourself (or have a friend do it.)  The system will not

perform as calculated if you miss the port frequency by as little

as 1 Hz.  Also, after you have reached a final box size, increase

the box volume by as much as 20% to account for volume used by the

drivers, bracing, and crossover network.

To find box diminsions from the box volume, take the volume

in cubic feet, multiply by 1728 cubic in/ft^3, to get in^3.  Then

take the cube root (or raise it to the 0.3333334 th power.)  Now

this number is the width in inches.  Multiply the width by 1.62

for the height, and multiply the width by 0.62 for the depth.

volume (ft^3) * 1728 = volume (in^3)

width  (in.)   =   (volume (in^3))^0.3333334

height (in.)   =   width * 1.62

depth  (in.)  =   width * 0.62

NOTE: All diminsions are internal, not external!

This will give the box a "typical speaker look" and also set

the diminsions up for minimum internal reflections.  PERFECT BOX

calculates these diminsions for you and prints them in the port

length table.

-5-

VI.  REFERENCES:

This document and the program PERFECT BOX are only tools for

the speaker designer.  One must know how to use these tools

properly before judging the usefulness of PERFECT BOX.  I suggest

that you read any of the following publications on the finer art

of speaker building.

SPEAKER BUILDER MAGAZINE (6 times a year)

Edward T. Dell, Jr., Publisher

PO Box 494

Peterborough, NH 03458

"The Loudspeaker Design Cookbook"

by Vance Dickason

Distributed by:

Old Colony Sound Lab

PO Box 243

Peterborough, NH 03458-0243

"How to Design, Build, & Test Complete

Speaker Systems"

by David B. Weems

Library of Congress # TK9968.W43

Published by:

TAB Books

Blue Ridge Summit, PA  17214

No. 1064

VII. VERSION 2.3 ADDITIONS:

The latest version of PERFECT BOX, v2.3, adds the following

options to the previous versions.

1.   The ability to change the path to the boxdata file.

This allows you to have multiple boxdata files, each in separate

subdirectory, or disk.  The current path is now displayed at the

opening menu.  The default DOS path is the initial drive path.

2.   The ability to save the current alignment (box volume,

box frequency, order, EQ frequency, and D,) to the BOXDATA file.

This allows you to save your favorite box alignment for each

driver.  You may also load previously saved parameters for a

starting point on a new design.

3.  Dual driver systems are now handled.  You may now choose

from three different configurations in series or parallel:

Standard --Drivers side-by-side, sharing the same box.

Push-Pull--Same box, mounted back-to-back or

front-to-front, wired out of phase.

Isobaric (constant pressure) systems--Drivers close-coupled

by a small air-tight tunnel.

See "The Loudspeaker Design Cookbook" for more information on

these variations in system design.

-6-

4.   A BOXDATA file print option which makes a complete

hardcopy of the current BOXDATA file drivers, including all

parameters.

5.   Maximum SPL and minimum power values above 20 Hz. are

denoted on the Excursion limit plot, for quick reference.  Also

these values are helpful when designing dual-driver or very heavy

power rated systems.

6.   One of the prompts in the LOAD driver sequence has been

deleted to speed up the process of parameter loading.

VIII.  ERRATA:

PERFECT BOX is under constant scrutany for improvement.  If

you have any ideas for new features/functions, send them to me at

AUSTIN ACOUSTICS.  Your suggestion could be included in the next

revision of PERFECT BOX, of which you will recieve the first copy!

By registering your copy, you will be notified of major

PERFECT BOX revisions, with new BOXDATA file parameters.  If

you send a disk, I will update to the present version, and append any

BOXDATA parameters I have recieved from others to your disk.  Please

send a COPY of your PERFECT BOX program disk (5.25 or 3.5) in a U.S.

Post Office approved disk envelope, and $40 (US) (or $51 for

Overnight Express Return.) Or, just send $40 (US) to me for program

registration instructions.

Please note that a registered user will not have to see the

annoying "Please Register" boxes that are sprinkled throughout the

program.  But this doesn't mean that the un-registered copy is

"crippled" in any way, because it isn't.  All PERFECT BOX

functions are available on the un-registered program.

Please make checks payable to:

WARREN MERKEL

9874 LANCEWOOD STREET

ORLANDO, FL  32817-4251

12/87            *** End of Document ***

-7-