Read Me for Ioline Corporation ADI 4.2 Driver PLIOL.EXP

-------------------------------------------------------

This file contains the following information:

1.  Brief history of the driver

2.  Installation procedures

3.  Driver specific information

4.  Troubleshooting errors

5.  Features to be added

6.  Error Reporting

Driver History

--------------

PLIOL.EXP was created as a protected mode driver for AutoCAD versions

11 and 12.  The driver should operate under both versions, however, it

has only been tested under version 12.

If any bugs are found please contact a Customer Service representative.

Your help in fixing the problems as they arise will be greatly

appreciated.

Installation Procedures

-----------------------

To use the protected mode ADI driver copy the file PLIOL.EXP to the

AutoCAD drivers sub-directory on your hard disk.  The command to do

this would be:

COPY B:PLIOL.EXP C:\ACAD\DRV

if AutoCAD were installed on the C drive with ACAD being the root

AutoCAD directory, DRV the drivers sub-directory, and the file PLIOL.EXP

was located on the B drive.  If the sub-directories were named

differently or AutoCAD is on another drive, you will need to use your

path to the AutoCAD drivers in place of the C:\ACAD\DRV command.

The following procedures are for configuring the driver in AutoCAD

version 12.

After the driver has been saved to the hard disk, enter AutoCAD.  Once

in AutoCAD, go to the configuration menu by either typing "CONFIG" at

the command line or selecting "CONFIGURE" in the "FILE" window.

From the configuration menu select the configure plotter option. At

this point, choose the appropriate command.  Either add a new plotter

if you have more than one plotter or change a plotter configuration.  If

you chose change a plotter configuration, answer yes to the question do

you want another driver.

Now a list of available drivers should be listed on the screen.  If you

do not see a driver selection labeled

"Ioline Corporation ADI 4.2 - by Scott Stone"

among the choices, exit AutoCAD and make sure PLIOL.EXP was copied to

the correct sub-directory.  If this selection is present, choose the

desired plotter model and follow through the detailed configuration.

When you are finished with the configuration exit out of the

configuration menu and save the changes.  The driver should now be ready

for plotting.

Driver Specific Information

---------------------------

The driver PLIOL.EXP supports all of the Ioline plotters currently in

production.  The driver uses standard DMPL command language and is able

to run under DOS and WINDOWS platforms.

Troubleshooting Errors

----------------------

Here are some problems that may occur during plotting and a possible

solution.

Problem:

AutoCAD gives an error saying check to see if plotter is on or paper

not loaded in the middle of a plot.

Solution:

Increase the time-out length in the configuration.  This error

usually signifies that the plotter buffer was full and the program

timed out before the buffer emptied and allowed more data to be

transferred.

Problem:

Plotter beeps and lights error message at start of plot.

Solution:

Make sure plotter is set for DMPL language and not HP/GL.

Problem:

Setting the pen speed in AutoCAD seems to have no affect on the

plot speeds.

Solution:

Make sure the speed override feature is turned off on the plotter.

If it is left on, the speed will be set by the plotter settings.

Problem:

Plotter does not recognize pen change commands.

Solution:

Make sure the pen change option is set on the plotter.  If the pen

change override is on, the plotter will ignore pen change commands.

Features To Be Added

--------------------

The big feature that is in the works right now is to add longplot

capability for the Signature 5000, SignMaker 5000, Studio 7, and SUMMIT

plotters.  Currently each of these plotters are allowed a maximum width

of 65 inches.  The longplot feature will allow up to 838 inch plots on

the Studio 7's and 5000 series plotters and 104 x 73.5 inch plots for

the SUMMIT.

Eventually if there is enough interest we may make an HP/GL compatible

driver also.

Error Reporting

---------------

If an error does occur try to make the same problem happen again.  If

it does happen again, please contact Customer Service for assistance.

We will try our best to fix the problem and get an updated version of

the driver to you as soon as possible if an error is found in the

driver.

If there is something you would like to see added to the driver, let us

know and we will see what we can do to implement the change in a future

version of the driver.