cadence.nov  GDK

batson

by Ed Batson

At  our  engineering  machine  shop in Tempe,  Arizona,  most  of  our

machining  is  done using drawings created in AutoCAD. But  if  a  job

comes  in  with  only  paper drawings  or  sketches  supplied  by  the

customer,  we create an AutoCAD drawing containing the features to  be

machined.  (This  has  the  added benefit  of  allowing  a  check  for

dimensions of the customer's part drawing.)

Using  AutoCAD  and AutoLISP, we have developed a  method  to  quickly

generate  an  ASCII  file containing the  x,y,z  coordinates  and  the

machining instructions required by our CNC equipment.

We use polylines to prepare a drawing for CNC code generation  because

we  can  edit  individual vertices without changing  the  starting  or

ending  point,  which  determine direction of  tool  travel.  This  is

extremely  important  in machining. Also, since  polylines  cannot  be

joined  unless  they are dimensionally correct, the  tool  coordinates

will all be qualified.

To manage 3D surfaces, we use 3Dpolys as well as meshes and  rulesurfs

converted  to 3Dpolys. To handle pockets and obstructions, we use  the

Hatch  command  and convert the hatch entity to polylines.  To  handle

text for engraving work, we use a routine that reads the SHP file  and

redraws the text as polylines.

We chose these relatively basic methods because the drawings  supplied

to us are created in various versions of AutoCAD, as well as other CAD

systems  outputting IGES files. We needed quick editing  methods_thus,

polylines.

The CNC equipment we use can calculate the proper tool path given the

finished  part dimension coordinates and the cutter  diameter.  Proper

entry  and  exit  points of the tool are  required  for  this  "cutter

compensation."  For  this,  we  insert  exploded  blocks  (which   are

polylines)  and  attach  them  to  the  polyline  defining  the  part.

Alternatively,  we  offset  the polyline in the  proper  direction  to

simulate a tool path for a specific cutter's diameter.

The  polyline  is  usually  "moved" to the  proper  elevation  in  the

drawing,  and  additional copies at other elevations can  be  used  to

simulate multiple cutter passes.

The  AutoLISP  routine  MILLNC.LSP (Listing 1), used  to  extract  the

polyline   data   and  export  it  to  an  ASCII   file,   is   fairly

straightforward. It does the following:

1. Opens a file for either writing or appending.

2. Prompts for selection of a base point.

3. Allows polyline selection(s).

4. Prompts for additional data required by the CNC machining code.

5. Reads the data contained in the selected polyline.

6. Writes the x,y,z data and the proper CNC codes to file.

7. Closes the file.

In  addition to the x,y,z coordinates, the CNC code must also  contain

the  proper G-codes, feeds, tool diameter, tool offsets, spindle  RPM,

and  other special codes such as Coolant = On, etc. Some of this  data

is  assimilated  at the time the polyline is selected by  reading  the

layer name and the z data. The balance of the required information  is

prompted for by the AutoLISP routine.

As  much  of this information as is practical is  retained  in  global

variables  to  speed up the selection of  additional  polylines  while

still in the drawing editor.

MILLNC.LSP also prompts for a base point to be used as an x,y,z  0,0,0

for all coordinates because the base must match the logical base point

to  be used on the mill. Usually the "back jaw," or stationary jaw  of

the  mill vise, is "Y = 0.0," and a "stop" placed to the left  of  the

mill vise is "X 0.0." The "Z = 0.0" is determined by the machinist  at

setup  time  and is usually the top of the part. In  some  cases,  the

"center" of a part will be 0,0,0, requiring the ability to select  any

point in the drawing as a base point.

Reading  the  vertices of a polyline record in  the  AutoCAD  database

required  a  good understanding of the information contained  in  each

field.  The AutoCAD Reference Manual is a bit cryptic, but between  it

and looking at the records of the polylines, they can be figured out.

The conversion of a rulesurf to a polyline that can be used as a  tool

path requires the following:

1.  Set SurfTab1 (line spacing) to match the required  cutter  offset.

[SurfTab1 = ((shortest end of the rulesurf

divided by the cutter diameter (minus some overlap)) ]

2.  Using AutoLISP, read the vertices of the rulesurf and  trace  over

them  with a polyline defining the new path. (You have to  change  the

order of the vertices.)

3.  Also  using AutoLISP, calculate the z offset of the  new  polyline

toolpath.  Usually  this  is to accommodate a ball-end  mill  kind  of

cutter. (Most CNC controls do not allow for cutter compensation in the

z axis.)

The conversion of a mesh to a 3Dpoly is much the same as converting  a

rulesurf, and the setting of SurfTab2 also comes into play.

Hatching is used to obtain a tool path for "pocket" milling as well as

a tool path around obstructions. Reading the information in the  hatch

entity  (which  is a block) requires stepping through  the  block  one

entity  at  a time and saving the starting and ending  points  of  the

hatch  lines  in  a  list or a file. This data can  then  be  used  to

overdraw the polyline and obtain the proper tool path.

Creating  a hatch to use in making a tool path requires the  placement

of  a "dummy line" between the objects to be hatched and  the  AutoCAD

0,0  point.  Include  this line as one of  the  objects  selected  for

hatching. This will cause the hatch lines to begin at the edge of  the

part rather than somewhere out in the middle (which makes a crazy tool

path).  Use "user" hatch, with the line spacing the equivalent of  the

required tool offset.

This  peculiarity  of hatching is caused by the fact  that  the  hatch

array  starts  about  halfway  between the  extremes  of  the  objects

selected.  By  placing  a dummy line in  the  proper  orientation  (90

degrees  from the hatch pattern), the hatch array will  start  halfway

between  the  dummy  line  and the farthest  point  from  0,0  in  the

selection set.

We  use two different methods to verify a tool path. One method is  to

"measure" the polyline with a block created from a circle representing

the tool diameter. (By setting the "measure" amount to the  resolution

needed,  a  visual confirmation of the metal that will be  removed  is

obtained.) Another method is to change the polyline width to match the

tool  diameter  and turn on fill. (This only works when  the  polyline

represents the center of the tool path.)

We  developed  several routines to facilitate  changing  objects  into

polylines and editing polylines. Most of these are simple menu macros;

others  are full-fledged LISP routines. By using simple  polylines  to

represent  tool  paths,  numerous  AutoCAD commands  can  be  used  to

manipulate,  measure, and edit the tool path before  extraction.  When

anything  other  than  a simple change is required  to  a  part  being

machined, make the change to the AutoCAD drawing, and rerun as much of

the MILLNC program as needed. This ensure

s accuracy of the CNC code.

These  methods have been adequate thus far, and at present there's  no

need for a more sophisticated package. So far, we have seen no benefit

in  using AME for CNC code extraction since all the data  required  is

present in the polyline vertices.  ?

Ed  Batson is president of Batson Tool Corp. Engineering Machine  Shop

in Tempe, Ariz.

cadence.nov  GDK

userlibrary

GCLOCK.LSP

by Paul J. Viellieux

GCLOCK  is  a graphic clock for AutoCAD users who don't  like  to  see

their  time displayed only in a numeric format. (It's also  for  clock

watchers who don't want the boss to catch them looking up at the clock

on the wall.)

After being loaded, the user can type "gc" at any time to display  the

clock. First the program looks at the variables VIEWSIZE and  VIEWCTR.

It will use VIEWCTR as the center point of the clock and VIEWSIZE as a

scale  factor for the clock face. The clock face will always fill  the

screen,  no  matter what the zoom factor is. ANGBASE  and  ANGDIR  are

changed  so  the angle of rotation is clockwise from 12  o'clock.  The

screen is cleared using grclear and points p1 through p41 are used  to

draw the face of the clock wi

th  the  grdraw  function.  The current  time  is  then  accessed  and

calculated  from the CDATE variable. A little trigonometry is used  to

calculate the location of the minute and hour hands, and they are also

drawn.

Editor's  Note: This clock is such a nice idea I could not resist  the

desire  to  expand  on  it just a little.  The  clock  was  originally

intended  to  be a static representation of the current  time  as  the

program  was  invoked. However, by using the grread function  and  its

scan  option,  the time displayed on the clock is  now  updated  every

second. In addition to the sweep second hand, a digital clock has been

added to the menu area for those who have lost the ability to decipher

the  complex  structure of analog time displays. As  in  the  original

program,  pressing  any key or the mouse button will  exit  the  clock

program. Any movement of the crosshairs will also halt the program.

Paul  J.  Viellieux  is  a  CAD  operator  and  programmer  at  Nercom

Engineering  and  Manufacturing in Neenah, Wis. He can be  reached  at

2297 Country Ln., Neenah, WI 54956.

cadence.nov   GDK

eriksen

Beginning AutoLISP

Entity Access, Part II

by Leif Eriksen

As  discussed  last month, entity access is the  process  of  finding,

updating,  deleting,  and creating entities in AutoCAD.  Entities  are

anything  you  draw inside the AutoCAD editor, such as  text,  points,

dimensions, lines, circles, etc.

All entities in AutoCAD have a unique entity name, but the name is not

permanent.  While  in  the drawing editor, the entity  name  will  not

change,  but if you exit and re-enter the editor an entity's name  may

change. This is why entity handles are provided. They will not  change

between drawing sessions.

The  entnext function may be used to determine an entity name. It  has

one optional argument. If the argument is absent, the first entity  in

the drawing is returned. Otherwise, if a valid entity name is given as

an argument, the next entity name in the drawing is returned.

Entget  returns an association list description when given  an  entity

name.  It lists every characteristic (such as layer,  color,  defining

points, etc.) of the entity and uses DXF code numbers to organize  the

list into entity characteristics. An entity with extended data will be

returned  by entget if the application name is provided as the  second

optional argument.

Entdel will delete an existing entity or restore a deleted entity. You

must provide the entity name as an argument to entdel. Entlast returns

the last entity name in the drawing.

Entsel

Entnext and entlast return an entity name do not allow you to select a

particular  entity  from  the screen. Entsel is  used  to  graphically

select  an  entity and return its entity name. In  fact,  entsel  also

returns  the  point at which the entity was selected. Figure 1  is  an

input/process/output (IPO) diagram for entsel.

Entsel  has  one  optional argument that is a  prompt  message,  as  a

string, displayed before waiting for input. For example,

Command:(entsel "\nSelect an entity: ")

Select an entity: {pick an entity}

(<Entity name: 6000001a> (0.769688 0.726378 0.0))

The first element of the list is the entity name, and the second is  a

list  representing the point picked. To get the entity list, you  must

strip  the  first  argument (entity name) from the  list  returned  by

entsel.  The  car function returns the first element in  a  list.  For

example,

Command:(setq enpt (entsel "\nSelect an entity: "))

Select an entity: {pick an entity}

(<Entity name: 6000001a> (0.769688 0.726378 0.0))

Command:(setq en (car enpt))

<Entity name: 6000001a>

Command:(entget en)

((-1 . <Entity name: 6000001a>)

(0 . "LINE")

(8 . "0")

(10 0.0 0.0 0.0)

(11 1.0 1.0 0.0)

(210 0.0 0.0 1.0)

A  shorthand  way  to write the code if you do not need  to  save  the

intermediate results is:

Command:(entget (car (entsel "\nSelect an entity: ")))

Select an entity: {pick an entity}

((-1 . <Entity name: 6000001a>)

(0 . "LINE")

(8 . "0")

(10 0.0 0.0 0.0)

(11 1.0 1.0 0.0)

(210 0.0 0.0 1.0)

If we always remove the entity name from the list returned by  entsel,

why  didn't  Autodesk design entsel to just return  the  entity  name?

Because  some AutoCAD commands require more than just an entity  name.

For  example, when you use the Trim command, AutoCAD prompts  you  for

cutting  edge(s)  and  then the object to trim. If you  reply  to  the

object  to  trim  with just an entity name,  the  Trim  command  quits

without performing the trim. Without a coordinate, Trim does not  know

on which side of the cutting edge to trim.

The  entsel list can be used as input to editing commands. If you  are

performing an AutoCAD command via the command function, the safest way

to select an object is to use an entsel list. Using only a  coordinate

may select a nearby object instead.

Complex Entities

There  are  two types of entities in AutoCAD: simple  and  complex.  A

simple entity is the most common and returns a full entity description

with  one  entget. A complex entity includes polylines,  inserts  with

attributes,  and blocks. A complex entity requires two or more  entity

names along with two or more entgets to fully describe the entity.

A  polyline  is a complex entity and starts with a  "POLYLINE"  entity

type.  The  overall  characteristics of the polyline  defined  by  the

"POLYLINE"  entity.  Next,  two  or  more  "VERTEX"  entities   define

definition  points for the polyline. Additional vertices may be  added

to  the polyline if the polyline was fitted or splined. A polyline  or

an  insert entity always ends with a "SEQEND" entity. This  allows  an

AutoLISP  program  to  determine when it has reached the  end  of  the

complex entity. For example,

Command:Pline

From point: 0,0

Current line-width is 0.0000

Arc/Close/Halfwidth/Length/Undo/Width/<Endpoint of line>: 1,1

Arc/Close/Halfwidth/Length/Undo/Width/<Endpoint of line>:

Command:(setq en (entlast))

<Entity name:6000001a>

Command:(entget en)

((-1 . <Entity name:6000001a>)

(0 . "POLYLINE")

(8 . "0")

(66 . 1)

(10 0.0 0.0 0.0)

(70 . 0)

(40 . 0.0)

(41 . 0.0)

(210 0.0 0.0 1.0)

(71 . 0)

(72 . 0)

(73 . 0)

(74 . 0)

(75 . 0)

)

Command:(setq en (entnext en))

<Entity name:60000034>

Command:(entget en)

((-1 . <Entity name:60000034>)

(0 . "VERTEX")

(8 . "0")

(10 0.0 0.0 0.0)

(40 . 0.0)

(41 . 0.0)

(42 . 0.0)

(70 . 0.0)

(50 . 0.0)

(71 . 0)

(72 . 0)

(73 . 0)

(74 . 0)

)

Command:(setq en (entnext en))

<Entity name:6000004e>

Command:(entget en)

((-1 , <Entity name:6000004e>)

(0 . "VERTEX")

(8 . "0")

(10 1.0 1.0 0.0)

(40 . 0.0)

(41 . 0.0)

(42 . 0.0)

(70 . 0.0)

(50 . 0.0)

(71 . 0)

(72 . 0)

(73 . 0)

(74 . 0)

)

Command:(setq en (entnext en))

<Entity name:60000068>

Command:(entget en)

((-1 . <Entity name:60000068>)

(0 . "SEQEND")

(8 . "0")

(-2 . <Entity name:6000001a>)

)

An  insert entity with attributes will have multiple entities.  First,

it  will have an "INSERT" name to define name, location, and  scaling.

This is followed by attributes (if defined in the block with attribute

definitions). The last attribute is followed by a SEQEND entity.

Nentsel

When  you  pick a complex entity with entsel, it  returns  the  entire

entity's  name, not the portion of the entity picked. For a  polyline,

if  you  pick anywhere on the polyline, the polyline  entity  name  is

returned, not the vertex selected. For a block, the insert entity name

is returned and not the line, arc, or other entity you selected within

the block.

The  nentsel  function  addresses these problems. When  you  select  a

simple entity, it returns an entity name and coordinate list just like

entsel.  If you select a complex entity, the component entity name  is

returned, along with other information. If you select a polyline,  the

entity  name  of the vertex before the select point is  returned.  For

example,  if  you pick the fourth segment of a  polyline,  the  fourth

vertex  entity  name  is  returned. Figure 2 is  an  IPO  diagram  for

nentsel.

If  you select a block reference (insert entity), the entity  name  of

the  entity  within the block is returned. In addition to  the  entity

name  and pick point, two other pieces of information for  blocks  are

returned  (except  for attributes). The third element is  a  model  to

world  transformation  matrix. It is used to convert  from  the  block

coordinates  (relative to the block insertion point) to the  WCS  (not

the UCS).

The  fourth element of the nentsel list is a list of containing  block

entity  names.  The  containing block entity  names  starts  with  the

innermost  block first and outermost block last. For example,  suppose

you select an entity inside block C (<Entity name:40000163>), which in

turn is in block B (<Entity name:40000220>), which is in turn in block

A (<Entity name:600001ba>), which is in the current drawing:

Command:(nentsel "\nSelect an entity: ")

Select an entity: {pick an entity}

(<Entity name:40000096>

(1.26927 3,26512 0.0)

(1.0 0.0 0.0)

(0.0 1.0 0.0)

(0.0 0.0 1.0)

(4.97671 4.15295 0.0)

(<Entity name:40000163>

<Entity name:40000220>

<Entity name:600001ba>)

)

BlkPath

The demonstration program this month is blkpath (block path). It lists

the entity type selected and the block that contains it. It also lists

the  names  of  any containing blocks from  the  inner-  to  outermost

blocks. The AutoLISP source code for blkpath is in Listing 1.

The program first checks if the entity selected is within a block.  If

not,  nentsel  will  return a list of only  two  elements  (just  like

entsel).  If the entity is an attribute in a block, it only returns  a

two-element list, so its path is not displayed.

Next  the  entity  type  is  extracted  and  displayed.  The  list  of

containing  blocks is extracted from nlist and placed in blklist.  The

first  element  of blklist is extracted, and the block name  is  found

(blkname) and displayed.

Within  a loop, the next block entity name is extracted from  blklist.

The  block  name  is extracted and displayed until  blklist  is  empty

(nil).

Last  month  I started a description of entity access.  The  functions

reviewed  allow  sequential access to entities in your  drawing.  This

month  you  have seen entsel and nentsel used to directly  select  the

entity  to  be checked. Next month we will  investigate  functions  to

modify and create new entities. If you have any comments or questions,

I  can  be  reached  through CADENCE  magazine  or  on  CompuServe  at

73177,365.  ?

Leif  Eriksen is president of Discovery Systems, an  AutoCAD  training

and consulting firm in Irvine, Calif.

Problems

1. What is returned by entsel?

2. What is returned if nentsel selects a circle?

3. What is returned if nentsel selects a circle within a block?

4.      What  is different in what is returned by entsel  and  nentsel

when a polyline is picked?

Answers

1.Entsel  returns a list with two elements. The first is the  selected

entity name. The second element is the 3D point picked.

2.For  simple  entities, such as circles, the same list as  entsel  is

returned.

3.If an entity, such as a circle, is selected within a block, a  four-

element  list is returned. The first element of the returned  list  is

the  entity name of the circle. Entsel, on the other hand, would  just

return the entity name of the insert entity. The second element is the

coordinate picked. The third is a transformation matrix. It allows you

to  translate  coordinates  to WCS. The fourth element is  a  list  of

containing block entity names.

4.Entsel returns the polyline entity name. Nentsel returns the  vertex

immediately before the selected point on the polyline.

head.cad.nov  GDK

Making Turns

by George Head

The  most valuable thing you should have learned in working in  3D  is

the  importance of controlling the User Coordinate System (UCS).  This

gives you the ability to "point and shoot" with just about any AutoCAD

command.  The  UCS  constantly  places you in  2D,  anywhere  in  your

drawing.

Controlling  the icon is anything but easy. I always know which way  I

want  the  x or y arrow to point, but even now, when I issue  the  UCS

command  and  do  a rotate around x or y, it usually  goes  the  wrong

direction.

Working in 3D in AutoCAD should be visual and intuitive. You shouldn't

have  to think about rotating y around x. You should be able to hit  a

key  and move the icon around in the direction you want it to  go.  If

it's  not  the right direction, keep hitting the key,  and  the  arrow

continues to turn into position.

That's  the  beauty  and simplicity of AutoLISP.  Because  of  my  own

frustration  in  getting  the UCS icon arrows to go  the  direction  I

wanted,  I  wrote a simple AutoLISP program. It should save  you  much

aggravation and frustration.

Over  the years while working in 3D, I noticed that once the  UCS  was

set  on my model in a given plane, most of my shifting of the UCS  was

generally  90  degrees at a time. A good example of this was  in  last

month's article on dimensioning in 3D. You learned that the entity you

were dimensioning had to be parallel to your current UCS. The  easiest

way  to do that was to use UCS Entity, which forced your  current  UCS

parallel.  But you had no control over the direction of the  x  arrow.

Many  times  you needed to simply rotate the x arrow in  the  positive

direction you were dimensioning so your dimension text would  properly

line up.

Dimensioning is only one example of how much of your UCS alignment  is

at  90-degree  angles  to where you  currently  are.  Therefore,  this

AutoLISP program quickly and visually rotates the UCS counterclockwise

90  degrees at a time either vertically or horizontally. Just press  x

or  y to tell it which arrow you want to point, then press  the  space

bar  and the x or y arrow will rotate 90 degrees. Each time you  press

the  space  bar,  the arrow will rotate another 90  degrees.  Thus  by

pressing  the space bar two or three times, you have the x or y  arrow

going  the  way you want it to. This is certainly a  lot  easier  than

using the pull-down menus or even typing in the UCS command. It's much

more  intuitive to say you want to move the x arrow than  to  indicate

that you want to rotate around z even though the latter is technically

more accurate.

To  use  the program, first type the following code into  an  ordinary

text file using any text editor (e.g., EDLIN). You may also use a word

processor, but remember to save it as a DOS text file (ASCII), not  in

a word processing format.

I  call the program Z since you are changing the direction of  z.  Run

the  program by typing the letter Z and hitting <Enter> or  the  space

bar. You can call the program anything you wish.

If  you call the program Z, put it in a file called Z.LSP.  (Actually,

you  will probably want to place it in the ACAD.LSP file, but more  on

that later.) Once you have created the file and typed in the  program,

it  must  be  loaded each time you enter the drawing  editor.  At  the

AutoCAD command prompt, type:

(load "Z") <Enter>

This  will  load  the  AutoLISP file. Now when you  want  to  use  the

program,  type  Z <Enter>, and the program will  begin.  (Actually,  I

always  use the space bar instead of <Enter> because it's  closer  and

only takes one hand.)

A  preferred  method of loading the program would be to append  it  to

your  current  ACAD.LSP file in your \ACAD directory. If  one  doesn't

exist,  you  may create it. If it does exist, make certain  you  don't

delete  or replace it. It's generally there for a reason.  Almost  all

third-party  software  uses an ACAD.LSP file in some  manner.  If  you

already have one, then go to the end of the file and begin typing this

program at the end. The advantage of this is that the ACAD.LSP file is

always  automatically loaded each time you enter the  drawing  editor.

Therefore,  if  your AutoLISP programs are in the ACAD.LSP  file,  you

don't have to load them every time.

To use the program most effectively, I like to set the Ucsicon to  the

origin.

Type: Ucsicon <Enter>

Type: Or <Enter>

This  will  place  the UCS icon right on your  object  each  time  the

program is used. It's much easier to see the direction the arrows  are

going if they are sitting on the model itself.

In  Figure 1 there are a series of lines that have been drawn in  plan

view.  At  the  end of the line you want to turn the line  up  in  the

positive z direction. Now begin the program.

Type: Z <Enter>

The program asks you to select the point of origin or <Enter> for  the

last  point. (For each <Enter> it's easier to press the space  bar  on

the keyboard.)

Type: <Enter>

Notice how the UCS icon snaps immediately to the endpoint of the line,

which  was the last point entered (Figure 1). If the Ortho is on,  you

are  only able to stay on the current plane. I like to keep  Ortho  on

when I'm using this program since it is designed to rotate the UCS  in

90-degree increments. The Ortho command helps me visualize which way I

can go without getting into trouble.

Now type X and then continue to press the space bar. Notice how the  x

arrow  rotates  counterclockwise in one direction (Figure 2).  If  you

type T, which stands for toggle, it remembers that you are still in  x

mode.  Now each time you press the space bar, it reverses the x and  y

and  rotates  around  the opposite plane. Play  with  the  program  by

choosing X, Y, and T, and continue to press the space bar until the  x

and  y  arrows  are pointing up as in Figure 2. Once  the  arrows  are

pointing the way you want them to, ty

pe  S. This stands for stop and exits you from the  program.  Continue

with  the Line command. With Ortho on, it's now easy to draw  straight

up (Figure 3).

Do  you see how easy this makes life in 3D? The program does a  couple

of things for you. If you have set your Ucsicon to origin, the program

either pops the icon to your last point or lets you set a new  origin.

Many  times  I use the program even when the arrows don't need  to  be

adjusted  just  to pop the icon to a new origin or the  last  point  I

picked. Then I immediately type S to stop the program.

There  are innumerable ways the program can be used. This is just  one

example.  At times you will want to use one of AutoCAD's  2D  commands

such as Rotate. Look at Figure 4. If the UCS remains where it is,  you

can't  rotate the circle. By rotating the UCS 90 degrees, you can  use

the Rotate command and rotate the circle vertically (Figure 5).

No  program  is  absolutely perfect in all situations.  The  way  this

program  is written works for me. But that doesn't mean that it  can't

be modified. You can use it as a base to create other programs or as a

routine inside of a special program to accomplish specific goals.  For

example, you may want to change the increment to a 45-degree  rotation

instead of 90. You may want to give the user input as needed as to the

degree of rotation. (C:Z) might be a call from within another program.

The variations are endless.

Let's look at how the program works so you can modify it.

(defun C:Z ()

This defines the function with Z as its name.

(setq lpt (getpoint "\nOrigin <Last Point> "))

(if (/= nil lpt) (command "Id" lpt))

The  first  line asks for the point of origin and  assigns  the  point

picked  to the variable lpt. If you press <Enter>, then lpt is set  to

nil. If lpt is not nil, then the AutoCAD command Id is issued with lpt

as the point identified. This makes lpt the last point picked.

(command "UCS" "O" "@")

This  is  the  UCS command that places the origin  at  "@,"  which  is

AutoCAD shorthand for the last point picked.

(setq e 1)

(while e

This begins a continuous loop until the variable e is purposely set by

the program to nil, which stops the loop.

(setq b (getstring))

(if (= b "") (setq b b1))

(setq b (strcase b))

This prompts you for text and is terminated with <Enter> or the  space

bar. If you hit <Enter> or the space bar, the variable b is set to b1,

which is the last text that was entered. The variable b is then set to

all uppercase.

(if (= b "Y") (command "UCS" "X" "-90"))

(if (= b "X") (command "UCS" "Y" "90"))

(if (= b "T")

(progn

(command "UCS" "Z" "90)

(setq b b1)

)

)

If  b is "Y," then the UCS is rotated around x. If b is "X," then  the

UCS is rotated around y. If b is "T," then the UCS is rotated around z

and the variable b is set to b1, which was the last X or Y entry made.

You don't want b to contain the "T" for the next time around. Then the

(progn  group  statement  is closed with a  parenthesis  and  the  (if

statement is closed.

(if (= b "S") (setq e nil)

(setq b1 b)

)

)

If  b is "S," then you want the loop to stop so the variable e,  which

is the loop controller, is set to nil. Then b1 is set to the  variable

b so that the default of the previous entry can continue. The loop and

the program are then closed.

As  you can see, even some simple AutoLISP programs can make  3D  more

usable  and life in 3D a lot simpler. The programs do not have  to  be

long  and complicated. But if you find yourself doing the same  things

repeatedly, think of a way to automate the procedure, to take some  of

the  time-consuming thinking out of it. Let AutoLISP do  the  thinking

for you.  ?

George  Head's  column teaches the fundamentals of drawing in  3D.  He

welcomes  your suggestions and comments. Write to him c/o  CADENCE  at

Ariel Communications Inc., PO Box 203550, Austin, TX 78720.

Cutlines

Figure 1. The UCS icon snaps to the last point entered, preparatory to

manipulating the icon.

Figure  2.  By typing X and hitting the space bar, the  x  arrow  will

rotate.  By  typing  T, you can change models and  rotate  around  the

opposite plane.

Figure 3. With the UCS icon oriented as in Figure 2, continue to draw.

With Ortho on, it is now easy to draw straight up.

Figure  4.  By  leaving the icon where it is, you  cannot  rotate  the

circle, but . . .

Figure  5.  rotate the UCS 90 degrees, and you can rotate  the  circle

vertically.

cadence.nov.kp

kramer

advanced autolisp concepts

The AutoLISP Compiler

by Bill and Denise Kramer

The  AutoLISP  "compiler,"  a product of Autodesk  Ltd.  in  the  U.K.

available   to  European  AutoCAD  third-party  developers,   compiles

AutoLISP source code and creates a binary load file. Programs compiled

under  this system reportedly run and load several times  faster  than

normal  AutoLISP programs. Several European software  developers  have

given  us  glorious reports about the operational  characteristics  of

programs  using  this  compiler. These reports intrigued  us,  and  we

thought our U.S. and Canadian readers might be interested in  learning

about  this utility as well. Recently, while working with  a  European

developer, we got the chance to try out the compiler system for a  few

days.  The results were both impressive and disappointing at the  same

time.

Perhaps  the  first  item to clear up concerns  the  term  "compiler."

Although there are several gray levels in between, most languages  can

be  classified  as being either compiled or  interpretive.  In  simple

terms,  a compiled language is one that must first be processed  by  a

compiler  program; an interpretive language requires  an  interpreter.

Let's consider each one in more detail.

COMPILED LANGUAGES

A  compiler  program reads source code and converts it  into  machine-

specific  command sequences. The output of a compiler is often  called

object  code.  Compiled languages were first used in the  1950s  as  a

method  of  increasing programmers' productivity. For example,  it  is

much easier to think of an equation in the form C = A + B than in  the

multitude  of steps required for the machine to actually  perform  the

operation.  For  simple values, which can be placed  into  a  register

within  the  machine, adding two numbers and saving the  result  in  a

different  memory cell is not that difficult. But the problem  becomes

complex when the numbers are larger than the registers of the computer

and  must  be  dealt with in segments.  (Consider  adding  two  32-bit

numbers  in  an  8-bit  machine.  Our  simple  example  becomes  quite

complex.)  The first compiled languages to gain widespread  acceptance

were  FORTRAN for engineers and scientists and COBOL  for  accountants

and bookkeepers.

When compiling a source program into an object program, the first step

is to make sure all statements in the source program can be understood

by the compiler. The variables used are checked to make sure they  are

of  the  correct data type and the actions to be performed  with  them

make sense. Modern compilers have a great deal of flexibility and  are

the  result of many years of research into programming  practices  and

methods. After checking that all the input source program is  workable

by  the  compiler,  a  series  of  machine-specific  instructions  are

generated, often called the object program. Now instead of referencing

a  variable by a name, a relative location is used where the  variable

data may be found or stored.

Compiling  generally will not yield a final executable program.  After

all the segments of a computer program have been compiled into  object

programs,  a  load  module  (executable  program)  must  be   created.

Typically,  another  program is used to create the  load  module  from

several  object programs. Programs that create load modules are  often

called  linkers  or load module generators. Although  there  are  many

programming  compilers with integrated linkers, linking is a  separate

step  in  the evolution of a source program to a  compiled  executable

program.

INTERPRETIVE LANGUAGES

Interpreters,  on  the other hand, process a program  by  reading  the

source code and acting on each statement as it is encountered. An easy

way  to think of an interpreter is that each statement  is  "compiled"

and  then executed immediately. Interpretive programs require more  of

the computer at run time than compiled programs. This is because  each

statement in the program must be evaluated to determine what is to  be

done  each  time  it  is encountered in  the  program.  As  a  result,

interpreted programs run much slower than compiled programs. For  most

engineering  applications  where a great deal of number  crunching  is

required, interpretive programs are considered too slow.

But interpreters do offer some unique capabilities over compiler-based

languages. For example, in an interpretive environment, statements can

be evaluated as they are typed into the computer. If all we wanted  to

know is the result of dividing 16,345 by 37.5, it would be  bothersome

to write a small program, compile it, and then execute it. Instead, we

would reach for a calculator or, if we are inside AutoCAD at the time,

type (/ 16345.0 37.5) to get an "instant" result. Developing  programs

is much faster inside an interpreter since we can easily test the code

in small fragments as it is being developed. This is why  interpreters

were  created. People time (i.e., development) is considered far  more

expensive than computer execution time. The best of both worlds exists

when a program can be developed in interpretive mode and compiled into

a  faster-executing  program when completed. (An example  of  this  is

BASIC, which exists in both interpretive and compiled forms.)

AutoLISP is an interpretive language. Each time a parentheses pair  is

opened,  the contents in that list are evaluated to determine what  is

to be done. If the first member of the list is a subroutine or defined

function,  the remainder of the list is considered data or  parameters

for  use by the function (or subroutine). When the rumor started  that

an  AutoLISP compiler might exist that works properly in  the  AutoCAD

environment, a great deal of interest was spawned.

The AutoLISP "compiler" was created in the Soviet Union several  years

ago  and  then sold to Autodesk Ltd. in the United  Kingdom.  Autodesk

U.K.  then  released  the  "compiler"  to  the  European   development

community   for  developing  complex  applications.  It   has   gained

widespread  acceptance  in Europe, and we have heard  that  some  U.S.

based developers have also started to use it.

Autodesk in the United States has not released the "compiler" to  U.S.

and Canadian developers since it perceived the values of the increased

speed versus coding limitations to be insignificant. In addition,  the

programming  style  used to develop the so-called  compiler  was  very

different  than  that of the Autodesk core group,  and  the  resources

required to clean up the code was not considered a wise expenditure as

opposed to improving the base AutoCAD system.

This is not to say that Autodesk is not interested in speeding up  the

AutoLISP  environment; it is interested. Instead, it  is  recommending

that  developers  switch  to ADS if speed is an  issue.  Autodesk  has

stated  it  will  implement some of the  concepts  from  the  European

"compiler"   to  improve  the  performance  of  AutoLISP   while   not

compromising the flexibility of the overall environment.

So  how does the AutoLISP "compiler" work? To begin with, it is not  a

compiler  in  the true sense of the term. Instead, it is more  like  a

"tokenizer."  Variables are reduced to smaller byte-sized  units  that

can  be referenced faster inside the evaluation phase of AutoLISP.  In

addition, the ATOMLIST is replaced by what is commonly referred to  as

a  hash  table. The use of a hash table reduces the time  required  to

locate  a  symbol.  In  normal  AutoLISP,  the  ATOMLIST  is  searched

sequentially  each  time  a  symbol  is  referenced.  The  bigger  the

ATOMLIST, the longer the search time. A hash table, on the other hand,

permits faster access through a keyed access, in the same manner as an

indexed  database. The combination of reducing the size of the  symbol

names and using a hash table improves the overall performance of  most

applications.

To  accomplish  this, the ACADL.EXP file (for AutoCAD Release  11)  is

replaced  by  a  new  ACADL.EXP, which is  slightly  larger.  The  new

ACADL.EXP works with both the "compiled" (tokenized) LISP code as well

as normal AutoLISP code.

Another program is provided that tokenizes AutoLISP ASCII source code,

creating a BI? file. (The ? is replaced by a number that reflects  the

release  being used. In the case of AutoCAD Release 11, the files  are

BI4.)  If a BI? file is to be loaded into AutoLISP, the BI?  extension

must  be specified. BI? files may be renamed to LSP if desired so  the

extension  does  not have to be part of the load expression.  (But  be

careful not to destroy the original LSP source!)

To  test  the AutoLISP tokenized system, a series of  small  functions

were created (Listings 1 and 2). Six different AutoLISP features  were

tested  to  determine  if the  tokenized  system  would  significantly

improve program performance.

Most  of  the  functions  in Listing 2 are  simple.  Each  performs  a

specific  action that exercises a particular aspect of  AutoLISP.  For

example,  the  functions  loop1 and loop2 test the  repeat  and  while

loops, respectively. Listing 1, however, does contain some interesting

functions that should be explained. The first is timer, which accesses

the  system date and time using the DATE system variable. DATE is  the

current  date  and  time supplied in Julian time  as  a  real  number.

(Julian  time is a count of the number of days since 46 B.C.  The  day

number  is found to the left of the decimal and the fraction of a  day

is  found to the right. When timer is invoked, it accesses the  Julian

date  setting. If the global variable clock is bound to nil, clock  is

set to the value retrieved from the DATE variable. If clock is already

set  to  a  value  (from  a previous run  of  timer),  the  result  of

subtracting  the current time from the value in clock is saved in  the

variable now. Clock is then set to nil so that the next time timer  is

run,  it  is initialized to the current time. Timer then  returns  the

elapsed time if clock is bound to nil or nil if clock has a value.

The  function c:test then runs timer, a test routine (from Listing  2)

and then saves the result of running timer again. Each time c:test  is

run,  the lists t1, t2, . . . t6 are updated by saving the  result  of

the  run. Once c:test has been run several times (we ran it  10  times

for  each  setup),  the function c:aves can be run to  report  on  the

average  time  as  saved in the t1, . . . t6  lists.  To  compute  the

average times, apply is employed to add the time results together. The

sum is then divided by the number of members in the list and passed to

the report function. Report outputs the concatenation of a  descriptor

string and the result of converting the elapsed average time to a real

number. Note that the elapsed average time is multiplied by 86,400  to

convert  it  from  fractions of a day to seconds (24  hours/day  5  60

minutes/hour 5 60 seconds/minute = 86,400 seconds in a day).

The  results of our testing are shown in Table 1. We ran each test  10

times  using  the  normal  AutoLISP  environment,  the  modified   EXP

environment  with  "compiled" code, and again with  the  modified  EXP

environment  without  "compiled" code. The standard deviation  of  the

normal  and "compiled" code was very small, but the  interpreted  code

running  in  the modified EXP environment was  quite  erratic;  hence,

those  values were not reported. The extreme deviations seemed  to  be

related to a heap management problem; however,

time did not permit more extensive testing.

Overall,  "compiled"  code did run significantly  faster  than  normal

interpreted code. For mathematically bound functions, the time savings

were about 4:1. Although this ratio is good, it is several factors off

when  compared to ADS-based applications where the  timesaving  ratios

are  measured  in the tens to one. Repeat and while  loop  timing  was

improved as well. The disparity in the while loop may have been caused

by  using  a numeric relation (i.e., (< II N)) instead of  some  other

method. String manipulation and output did not yield faster execution,

which was somewhat disappointing. An interesting side note is that the

greatest  fluctuations  of timing came about when  using  "uncompiled"

code  with  the  modified  AutoLISP  EXP.  String  manipulation   with

"uncompiled"  code was about two to three times slower on  average  as

compared with normal AutoLISP.

The  results  of our experiments show that although some time  may  be

saved  by  using  a "compiler," there is much more  to  be  gained  by

working  with  ADS.  Although ADS is  not  for  everyone,  third-party

developers  should  seriously consider the compiled C  environment  if

execution speed is a problem. In addition, most AutoLISP routines  are

interactive  in nature and manipulate strings as much as numbers.  For

these kinds of applications, there is nothing significant to be gained

by switching to the AutoLISP "compiler."

In another, more extensive experiment, we tested loading times for BI?

files  versus  LSP files. Our experiments showed that BI?  files  load

about  three to four times faster than the original LSP  files  which,

for  some  applications,  can be very rewarding.  Another  aspect  for

third-party developers to consider is that "compiled" AutoLISP code is

somewhat   protected.   Our  investigations  did  not   turn   up   an

"uncompiler,"  whereas  there are several programs  available  in  the

United States and Canada to unprotect protected code.

To test the "compiler" in a truly rigorous manner, we turned it  loose

on  NC-AUTO-CODE and APT-AUTO-CODE to see what could be  gained.  This

can  be classified as "abusive" testing since the  AUTO-CODE  products

were  not originally written to be used by the compiler.  Our  results

were  very disappointing, since one of the NC-AUTO-CODE modules  could

not "compile" at all. There are limitations in the AutoLISP "compiler"

system  that  restrict development. (If you knew about them  to  begin

with,  you  would  code  the application  differently!)  Some  of  the

restrictions  had  to  do  with code size  and  the  number  of  local

variables  that  can be defined in a single function.  Large  programs

cannot be "compiled" unless they are broken up into smaller  segments.

In addition, the same large programs will not even load and run in the

interpreter.

Our advice to U.S. and Canadian developers is not to use the  AutoLISP

"compiler."  The potential for conflicts with other user products  are

very  real.  In those cases, who would be considered  the  responsible

party?  For users, the compiler might represent a method of  improving

throughput  of existing software, but only if you have access  to  the

source code to begin with. We had also heard that earlier versions  of

the modified AutoLISP EXP could not work with protected AutoLISP code,

which signifies another problem regarding new versions of AutoCAD.  In

addition,  it was reported by several developers that new versions  of

the AutoLISP "compiler" normally lag behind the release of AutoCAD  by

several  months and each different version of AutoCAD requires  a  new

EXP as well as "compiler."

The  experiment was fun, however, and we certainly hope that  Autodesk

will  make use of the concepts used in the "compiler." The hash  table

did improve load and symbol access time, which is of great interest to

applications  developers.  On  the  other  hand,  it  would  be   very

disappointing  to  see  the  limitations  of  the  so-called  compiler

integrated in future AutoLISP releases. Finally, if you are interested

in improving the speed of an application, switch to ADS.

We  are  interested  in  hearing from other  developers  who  use  the

AutoLISP  "compiler." Send comments or questions to Bill  Kramer,  c/o

Ariel   Communications.  Until  next  month,  keep  your   parentheses

balanced!  ?

Bill and Denise Kramer are the principals of Kramer Consulting Inc., a

Dublin, Ohio software development company that specializes in  CAE/CAM

development for AutoCAD.

Table 1. Test results of normal "compiled" code.

cadence.oct zinsmeister   GDK

Poor Man's Spell-Checking ... and More

by George E. Zinsmeister

If you find proofreading and spell-checking AutoCAD drawings with lots

of text or attributes boring, time-consuming, and fraught with  error,

read on.

The  programs EXTTXT.LSP (Listing 1) and RESTORE.LSP (Listing 2)  work

in  conjunction with your word processor's spell-checker. Just  follow

these steps:

1.  If the drawing does not have handles turned on, turn them on  with

the Handles command.

2.  Load EXTTXT.LSP by typing (load "EXTTXT") at the  command  prompt,

and  run it by typing EXTTXT at the command prompt. This will  extract

the desired text from a drawing and write it to a file named $.EDT  in

the current directory. You can choose to extract all the text from the

drawing or only what you select.

3. Spell-check $.EDT and save it as an ASCII file. This can be done in

the  AutoCAD  shell if your word processor fits in the  shell.  If  it

doesn't, save the drawing, quit AutoCAD, and spell-check $.EDT outside

AutoCAD.

4.  Get back into AutoCAD and reload the drawing. Load RESTORE.LSP  by

typing  (load "RESTORE") at the command prompt, and run it  by  typing

RESTORE at the command prompt. Any text changed in the $.EDT file will

be  updated  in the drawing. The RESTORE program asks if you  want  to

restore  the  attributes  as processed. When  restoring  text,  always

answer  "No"  to this prompt because text is always updated as  it  is

processed. The results are the same if you answer "Yes," but the extra

processing is unnecessary.

5.  If  handles  are not needed for other applications,  they  can  be

destroyed using the Handles command.

To  spell  check  attributes, use EXTATT.LSP (Listing  3)  instead  of

EXTTXT.LSP  and select block inserts with attributes instead of  text.

When  the  RESTORE program asks if you want to restore  attributes  as

processed,  answer "Yes" to see the attributes restored on the  screen

as  they  are  modified. If there are many  block  inserts  with  many

attributes to update, this could take a long time. If you answer "No,"

the processing will be faster, but you will not see the changes on the

screen.  To  see  the changes, the drawing must  be  regenerated  when

RESTORE is finished. You do not have to regenerate the drawing  unless

you want to check the changes.

ADDITIONAL INFORMATION

Lines  will be restored to their respective positions in the  drawing.

Blank lines will also be restored, leaving text entities or attributes

with  no  text  in them. It is better to erase unwanted  text  in  the

AutoCAD editor. Attributes with no text may be desired so this is  not

a problem with attributes.

If $.EDT is edited so that it has fewer lines than the original  $.EDT

file,  RESTORE will only restore the lines that are in the  file.  The

remaining lines will not be changed. However, make sure that there are

no unwanted blank lines at the end of the file. If $.EDT is edited  so

it  has  more lines than the original $.EDT file,  RESTORE  will  only

restore  the original number of lines and prompt that the extra  lines

were not restored.

Text  or block inserts with attributes may be selected using a  window

without worrying about accidentally selecting other types of entities.

EXTTXT and EXTATT will filter out the appropriate entities to process.

When  the  "All"  option for text selection is used or  when  text  is

selected using a window, the text lines will appear in the $.EDT  file

in  the order in which they were created. This will make it easier  to

edit  the file, since it is likely that any long blocks of  text  were

created in the order they are to be read. If individual lines of  text

are selected, they will appear in the reverse order in which they  are

selected. Thus, to make the individually selected lines appear in  the

order they are to be read, pick them in reverse order.

Attributes  will always appear together as complete sets in the  $.EDT

file  in the order they are displayed onscreen. When the "All"  option

for  attributes  is used or when block inserts are  selected  using  a

window,  the sets of attributes will appear in the order in which  the

block  inserts were created. Individually selected block inserts  will

appear in the reverse order in which they are selected.

Always  keep  a  backup  of your  drawing  when  using  programs  that

automatically make changes in your drawing. While correct changes  can

be made very quickly, so can incorrect changes.

OTHER IDEAS

Use any of your word processor's features to edit $.EDT. For  example,

use  global search and replace to change text globally in  a  drawing.

Global  search and replace is also possible with attributes, but  this

feature is already available with the AutoCAD Attedit command. You can

also create specialized dictionaries for various disciplines, vendors,

or clients to facilitate spell-checking.

Try importing text into an AutoCAD drawing. To do this, first create a

single  line  of dummy text in the drawing. The contents of  the  line

need  be only one character. Then array this text entity downward  the

number of lines you expect you will need (or more to be on the  safe).

Array other columns of text to the right at the same time if you  want

several  columns of text. Run EXTTXT and select the dummy text with  a

window. Edit $.EDT or even create a new $.EDT file; then run  RESTORE.

The new text will appear in the drawing in place of the dummy text.

You  can  free  up an AutoCAD workstation by extracting  the  text  or

attributes  and using a different computer for editing $.EDT. You  can

also speed up AutoCAD text editing by using a windowing or application

switching  capability.  For  example, I  use  Software  Carousel  from

SoftLogic  Solutions.  This lets me run EXTTXT or EXTATT  in  AutoCAD,

switch  to  WordPerfect  to edit the $.EDT file, and  switch  back  to

AutoCAD without having to exit either of the programs.

HOW IT ALL WORKS

The  concept for these programs is simple. The front end of EXTTXT  is

the  text selection process. An ssget "X" filter is used to  create  a

selection set of all the text in the drawing. If all the text is to be

processed,  then  this  selection set is used. However,  if  the  user

chooses  to select the text, a selection set of the selected  entities

is created. Then the intersection (i.e., common entities) of these two

selection  sets  is  found  using INTERS.LSP  (Listing  4),  and  this

intersection is used for processing.

This assures that only text entities are processed.

EXTTXT  produces  three  files that should be  deleted  when  you  are

finished:  $.EDT,  $.HDL  (which  contains the  handles  of  the  text

entities in the same order as the entities appear in $.EDT), and $.CMP

(which is identical to $.EDT). $.CMP is used to compare lines of  text

with  those in $.EDT to find which ones have been changed, so  do  not

edit it.

EXTATT  operates similarly to EXTTXT except that a list,  entlist,  is

used  to  collect the attributes in the proper order  for  writing  to

$.EDT.

RESTORE reads the three files line by line. If the line from $.EDT  is

different  from the line read from $.CMP, then the  entity  associated

with the handle read from $.HDL is updated.

With practice, you will find these programs easy to use and  versatile

and will discover additional ways to use them.  ?

George  E. Zinsmeister is in the mechanical engineering department  at

the University of Massachusetts in Amherst.

Tips And Tricks  GDK

by Tandy Willeby

CADENCE Staff

This  month we have four small routines that might help you  with  the

tedious  task of dimensioning. The first two routines aid in  changing

the  look of some dimensioning entities to a form more  familiar  with

non-CAD   drafting.  The  other  two  are  utilities  to  help   place

dimensions.

AL.LSP

This  function  creates  a leader line more  suited  to  architectural

drawings. Normal AutoCAD leaders consist of only straight lines  while

architectural  leaders  are  created  using  smooth  freehand  curves.

Although true freehand type curves are difficult to create in AutoCAD,

a  splined  polyline is the next best thing. AL.LSP creates  a  zigzag

Pline  from the arrow location to the text location and  then  splines

the  Pline.  After the smooth curve is created, the  leader  arrow  is

placed on the end and the Dtext command is invoked to place the leader

text in the drawing. Because the DIMSCALE and DIMTEXT system variables

are  used in conjunction with the dimension leader command,  the  text

and arrowhead sizes match that of other dimensions in the drawing.

IB.LSP

When dimensioning arcs and circles, AutoCAD's dimension program places

the diameter symbol (0) and the radius symbol (r) at the beginning  of

the text string corresponding to the dimension. In some cases, drawing

standards  require  a  diameter  dimension  to  be  followed  by   the

abbreviation DIA instead of the normal diameter symbol. Likewise,  the

radius abbreviation R sometimes needs to follow the dimension text.

The  IB  (In  Back)  function is an automated  way  to  achieve  these

results.  The  program explodes the  selected  associative  dimension,

retrieves  the  text  information,  restores  the  dimension  to   its

associative  state and updates the dimension with the new edited  text

string.  For example, 012.67 would convert to 12.67 DIA;  r5.76  would

convert to 5.76 R.

MOVEDIM.LSP

Sometimes the location of a dimension text string selected by  AutoCAD

is  not ideal for the drawing in which it is placed.  However,  moving

the  text  in a dimension can be easily  accomplished  using  MOVEDIM.

After selecting the dimension text that needs to be moved, the program

runs  through a grread scan loop and updates the text location to  the

current  cursor location on each pass. When a point is  selected,  the

text stays in that location and retains its associative link with  the

dimension.

APT.LSP

This  function lets you snap to the top the endpoint of an  arrowhead.

Arrowheads are created using solids, so the Endp Osnap option will not

work  when the location of the arrow tip is needed. APT  collects  the

solid information from the arrowhead selected, extracts the  associate

12 value (the endpoint), and returns that value to AutoCAD. To  ensure

that an arrow is easy to select, the program picks up the entity using

a  crossing  window the same size as the arrow  being  selected.  This

allows the pick point to be slightly off the actual arrow. To use  the

APT  routine, simply load APT and enter (apt) at any prompt  that  you

would normally use any other Osnap filter.