T H E

I N S P E C T O R ' S

A S S I S T A N T

VER 1.1

12/10/91

TENSILE STRENGTH

The primary destructive test used to determine the properties of a

metal would probably be the "TENSILE TEST".  This test provides the

greatest a amount of pertinent information about the specimen.

Before a tensile test can be performed, much work goes into the

machining of the specimen.  All surface deformations must be removed, as

this may cause Stress Risers.  Normally, a Tensile Specimen, will be

reduced down in the middle to force the break to take place in a given

location.

A successful tensile test is one that meets one or the other

criteria:

1) Fails in the Base Metal

2) Fails in the Weld Metal as long as the failure is

above the specified strength of the base metal.

The Tensile Test provides information such as the Ductility of a

material.  It can also provide the UTS (Ultimate Tensile Strength), Yield

Strength, Percent Elongation, Percent Reduction of Area, Modulus of

Elasticity, and Elastic Limits.

PERCENT OF ELONGATION

To determine the Percent of Elongation, two precise guage marks

must be applied beforehand to the specimen.  These guage marks are

carefully measured and documented.  After the Tensile Test, the specimen is

placed end to end and another precise measurement is taken.  Therefore you

can compute the Percent of Elongation.  This is an expression of a Metal's

Ductility.

PERCENT REDUCTION OF AREA

To determine the Percent Reduction of Area, the actual surface area

of a specimen is calculated.  Remember, this would be an imaginary plane

perpendicular to the specimen.  Use the appropriate formula to figure the

Original Area, whether you have a round or rectangular specimen.  After

the Tensile Test is completed, carefully calculate the mean surface area of

the ends of the specimen (at the break).   Again the Percent Reduction of

Area, is an expression of a Metal's Ductility.

If you have comments, please forward them to me.  I'd like to hear from

you!

Jeff W. Waldrop

1018 Bee Tree Street

Westlake, LA 70669

(318) 436-7304

GEnie: J.WALDROP1