USER'S MANUAL

******************************************

PARTFULL 1.1 - Gravity Flow in Round Pipes

******************************************

Solves capacity of sloped drains and sewers

that are part full. Computes flow, actual

depth, and actual velocity. Also calculates

critical velocity and critical depth for hy-

draulic jump analysis. User can select either

of two types of hydraulic elements; Manning

based or those based on observed data. Also,

User can choose either of two types of flow

formulas; the Manning or Kutter Formulas. Size

can be changed from 40 to 5 lines to increase

speed. Toggles between English and Metric

system with all input and output data convert-

ed.

The functions are extremely complex and no

simple solution for a PC has been published

before. Accuracy and speed is obtained using

interpolation in lookup tables.

For all kinds of engineers that use round

pipes, hydraulic specialists, utility workers,

road and site designers, and contractors. The

program is written in a familiar spreadsheet

format to decrease learning time. Many spread-

sheet type commands are incorporated.

Will run on any IBM compatible with 512 K and

either one high density floppy; two low densi-

ty floppies; or a hard disk. Includes a con-

figuration program for most types of monitors

and printers. Display colors can be changed as

preferred.

*******************************

Copyright, 1991. All rights reserved.

WHEELER GRAPHICS, 75 Stelfox Street, Demarest, NJ 07627

Voice Mail 201-767-3689 -- FAX 201-767-5620

2

CONTENTS

SUBJECT                             PAGE

Running The Program..................2

Introduction.........................2

Economic Impact......................3

Two Hydraulic Element Methods........4

Hydraulic Jump.......................4

Comparisons with Other Equations.....5

Research Needs.......................5

Notes................................5

General Description..................6

Operating the Program................6

Documentation........................9

Verifying the Program...............10

Registration........................10

What Shareware is...................10

Disclaimer-Agreement................11

Registration........................12

Registration Form...................13

Technical Support...................14

RUNNING THE PROGRAM

All of the files should be copied on to a hard disk or a floppy. To

start type:

RUN   (or)

RUN PARTFULL

The program operates like a spreadsheet. To change pipe size,

slope, n-value, starting flow, or flow increment, simply overwrite

the highlighted value and press enter. For more details, see

OPERATING THE PROGRAM.

INTRODUCTION

One of the main reasons for writing this program is the complexity

of the equations and methods. It is almost too tedious to compute

depth, velocities and the tendency for hydraulic jump in part full

pipes with gravity flow. This difficulty causes too many engineers

to skip past these calculations when designing sewers or checking

existing.

The common practice is to design drains and sewers based on the

capacity and velocities when flowing full. With this approach the

investigator doesn't have to be concerned with part full depths and

velocities, or hydraulic jump. Hydraulic jump cannot occur in a

full pipe. The reasoning has always been that if the pipe and slope

is adequate when flowing full, it will be adequate when part full.

Therefore why bother with hours of complex calculations?

3

Actually, a gravity flow pipe almost never flows full. In most

cases, the pipe must carry whatever flow rate that is put into it

and this rate is constantly changing. This causes hydraulic levels

in the pipe to oscillate up and down.

There are many good reasons why it is necessary to know what

depths, velocities and the tendency for hydraulic jump might be.

For example, it might be desirable to eliminate the possibility of

hydraulic jump. Hydraulic jump causes turbulence, which in turn,

causes the release of sewer gases. In 1991 Kienow reported that

these gases when released can accelerate sewer corrosion. Also,

odors can be released in residential neighborhoods. Jones describes

a pumping station design to avoid wet well turbulence and the res-

ulting odors.

Another reason, is that hydraulic jump can cause unaccounted for

hydraulic losses. The capacity of an existing sewer cannot be

accurately determined under these circumstances.

The advantages of knowing levels and velocities should be obvious.

If the design of a wet well is going to take advantage of unused

sewer volumes to reduce wet well size then there must be confidence

in the calculations. Sediment deposition has always been a design

criteria and minimum sewer velocities should always be calculated

accurately.

ECONOMIC IMPACT

As a rule of thumb, about two-thirds of the cost of a sewerage

system is in the design and construction of sewers. As huge amounts

of money are spent on these systems world-wide, the design should

be as perfect as modern computation methods allow. Computations

methods should not be short cut methods but should be as faultless

as economical methods allow.

Also, a round pipe is probably the most common structure used in

environmental engineering construction. Still, the methods of

computing capacity in some areas may be too difficult for practical

use.

The difference in results can be very large depending on the

computation methods used. For example, consider a case where a

sewer is designed to avoid hydraulic jump at all flow conditions.

If Manning based hydraulic elements are used, then the capacity

will be some 30 percent less than if observation based elements are

employed. A 30 percent difference in capacity would be too much for

many engineers to ignore.

Most are not aware of these and other large differences because of

the difficulty of making comparisons both ways and for all

conditions. Sewer design should be based on full knowledge of all

conditions. A designer should not be required to make assumptions

4

simply because computation methods have not kept pace with

technology.

TWO HYDRAULIC ELEMENT METHODS

The American Society of Civil Engineers in Manual of Practice No.

60 describes two methods of computing the hydraulic elements of a

round pipe. The hydraulic elements are the relation of part full

depth and velocity in relation to full. These two methods use

either a constant or a variable n. The terminology may obscure what

is meant. A variable n means that part full depth and velocity are

based on observations. Also, a fixed n means that the hydraulic

elements are based on values derived from the Manning formula. The

ASCE says that the decision as to what hydraulic elements to be

used "must be left to the engineer." Saatci recently questioned the

use of observed values. On the other hand Escritt espouses that the

use of the "theoretical" values "has been proved a fallacy."

A review of all this information, including Camp's original paper,

suggests that the actual observations cannot be ignored. The use of

the Manning based hydraulic elements may have strong roots in

tradition, but this should not be a basis for sewer design. A

conservative design approach, might be to run calculations both

ways and use the most severe results. Still, economy of con-

struction cannot be disregarded.

The program allows computation both ways but notes that Manning

based hydraulic elements might not be accurate.

HYDRAULIC JUMP

The term hydraulic jump usually means that under conditions of free

flow, the water surface suddenly changes from a low stage to a high

stage. Turbulence results and is the cause of head loss. Hydraulic

jump can occur when the velocity exceeds the critical velocity or

when the depth is less than the critical depth.

There is abundant information available on the occurrence of

hydraulic jump in rectangular channels, but the literature is less

than abundant on circular channels. The program computes the

critical velocity at the actual depth of flow; and the critical

depth at the actual velocity. These are compared by ratios. The

term critical flow, means that flow which produces critical depth

and velocity. If the actual velocity is the same as the critical

velocity then the flow is critical.

The actual depth of flow and velocity is computed by the hydraulic

elements which in turn is computed from the flow formula. Thus the

tendency for hydraulic jump can be influenced by all data input,

including the flow formula, the n value, the slope and the type

hydraulic elements.

5

As both velocity and depth are critical simultaneously, either is

a measure of the tendency for hydraulic jump. The exact point at

which hydraulic jump will start cannot always be determined. If

hydraulic jump is to be avoided then many feel that the critical

velocity and depth must not be within 10 to 20 percent of the

actual.

COMPARISONS WITH OTHER EQUATIONS

There is at least one approximate formula and at least one

nomograph that computes depth directly from the flow and diameter

and does not take into consideration the other variables. These

approximate methods check reasonably well for typical values.

Still, it was found for the same flow and diameter the tendency for

hydraulic jump changes with slope, type of hydraulic elements, head

loss formula, and n value. It is not difficult to get results that

vary 15 percent or more from the true values.

As you run PARTFULL you will see that as input changes, other than

flow and diameter, the tendency for hydraulic jump also changes.

Therefore any Equation or graphic solution that does allow input of

all of the variables will probably not be the most accurate

solution.

RESEARCH NEEDS

Many will contend that more research is needed in the area of

hydraulic elements and hydraulic jump for round pipe. It is

particularly difficult to accept that the hydraulic elements do not

change from a 4-inch to 12-foot diameter pipe. Still, this is what

our current data tells us. This research is especially important as

so much of civilization's infrastructure uses gravity flow round

pipes.

Yet, the best analytical methods available should be used until our

data is updated. It is believed that this program uses the most

current information published. It is also believed that the use of

this program would be faster and more accurate than other methods

proposed.

NOTES

This program produces results of far greater accuracy than is

justified by the hydraulic formulas used. The intent is not to

introduce computational errors by approximations. It should not be

inferred that such accuracy is possible.

Part of this program was developed for a discussion on Saatci's

paper that is expected to be published in the Journal of the ASCE,

Hydraulics Division in 1992.

William Wheeler, P.E. has a bachelor's and master's degree in Civil

Engineering, is a Diplomate in the AAEE and a Fellow in the ASCE.

6

GENERAL DESCRIPTION

RUN.EXE, RUN.OVL, RUN.SET CONFIG.EXE, and PARTFULL.JWK - These are

files in object code for compiled, complete standalone program with

a configuration program. The compiled program does not require the

use of another program and provides filing, display and printing

options. Formulas are hidden and no revisions can be made.

OPERATING THE PROGRAM

RUNNING THE PROGRAM - The program is installed for an EGA monitor.

It will run on CGA, VGA and some monochrome monitors as they are.

If the program does not run, or does not run well, please run the

configuration program. This can be done only after unpacking the

files. To configure type:

C: (A: or B:)>CONFIG <Enter>

The configuration program will give step by step instructions with

menus for selecting the printer, the monitor and will let you

select display colors as you choose.

To run the program type at a DOS prompt:

C: (A: or B:)>RUN <Enter>

The program will prompt the User to select the program displayed.

(Other programs may use the same engines. Also, you may wish to

save customized versions of the program under a different name.

Then, you could call up the version needed for a specific applica-

tion).

OPERATING - As you will see when the program has been loaded, the

program is in spreadsheet format and produces a table of flows,

depths and velocities for a given size and slope of pipe. The

spreadsheet format is probably the most familiar mode among

technical users and should allow intuitive operation for most.

A five line table is the initial display but the table size can be

increased to 40 lines, if desired. Operation is extremely simple

and no menus are needed. Just enter the values you want, overwrit-

ing the (highlighted) values on the screen, and the program will

recalculate the entire table. This can be done using only the

cursor keys and the enter key. Almost no typing is needed.

The program as received will have values already entered. These

values can be overwritten and the program will recalculate after

pressing Enter.

The program calculates depths, velocities and critical velocities

for; both Manning's and Kutter's formula; for both Manning based

and observation based hydraulic elements; and for both the English

and Metric systems of units. The table is prepared from flow input,

7

and the starting flow on the first line of the table. The next

value of flow, and subsequent values are spaced by the increment

selected. Any diameter, n value, and pipe length can be input.

Length is not necessary except to compute fall. Note that there is

a "Round-Plcs" number that can be entered or changed. This allows

any degree of rounding desired but does not always change the

number of digits displayed. Also note that there is a "Marginal

Point" that can be changed. This changes the point at which the

condition changes from "OK-subcr" to "Marginal." When either ratio

is greater than the marginal point entered, then the work

"Marginal" will be displayed.

If either the Flow or Flow Increment values are zero, the program

will use default values to calculate a table. For a 5-line display

the flows will be spaced 20 percent apart. For the 40-line display,

they will be 2.5 percent apart. The defaults are automatically

overridden by entering a value in the blank space. This permits the

User to customize the flows to any degree desired.

The function key tasks are as follows:

F1 - Changes the table from the Observed values of hydraulic

elements to the Manning based values. Pressing F1 again toggles the

display back to Observed values. Generally, the observed values are

preferred and should be used is design but there are some instances

where the Manning based values would be used.

F3 - A toggles that changes the table from Manning Formula

values to Kutter Formula and back again. The Manning Formula has

gained widespread use because of its simplicity. Many believe that

the Kutter Formula might be more accurate.

F5 - Changes the table size from five lines to 40 and back to

five lines again. The five line display will recompute must faster.

F6 - Prints the entire page of the table.

F7 - Leaves the program to a DOS prompt.

F10 - Converts values and units from the English System with

flows in mgd, then cfs, and then to the Metric System. If the F10

key is pressed again, units will return to units at the start. All

of the inputs and the outputs are converted.  This key lets the

user switch back and forth from the Metric to the English System

and back to the Metric System. Mixed units can be entered while in

the second system. For example, if you are working in the Metric

System but want to enter a 14-inch pipe. First, switch to the

English System, then enter only 14 inches keeping the other data.

Now switch back to the Metric and the conversion will be made.

F2 - Allows the editing of a cell if there is text. Where the

cell value is a computed number, the F2 key will display all of the

digits the rounding digits entered will allow. Changing the "Round-

8

Plcs" number will display more or less digits.

F9 - Recalculates the display. This is only useful if the

recalculation method is changed by the "/-command" menu.

Note that by pressing the "/" key a menu is displayed. This is a

"generic spreadsheet" menu than will enable the User to perform

some filing, formatting and other functions in the same mode as a

spreadsheet. For example, if you want the program to wake up in the

Metric system, with Manning based Values, and with certain discrete

values installed, save the spreadsheet this way using the menu

commands and overwrite the existing file or write a new one.

Previously stored files can be recovered by using this menu. No

changes made this way would affect the packed file. Thus, the

original file can always be recovered.

The "/-command" menu can be useful for changing the width of

columns to either display more digits or have more blank space

between columns. The printing options, such as margins or printing

codes can also be changed. The program wakes up set for 10-pitch

printing but this might not suit your needs.

Suppose, as an example, you are studying a particular section of

sewer and you are publishing this information in a report. It is

necessary to save the display so that it can be recalled without

reentering data or changing the formatting. This can be done by

saving this version of the program using the "/", "F" and "S" keys,

in order. You will then be asked to insert the new file name, and

the file will be saved with the data entered.

The command menu works similar to spreadsheet programs and has many

other uses. Please see a typical User's Manual on spreadsheets for

operating details.

If you have several versions of this program stored under different

names, then typing "RUN" alone will allow you to choose the program

desired. The "/-F-R"  command can be used instead.

The compiled program has an advantage in that formulas cannot be

either viewed or changed. This, of course, can be a disadvantage if

you want to change a formula. There are some unprotected cells on

the top of the table that can be overwritten if a project or an

author's name is to be inserted before printing or saving.

By pressing the TAB key several times, the lookup tables will be

displayed. Lookup table values can be overwritten but this is not

advised. Also displayed will be the unit names and conversions

values. These can be changed if desired, but the order of rotation

is fixed. For example, you might prefer "Feet" to "Ft". Changing

the conversion constants is also possible except for the primary

flow value.

The program wakes up with the Global Protection on. This means that

9

cells that only the highlighted can be changed. By disabling the

Global Protection all areas, except those with formulas can be

overwritten. This could be a useful feature if the program is

operated by non-English speaking Users or there are special

terminology requirements.

DOCUMENTATION

METHODOLOGY - Both programs perform interpolations from lookup

tables for computing depth of flow in the pipe, the velocity and

the critical velocity. Unit conversion and lookup interpolations

are made by revising the values in a table by transferring them

from other tables. The formulas are not changed.

Logic is written in equations that prevents any flow greater

than full flow from being used. The hydraulic elements show

that flow can sometimes be greater than full flow at between

80 percent and 100 percent of depth. This is considered as an

unstable condition and has been eliminated for that reason.

This upper limit that changes pipe depth to full can be

revised, if desired.

Hydraulic elements are from the American Society of Civil Engineers

Manual of Practice No. 60, "Gravity Sanitary Sewer Design and

Construction, 1982, Figure 5-21. From this Figure, Manning based

hydraulic elements are for n and f constant. Observed hydraulic

elements are from n and f variable with depth. Equations for this

data are best solved with lookup tables. Points in the tables are

spaced 1 percent apart so that straight line interpolation can be

very accurate. (Accuracy is much greater than is justified by the

data).

The tendency for hydraulic jump is assumed to be a function of the

critical velocity and depth. As the actual velocity and depth

approach the critical velocity and depth, the possible occurrence

of hydraulic jump increases. Critical velocity and depth is

calculated using long accepted critical velocity equations, again

using lookup.

HARDWARE AND SOFTWARE PROBLEMS - The size of the program has been

optimized for IBM compatible 16-bit computers running at 12 to 16

Hz and with a coprocessor. Some older computers may run the program

slowly. It should take about 4 to 8 seconds to recompute the entire

40-line table with a 16-bit machine and a coprocessor. The

five-line table should be recalculated in a blink. If your machine

is much slower, or you don't have a coprocessor, you may want to

operate only in the five-line mode. This would probably give you

adequate information.

The program should run on any IBM compatible computer with 512K

memory. A coprocessor is not necessary but is recommended. The

program will run on machines with only two 360 K floppies and no

hard disk. You may need another computer to rearrange the files so

10

they will fit in the right order on each floppy. The files RUN.OVL

and CONFIG.EXE should be installed on the second floppy and all

others on the first. It would also be helpful if the path was set:

A:>PATH A:,B: <Enter>.

VERIFYING THE PROGRAM

The methods for solving both hydraulic elements and hydraulic jump

criteria, are extremely complex. Many equations, graphs and a

definition sketch are necessary for full understanding. Unfortu-

nately, the limitations of an ASCII file does not allow this

information to be presented here. Also, much of this information

had to be specially developed for this program and is not available

from other references.

As a result, the computational results of the program will be

difficult to verify. To assist in this verification and to permit

full understanding of the program, the following information is

available:

- Equations used

- The references for these equations, or how they were derived

- Other approximate methods used for independent cross

checking

- Specially developed tables of hydraulic elements, critical

depth and velocity

It is strongly recommended that before the program is used on real

world problems, the equations and computational results be

verified. This is not only needed to ensure that the results are

accurate but also to confirm that the User knows what is meant by

the terminology and expressions used.

Detailed information will be sent to assist in verification when

the program is registered.

REGISTRATION

PARTFULL is shareware. No sales commissions or fees are paid. No

demo disks are sent because the full program is available on a

trial basis.

For information on customized versions or site licenses please

call or FAX.

WHAT SHAREWARE IS

Shareware distribution gives users a chance to try software

before buying it. If you try a Shareware program and continue

using it, you are expected to register.

11

Shareware is a distribution method, not a type of software. The

Shareware system makes fitting your needs easier, because you can

try before you buy. And because the overhead is low, prices are

low also. Shareware has the ultimate money-back guarantee -- if

you don't use the product, you don't pay for it.

DISCLAIMER - AGREEMENT

Users of PARTFULL must accept this disclaimer of warranty:

"PARTFULL is supplied as is.  The author disclaims all warran-

ties, expressed or implied, including, without limitation, the

warranties of merchantability and of fitness for any purpose. The

author assumes no liability for damages, direct or consequential,

which may result from the use of PARTFULL."

PARTFULL is a "shareware program" and is provided at no charge

to the user for evaluation.  Feel free to share it with your

friends, but please do not give it away altered or as part of

another system.  The essence of "user-supported" software is to

provide personal computer users with quality software without

high prices, and yet to provide incentive for programmers to

continue to develop new products.  If you find this program

useful and find that you are using PARTFULL and continue to use

PARTFULL after a 30 DAY trial period, you must make a registra-

tion payment of $15. to Wheeler Graphics.  The $15. registration

fee will license one copy for use on any one computer at any one

time.  You must treat this software just like a book.  An example

is that this software may be used by any number of people and may

be freely moved from one computer location to another, so long as

there is no possibility of it being used at one location while

it's being used at another. Just as a book cannot be read by two

different persons at the same time.

Users of PARTFULL must register and pay for their copies of

PARTFULL within 30 days of first use or their license is with-

drawn. Anyone distributing PARTFULL for any kind of remuneration

must first contact Wheeler Graphics for authorization.

Teachers may distribute PARTFULL but they must not charge for the

program. The verification data given when the program is regis-

tered cannot be copied and can only be distributed by Wheeler

Graphics.

This authorization will be automatically granted to distributors

recognized by the (ASP) as adhering to its guidelines for

shareware distributors, and such distributors may begin offering

PARTFULL immediately (However Wheeler Graphics must still be

advised so that the distributor can be kept up-to-date with the

latest version of PARTFULL).

You are encouraged to pass a copy of PARTFULL along to your

friends for evaluation.  A User's Manual is on disk that can also

be distributed. Please encourage them to register their copy if

12

they find that they can use it.

REGISTRATION

All registered users will receive the following:

- A printed description of how the program operates and

the methods used for making the calculations.

- Equations used in the program for hydraulic elements

and hydraulic jump. Also the source of these equations

or how they were derived.

- Graphs and figures essential for the verification and

understanding of the program.

- Selected references

- Tables for the quick solution of hydraulic elements,

critical velocity and depth for those instances where a

rapid verification of conditions is necessary, or a

computer is not available. Instructions for use of the

tables with an example is included.

The registration fee does not include a floppy disk with the

latest version of the program. If the most recent version is

desired there is an additional copying and mailing charge of $15.

If both an additional program disk and a registration is desired,

the fee is $30. New Jersey residents please add sales tax. Please

send to:

Wheeler Graphics

75 Stelfox Street, Demarest, NJ 07627

Voice Mail . . . . . 201-767-3689

FAX  . . . . . . . . 201-767-5620

Development of new software related to PARTFULL and in the

environmental field will continue. Your financial support will be

greatly appreciated. It will put more resources to work and will

make available more new software sooner.

REGISTRATION FORM

For your convenience a Registration Form/Invoice that can be

printed or copied, is on the next page:

13

*****************************************************************

REGISTRATION FORM AND INVOICE

*****************************************************************

Date:___________________

Mail to:  Wheeler Graphics

75 Stelfox Street

Demarest, NJ 07627

PO # (If Needed)___________________

From:____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

_____Enclosed is a check for $15. Please register the program and

send me all of the verification information including equations,

sources, derivations, graphs, figure, tables, instructions and

references.

_____Enclosed is a check for $30. Please register the program and

send me all of the verification information as described above.

Also send me a copy of the latest version of the program. Please

send copy on a ________5-inch, or a________3.5-inch size disk.

New Jersey residents please include sales tax. Only checks drawn

on a US bank in US funds can be honored. Cost of transfer of

other funds is prohibitively expensive.

Comments are welcome. Are there any changes or additions you

would like to see incorporated?

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

14

TECHNICAL SUPPORT

Free technical support is offered at the above address and

telephone numbers. Support is offered to all Users whether

registered or unregistered.

PRINTING THIS MANUAL - This manual has been written an ASCII

format and should be read by almost any word processor. Using a

word processor would give the User control over printing and

allows formatting. This file also can be printed (without control

and formatting) by using the DOS commands:

A:>PRINT USER-MAN.DOC <Enter>