191

CHAPTER 19 - STRINGS

Sometimes we want to deal with long strings of information. Here

long means hundreds or thousands of bytes, not tens of bytes. The

8086 provides a group of instructions to move and compare

strings. These instructions have a rigid structure, but with a

little bit of effort we can get them to work easily for us. We

will start with SCAS, since it is simple, yet embodies all the

rigid features of these instructions.

SCAS (scan string) compares either a byte to AL or a word to AX.

The byte or word must be in memory, and the register must be AL

or AX. SCAS also increments or decrements the pointer. First, the

size:

scasb

compares a byte to AL, while:

scasw

compares a word to AX. But where's the pointer? You have no

choice, it's DI. Not only is it DI, but it MUST be ES:DI. The ES

segment is coded into the 8086 microcode; the DI register is

coded into the 8086 microcode; there is nothing you can do to

change it. What about incrementing or decrementing? In the flags

register, there is a flag called the direction flag. It is set

manually by the program. If DF = 0, SCAS increments DI; if DF =

1, SCAS decrements DI.{1} The equivalent software for the

instruction would be:

(scasb)                       (scasw)

DF = 0        cmp  al, es:[di]              cmp  ax, es:[di]

pushf                         pushf

add  di, 1                    add  di, 2

popf {2}                      popf

DF = 1        cmp  al, es:[di]              cmp  ax, es:[di]

pushf                         pushf

sub  di, 1                    sub  di, 2

popf                          popf

____________________

1. Every time you have called show_regs DF has been there; it

doesn't show 0 and 1, it shows + and -  (+ = 0 , - = 1).

2. The microcode doesn't really push and pop the flags. This

is only to indicate that the order of operations is (1) get the

byte (word) from the string, (2) compare and set the flags, and

finally (3) increment (decrement) the pointer without changing

any of the flags.

______________________

The PC Assembler Tutor - Copyright (C) 1989 Chuck Nelson

The PC Assembler Tutor                                        192

______________________

Thus, at the end of the end of the instruction, DI is in a new

place and the flags are set according to the compare result. DI

is incremented by a byte for the byte instructions; it is

incremented by a word for the word instructions. The same pattern

holds true for decrementing DI.

We set the direction flag with the instruction STD (set direction

flag) and we clear the direction flag by using CLD (clear

direction flag). It only needs to be set or cleared once, and

this should be done before starting the operation. DF is only

changed by those specific instructions from the program - it

can't be changed by any arithmetical or logical operation on the

chip.

If you have a string and you are looking for a specific number,

(27 for instance), you simply put that number in AL (or AX) and

run a loop. If:

long_string  db 5000 dup (?)

contains data and we want to look for a 27d then the operation

is:

lea  di, long_string{3}

mov  al, 27

cld

search_loop:

scasb

jne  search_loop

on exiting, DI will point 1 PAST the matching byte (word). You

move back one byte (word) to find the match. Why would anyone

want this instruction? With a 0 in AL, it will find the end of a

C (0d terminated) string quickly. Also, that number 27 is no

accident. 27d is the ASCII escape character. For a lot of

hardware, 27d indicates that the bytes that follow are not ASCII

characters but are technical information. For instance, on my

printer the sequence (27d, 65d, 0d, 11d) sets tabs every 11

columns. SCAS can find where these substrings are so the program

can operate on them.

In order to use string instructions, we need strings to work on.

The one we will use is called CH1STR.OBJ. It is in \XTRAFILE.  It

is an object file that contains one data segment containing a

string of lower case characters. The string is several thousand

bytes long, it is terminated by 0, and it contains ONLY the lower

case letters (a-z). It is the first draft of part of chapter 0

with all punctuation, numbers, spaces, carriage returns etc.

deleted. All upper case letters have been converted to lower case

so we don't have to worry about the difference between A and a, Q

and q.

The name of the array in CH1STR.OBJ is CH1STR and it is defined:

____________________

3. Assuming that long_string's segment address is in ES.

Chapter 19 - Strings                                          193

____________________

PUBLIC  ch1str

so that you can access it with the SEG and OFFSET operators.

First, let's find out how long the string is.{4}

MYPROG1.ASM

; - - - - - - - - - -

STRINGSTUFF  SEGMENT  PUBLIC 'DATA'

EXTRN  ch1str:BYTE

STRINGSTUFF  ENDS

; - - - - - - - - - -

;- - - - - - - - - - PUT CODE BELOW THIS LINE

mov  ax, seg ch1str      ; segment address of ch1str

mov  es, ax

mov  di, offset ch1str   ; offset address of ch1str

mov  al, 0               ; try to match zero

cld                      ; increment (DF = 0)

string_end_loop:

scasb

jne  string_end_loop

dec  di                  ; back up one

mov  ax, di

sub  ax, offset ch1str

call print_unsigned

;- - - - - - - - - - PUT CODE ABOVE THIS LINE

In all these string operations, we need to be careful about

boundary conditions. What if there is no valid data? What if

there is one valid item? What if the string is empty?

On exiting the loop, DI will point 1 past the first 0d, so we

need to back up one to point to the first 0d. Then subtracting

the starting position will give us the count.{5} Try it out and

find out how long it is. Since we now have 3 object modules, the

link instruction must read:

link  myprog+ch1str+asmhelp ;

assuming that you name your program myprog.asm. Save the result

because we will need to use this number several times.

____________________

4. Just to keep it from being too easy, I have put garbage

both in front of the string and behind the string. That means

that the string length is shorter than the length of the object

file and the string does not start with the first byte of the

object file.

5. If the first byte in the string is 0d, we move one, then

move back one which gives the length zero.

The PC Assembler Tutor                                        194

______________________

You will notice that we have gotten the segment address by using

the SEG operator. You don't need to know the name of the segment.

The segment doesn't even have to be PUBLIC. As long as the

VARIABLE is either in the same file or is in another file and

PUBLIC, the linker will find the correct segment address and put

it there.

To make things a little more complicated, we will make another

infinite loop. This time you will enter a character, and the

program will find the first occurance of that character. We need

to add some error checking here. Since you will probably be

dreaming about taking your next vacation in Hawaii while you are

entering the data, a few characters that don't exist in the

string (things like G $ ? ~ ) might creep in. It would be

possible to run way past the end of the string before you found

that character. We'll put the length of the string (from the last

program) in CX, have a regular loop so we can't go too far, and

jump out of the loop if we find a match.

MYPROG2.ASM

; - - - - - - - - - -

STRINGSTUFF  SEGMENT  PUBLIC 'DATA'

EXTRN  ch1str:BYTE

STRINGSTUFF  ENDS

; - - - - - - - - - -

;- - - - - - - - - - PUT CODE BELOW THIS LINE

mov  ax, seg ch1str

mov  es, ax

outer_loop:

call get_ascii_byte           ; returns character in al

mov  cx, $$$$$$$              ; enter string length here

mov  di, offset ch1str

cld                           ; increment (DF = 0)

string_end_loop:

scasb

je   after_loop               ; if equal, we found the char

loop string_end_loop

mov  ax, 0                    ; we fell through the loop

call print_unsigned

jmp  outer_loop

after_loop:

mov  ax, di                   ; move for printing

sub  ax, offset ch1str        ; number of bytes

call print_unsigned

jmp  outer_loop

;- - - - - - - - - - PUT CODE ABOVE THIS LINE

Chapter 19 - Strings                                          195

____________________

Those dollar signs are the place to enter the exact length of the

string that you got from the last program. This time we jump out

of the loop if we find a match; DI will be 1 past the matching

character, but this will give us the right count (if we find the

character in the first space, we increment once). If we can't

find a match we fall through the loop and print a 0. Remember to

link all 3 modules when you run the program. Run the program and

then we'll move forward.

This type of thing is so common with string operations that there

is a special prefix for SCAS and all other string operations

which makes the coding simpler. It has several forms:

rep       decrement cx ; repeat if cx is not zero

repe      decrement cx ; repeat if cx not zero and zf = 1

repz      decrement cx ; repeat if cx not zero and zf = 1

repne     decrement cx ; repeat if cx not zero and zf = 0

repnz     decrement cx ; repeat if cx not zero and zf = 0

REP is for the move instructions which we will see later - it

won't work here. For each prefix, if either (or both) of the

conditions is not true, the repitition stops. For instance, with

REPE, if cx is zero, and/or if the comparison was not equal (so

the zero flag was not set), the instruction will stop. For our

program, the coding is:

repne scasb

That's it. That replaces the whole inner loop. Here is our new

coding of the last program.

MYPROG3.ASM

; - - - - - - - - - -

STRINGSTUFF  SEGMENT  PUBLIC 'DATA'

EXTRN  ch1str:BYTE

STRINGSTUFF  ENDS

; - - - - - - - - - -

;- - - - - - - - - - PUT CODE BELOW THIS LINE

mov  ax, STRINGSTUFF

mov  es, ax

cld                           ; increment (DF = 0)

outer_loop:

call get_ascii_byte           ; returns character in al

mov  cx, $$$$$$$              ; enter string length here

lea  di, ch1str               ; address of string

repne  scasb

je   found_the_char           ; an equal comparison

mov  ax, 0                    ; we didn't find a match

call print_unsigned

jmp  outer_loop

found_the_char:

mov  ax, di                   ; move for printing

The PC Assembler Tutor                                        196

______________________

sub  ax, offset ch1str        ; number of bytes

call print_unsigned

jmp  outer_loop

;- - - - - - - - - - PUT CODE ABOVE THIS LINE

There are two possibilities for exiting the 'repne scasb'

instruction. Either we found an equal comparison or we exhausted

all the characters in ch1str. If we found an equal comparison, JE

will send us to the print routine. Otherwise we print a 0 because

we finished the loop without finding anything.

STOS

We can ask the operating system to allocate memory for us while

the program is running.{6} When you get it, however, it will

contain trash. The fast way to clear it is to use STOS (store to

string). The instruction is:

stosb

or:

stosw

The equivalent action (not counting changing the value of DI) is:

mov  es:[di], ax    ; or AL for byte moves

Once again (1) the pointer is the ES:DI pair, which is mandatory,

and (2) DI is incremented or decremented (by 1 for byte, by 2 for

word) depending on the status of DF, the direction flag. The

instruction moves a byte (a word) from the AL (AX) register to

the memory address pointed to by ES:DI. We can use the REP{7}

instruction to speed things up a bit. If we have a 11,872 word

block of memory, we can clear it with the following instructions:

; - - - - - - -

DATASTUFF  SEGMENT

my_bufferdw  11872 dup (?)

DATASTUFF  ENDS

; - - - - - - -

mov  ax, seg my_buffer

mov  es, ax

cld                           ; increment (DF = 0)

mov  ax, 0               ; clear the buffer with 0s

mov  di, offset my_buffer

mov  cx, 11872

rep  stosw

____________________

6. Cf. You-know-who's Programmer's Guide to You-know-what or

"DOS Programmer's Reference."

7. There is no comparison here, so REPE or REPNE doesn't make

any sense.

Chapter 19 - Strings                                          197

____________________

That's as fast as it gets. Why does the STOS instruction use AX?

Because that's the register that port i/o uses. If you are

writing a communications program, you need speed. You can have

the following:

; - - - - - - - - -

DATASTUFF  SEGMENT

port_address   dw   0F2A8h        ; this address is legal but

; there's nothing there.

input_buffer  db   4000h     dup (?)

output_buffer  db   4000h     dup (?)

DATASTUFF  ENDS

; - - - - - - - - -

mov  ax, DATASTUFF

mov  es, ax

cld                           ; increment (DF = 0)

mov  di, offset input_buffer

mov  dx, port_address

input_loop:

in   al, dx

stosb

jmp  input_loop

; - - - - - - - - -

A real program would be much more complicated because we would

have to check to see if data was ready to come in and we might

need to check the data for errors. Also we would occasionally

have to clear the buffer. The port address F2A8h is just an

arbitrary address. It's a legal address but there's nothing

there.

We should write a program, so let's input a character and have it

fill the screen. We'll leave the last line of the screen alone so

you can see your input. Move your cursor to the last line before

beginning the program.

; - - - - - ENTER CODE BELOW THIS LINE

mov  ax, 0B800h          ; or 0B000h for a monochrome card

mov  es, ax

cld                      ; increment (DF = 0)

outer_loop:

call get_ascii_byte      ; AL = fill char from input

mov  ah, 07h             ; black background, white letters

sub  di, di              ; set di to zero

mov  cx, 1920            ; 24 lines X 80 chars

rep  stosw

jmp  outer_loop

; - - - - - ENTER CODE ABOVE THIS LINE

If you have a monochrome card, the segment address is 0B000h. If

The PC Assembler Tutor                                        198

______________________

you have a color card and are in text mode, the segment address

should be 0B800h. This fills the first 24 lines with the input

character. The STOS instruction has no effect on the cursor.

LODS

The opposite of STOS is LODS (load string) It moves a byte (word)

from the string to the AL (AX) register. This time, for a change,

we use the SI register as a pointer, and the default register is

DS.{8} As always, SI is incremented or decremented by a byte

(word) depending on the setting of DF, the direction flag. The

two possibilities are:

lodsb

and

lodsw

The equivalent action (not counting changing the value of SI) is:

mov  ax, [si]  ; or AL for byte moves

This is an instruction for people that write device drivers. You

could use it if you are sending a string of characters to the

printer, but that's about it. Code for doing that would have the

following form:

; - - - - - - -

buffer   db   1000 dup (?)

; - - - - - - -

lea  si, buffer   ; the buffer must be in the ds segment

cld                      ; increment

out_loop:

lodsb

and  al, al              ; if 0, end of string

jz   quit_loop

mov  dl, al              ; move character to dl {9}

mov  ah, 5               ; int 21h function 5

int  21h                 ; print a character

jmp  out_loop

quit_loop:

; continue with the program

If you actually run this program, many printers will not print

anything until they get an end of line signal ( 10d, 13d).

____________________

8. Register DS can be overriden. We'll talk about that in the

second part of this chapter.

9. Int 21h (AH = 5) prints one character from DL to the

printer. Why it's DL and not AL is a mystery.