This program is adapted from

DTR and AT

by

Donavon Kuhn  &  Jon Niedfeldt

(c) 1985

Adaptation for Clipper

by

John Karns

Clipper/Modem Interface

CLIPDIAL.ASM

v1.0  10/26/88

In general terms:

Call DIALIT with "Hayes command"

where the hayes command is a character string

without the AT on the front.  (Since all commands

start with an AT, the module will supply it).

COM1 is the default unless you specify otherwise.

Specifically:

Call DIALIT with "Com2:DT1234567"  && Can use COM1 thru 4

Call DIALIT with "DT1234567"       && Dial # 1234567 (default COM1)

Call DIALIT with "H"               && Hang up

(Upper or lower case may be used.)

=========================================================================

Example:

m_PHONE = SPACE(10)

CLEAR

@ 10, 25 SAY "Enter Phone No. " GET m_phone PICTURE "@R (999) 999-9999"

READ

prefix = "DT"

cmd = prefix + m_phone

@ 12, 25 SAY "Dialing - Please wait..."

Errcode = DIALIT(cmd)

prompt1 = "Return Code = " + ltrim(str(errcode))

@ 14, 25 SAY prompt1

@ 23, 25 SAY "Press any key to hang up..."

inkey(0)

cmd = "H"

Errcode = DIALIT(cmd)   && Must use this form to maintain consis-

&&   tency with above function call...

*          CALL DIALIT with cmd  && This will cause Clipper to generate a

&&   compile-time error (see note below).

CLEAR

@ 12, 35 SAY "All Done..."

return

This module may be called as a procedure or as a function.

If called as a function, an error code will be returned.

For example:

Errcode = DIALIT("DT1234567")

where:

Errcode = 0 - No errors detected

1 - Specified Com port > 4

2 - Com port not found

���������������������������������������������������������������������������Ŀ

�  NOTE: If you call it as BOTH a function and as a procedure, Clipper      �

�        will give the compile-time error "Symbol Redefinition Error",      �

�        so you must be consistent - i.e., use one form or the other!       �

�����������������������������������������������������������������������������

This is a straight-forward adaptation of the above mentioned set of

programs.  A modem which uses the standard hayes command set is

required.

The two sets of modules, DIALIT86.* and DIALIT87.* are for Clipper

versions Autumn '86 and Summer '87 respectively.

The intended purpose of the module is to provide a convenient

interface between a Clipper application and a dialing device.  It

should be fully possible to send any of the full set of "AT"

commands to a modem via DIALIT.  The full command string would be

Call DIALIT with "COMx:command"

where:  1 <= x <= 4

and 'command' is any modem command. The AT prefix IS SUPPLIED by

DIALIT, so don't use it!  Just start with the third character in the

string.  So instead of ATDT---------  use  DT-----------, get it?

The COM spec. is optional, defaulting to COM1.

Before attempting this adaptation, I searched many of the popular

Clipper/dBase BBS's for such a utility.  I did find one in a shareware

library, but then I came upon the two programs, AT.COM and DTR.COM

which were written to function from DOS, and were contributed freely

by the authors for others to use for their own purposes.  I likewise am

happy to make a small contribution in that same spirit of sharing

with the user/programmer community.

This is my first (successful) attempt at doing anything with

808x/80x86 assembler, particularly relating to Clipper.  I think I

learned a few basics in working in both domains, and hope to do some

more in the future.  It is great fun!  But I make no claims toward being

an expert in assembler.  You are free to make your own adjustments as

you see fit.

When called from Clipper, DIALIT will set DTR before dialing.

For the uninitiated, DTR is a mnemonic for Data

Terminal Ready.  It must be set prior to sending the

dial string in order for the modem to dial out.

"Dropping DTR" will effectively hang up the modem, and

can override a frozen connection - i.e., it FORCES the

modem to go on-hook (Hang up).

I thought it best to leave DTR set in order not to disrupt any other

pending process.

DISCLAIMER

These programs are provided as is.  Whomsoever will use these will

assume all responsibility in determining that these programs modules

are suitable for any intended use.  Clipper is a trademark of

Nantucket Corporation.