POST 2.2

Introduction:  This document describes the MS-DOS utility POST.

POST is a utility that gives MS-DOS based PC's the ability to

print directly to PostScript printers from the DOS prompt.  This

information applies to version 2.2 of the POST utility.

Background:  The first PostScript based printer was introduced by

Apple Computer.  This printer helped establish the desktop

publishing industry because it incorporated PostScript, the

sophisticated Page Description Language (PDL), developed by

Adobe.  The printer was designed to work with the Apple Macintosh

using an Apple developed interface called AppleTalk.  The printer

designers did however include an RS-232 port so that it could be

connected to other computers such as the IBM-PC and clones.

Software developers soon discovered the power of PostScript and

began to write printer drivers for PC application programs.  One

serious drawback however, is the lack of a PostScript driver for

MS-DOS.  The POST utility is an attempt to solve this oversight.

Purpose:  POST allows users of PC's to print ASCII files directly

to PostScript printers.  It is intended to provide PostScript

support for programs that do not have the necessary drivers, and

to allow direct printing of DOS files.  POST allows directory

listings to be piped through it directly to a PostScript printer.

POST also allows the user to print files that have been created

by spreadsheet and database programs that can print to ASCII

files.

The user can specify a variety of parameters that will affect the

appearance of the output.  These parameters include the top, bottom,

left margins, and the font style, size and leading.  Options for

scaling, rotation and translation are also provided.  Version 2.0

also allows PostScript commands to be embedded in the source file.

This can be used to include box drawing, shading, font changes,

logos and other PostScript functions on the printed page.

(This feature requires some knowledge of the PostScript language.)

The ability to include PostScript commands is very powerful and can

be used to produce almost anything.

Function:  POST Reads data from the standard input device,

converts it to PostScript format, then writes the data to

the standard output device.

1

POST 2.2

Command Line Format:  (options are explained on next pages)

[d:][path]POST[option 1 value(s) ... option n value(s)]

Upper or lower case is OK except for Font names (this will be

explained later).  There are 72 points per inch.  The default

page origin is located at the lower left corner with positive X

to the right and positive Y upwards.

The output filename must be different from the input filename.

POST will space to the next column position if it finds a tab

character.  Column positions can be defined as preset distances

from the left margin.  POST will skip to top of form if it finds

a form feed character. POST will ignore control characters other

than line feed, form feed and tab.

OPTIONS:         LETTER:    VALUE(S):                          DEFAULTS:

Left Margin         L       Left margin in points              L 54

Top Print Line      T       Top print position in points       T 720

Bottom Print        B       Bottom print limit in points       B 54

Column Tab          C       Column tab spacing in points       C 56

Typeface Size       P       Typeface size in points            P 12

Leading             D       Vertical line spacing in points    D 12

Typeface Font       F       Selected font for printing         F Courier

Scaling             S       X and Y scaling multipliers        S 1.0 1.0

Page Origin         X       X and Y page translation points    X 0 0

Page Orientation    R       Page rotation angle in degrees     R 0

Paper Tray Number   N       Valid Paper Tray number            N 1

Details of Options:

L:   The left margin is the value assigned in points (72 per inch) to

the leftmost printing position on the page.  For instance, if you

want to start printing one inch from the left side of the page you

would include the option - L 72.

T:   The top margin is used by the printer every time a new page is

started. Its value is the number of points (72 per inch) from the

bottom of the page to the beginning character position of the first

line on the page.  Remember that if you decide to print in the

landscape mode, you will have a smaller number of points to the

first line.  POST uses 720 points as the first line default (10

inches times 72 points per inch).

2

POST 2.2

B:   The bottom margin is used by the printer to decide when to start a

new page.  Every time a new line is started the amount of

line-to-line space defined (see D - leading) is subtracted from the

current position on the page to see if enough room is available.

If not, the current page is printed and printing starts at the top

of a new page.  If you are using embedded form feed control, be

sure to set the bottom margin to a value that will keep from

automatically paging before you want to.  Also, most PostScript

printers cannot print closer than one fourth to one half inch of

the edge of the paper.

C:   The column tab spacing is the increment in points to be added to

the left margin for each tab column across the page.  The default

tab column for the first tab is 54 points for the left margin plus

56 points.  The next tab column is 56 points more than the first.

P:   One advantage of PostScript printers is that they can print

characters in any size desired because they are defined in a manner

that allows them to be scaled very accurately.  POST allows any

integer value to be used.  The default size is 12 points which is

10 characters per inch.  Do not forget that the line-to-line

spacing must be adjusted in conjunction with the typeface size or

overprinting can result.  Normally the line-to-line (leading -

option D) spacing should be equal to or greater than the typeface

size.

D:   Leading is the line-to-line spacing value.  The name is derived

from when type was made from the metal of the same name - lead.

The lead was poured to make rows of letters and the space between

rows was the amount of leading.  Be sure to increase the leading

when you increase the type size to avoid overprinting.  The leading

value is also used to determine the number of lines per page by

subtracting its value from the calculated line position each time a

new line is started.

F:   The default font is Courier.  Any other font that is already in the

PostScript printer can be used.  Be sure to use upper and lower

case as described by the printer manual.  Examples for Courier are:

Courier, Courier-Bold, Courier-Oblique and Courier-BoldOblique.  If

you have downloaded a special font, you can select it.  The

PostScript printer will ignore requests for incorrectly identified

fonts.  Please remember that most of the fonts other than Courier

are proportional and that column data will not be lined up.

Examples other than Courier are not shown because most of the fonts

are copyrighted.

3

POST 2.2

S:   Scaling is done by the PostScript printer.  It multiplies each X

and Y coordinate by the X and Y scaling factors that you have

provided.  The printout can be enlarged or reduced.  The X and Y

scaling factors are the only ones that POST allows to be

non-integers.  POST sends translation, orientation and finally

scaling values sequentially to the printer.

X:   Page Origin:  This will allow you to translate the page origin to

any place on the page.  The values specified must be in points.

Remember that if you translate the origin, X and Y positive values

are still up and to the right.  If you translate the X coordinate

of the origin to the top of the page, nothing will print because it

will be off the page!  Also, any characters that extend beyond the

right edge of the page will also not print.  Generally if you

translate the page origin you also need change the orientation of

the page with the Rotate command (option R).

R:   Rotation of the coordinate system can be very confusing.  The

PostScript printer rotates the page image counter-clockwise by the

number of degrees requested.  POST expects the number to be an

integer - i.e. one degree increments.  POST sends the rotation

value after it sends the translation values but before scaling.

See the example of landscape page orientation for an example

and experiment using variations.

N:   Some of the newer PostScript printers have multiple paper trays.

This option allows you to select another papertray.  The printer

will abort if you select a papertray it doesn't know about.

Examples: The following performs a standard directory function, pipes

the output as a standard input to POST and POST converts the

directory listing to PostScript format and sends it to the

PostScript printer:

C>DIR | POST > PRN

The following sends the file infile.ext to POST, POST converts

it to PostScript format and sends it to the file outfile.ext:

C>POST < infile.ext > outfile.ext

4

POST 2.2

The following prints a page in the landscape mode by setting

the top of page at 540 points, translating the origin to the

upper left corner and rotating the page counter-clockwise 270

degrees:

C>POST T 540 X 0 792 R 270 < infile.ext > PRN

The following prints very full pages by setting the left

margin to 24 points, top of page to 756 points and the bottom

of page to 24 points:

C>POST l 24 t 756 b 24 < infile.ext > PRN

Hint:     If there are specific functions that you are going to use

often, create batch files with the required options.  See your

DOS manual for more information about creating batch files.

Sample batch files are included with this documentation.

5

POST 2.2

Computer Connections:

The purpose of this section is to help you establish the correct

connection and communication method between your computer and PostScript

printer.

POST was developed for people who have a PC (or clone) with a PostScript

printer connected to the parallel port of their PC.  That is why the

examples are using "PRN" as the output device.  If you have this

configuration, you should have no trouble using the POST utility.

Making the PostScript printer work with a serial connection is slightly

more difficult.  The printer and the PC must be set to communicate at

the same speed.  9600 bits-per-second must be used on the LaserWriter.

To set up DOS to communicate correctly, enter the following incantation

at the DOS prompt.  Substitute the correct COM port if yours is

different.

MODE COM1:9600,N,8,1,P

This tells DOS to talk to serial device COM1 at 9600 bits-per-second,

with no parity, eight data bits, one stop bit and to continuously retry

to send upon receipt of time out errors from the asynchronous adapter.

You may also want to enter the following:

MODE LPT1:=COM1:

This tells DOS to re-route information intended for the parallel port to

the specified serial port.  You may not need this.

Next set your printer to use the serial port at 9600 bits per second

(see your printer manual).

There is one problem that frequently occurs when a PostScript printer is

attached to a PC via the serial ports on both devices.  The problem

occurs because the default handshaking (or synchronization) method used

by the devices are different.  By default, the Laserwriter and other

PostScript printers use XON/XOFF (software) handshaking.  The PC, on the

other hand, uses DSR/DTR (hardware) handshaking.  The problem is

exacerbated by the fact that many application programs for the PC

control the serial port directly and implement the software handshake

protocol.  This means that your word processor could produce perfect

documents, but the POST utility would not work from the DOS prompt.

6

POST 2.2

In order to solve this problem, a couple of options are available.

The first is to change the default handshaking method of the printer,

and the second is to change the default handshaking method of the PC.

To change the default handshaking method of the printer, you send a

small PostScript program to it.  On some printers (such as the NT and

the NTX) you can also change it via the DIP switches.  Early versions of

the Laserwriter will not allow you to change the default handshaking

method.  If you change the default handshaking method of the printer, a

correctly wired cable is essential.  The PostScript program is shown

below.  Copy it to the printer using the DOS COPY command.

0000 % Server Password (zeros, not oh's)

statusdict begin 25 sccbatch 4 ne exch 9600 ne or {serverdict begin

exitserver}{pop end stop}ifelse statusdict  begin

25  9600  4  setsccbatch

end

Create the file listed above using EDLIN or a wordprocessor that can

create a simple ASCII file.  Embed a control D as the last character to

tell the printer that this is the end of the data.  (Most MS-DOS based

word processors and text editors support the default method of holding

down the ALT key and entering the decimal value of the required control

character on the keypad.  To enter a control D hold down the ALT key and

enter 004 on the keypad.  To enter a control character with EDLIN, type

Ctrl-V, then press the Capital D.)  This program makes a semi-permanent

change to the printer.  That is - it will stay in DSR/DTR protocol until

you change it again.  To switch back to XON/XOFF change the two "4"s to

"0"s (zeros, not oh's).

To change the default handshaking method of the PC, you may need to use

a copy of the program XONXOF.COM included in the ARC file.  This program

is a TSR which means it does not go away until you turn off the

computer.  Some PC clones allow you to specify the handshaking mode of

the serial ports.  If yours does you should have no trouble once you set

it for XON/XOFF.

NOTE:

If you have been using POST.COM (a previous version of this utility)

you will need to remove it from the directory when you install POST.EXE.

Otherwise, DOS will execute the old version.  DOS always executes the

file with the .COM extension if it exits even if you type in the .EXE

extension.

7

POST 2.2

Embedded PostScript:

Version 2.2 has incorporated the ability to send PostScript commands

directly to the printer without intervention.  This capability can be

used to include a company logo on the page, or to draw boxes on the

page to highlight columns or rows.  Almost any valid PostScript commands

can be sent using this feature (some knowledge of the PostScript

language is required). To embed PostScript commands in your file you

should enter a CTRL-^ (hold down the CTRL key on the keyboard and

press the ^ key).  On most PCs, the ^ is above the 6 and CTRL-6 will

sometimes work.  Next enter a <RETURN> followed by the desired set

of PostScript commands. Next enter a <RETURN> followed by another CTRL-^.

The POST utility interprets the CTRL-^ as follows:

When the first CTRL-^ (Decimal 30 or Hex 1E or Octal 36) is found,

POST sets a flag and sends all other characters directly to the

printer (or file).  When the next CTRL-^ is encountered, POST

reverts to its normal self.  In this fashion, all sorts of control

can be exerted upon the output of a PostScript printer.   For example,

if the following code were embedded in a normal ASCII file, a shaded

logo would show up on the background of the output.  Please make sure

that the CTRL-^ is the first character of the embedded PostScript

code and it is recommended that you use either a save-restore pair or a

gsave-grestore pair to surround your embedded code to avoid corrupting

the remainder of the code generated by the POST utility.

Example:

#

gsave    % there is a CTRL-^ before the gsave

200 175 moveto

500 175 lineto

500 225 lineto

200 225 lineto

closepath

.5 setgray

fill

/Times-Roman findfont 50 scalefont setfont

250 185 moveto

1 setgray

(POST 2.2) show

gsave 250 185 moveto 0 setgray

(POST 2.2) true charpath stroke grestore

grestore  % there is a CTRL-^ after the grestore

#

8

POST 2.0

The save-restore (or gsave-grestore) should surround the PostScript

code if it is independent of the current page.  Embedded PostScript

commands can be included anywhere in a file (the result of the code

will be printed on the page that is currently being rasterized.)

Shareware:      This program is distributed as Shareware.  If you

find this program useful, send $10.00/computer to:

F. C. Betts

c/o Veda Incorporated

Suite 200

5200 Springfield Pike

Dayton, OH 45431

Any comments and/or suggestions will be appreciated and a

response will be sent if possible.

9