The following files are part of COMM.ARC:

COMM.C - Used to create a .OBJ which contains all the communications

functions.

COMM.H - File with documentation for functions, and prototypes.

TERM.C - Sample terminal program using the communications functions.

The COMM.C file creates functions for an interrupt-driven communications

handler.  The default buffer size is 4K but can be increased to a larger

value by performing the following in your code:

extern int BufSize;

main()

{

BufSize=NewBufferSize;

Please note that the above assignment must be done before the functions

are called to setup the interrupts.

To compile the COMM.C file, using one of the following lines:

MicroSoft:

CL (or QCL) /c COMM.C

Turbo:

TCC -c COMM.C

This will create a COMM.OBJ file which contains the functions.  To test

them, compile the TERM.C program as follows:

MicroSoft:

CL (or QCL) TERM.C COMM.OBJ

TurboC:

TCC TERM.C COMM.OBJ

When the above steps are performed, the TERM.EXE file should have been

created.  This program is hard coded to work with a modem at 1200 baud

connect to COM1:.  Read the .H file for information on changing the

baud or other settings  via the PortInit function.

The TERM.EXE program is very simple and will allow you to communicate

with your modem as a dumb terminal.  Modem commands like "ATH" to hangup

and "ATDTxxxxxx" to dial are required.  Make sure your modem is on and

connected to your computer.  If all goes well, characters you type on

the keyboard are sent to the modem, the modem then sends them back via

the COM port, where TERM.EXE will get them back and display them for you.

Mario Giannini

Compuserve ID 76276,1576