;  Date: 07-31-92  22:35

;  From: Larry Teghtmeyer

;----------------------------------------------------------------------

;Ok, so I set down last night and came up with a couple of routines  and

;thought I would try them out and see if I could get them into my

;computer as good as they are in my head and here is what I came up with.

;Maybe you can use the information, but I know you can see it can be done

;with A86.  The first program is to be typed in or transfered to a text

;file.  I could suggest test.asm.

;CUT HERE-!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!-

PUBLIC a86test

data segment para public 'data'

message1 db 'This text stored in the assembly data area'

message1l equ $-message1

row db 0

col db 0

attr db 0

rowlen dw 160

code segment para public 'code'

a86test:push bp               ;got to save it for qb later

mov bp,sp                     ;now we can access the passed parameters

push ds,es                    ;we are now saving DS and ES

mov ax,seg message1

mov es,ax

mov si,[bp+6]                 ;lets get the attr calculated in qb

mov ax,[si]

mov es:attr,al

mov si,[bp+8]                 ;get the integer val of col

mov ax,[si]

mov es:col,al                 ;store val of col in row

mov si,[bp+10]                ;now get row

mov ax,[si]

mov es:row,al

mov ax,[bp+12]                ;now we are going for the string passed

from qb

mov bp,ax                     ;store the string location in bp

mov ax,[bp]                   ;get the string len

mov cx,ax                     ;put the string len into cx

mov ax,[bp+2]

mov bp,ax

mov bh,0

mov bl,es:attr

mov dh,es:row                 ;remember this is 0 based instead of 1

based

mov dl,es:col                 ;as in qb

mov es,ds                ;a86 takes care of register to register transfer

mov al,0

mov ah,013                    ;the 0 in front of 13 makes this a hex

number

int 010                       ;bios call to print string

mov cx,message1l              ;this routine will be a direct screen write

mov es,0b800                  ;for b&w use 0b000

mov ax,seg message1           ;get the segment address of our message

mov ds,ax                     ;store it in ds

lea si,message1               ;get the location of our message into si

add row,2       ;we need to print the second string at a different

location

mov al,row

cbw                           ;make it into a word

mul rowlen

add al,col

if c add ah,1

add al,col                    ;this calculates the screen pos to print to

if c add ah,1

mov di,ax                     ;ES:DI = destination;DS:SI = sourse

mov ah,attr

m1:lodsb                      ;loads al with a byte from string

stosw                         ; stores the word in screen memory

loop m1                       ;do this cx times

pop es,ds,bp                  ;one call to pop all trashed registers

retf 4*2                      ;far ret for basic and clean up stack

;CUT HERE-!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!-

;Notice there are no assumes, no memory management, no endp,no ends, no

;other extranious BS. Just code. You don't have to even declare the

;public names, but I do just so no other lables become public and can be

;used by other procedures.

;    Now you have the text file "test.asm".  Just tpye in from the DOS

;command line: A86 +o test.asm

;In an eyeblink, you wil have test.obj to play around with.  Make a

;library and .qlb in the normal manner from DOS. (ie. lib test,

;test.obj;) and (link /q test,etc).  I'm sorry, but *I* type in only what

;*I* need and then follow the prompts!  (I can't remember all the systax

;I need. (Sheepish grin))

;    Now that you have your new library, I have a small bit of qb code

;for you to test it out with:

;CUT HERE-!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!-

DECLARE SUB a86test (a$, row%, col%, attr%)

b$ = "This string stored in qb"

row% = 5

col% = 1

attr% = 31       'BLUE backgroun,white foreground (or 16 * backgroung +

'foreground

COLOR 7, 1: CLS

CALL a86test(b$, row%, col%, attr%)

CUT HERE-!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!-

'load this into qb with the quick library created above and run it.  You

'may be *amaized* to see that it may even work!  Let me know how it turns

'out for you ...

'Type you later.

'                                       Larry Teghtmeyer