Card #150

Date:   September 21, 1987

Number: 195

Title: PRINTING FROM NONDEDICATED FILE SERVERS

It has been found that version 2.01 of Lotus 1-2-3 (and subsequent releases)

will not print from a nondedicated file server to the network printer.  (This

is true all for versions of nondedicated Netware.)

Prior to a printing operation, Lotus will check the equipment list to see

that a printer is available.  Because the printer port has been defined as a

network printer, the equipment list is cleared.  A SPOOL command must be

issued before any applications can print.  Any applications, such as BASIC,

that check for the presence of the printer will see that none are available

according to the equipment list and will not allow printing to occur.

By running "PRFIX.COM" prior to entering the application, the equipment list

will be modified to show that a printer is available and allow the application

to print.  Remember that a SPOOL command must be entered before the PR.FIX

command is entered.

The program "PRFIX.COM" is available from NetWire or LANSWER.

Lotus and Lotus 1-2-3 are trademarks of Lotus Development Corporation.

NetWire is a servicemark of Novell, Inc.

NetWare is a trademark of Novell, Inc.

Card #146

Date:   October 28, 1987

Number: 198

Title: PRINTING OPERATIONS ON NONDEDICATED 286 FILE SERVERS

THIS TECHNICAL BULLETIN PROVIDES FURTHER INFORMATION RELATING TO

BULLETIN #195

It has been discovered that printing in BASIC or with Lotus 1-2-3 (version

2.01 and above) appears to cause the nondedicated workstation to hang for a

short time, after which a "device unavailable" error is generated to the

screen and the print process is aborted.  (This happens regardless of whether

the workstation is spooled and on all versions of nondedicated NetWare.)

To do printing from a nondedicated server, either a local printer or a

server printer may be installed on a given LPT port, but not both.  This is

done to prevent any chance of interleaving the middle of a local print job

with a server print job or vice versa.

During server installation, LPT1, by default, is installed as a server

printer port.  When the server is booted, it zeroes out the DOS LPT1 port

address area in memory to indicate that LPT1 is not available to do local

printing.  If the port address is zero, the workstation will timeout and

generate the error message to the screen.

The solution is to place a non-zero value in the LPT1 port address area.

To do this, the program PRFIX.COM was written.  It can be put into the login

script or be run after logging in, but must be done before the first printing

by any Lotus 1-2-3 or BASIC program.  PRFIX.COM only needs to be run once,

since the value it places in the LPT1 port address remains unchanged until

the server is rebooted again.  Logging in and out will not change it.

When the file server/workstation is used to run applications, it should be

noted that file server processes must share CPU time with the running

applications.  Depending on the intensity of the application, the impact on

file server processes will vary.  It is most noted when printing from the

file server.

When the file server/workstation is running an application, the printer

may run slower than if the file server/workstation is sitting at a DOS prompt

or is in a "console" mode.  Whenever multiple large print operations must be

performed, it is recommended that the file server/workstation either be left

at a DOS prompt or placed in console mode.

PRFIX.COM is available from NetWire or the LANSWER group.

Lotus is a trademark of Lotus Development Corporation.

NetWare is a registered trademark of Novell, Inc.

NetWire is a registered trademark of Novell, Inc.