ETHERNET REMOTE PROGRAM LOAD

Use ETHERRPL to enable remote booting of workstations (mostly diskless) that

have Ethernet network boards installed with IBM RPL style bootroms.

Command format

LOAD [path]ETHERRPL

Command parameters

Replace path with a full directory path to the directory containing the

module; begin with a DOS drive letter or a NetWare volume name.  If you do

not specify a path and the SYS volume is mounted, the operating system

assumes that the loadable module is located in the SYS:SYSTEM directory.

(Use SEARCH to set up additional paths for automatic searching.)

Additional information

To remotely boot a diskless Ethernet workstation, you must complete the

following:

�     Load ETHERRPL.

�     Configure the workstation boot files for remote booting.  Follow the

DOSGEN instructions in Appendix D of the NetWare 386 Installation

manual.

�     Bind the ETHRPL protocol to the network boards in the file server.

To load ETHERRPL, complete the following steps.

1.    Type

LOAD ETHERRPL  <Enter>

2.    Bind the ETHRPL protocol to the driver for the Ethernet network

board(s) in the file server.  Type

BIND ETHRPL LAN_driver  <Enter>

Replace LAN_driver with the name of the driver you are using.  If you

have loaded the driver more than once, you will need to repeat this

step.

3.    Edit the AUTOEXEC.NCF file.  Add the following lines to load and bind

ETHERRPL:

LOAD ETHERRPL

BIND ETHRPL LAN_driver driver_parameters

Replace LAN_driver with the name of the driver you used in Step 2.

Replace driver_parameters with the parameters you used to load the LAN

driver.