AutoCAD Interface for a

TEK222 Digital Storage Oscilloscope

ShareWare Version

Installation Instructions

By: Tom Johnston Jr.

Spring, 1994

AutoCAD Interface for a TEK222 Digital Storage Oscilloscope

General

This software package was created using AutoCAD Release 12's

customization features to provide a graphical front end for a

TEK222 Digital Storage Oscilloscope. This software will enable

the user to transfer, modify and view waveforms and front panel

set-ups. It will also provide the functionality to simulate a

button press on the scope and report diagnostic codes and

communications status available through the DSO's RS-232

interface. Using AutoCAD to save and view waveforms provides a

means to save the imported waveforms as AutoCAD drawing files for

later use in machine service documentation, performance

presentations, troubleshooting analysis and training tools.

Software Details

The structure used in creating this software package

utilized three of AutoCAD's powerful customization features.

AutoLISP, a specialized implementation of the LISP programming

language, serves as the primary or "main" part of the

software. AutoCAD Development System (ADS), a programming

interface which lets you use the C programming language was

used to program the individual functions of the software package.

These C subroutine functions are then loaded by, and called from

the AutoLISP interpreter. The interaction between the user and

the AutoCAD interface was designed and implemented using

AutoCAD's Programmable Dialog Boxes. Dialogue boxes are defined

by ascii files written in Dialogue Control Language (DCL). The

DCL description of a dialogue box defines how the box will appear

and what it will contain: buttons, lists, text and so forth. Both

AutoLISP and the AutoCAD Development System provide functions for

displaying them, and responding to the user's choices.

Functionality

A simple terminal program was used to establish the

communications between the computers RS-232 port and the

oscilloscopes RS-232 interface. The terminal program was

programmed as an ADS function. The commands sent to

the oscilloscope and responses received from it will be saved as

ascii text files. These text files will serve as a means to

archive front panel set-ups and waveforms. The text files

uploaded from the scope will be modified from additional

functions for viewing and changing front panel set-ups. The

modified text files can then be downloaded to the scope to change

the current, or one of four storable front panel setups in the

oscilloscope itself. The ascii text files saved as waveforms,

are used for importing into AutoCad drawings on a previously

created "face plate". This face plate serves two purposes;

first, it will provide the boundaries for the import, and second,

it will provide the user with different ways to present the

information uploaded from the oscilloscope.

TEK222 RS-232 Communications

Communications between the 222 DSO and the PC running

AutoCAD Release 12 software is provided via an RS-232

communications interface. Setting the communications baud rate

for the 222 can only be done using the menu selections on the

scope itself. Commands, front panel setups, and waveforms sent

over the RS-232 interface are composed of ascii character

strings.

The ascii characters represent hexadecimal numbers, with

two characters per hexadecimal value.

The general command format is as follows:

cmnd arg:data;

where cmnd is the command; arg is an argument to the command, and

data is any additional information needed for the command. Refer

to Tektronics "222 RS-232 Interfacing Guide" for more specific

details on Codes and expected formats.

NOTICE: COM PORT, BAUDRATE and PARITY are fixed in this shareware

version to COM2, 9600, NONE. It will always search for the scope

on the defined com port. ENTITY Colors creatred by the

software are fixed also. In the REGISTERED VERSION, a configuration

dialog is included which allows the operator to change operating

parameters of the software.

References and Sources

Autodesk Inc., AutoCAD Customization Manual, Autodesk, 1992.

(AUTODESK Publication number 100891-01)

Autodesk Inc., AutoLISP Programmer's Reference Manual, Autodesk,

1992. (AUTODESK Publication number 100191-01)

Autodesk Inc., AutoCAD Development System Programmer's Reference

Manual,  Autodesk, 1992. (AUTODESK Publication number 100192-01)

MIS: PRESS, Serial Communications in C and C++, Management and

Information Source Inc, 1992.

Tektronix, Inc. 222 RS-232 Interfacing Guide, Tektronix, 1989.

How to Install the TEK AutoCAD Interface

It will be assumed that AutoCAD was installed with the default

Directories and can be run on your work station. The installation

is a very simple procedure.

Simply copy all the files from the distribution diskette into

your ACAD\ADS directory. The files included on the distribution

diskette are listed below.

TEK.LSP           <-- AutoLISP program for Main Tek Dialog.

TEKFP.EXE      <-- ADS Executable: Front Panel Interface.

TEKON.EXE      <-- ADS Executable: ONLINE Interface.

TEKWF.EXE      <-- ADS Executable: Waveform Interface.

TEK.DCL        <-- Dialog Control Language File.

TEKHLPF.HLP    <-- Front Panel Interface Help File.

TEKHLPO.HLP    <-- Online Interface Help File.

TEKHLPW.HLP    <-- Waveform Interface Help File.

*.WF Files     <-- Example Waveform Files.

*.FPS Files    <-- Example Front Panel Set-up Files.

NOTICE: Your work station monitor must be capable of a display

resolution of 800 X 600. If AutoCAD is not configured for this

resolution, or higher, the application will not run on your work

station.

The TEK.LSP file can be automatically loaded when you start

AutoCAD by entering the following line in the ACAD.LSP text file:

(load "tek"   )

Refer to the Tutorial and User's Guide document for instructions

on how to start and run the application files.

The "\ACAD\ADS" path must be included in your ACADR12.BAT file

so that AutoCAD can find the application files.

Example ACADR12.BAT File:

SET ACAD=C:\ACAD\SUPPORT;C:\ACAD\FONTS;C:\ACAD\ADS

SET ACADCFG=C:\ACAD

SET ACADDRV=C:\ACAD\DRV

C:\ACAD\ACAD %1 %2

TECHNICAL SUPPORT:

Users of the TEK222 AutoCAD Interface to can obtain technical

support by contacting: Tom Johnston either by phone, US mail

or CompuServe Email. Phone hours are limited to 6:00pm -

10:00 pm, CST.

Tom Johnston Jr.

2005 Cliff-Alex Ct. S. #2

Waukesha, WI. 53186

(414) 547-2174

Compuserve: 70614,1533

REGISTRATION:

The TEK222 AutoCAD Interface is provided to you as a shareware

product.  You MUST register the package if you plan on using it for

commercial applications.

Registration Price per Installation is $149.00

With your registration, you will receive the Registered Version

of the software, which includes a CONFIGURATION utility where

the user can define and set Operating Parameters for the COM

Port and other configurable options (Yes, even a toggle to tell

the program(s) NOT to communicate with the oscilloscope). You will

also receive the original Users Manual with figures, and documents

including technical discussions on how the program was created

(including pseudo code and function documentation). Actual ADS Code

listings are avaiable with additional cost. Contact me for prices.

HOW TO REGISTER / ORDER:

Enclose a money order or check (in US Funds ONLY Please).

____ TEK222 AutoCAD Package & Registration   @ $149.00 ea  $ _________

US Shipping and handling:                                  $      3.00

Total (US funds):                                          $ _________

Payment by:  ( ) Check         ( ) Money Order

I received ACADTEK  via: ( ) CompuServe

( ) Other: ___________________________________

Name:    ______________________________________________________________

Company: ______________________________________________________________

Address: ______________________________________________________________

______________________________________________________________

______________________________________________________________

Country: _________________________     Date: __________________________

Phone: (______)_______-___________     Fax: (______)_______-___________

Thank you for your order!