Jeffrey D. Prouty                                      18 December 1985

Jordan Oil & Gas Company

P.O. Box 1919

Santa Rosa, CA  95402

Dennis Dubose, et al

Radian Corporation

P.O. Box 9948

Austin, TX  78766

Re:  Use of AutoCAD for data input to CPS/PC

AutoCAD by AutoDesk may be used to prepare x, y, z, polyline, text, and

symbol data for input to CPS/PC.  This may be done as one is drawing a

basemap on which CPS-generated contours may later be plotted.  Seismic

time horizons may also be digitized with AutoCAD, plotted at shotpoints

on the basemap and then extracted as data for CPS.

Basemap preparation begins by starting AutoCAD with a prototype drawing

which sets up the initial drawing environment.  My prototype basemap is

called MAP.DWG and is loaded by default.  It calls my mapping menu,

ATT0.MNU, which contains four submenus which may be accessed from the

screen, cursor buttons, or tablet menu area.

I proceed to build the basemap, usually with land grid from USGS topos,

wells from PI lease maps, and seismic from shot point maps, placing data

on the appropriate layers.  Cursor buttons 8 thru D spot and scale the

well symbols.   Well tops may be input as the wells are spotted by calling

up the ATT1 OR ATT2 menus which prompt for one or two data per well.

These data are stored as attributes WDATA1 and WDATA2 which may be made

visible by setting ATTDISP to ON.  If ATT0 or ATT1 are used both or one

attribute will be given the default value of 99999 (my INDT value in CPS).

Of course, any number of data may be stored in this manner, including

well name, operator, KB, etc.  The well symbol type is inherent to the

block name and will be extracted as the CPS symbol code for each well.

Seismic lines are usually plotted on separate layers using the seismic

submenu.  Lines must be posted in ascending order if data is to be

automatically inserted (see below).  Button 7 (BOL) begins a line, setting

the initial shotpoint number to 100 and the label increment to 10.   These

may be changed via AutoCAD's LISP language, ie. "(setq sp 94)" or "(setq

inc 9)".  Remaining shotpoints are plotted using buttons 4, 5, 8, and 9.

Major shotpoints are labeled with sp which is incremented by inc. All

shotpoints have invisible attributes tagged SDATA.  Menus could allow

immediate data entry as with the wells, or digitized data may later be

added to each plotted shotpoint via a menu generated by a program (see

below).  The line may be ended with button B (EOL) and labeled with C.

Data stored as attributes by wells and shotpoints may be edited with the

ATTEDIT command and extracted with the ATTEXT command.  The manual has

details.  Choose the SDF extraction format.  Examples of my template files

are included as *.TXT files.

I should note that most of my most frequently used AutoCAD command

sequences are stored as macros under SuperKey and therefore do not appear

as menu selections.  Also, viewing and editing menus, scripts, data, and

template files while still in AutoCAD is easy with SideKick's Notepad (use

the MENU command to retrieve edited menus and try out changes).  The SHELL

command will let you out to DOS to do such things but may not let you back

into AutoCAD - save your drawing first!

Digitizing horizons from seismic sections is more complicated but can be

very efficient when working with many horizons on many lines in a given

area.  Begin by starting a new drawing, usually with the name of the

seismic line, using SEIS.DWG as the prototype, ie. "DB01=SEIS".  Call the

SeisSec submenu.  Snap aspect is set to 1 inch horizontal which must

correspond to your sampling interval (usually every fifth shotpoint).

Change if necessary, but leave the vertical snap resolution at .0001 inch.

Calibrate the tablet to your seismic section in inches.  Place 0,0 at

about 1.5 seconds on the leftmost sampled shotpoint.  Note this shotpoint

and time for later use.  Choose the second point at least 10 horz. and 10

vert. inches away, but somewhere in the upper right corner of your

section.  Use "inches" as scaled on the section, not true inches, to

adjust for printer and paper stretch.

Number the horizons and record them by moving the cursor along the

horizons and pressing the horizon number on the cursor keypad at each

sample interval.  The crosshairs should snap between horizontal sample

shotpoints but have full vertical freedom.  ATTEXT with the F key using

SEIS.TXT as the template and the drawing name as the extract file name

(the default).

Now, you may either END the drawing or use the SHELL command to return to

DOS and run the BASIC program SPOST.  This pgm. reads the extract file and

converts x and y coordinates to shotpoints and time.  You may also take

isochrons between any horizons, print tables of the results, and create a

.MNU file with the name of the seismic line.  Running the program should

be self explanatory.  You may then return to AutoCAD to digitize a new

section.

The menu files created by SPOST may be called in to automatically ATTEDIT

the seismic lines posted above on your basemap.  Freeze all but a set of

non-intersecting seismic lines.  Load one seismic line menu.  Select

horizon or isochron to post.  Window that line and the digitized values

will be edited into INDT attribute positions.  Repeat for all lines on the

screen then freeze these, thaw a new set of lines, and continue.  ATTEXT

using SDATA as a template file to create an x,y,z data file ready for CPS.

These are only a few, simple ways AutoCAD may be used for IO to and from

other programs.  I've enclosed an excellent article from CADalyst which

surveys the many roads into and out of AutoCAD.  I'm sure it will generate

many new ideas usefull to your CPS interface.  If I may be of any further

assistance feel free to call (707) 523-0900.  Keep me posted!

Sincerely,

Jeff D. Prouty