ADI2PIC AutoCAD -to- Lotus .PIC File Plotter Driver

Copyright 1987 by William J. Hinkle

Installation Guide

This ADI plotter driver was created to allow AutoCAD (from Autodesk, Inc.)

to produce plotted files in the Lotus .PIC file format.  This is the

graphics file format used by Lotus 1-2-3 graphs.  My primary goal was to

allow AutoCAD drawings to become importable by other programs -- specifically

the Lotus Manuscript technical word processor.  Naturally, Lotus only

allowed graphic imports from their own programs, and a few scanners.

By plotting AutoCAD drawings as .PIC files, Manuscript (and any other

program that handles these files) thinks the file is some sort of 1-2-3

bargraph or piechart and is perfectly happy to integrate the picture.

The plotter driver is a single file: ADI2PIC.EXE.  It can provide the same

functionality and performance as the device drivers distributed with AutoCAD.  The only difference is that

Note that the driver is being released into the public domain by the author

and is NOT supported by Autodesk, Inc.

Follow the instructions for configuring AutoCAD in the AutoCAD

User Reference.  Also, see the AutoCAD Drafting Package Installation

Guide for general information on using AutoCAD with various

peripherals.  If you are using the IBM PC version of AutoCAD with an

IBM PC "work-alike," be sure to the chapter on "work-alikes" in the

Installation Guide.

ADI drivers differ from drivers distributed with AutoCAD in that

they must be loaded BEFORE you begin AutoCAD.  To use the ADI2PIC

plotter driver with AutoCAD, copy ADI2PIC.EXE onto the floppy

disk you use to run AutoCAD, or into the AutoCAD directory on

your hard disk, or to another directory accessible in your DOS

path.

OPERATION

=========

You have two different choices as to using the driver: as an installed

driver (recommended), or as a file translator (if you are short on memory).

INSTALLED DRIVER

================

To use as an installed driver, type in:

ADI2PIC /i

at the DOS command prompt.  This installs the driver in memory; a short

message of acknowledgement is displayed.  (You can perform this from your

AUTOEXEC.BAT file if desired).  Then proceed to invoke ACAD (you must

configure AutoCAD to use this built-in driver; see "Configuration", below).

All plots will be plotted directly to a Lotus .PIC file, whose name you

must specify at the appropriate prompt.  If you want to remove the driver

from memory (to reclaim the RAM space of about 22kbytes), simply type in:

ADI2PIC /r

at the DOS command prompt.  Again, a short message will be displayed upon

removal of the driver.

Note that the driver uses the default ADI plotter interrupt vector: 78H.

You can specify another vector if you have some weird setup; see "Compleat

Usage", below.  Make sure you specify the same vector when installing and

removing the driver.  You can also specify scaling factors, but the defaults

work best.

FILE TRANSLATOR

===============

To use the driver as an ADI plot file translator, first plot your drawing

as a standard ADI .PLT file (see configuration, below).  Then type in:

ADI2PIC fname

or

ADI2PIC fname.PLT

or

ADI2PIC fname.PLT fname.PIC

from the DOS command prompt.  The .PLT and .PIC extensions are assumed if

not entered.  The .PIC file name if made the same as the .PLT if not

otherwise specified.  Full DOS pathnames can be specified.

The .PLT file will be translated to a .PIC file.

The .PLT file will not be erased.  You can also specify scaling factors, but

the defaults work the best.

CONFIGURATION

=============

The first time you use AutoCAD with the ADI driver, you need to

configure AutoCAD for the ADI device.  Select Configure AutoCAD (5)

from the AutoCAD MAIN MENU.  (If AutoCAD was not previously

configured, you will be automatically put into the configuration

process when you start AutoCAD).  Select the ADI plotter option (5)

from the configuration menu.  If you do not have this menu option, check

to see that you have installed your AutoCAD DRV diskette(s) and

(if applicable) any new drivers (files with extension .DRV) from

the AutoCAD Driver Update Disk (available from your Authorized

AutoCAD Dealer).

Follow the remaining configuration questions as detailed in the

AutoCAD User Reference.  If you are using an installed driver (/i)

then specify the installed driver (use the default interrupt vector 78H

unless you have some special problem); otherwise specify either ASCII or

binary ADI output format.

Note that colors (pens) & linetypes are mapped to PIC colors, so multiple

pens and linetypes may be specified.

The most common setup is as follows.  Be sure to use the indicated scaling

factors (X & Y plot sizes, steps, pen width, etc.).

-----

Does the plotter have multiple pens? <Y>

How many pens does it have, 2 to 127 <8>:

Does the plotter have multiple dashed line styles? <Y>

How many line styles are there, 2 to 127 <8>:

Is the plotter variable speed? <N>

Specify plot size in millimeters? <N>

Maximum horizontal (X) plot size in inches <3.1990>:

Plotter steps per inch in the horizontal (X) direction <1000.0000>:

Maximum vertical (Y) plot size in inches <2.3100>:

Plotter steps per inch in the vertical (Y) direction <1000.0000>:

Would you like to calibrate your plotter? <N>

Size units (Inches or Millimeters) <I>:

Plot origin in Inches <0.00,0.00>:

Enter the Size or Width,Height (in Inches) <MAX>:

Rotate 2D plots 90 degrees clockwise? <N>

Pen width <0.001>:

Adjust area fill boundaries for pen width? <N>

Remove hidden lines? <N>

Plotted Inches=Drawing Units or Fit or ? <F>:

-----

COMPLEAT USAGE

==============

If you invoke ADI2PIC from DOS without any other parameters, it will

display the following help message.  This sort of explains some of the

other parametric options.

-----

ADI-to-PIC Translator: V2.0 ... Copyright 1987 by W.J.Hinkle

Usage: ADI2PIC fname1.PLT [fname2.PIC] [/xN][/yN][/v][/i[N]][/r[N]]

where: fname1.PLT ... AutoCad ADI plotter format file

in ASCII or binary format

fname2.PIC ... Lotus PIC output file (default is fname1.PIC)

/xN        ... optional x plot scale = N points (default is 3200)

/yN        ... optional y plot scale = N points (default is 2311)

/iN        ... optional driver installation flag

/rN        ... optional driver removal flag

where N = optional int vector in hex (default is 78H)

ADI2PIC can function as a translator to .PIC files or as an

AutoCad installed plotter device driver (using the /i flag).

For best results, configure your AutoCad plotter as the

ADI (virtual) plotter: ASCII or binary format (without /i flag)

or installed driver (with the /i flag).  If you can't use the default

installation int vector (78H) add a different vector number: e.g. /i70.

Set the X width to 3.199, Y height to 2.310, both with 1000 points/unit.

Set the pen width to 0.001 units.  Colors & linetypes are mapped

to PIC colors, so multiple pens and linetypes may be specified.

Other scaling factors may be used if you specify /xN /yN, above.

-----

CAVEAT

======

Should you have any difficulties configuring or using AutoCAD

with this ADI driver, do not contact Autodesk.  They don't know

anything about it.  Since it's free, the author doesn't either.

COPYRIGHT INFO

==============

This driver ADI2PIC.EXE was written by

William J. Hinkle

who retains the copyright.  Copy and use the driver freely.  Distribute

it to whomever you want.  But:

=== Do not distribute modified versions.

=== Do not remove the copyright notice.

=== Always distribute with the documentation.