AUTODESK, INC.

Autodesk Device Interface / ADI(tm)

Driver Development Kit

Version 4.0

Graphics Displays

Rendering Devices

Digitizers

Plotters

Printer/Plotters

Publication TDADIDOC01.04                                  October 7, 1988

Copyright (C) 1986,87,88     Autodesk, Inc.

All Rights Reserved

This publication is distributed by Autodesk, Inc. so that independent parties

can develop peripheral device drivers for use with Autodesk software products.

This publication, or parts thereof, as well as associated materials distributed

on diskette, and software developed using these materials, i.e., ADI device

drivers intended to function with Autodesk software products, may be

distributed freely without royalty to Autodesk, Inc., subject to the following

conditions.

Users of the ADI Driver Development Kit are granted a limited license to use

and/or make copies of these materials for use by others in order to develop

graphics peripheral device drivers for Autodesk products.  This limited license

does not include:

1. Distributing the ADI Kit in connection with any other product.

2. Making the ADI Kit available for any consideration or "disk fee."

3. Distributing the ADI Kit in modified form.

Autodesk, Inc.  makes no warranty, either expressed or implied, including but

not limited to any implied warranties of merchantability or fitness for a par-

ticular purpose, regarding these materials and makes such materials available

solely on an "as-is" basis.

In no event shall Autodesk, Inc. be liable to anyone for special, collateral,

incidental, or consequential damages in connection with or arising out of pur-

chase or use of these materials.  The sole and exclusive liability to Autodesk,

Inc., regardless of the form of action, shall not exceed the purchase price of

the materials described herein.

For condition of use and permission to use these materials for publication in

other than the English language, contact Autodesk, Inc.

Autodesk, Inc. reserves the right to revise and improve its products as it sees

fit.  This publication describes the state of this product at the time of its

publication, and may not reflect the product at all times in the future.

The latest version of the ADI Driver Development Kit, including printed docu-

mentation, may be obtained by sending US$100 in check, VISA, or MasterCard to:

ADI Program

Autodesk, Inc.

2320 Marinship Way

Sausalito, CA  94965

Add $6 sales tax if you are located in California.

Comments on these materials are welcome.  Correspondence may also be exchanged

on the Autodesk Forum on CompuServe(R).  Type GO ADESK when you log on.

This manual was published in October, 1988, based on Version 4.0 of the ADI

Driver Development Kit.  It was prepared using the GML/PC(tm) document composi-

tion system from Command Technology Corp., Oakland, CA.

Autodesk, the Autodesk logo, AutoCAD, AutoCAD AEC, AutoLISP, AutoSketch, and

CAD/camera are U.S. registered trademarks of Autodesk, Inc.  ACAD, ADE,

Autodesk Device Interface, ADI, Advanced User Interface, AUI, AutoFlix,

AutoShade, AutoSolid, and DXF are additional trademarks of Autodesk, Inc.

CompuServe is a registered trademark of CompuServe Incorporated.  IBM and

PC-DOS are registered trademarks of International Business Machines Corpora-

tion.  Intel is a trademark of Intel Corporation.  MS-DOS is a trademark of

Microsoft Corporation.  PostScript is a registered trademark of Adobe Systems

Incorporated. UNIX is a trademark of AT&T Bell Laboratories.

Chapter 1

The Autodesk Device Interface/ADI

Overview

The Autodesk Device Interface (ADI) is an interface specification that allows

manufacturers, dealers, and users to develop customized drivers for peripheral

devices that will work with PC-DOS/MS-DOS and UNIX versions of AutoCAD(R), and

PC-DOS/MS-DOS versions of AutoShade(tm), AutoSketch(R), and other Autodesk

products.

Autodesk Product                Supported ADI Drivers

Graphics  Rendering  Digitizer  Plotter  Printer/

Display    Device                        Plotter

AutoCAD              o                    oo        oo        oo

AutoSketch           o                     o         o        o

AutoShade            o          o          o

KEY:   o       ADI interface available under PC-DOS/MS-DOS only

oo      ADI interface available under both PC-DOS/MS-DOS and UNIX

NOTE:  Early versions of AutoSketch supported only the ADI digitizer driver.

ADI graphics display, plotter, and printer/plotter support were added in Ver-

sion 1.04.

By using the Autodesk Device Interface, a developer can write, test, distrib-

ute, and support a peripheral driver that is compatible with Autodesk software

products without relying on Autodesk to implement the driver.  ADI driver

development requires a fairly high degree of programming skill; this kit is not

intended for use by novices.

The following diagram depicts the relationships and chain of communication

between the Autodesk software product, your ADI device driver, and the periph-

eral device.

PC-DOS/MS-DOS ADI              UNIX ADI

+--------------------+      +-------------------+

|  Autodesk product  |      |      AutoCAD      |

+--------------------+      +-------------------+

^    |                       ^   |

|    v                       |   v

+--------------------+            +------+

|  xxGEN.DRV generic |             | P  |

|    device driver   |             | I  |

|   (AutoCAD only)   |             | P  |

+--------------------+             | E  |

^    |                      +------+

|    |                        ^   |

|    v                        |   v

+--------------------+      +-------------------+

|  Your ADI driver   |      |   Your ADI driver |

+--------------------+      +-------------------+

^    v                        ^   v

+--------------------+      +-------------------+

|  Peripheral device |      | Peripheral device |

+--------------------+      +-------------------+

1

Autodesk Device Interface/ADI

Developer's Responsibilities

ADI drivers are distributed and supported by the developer (you), not by

Autodesk.  It is your responsibility to inform users how to install and config-

ure your driver and peripheral.

If you are a peripheral manufacturer, and wish

to use the ADI Driver Development Kit, you must

supply Autodesk with the name of a technical

support person in your organization.  Autodesk's

Product Support staff will be supplied with a

list of these names, and questions from users

about your driver will be referred to a knowl-

edgeable individual in your organization.

What You Need

To implement an ADI driver, you should be familiar with the Autodesk products

you intend to support, and their operation.

UNIX ADI drivers may be written in any language you choose; PC-DOS/MS-DOS ADI

drivers must be written in a language that can utilize PC-DOS/MS-DOS software

interrupts.  The examples in this documentation are written either in C, or in

assembly language.

The C language examples presented in this documen-

tation and included in the ADI Driver Development

Kit are intended for use with Lattice C version

3.1, Microsoft C versions 4.0 and 5.0, or Borland

Turbo C versions 1.0 or 1.5.  If you are using

another C language compiler, you may need to

modify the examples.

What The ADI Kit Includes

The ADI kit contains C and assembly language sample source code.  The sample

source code, or any portion thereof, can be used in your ADI driver implementa-

2

(1) The Autodesk Device Interface/ADI

tion without any royalties or additional fees paid to Autodesk, Inc.

The ADI kit also includes batch files, link files, sample installation-guide

text, and debugging aids.

Working with ADI

Because of resource limitations, Autodesk cannot provide technical assistance

to ADI Driver Developers.  ADI kit users are expected to have sufficient tech-

nical skills to implement their drivers without assistance from Autodesk.

ADI kit users may wish to use the Developer's Corner in the Autodesk Forum on

CompuServe to exchange information; if a bug or deficiency is discovered in the

ADI kit, developers will be notified via that medium.  To enter the Autodesk

Forum, type GO ADESK when you log onto CompuServe.

Testing

The importance of testing your finished ADI driver cannot be overemphasized.

This documentation describes the tests that Autodesk's Quality Assurance

Department performs as a minimum for every device driver supported.

Testing is critical to the success of your driver, and thorough testing can

reduce the volume of user support calls generated by a malfunctioning driver.

Remember that you are responsible for the support of the finished ADI driver,

and that user support calls concerning your driver will be directed to you.

Releasing Your Driver

How you distribute your ADI driver is up to you.  You may want to ship it on a

disk included with your product, or make it available to your dealers.

You may not distribute the AutoCAD files DSGEN4.DRV, DGGEN.DRV, PLGEN.DRV, and

PPGEN.DRV. These drivers are copyrighted, proprietary Autodesk software prod-

ucts and are specific to individual AutoCAD releases.  AutoCAD users may obtain

updates of these drivers (if necessary) from their authorized AutoCAD dealer.

Versions of these drivers are included in each AutoCAD PC-DOS/MS-DOS release

(along with the other peripheral device drivers).

Your driver should continue to work with new releases of Autodesk software

products without any changes required.  New features added to the ADI interface

may require that you update your ADI driver if you want to take advantage of

them.

Autodesk, Inc.  publishes a list of hardware manufacturers and other developers

known to have completed ADI device drivers.  This list is distributed to deal-

3

Autodesk Device Interface/ADI

ers and posted in the Autodesk Forum on CompuServe.  To be included in this

list, send a copy of your completed ADI driver on disk, and all user documenta-

tion to:

ADI Program

Autodesk, Inc.

2320 Marinship Way

Sausalito, CA  94965

4

Chapter 2

ADI Calling Sequences and Coding Practices

This chapter describes the calling sequences, conventions, and coding practices

used in PC-DOS/MS-DOS ADI drivers for all device types. Similar information is

provided for UNIX ADI drivers in Chapter 8.

Sample Drivers and Development Aids

The ADI Driver Development Kit contains several examples and tools to aid you

in writing and debugging a PC-DOS/MS-DOS ADI device driver.  The files provided

include:

o  DSEGA4.C, DSEGAA4.ASM, DSEGA4.EXE, M5-EGA4, M-EGA4, B-EGA4, L-EGA4,

B15-EGA4 - Source, make files and executable code for a sample ADI 4.0

Packet-mode EGA display driver using a display list.  This driver

incorporates numerous debugging features.

o  DSEGA.C, DSEGAA.ASM, DSEGA.EXE, M-EGA, L-EGA, B15-EGA, B-EGA - Source,

make files and executable code for a sample ADI 4.0 Packet-mode

driver, with no display list.  This driver also incorporates numerous

debugging features.

o  RDPGCP.C, RDPGCPA.ASM, RDPGCP.EXE - Source and executable code for an

IBM Professional Graphics Controller (PGC) rendering driver.

o  RDPJ.C, M5-PJ, M-PJ, L-PJ - Source, make files and executable code for

a sample HP-PaintJet rendering hard copy driver.

o  DGDEMO.C, DGDEMO.EXE - Source and executable code for a Microsoft

mouse digitizer driver.

o  DGSUM.C, DGSUM.EXE, M5-SUM, M-SUM, L-SUM, B15-SUM, B-SUM - Source,

make files and executable code for a sample tablet digitizer driver.

o  PPDEMO.C, PPDEMO.EXE - Source and executable code for a sample

printer/plotter driver.

o  IWAIT.ASM, PCALL.ASM - Assembly language support routines for ADI

drivers written in the C language.

o  ADI.H - C language definitions used in all PC-DOS/MS-DOS ADI device

drivers.

o  COLOURS.H - C language definitions for standard physical and logical

color assignments.

o  DGADI.H - C language definitions for ADI digitizer drivers.

5

Autodesk Device Interface/ADI

o  DSADI.H - C language definitions for ADI graphics display drivers.

o  RDADI.H - C language definitions for ADI rendering device drivers.

o  README.DOC - Last-minute documentation changes and clarifications.

Make files (in Microsoft MAKE format) are distributed with several of

the sample drivers. Each make file is named with a prefix indicating

which compiler it is designed for, as follows:

Prefix     Description

B       Borland Turbo C Version 1.0

B15     Borland Turbo C Version 1.5

L       Lattice C Version 3.1

M       Microsoft C Version 4.0

M5      Microsoft C Version 5.0

The mnemonics for request codes, parameters, and returned values used through-

out this document are those defined in the various .H C language header files

supplied with this kit.

The Direction Flag (DF)

PC-DOS/MS-DOS, various C compilers, and all Autodesk

products rely on the 8086-family CPU's direction flag

(DF) being CLEAR.  If your driver sets the direction

flag (by performing a STD instruction) and neglects to

clear it before returning (by performing a CLD instruc-

tion), unpredictable and potentially disastrous conse-

quences may result.

Interrupt Vectors

ADI device drivers are implemented as DOS programs that terminate and stay

resident.  They are not true MS-DOS device drivers (although they may be imple-

mented as such).  Your installed driver should patch itself into a vacant

interrupt location, then exit to DOS, remaining resident in memory.  Installa-

tion of the driver must be accomplished before the Autodesk product that uses

it is executed.

Since the Autodesk product does not know where in memory the driver is located,

an INT request to a particular interrupt vector is used to activate the driver.

Some drivers supply information that allows packet-mode communication subse-

quent to this initial request.

When configuring an Autodesk product for use with an ADI driver, the user sup-

plies the interrupt vector number the Autodesk product will use to communicate

6

(2) ADI Calling Sequences and Coding Practices

with the driver.  Be sure to identify the interrupt vector clearly in your user

documentation.  (You should fully acquaint yourself with the Autodesk product's

configuration process in order to provide clear documentation for users of your

ADI driver.) The default ADI interrupt vectors for the various device types are

shown in the following table.

Device Type    Default INT Vector

Plotter                   78h

Digitizer                 79h

Graphics display          7Ah

Rendering device          7Ah

Printer/plotter           7Bh

For instance, a generic digitizer driver is integrated into all Autodesk soft-

ware products (for AutoCAD, it's the file DGGEN.DRV), and appears on the digi-

tizer configuration menu as "ADI digitizer" or "ADI pointer".  When the user

selects the ADI driver from the configuration menu, the generic driver prompts

for the following information:

Hexadecimal interrupt code (INT 0XXh) <79>:

In response to this prompt, the user must supply the interrupt vector number

that will be used to communicate with your installed driver.  Your documenta-

tion must tell the user which interrupt to use.  The default interrupt vector

is shown in the prompt.

Note that the graphics display and rendering device have the same default

interrupt vector (7Ah).  This is for the convenience of drivers that implement

both sets of functions.  If your rendering device is separate from the graphics

display, you should specify a different interrupt vector for it.

ADI Driver Initialization

When your driver is first loaded, it should perform any one-time initializa-

tion, check for presence and suitability of the hardware device (if feasible),

register an interrupt service routine at the desired interrupt vector, and then

terminate but stay resident.  To aid in identifying your ADI display driver,

the initialization sequence should display a message indicating the version

number of the driver, the name of the developer, and the date the driver was

released.

If your driver requires the user to provide configuration information, you

should include any desired questions in your driver's initialization sequence.

Alternatively, such information can be obtained from the command that first

loads the driver into memory.

Driver Compatibility

The ADI specification may be extended from time to time as new capabilities are

added to Autodesk products.  All Autodesk products supply an ADI level indica-

7

Autodesk Device Interface/ADI

tor to ADI drivers during driver initialization.  This level number is

increased when new functions are added to the interface.

Some ADI drivers support optional features that are not supported by other

drivers.  Similarly, some Autodesk products require more from a driver than do

others.  Therefore, means are provided, during initialization, for the Autodesk

products and ADI drivers to communicate about the features they require and

support.

These mechanisms allow a driver to determine whether or not it is compatible

with a particular Autodesk product.  Drivers can safely ignore Autodesk product

features they do not need, and enhancements they do not support.  Similarly,

Autodesk products can often be used with drivers that do not support all the

optional features.  In practice, this means that drivers written for use with

one level of an Autodesk product generally will continue to work with future

versions, and may work with older versions as well.  Of course, if a driver is

to take advantage of new features added in a later version of the ADI inter-

face, it must be revised.

INT-mode Calling Sequence

Autodesk products begin communication with an ADI driver by issuing an inter-

rupt (INT) request to the driver's configured interrupt vector, passing infor-

mation back and forth through the AX, BX, CX, and DX registers.  When the

driver's interrupt handler receives control, the AX register always contains a

request code specifying the task the driver is instructed to perform.  The use

of the other registers varies from request to request.  The driver returns

status codes and results in the same four registers.  The meaning of the

results returned depends upon the request.

The ADI Driver Development Kit contains an assembly language routine that can

mediate between an Autodesk product's INT functions and your driver code writ-

ten in C.  This routine is called iwait and its source file is IWAIT.ASM.  An

ADI driver using this routine should be written essentially as a large loop.

After doing any one-time initialization, the driver should fall into a loop as

follows:

for ( ; ; ) {

iwait(VECTOR, &iregs, &oregs);

... processing ...

}

where VECTOR is the interrupt vector utilized by your driver and entered by the

user during configuration, as described above.  iregs and oregs are structures

consisting of four short values corresponding to the AX, BX, CX, and DX regis-

ters.  For example:

8

(2) ADI Calling Sequences and Coding Practices

struct cregs {

short ax;

short bx;

short cx;

short dx;

};

static struct cregs iregs, oregs;

To simplify making your driver a terminate and stay resident program (TSR), the

first call to iwait installs the driver at the interrupt vector VECTOR and then

terminates but remains resident.  The iwait routine will return to its caller

only upon receipt of an interrupt to VECTOR from an Autodesk product.

When the Autodesk product performs an INT to VECTOR, it stores the contents of

the AX, BX, CX, and DX registers into iregs, and transfers to iwait to return

control to your driver.  The driver performs the requested task, stores its

results in oregs, and then calls iwait to return control to the Autodesk prod-

uct.

You may notice that iwait is a complement of the Computer Innovations C86

library function, sysint, except that it uses fewer registers.  In fact, a pair

of programs doing sysint's and iwait's form a co-routine linkage.  iregs and

oregs may (and usually will) be the same structure.  Several of the INT

requests are designed with this convention in mind, although there is no

requirement either way.

Packet-mode Calling Sequence

All ADI drivers begin with an INT request as described above.  After this ini-

tialization interrupt, the Autodesk product can continue to communicate with

your ADI driver using the INT interface.  For graphics displays and rendering

devices, however, an alternative communication method is possible.  This

method, called Packet mode, uses far calls and passes arguments and results in

packets rather than in registers.  When it returns from the initialization

interrupt, the driver can supply the memory address of its packet handler and

instruct the Autodesk product to switch to Packet mode for all subsequent com-

munication.

Packet mode is the preferred interface for an ADI graphics display driver, and

the only interface for a rendering device driver.  For a graphics display, the

Packet interface is not only more efficient than the older INT interface; it is

also more straightforward, therefore making it much easier to develop graphics

display drivers.  Also, whereas the INT interface for graphics displays is sup-

ported only by AutoCAD, the Packet interface is applicable to all Autodesk

software products.  This section describes the calling sequences and conven-

tions used by Packet mode.

The following calling sequence is used by the Autodesk product when issuing

packet-mode requests:

9

Autodesk Device Interface/ADI

push[PKT_LENGTH]

push[SEGMENT]

push[OFFSET]

callfar [ENTRY]

where PKT_LENGTH is the length of the request packet in bytes (supplied for

those who wish to copy the packet to local storage), and SEGMENT:OFFSET is the

packet address.  ENTRY is the address (SEGMENT:OFFSET) of the driver's packet

processing routine which was returned in response to the INT-mode startup

request.

When control reaches the driver's packet handler entry point, the contents of

the DS and ES registers are undefined, but they must be preserved by the

packet-processing routine.

If you wish to write your packet-mode ADI driver in C, you may use the assembly

language routines supplied in PCALL.ASM to mediate the interface with the

Autodesk product. These routines are written to work with Lattice C version

3.10 "-mp" memory model, Microsoft C version 4.0 or 5.0 medium model and Bor-

land C version 1.0 or 1.5 medium model depending on ifdef conditional tests.

See the MAKE files for examples of each environment's settings.

10

(2) ADI Calling Sequences and Coding Practices

When using the PCALL interface in a graphics display driver, you must define a

global variable:

struct pkfille packet;

In a rendering device driver, this would be:

struct rendreq packet;

(If the driver is to support both the graphics display and rendering functions,

packet should be a union composed of a struct pkinit and a struct rendreq, to

ensure that it's large enough to handle the largest packet sent to either type

of device.)

Determining Maximum Packet Size

In the ADI 3.1 sample drivers, the packet communication structure was defined

as follows:

struct pkinit packet;

#define P ((struct PT *) &packet)

This worked because pkinit was the largest packet. In ADI version 4.0 pkfille

is the largest packet, so we must define the packet communication structure as

follows:

struct pkfille packet;

#define P ((struct PT *) &packet)

Future revisions of ADI are likely to implement new packet types, and the long-

est packet may become still larger.  To help ensure upward compatibility of

display drivers with later revisions of ADI, the assembly code in PCALL.ASM has

been modified to check the size of each packet which arrives from the Autodesk

product.  If a packet is larger than the largest packet the driver knows about,

it will be truncated by PCALL.  In addition, drivers should henceforth ignore

unrecognized packets (instead of complaining about them, as in the past).

To use the PCALL interface, the driver must also provide an externally-

accessible function named propkt that will be called with no arguments to pro-

cess each packet.  When this function is called, the packet passed by the

Autodesk product will be found in the local buffer, packet, which may be cast

to the packet type used by a particular request.  Examination of the sample

driver source provided in DSEGA.C and RDPGCP.C will elucidate the fine points

of this interface.

To utilize this interface, use the following C code to handle the INIT1

request:

11

Autodesk Device Interface/ADI

unsigned short cvec[2];

c.arg1 = 0;      /* Request packet mode */

pksetup(cvec);   /* Obtain packet handler entry point */

c.arg2 = cvec[0];/* Entry point offset */

c.arg3 = cvec[1];/* Entry point segment */

c.code = 3;      /* Success indicator */

where c is the oregs structure used to pass results back to the Autodesk prod-

uct (via iwait).

The pksetup routine will store a function pointer to an assembly language rou-

tine in PCALL.ASM.  That routine will mediate between the Autodesk product and

your C driver function, propkt.

The propkt function is expected to return a result of type int.  If this return

value is nonzero, the local packet will be copied back to the Autodesk product.

Otherwise, the packet is not copied.  Only functions that return information to

the Autodesk product should return nonzero values from propkt.

All packet fields are stored with 8086-family byte ordering and no slack bytes.

Each packet-mode request uses a different packet format, but the first field in

every request packet is pfunc, a 16-bit field that specifies the function the

driver is to perform.  The next chapter describes the packet-mode requests,

their function codes, packet formats, parameters, and purposes.

Use of Expanded Memory

ADI display, printer/plotter, and plotter drivers (but not ADI digitizer or

rendering drivers) have free use of Lotus-Intel-Microsoft Expanded Memory Spec-

ification (LIM EMS) physical pages from the time they are initialized by an

Autodesk product until the time they are terminated.

The ADI driver should acquire whatever physical space it needs when it is first

executed, although as much expanded memory as possible should be left for use

by the Autodesk product. (Currently, AutoCAD is the only Autodesk product that

uses expanded memory; it will address up to four megabytes of expanded memory

as extended I/O page space.) If necessary, the amount of expanded memory

AutoCAD uses can be controlled by the system variable ACADLIMEM.

It is not necessary to map physical pages to logical pages for each request

issued by an Autodesk product.  Mappings established at the time the initial-

ization request is issued remain in effect for all subsequent requests up to

the matching terminate request.  The only exception is for display drivers

during the execution of a SHELL command.

Obviously, any ADI driver functions that execute as the result of timer or

other (non-Autodesk product) interrupts must follow the LIM EMS practices of

all other interrupt handlers.

12

(2) ADI Calling Sequences and Coding Practices

Use of Extended Memory

With the release of ADI 4.0, the use of expanded and extended memory by display

list drivers is likely to increase.  Since both AutoCAD and Extended AutoLISP

use extended memory, developers must exercise some caution to avoid conflicts.

AutoCAD can use up to 4 megabytes of extended memory as additional I/O page

space.  The amount and location of the extended memory AutoCAD uses can be con-

trolled by the system variable ACADXMEM.  If no ACADXMEM setting is supplied,

AutoCAD uses all extended memory (up to 4 megabytes) that is not in use by a

standard RAM disk (VDISK) driver.

Extended AutoLISP uses the DOS/16M DOS extender from Rational Systems, Inc.

Extended AutoLISP informs other applications of its use of extended memory by

chaining onto INT 15 FUNC 88 while loading.  Extended AutoLISP also checks an

environment variable (LISPXMEM) to determine its use of the available memory;

as with AutoCAD, this setting is honored if it does not conflict with a VDISK

allocation.

Display list drivers that want to use extended memory should chain onto INT 15

FUNC 88 so that they can cooperate with AutoCAD and Extended AutoLISP in the

use of extended memory.  If necessary, you can also instruct your users to set

the environment variables ACADXMEM or LISPXMEM to control the Autodesk prod-

ucts' use of extended memory.

In addition, it is good practice for ADI drivers to check for VDISK headers.

In a system with a lot of extended memory, a user might have one or more

VDISKS, Extended AutoLISP, a display list driver, and AutoCAD, all using dif-

ferent portions of extended memory.

Both Extended AutoLISP and any ADI drivers must be loaded before AutoCAD; the

sequence in which these drivers are loaded is also important.  Loading an ADI

driver after loading Extended AutoLISP will prevent remlisp.exe from recovering

the available memory when removing Extended AutoLISP.  Your driver documenta-

tion should explain clearly the order in which drivers must be loaded, as well

as any environment variable settings required to avoid memory conflicts.

Some existing ADI 3.1 drivers use products like

CEMM to convert extended memory into expanded

memory for use by the driver.  Unfortunately,

CEMM uses Virtual 8086 mode in a manner which

blocks the use of extended memory by Rational

Systems' DOS extender; thus, using CEMM blocks

the use of Extended AutoLISP.

Drivers should exercise caution in using INT 15 on a non-IBM BIOS.  Check to

see that there is a vector on INT 15 before using it, if the driver is loaded

before Extended AutoLISP chains onto the interrupt.  Try to check for valid

code before executing the function, and check that the returned data is reason-

13

Autodesk Device Interface/ADI

able.  If all else fails, you might try having your driver pretend to be a

VDISK.

Generally, if a clone does not implement INT 15 properly, Extended AutoLISP

will not operate.  (Of course, Extended AutoLISP only runs on 80286 or 80386

machines.)

Here are a few pitfalls that a driver should watch for when using INT 15:

1. Many BIOSs (including IBM's) fail to clear the carry flag on success,

so you probably should do this before calling INT 15.

2. Some BIOSs (such as AT&T's Version 1.0) don't even return from INT 15.

You may want to check for known special cases. In addition, you may

want to check for the presence of a -286 or -386 CPU before trying INT

15.

3. Some BIOSs return impossible numbers for extended memory.

4. Some BIOSs return with interrupts disabled.

The following code fragment shows how to check for the AT&T BIOS problem (in a

large code, small data program):

dummy   proc    near                ; That's right, **near**

ATTsqnc label   byte

stc

mov     ah,86h

ret     2

ATTsqncL equ    $-ATTsqnc

dummy   endp

getextendedsize:

xor     ax,ax

mov     es,ax               ; set up to read INT 15h vector

les     di,es:[15h*4]       ; load int vec ptr into ES:DI

mov     ax,es

or      ax,di

jz      noxmem              ; If zero--don't dare use it!

mov     si,OFFSET ATTsqnc

mov     cx,ATTsqncL

repe  cmpsb                 ; If AT&T instruction

jz      noxmem              ; sequence--don't use it!

clc                         ; Even IBM AT forgets to clear CY

mov     ah,88h              ; on success--it sets it on failure

int     15h                 ; get extended memory size

sti

cld

jc      noxmem              ; if AH=86H and carry set...failure

or      ax,ax               ; AX has # of 1K byte pages

jle     noxmem              ; Don't take ridiculous values

cmp     ax,16384

jae     noxmem

14

(2) ADI Calling Sequences and Coding Practices

Don't forget that AutoCAD also uses expanded (LIM) memory.  If your driver uses

expanded memory, it should allocate all the LIM memory it needs before it ter-

minates and stays resident.  See the section on expanded memory above for more

details on this.

Suggested Standards for Floating Point Operations

You can assume the presence of a math coprocessor for floating point operations

in AutoCAD Release 9 and later.

1. To keep from making your TSR larger with emulation library code, we

recommend that you use a compiler option generating in-line floating

point code.

2. Do not alter any floating point traps or control bits within the 80x87

during execution.  It's OK to do so during TSR initialization since

that happens before AutoCAD starts running, and we will reset the

environment to what AutoCAD wants.  If your code alters any of the

status bits, please restore them as they were on entry to your driver

before you return to AutoCAD.

3. Make no assumptions about the various floating point exception traps,

rounding directions, number of infinities, etc.  We currently do not

enable floating point traps, but may do so in a future release; as a

result, any of the floating point states might change from one version

of AutoCAD to another.

4. Write defensive code which never generates any floating point excep-

tions, NANs or infinities - at least assuming that AutoCAD doesn't

pass any in.

5. There will be small roundoff errors in any floating point numbers

passed to you from AutoCAD.  You should allow some small "fuzz" factor

if you attempt a test for equality (perhaps 1E-10).  It is possible to

write a big screen driver which does not use floating point tests for

equality.

6. Although this isn't necessarily a floating point issue, please note

that if AutoCAD is asked to split the screen into two "equal" view-

ports, they may be slightly different in size because the starting

screen size available for the division was odd in the direction of

division.

Direct Hardware Programming vs. BIOS Calls

Most ADI drivers must establish certain modes for the hardware device (e.g.,

graphics modes or color palette for a display, data rate and parity for a

serial I/O port, etc.).  In many cases, the computer's BIOS (Basic I/O System)

provides hooks for setting these modes and for letting the operating system

15

Autodesk Device Interface/ADI

know what's been set.  When such hooks exist, by all means use them; if the

operating system knows what modes are set, operation of the driver in nonstand-

ard environments (with other TSR programs, odd operating systems, etc.) is less

prone to failure.  If, on the other hand, appropriate BIOS calls do not exist

or are incapable of setting the particular modes you need, direct manipulation

of hardware registers and special memory locations may be the only method

available.

Sometimes, BIOS calls are provided for graphics drawing once the operational

modes have been established, but these calls may be inefficient or not quite

what is needed.  In these cases (high usage, where performance counts), it's

fine to do direct hardware programming.  Since mode-setting is needed less fre-

quently, efficiency is not an important issue and BIOS calls should be used

when possible.

Debugging Your Driver

AutoCAD cannot be run under a debugger.  Experience has shown that debugging of

ADI drivers is best accomplished by inserting "print" statements at key points

in the code.  For a display driver, it may be necessary to direct the debugging

output to a file.  For instance, the DSEGA.C and DSEGA4.C sample drivers con-

tain trace code that, when enabled, shows the contents of all request packets.

This can help demystify function sequences and show actual packet field values.

16

Chapter 3

ADI Graphics Display Specification

This chapter contains information that you will need to develop an ADI graphics

display driver for use with Autodesk software products running under

PC-DOS/MS-DOS.

Configuration

A generic graphics display driver is integrated into each Autodesk product and

appears on each product's display configuration menu as "ADI display v#.#"

(where #.# is the version number).  For AutoCAD, the generic display driver is

a separate file named DSGEN4.DRV, which was first distributed (as DSGEN.DRV)

with the IBM PC/XT/AT version of AutoCAD 2.18 in March of 1986.

The generic display driver operates by calling a DOS terminate and stay resi-

dent (TSR) graphics display driver (the one you create) that is installed prior

to entering the Autodesk product.  When configuring the Autodesk product, the

user must select the ADI display driver (the generic driver) from the configu-

ration menu.

For AutoCAD Version 2.18

DSGEN.DRV appears as "ADI display v2.18" on the video display configuration

menu.  AutoCAD 2.18 users must have the version of the DSGEN.DRV display driver

date-stamped March, 1986.  A DSGEN.DRV with an earlier file date-stamp will not

function correctly.  This was ADI version 1.0 (interface level 1), and

supported only the INT interface.  For a driver to work with this level of the

ADI package, it must use INT mode and must not rely on features added in later

levels (see Appendix C).

For AutoCAD Versions 2.5x and 2.6x

DSGEN.DRV appears as "ADI display v2.1" on the video display configuration

menu.  AutoCAD 2.5x and 2.6x users must use the version of the DSGEN.DRV dis-

play driver distributed with their version of AutoCAD.  These were ADI versions

2.0 and 2.1 (interface levels 2 and 3), and supported only the INT interface.

For a driver to work with these levels of ADI, it must use INT mode and must

not rely on features added in later levels (see Appendix C).

17

Autodesk Device Interface/ADI

For AutoCAD Release 9

AutoCAD's user interface was significantly enhanced in Release 9, so the ADI

graphics display interface was extended to take advantage of the advanced fea-

tures.  Initial shipments of Release 9 included ADI version 3.0 (interface

level 4).  A subsequent driver update featured ADI version 3.1 (interface level

5).

In addition to the existing Interrupt (INT) interface for ADI display drivers,

an entirely new Packet interface was added.  To use all of the extensions added

to the INT interface, and to take advantage of the new Packet interface intro-

duced in version 3.0 of the ADI Driver Development Kit, users must have AutoCAD

Release 9 and the DSGEN.DRV display driver that appears in the AutoCAD Video

Display configuration menu as "ADI display v 3.0".

For AutoCAD Release 10

AutoCAD's user interface has been further enhanced in Release 10 with the addi-

tion of multiple viewports.  Version 4.0 of the ADI specification (interface

level 6) includes additional functions which allow display drivers using the

packet mode interface to manage these viewports.  In addition, version 4.0 pro-

vides more direct support for vendors who choose to write display list drivers.

Most non-display list drivers should continue to function (in single-viewport

mode) as in the past, though clear-screen operations in multiple viewports will

be noticeably slower.  (Minor programming changes will allow old non-display

list drivers to clear the screen much more quickly.) Old display list drivers

should continue to work in a single viewport, but will probably not work at all

in multiple viewports; they will need to be rewritten to handle multiple view-

ports properly and to make use of the new features provided in version 4.0.

Display drivers that use the Interrupt (INT) interface will continue to work,

but this interface has not been extended to support the enhancements mentioned

above.

Support for the INT interface will be dropped in

the next major AutoCAD update after Release 10.

Autodesk strongly recommends that all new driv-

ers use packet mode.

For Other Autodesk Products

To use an ADI display driver with another Autodesk product, select the "ADI

display" item from the configuration menu.  If a hexadecimal interrupt code is

requested, refer to "Interrupt Vectors" on page 6 for instructions.  The ADI

display drivers in all products other than AutoCAD support only the Packet

interface, and have interface levels of 4 or higher.

18

(3) ADI Display

The Graphics Display Environment

In order to write a graphics display driver for use with an Autodesk product,

you must understand what that product's display interface looks like and how it

works.  We suggest that you first experiment with the product's features as

they relate to the display, using a display already supported by the software.

The following sections should help you get started.

AutoCAD

Conceptually, AutoCAD's Drawing Editor uses two display screens: a text screen

(the hidden screen), and a graphics screen (see the illustrations in Chapter 1

of the AutoCAD Reference Manual).  For systems with one display, the AutoCAD

user can flip between these screens at any time by pressing a function key

(except while a dialogue box is displayed on the screen or the DVIEW, ZOOM D or

VPOINT commands are in progress).

The text screen contains the last screenful (generally the last 25 lines) of

text.  The graphics screen is logically divided into four windows: the graphics

area, the text-scrolling area, the status line, and the screen-menu area.  With

the exception of the graphics area, most display drivers permit the user to

turn each of these areas off to maximize the portion of the screen devoted to

the drawing.  Standard placement of these windows is with the text-scrolling

area at the bottom of the display, the screen-menu area at the right, the

status line at the top, and the remainder devoted to the graphics area.

19

Autodesk Device Interface/ADI

The ADI driver must communicate the sizes of these various screen areas to

AutoCAD.  The following illustration shows most of the variables involved.

[ See AutoCAD drawing file ADI.DWG ]

Since not all display hardware has the ability to store and manipulate a com-

plete graphics bitmap and a full screen of text, AutoCAD will take care of cer-

tain functions that the display device cannot handle.  During driver

initialization, the ADI driver must tell AutoCAD which capabilities it has.

For example, if the display device and driver are not capable of storing and

scrolling a text screen independent of the graphics screen, AutoCAD will keep a

buffer of text and scroll it as needed.

Unlike earlier versions of AutoCAD, Release 10

uses a temporary file to store the text screen;

retrieving this text for display will be some-

what slower than in earlier versions.  ADI dis-

play drivers that can handle the text screen

themselves should do so, since this will proba-

bly improve performance.

There are two forms of display devices: those that can operate in a single-

screen mode, and those that operate in a dual-screen mode.  Dual-screen devices

use one monitor for the graphics screen and another monitor for the text

screen.  Examples of currently supported dual-screen displays are the Control

Systems Artist I and the Number Nine Revolution board.  Single-screen displays

use only one monitor.  Examples of single-screen displays are the Hercules

Graphics Card and the IBM Enhanced Graphics Adapter.  Some devices can be oper-

ated in either single-screen or dual-screen mode, depending on the hardware

configuration of the user's machine.  An example of such a device is the IBM

Color/Graphics Adapter.  For dual-screen systems, the text-scrolling area is

replaced by a one-line prompt area, although the term "text-scrolling area" is

often used to refer to whichever of these is present.  Further information on

the differences between single-screen and dual-screen drivers is presented in

"Single-Screen vs. Dual-Screen AutoCAD Drivers" on page 22.

Advanced User Interface

AutoCAD Release 9 (ADE-3) added support for a menu bar, pull-down menus, icon

menus, and dialogue boxes.  Collectively, these features are called the

Advanced User Interface, or AUI.  The pull-down menus require a pointing

device; the keyboard cannot be used to access them.  When the user moves the

pointing device to the top of the graphics area (into the status line), the

menu bar replaces the status line.  Pull-down menus are then accessed from the

menu bar.  The menu bar feature relies on the status line being configured ON

and being positioned at the top of the screen.

In order to use the Advanced User Interface features, the display driver must

be capable of saving and restoring the contents of the screen where the menu

20

(3) ADI Display

bar, pull-down menu, icon menu, or dialogue box will be displayed.  Further-

more, dialogue boxes may be "stacked" one on top of another.  In AutoCAD, the

sum total of dialogue boxes on the stack at any time may exceed one physical

screen in size.  For AutoShade and AutoSketch, however, the total never exceeds

one screen.  The most recent box pushed onto the stack is always the first one

to be popped off it.

A display driver may ask the Autodesk product to handle storage for any box

push requests which the driver can't handle locally.  AutoCAD automatically

pages such requests through I/O page space to extended RAM or disk.  AutoSketch

and AutoShade will also give the display driver some help in storing box

pushes, but they can only provide a limited memory allocation, defined by a

value (psavebl) supplied by the ADI driver during initialization.  If the

driver requires more non-local storage than these products can provide, it must

page to extended/expanded memory or disk directly.

ADI Version 4.0 adds support for multiple viewports, a feature new to AutoCAD

Release 10.  In addition, Version 4.0 provides increased support for "big-

screen" (large bit-map or display list) drivers.  The illustration below dia-

grams the relationship (for such drivers) between "real", "logical" and

viewport-independant "pixel" (or LLG) coordinates for a given viewport, and the

variables used to reference each.

[ See AutoCAD drawing file BIGSCRN1.DWG ]

AutoSketch

AutoSketch's display environment is much simpler than that of AutoCAD.  A menu

bar occupies the top text line of the screen, and the bottom line of the screen

is used for prompts.  The remainder of the screen is for the drawing.

Pull-down menus and dialogue boxes are the only means of command selection and

mode setting in AutoSketch.  These features use the same driver functions as

the AUI features in AutoCAD, but support of these functions is required in

order for a driver to be used with AutoSketch.

AutoSketch can provide up to 40k of box-saving space, but this subtracts from

drawing space and limits the user to smaller drawings.  If your driver requires

additional storage for box-saving and is to work with AutoSketch, it will have

to perform the paging to disk or extended/expanded memory.

AutoShade

Like AutoCAD, AutoShade employs two screens.  A graphics display with an

appearance much like AutoSketch's is used for command selection and parameter

setting by means of pull-down menus and dialogue boxes.  This screen is also

used when plan or wireframe images are displayed.  AutoShade's second screen is

the rendering screen; it is here that fast-shaded and full-shaded images are

displayed.  The two screens can be separate display devices, or the same

device.

21

Autodesk Device Interface/ADI

AutoShade can provide up to 100k of box-saving space (in a 640k system), but

asking for this much will degrade AutoShade's performance. Some graphics boards

have local memory (like the upper 128K of a 256K EGA board) for saving box

pushes.  If your board doesn't, you will be requesting memory from the Autodesk

product, we suggest using a single bit plane for menus and dialogue boxes, to

minimize the amount of memory requested from the Autodesk product.  (Since

there are no box pushes or pops in rendering mode, you can use all bit planes

for it.)  Alternatively, your driver could page to disk or extended/expanded

memory on its own.

Writing Display Drivers

There are two forms of interface for ADI graphics display drivers:  the origi-

nal Interrupt (INT) interface, and a new Packet interface.

The interrupt interface supports ADI display drivers written for AutoCAD ver-

sions 2.18, 2.5x, 2.6x, Release 9 and Release 10. Version 3.0 of the ADI dis-

play driver contained a number of extensions to this interface, to support the

new Advanced User Interface with pull-down menus introduced with AutoCAD

Release 9.

To enable ADI drivers to run as fast as built-in drivers, and to incorporate

enhancements to the new Autodesk user interface, Autodesk developed an entirely

new packet interface.  This interface is not only more efficient, it is more

straightforward, simplifying the task of writing graphics display drivers for

AutoCAD. Although drivers written for the interrupt interface will continue to

work in Release 10, the new features of ADI version 4.0 are only supported via

the packet interface. This interface includes a special fast vector mode that

allows a carefully coded ADI graphics display driver to run faster than a

driver built into AutoCAD.

All new drivers should use the packet interface.

While AutoCAD supports both the interrupt and

the packet interfaces, newer Autodesk products

(such as AutoShade and AutoSketch) work only

with the packet interface.  Future releases of

AutoCAD will support only the packet interface.

Single-Screen vs. Dual-Screen AutoCAD Drivers

Dual-screen-only AutoCAD display drivers are quite a bit simpler than single-

screen drivers.  There is one unique dual-screen function call (to move the

text cursor to the prompt area), and a dual-screen driver does not have to deal

with a text-scrolling area.  Since the driver also doesn't have to worry about

characters sent to the text screen, the whole issue of special characters is

sidestepped.

22

(3) ADI Display

Technically, a single-screen driver has a text-scrolling area at the bottom of

the graphics screen, and a dual-screen driver has a one-line prompt area there.

The prompt area (of a dual-screen system) contains only messages that are

internally designated as prompts.  Informational messages and echoing of user

input does not appear in the prompt area.  In addition, the way text is sent to

the two areas differs quite a bit.

For a single-screen driver, text is sent to the text-scrolling area one charac-

ter at a time via PWRSPLIT.  The characters sent can include carriage return,

backspace, new-line, tab, and (in certain rare cases) any character except NUL.

The driver's PWRSPLIT handler must scroll the text area when a new-line charac-

ter is encountered on the bottom line of the screen, or when the last column of

the bottom line is reached.

For dual-screen drivers, text is sent to the prompt area by a PTPROMPT call

(which sets the text cursor to the start of the prompt area) followed by a

series of PCHAR calls (with pretty well-behaved characters) and a PECHAR call

to mark the end of the string of PCHAR calls.  With the exception that the

sequence begins with a PTPROMPT call instead of PMNUCURS, this is exactly the

same mechanism as that used to display an item in the screen menu.  The only

things to watch out for with this mechanism are:

1. Ignore any character that you don't want to display.  Most drivers

ignore everything less than 32 (space) or greater than 127.

2. Ignore anything beyond the maximum length of the prompt area.  The

worst case prompt (in the English-language version of AutoCAD) is the

PLINE command's prompt for input when you are starting an arc segment

of a polyline.  (That prompt actually has a new-line character embed-

ded in it!)  Also note that the driver should not wrap to the begin-

ning of the line if the input string is too long.  This is because

AutoCAD pads each prompt with enough spaces to ensure that the previ-

ous prompt is blanked out.  If the driver wraps, then a short prompt

that happens to follow a prompt that took more than one line would

wind up being invisible due to the extra spaces at the end of it.

On a dual-screen system, AutoCAD uses DOS to display characters on the text

screen, without calling your display driver for anything. (See "ADI Graphics

Display Test Procedures" on page 83 for more details on the relationship

between the graphics and text screens, and some hints on how to test your

driver for proper performance.)

Natural and Biased Coordinates

For most display boards, natural (also known as "ULS") pixel coordinates have

their origin at the upper left corner of the screen.  Autodesk products also

use biased Y coordinates ("LLG" coordinates) that place the origin at the lower

left corner of the drawing area (which may not be the lower left corner of the

screen due to the possible presence of the text-scrolling area below the graph-

ics area).

23

Autodesk Device Interface/ADI

Generally, ADI requests used in actual drawing are in biased Y coordinates.

ADI requests involving the advanced user interface use natural Y coordinates.

Thus, natural Y coordinates are used in displaying the menu bar and the line

under it, in creating pull-down menus, dialogue boxes, and in displaying the

arrow cursor that is part of the advanced user interface.

24

(3) ADI Display

The following ADI commands are used only as part of the advanced user interface

and use natural Y (ULS) coordinates:

PACKET Interface  INT Interface

PBOXSIZE     BOX1

PBOXPUSH     BOXCLR

PBOXPOP      BOXSIZE

PLINE        BOXPUSH

PDCURS       BOXPOP

PCCURS       VMOVE

VDRAW

The following commands use biased Y (LLG) coordinates:

PACKET Interface  INT Interface

PDOT         DOT

PVEC         MOVE

PFILL        DRAW

PMARK        FLOOD

PCMARK       MARK

FASTDRAW     CMARK

PIXWID and PIXHGT

To find the physical screen size, either use a ruler to measure the actual

size, or use the display's specified aspect ratio as screen size, then compute

the actual size of a pixel (screen size/# of pixels) and reduce to as small an

integer as practical, maintaining proportion between X and Y.  The driver must

supply these values to the Autodesk product.  PIXWID is the screen width, and

PIXHGT is the screen height.

Some common video monitor aspect ratios (width:height) are as follows:

EGA          4:3

VGA          4:3

IBM monochrome  3:2

The following equations calculate PIXWID and PIXHGT for the EGA and VGA (high-

resolution mode) displays.  First, for the EGA:

(4 / 640) / (3 / 400) = (4 * 400) / (3 * 640) = 1600 / 1920 = 5 / 6

Thus, PIXWID = 5 and PIXHGT = 6.

For the high-resolution VGA:

(4 / 640) / (3 / 480) = (4 * 480) / (3 * 640) = 1920 / 1920 = 1

Thus, PIXWID = 1 and PIXHGT = 1.

25

Autodesk Device Interface/ADI

Finally, for a monochrome Hercules Graphics Card using an IBM monochrome dis-

play:

(3 / 720) / (2 / 350) = (3 * 350) / (2 * 720) = 1050 / 1440 = 35 / 48

Thus, PIXWID = 35 and PIXHGT = 48.  Note that we use 350 for the number of scan

lines, even though the Hercules Graphics Card supports only 348.  We had to use

the 350 figure, because the 3:2 aspect ratio is defined for 350.

Notification of SHELL Commands

Notification that AutoCAD is executing a SHELL command is essential for ADI

display drivers that are subject to having their graphics area or controller

state altered by a SHELL program, and for drivers that wish to use the Lotus-

Intel-Microsoft Expanded Memory Specification (LIM EMS) efficiently.  (See "Use

of Expanded Memory" on page 12.)

During initialization, an ADI display driver can tell the Autodesk product that

it wants to be notified of the start and end of SHELL commands.  When notifica-

tion of an impending SHELL command occurs, the driver should save whatever

state it needs and relinquish any resources that it does not wish to maintain

during the execution of another program.  For a display that is also the

primary system display, this would normally involve restoring the state of the

hardware to that expected by the operating system and other programs.

For drivers that have mapped any LIM EMS expanded pages to physical pages, they

should assume the physical pages have been unmapped once they return from the

"before SHELL" request.  Following a "before SHELL" notification, the driver

should not use expanded memory until the "after SHELL" notification is given.

At that time, it may remap any LIM EMS expanded pages it had acquired.

Upon receipt of the "after SHELL" notification, the driver should restore what-

ever state it saved and re-acquire any resources it needs.  It is common prac-

tice to set a flag that will result in repainting the screen on the next

GOGRAPH request if there is danger that the SHELL program may have altered the

screen buffer contents.

Function Usage Table

The following table summarizes which display (and rendering) driver functions

are used by each Autodesk software product.

Packet mode INT mode   Auto- AutoShade  Auto- Org       Comments

CAD  Disp Rend Sketch

1 INIT1        x     x    x     x       primary initializa-

tion

continued ...

26

(3) ADI Display

Packet mode INT mode   Auto- AutoShade  Auto- Org       Comments

CAD  Disp Rend Sketch

2-6 INITn      x                        further INT-mode

initialization

1 PINIT                   x     x          x       further packet-mode

initialization

2 PTERM    9 TERM         x     x          x       termination

3 PCLEAR   10 CLEAR       x     x          x       clear graphics

screen

4 PDOT     11 DOT         x     F          x   LLG grid dots

12 MOVE        x                    LLG most drawing if no

fastdraw, arrow

cursor in AUI

5 PVEC     13 DRAW        x     x    U     x   LLG most drawing if no

L   fastdraw; arrow

cursor in AUI.

Returned in L coor-

dinates if EF_BS is

set.

6 PFILL    14 FLOOD       x     F          x   LLG solid fill; smart

L   devices only.

Returned in L coor-

dinates if EF_BS is

set.

7 PMARK    15 MARK        x     F          x   LLG Draw graphics

cursor.  If EF_BS

is set, driver must

clip to viewport.

8 PCMARK   16 CMARK       x     F          x   LLG Clear graphics

cursor.  If EF_BS

is set, driver must

clip to viewport.

9 PCHAR    20 CHAR        x                        draw text char at

text cursor posi-

tion

10 PECHAR  21 ECHAR       x                        end of text

11 PHLITE  22 MHLITE      x                        highlight screen

menu box

12 PDHLITE 23 MDHLITE     x                        dehighlight screen

menu box

13 PMNUCURS24 MNUCUR      x                        move text cursor to

screen menu box

14 PTPROMPT30 TPROMPT     x                        dual-screen only;

move text cursor to

prompt line

15 PQPLOT  31 QPLOT       x                        dump graphics

screen to printer

16 PRPEN   32 RPEN        x                    LLG read light pen

17 PGOTEXT 33 GOTEXT      x                        single-screen only;

flip to hidden text

screen and clear it

continued ...

27

Autodesk Device Interface/ADI

Packet mode INT mode   Auto- AutoShade  Auto- Org       Comments

CAD  Disp Rend Sketch

18 PGOTEXTU34 GOTEXTU     x                        single-screen only;

flip to hidden text

screen, display

hidden text

19 PGOGRAPH35 GOGRAPH     x                        single-screen only;

flip to graphic

screen

20 PWRSPLIT36 WRSPLIT     x                        write text char to

text scrolling area

21 PMODELINE  37 MODELINE x                        move text cursor to

status line

22 PCOORDLINE 38 COORDLINEx                        move text cursor to

coordinate line

23 PSYNC   50 SYNC        x                F       flush local display

buffers

24 PREDRAW 51 REDRAW      x                F       redraw graphics

screen

25 PCOMMAND52 DSCOMAND    x                        execute driver-

implemented command

26 PXPCOMD 53 DSXPCOMD    x                        CTRL-L initiated

transparent opera-

tion

60 BOX1        x                    ULS AUI - set lower

left corner of box

27 PBOXCLR 61 BOXCLR      x     x          x   ULS AUI - clear box to

specified color

28 PBOXSIZE62 BOXSIZE     x     x          x   ULS AUI - establish

buffer size to save

box

29 PBOXPUSH63 BOXPUSH     x     x          x   ULS AUI - save box on

stack

30 PBOXPOP 64 BOXPOP      x     x          x   ULS AUI - pop box from

stack

65 TATTR       x                        AUI - set raster

text attributes

31 PTEXT   66 TEXT        x     x    U     x   ULS AUI - draw raster

text

67 VMOVE       x                    ULS AUI - set line

"from" point

32 PLINE   68 VDRAW       x                    ULS AUI - draw line

69 BOXBUF      x                        AUI - buffer

address for BOXPUSH

/ BOXPOP

33 PDCURS                 x     x          x   ULS draw raster cursor

34 PCCURS                 x     x          x   ULS clear raster cursor

35 PBSHELL 54 BSHELL      x                        SHELL command

starting

36 PASHELL 55 ASHELL      x                        SHELL command com-

plete

37 PDIGTIZE56 DIGTIZE     x                        Digitizer input

38 POPENVP                x                        open viewport

continued ...

28

(3) ADI Display

Packet mode INT mode   Auto- AutoShade  Auto- Org       Comments

CAD  Disp Rend Sketch

39 PCLOSEVP               x                        close viewport

40 PVIEWPORT              x                        respecify viewport

in logical space

41 POCONIN                x                        override keyboard

input

42 PTABCFG               x                        supply coordinates

of screen pointing

area

43 PDRAG                  x                    LLG Manipulate drag

L   lists.

dmode=1  LLG coor-

dinates for all

vectors

dmode=2  L coordi-

nates for all vec-

tors

44 PLOPEN                 x                        supply projection

identification

number

45 PCLVP                  x                        fast clearscreen

FASTDRAW                  x                    LLG EF_BS=0  PV coordi-

PV  nates sent to PVEC

L   EF_BS > 0  L coor-

PV  dinates sent

NO_BS > 0 PV

coordinates sent

2000 RDLINKUP                   V    x

2001 RDINIT                     V    x

2002 RDSTART                    V    x

2003 RDEND                           x

2004 RDTERM                     V    x

2005 RDRESIZE                   V    x             on windowing sys-

tems

2006 RDCLEAR                         x

2007 RDCMAPB                         x

2008 RDCMAP                          x

2009 RDCMAPE                         x

2010 RDPOLY                          x         LLG

2011 RDFNAME                    V

2012 RDCRANGE                        x

2013 RDCPOLY                         x         LLG

KEY:

x    indicates usage

F    indicates likely future usage (but anything is possible)

AUI  Advanced user interface

Org  Coordinate origin point

29

Autodesk Device Interface/ADI

L    viewport-relative logical coordinates

LLG  viewport-independant pixel coordinates, with origin at lower left

corner of graphics screen ("Y biased" coordinates)

ULS  upper left of screen ("natural")

PV   LLG viewport 0 vectors are sent to PVEC

U    indicates a display driver command that may be sent to an

AutoShade display driver during rendering (after RDSTART)

V    indicates a rendering driver command that may be sent to the ren-

dering driver while in display mode (before RDSTART).

30

(3) ADI Display

Packet Interface for ADI Graphics Display Drivers

The ADI Packet interface is the preferred interface for an ADI graphics display

driver.  The Packet interface is not only more efficient than the older INT

interface; it is also more straightforward, therefore making it much easier to

develop graphics display drivers.  Also, whereas the INT interface is supported

only by AutoCAD, the Packet interface is applicable to all Autodesk software

products.  This section describes the requests issued by Autodesk products to

communicate with an installed ADI graphics display driver using the Packet

interface.

Included in the Packet interface is a special fast vector mode that allows a

carefully coded ADI graphics display driver to actually run faster than a

driver built into AutoCAD.

General information on packet-mode calling sequences can be found in Chapter 2.

Each packet-mode request, its function code, packet format, parameters, and

purpose, is described in detail below.

Packet-mode Display Driver Requests

This section describes the requests used by Autodesk products to communicate

with an installed ADI graphics display driver employing the Packet interface.

--------------------  INIT1 (1)  --------------------

Communication with a packet-mode display driver begins with an INIT1 request

(function code 1) issued through the INT interface.  The results returned by

the driver in response to the INIT1 request determine which mode the Autodesk

product will use for further communication with the driver.

Parameters:  Code    (AX):   1

Arg1    (BX):   CONFIG

Arg2    (CX):   (reserved,INTLEVEL)

Return values:  OK   (AX):   0  =  unsuccessful initialization

1  =

pre-ADI 4.0 driver successfully initialized

3  =

post-ADI 3.1 driver successfully initialized

Res1    (BX):   0

Res2    (CX):   OFFSET

Res3    (DX):   SEGMENT

In the INT interface, Res1 would be returned with the width of the graphics

area in pixels.  Here, the 0 returned for Res1 is an obviously invalid specifi-

cation for the screen width; the Autodesk product recognizes this as a request

to begin packet-mode communication.

31

Autodesk Device Interface/ADI

Res2 and Res3 specify the offset and segment, respectively, of the driver's

entry point to be called from the Autodesk product to process each subsequent

ADI request packet.  If your driver is written in C and uses the IWAIT and

PCALL assembly language support functions, refer to Chapter 2 for information

on how you can do this from your C code.

If feasible, your driver should perform tests to determine whether the display

device is present and suitably configured from a hardware standpoint. If all is

well, the driver should set OK to either 1 or 3 before returning; otherwise, it

should set OK to zero.  Returning a value of 1 in OK indicates successful ini-

tialization.  Setting OK to 3 indicates that initialization was successful and

that the driver is a post-ADI 3.1 driver which can truncate excessively long

packets.  This will cause the pkinit packet passed by PINIT to be extended by

adding ybias, and the PFILL (6) packet to be extended by adding vpnumber.

The first packet-mode request will be a PINIT (1) call to obtain initialization

parameters.  The PINIT request will supply copies of the CONFIG and INTLEVEL

arguments, so you can ignore the arguments supplied by the INIT1 request.

Future revisions of the ADI Display interface

may allow for additional packet-mode requests to

occur between the INIT1 handshake and PINIT.

--------------------  PINIT (1)  --------------------

The packet is provided to the driver with fields pfunc, pconfig, and pintlev

initialized by the Autodesk product.  All other fields should be filled in by

the driver and returned to the Autodesk product.

struct pkinit {

short   pfunc;

short   pstat, pconfig, pintlev;

short   pixdots, piydots, pixdotsm;

short   pmodelinl, pmnuchars, pmaxboxes;

short   pmodes;

char    psplit, phwfill;

short   pixwid, pixhgt;

char    phidlines, phidcols;

long    pefmodes;

short   pymenumax;

short   pnuxdots, pnuydots;

short   pnuxchars, pnuychars;

short   pnuxbpc, pnuybpc;

long    pfdraw, psavebl;

short   ybias;

};

32

(3) ADI Display / Packet Mode

Several of the packet fields are used only by AutoCAD; this will be noted in

the individual field descriptions.

The ADI driver must set pstat to 1 if the device was initialized correctly;

otherwise, pstat should be set to 0.  If the driver requires an interface level

other than that specified by pintlev, or features not indicated as present by

pconfig, it should return with pstat set to 0.

pintlev identifies the version of the driver interface for upward

compatibility.  The current version of the driver interface (Version 4.0)

assigns the value 6 to pintlev.  Driver interface levels 3 and higher support

packet-mode requests.

pconfig is a bit-coded field that specifies the driver features supported or

required by the particular Autodesk product in use, as well as the screen con-

figuration defined by the user during configuration.  Currently defined bits

for pconfig are as follows:

Mnemonic   Bit  Value                      Meaning

CF_TEXT   0     1    set if text scrolling area defined (AutoCAD

only)

CF_MENU   1     2    set if screen menu area defined (AutoCAD only)

CF_STATUS 2     4    set if status line defined (AutoCAD only)

CF_CURS   3     8    set if raster cursor support required

CF_DIGTIZE4     16   set if digitizer filtering supported

CF_ZOUT   5     32   0 = initial display is zoomed in

1 = initial display is zoomed out

CF_SYNC   6     64   set if PSYNC (23) request supported

CF_SHADE  13   8192  set if product is AutoShade

CF_SKETCH 14  16384  set if product is AutoSketch

CF_ACAD   15  32768  set if product is AutoCAD

If CF_CURS (pconfig bit 3) is set, the driver must provide support for the

raster cursor requests, PDCURS (33) and PCCURS (34).  AutoCAD does not

currently require raster cursor support and therefore does not set this bit in

pconfig.  However, AutoShade and AutoSketch do require raster cursor support.

If CF_CURS is set and the driver does not support raster cursors, it should

return with pstat set to 0 to indicate initialization failure.  If an ADI

driver is to work with AutoSketch and AutoShade, it must support the raster

cursor requests and respond by setting both the CF_CURS bit in the pconfig

field, and the EF_CURS bit in the pefmodes field (described below).  (Autodesk

products that do not require raster cursor support may use it if the driver

sets the EF_CURS bit.)

CF_ZOUT, CF_SYNC, and the product identification bits (CF_SHADE, CF_SKETCH, and

CF_ACAD) are only meaningful for pintlev values 5 and greater.

The driver should set pixdots to one less than the horizontal size of the

graphics area (minus the screen menu area), and piydots to one less than the

vertical size of the graphics area (minus the text scrolling area and status

line).  (AutoCAD only)

33

Autodesk Device Interface/ADI

pixdotsm is the maximum horizontal pixel number (including screen menu area)

beginning at zero.  The total number of pixels in the X direction is

pixdotsm+1.  (AutoCAD only)

pmodelinl is the number of characters available for the status line.  pmodelinl

must be at least 37 to avoid an abbreviated status line.  The count does not

include characters available for the coordinate display.  (AutoCAD only)

pmnuchars is the number of characters available for screen menu items, and

pmaxboxes is the number of screen menu boxes on the screen (zero if no screen

menu is configured).  See the CF_MENU bit of the pconfig field.  (AutoCAD only)

pmodes is a bit-coded field that specifies modes pertaining to the operation of

the display device.  Currently defined bits for pmodes are as follows:

Mnemonic  Bit  Value                 Meaning

DM_CCECH   0     1    0 = echo CTRL-C

1 = suppress echo of CTRL-C

DM_HLEN    1     2    set if display supports highlighting

DM_GROK    2     4    0 = dual-screen operation

1 = single-screen operation

DM_CCECH (pmodes bit 0) indicates that echoing a CTRL-C to the screen while in

graphics mode will destroy the display, and that CTRL-C should therefore be

suppressed.  This bit should be set only for single-screen displays that lack

separate image planes for text and graphics.  Setting this bit will inhibit a

CTRL-C that is typed ahead (follows other as yet unprocessed keyboard input)

from taking effect immediately.

DM_HLEN (pmodes bit 1) should be set if the display implements highlighting for

selected objects (preferably by drawing them with dashed lines).

DM_GROK (pmodes bit 2) should be set for single-screen operation that requires

the driver to switch between a text screen and a graphics screen.  DM_GROK

should be cleared if a separate graphics screen will be used.  A driver that

supports both single- and dual-screen modes must find out for itself whether a

second screen is present (i.e., check equipment byte, etc.), and then set

DM_GROK accordingly.

If DM_GROK is set (specifying single-screen operation), psplit should return

the number of lines of text that appear in the text scrolling area (normally

three).  Otherwise (dual-screen), the driver should set psplit to 0.

phwfill should be assigned the value 0 for devices without hardware fill, or 2

for devices with polygon fill (1 is not used).

pixwid specifies the horizontal physical spacing between pixels, and pixhgt

specifies the vertical physical spacing between pixels.  The ratio of pixwid to

pixhgt is used to correct the aspect ratio so that the horizontal and vertical

measurements of objects drawn on the screen are equal.  On a device with square

pixels, pixwid and pixhgt should both be assigned the value 1.  On devices with

different X and Y scales, pixwid and pixhgt should be assigned a value based on

the physical screen measurement in each direction divided by the number of

pixels on that axis.  Only the ratio of pixwid to pixhgt is significant, so any

34

(3) ADI Display / Packet Mode

desired units may be used.  However, the values must be greater than 0 and less

than 32767, and should be made as accurate as possible, since changes in the

second decimal place of the ratio can result in visible changes on the screen.

See "PIXWID and PIXHGT" on page 25 for more information and example calcula-

tions.

The values assigned to phidlines and phidcols (normally 25 and 80,

respectively) specify the number of lines and characters to be used for the

hidden screen (AutoCAD only).  If the driver does not require the Autodesk

product to maintain a hidden screen, both of these values should be assigned

the value 0.  Refer to the PGOTEXT (17) request for details. Note that Release

10 (unlike earlier versions of AutoCAD) uses a temporary file to store the

hidden text screen; this can slow performance.  If possible, display drivers

should handle the text screen themselves.

pefmodes is a 32-bit, bit-coded options field.  The status of certain bits in

this field specify the presence (or absence) in the driver of a handler for

various extended requests.  The currently defined bits, and the requests they

represent, are as follows:

MnemonicBit  Value                       Meaning

EF_SYNC   0     1    set to use PSYNC to synchronize the display with

the local display list, or to request special

functions from the Autodesk product.

EF_REDRAW 1     2    set to use PREDRAW to redraw from local display

list. Post ADI 3.1 drivers that set this bit

should also set EF_BS and EF_MV in PINIT, and

must support the new "big screen" features of

ADI Version 4.0.

EF_CMD    2     4    set to use PCOMMAND to process unrecognized com-

mands

EF_XPCMD  3     8    set to use PXPCOMD to process transparent com-

mands

EF_NEWI   4     16   set if driver supports Advanced User Interface

(AUI) requests

EF_SAVEB  5     32   set to request AutoCAD to provide screen-save

buffers (BOXSIZE - 62)

EF_FDRAW  6     64   set if fast vector draw routine is supplied in

pfdraw field

EF_CURS   7    128   set if raster cursor requests are supported

EF_SHELL  8    256   set if SHELL command notification is desired

EF_DIGTIZE9    512   set to use PDIGTIZE to filter digitizer input

EF_MV     10   1024  0 = pre-version 4.0 driver; no support for mul-

tiple viewports

1 = some support for multiple viewports

EF_DRAG   11   2048  set if driver supports drag lists

EF_BS     12   4096  set if this is a "big screen" device

If the driver supports the Advanced User Interface (AUI) requests, it should

set EF_NEWI (pefmodes bit 4) and initialize the following fields: pnuxdots,

pnuydots, pnuxchars, pnuychars, pnuxbpc, and pnuybpc.  Otherwise, the driver

should clear the EF_NEWI bit and set these fields to 0.

35

Autodesk Device Interface/ADI

To support the Advanced User Interface (AUI), the driver must be capable of

saving a screen rectangle and subsequently restoring it (see the PBOXSIZE (28),

PBOXPUSH (29), and PBOXPOP (30) requests below).

Screen data may be saved:

1. in local memory provided by the display adapter;

2. in memory obtained from AutoCAD (AutoSketch and AutoShade don't offer

this option); or

3. in a combination of (1) and (2), depending on the amount of screen

data to be saved.

If EF_SAVEB (pefmodes bit 5) is set, the driver requires AutoCAD to provide

screen-save buffers.  Otherwise, the driver is responsible for buffering screen

images. (See psavebl, below.)

If EF_FDRAW (pefmodes bit 6) is set, the driver supports a fast vector draw

routine.  (See pfdraw, below.)

If your driver supports viewport-specific PCLEAR

but is not a big-screen driver, it should set

EV_MV (pefmodes bit 10) but not EF_BS.  Older

drivers which don't set EF_MV will be sent a

large, filled polygon to clear a viewport.  This

is quite slow for most drivers.

Setting EF_DRAG (pefmodes bit 11) indicates that the driver supports the new,

faster dragging abilities of ADI 4.0.  If this bit is set, the driver receives

PDRAG (43) packets.  This function sends colored vectors (instead of XOR vec-

tors through PVEC) for the entities to be dragged.  When the driver has been

given a list of vectors to drag it may then be given a displacement to apply to

the list.  Another packet will tell you to "forget" the vectors you have been

dragging.  You do not have to be a big screen driver to use PDRAG.  You may or

may not choose to implement color XOR - regular XOR will also work as long as

you are consistent.

If the driver sets EF_BS (pefmodes bit 12), a new field (short vpnumber) will

be added to the end of several ADI function packets and several new functions

will be activated.  When vpnumber is 0, the driver receives information which

is viewport-independent; coordinates are passed in the LLG (lower left corner

of graphic screen origin) ixdots, iydots system.  If vpnumber is nonzero, the

driver must handle a viewport-specific operation in the specified viewport.

The coordinate system will be "logical" coordinates for the specified viewport.

If EF_BS is set, EF_MV should also be set.

The remaining bits in pefmodes are currently unused, and should be explicitly

cleared by the driver to avoid incompatibilities with future versions of ADI.

36

(3) ADI Display / Packet Mode

In a DOS environment the total number of viewports open at one time is limited

to 4 or less, due to memory constraints.  The user may open and close viewports

repeatedly during a single session, however.  Each newly opened viewport is

assigned a new number, resulting in viewport numbers which may be large.  The

driver must maintain a table of the currently open viewport numbers, perhaps

translating these to smaller numbers from 1-4.  If the driver receives any

request (other than POPENVP (38)) for a number not in the table, it is an

AutoCAD error and should be ignored.

37

Autodesk Device Interface/ADI

Master and slave viewports

If the projection for two or more viewports is identical, one viewport may be

treated as a subset (or "slave") of another; this is the case if one viewport

(the slave) is completely contained in another viewport's logical space, and

the number of physical pixels (in each direction) in the slave viewport is less

than the number of logical pixels in the master's logical space (i.e., the

slave can't be zoomed too far in).  See the following illustration.

[ See AutoCAD drawing file BIGSCRN2.DWG ]

In such cases, the driver can optionally maintain a single display list for two

or more viewports.  The driver is passed a projid number (a short) for each

viewport (see PLOPEN (44)); this number will change for each new projection, to

allow you to check for such cases.  If projections are identical, it is the

driver's responsibility to determine if one viewport is a subset of another

viewport's logical space, based on the floating-point coordinates of the poten-

tial slave viewport and the floating-point coordinates of the potential

master's logical space.  (See the sample driver DSEGA4.C for an example.) If

required, the driver can then tell AutoCAD to pass only non-drawing vectors for

the subset viewport(s). (Non-drawing vectors are those used to generate tempo-

rary display artifacts, such as grid, marker blips, etc.)

pymenumax is the maximum vertical pixel assigned to the screen menu area, and

is normally equal to the value assigned to piydots, but may be assigned another

value if the screen menu area has a height different from the graphics portion

of the screen.  (AutoCAD only)

pnuxdots is the width of the physical screen in pixels.  Note that this is the

total width of the screen, not the highest-numbered pixel.  pnuydots is the

height of the physical screen in pixels. AutoSketch and AutoShade require that

the screen size reported in pnuxdots, pnuydots be an exact number of characters

of size pnuxbpc, pnuybpc.  This requirement forces some display drivers to

leave a few pixels on the screen unused.  Failure to observe this requirement

will produce a mis-alignment of text and vectors in the user interface (pull

down menus).  AutoCAD does not have this strict requirement, but it is advis-

able to write display drivers which can work with all three products.

38

(3) ADI Display / Packet Mode

Below is a fragment of code from DSEGA.C, the EGA sample driver, which illus-

trates how to enforce the required screen size parameters.  Note that

AutoSketch and AutoShade also require a ybias correction.

/* if you decide to use a different character size for the */

/* AUI, be sure your PTEXT routine knows to space characters */

/* correctly, and be sure the parameters below are set to */

/* make the AUI screen an exact number of characters large */

P->pnuxbpc = CHARWID;          /* PTEXT routine uses CHARWID */

P->pnuybpc = CHARHGT;          /* to space characters */

P->pnuxchars = XSIZE/P->pnuxbpc;

P->pnuychars = YSIZE/P->pnuybpc;

P->pnuxdots = P->pnuxbpc*P->pnuxchars; /* must be exact multiple */

P->pnuydots = P->pnuybpc*P->pnuychars; /* of character cell size! */

/* Do this after we figure pnuydots and basic ybias...

Now we care which product is in use in case pnuydots <> YSIZE.

AutoCAD doesn't care about this case, but sketch and shade

both require a ybias correction. */

if (acad == 0) {

yb = ybias -= YSIZE - P->pnuydots;

}

P->ybias = ybias;

The screen is presumed to be divided into a matrix of fixed-size character

cells.  pnuxchars specifies the width of the screen in characters, and pnuy-

chars specifies the screen height in characters.  pnuxbpc and pnuybpc specify

the width and height, respectively, of a character cell in pixels. (Drivers for

high-resolution displays which make use of AutoCAD's Advanced User Interface

(AUI) should take care to choose a character set of appropriate size and legi-

bility.)

If CF_CURS (pconfig bit 3) is set, the product calling the driver requires that

the driver provide support for the raster cursor requests, PDCURS (33) and

PCCURS (34).  The driver must confirm that it supports these requests by set-

ting EF_CURS (pefmodes bit 7) or the product will refuse to accept the driver.

(Autodesk products that do not require raster cursor support may use it if the

driver sets the EF_CURS bit.)

pfdraw should return the entry point of the driver's fast vector draw function.

The interface support for this feature is described in "Fast Vector Mode" on

page 77.

If the Autodesk product provides memory for screen images (EF_SAVEB in pefmodes

is set), the psavebl field must be assigned a long value approximating the

number of bytes required to save the entire graphics screen.  Care should be

taken in assigning this value, since the amount of screen-save memory a given

Autodesk product can make available may be limited; larger values should not be

assigned to psavebl.  The value in psavebl need not be absolutely precise, how-

ever.  It is used to guard against running out of memory and should err, if

possible, slightly on the high side.  (AutoSketch and AutoShade use psavebl,

39

Autodesk Device Interface/ADI

but AutoCAD does not; it automatically pages to available space in extended

memory or to disk.)

After the driver has completed its initialization in response to the PINIT

request, the screen should be in graphics mode, cleared, any borders between

logical screen areas drawn, and ready to accept graphics-mode requests.

If EF_BS (pefmodes bit 12) is set, the driver will receive POPENVP (38) shortly

after returning PINIT.  (POPENVP is not guaranteed to be the first packet fol-

lowing PINIT.) This function specifies lgxdots and lgydots.

40

(3) ADI Display / Packet Mode

--------------------  PTERM (2)  --------------------

The PTERM request indicates that the Autodesk product is exiting graphics mode

and that the driver should perform any required cleanup.

struct pkfonly {

short  pfunc;

};

The driver should not clear the text screen, since this would erase any error

messages displayed by the Autodesk product should it exit abnormally.

A single-screen display will always be in text mode when this request is

issued.

--------------------  PCLEAR (3)  --------------------

PCLEAR instructs the driver to clear the graphics portion of the screen.  If

possible, the screen menu area, scrolling area, and status line should not be

disturbed.  If vpnumber is 0, or if EF_BS is not set in pefmodes at PINIT time,

the driver should clear the entire graphics screen (all viewports).  Otherwise,

the driver should clear only the specified viewport, without disturbing its

borders.

struct pkclear {

short  pfunc;

short  prepaint;

short  vpnumber;

};

prepaint is a bit-coded field used to determine which screen areas, if any,

should be repainted following the PCLEAR request.

If the driver is capable of clearing only the graphics portion of the screen

without disturbing the screen menu area, text scrolling area, and status line,

prepaint should be assigned the value 0.  Otherwise, the following bits in pre-

paint should be set, depending on which areas of the screen must be redrawn:

Mnemonic   Bit  Value                    Meaning

RP_STAT     0     1    set if status line should be redrawn

RP_MENU     1     2    set if screen menu area should be redrawn

RP_SCROLL   2     4    set if text scrolling area should be redrawn

RP_GRAPH    3     8    set if graphics area should be redrawn

If the driver draws borders on the screen and they are destroyed by clearing

the screen, PCLEAR should replace them.

41

Autodesk Device Interface/ADI

--------------------  PDOT (4)  --------------------

PDOT instructs the driver to XOR the dot at the location given by (px,py).

struct pkdot {

short   pfunc;

short   px, py;

};

If the device is a multi-plane color display, the dot should be XORed in all

planes.  This function is currently used by the GRID command in AutoCAD and may

be tested by using that command.

--------------------  PVEC (5)  --------------------

PVEC instructs the driver to draw a vector.

struct pkvec {

short   pfunc;

short   pfx, pfy;

short   ptx, pty;

short   pcolour;

short   pdrawm;

short   vpnumber;

};

The driver should draw a vector from (pfx,pfy) to (ptx,pty).  For higher per-

formance, a driver should also implement fast vector mode; see "Fast Vector

Mode" on page 77 for details.

If EF_BS is set in pefmodes at PINIT (1) and vpnumber is 0, then pfx, pfy, ptx,

and pty are LLG viewport-independent coordinates.  If vpnumber is nonzero, then

these are viewport-relative logical coordinates.  If vpnumber is neither a

master nor a slave viewport, the driver should scale, clip and draw the vector

in the indicated viewport.  If vpnumber is a slave viewport, these vectors are

from the master viewport's logical space.  They should be scaled and clipped

for the slave viewport and drawn only in the slave viewport.  Drawing vectors

directed to a master viewport (in drivers which don't implement FASTDRAW)

should be drawn in both the master and its slave viewports, with appropriate

scaling and clipping for each.

Nondrawing vectors should only be drawn in view-

port vpnumber and should never be copied from

master to slave!

42

(3) ADI Display / Packet Mode

If your driver implements FASTDRAW, all drawing vectors will normally arrive

through FASTDRAW, and PVEC will get nondrawing vectors.  If the driver sets the

NO_BS bit in PVIEWPORT (40), then drawing vectors for that viewport will arrive

here (in pfx, pfy, ptx, and pty) instead of FASTDRAW; these vectors will be

expressed in vpnumber 0, LLG viewport-independent coordinates.

It is not unusual for some graphics controllers to draw a different set of

pixels depending on the direction the vector is drawn.  If garbage is left on

the screen after erasing a polyline or solid, you might try sorting the coordi-

nates before drawing the vector.

If the driver supports the Advanced User Interface (AUI), pcolour may be less

than -1, indicating a logical color number.  In this case, the driver should

substitute the appropriate physical color corresponding to the requested logi-

cal color.  If the (possibly adjusted) pcolour is neither zero nor -1, that

color should be used to draw the vector.  Refer to Appendices A and B for

information on color numbers.

If pcolour is zero, the vector should be erased (preserving the background

color if it is something other than black).  If pcolour is -1, the vector

should be XORed with the background color in all image planes.  pdrawm is a

bit-coded field in which the bits have the following meaning:

Mnemonic   Bit  Value                  Meaning

DR_HILITE   0     1    0 = do not use highlighting

1 = draw using highlighting

DR_NORDRW   1     2    0 = vector is part of drawing entity

1 = vector is not part of drawing entity

If the driver implements highlighting, pcolour is greater than zero, and

DR_HILITE (pdrawm bit 0) is set, the vector should be drawn highlighted (pref-

erably in a dashed-line font, but the driver may use other forms of highlight-

ing if they are more effective).  Otherwise, a solid vector should be drawn.

If the driver supports the Advanced User Interface

(EF_NEWI was set in the pefmodes field of the PINIT

(1) reply packet), it must be capable of drawing a

highlighted line in either normal drawing (pcolour >

0) or XOR mode (pcolour = -1).

DR_NORDRW (pdrawm bit 1) marks vectors that are not part of any drawing entity,

but are simply screen artifacts (for example, the two lines forming a marker

blip). This is provided for "big screen" drivers that use the PREDRAW (24)

request. If DR_NORDRW is set, the vector should not be redrawn by the next

PREDRAW request.  Drivers that do not use the PREDRAW request can ignore

DR_NORDRW.

43

Autodesk Device Interface/ADI

--------------------  PFILL (6)  --------------------

The PFILL request is issued only if the phwfill field is returned as a nonzero

value in the PINIT (1) reply packet.  If a big screen driver does not do its

own fills, AutoCAD sends vectors for filled shapes through the normal drawing

vector facilty (PVEC or FASTDRAW); if NO_VEC is set for that viewport, no vec-

tors are sent.

Fills performed by AutoCAD for a big screen

driver will be very slow, since many logical

fill vectors map to a single displayed vector.

One work-around is to suggest that users turn

FILL off in AutoCAD.  Implementing PFILL is the

best solution.

struct pkfille {

short   pfunc;

short   pcolour;

short   pdrawm;

short   pnvert;

short   pvert[12][2];

short   vpnumber;

};

vpnumber is included only if the driver returned a value of 3 in the AX regis-

ter in response to INIT 1.

If EF_BS in pefmodes is set at PINIT (1) the vertex coordinates passed in

pvert[][] will be viewport-relative logical coordinates. If EF_BS was not set

vpnumber should be ignored.

If the phwfill field is assigned the value 2 (1 is not used), PFILL instructs

the driver to draw a filled polygon.

If the driver supports the Advanced User Interface, pcolour may be less than

-1, indicating a logical color number.  In this case, the driver should substi-

tute the appropriate physical color corresponding to the requested logical

color.  If the (possibly adjusted) pcolour is neither zero nor -1, that color

should be used to draw the filled area.  Refer to Appendices A and B for infor-

mation on color numbers.

If pcolour is zero, the area should be erased (preserving the background color

if it is something other than black).

If the driver implements highlighting, pcolour is greater than zero, and

DR_HILITE (pdrawm bit 0) is set, the filled area should be highlighted (usually

by highlighting every fifth line of the fill).

44

(3) ADI Display / Packet Mode

The polygon to be drawn is made up of pnvert vertices, which are defined in a

two-dimensional table, pvert, containing 12 columns representing up to 12 ver-

tices, and two rows representing X and Y coordinates.  Valid column subscript

values must be within the range 0 to pnvert-1.  Valid row subscripts are 0 for

X coordinates, and 1 for Y coordinates. Although 12 vertices can be

represented, AutoCAD will never pass a polygon with more than 10 vertices to

PFILL.

45

Autodesk Device Interface/ADI

--------------------  PMARK (7)  --------------------

PMARK instructs the driver to draw a cursor of type pcursel at screen coordi-

nates (px,py).

struct pkmark {

short   pfunc;

short   pstat;

short   px, py;

short   pcursel;

short   vpnumber;

};

For all values of pcursel other than 0 or 3, the driver should simply return a

value of 0 in pstat, and the Autodesk product will draw the cursor itself.

pcursel values of 0 and 3 specify simple orthogonal crosshairs, and the display

driver may choose to draw these with optimized code.  Autodesk products

normally draw a full-screen crosshair, but the driver can draw it smaller (or

allow the size to be configured by the user) if desired.

If the driver set EF_BS in pefmodes at PINIT (1) the coordinates passed in px,

py will be viewport-independent pixel coordinates.  It is the driver's respon-

sibility to clip the cursor to the specified viewport.

If the driver draws the cursor, it should return a value of 1 in pstat.  Other-

wise, it should return with pstat set to zero, instructing the Autodesk product

to draw the cursor itself.

--------------------  PCMARK (8)  --------------------

PCMARK instructs the driver to clear a cursor of type pcursel (see the PMARK

(7) request above) at screen coordinates (px,py).

struct pkmark {

short   pfunc;

short   pstat;

short   px, py;

short   pcursel;

short   vpnumber;

};

For all values of pcursel other than 0 or 3, the driver should simply return

with pstat set to zero.

If the driver clears the cursor, it should return a value of 1 in pstat.  Oth-

erwise, it should return with pstat set to zero, instructing the Autodesk prod-

uct to clear the cursor itself.

46

(3) ADI Display / Packet Mode

If the driver set EF_BS in pefmodes at PINIT (1) the coordinates passed in px,

py will be viewport-independent pixel coordinates. It is the driver's responsi-

bility to clip the cursor to the specified viewport.

47

Autodesk Device Interface/ADI

--------------------  PCHAR (9)  --------------------

PCHAR instructs the driver to draw the character specified by pchar on the

graphics screen at the current text position.  The current text position should

then be moved one character to the right.

struct pkchar {

short   pfunc;

short   pchar;

};

A sequence of PCHAR requests is always terminated by an PECHAR (10) request, so

drivers can buffer the characters and process them when the PECHAR request is

received if they so choose.

See requests below that set the current text position.

--------------------  PECHAR (10)  --------------------

PECHAR indicates the end of a string of characters sent by a sequence of PCHAR

(9) requests.

struct pkfonly {

short   pfunc;

};

--------------------  PHLITE (11)  --------------------

PHLITE instructs the driver to highlight the menu box specified by the current

text position.

struct pkhlite {

short   pfunc;

short   pstat;

short   pnchars;

};

pnchars is the number of characters in the menu box.

If the driver can highlight the box without knowing the characters within it,

it should return pstat equal to 0.  Otherwise, it should return pstat equal to

1 and the Autodesk product will send the contents of the box to the driver with

pnchars PCHAR (9) requests, followed by a PECHAR (10) request.

48

(3) ADI Display / Packet Mode

--------------------  PDHLITE (12)  --------------------

PDHLITE instructs the driver to dehighlight the menu box specified by the cur-

rent text position.

struct pkhlite {

short   pfunc;

short   pstat;

short   pnchars;

};

pnchars is the number of characters in the menu box.

If the driver can dehighlight the box without knowing the characters within it,

it should return pstat equal to 0.  Otherwise, it should return pstat equal to

1 and the Autodesk product will send the contents of the box to the driver with

pnchars PCHAR (9) requests, terminated by a PECHAR (10) request.

--------------------  PMNUCURS (13)  --------------------

PMNUCURS instructs the driver to set the current text cursor to screen menu box

pboxno (0 is the top box).

struct pkmnucur {

short   pfunc;

short   pboxno;

short   pnboxes;

};

There are pnboxes on the screen.  This request may be followed by a sequence of

PCHAR (9) requests to write into the box, or by a PHLITE (11) or a PDHLITE (12)

request to highlight or dehighlight the box.

--------------------  PTPROMPT (14)  --------------------

This request is issued only on dual-screen displays, indicated by DM_GROK (bit

2 of the pmodes field) cleared in the PINIT (1) reply packet.

struct pkfonly {

short  pfunc;

};

PTPROMPT instructs the driver to set the current text cursor to the prompt line

on the screen.  This request will be followed by a sequence of PCHAR (9)

requests to write to the prompt line.  Text output to the prompt line will be

terminated by a final PECHAR (10) request.

49

Autodesk Device Interface/ADI

--------------------  PQPLOT (15)  --------------------

PQPLOT instructs the driver to perform a screen dump (all viewports) to the

printer. If the driver performs the screen dump, it should return pstat equal

to 1.  Otherwise, the driver should do nothing, and return pstat equal to 0.

This call is initiated by AutoCAD's QPLOT command.

struct pkqplot {

short   pfunc;

short   pstat;

};

--------------------  PRPEN (16)  --------------------

PRPEN instructs the driver to sense the position of the screen-pointing device

(e.g.  light pen) and return its current position (in terms of pixels on the

graphics screen) in px and py. These coordinates should be returned in

viewport-independent coordinates.

struct pkrpen {

short   pfunc;

short   pstat;

short   px, py;

};

The driver should return pstat, px, and py as indicated in the following table.

Response     pstat     px        py

No response       0

Tracking point    2     X-coord  Y-coord

Picked point      3     X-coord  Y-coord

Button pick       4    Button #

In AutoCAD, button picks returned by PRPEN (pstat = 4) access the "AUX1" menu

and can be used to support a multi-button function box that is not necessarily

associated with the light pen.  Button code 0 references the first item in the

"AUX1" menu. If the "AUX1" portion of the menu file contains items that submit

individual characters (as in "A\"), the driver can use them in sequence to

submit any command sequence it needs.

50

(3) ADI Display / Packet Mode

--------------------  PGOTEXT (17)  --------------------

The PGOTEXT request is issued only on single-screen systems, indicated by

DM_GROK (bit 2) set in the pmodes field of the PINIT (1) reply packet.

struct pkfonly {

short   pfunc;

};

PGOTEXT instructs the driver to switch to the text screen and clear it.  The

text screen cursor should be left at the top of the screen.  Subsequent to this

call, the Autodesk product will use regular DOS I/O to write to the text

screen.

--------------------  PGOTEXTU (18)  --------------------

The PGOTEXTU request is issued only on single-screen systems, indicated by

DM_GROK (bit 2) set in the pmodes field of the PINIT (1) reply packet.

struct pkfonly {

short   pfunc;

};

PGOTEXTU instructs the driver to switch to the text screen, displaying hidden

screen information.

If the driver maintains its own hidden screen (indicated by returning both

phidlines and phidcols equal to 0 in the PINIT (1) reply packet), it should

switch to the text screen, display the hidden screen information, and restore

the cursor to its previous position on the text screen.

If the hidden screen is maintained by the Autodesk product (indicated by

nonzero values returned in phidlines and phidcols in the PINIT (1) reply

packet), the driver should switch to the text screen, clear it, and leave the

cursor at the top.  The Autodesk product will automatically rewrite the hidden

screen.

Subsequent to this call, the Autodesk product will use regular DOS I/O to write

to the text screen.

51

Autodesk Device Interface/ADI

--------------------  PGOGRAPH (19)  --------------------

The PGOGRAPH request is issued only on single-screen systems, indicated by

DM_GROK (bit 2) set in the pmodes field of the PINIT (1) reply packet.

struct pkgograph {

short   pfunc;

short   prepaint;

};

PGOGRAPH instructs the driver to switch to the graphics screen.  Ideally, this

operation should be nondestructive, displaying the picture, status line, and

screen menu area exactly as they were when the display was switched to the text

screen.  The text scrolling area should reflect the last few lines of the text

screen if possible.

prepaint is a bit-coded field identifying the screen areas, if any, that should

be repainted by requests from the Autodesk product following the PGOGRAPH

request.

If the driver can switch to the graphics screen nondestructively, it should

return prepaint equal to 0.  Otherwise, the following bits in prepaint should

be set, depending on which areas of the screen must be redrawn:

Mnemonic   Bit  Value                    Meaning

RP_STAT     0     1    set if status line should be redrawn

RP_MENU     1     2    set if screen menu area should be redrawn

RP_SCROLL   2     4    set if text scrolling area should be redrawn

RP_GRAPH    3     8    set if graphics area should be redrawn

If RP_GRAPH (prepaint bit 3) is set, requiring the Autodesk product to redraw

the picture, the redraw process will issue the PCLEAR (3) request.  If the

PCLEAR request in turn uses prepaint to restore the screen menu area, text

scrolling area, or status line, there is no need to request restoration of

these screen areas in the prepaint field returned by the PGOGRAPH request.

Note that the Autodesk product cannot redraw the text scrolling area if the

driver maintains its own hidden text screen (as indicated by zero values for

phidlines and phidcols returned by the PINIT (1) request).

If the driver draws borders (or other information not known to the Autodesk

product) that are destroyed by switching to the text screen and back to the

graphics screen, it is the responsibility of PGOGRAPH to replace this informa-

tion.

52

(3) ADI Display / Packet Mode

--------------------  PWRSPLIT (20)  --------------------

PWRSPLIT instructs the driver to write the character specified by pchar to the

text scrolling area of the graphics screen at the current text scrolling area

position.  Following the PWRSPLIT request, the current text scrolling area

position should be moved one character to the right.  The driver should main-

tain a current text scrolling area position distinct from the text position

used by other requests.

struct pkwrsplit {

short   pfunc;

short   pchar;

};

If the driver maintains its own hidden screen (indicated by returning both

phidlines and phidcols equal to 0 in response to the PINIT (1) request), pchar

should also be saved on the hidden screen.

If the hidden screen is maintained by the Autodesk product (indicated by

nonzero values returned in phidlines and phidcols in response to the PINIT (1)

request), the driver need only draw the character on the graphics screen.  The

Autodesk product will take care of saving it.

--------------------  PMODELINE (21)  --------------------

PMODELINE instructs the driver to set the current text position to the start of

the status line.

struct pkmodeline {

short   pfunc;

short   pnchars;

short   pcolour;

};

The string to be displayed is written to the status line by pnchars PCHAR (9)

requests followed by a PECHAR (10) request.

The current drawing color (the color in which new entities will appear, however

derived) is passed to the driver in pcolour so that the driver can, if it

chooses, represent this on the status line.

--------------------  PCOORDLINE (22)  --------------------

PCOORDLINE instructs the driver to set the current text position to the start

of the coordinate-status line, usually pmodelinl+1 (the value of pmodelinl is

returned in the PINIT (1) reply packet).

53

Autodesk Device Interface/ADI

struct pkcoordline {

short   pfunc;

short   pnchars;

};

The string to be displayed is written to the coordinate-status line by pnchars

PCHAR (9) requests followed by a PECHAR (10) request.

54

(3) ADI Display / Packet Mode

--------------------  PSYNC (23)  --------------------

The PSYNC request is issued only if CF_SYNC was set in the pconfig field of the

PINIT (1) request packet, and the driver set the EF_SYNC bit in the pefmodes

field of PINIT's reply packet.

struct pksync {

short   pfunc;

short   prequest;

short   paperture;

short   vpnumber;

};

For drivers that maintain local display buffers (vector lists), the PSYNC

request forces the local display buffers (queued vectors) to be drawn, bringing

the picture into synchronization with the Autodesk product's internal state.

For example, this request is issued before digitizer samples are taken, to

ensure that the entire picture is on the screen for the user to pick from.

prequest is a bit-coded field that lets the ADI driver make special requests of

the Autodesk product.  Currently defined bits are:

Mnemonic   Bit  Value                     Meaning

SY_APERTURE0     1    set to change aperture (and pickbox) size

SY_HILITE  1     2    set to redraw nondrawing vectors for viewport

vpnumber

SY_PROMPT  2     4    set to reissue current prompt

AutoCAD supplies the current size (in pixels) of the object snap aperture in

the paperture field.  If the driver wishes to change the aperture size, it

should store the new size in paperture and set the SY_APERTURE bit in the pre-

quest field before returning.  AutoCAD will then set the object snap aperture

to the specified value, and will also set the object selection pick box to one

third of the aperture size. The aperture setting is stored in AutoCAD's config-

uration file, so it is normal for an aperture change to cause a disk access. In

the future, the aperture size may be used to govern the size of marker blips

and other drawing features as well.

The SY_HILITE bit in prequest, if set by the driver, instructs the Autodesk

product to redraw any vectors that were not part of the drawing proper, such as

highlighting.  Marker blips are never redrawn, however. If EF_BS in pefmodes is

set at PINIT (1) and SY_HILITE is requested, the driver can return a legal

viewport number in vpnumber to request fresh non-drawing vectors for that view-

port. Returning 0 in vpnumber references all viewports.

The driver can set the SY_PROMPT bit in prequest to instruct the Autodesk prod-

uct to reissue the current text prompt.

55

Autodesk Device Interface/ADI

--------------------  PREDRAW (24)  --------------------

This request is issued only if EF_REDRAW (bit 1) is set in the pefmodes field

of the PINIT (1) reply packet. (Post- ADI 3.1 drivers that set EF_REDRAW should

also set EF_BS and EF_MV, and must support the new, "big screen" features of

ADI Version 4.0.)

struct pkredraw {

short   pfunc;

short   pstat;

short   pflush;

short   vpnumber;

};

When a redraw of the picture is necessary, the Autodesk product will issue the

PREDRAW request with pflush set to 0, instructing the driver to redraw all vec-

tors.  The Autodesk product will still issue a PCLEAR (3) request prior to the

PREDRAW request, and will draw grid dots, etc.  afterwards.

If EF_BS was set in pefmodes at PINIT (1) and vpnumber is zero, the driver

should redraw the entire graphics area.  If vpnumber is nonzero then the driver

should redraw the specified viewport.

If the driver is able to complete the redraw, it should set pstat to 1.  Set-

ting pstat to 0 indicates that the driver needs a fresh display list.  In this

case, AutoCAD's action depends on the value you pass in vpnumber.  If vpnumber

was passed as 0, the driver should return it as 0, and the entire graphics

screen will be redrawn.  If vpnumber was nonzero, the driver can either return

the same value and get just the current viewport redrawn or it can change

vpnumber to 0 to request a redraw of the entire graphics area.  The driver

should not set vpnumber to any other value!

When the current contents of any display buffer are no longer valid, the

Autodesk product will issue the PREDRAW request with pflush set to 1 to indi-

cate that the vector space should be cleared.  In this case, no return status

is necessary.

56

(3) ADI Display / Packet Mode

--------------------  PCOMMAND (25)  --------------------

The PCOMMAND request is issued only if EF_CMD (bit 2) is set in the pefmodes

field of the PINIT (1) reply packet.

struct pkcommand {

short          pfunc;

short          pstat;

short          pstrlen;

unsigned short poffset, pseg;

};

PCOMMAND allows the driver to examine all input commands that are not recog-

nized by AutoCAD.

The command string location is passed as a SEGMENT:OFFSET pair in pseg and

poffset, respectively, with the string length in pstrlen.

The command string is terminated by a null character in the usual C fashion.

If the driver returns 0 in pstat, AutoCAD proceeds with its unknown-command

handling.  Otherwise, AutoCAD assumes that the string has been recognized and

processed by the driver, and behaves as if it were any valid AutoCAD command

(i.e., a null line will repeat the command).

If EF_CMD in pefmodes is set, the driver will also receive unrecognized "trans-

parent" commands (those prefaced by " ' ", such as 'ZOOM) in addition to unrec-

ognized regular commands.  The strings passed from such commands are "raw";

that is, no case conversion takes place and the leading " ' " will still be

present.  Transparent commands will be sent to the driver only from Release 10

AutoCAD (not from earlier versions).

--------------------  PXPCOMD (26)  --------------------

The PXPCOMD request is issued only if EF_XPCMD (bit 3) is set in the pefmodes

field of the PINIT (1) reply packet.

struct pkxpcomd {

short   pfunc;

short   pstat;

short   pchar;

};

PXPCOMD notifies the driver when a special transparent display-driver command

character is entered.  Currently, there is only one such character: CTRL-L (0C

hexadecimal).

The command character is passed to the driver in pchar.

57

Autodesk Device Interface/ADI

If the driver returns 0 in pstat, AutoCAD proceeds with its unknown-character

handling.  Otherwise, AutoCAD assumes that the character has been recognized

and processed by the driver.

The transparent control character may be entered at any time by the user, and

will be immediately passed to the driver.  This is the mechanism normally used

to invoke such functions as hardware pan and zoom.

58

(3) ADI Display / Packet Mode

--------------------  PBOXCLR (27)  --------------------

The PBOXCLR request is issued only if the Advanced User Interface (AUI) has

been enabled by setting EF_NEWI (bit 4) in the pefmodes field of the PINIT (1)

reply packet.

struct pkboxclr {

short   pfunc;

short   pxl, pyl, pxh, pyh;

short   pcolour;

};

PBOXCLR instructs the driver to clear, to the color specified by pcolour, the

rectangle inclusively delimited by the points (pxl,pyl) and (pxh,pyh).

--------------------  PBOXSIZE (28)  --------------------

To support the Advanced User Interface, the driver must be capable of saving a

screen rectangle and subsequently restoring it (see the PBOXPUSH (29) and

PBOXPOP (30) requests below).  If necessitated by space requirements, the rect-

angle can be broken up into horizontal strips.  Each such strip will be the

full width of the rectangle and one or more pixels high.  Screen data may be

saved in one of the following places:

1. in local memory provided by the display adapter;

2. in memory obtained from AutoCAD; or

3. in a combination of (1) and (2), depending on the amount of screen

data to be saved

The PBOXSIZE request is issued only if the Advanced User Interface has been

enabled (by setting EF_NEWI (bit 4) in the pefmodes field of the PINIT (1)

reply packet), and the driver requires AutoCAD to provide memory for screen-

save buffers (indicated by setting EF_SAVEB (bit 5) in the pefmodes field of

the PINIT reply packet).

struct pkboxsize {

short    pfunc;

short    pxl, pyl, pxh, pyh;

long     pbsize;

};

When the driver receives a PBOXSIZE request, it should calculate the amount of

memory (in bytes) required to save the screen rectangle inclusively delimited

by the points (pxl,pyl) and (pxh,pyh), plus any control storage the driver

needs, and return the result in pbsize.

Also see the PBOXPUSH (29) and PBOXPOP (30) requests below.

59

Autodesk Device Interface/ADI

Many drivers have some space available in otherwise unused portions of the dis-

play adapter's memory.  This local memory should be exhausted before requesting

memory from AutoCAD.

If enough local memory is available to save a horizontal strip (at least one

pixel high) all the way across the screen rectangle, then the driver should

prepare to use this local memory, and return pbsize equal to the total amount

of memory required, less the amount of local memory available.

If pbsize is returned as a nonzero value, AutoCAD will attempt to allocate the

requested amount of memory.  Presently, the maximum amount of memory AutoCAD

will allocate in a single request is 65535 bytes.  Drivers should not count on

being able to obtain all of the requested memory in one block, since AutoCAD

may allocate a buffer smaller than the requested size.  In this case, the

driver will be expected to save the largest horizontal strip of the screen

rectangle that will fit in the buffer, and AutoCAD will continue to supply

buffers to hold the rest of the screen rectangle.

The address of the allocated buffer is passed to the driver by a subsequent

BOXPUSH or BOXPOP request.

If AutoCAD is requested to provide a buffer for a subsequent BOXPUSH request,

and is unable to obtain even a minimum size buffer (2K), then the BOXPUSH

request to save the screen rectangle will not be issued, and the area of the

screen will be lost.  This, however, should happen only very rarely.

--------------------  PBOXPUSH (29)  --------------------

The PBOXPUSH request is issued only if the Advanced User Interface has been

enabled by setting EF_NEWI (bit 4) in the pefmodes field of the PINIT (1) reply

packet.

struct pkboxsave {

short          pfunc;

short          pxl, pyl, pxh, pyh;

unsigned short pbufoffs, pbufseg;

long           pbufl;

short          pheight;

long           plocate;

};

PBOXPUSH instructs the driver to save the contents of the screen rectangle

inclusively delimited by the points (pxl,pyl) and (pxh,pyh).

If AutoCAD provides memory for the saved image (indicated by bit 5 (EF_SAVEB)

set in the PINIT reply packet and a nonzero pbsize in the PBOXSIZE (28) reply

packet), the address of the buffer allocated by AutoCAD is passed to the driver

as a SEGMENT:OFFSET pair in pbufseg and pbufoffs, respectively.

The size of the buffer (in bytes) is passed in pbufl.  This size of this buffer

may be smaller than the requested size, but will be at least 2K.

60

(3) ADI Display / Packet Mode

If the driver provides its own screen-save buffer, then pbufseg, pbufoffs, and

pbufl will be 0.

Upon return, pheight must be set to the number of pixel rows the driver was

able to save in the space available.  If the entire rectangle was saved, this

would be pyh - pyl + 1.

The return value plocate is for the driver's convenience, and may be used to

identify where in local memory the screen rectangle has been saved.  The value

assigned to plocate will be passed as an argument to the matching PBOXPOP (30)

request.

61

Autodesk Device Interface/ADI

--------------------  PBOXPOP (30)  --------------------

The PBOXPOP request is issued only if the Advanced User Interface has been

enabled by setting EF_NEWI (bit 4) in the pefmodes field of the PINIT (1) reply

packet.

struct pkboxsave {

short          pfunc;

short          pxl, pyl, pxl, pyh;

unsigned short pbufoffs, pbufseg;

long           pbufl;

short          pheight;

long           plocate;

};

Note that PBOXPOP uses the same packet format as the PBOXPUSH (29) request.

PBOXPOP instructs the driver to restore the screen rectangle inclusively

defined by the points (pxl,pyl) and (pxh,pyh).  This will always be the screen

rectangle most recently saved with PBOXPUSH.

If AutoCAD provides memory for the saved image (indicated by (EF_SAVEB (bit 5)

set in the pefmodes field of the PINIT (1) reply packet and a nonzero pbsize in

the PBOXSIZE (28) reply packet), the address of the buffer allocated by AutoCAD

is passed to the driver as a SEGMENT:OFFSET pair in pbufseg and pbufoffs,

respectively.

The size of the buffer (in bytes) is passed in pbufl.  If the driver provides

its own screen-save buffer, then pbufseg, pbufoffs, and pbufl will be 0.

The value of the plocate field in the PBOXPOP packet is supplied by AutoCAD,

and is equal to the value of the plocate field returned to AutoCAD in the reply

packet of the matching PBOXPUSH (29) request.  This parameter is for the

driver's convenience and may be used to identify where in local memory the

screen rectangle has been saved, or for any other purpose the driver requires.

The value of the pyh field is always equal to (pyl+pheight-1) for the PBOXPOP

request.

In single-screen systems, everything that has been pushed will be popped prior

to a request to flip to AutoCAD's text screen or to AutoShade's rendering

screen.

62

(3) ADI Display / Packet Mode

--------------------  PTEXT (31)  --------------------

PTEXT is used to write raster text on the screen.  The first character should

be drawn in the character cell specified by (px,py).  Successive characters

should be drawn at ascending X locations.  The PTEXT request is issued only if

EF_NEWI (bit 4) is set in the pefmodes field of the PINIT (1) reply packet.

struct pktext {

short          pfunc;

short          px, py;

short          pattr;

short          ptlen;

unsigned short poffset, pseg;

short          pbgcolor;

short          pfgcolor;

};

A maximum of one line of text will be written by a single PTEXT request.  The

text should be displayed with the attributes specified by the pattr field.

pattr is a bit-coded field in which the bits have the following meanings:

Mnemonic  Bit  Value             Meaning

TANORM    all    0    Normal text (all bits clear)

TAINV      0     1    Reverse video text

TAGREY     1     2    Disabled text

TANORM specifies normal text, whose background is the same color as the back-

ground of the screen and whose intensity is normal.

TAINV causes text to be written in reverse video, with the background and fore-

ground reversed or otherwise highlighted at the driver's discretion.

TAGREY signifies that the text denotes a disabled function.  Such text is gen-

erally "greyed out" by forcing every other pixel in a checkerboard pattern to

the background color (but a driver is free to use other visual cues if they

work better).  The driver must be able to handle TAGREY in combination with

TANORM either set or cleared.  See "ADI Graphics Display Test Procedures" on

page 83 for tips on testing greyed-out text.

The text string to be displayed is passed as a SEGMENT:OFFSET pair in pseg and

poffset, respectively.

The actual number of characters written to the screen is determined in one of

two ways, depending on the value contained in the ptlen field.  If ptlen is

zero, the string is assumed to be zero-terminated in normal C fashion.  In this

case, the driver should output the string up to but not including the terminat-

ing character.  Otherwise, exactly ptlen characters should be written to the

screen.

If the string contains less than ptlen characters, blank characters must be

appended to the string.  If the string contains more than ptlen characters, the

string must be truncated.

63

Autodesk Device Interface/ADI

Character codes comprising the text string will, for the U.S.  domestic version

of the Autodesk product, always be in the inclusive range from 32 to 128, with

128 signifying a check mark.  Editions of Autodesk products in languages other

than English may use additional character codes beyond 128, and may use a dif-

ferent code for the check mark character.

If it is impossible for the driver to display a check mark, a capital X is an

acceptable substitute.

pbgcolor and pfgcolor are the colors (usually logical colors) used to draw the

text.  The entire character cell should be filled with pixels of one of the two

colors.

--------------------  PLINE (32)  --------------------

The PLINE request is issued only if the Advanced User Interface is enabled,

indicated by EF_NEWI (bit 4) set in the pefmodes field of the PINIT (1) reply

packet.  Note that this request has nothing at all to do with AutoCAD's PLINE

command.

struct pkvec {

short   pfunc;

short   pfx, pfy;

short   ptx, pty;

short   pcolour;

short   pdrawm;

};

PLINE instructs the driver to draw a vector from (pfx,pfy) to (ptx,pty) in the

color specified by pcolour.  The coordinates passed to the driver by this

request are in the natural device coordinate system with (0,0) representing the

upper left corner of the display.

If pcolour is -1, the vector should be XORed with the background color in all

image planes.

A vector drawn with the PLINE request will never be highlighted and will never

be part of a drawing entity, and therefore should never be saved in a local

display list.

For compatibility with the PVEC (5) request, the following bits are set in the

pdrawm field:

Mnemonic   Bit  Value                   Meaning

DR_HILITE   0     1    0 = do not use highlighting

1 = use highlighting

DR_NORDRW   1     2    0 = vector is part of a drawing entity

1 = vector is not part of a drawing entity

64

(3) ADI Display / Packet Mode

--------------------  PDCURS (33)  --------------------

PDCURS instructs the driver to draw a raster cursor as described by the rcursor

structure pointed to by the SEGMENT:OFFSET pair pcseg and pcoff.

The PDCURS request is issued only if the driver set the EF_CURS bit in the pef-

modes field of the PINIT (1) reply packet.  If an ADI driver is to work with

AutoSketch or AutoShade, it must support raster cursor requests.

struct rcursor {

short          curx, cury;

unsigned short curand[16], curxor[16];

};

struct pkdcurs {

short          pfunc;

short          px, py;

unsigned short pcoff, pcseg;

};

The cursor to be drawn is described in the rcursor structure by two 16 by 16

bit masks.  The first mask, curand, is logically ANDed with the contents of the

screen.  The second mask, curxor, is then XORed with the screen contents.

Two fields in the rcursor structure, curx and cury, specify the bit and word

number, respectively, of the bit in the cursor that overlays the screen coordi-

nates given by (px,py) in the PDCURS request packet (this is often called the

cursor's hot spot).

If curx and cury are both zero, the upper left-most point of the cursor will

align with the screen coordinates.  If curx and cury are both 8, the cursor

will be centered around the screen point.

px and py specify the natural screen coordinates over which the cursor align-

ment point should be positioned. These coordinates are always viewport indepen-

dent.

px and py can be any valid screen coordinates,

and it is the responsibility of the driver to

properly clip the cursor at the screen edges.

On a display with multiple bit planes, the AND

and XOR operations should be performed in all

bit planes.

The original screen area overlaid by the cursor should be saved in a buffer so

that it may be restored by the PCCURS request.

65

Autodesk Device Interface/ADI

--------------------  PCCURS (34)  --------------------

PCCURS instructs the driver to restore the screen contents saved by the most

recent PDCURS (33) request.

The PCCURS request is issued only if the driver set the EF_CURS bit in the pef-

modes field of the PINIT (1) reply packet.  If an ADI driver is to work with

AutoSketch or AutoShade, it must support raster cursor requests.

struct pkfonly {

short   pfunc;

};

PDCURS and PCCURS requests are always made in pairs and are never nested, so

only one save buffer is required within the driver.

--------------------  PBSHELL (35)  --------------------

The PBSHELL request is issued when a SHELL command is about to begin, if

EF_SHELL (bit 8) was set in the pefmodes field of the PINIT (1) reply packet to

let the Autodesk product know that the driver wants to be informed of SHELL

commands.

struct pkfonly {

short   pfunc;

};

The PBSHELL request is issued to protect driver page mappings when a SHELL com-

mand is used.

If the driver is to use LIM EMS expanded memory, it should request it before

terminating and remaining resident (TSR), mapping 4 logical->physical pages.

AutoCAD will then take the rest of expanded memory when it starts.  AutoCAD

will save and restore the driver's page maps, but the SHELL command lets other

programs trash them, so the PBSHELL notification is issued to let drivers pro-

tect their page maps.

After PBSHELL, the driver should not use expanded memory until a PASHELL (36)

is issued.

--------------------  PASHELL (36)  --------------------

The PASHELL request is issued when a SHELL command is complete, if EF_SHELL

(bit 8) was set in the pefmodes field of the PINIT (1) reply packet to let the

Autodesk product know that the driver wants to be informed of SHELL commands.

66

(3) ADI Display / Packet Mode

struct pkfonly {

short   pfunc;

};

If the driver is to use LIM EMS expanded memory, it should request it before

terminating and remaining resident (TSR), mapping 4 logical->physical pages.

AutoCAD will then take the rest of expanded memory when it starts.  AutoCAD

will save and restore drivers' page maps, but the SHELL command lets other pro-

grams trash them, so BSHELL is issued to protect drivers' page maps.  PASHELL

is issued when the SHELL command is complete so that driver may again use

expanded memory.

67

Autodesk Device Interface/ADI

--------------------  PDIGTIZE (37)  --------------------

The PDIGTIZE request is issued whenever a digitizer sample is obtained from the

configured digitizer, and permits the display driver to intercept and/or alter

digitizer samples.  This request is only issued if the CF_DIGTIZE bit was set

in the pconfig field of the PINIT (1) request packet, and the driver sets the

EF_DIGTIZE bit in the pefmodes field of the reply.  Currently, only AutoCAD

supports this request

struct pkrpen {

short   pfunc;

short   pstat;

short   px, py;

};

When a digitizer sample has been passed with this request, the ADI driver can

take several actions.  It can do nothing, change the sample, request another

sample, or request that some digitizer coordinates be mapped to screen coordi-

nates.  Finally, the filtered coordinates are returned to the Autodesk product.

The interface provided by this request is similar to an ADI digitizer driver

SENSE function (although some of the encodings vary slightly).  Each PDIGTIZE

request passes in a status, an X and a Y.  The ADI driver returns the same

fields (possibly having altered the values).  Except for packets with a pstat

value of 1, the px and py values are in digitizer space with 0,0 being the

bottom left corner of the digitizer and 20480,20480 being the upper right

corner.  Note that not all of the digitizer surface is necessarily part of the

screen pointing area and that some digitizers may (inappropriately) return

coordinates all the way up to 32767.

Possible pstat values sent to the ADI driver are:

Value                         Meaning

0       No sample available (px and py are meaningless).

1       px and py are the requested screen equivalents

of the px and py values most recently passed

back from the ADI driver.  (See status 1

returned by the ADI driver, below.) px will be

between 0 and ixdots.  py will be between 0 and

ymenumax or iydots (whichever is greater).  This

status value is only returned immediately fol-

lowing a previous PDIGTIZE request to which the

driver replied with a status of 1.

2       px and py are a tracking coordinate.

3       px and py are a pick coordinate.

-1, -2, ...  Button number 1, 2, ...  was pressed.  px and py

are the coordinates at which this occurred, or

are both -1 if no coordinates were returned by

the digitizer.

The ADI driver can return any of the preceeding status values (except 1) with

the corresponding px and py coordinates, and the Autodesk product will perform

68

(3) ADI Display / Packet Mode

the appropriate action.  The ADI driver may return a status of 1 or 4 to

request special action.  For these two returned pstat values, the immediately

following packet request will be another PDIGTIZE.  No user keyboard input (not

even CTRL-C) is permitted before the next request.

Value                      Meaning

1    Convert the returned px and py values to screen

coordinates and call the ADI driver immediately

with the converted values of px and py.

4    Do not use the returned px and py values.

Instead, immediately request another digitizer

sample and return it in the next PDIGTIZE

request.

Note that AutoCAD's digitizer interface does not permit tracking coordinates to

be returned while a pick button is depressed, nor does it distinguish a short

button click from a long one.

--------------------  POPENVP (38)  --------------------

POPENVP is issued each time a viewport is opened, if the driver set EF_BS (bit

12) in pefmodes at PINIT (1).

struct pkovp {

short   pfunc;

short   vpnumber;

short   vpxmin, vpymin;

short   vpxmax, vpymax;

short   lgxdots, lgydots;

};

vpxmin, vpymin, vpxmax, and vpymax are the corners of the viewport, expressed

in LLG (ixdots,iydots) pixel coordinates.  lgxdots and lgydots are in pixel

units, expressing a size, not a position.

It is the big screen driver's responsibility to maintain a table of valid

(properly opened) viewports, their numbers and extents.  Upon return, the

driver must set lgxdots and lgydots to the size of the logical screen it main-

tains.  This size must be an integral multiple of the viewport size - 1.

The largest possible value for either lgxdots or lgydots is 0x7FFE or 32766.

The value 0x7FFF or 32767 is reserved for use as a flag.  The driver's logical

space should be as large as possible within these limitations, because you

can't zoom in (without a regen from AutoCAD) past the point where logical and

viewport physical pixels are equal in size.

The initial value passed to the driver in lgxdots, lgydots is the largest pos-

sible logical space for this viewport (computed by the code fragment below).

Most drivers will use these default values.  The driver may have to reduce

these values if it maintains a large bit-map instead of a display list.

69

Autodesk Device Interface/ADI

vpxdots = vpxmax - vpxmin;

vpydots = vpymax - vpymin;

sfac = 32766/max(vpydots, vpxdots);

lxgdots = vpxdots * sfac;

lgydots = vpydots * sfac;

--------------------  PCLOSEVP (39)  --------------------

This function is called to close a viewport, if EF_BS (bit 12) in pefmodes was

set at PINIT (1).

struct pkcvp {

short   pfunc;

short   vpnumber;

};

The driver should discard any display list data associated with the viewport

and remove it from the list of open viewports.

70

(3) ADI Display / Packet Mode

--------------------  PVIEWPORT (40)  --------------------

This function is called if you set EF_BS in pefmodes at PINIT.

struct pkview {

short   pfunc;

short   vpnumber;

short   lgxl, lgyl;

short   lgxh, lgyh;

short   master;

char    vflags;

};

lgxl, lgyl, lgxh, and lgyh are in viewport-relative logical coordinates.

PVIEWPORT indicates what portion of a viewport's logical space will be visible.

It is not a redraw or clear screen request.  It is usually followed either by a

redraw request (an AutoCAD PAN or ZOOM), or by an entirely new set of vectors

(an AutoCAD REGEN).

If the viewport is a subset of an existing viewport's logical space, and you do

not want to establish a separate display list for it, the driver may set the

NO_VEC bit in vflags to tell AutoCAD not to send you drawing vectors for this

viewport.  AutoCAD will continue to send non-drawing vectors and entities such

as the grid.  If NO_VEC is set, the driver should return the number of the

master viewport in master; otherwise it should return vpnumber in master.

If the viewport's logical space is a superset of another viewport for which the

driver is already maintaining a display list, the driver may use the PREVEC

command for the smaller viewport so that you can set NO_VEC for it, while col-

lecting vectors for the master viewport's larger logical space.

If a big screen driver is unable to handle this viewport in big screen mode

(perhaps due to lack of display list memory), it may set the NO_BS bit in

vflags to let AutoCAD know.  The logical size of the viewport will be reduced

to the viewport size and the driver will be passed viewport-independent pixel

coordinates (through PVEC) for display directly on the screen.  (Vectors passed

to the driver for the NO_BS viewport will all be tagged for viewport 0).

Mnemonic  Bit  Value                 Meaning

NO_VEC     0     1    0 = send vectors

1 = don't send vectors

NO_BS      1     2    0 = this is a big screen viewport

1 = this is not a big screen viewport

It is unreasonable to set both NO_VEC and NO_BS at the same time.

71

Autodesk Device Interface/ADI

--------------------  PCONIN (41)  --------------------

struct pkconin {

short   pfunc;

short   conin;

};

If EF_MV in pefmodes is set at PINIT, this function allows the display driver

to submit normal AutoCAD commands by overriding keyboard input.  The return

value (if nonzero) is used as the next keystroke sent to AutoCAD and will be

echoed on screen.

Note that this function is called only when AutoCAD is about to resort to the

keyboard for more input; scripts, AutoLISP input and menu items in progress

will complete before AutoCAD looks to the display driver for more input.

The initial purpose of this function is to make it possible for a big screen

display driver to request transparent pans and zooms, allowing extensions to

AutoCAD's user interface.  A special option ($0) has been added to the ZOOM

command to allow drivers to request a "ZOOM W" with the viewport corners speci-

fied in "reals" (floating-point space).  Thus you can issue a command:

zoom $0 45.3 -2.1e-002 100.54 250

Note that the four corners are xl, yl, xh, and yh, separated by spaces.  A

space at the end of the string is required to cause AutoCAD to execute the com-

mand.

--------------------  PTABCFG (42)  --------------------

This function is called only if you set two flags in pefmodes at PINIT (1):

EF_DIGTIZE and EF_MV.

struct pktconfig {

short          pfunc;

unsigned short llx, lly;

unsigned short urx, ury;

};

llx, lly, urx, and ury are in raw tablet coordinates.

This is called during initialization (sometime after PINIT) and after every

tablet configuration.  It gives you the raw digitizer coordinates of the cor-

ners of the "screen pointing area". These values, used along with PDIGTIZE,

will allow easier implementation of display driver commands that use pointer

input.

72

(3) ADI Display / Packet Mode

--------------------  PDRAG (43)  --------------------

This function is called if the driver set EF_DRAG in pefmodes at PINIT (1).

The action to take depends on the value of dmode.

struct pkdrag {

short   pfunc;

short   pfx, pfy;

short   ptx, pty;

short   ptcolor;

short   dmode;

short   vpnumber;

};

pfx, pfy, ptx, and pty are in viewport-relative logical coordinates.

Fill mode is off during drag sequences; filled

shapes are sent as outlines.

There are several states a viewport may be in with respect to drag vectors

(typically represented by a local flag, dstate).  In addition, if the driver

has used up its local memory, it should set an overflow flag.  (See the sample

display driver dsega4.c for examples.)

dmode  dstate  Overflow     Coordinate Type       Add to drag list

1      0        No              LLG                    Yes

1      1       Yes              LLG                    No

2      2        No     L (viewport relative)           Yes

2      3       Yes               L                     No

4      4        No              LLG           Redrawing moved image

4      6        No               L            Redrawing moved image

dmode = 0Add the vector to the drag list for the specified viewport and

display it (even if the display list has overflowed).  The driver

must remember information sent to it by dmode 1 or 2 to know if

vectors sent for this viewport are pixel or logical, and handle

them accordingly.

dmode = 1Initialization call.  The driver should forget any existing drag

list for the viewport in vpnumber.  Future vectors sent with

dmode 0 for this viewport will be absolute pixel vectors.  This

call precedes the vector list associated with AutoCAD operations

such as ROTATE where a simple displacement of the drag list is

not useful.  The next movement in a drag sequence will instruct

the driver to erase the drag image.  It is not necessary to scale

or clip any vectors sent with a viewport initialized with dmode =

1.

73

Autodesk Device Interface/ADI

dmode = 2Initialization call.  The driver should forget any existing drag

list for the viewport in vpnumber.  Future vectors sent through

dmode 0 for this viewport will be logical vectors.  This call

precedes the vector list associated with AutoCAD operations such

as MOVE, where a simple displacement of the drag list is useful.

The next movement in a drag sequence will probably instruct the

driver to erase and redraw the drag image after applying a dis-

placement to it (this requires the driver to scale and clip the

vectors to the current viewport).

dmode = 3Instructs the driver to erase anything already drawn from the

drag list.  If the driver buffers vectors and hasn't drawn all of

them, it should erase the ones already drawn and flush the rest

from the buffer.  If this function is called when a movable drag

image is partly redrawn in its new position (because of fresh

user input), the driver should erase what has been drawn and stop

that redraw sequence (you'll probably soon get a fresh dmode = 4

call).  If the driver is able to erase any drag vectors already

drawn, it should return dmode = 1 to signal success.  If the

driver cannot do the erase (because the drag list space over-

flowed as vectors came in), it should not erase anything, and

return dmode = 0 to tell AutoCAD.  AutoCAD will send XOR vectors

through dmode = 0 to erase it.  Any vectors sent to this viewport

through dmode = 0 should be drawn (which erases) until you get a

fresh initialization for this viewport's drag mode.

dmode = 4Instructs the driver to displace (move) all of the vectors in

the drag list by the specified amount.  The driver is responsible

for redrawing the vectors in the new position.  (This will have

been preceded by a dmode = 3 call to erase the old image.)

If there are many vectors to redraw, the driver should not do

them all at once.  After 100 milliseconds (or so), the driver

should return with dmode set to 1 to indicate that more redraw is

required.  After AutoCAD checks for input from the user, the

driver will be called again with dmode set to 4 to let the driver

continue redrawing.  It is the driver's responsibility to remem-

ber where to continue redrawing.

If the driver can't perform the requested displacement (because

the display list overflowed, for example), it should return dmode

= 0.  AutoCAD will handle erasing and redrawing the image by

sending vectors through the drag vector facility.  The driver

should leave the overflow flag set so the vector facility will

know to just draw and not add the vectors to a drag list.

The driver should return with dmode = 2 when the image has been completely

redrawn.

--------------------  PLOPEN (44)  --------------------

74

(3) ADI Display / Packet Mode

struct pklopen {

short   pfunc;

short   vpnumber;

short   projid;

double  rxl, ryl;

double  rxh, ryh;

};

rxl, ryl, rxh, and ryh are in floating-point space coordinates.

This function is called whenever an action (such as a REGEN) is performed which

could move a viewport's logical space or change its projection id.

If EF_BS in pefmodes was set at PINIT (1), this function provides a projection

identification number to compare with any other viewports maintained by the

driver.  It also provides real coordinates for the corners of the viewport's

logical space in floating-point space, useful for determining if one viewport

is a subset of another.  Note that the data supplied in PLOPEN refers to the

corners of the entire logical space, not just the portion visible on the

screen.

75

Autodesk Device Interface/ADI

--------------------  PCLVP (45)  --------------------

struct pkclvp {

short   pfunc;

short   vpxmin, vpymin;

short   vpxmax, vpymax;

};

This function allows fast clearing of a viewport for non-big screen drivers if

EF_MV was set and EF_BS was cleared in pefmodes at PINIT.  It is called

whenever a viewport is to be cleared (such as during a PAN, ZOOM, REGEN or

REDRAW) and erases the indicated rectangle.  vpxmin, vpymin, vpxmax and vpymax

are expressed in LLG coordinates.

--------------------  PREVEC (46)  --------------------

struct pkrvec {

short   pfunc;

short   rvflags;

short   vpnumber;

short   master;

};

This function is called after a PVIEWPORT (40), PCLOSEVP (39), or any change

which could alter the relationship of master and slave viewports.  It will be

called only if the driver set EF_BS in pefmodes at PINIT.

Mnemonic  rvflags bit  Value             Meaning

YES_VEC       0         1    Turn off NO_VEC for viewport

vpnumber

UN_VEC        1         2    Turn on NO_VEC for viewport

vpnumber

AGAIN        2         4    Call PREVEC again soon

This function facilitates handling of slave viewports which are subsets of

another (master) viewport.  If the master viewport is current and it is panned

or zoomed so that it is no longer a superset of one or more slaves, PREVEC may

be an attractive alternative to PCONIN for asking AutoCAD to turn vectors back

on for the freed slave.  An additional bit makes it easier to enslave viewports

if a fresh master is created by opening or moving a viewport; it allows you to

turn off vectors for any legal viewport.

If UN_VEC is set in rvflags, be sure to set master equal to the viewport number

of the slave's master viewport.  Otherwise master should be equal to vpnumber.

UN_VEC and YES_VEC cannot both be set for the same vpnumber.

Setting AGAIN in rvflags facilitates changing the status of several viewports

in rapid succession.

76

(3) ADI Display / Fast Vector Mode

Fast Vector Mode

Although using packet mode dramatically increases the efficiency of an ADI

graphics display driver compared to a driver using the INT Interface, a packet

mode ADI driver will still redraw slightly slower than one built into AutoCAD.

Although measurements vary widely based on the driver's performance and the

average vector length of the drawing used to make the measurement, a typical

slowdown of 10% may be expected for a conventional bit-mapped driver.

Since redraw speed is not the most important factor in AutoCAD's performance,

this slight overhead can usually be tolerated.  If the ultimate in performance

is desired, however, the ADI specification contains a special fast vector mode

which permits an ADI driver to actually run faster than a driver built into

AutoCAD.

Using fast vector mode requires that the driver's routine for vector drawing be

coded in assembly language and that it conform precisely to the specifications

given herein.  Deviation from the specifications can lead to unreliable opera-

tion and/or random crashes of AutoCAD.

To use fast vector mode, your driver must use ADI packet mode.  You should

first develop and test your driver thoroughly in normal packet mode before you

upgrade it to fast vector mode.  Because fast vector mode is used only for

time-critical vectors, all of the existing code in your packet mode driver will

continue to be used by AutoCAD even though you are using fast vector mode.

To enable fast vector mode, set EF_FDRAW (bit 6) in the pefmodes field of the

packet returned by the PINIT request.  When this bit is set, the pfdraw field

in the PINIT reply packet must be set to the far call address of your assembly

language vector draw routine.  The long value in the packet should contain the

address in the SEGMENT:OFFSET form used to make a far call through a pointer.

How you get it there is your business.

Refer to the sample packet mode EGA drivers (DSEGA.C and DSEGA4.C) and their

accompanying assembly modules (DSEGAA.ASM and DSEGA4.ASM), supplied with the

ADI Driver Development Kit, for examples compatible with Lattice C, Microsoft C

and Borland C.

Once you have enabled fast vector mode, requests to draw normal vectors from

AutoCAD will come directly to your assembly language driver, bypassing the ADI

driver within AutoCAD and your packet handler.  Skipping these intermediate

call levels is what permits a fast vector mode ADI driver to outperform a

built-in AutoCAD driver.

If EF_DRAW (bit 6) is set and EF_BS (bit 12) is 0 in pefmodes at PINIT, AutoCAD

uses the following calling sequence to draw a vector:

77

Autodesk Device Interface/ADI

push[non_drawing_flag]

push[highlight]

push[color]

push[to_y]

push[to_x]

push[from_y]

push[from_x]

callfar [pfdraw]

where pfdraw is the pointer returned in the PINIT reply packet.

If both EF_DRAW and EF_BS are set, AutoCAD uses the following calling sequence:

push[vpnumber]

push[highlight]

push[color]

push[to_y]

push[to_x]

push[from_y]

push[from_x]

callfar [pfdraw]

The driver should draw a vector from (from_x,from_y) to (to_x,to_y) in the

color specified by color.  color will be zero to erase a vector or positive to

specify one of the standard color numbers used by Autodesk products (see Appen-

dix A).

The fast vector draw routine will not be used for XOR vectors, so it need not

contain code to handle them.  This can usually speed up the routine signifi-

cantly.

If highlight is set, the vector should be highlighted on the screen (usually by

drawing it with a dashed line font).

The non_drawing_flag was used in earlier versions of ADI to enable displays

with a local vector list to identify vectors which should not be saved for

local redraw.  This is no longer necessary.  Such drivers should set EF_BS in

PINIT.  All vectors subsequently arriving through FASTDRAW should be added to

the driver's display list.  FASTDRAW will be the only source of vectors belong-

ing in the display list (except for PFILL, if the driver has "smart fill" capa-

bility). Vectors sent through PLINE will be non-drawing entities. Vectors sent

through PVEC will be non-drawing entities or drawing vectors for NO_BS view-

ports (which do not get added to the display list).

Both the highlight and non_drawing_flag will be 1 if true and 0 if false.

If both EF_DRAW and EF_BS are set, vpnumber will be added to the pfdraw calling

sequence.  Its meaning is basically the same as in PVEC.  vpnumber is always

nonzero.  The driver should interpret the endpoints passed to pfdraw as logical

coordinates relative to the specified viewport.  The driver must clip and scale

the vectors as necessary for display purposes.

78

(3) ADI Display / Fast Vector Mode

When control reaches your fast vector handler, the arguments will be found on

the stack starting at 4 bytes above SP.  CS will, of course, point to the code

segment of your fast vector handler.

The contents of DS and ES are undefined, but must be saved and restored if the

driver changes them.  The driver must also save and restore the BP register and

leave with SP and SS set as they were on entry to your routine.

All other registers may be changed at will by the fast vector driver.

If ever there was a place where every instruction counts, the fast vector draw

routine is it.  All the effort you expend to optimize the execution of this

routine will be directly repaid in how fast your driver draws on the screen.

79

Autodesk Device Interface/ADI

Development Hints for Display Drivers

This section briefly lists various hints which you may find useful in updating

an existing ADI display driver, or developing a new one.

Existing ADI 3.1 drivers

If you are updating an ADI 3.1 non-display list driver, you can ignore much of

the material relevant to version 4.0. Here is what you need to do:

1. Use the new dsadi.h and pcall.asm.  Read the section on INIT1 to see

the new "driver successfully installed" flag and update your code

accordingly.  In addition, your code should now truncate any packets

longer than it expects and ignore packets it doesn't understand.  This

will help ensure compatibility with future extensions of ADI.

2. Implement PCLVP to speed up viewport clear.  If your driver already

supports the AUI, you probably can use the same routine which imple-

ments PBOXCLR, substituting the proper coordinates.

New ADI 4.0 non-display list drivers

Here is a suggested sequence for developing an ADI 4.0 non-display list driver:

1. Implement and debug a stripped-down driver without AUI, PFILL, smart

dragging, raster cursor or FASTDRAW.

2. Implement and debug the extra features which you choose to support.

3. Consider using the packet tracing and logging code from Autodesk's

sample drivers in your driver during development.

ADI 4.0 Display list drivers

If you plan to implement a display list driver for ADI 4.0 you may find it less

painful if you proceed in stages, as described below:

1. Begin by implementing and debugging a non-display list ADI 4.0 driver

without FASTDRAW.

2. Implement and debug FASTDRAW.

3. Implement and debug your display list without optional features (such

as NO_VEC slave viewports and EF_DRAG).

4. Implement and debug optional features such as slave viewports and

smart dragging.

80

(3) Development Hints for Display Drivers

Please test your driver carefully - there are

far more cases to test with multiple viewports,

master/slave viewports, etc.

81

Autodesk Device Interface/ADI

Summary of changes in ADI 4.0

All of the packets or functions in the following table have undergone some

change in ADI 4.0.  Some of these changes affect all display drivers running

under ADI 4.0; others apply only to "big screen" drivers, or those which imple-

ment new, optional features.

If an "x" appears in the "Change Required?" column, this means that pre-ADI 4.0

drivers must make some change to this function in order to run with ADI 4.0.

Other pre-4.0 packets have been changed but don't require that a pre-4.0 driver

be updated in order to use them.  New packets implement changes to support "big

screen" displays or to provide additional, optional, capabilities.

Function         Drivers       Change             Comments

Name          Affected     Required?

INIT1              All           x      Revise "success" flag

PINIT (1)          All           x      Check EF_MV, EF_DRAG, EF_BS

and ybias.

PCLEAR (3)     Big Screen               Pre-ADI 4.0 packet

PVEC (5)       Big Screen               Pre-ADI 4.0 packet

PFILL (6)      Big Screen               Pre-ADI 4.0 packet

PMARK (7)      Big Screen               Pre-ADI 4.0 packet

PCMARK (8)     Big Screen               Pre-ADI 4.0 packet

PSYNCH (23)    Big Screen               Pre-ADI 4.0 packet

PREDRAW (24)   Big Screen               Pre-ADI 4.0 packet

PCOMMAND (25)      All                  Optional feature

FASTDRAW       Big Screen               Optional feature; not a

packet

POPENVP (38)   Big Screen               New packet

PCLOSEVP (39)  Big Screen               New packet

PVIEWPORT (40) Big Screen               New packet

PCONIN (41)        All                  Optional feature

PTABCFG (42)       All                  Optional feature

PDRAG (43)         All                  Optional feature

PLOPEN (44)    Big Screen               New packet

PCLVP (45)   Non-Big Screen      x      Required for non-big screen

drivers

PREVEC (46)    Big Screen               New packet

82

(3) ADI Display / Test Procedures

ADI Graphics Display Test Procedures

Included in the ADI Driver Development Kit is a set of files for testing graph-

ics display drivers.  These files use the AutoLISP LISP interpreter built into

AutoCAD to automate the testing of graphics display drivers.

To load these files type (load "DSTEST") at the AutoCAD Command: prompt, or

rename DSTEST.LSP to ACAD.LSP to have the program loaded automatically when

AutoCAD is started. (The files are shipped in the DSTEST directory on the

ADIKIT disk.)

Type single or dual at the AutoCAD prompt to test those configurations.  Read

the comments at the beginning of DSTEST.LSP for a further explanation of the

tests and how to run them.

If you expect your driver to be used by

AutoShade or AutoSketch, you should make a point

of checking the alignment of vectors and text in

the pull down menus in those products.  There

are problems which show up here and not in

AutoCAD.

If you wish, you may also perform the manual test procedures outlined below.

Most - but not all - of these procedures are a subset of the tests performed by

DSTEST.LSP. It should not be considered a thorough test for a display driver,

but you may find it useful in the early stages of development.

1. Set up the machine to be tested with the display board, an appropriate

monitor, and a digitizer.

2. Copy the AutoCAD release into a test directory. Configure AutoCAD for

the board, monitor, and digitizer you are using.

3. Start a new drawing.

4. Draw a circle and DRAG it around the screen.  No garbage should appear

or be left behind on the screen.  ERASE the circle.  The circle should

be highlighted when selected.  On a single-screen display configura-

tion, use the flipscreen key to toggle the display from the graphics

screen to text screen, and back to the graphics screen.  Neither the

circle, nor any garbage should be displayed on the screen, and the

border should be intact.

5. Draw a box around the edges of the graphics area, overlaying the bor-

ders.  This is most easily done using the LINE command to draw a line

from one corner to each of the remaining corners.  Then ERASE the box.

The borders of the graphics area should be intact.

6. Configure the display with the Menu Area ON and the Command Prompt

Area OFF.  Scroll through the menu with the pointing device from top

83

Autodesk Device Interface/ADI

to bottom and back to the top.  The menu items should retain their

position on the screen.  They should not scroll (move).  Scroll

through the menu from bottom to top and back to the bottom.  Again,

the menu items should not scroll.

7. Configure the display with the Command Prompt Area ON.  Enter the

drawing editor using a large drawing (e.g.  CHROMA).  Point to the

last item in the menu area with the pointing device.  Type REGEN.

Enter CTRL-C while the REGEN is in progress.  After the REGEN is can-

celed, enter several more commands.  The command text area should not

scroll into the drawing area.

8. For color display drivers, check that the colors are correct.  Also,

if colors for graphics background, menu text, and prompt text are con-

figurable, be sure that the color assignments actually work.

9. For color display drivers, turn AXIS  ON, draw something with any

color (other than that used for the axis), ERASE the objects, and flip

the screen display from graphics screen to text screen and back again.

The AXIS should not be redrawn with the color you used for the object.

Also, the border should be intact.

10. Draw a wide polyline.  MOVE the polyline.  Garbage should not appear

or be left behind on the screen.  ERASE the polyline.  There should be

no garbage left on the screen.  Perform the same steps with a solid.

It is not unusual for some graphics controllers to draw a different

set of pixels depending on the direction the vector is drawn.  If gar-

bage is left on the screen after erasing a polyline or solid, you

might try sorting the coordinates before drawing the vector.

11. Test any configurable items you've chosen to implement, such as dis-

play resolution, text size, number of colors displayed simultaneously,

background/text/menu colors, or special commands like hardware pan and

zoom.

12. If the display driver supports the advanced user interface, load the

standard AutoCAD menu.  Note that the menu bar and pull-down menus

work only when the status line is configured, and that they are only

accessible using a pointing device.

Move the pointer into the status line; the menu bar should appear.  As

you move the pointer left and right, the menu bar's titles should

highlight.  Try selecting each of the titles; the corresponding pull-

down menu should appear.  Moving the pointer within a pull-down menu

should highlight each of its items.  (You should also test a menu that

contains "greyed out" items.)

Use the "Draw/Hatch" item or the "Options/Fonts" item to test the icon

menus.  While an icon menu is displayed, the cursor should appear as

an arrow.  Moving the arrow over the box to the left of an icon should

cause the box to highlight and a box to be drawn around the icon.

84

(3) ADI Display / Test Procedures

To give the dialogue boxes a workout, first create a new layer and

freeze it (for instance: "LAYER New ice Freeze ice <RETURN>") and then

choose "Entity Creation" from the "Modes" pull-down menu (or enter the

DDEMODES command).  Move the arrow to the "Layer" area and press the

"pick" button -- the "Select layer" dialogue box should appear "on top

of" the "Entity creation modes" dialogue box.  The entry for the

frozen layer should appear "greyed-out".  Now, if you point to the

"Layer" area at the bottom of the dialogue box and type in an invalid

layer name, such as "junk", "OK" and "Cancel" buttons should appear to

the right.  If you pick the "OK" button or press the RETURN key, an

alert box should appear on top of everything else informing you that

this layer does not exist.  You can use pick the "OK" or "Cancel" but-

tons (or press the ESC key) to dismiss the alert box and each of the

dialogue boxes; as you do so, the underlying dialogue box or graphics

should reappear.

If the driver supports the advanced user interface, the Autodesk prod-

uct differentiates "crossing" selection boxes from "window" selection

boxes by highlighting the "crossing" boxes.  This is the only instance

where the driver is asked to highlight and XOR at the same time, so be

sure to test this case.

Testing hidden text screen behavior

As noted previously, single-screen ADI display drivers can request that AutoCAD

maintain the "hidden" text screen.  Release 10 of AutoCAD uses a temporary file

to store this text, however, and this can impact the driver's performance when-

ever a flipscreen is performed.  Drivers that can store the text screen them-

selves should probably do so.

You should be able to flip back and forth repeatedly between the graphics and

text screens without losing any text on the hidden screen.  (You can test this

by invoking the STATUS command to cause AutoCAD to flip to the text screen.

Now flip back and forth between the status display and the graphics screen.)

You should also be able to enter AutoCAD commands from either the graphics

screen or the text screen and not lose text if the display is forced from one

screen to the other.  (Test this by calling DVIEW from the text screen, or

invoking the FILES command from the graphic screen.)

Consider a display which can show 24 lines of text on the text page and three

lines of scrolling text on the graphics screen.  You should be able to type a

series of commands while on the graphics screen, flip to the text screen, and

find 24 lines of text.  If you flip back to the graphics screen , you should

see the last three non-blank lines of text in the scrolling text area.  If you

immediately flip back to the text screen, you should see all 24 lines of text

again.

Additional tests for Display List ("big screen") drivers

1. Check that your driver handles display list overflow properly, by

working with a very large drawing.

85

Autodesk Device Interface/ADI

2. If you are checking for "master/slave" status of viewports, you should

exercise this code thoroughly by performing a variety of extreme ZOOM

operations; this will force switching of masters' and slaves' NO_VEC

status.

As a simple test, create a drawing with a circle in it.  Split the

drawing into 4 viewports.  Zoom the viewports to force one to be a

master to the others.  Try a REDRAW in each viewport; then try a

REGEN.  Try REDRAWALL and REGENALL.  Now zoom one of the slaves to

make it the master of the original master.  Recheck to see that all 4

viewports still work properly.

3. Use the PAN command to check for proper clipping to the edges of view-

ports.  (You should also use PAN - like ZOOM, above - to check for

correct master/slave "flip/flops".)

4. Check that crosshair cursors are clipped properly to the exact view-

port boundary in multiple viewports.

5. Check that the viewport selection cursor displays and works correctly.

6. Check TABLET mode at several levels of zoom, to be sure that tablet

data has not been corrupted.

Dynamic Dragging

If your driver implements dynamic dragging (by setting EF_DRAG in pefmodes at

PINIT) you should add the following tests:

1. Check dragging in multiple viewports simultaneously.

2. Check the clipping of dragged images in one (or more) viewports works

correctly.

3. Check your handling of drag display list overflow by dragging a huge

shape.

4. Check dragging in a slave viewport.  (Zoom out quite a bit in a view-

port to force it to become a master to a zoomed-in slave.)

The sample drivers DSEGA.C and DSEGA4.C (and

their associated assembly modules) contain

debugging code which you may want to use during

the development and testing phase of implement-

ing your display driver.  With this code in

place, pressing ^L will dump vpinfo, allowing

you to prove to yourself that you have zoomed or

panned enough to force master/slave relation-

ships to change.

86

(3) ADI Display / Test Procedures

Checking configuration options

The next group of procedures involves configuring the Status Line, Command

Prompt Area, and Menu Area in the following ON/OFF combinations and performing

tests with each configuration.  (You can use the following chart to track your

progress.)

Status      Prompt     Menu Area   Pass/Fail

Line        Area

Config 1      Off         Off         Off

Config 2      Off         Off         On

Config 3      Off         On          Off

Config 4      Off         On          On

Config 5      On          Off         Off

Config 6      On          Off         On

Config 7      On          On          Off

Config 8      On          On          On

1. Go to the AutoCAD configuration menu and configure the Status Line,

Command Prompt Area, and Menu Area according to the desired configura-

tion in the chart.

2. Edit the existing drawing, DSTEST.DWG.

3. Draw a box around the limits of the display (as in step 5 above).

Make sure that the edges of the display are accessible.

4. Check the Status Line.  If you have configured the Status Line OFF, it

should not appear at the top of the screen.  If you have configured

the Status Line ON, it should be properly positioned at the top of the

screen, and should contain the correct layer, mode, and coordinate

information.

5. Check the Command Prompt Area.  If you have configured the Command

Prompt Area OFF, it should not be appear at the bottom of the screen.

If you have configured the Command Prompt Area ON, it should be posi-

tioned properly, and there should be usable text lines at the bottom

of the screen for entering commands or data (3 lines for single-screen

configurations, 1 line for dual-screen configurations).

6. Check the Menu Area.  If you have configured the Menu Area OFF, it

should not appear on the right hand-side of the screen.  If you have

configured the Menu Area ON, it should be positioned properly at the

right hand-side of the screen.  Scroll through the Menu Area from top

87

Autodesk Device Interface/ADI

to bottom and from bottom to top.  Make sure that you can pick each

menu item, and that each operation functions correctly.

7. Use the flipscreen key to toggle the display from the graphics screen

to the text screen, and back.  The lines bordering the display area

should be intact.

8. Draw a unit circle in the middle of the screen.  ZOOM to extents.  The

border should not clip the circle at the top of the screen.  ERASE the

circle.  The borders at the top and bottom edges of the screen should

not be erased.  UNDO the ERASE operation to restore the circle.  The

borders at the top and bottom edges of the screen should remain

intact.  ZOOM out and draw another unit circle to the right of the

first one so that the distance between the centers is greater than the

diameter.  ZOOM to extents.  The border should not clip the circles at

the sides of the screen.  ERASE both circles.  This should not erase

the borders at the sides of the screen.

9. Repeat the above procedures with the next configuration.

88

Chapter 4

ADI Rendering Driver Specification

This section describes the requests issued by Autodesk software products to

communicate with an installed ADI driver written to support rendering devices.

Only one Autodesk product, AutoShade, currently supports rendering devices.

Because of the nature of AutoShade, its driver requirements are somewhat more

complex than other Autodesk products.  Drivers written to work with AutoShade

require both a display interface, and a rendering interface (used to produce

color renderings).

You may choose to write a single driver that will work with AutoCAD,

AutoSketch, and AutoShade, or you may decide to write separate drivers for dis-

play devices and AutoShade rendering devices.  In fact, the rendering device

need not be a video display.

Rendering Driver Requests

The rendering device driver requests are extensions to the Packet interface for

graphics display drivers, and are presented in the same format.

--------------------  RDLINKUP (2000)  --------------------

Like an ADI packet-mode graphics display driver, communication with an ADI

packet-mode rendering driver begins with a request made through the INT inter-

face.  This initial request, known by AutoShade as RDLINKUP, is performed as

described below.  The value 0 returned in Res1 requests packet mode.

Parameters:  Code    (Ax):   2000

Arg1    (BX):   0

Arg2    (CX):   (0,INTLEVEL)

Return values:  Res1 (BX):   0

Res2    (CX):   OFFSET

Res3    (DX):   SEGMENT

Res2 and Res3 specify the OFFSET and SEGMENT, respectively, of the entry point

to be called from AutoShade to process each ADI request packet.

Once the RDLINKUP request is issued, the ADI driver within AutoShade will begin

communicating with the installed driver using packet mode.  Examination of the

sample driver source provided in RDPGCP.C will elucidate the fine points of

this interface.

89

Autodesk Device Interface/ADI

The first packet-mode request will be an RDINIT call to obtain initialization

parameters.

90

(4) ADI Rendering Device

Each packet-mode request, its function code, packet format, parameters, and

purpose, is described in detail below.  The explanations assume that the fol-

lowing structure has been defined:

union rd_mastr {

struct rd_init   rinit;

struct rd_start  rstart;

struct rd_poly   rpoly;

struct rd_cmap   rcmap;

struct rd_fname  rfname;

struct rd_crange rcrange;

struct rd_cpoly  rcpoly;

};

struct rendreq {

short            rfunc;

union rd_mstr    rpkt;

};

--------------------  RDINIT (2001)  --------------------

The RDINIT packet is provided to the driver with fields rfunc and flags ini-

tialized by AutoShade.

struct rd_init {

unsigned short flags;

short          xdots, ydots;

unsigned short pixwid, pixhgt;

short          ncolour;

unsigned short maxintens;

unsigned short nshades;

far char       *name;

far char       *rendconf;

};

All other fields should be filled in by the driver and returned to AutoShade.

flags is a 16-bit, bit-coded field.  Currently defined bits for flags are

listed below.  Mnemonics for these bits are defined in RDADI.H.

Mnemonic     Bit  Value                   Meaning

RF_SINGLSCR     0     1    0 = dual-screen configuration

1 = single-screen configuration

RF_REDRAWSCR    1     2    0 = redraw not required on flip screen

1 = redraw required on flip screen

RF_FAILINIT     2     4    0 = driver initialization successful

1 = driver initialization failed

RF_FILEOUTPUT   3     8    0 = driver does not write output to a file

1 = driver writes output to a file

RF_NAME         4    16    0 = driver does not return name

1 = driver returns name

91

Autodesk Device Interface/ADI

Drivers should change only the bits defined in the table above, leaving the

rest unchanged. (Some are used by AutoShade for internal purposes.)

RF_SINGLESCR and RF_REDRAWSCR (flags bits 0 and 1) are set by AutoShade in

accordance with information provided by the user during the configuration pro-

cess, and can be examined by the driver if desired.  The remaining flag bits

should be set or cleared by the driver before returning to AutoShade, to inform

AutoShade of various driver capabilities.

RF_SINGLSCR will be set if the user configured AutoShade for single-screen

operation, wherein the display device is also used for color renderings.  In

this case, the driver must not switch to rendering mode until it receives an

RDSTART request.

In a single-screen configuration, RF_REDRAWSCR will be set if the user

indicated that a FLIPSCREEN operation requires a redraw.  An RDTERM request

will signal such an operation.  AutoShade never saves the image from either the

display screen or the rendering screen, but if RF_REDRAWSCR was set, it will

clear the display screen and redraw its borders and menu bar whenever the user

flips from the rendering screen to the display screen. If the user indicates

that no redraw is necessary for a FLIPSCREEN, the rendering is left on the

screen until a new wireframe image is drawn.  Wireframe images are preceded by

a call to clear the display screen.

The driver should clear RF_FAILINIT (flags bit 2) in the reply packet upon suc-

cessful initialization.  If the driver detects an error during initialization,

it should return with this bit set and AutoShade will abort.

The driver should set RF_FILEOUTPUT (flags bit 3) in the reply packet if the

driver's purpose is to write to an output file (such as a PostScript(tm) file)

rather than to a hardware device.  When this bit is set, the Record item on

AutoShade's Display Menu will be greyed out so that the user will not be able

to activate Record mode.

The driver should return a SEGMENT:OFFSET pointer to a text string in the reply

packet's name field.  The text string is a NUL-terminated ASCII string that

will be written at the beginning of all rendering recording files to identify

the driver used to produce the file.  For example, the IBM PGC driver returns a

pointer to the string, "IBM Professional Graphics Controller (PGC)." The string

must not exceed 65 characters in length.  If an identification string is being

returned, the driver should also set RF_NAME (flags bit 4) in the reply packet.

The values returned by xdots and ydots specify the size (in pixels) of the

graphics area of the screen.  xdots is the horizontal size of the graphics area

of the screen, and ydots is the vertical size of the graphics area.  These

values determine the units used to specify polygon vertices that are passed to

the driver by the RDPOLY request.  Vertex values range from (0 to xdots-1) and

(0 to ydots-1) with (0,0) representing the lower left corner of the screen.

pixwid specifies the horizontal physical spacing between pixels, and pixhgt

specifies the vertical physical spacing between pixels.  The ratio of pixwid to

pixhgt is used to correct the aspect ratio so that the horizontal and vertical

measurements of objects drawn on the screen are equal.

92

(4) ADI Rendering Device

On a device with square pixels, pixwid and pixhgt should both be 1.  On devices

with different X and Y scales, pixwid and pixhgt should be set based on the

physical screen measurement in each direction divided by the number of pixels

on the appropriate axis.

Only the ratio of pixwid to pixhgt is significant, so any desired units may be

used.  The values should as accurate as possible, however, since changes in the

second decimal place of the ratio result in visible changes on the screen.  The

values returned in pixwid and pixhgt must be greater than 0 and less than

32767.

ncolour specifies the total number of entries in the color look-up table (LUT),

and determines the number of colors that can be simultaneously displayed on the

screen.  For example, the IBM Professional Graphics Controller supports 256

simultaneous colors.  Values used to index into the color look-up table (for

example, the value assigned to the code variable in the RDCMAP request) must be

within the range 0 to ncolour-1.

If the driver returns ncolour equal to 0, AutoShade will treat the device as a

continuous-tone device, and will never issue the RDCMAPB, RDCMAP, RDCMAPE, or

RDPOLY requests.  Instead, the RDCRANGE request will be issued at the start of

a rendering. If your rendering device is capable of more than 256 RGB shades,

you may wish to treat it as a continuous-tone device.  The PostScript driver is

an example of a continuous-tone driver (due to the large number of monochrome

dithering patterns available).  Also, you might specify continuous-tone if your

device doesn't use RGB and the continuous-tone representation eases the trans-

lation to your device's color system.

maxintens specifies the maximum value that can be assigned to the red, green,

or blue components of a color look-up entry.  For example, the values assigned

to the red, green, and blue variables passed to the driver via the RDCMAP

request will be within the range 0 to maxintens.  Similarly, AutoShade will

return color palette RGB values within the range 0 to maxintens.  For the IBM

Professional Graphics Controller, the value assigned to maxintens is 255.

nshades specifies the number of shades available per color, which in turn

determines the permissible number of intensity values for a single RGB gun.

For most devices, nshades = maxintens+1.  However, some devices allow a smaller

number of shades per RGB gun than the value assigned to maxintens.  For the IBM

PGC, for example, nshades is 15, even though maxintens is 255.  nshades may

also be expressed as nshades=2n, where n is the number of bits in a color

look-up entry.

rendconf is a SEGMENT:OFFSET pointer to a 32-byte driver configuration area.

This area is not currently used as a configuration area.

93

Autodesk Device Interface/ADI

--------------------  RDSTART (2002)  --------------------

The RDSTART request indicates that AutoShade is beginning a rendering.

struct rd_start {

unsigned short flags;

};

When the driver receives this request, it should switch to the rendering

screen.

If configured for single-screen operation, indicated by RF_SINGLSCR (flags bit

0) set in the RDINIT reply packet, the driver should switch the display from

display mode to rendering mode.

An RDSTART request will always be followed by an RDCLEAR request to clear the

screen.

In general, AutoShade attempts to terminate a rendering initiated by an RDSTART

request with a matching RDEND request.  However, it is possible that multiple

RDSTART requests may be issued without matching RDEND requests.

flags is a 16-bit, bit-coded options field that is currently unused.

--------------------  RDEND (2003)  --------------------

The RDEND request indicates that the rendering process is complete.

struct rendreq {

short rfunc;

};

When the driver receives this request, it should switch to the display screen.

If configured for single-screen operation (indicated by RF_SINGLSCR (bit 0) of

the flags field set in the RDINIT reply packet) the driver should switch the

display from rendering mode to display mode.

In general, AutoShade attempts to terminate a rendering initiated by an RDSTART

request with a matching RDEND request.  However, it is possible that multiple

RDEND requests may be issued that do not correspond to matching RDSTART

requests.

94

(4) ADI Rendering Device

--------------------  RDTERM (2004)  --------------------

The RDTERM request indicates that all rendering is complete, and that the

driver can terminate the rendering process.

struct rendreq {

short rfunc;

};

The RDTERM request indicates that AutoShade is exiting to DOS.  No further ren-

dering requests will be issued during the current execution of AutoShade.

--------------------  RDRESIZE (2005)  --------------------

The RDRESIZE request is only issued on windowing systems after the operating

system has informed AutoShade that the window size has changed.

struct rd_init {

unsigned short flags;

short          xdots, ydots;

unsigned short pixwid, pixhgt;

short          ncolour;

unsigned short maxintens;

unsigned short nshades;

far char       *rendconf;

};

Upon receipt of this request, the driver must return the current status of the

values returned in the RDINIT reply packet.

This request is currently issued only on UNIX(tm) systems.

95

Autodesk Device Interface/ADI

--------------------  RDCLEAR (2006)  --------------------

The RDCLEAR request, typically issued following an RDSTART request, instructs

the driver to clear the rendering screen to the current background color.

struct rendreq {

short rfunc;

};

The background color is determined by entry 0 in the color look-up table, and

may be changed by the RDCMAP request.

If the RDCLEAR request is issued before the color look-up table has been set

up, the driver should clear the screen to black.

--------------------  RDCMAPB (2007)  --------------------

The RDCMAPB request signals the beginning of changes to the color look-up

table, and will be followed by multiple RDCMAP requests.

struct rendreq {

short rfunc;

};

96

(4) ADI Rendering Device

--------------------  RDCMAP (2008)  --------------------

The RDCMAP request provides the red, green, and blue intensities for the color

look-up table entry specified by code.  In other words, the RDCMAP request is

used to map colors into the color look-up table.

struct rd_cmap {

short          code;

unsigned short red, green, blue;

};

The values for code will fall within the range 0 to ncolours-1.

The values for red, green, and blue will fall within the range 0 to maxintens.

Values are assigned to both ncolours and maxintens in response to the RDINIT

request.

Color look-up table entry 0 is reserved by AutoShade for the current screen

background color.  This entry should be used by the RDCLEAR request to clear

the screen to the background color.

If a RDCMAP request maps a new color into entry 0 of the color look-up table,

the rendering screen background color should be changed accordingly.  If this

does not occur automatically as a result of setting the new value for entry 0

of the color look-up table, the driver can wait until after the RDCMAPE request

is issued to display the new background color.

--------------------  RDCMAPE (2009)  --------------------

The RDCMAPE request indicates that all changes to the color look-up table (per-

formed by multiple RDCMAP requests) have been completed.

struct rendreq {

short rfunc;

};

The driver should ensure that the background color specified by entry 0 in the

color look-up table is displayed on the rendering screen.

When changes are made to the color look-up table, AutoShade may not use the

entire look-up table, but it is guaranteed to use at least 1 color entry, and a

maximum of ncolour (returned in the RDINIT reply packet) color entries.

Changes are performed sequentially, from entry 0 to entry ncolours-1.

97

Autodesk Device Interface/ADI

--------------------  RDPOLY (2010)  --------------------

AutoShade passes polygon vertices to the driver using the RDPOLY request.

struct rd_poly {

unsigned short flags;

short          colour;

short          ecolour;

short          nvert;

short          vx[10];

short          vy[10];

};

Each RDPOLY request may pass a maximum of 10 polygon vertices to the driver.

If the polygon to be rendered contains more than 10 vertices, the remaining

vertices will be passed to the driver by additional RDPOLY requests.

flags is a 16-bit, bit-coded field.  Currently defined bits for flags are as

follows:

Mnemonic   Bit  Value                    Meaning

RF_MORE       0     1    0 = no more vertices left to send

1 = additional vertices to send

RF_CONT       1     2    0 = new RDPOLY request

1 = continuation of previous RDPOLY request

Top 10 bits              Invisible edges

RF_MORE (flags bit 0) should be set if additional RDPOLY requests are required

to pass additional polygon vertices to the driver.  RF_CONT (flags bit 1)

should be set if the current RDPOLY request is a continuation of a previous

RDPOLY request.

The top 10 bits (bits 6 through 15) of flags are used to indicate which edges

of the polygon should not be drawn in ecolour.  Each of these bits corresponds

to an edge defined by (vx[i],vy[i]) to (vx[i+1],vy[i+1]).  If colour and

ecolour are equal, then the driver should ignore the invisible edges, since any

border it draws would be invisible.

For example, if flags bit 6 is set, the edge from (vx[0],vy[0]) to

(vx[1],vy[1]) should not be drawn.  If flags bit 7 is set, the edge from

(vx[1],vy[1]) to (vx[2],vy[2]) should not be drawn.  If flags bit 8 is set, the

edge from (vx[2],vy[2]) to (vx[3],vy[3]) should not be drawn, etc.

colour and ecolour are the look-up table indices specifying the colors to be

used to fill the polygon and to draw its edges, respectively.  In the current

implementation, the two are always equal so that no edge outlining need be per-

formed.  However, ecolour is used whenever nvert is 1 or 2 (see description of

nvert below).

nvert contains the total number of vertices in the polygon, not the number of

vertices contained in the vertex list (unless the total number of vertices in

the polygon is 10 or less).

98

(4) ADI Rendering Device

If multiple RDPOLY requests are issued to the driver to pass a complete poly-

gon, the driver must keep track of the number of vertices passed with each

RDPOLY request in order to determine the number of RDPOLY requests required to

complete the polygon, as well as the number of vertices it will receive in the

last RDPOLY request.

The last RDPOLY request issued for a given polygon will always have RF_MORE

(flags bit 0) cleared (0).

Two special cases exist where the driver receives an RDPOLY request, but does

not draw a polygon.  If nvert is 1, a single pixel is drawn with the color

specified by ecolour at coordinates (vx[0],vy[0]).  If nvert is 2, a vector is

drawn with the color specified by ecolour from coordinates (vx[0],vy[0]) to

(vx[1],vy[1]).

vx[] and vy[] may contain up to 10 vertices per each RDPOLY request.

--------------------  RDFNAME (2011)  --------------------

The RDFNAME request is used to pass an output filename to the driver for its

own use.  This request occurs while still in display mode - after the user has

requested a rendering, but before the RDSTART request has been issued.

struct rd_fname {

unsigned short flags;

far char       *fname;

far char       *errormsg;

far char       *ftitle;

};

If the user picked the Record item from the Display menu, AutoShade first

prompts for the name of the recording file.  Following this (or if the user did

not select Record mode), AutoShade issues a series of RDFNAME requests to let

the driver obtain a file name from the user for its own use during the render-

ing process.  (If the driver's purpose is to write an output file, Record mode

is probably superfluous and should be disabled by setting the RF_FILEOUTPUT bit

in the RDINIT reply packet.)

flags is a 16-bit, bit-coded field used to control the flow of information

between AutoShade and the driver.  Currently defined bits for flags are as fol-

lows:

Mnemonic    Bit  Value                      Meaning

RF_TITLE    0     1    1 = AutoShade requests a title string from the

driver

RF_CANCEL   1     2    1 = user cancelled filename request

RF_RECORDING2     4    1 = AutoShade performs file output during ren-

dering

RF_GETFNAME 3     8    1 = AutoShade should prompt user for filename

RF_ERROR    4     16   1 = AutoShade should display alert box with

error message

99

Autodesk Device Interface/ADI

RF_RECORDING (flags bit 2) is set by AutoShade to inform the driver that

AutoShade is performing output to a recording file during the rendering pro-

cess.  The driver may ignore this bit, but should take care not to alter it.

The first RDFNAME request will have RF_TITLE (flags bit 0) set, indicating that

AutoShade is requesting a title for its filename dialogue box and that the

fname field does not yet contain a filename pointer.  If the driver doesn't

need a filename, it should simply ensure that RF_GETFNAME (flags bit 3) is

clear and return to AutoShade; no further RDFNAME requests will be issued for

this rendering.

If the driver does need a filename, it should indicate this by setting the

RF_GETFNAME flag bit in the reply packet.  If a custom dialogue title is

desired, the driver should store a SEGMENT:OFFSET pointer to the title string

(a NUL-terminated ASCII string not exceeding 70 characters in length) in the

ftitle field of the reply packet and set the RF_TITLE flag bit to signal that a

title is supplied.  For example, the PostScript rendering driver supplies the

title, "Create PostScript output file."  If the driver returns with RF_GETFNAME

set but RF_TITLE clear, AutoShade will use the default dialogue title.

Assuming that RF_GETFNAME was set in the reply packet, AutoShade will ask the

user for the filename and issue a second RDFNAME request, passing to the driver

the filename provided by the user.  If the user cancelled out of the dialogue

box, the RF_CANCEL flag bit will be set for this request, and the driver should

just clear all flag bits and return.

If the driver receives an RDFNAME request packet with neither RF_TITLE nor

RF_CANCEL set in the flags field, the fname field will contain a SEGMENT:OFFSET

pointer to the filename string entered by the user.  The filename does not nor-

mally include an extension, so the driver should expect to add one.  However,

the driver should never change or add to the string passed by AutoShade; if

this is necessary, the driver should first copy the string to its own local

space, and edit it there.  The driver should then attempt to open the file.

If it is unable to open the file, the driver should place a SEGMENT:OFFSET

pointer to an error message in errormsg, and set flag bits RF_ERROR, RF_TITLE,

and RF_GETFNAME in the reply packet to instruct AutoShade to display the error

message in an alert box and again ask the user for a filename.  The title

string should again be supplied in ftitle in this case.

If an error message is needed, it should be a NUL-terminated ASCII string and

may contain multiple lines (imbedded newline \n characters), but no single line

may exceed 70 characters in length, and the entire error message may not exceed

15 lines.

--------------------  RDCRANGE (2012)  --------------------

If the ncolour field is returned with a value of 0 in response to the RDINIT

request described above, the device will be treated as a continuous-tone

device, and the RDCMAPB, RDCMAP, RDCMAPE, and RDPOLY requests will never be

issued.  In this case, the RDCRANGE request will be issued in place of the

100

(4) ADI Rendering Device

RDCMAPB, RDCMAP, and RDCMAPE requests, and the RDCPOLY request described below

will be issued in place of the RDPOLY request.

struct rd_crange {

double        minshade;

double        maxshade;

short         rstretch;

struct rd_rgb bcolour;

};

struct rd_rgb {

double        r, g, b;

};

minshade and maxshade specify the range of floating-point shading factors that

will be passed to subsequent RDCPOLY requests.

If the user has requested that contrast in the picture be stretched to use all

of the device's dynamic range, rstretch will contain a nonzero value.  Other-

wise, it will contain the value 0.

AutoShade does not perform contrast stretching for a continuous-tone device.

If this feature is desired, it should be handled by the driver.

bcolour is a structure of type rd_rgb that contains three components:  r, g,

and b.  These components specify the user's requested background color as light

intensities from 0 to 1.  For example, black is represented by all three values

equal to 0; white by all three values equal to 1; and yellow by r and g equal

to 1, and b equal to 0.  This form of floating-point color specification is

used throughout for continuous-tone drivers thereby allowing any possible color

to be specified.

101

Autodesk Device Interface/ADI

--------------------  RDCPOLY (2013)  --------------------

The RDCPOLY request is used by AutoShade to pass polygon vertices to

continuous-tone drivers.

struct rd_cpoly {

unsigned short flags;

struct rd_rgb  colour;

struct rd_rgb  ecolour;

double         shadef;

short          nvert;

short          vx[10];

short          vy[10];

};

struct rd_rgb {

double         r, g, b;

};

Each RDCPOLY request may pass a maximum of 10 polygon vertices to the driver.

If the polygon to be rendered contains more than 10 vertices, the remaining

vertices will be passed to the driver by additional RDCPOLY requests.

flags is a 16-bit, bit-coded field.  Currently defined bits for flags are as

follows:

Mnemonic    Bit  Value                    Meaning

RF_MORE       0     1    0 = no more vertices left to send

1 = additional vertices to send

RF_CONT       1     2    0 = not a continuation of previous RDCPOLY

request

1 = continuation of previous RDCPOLY request

Top 10 bits              Invisible edges

RF_MORE (flags bit 0) should be set if additional RDCPOLY requests are required

to pass additional polygon vertices to the driver.

RF_CONT (flags bit 1) should be set if the current RDCPOLY request is a contin-

uation of a previous RDCPOLY request.

The top 10 bits (bits 6 through 15) of flags are used to indicate which edges

of the polygon should not be drawn in ecolour.  Each of these bits corresponds

to an edge defined by (vx[i],vy[i]) to (vx[i+1],vy[i+1]).  If colour and

ecolour are equal, then the driver should ignore the invisible edges, since any

border it draws would be invisible.

For example, if flags bit 6 is set, the edge from (vx[0],vy[0]) to

(vx[1],vy[1]) should not be drawn.  If flags bit 7 is set, the edge from

(vx[1],vy[1]) to (vx[2],vy[2]) should not be drawn.  If flags bit 8 is set, the

edge from (vx[2],vy[2]) to (vx[3],vy[3]) should not be drawn, etc.

colour and ecolour are structures of type rd_rgb that speciry the RGB compo-

nents of the face color and of the polygon's edges, respectively. In the cur-

102

(4) ADI Rendering Device

rent implementation, the two are always equal so that no edge outlining need be

performed.  However, ecolour is used whenever nvert is 1 or 2 (see description

of nvert below).

shadef is the shading factor to be applied to the basic color of the face.

Values for shadef will be within the range specified by the minshade and max-

shade fields of the RDCRANGE request packet.

The actual color components used to draw the face are calculated by multiplying

the r, g, and b components of colour and ecolour by the shade factor, shadef.

Colors are expressed in this form so that faces with common source color may be

identified.

If colored light sources are present, they will

be reflected in the r, g, and b components of

colour and ecolour.

nvert contains the total number of vertices in the polygon, not the number of

vertices contained in the vertex list (unless the total number of vertices in

the polygon is 10 or less).

If multiple RDCPOLY requests are issued to the driver to pass a complete poly-

gon, the driver must keep track of the number of vertices passed with each

RDCPOLY request in order to determine the number of RDCPOLY requests required

to complete the polygon, as well as the number of vertices it will receive in

the last RDCPOLY request.

The last RDCPOLY request issued for a given polygon will always have RF_MORE

(flags bit 0) cleared (0).

Two special cases exist where the driver receives an RDCPOLY request, but does

not draw a polygon.  If nvert is 1, a single pixel is drawn with the color

specified by ecolour at coordinates (vx[0],vy[0]).  If nvert is 2, a vector is

drawn with the color specified by ecolour from coordinates (vx[0],vy[0]) to

(vx[1],vy[1]).

vx[] and vy[] may contain up to 10 vertices per each RDCPOLY request.

103

104

Chapter 5

ADI Digitizer Specification

This chapter describes the interface for ADI digitizer drivers.

The ADI Driver Development Kit includes a demonstration driver for the Micro-

soft Mouse (DGDEMO.C).  This demonstration driver includes functions common to

all ADI drivers ("#include ADI.H"), and functions specific to ADI digitizer

drivers ("#include DGADI.H"), and should be linked with the object module

assembled from IWAIT.ASM.

The digitizer driver must preserve all segment registers as well as the SI and

DI registers.

Digitizer Driver Requests

--------------------  INIT (1)  --------------------

The INIT request is the initial request issued to the driver.

Parameters:  Code    (AX):   1

Arg1    (BX):   (0,INTLEVEL)

Return values:  Res1 (AX):   0  =  unsuccessful initialization

1  =  successful initialization

Res2    (BX):   0  =  relative pointing device

1  =  digitizing tablet

Res3    (CX):   0  =  pointer sends continuous samples

1  =  pointer pauses between samples

Upon receiving an INIT request, the driver should perform any required initial-

ization, and return a status code in AX.

Prior to issuing an INIT request to the driver, the Autodesk product tests

whether the configured interrupt vector is nonzero.  If it is zero, the message

** No driver is installed for interrupt vector 0XX **

is issued and the driver disabled.

INTLEVEL (BL) contains a value identifying the version of the driver interface

for upward compatibility.  The current version of the driver interface passes

the value 1 in INTLEVEL.

105

Autodesk Device Interface/ADI

The driver should return 1 in AX if the initialization was successful.  Other-

wise it should return 0 in AX.

If the initialization was successful, BX contains a value that identifies the

type of pointing device.  If BX is 0, the device is configured as a relative

pointing device such as a mouse, and tablet menus and table mode are disabled.

If BX is 1, the device is configured as a tablet and all tablet-related com-

mands are enabled.

If the initialization was successful, CX contains a value that indicates

whether the pointing device sends continuous samples or pauses between samples.

If the pointer always responds with a sample, or if pauses are meaningful (for

example, if pauses indicate that the cursor has been removed from the tablet

surface) the driver should return 0 in CX.

If the device occasionally returns no sample because of its inherent properties

(for example, a device that only reports changes in coordinates) the driver

should return 1 in CX.

Returning an invalid value in CX can cause the

crosshairs to blink.

If the driver returns 0 in AX, indicating initialization failure, the message

Could not initialize pointer driver.

is issued and the pointing device disabled.

--------------------  TERM (2)  --------------------

The TERM request is issued when the drawing editor returns to the AutoCAD Main

Menu.

Parameters:  Code    (AX):   2

Return values:  none

This request instructs the driver to terminate.

--------------------  SENSE (3)  --------------------

The SENSE request is issued by the program whenever digitizer input is

required.

106

(5) ADI Digitizer

Parameters:  Code    (AX):   3

Return values:  see below

Point coordinates returned by the digitizer driver should fall within the range

0 to 20480 with (0,0) at the lower left corner of the tablet and (20480,20480)

at the upper right.  Even relative pointing devices, such as mice, must return

absolute coordinates in the range 0 to 20480.  If a digitizer tablet has dif-

fering height and width, the full range of the smaller dimension should be

scaled from 0 to 20480, with the larger dimension's coordinates using the same

scale.  This scheme will result in the larger dimension's coordinates exceeding

20480, though they must remain less than 32768.

If Tablet Mode is to function properly, vertical and horizontal digitizer coor-

dinates must represent equal physical distances.

The driver should return one of the following status codes in AX, with addi-

tional arguments as indicated:

Response      AX     BX       CX       DX

Nothing          0

Tracking point   2   X-coord   Y-coord

Picked point     3   X-coord   Y-coord

Button pick      4   Button #

Button + coords  5   Button #  X-coord  Y-coord

AX = 0 indicates the pointer has nothing to report.  No action is taken by the

program.

The driver may return AX = 0 when waiting for a

button to be released following a selection.

The Autodesk product does not perform this func-

tion.  It is the responsibility of the installed

ADI driver, as is any debouncing of buttons that

may be required.

AX = 2 indicates a tracking coordinate pair is returned in BX and CX.  The X

coordinate returned in BX must fall within the range from 0 to 20480, inclu-

sive.  The Y coordinate returned in CX must also fall within this range.  This

code is used to return coordinates when the pick button is not pressed.  In

this case, the program will track with the screen or menu cursor.

AX = 3 indicates a picked coordinate pair is returned in BX and CX.  The X

coordinate returned in BX must fall within the range from 0 to 20480, inclu-

sive.  The Y coordinate returned in CX must also fall within this range.  This

code is used to return the coordinates when the pick button is pressed.

AX = 4 indicates a button has been pressed, which generates a button-menu pick.

The button number is returned in BX, with the first button indicated by 0, the

second by 1, and so on.

107

Autodesk Device Interface/ADI

AX = 5 indicates a button has been pressed, which generates a button-menu pick,

and that the driver has returned the coordinates of the pointer at the time of

the button pick.  The button number is returned in BX, the X coordinate (0 to

20480) in CX, and the Y coordinate in DX.  Drivers that return the current

pointer position with button-menu picks should return status code 5 and return

the coordinates.  Drivers that lack this capability should return status code

4, described above.

If the driver returns any value other than those given above in AX, the mes-

sage:

** Pointer driver returned illegal code X.  Pointer disabled **

is issued and the pointer disabled.

ADI Digitizer Test Procedures

These test procedures are for use with AutoCAD.  If you intend to use your ADI

digitizer driver with AutoSketch, we suggest that you create a simple drawing

with AutoSketch and edit it using all the options in the Change menu.  Be sure

that you are able to access the entire drawing area and that picking works cor-

rectly.

1. Set up your system for the digitizer you are testing.  Cable the digi-

tizer according to the instructions you will provide the user for this

purpose.

2. Configure AutoCAD for the digitizer.

3. Start a new drawing.  Crosshairs should appear on the screen immedi-

ately after bringing up the drawing.  Be sure that the pointing device

moves the crosshairs smoothly across the screen and that the

crosshairs can be moved over the entire drawing screen.  Moving the

pointing device from the bottom of the digitizer pointing area to the

top should move the crosshairs from the bottom of the screen to the

top (into the menu bar, if that is present).  Moving the pointing

device from the extreme left of the digitizer pointing area to the

right should move the crosshairs from the left of the screen to the

right (into the menu area if the menu area is configured ON).

Enter the LINE command and ensure that your pointer's

"pick" button allows point selection.

4. If you are using the standard AutoCAD menu, and the pointing device

has multiple buttons, ensure that the button menu options work as doc-

umented in Appendix A of the current AutoCAD Reference Manual.  For a

custom menu, check the documentation supplied with that menu.

5. Test the pointing device in SKETCH mode.  The normal button menu

cannot be used while a SKETCH command is in progress.

108

(5) ADI Digitizer

The following table lists the SKETCH subcommands and the pointer but-

tons that correspond to them:

Command Character  Pointer Button         Function

P              "Pick"          Raise/lower pen

.  (period)            1             Line to point

R                 2             Record lines

X, Space, RETURN         3         Record lines and exit

Q, CTRL C             4        Discard lines and exit

E                 5                 Erase

C                 6                Connect

Use the following procedure to test AutoCAD operations that are specific to

digitizing tablets.

1. If the tablet is large enough, use the standard AutoCAD tablet tem-

plate to test for proper operation of tablet menus.  Use the TABLET

CFG command to configure the four menu areas and the screen pointing

area.  Then try selecting several boxes from each of the four menu

areas.

2. To test Tablet mode, use the TABLET CAL command to calibrate the

tablet to area with known dimensions and check various points (with

tablet mode ON) for accuracy.  When calibrating the tablet, choose two

points that require the command to perform coordinate calculations

more difficult than those required by picking the origin and another

point on either the X or Y axis.

3. Moving the pointer out of proximity with the tablet should cause the

screen crosshairs to disappear, but should not interfere with keyboard

input.  Move the pointer out of proximity and type in some AutoCAD

commands.

4. If your driver supports several pointer types (puck, stylus, etc.),

run all the tests with each type of pointer.  If your driver supports

several digitizer models, run all the tests with each model.

109

110

Chapter 6

ADI Plotter Specification

The following describes the interface and configuration process for ADI plotter

drivers in AutoCAD; the actual configuration dialog in other Autodesk products

may vary slightly from that described below.

The ADI Development Kit allows users to interface Autodesk software products to

plotters and other vector-oriented devices that Autodesk does not directly sup-

port.  The driver is fully configurable and offers a variety of different

output formats.

The output from an ADI driver may be sent to an ASCII file or a binary file, or

may be configured to communicate with an installed INT driver.

Configuration

Because the driver is indeed generic, the process of configuring it is more

complex than that required by most other plotters.  All the hardware informa-

tion provided by a conventional driver must be supplied by the user when con-

figuring an ADI driver.  As a result, the dialog is extended.

First, the user specifies the driver's output format by responding to the fol-

lowing prompt:

Select output format:

0.  ASCII file

1.  Binary file

2.  AutoCAD DXB file

3.  Installed ADI driver

Output format, 0 to 3 <0>:

ASCII and binary file formats will be described later.  The DXB file format is

described in the AutoCAD Reference Manual.  The generic driver can also send

commands to a resident driver via an INT call.  If this option (3) is selected,

the additional question:

Hexadecimal interrupt code (INT 0XXh) <78>:

is issued.  In response to this prompt, the user should supply the interrupt

vector to be used to communicate with the plotter driver.  The default is 78h,

however any vector from 1 to 0FFh is permissible.  If an invalid value is

entered, the message:

** Interrupt number must be between 1 and 0FF

111

Autodesk Device Interface/ADI

is displayed and the prompt reissued. Following the output format dialog, the

user is asked whether the plotter is multi-pen:

Does the plotter have multiple pens? <N>

If the user answers 'YES' to this question, the following prompt is issued:

How many pens does it have, 2 to 127 <2>:

Next, information on plotter-implemented line styles is requested with the fol-

lowing questions:

Does the plotter have multiple dashed line styles? <N>

How many line styles are there, 2 to 127 <5>:

If the plotter provides multiple pen speeds, the user may define them by

responding the following questions:

Is the plotter variable speed? <N>

What is the numeric code for the fastest speed, 1 to 127 <10>

These questions supply information which is normally provided by the hardware

definition of a plotter within a driver. It is used to enable features in the

plot setup process and to provide rudimentary range checking of user specifica-

tions.

Next the maximum plot size and plotter step size are requested. Values may be

specified either in inches or millimeters. The units are selected by the fol-

lowing prompt:

Next you will specify the maximum plot size the plotter can make,

and the step size of the plotter.  You may specify these either in

millimeters or inches.  The next question selects the units.

Specify plot size in millimeters? <N>

If the user responds with the default, 'NO' the prompts that follow and their

responses will refer to inches; otherwise, they will refer to millimeters.  The

examples given below assume inches were selected.

Maximum horizontal (X) plot size in inches? <11.0>

Plotter steps per inch in the horizontal (X) direction: <1000.0>

Maximum vertical (Y) plot size in inches? <8.5>

Plotter steps per inch in the vertical (Y) direction: <1000.0>

These parameters are floating point numbers used to compute the plotter size in

plotter steps.  The calculated step count may not exceed 65535.  If the speci-

fications given exceed the limit, the following message is displayed (after the

steps per inch are specified):

** Error: Plot size multiplied by steps per inch cannot exceed 65535.

Specifications resulted in ?????.

112

(6) ADI Plotter

The question marks in the prompt are replaced by the step count value entered

by the user.  After displaying the above error message, the configuration pro-

cedure reissues the prompts for plot size and steps per unit.

Once the driver-specific configuration procedure is completed, the regular

plotter configuration procedure is performed.  The generic plotter driver

remembers all its configuration options and supplies them as defaults during

subsequent configurations.  The defaults given in the prompts above are those

assumed during initial configuration of an ADI plotter driver.

Using the Driver

If ASCII, binary, or DXB output format has been selected, the program will

prompt for a file name following the runtime plot configuration dialog:

Enter file name for plot:

The user should respond with the name of the file to which the plot data should

be written.  The name may be entered with a file name extension, if desired.

If none is specified, the extension .PLT (or .DXB for DXB file output) will be

used.  If the file cannot be opened, the program issues the following prompt:

Cannot open this file.  Try another name? <N>

If answered affirmatively, the file name prompt is reissued; otherwise, the

plot will be canceled.

Output Format

Regardless of whether the output is being written to a file or passed directly

to an installed driver, plot data is represented in a uniform fashion.  Each

plot command consists of a type code, possibly followed by arguments.  The fol-

lowing table list all valid plot codes and their arguments:

Command     Code  Type   Argument 1    Argument 2

Begin plot      1     B     File level

End plot        2     A

Move            3     C    X coordinate  Y coordinate

Draw            4     C    X coordinate  Y coordinate

New pen         5     B     Pen number

Select speed    6     B     Speed code

Set line type   7     B     Line type

Raise pen       8     A

Abort plot      9     A

The Type field in the above table specifies how the information is represented

in the file or passed to the driver.  This field is discussed in detail in the

File Format section below.

113

Autodesk Device Interface/ADI

The first command issued for a plot is the BEGIN PLOT command (code 1).  The

BEGIN PLOT command has a single argument, file level, which identifies the ver-

sion of the plot file which follows.  Plots generated by this version of the

ADI driver will have a file level of 1.

The RAISE PEN command (code 8) allows the driver to prepare for a possible

manual pen change (for example, by raising the pen and moving it to the middle

of the carriage).  However, the plotter should not physically change pens in

response to this command; that action is requested by the NEW PEN command

described below.  A RAISE PEN is issued at the beginning of every plot and

before every pen change.  It also occurs occasionally in the middle of a plot,

if the Autodesk software decides that the pen has been down too long in one

place.  This helps prevent puddling when database processing (skipping of

objects on turned-off layers, for instance) results in a long pause between

vectors.

The NEW PEN command (code 5) physically selects a new pen.  This command has a

single argument that specifies the pen number (refer to the chapter on "Plot-

ting" in the AutoCAD Reference Manual for information on how AutoCAD handles

the assignment of plotter pens to drawing colors).

The SELECT SPEED command (code 6) selects the drawing speed for a variable

speed plotter.  This command has a single argument that selects the speed.  The

argument is range checked against the maximum plotter speed specified during

configuration, but is not otherwise checked.  For a plotter with discrete

speeds, the driver should verify the speed argument.  If the value exceeds the

capabilities of the plotter, the driver should send the next SLOWER speed.

The SET LINE TYPE command (code 7) selects a plotter line font.  This command

has a single argument that selects the line font.  It is a universal convention

within AutoCAD that zero selects a continuous (solid line).  The meaning of the

other fonts is up to the driver.

The MOVE command (code 3) has two arguments, specifying the absolute coordi-

nates to which the pen should be moved with the pen raised.  The coordinates

are specified X first, Y second, both as unsigned numbers in the inclusive

range from 0 to 65535.

The DRAW command (code 4) has two arguments, specifying the absolute coordi-

nates to which the pen should be moved with the pen lowered.  The coordinates

are specified X first, Y second, both as unsigned numbers in the inclusive

range from 0 to 65535.

The END PLOT command (code 2) is issued at the end of a plot.  This command has

no arguments.

The ABORT PLOT command (code 9) is relevant only for installed drivers.  This

command will never be encountered in a plot file because aborting the plot

causes the output file to be deleted.  This command is passed to an installed

driver to indicate that the plot has been terminated by the user, and that

abnormal termination action should be taken (flushing a buffer, for example).

The ABORT PLOT command is followed by an END PLOT command, which should perform

the usual end-of-plot functions (such as putting the pen back).

114

(6) ADI Plotter

File Format

If the output format is an ASCII file, each plot command will appear on a new

line, starting in the first text column.  All command codes and arguments

appear as decimal numbers, separated by commas.  Type A codes appear by them-

selves.  Type B codes consist of the command code followed by the argument.

Type C codes consist of the command code, followed by the X coordinate and Y

coordinate, comma delimited.  All numbers appearing in a plot file are

unsigned.  Arguments to type B codes are in the range from 0 to 127, and argu-

ments to type C codes range from 0 to 65535.  The following is an actual plot

file generated by plotting a square.  The annotations did not appear in the

original file, but were added to explain the commands.

1,1              Begin plot, file level 1

8                Raise pen (start of pass)

5,1              Select new pen 1

7,0              Select line type 0 (solid)

6,1              Select speed 1

3,0,6953         Move to 0, 6953 - pen up

4,6954,6953      Draw to 6954, 6953 - top of square

4,6954,0         Draw to 6954, 0 - right side

4,0,0            Draw to 0,0 - bottom

4,0,6953         Draw to 0, 6953 - left side

2                End of plot

In a binary file, the codes and arguments appear precisely as they do in an

ASCII file, but are stored as binary numbers.  Type A codes are written as

single bytes.  Type B codes consist of a single-byte code, followed by a one-

byte argument.  Type C codes are written as a single-byte code, followed by two

16-bit arguments representing the X and Y coordinates.  Each 16-bit argument is

written least significant byte first, most significant byte last, the standard

8086 representation.  The following is a hexadecimal dump of a binary file gen-

erated by the same drawing which created the ASCII file above.  Spacing has

been added to the dump to aid readability.

01 01

08

05 01

07 00

06 01

03 00 00 29 1B

04 2A 1B 29 1B

04 2A 1B 00 00

04 00 00 00 00

04 00 00 29 1B

02

Installed Driver Operation

The generic plotter driver (PLGEN.DRV) can communicate with an installed

driver.  This allows the development of a driver which, once loaded in memory,

115

Autodesk Device Interface/ADI

acts exactly like a driver implemented within AutoCAD itself: plots are per-

formed in real time, without the need for an intermediate file.  Installed

drivers are implemented as MS-DOS programs that terminate and remain resident.

They are not true DOS device drivers (although an alternate approach is to use

a DOS device driver and send the file output to it).

The installed driver should patch itself into a vacant interrupt location then

exit to DOS, remaining resident in memory. Installation of the driver must be

accomplished before AutoCAD is executed. The standard interrupt vector for

printer/plotter drivers is 078 hexadecimal.

When configuring the generic driver, the installed driver is selected by choos-

ing option 3 in response to the output format prompt. The interrupt vector at

which the driver installed itself must also be specified at configuration time.

When a plot is made, the generic plotter driver verifies that the driver is

installed by testing the value of the interrupt vector, and will only proceed

if it is nonzero.  If the interrupt vector is zero, the message:

** No driver is installed for interrupt vector 0XX **

is issued, specifying the interrupt vector in hexadecimal, and the plot will be

canceled.

If the driver is properly installed, each plot command is sent to it by loading

the command code and arguments into registers and then calling the driver with

an INT instruction to the configured vector.  The command code is always passed

in AX.  For type B commands, the first argument is passed in BX.  For type C

commands, the X coordinate is passed in BX and the Y coordinate in CX.

The driver should return with AX zero.  (This is not currently checked, but may

be used in the future for a driver status).  The driver must preserve the con-

tents of all segment registers, as well as the SI and DI registers.

ADI Plotter Test Procedures

Use the PLTST2.DWG to test your plotter driver.

1. Configure AutoCAD for the plotter to be tested.  Cable the plotter

according to the instructions you will provide the user for this pur-

pose.

2. Scaling - Plot the view named SCALE to a specified scale (e.g.1=1).

Measure the dimensioned parts of the drawing checking that they are

plotted to the scale specified.

3. 90 Degree Rotation - Plot the drawing specifying 90-degree rotation,

MAX size paper, and FIT.  The plot should be rotated 90 degrees.  The

plot should not be truncated.  If it is, the paper size is incorrect.

4. Linetypes, Pen changes, and Pen speeds - If the plotter driver sup-

ports multiple pens, line types, or pen speeds, use AutoCAD's color

116

(6) ADI Plotter

assignment dialogue to select various combinations of these.  Ensure

that each pen, line type, and speed is accessible.  change the layers

to different pen numbers and pen speeds supported on the plotter.

Plot the drawing with representative combinations, and verify that the

appropriate line types and pens were used.  If the plotter supports

only one pen, test the "pause for pen change" option.

5. Origin Offset - Plot the drawing, specifying an offset from the

origin.  The plot should move the specified number of units from the

origin.

6. MAX Size - Plot EXTENTS, MAX size, FIT, no rotation. The drawing

should not be truncated.

7. If your driver supports several plotter resolutions, run the above

tests at each resolution.

8. If your driver supports several plotter models, run the above tests on

each model of plotter.

117

118

Chapter 7

ADI Printer/Plotter Specification

The following describes the interface and configuration process for ADI

printer/plotter drivers in AutoCAD; the actual configuration dialog in other

Autodesk products may vary slightly from that described below.

The ADI Development Kit allows manufacturers and users to interface Autodesk

software products to printers that Autodesk does not directly support.  The

sample included in the ADI Development Kit demonstrates a printer/plotter that

simply uses asterisks and spaces for output.

Configuration

Printer/plotters come in a variety of resolutions and sizes, and some support

color output.  The program must know these parameters before it can begin a

plot.  These parameters are set during configuration of the ADI

printer/plotter.

First, maximum paper size and resolution information are requested.  Values may

be specified either in inches or millimeters.  The units are selected by the

following prompt:

Next you will specify the maximum plot size the printer can make,

and its resolution (dots per unit).  You may specify these either

in millimeters or inches.  The next question selects the units.

Specify plot size in millimeters? <N>

If the user responds with the default, 'NO' the prompts that follow and their

responses will refer to inches; otherwise, they will refer to millimeters.  The

examples given below assume inches were selected.

Maximum horizontal (X) plot size in inches? <11.0>

Printer dots per inch in the horizontal (X) direction: <100.0>

Maximum vertical (Y) plot size in inches? <8.5>

Plotter dots per inch in the vertical (Y) direction: <100.0>

These parameters are floating-point numbers which are used to compute the

printer X and Y size in dots.  The calculated sizes may not exceed 65535.  If

the specifications given exceed the limit, the following message is issued

(after the dots per unit are specified):

** Error: Plot size multiplied by dots per inch cannot exceed 65535.

Specifications resulted in ?????.

119

Autodesk Device Interface/ADI

The question marks in the prompt are replaced by the size value entered by the

user.  After displaying the above error message, the configuration procedure

reissues the prompts for plot size and dots per unit.

Finally, the user specifies the driver's output format by responding to the

following prompt:

Select output format:

0.  Binary file

1.  CAD/camera image file

2.  Installed ADI driver

Output format, 0 to 2 <0>:

The binary file format is described below.  The CAD/camera image file format is

described in the AutoCAD Drafting Package Installation and Performance Guide

under "ADI Printer/Plotter." The generic driver can also send commands to a

resident driver via an INT call.  If this option (2) is selected, the

additional question:

Hexadecimal interrupt code (INT 0XXh) <7B>:

is issued.  In response to this prompt, the user should supply the interrupt

vector to be used to communicate with the printer/plotter driver.  The default

is 7Bh, however any vector from 1 to 0FFh is permissible.  If an invalid value

is entered, the message:

** Interrupt number must be between 1 and 0FF

is displayed and the prompt reissued.

If binary file format is selected, the user must specify whether color informa-

tion is to be written to the file (an installed driver provides this informa-

tion to to the program at initialization time, so this question does not appear

if an installed driver is selected.  The prompt is:

Does the printer/plotter support color? <N>

If the user responds with the default 'NO' the output file will be one bit per

pixel.  If color is selected, the output will be four bits per pixel, and will

contain color information.  See the AutoCAD Drafting Package Installation and

Performance Guide for examples of how various printers handle color informa-

tion.

Once the driver-specific configuration procedure is completed, the regular

printer/plotter configuration procedure is performed.  The generic

printer/plotter driver remembers all its configuration options and supplies

them as defaults during subsequent configurations.  The defaults given in the

prompts above are those assumed during initial configuration of an ADI

printer/plotter driver.

120

(7) ADI Printer/Plotter

Using the Driver

If binary file output format has been selected, the program will prompt for a

file name following the runtime printer plot configuration dialog:

Enter file name for plot:

The user should respond with the name of the file to which the printer plot

data should be written.  The name may be entered with a file name extension, if

desired.  If none is specified, the extension ".PRP" (or ".IMG" form CAD/camera

image file output) is used.  If the file cannot be opened, the program issues

the following prompt:

Cannot open this file.  Try another name? <N>

If answered affirmatively, the file name prompt is reissued; otherwise, the

plot will be canceled.

Output Format

Regardless of whether the output is being written to a binary file or passed

directly to an installed driver, printer plot data is represented in a uniform

fashion.  Each record in the file or message to the installed driver begins

with a single 16-bit value.  If the sign bit is set in this value, it is a con-

trol function, otherwise it is a count specifying the number of bytes of plot

data that follow.  The function codes are given below.

Function    Code          Arguments

Begin plot  8001h  FILE LEVEL, XDOTS, YDOTS,

COLOR

End plot    8002h

Abort plot  8003h

A binary file always begins with the BEGIN PLOT code (all 16-bit values in the

file are written in standard 8086 fashion: least significant byte first, most

significant byte last), followed by the 16-bit values FILE LEVEL, XDOTS, YDOTS,

and COLOR.  FILE LEVEL identifies the format of the file for upward compatibil-

ity with future extensions.  Files written by this version of the driver will

have a file level of 1.  XDOTS and YDOTS specify the horizontal and vertical

size, respectively, of the printer for which the file is generated.  COLOR is 0

if the file is monochrome format (one bit per pixel) and 1 if the file is color

format (4 bits per pixel).

Following the BEGIN PLOT record are zero or more line records.  Each line

record consists of a 16-bit byte count followed by that number of bytes of

raster data.  If the printer/plotter was configured as supporting color, each

byte will contain two pixels, with 4 bits per pixel indicating color.  If the

printer/plotter was configured as monochrome, 8 one-bit pixels will be stored

per byte.  In either case, pixels are arranged on the page left to right start-

ing with the most significant pixel of each byte, proceeding to the least sig-

nificant, then continuing to the next byte.  Records in the file are variable

121

Autodesk Device Interface/ADI

length; trailing zero bytes are not written.  A totally blank line on the

printer will be written with a byte count of 0 and no following data bytes.

Totally blank lines at the end of the plot are not written to the file.  Conse-

quently there may be fewer line records in the file than the YDOTS count given

in the BEGIN PLOT record.

The end of the plot is indicated by an END PLOT record instead of a byte count

word.

The ABORT PLOT code will never be encountered in a printer plot file because

aborting the plot causes the output file to be deleted.  It is passed to an

installed driver to apprise it of the abnormal termination of the printer plot.

Installed Driver Operation

The generic printer/plotter driver (PPGEN.DRV) can communicate with an

installed driver.  This allows the development of a driver which, once loaded

in memory, acts exactly like a driver implemented within AutoCAD itself:

printer plots are performed in real time, without the need for an intermediate

file.  Installed drivers are implemented as DOS programs that "terminate and

remain resident"; they are not true DOS device drivers.

The installed driver should patch itself into a vacant interrupt location then

exit to DOS, remaining resident in memory.  Installation of the driver must be

accomplished before AutoCAD is executed.  The standard interrupt vector for

printer/plotter drivers is 07B hexadecimal.

When configuring the generic driver, the installed driver is selected by choos-

ing option 2 in response to the output format prompt.  The interrupt vector at

which the driver installed itself must also be specified at configuration time.

When a printer plot is made, the generic plotter driver verifies that the

driver is installed by testing the value of the interrupt vector, and will only

proceed if it is nonzero.  If the interrupt vector is zero, the message:

** No printer driver is installed at interrupt vector 0XX **

is issued, specifying the interrupt vector in hexadecimal, and the plot will be

canceled.

Commands are passed to the installed driver through the registers.  The func-

tion code or length in bytes of the data is passed in the AX register.  The

driver should examine AX and react accordingly.

The BEGIN PLOT code is passed with the FILE LEVEL in BX.  The configured size

of the printer/plotter in dots is passed with the horizontal size in CX and the

vertical size in DX.  The driver must return with AX containing a status code:

1 if the printer was successfully initialized and 0 otherwise.  BX should

return a code that specifies monochrome or color output format, 0 for mono-

chrome (1bit per pixel), 1 for color (4 bits per pixel).

122

(7) ADI Printer/Plotter

Successive lines of plot data are passed with the length in bytes of each line

in AX.  BX contains the offset address of the line of data,and CX contains the

segment address of the printer data (thus, the full 32 bit address of the line

is in CX:BX).  DX contains the vertical position of the line on the page (since

blank lines within the plot are output, most drivers will not need this value

and may ignore it).  If the line is blank (AX is zero); CX and BX are meaning-

less.

The end of the plot is signaled by the END PLOT code in AX.  The driver should

eject the page and close out the printer plot.

If the printer plot is aborted by the user, the driver will receive the ABORT

PLOT code in AX.  If possible, the driver should discard information buffered

for the printer.  The driver will receive an ENDPLOT code after the ABORT PLOT

code.

A sample driver, intended to illustrate how an installed driver operates, and

to exercise the facility for testing, is provided on the disk as PPDEMO.C and

PPDEMO.EXE.

This driver sends its data to the printer in "ultimate dumb printer/plotter

mode": zero bits are printed as spaces and one bits as asterisks.  To configure

a 132 column printer for use with this driver, answer the configuration ques-

tions as follows:

No          <use inches>

13.2        <paper width>

10          <characters per inch horizontal>

10          <usable paper height>

6           <characters per inch vertical>

This driver supports only monochrome format (one bit per pixel).  The driver

will read a binary output file and send it to the printer when called with the

command:

PPDEMO <file>

The extension .PRP is supplied automatically.  The driver may be installed into

interrupt vector 07Bh by calling with the command:

PPDEMO -I

Once installed, printer/plotter output may be sent directly to it by using the

installed driver output option.

ADI Printer/Plotter Test Procedures

Use the test procedures outlined under ADI Plotter Test Procedures in the pre-

vious chapter to test your completed ADI Printer/Plotter driver.

In addition, for color printer/plotters, check that the colors assigned to the

AutoCAD color numbers plot as desired (and as noted in the documentation you

123

Autodesk Device Interface/ADI

will supply to the user).  Note that color numbers that are multiple of 16 are

not currently plotted by AutoCAD.

124

Chapter 8

Developing UNIX ADI Drivers

This chapter describes the calling sequences, conventions, and coding practices

used in UNIX ADI drivers for digitizers, plotters and printer plotters.  It

assumes that you are familiar with the MS-DOS ADI drivers, described in the

preceding chapters.

UNIX Sample Drivers and Development Aids

The ADI Driver Development Kit contains several examples and tools to aid you

in writing and debugging a UNIX ADI device driver.  The files provided include:

o  uadi.h - Definitions for all the structures and defines.

o  makefile - UNIX makefile to build all the drivers.  (dgadi, pladi,

pladip, and ppadi).

o  dgadi.c - Main program for ADI digitizer driver.  This is a completely

functional driver for Hitachi digitizers.

o  dghtcom.c - Hitachi Hicomscan digitizer driver.  Called by dgadi.c.

The driver is currently configured for a Hitachi 1111B with a stylus.

o  sio420.c - Serial port driver for BSD 4.2 systems.  Used by dghtcom.c.

o  pladip.c - Main program implementing an ADI plotter driver that out-

puts UNIX Plot(5) format files.

o  pladi.c - Main program for an ADI plotter that prints the AutoCAD plot

records.

o  ppadi.c - Main program for an ADI printer plotter that prints the

AutoCAD printer plotter records.

Configuration

When you select an installed ADI driver under UNIX, AutoCAD issues the follow-

ing prompt:

Enter the ADI device filename <filename>:

where device is the type of peripheral you are installing (digitizer, plotter

or printer/plotter).  Enter the filename of the installable ADI driver.  The

filename may be preceded by a complete path specification.  If no path is

given, AutoCAD uses the PATH environment variable to search for the ADI driver.

125

Autodesk Device Interface/ADI

The specified ADI driver is loaded automatically by AutoCAD when the drawing

editor is entered.

AutoCAD Loads UNIX ADI Drivers Automatically

When running under UNIX, AutoCAD invokes each ADI driver as a child process by

searching the PATH environment variable for the ADI filename specified during

the configuration process.  Communication between AutoCAD and the driver is

done using UNIX pipes.  The file descriptors for the driver's read and write

pipes are passed to the driver as the first and second command line arguments,

respectively.  In addition to the read and write pipes, AutoCAD preserves the

original standard input, output, and error files.  Be aware, however, that the

standard input, output, and error descriptors on most window-based systems will

probably be the original windows in which AutoCAD was invoked, rather than the

AutoCAD windows.  Also, if AutoCAD was invoked as a background process, it may

not be possible for the ADI driver to read or write to the standard input and

standard output.

After the driver is loaded, AutoCAD performs a read on the pipe for the

driver's "heartbeat".  AutoCAD expects to read two bytes (a short).  Thus,

after initialization, the driver must write two bytes to its write pipe to

release AutoCAD from the read.  A value of 1 indicates that the driver was suc-

cessfully loaded; any other value indicates that the driver was not

successfully loaded.

When reading/writing 2-byte values from/to the

pipe, AutoCAD accesses the value as an array of

two chars, reading or writing the first char

followed by the second char. The driver should

do the same to avoid swapping the bytes of the

value.

After AutoCAD receives the heartbeat, it sends an initialization packet to the

driver.  The format of the initialization packet and subsequent data packets is

different for each driver type.  Specific packet formats are given in the fol-

lowing sections.

UNIX ADI Digitizer Driver

Digitizer Initialization Packet

After receiving the heartbeat, AutoCAD sends the driver a digitizer initializa-

tion packet, as follows:

126

(8) Developing UNIX ADI Drivers

struct dgipkt {

short   dgcode;

short   level;

short   tablet;

short   pauses;

};

#define DGIPKTSIZE (sizeof(struct dgipkt))

To receive this initialization packet, the driver should read DGIPKTSIZE bytes

from its read pipe.  The fields of the digitizer initialization packet are

defined as follows:

Field                       Meaning

dgcode  Contains the command code DGINIT (1).  This

field is also used by the driver to return a

status code of 1 if the driver initialization is

successful, and 0 if initialization fails.

level   Identifies the format of the packets for compat-

ibility with future revisions.  The level passed

by this version is 1.

tablet  Set to 1 by the driver if the device is config-

ured as a tablet.  This enables all of AutoCAD's

tablet-related commands.  If tablet is set to 0,

the device is configured as a relative pointing

device such as a mouse; tablet menus and tablet

mode are disabled.

pauses  Set by the driver to indicate whether the point-

ing device returns samples when the cursor is

stationary.  If the pointer always responds with

a sample, or if pauses are meaningful (for exam-

ple, if pauses indicate that the cursor has been

removed from the tablet surface), the driver

should return 0.  If the device occasionally

returns no sample because of its inherent prop-

erties (for example, a device such as a mouse

that only reports changes in coordinates), the

driver should return 1.  Returning an invalid

value in pauses can cause the crosshairs to

blink.

If the driver accepts this initialization request, it should set tablet and

pauses appropriately and send the packet back to AutoCAD by writing the packet

(DGIPKTSIZE bytes) to the write pipe.  If the driver cannot accept the initial-

ization request it should set the dgcode field to 0, send the packet to

AutoCAD, and exit.

Digitizer Command Packet

After the initialization packet has been returned, AutoCAD begins sending com-

mand packets.  For each command packet the digitizer receives, it must compose

a reply packet and send it back to AutoCAD.  The time between when the driver

receives the command packet and the when the driver sends the reply should be

127

Autodesk Device Interface/ADI

as short as possible, since AutoCAD blocks on a read of the reply.  If the

delay is longer than 50 to 100 milliseconds, AutoCAD's cursor will appear jumpy

and unresponsive.  Command packets are defined as follows:

struct dgpkt {

short   dgcode;

short   buttn;

short   x, y;

};

#define DGPKTSIZE (sizeof(struct dgpkt))

The driver receives a command packet by reading DGPKTSIZE bytes from its read

pipe.  The fields of the digitizer command packet are defined as follows:

Field                       Meaning

dgcode  Contains the command code DGSENSE (2) or DGTERM

(3) from AutoCAD.  In response to a DGSENSE

request, the driver sets dgcode to one of the

response codes listed in the next table.  When

the driver receives a DGTERM command code in the

dgcode field of a command packet, it should

clean up and exit.

buttn  Button number returned by driver.

X    X position of cursor.

Y    Y position of cursor.

AutoCAD sends the DGSENSE command when digitizer input is required.  Response

codes that may be returned by the driver are summarized in the following table:

dgcode   Value      Buttn           X             Y

P_NONE     0

P_COORD    2                   X-coordinate  Y-coordinate

P_POINT    3                   X-coordinate  Y-coordinate

P_BUTTN    4    Button number

P_BUTTP    5    Button number  X-coordinate  Y-coordinate

P_NONE indicates that the pointer has nothing to report.  No action is taken by

AutoCAD.  The driver may return this code when waiting for a button to be

released following a selection.  AutoCAD does not perform this function; it is

the responsibility of the installed driver, as is any debouncing of buttons

that may be required.

P_COORD indicates that a tracking coordinate pair is returned in X and Y.  The

X and Y coordinates returned must fall within the range from 0 to 20480 inclu-

sive.  P_COORD should be used to return coordinates when the pick button is not

pressed.  In this case, AutoCAD will track with the screen or menu cursor.

P_POINT indicates that a picked coordinate pair is returned in X and Y.  The X

and Y coordinates returned must fall within the range from 0 to 20480 inclu-

sive.  P_POINT is used to return the coordinates when the pick button is

pressed.

128

(8) Developing UNIX ADI Drivers

P_BUTTN should not be used unless the driver is unable to return the button

number and the coordinates of the pointer at the time of a button pick.  Use

P_BUTTP instead of this function if possible.

P_BUTTP indicates a button was pressed and that button number and the coordi-

nates of the pointer at the time of the button pick are returned.  The button

number is returned in buttn and the coordinates of the picked point are

returned in X and Y.

UNIX ADI Plotter Driver

Plotter Initialization Packet

After AutoCAD receives the heartbeat, it sends the driver a plotter initializa-

tion packet:

struct plipkt {

short   code;

short   level;

real    plxmax, plymax;

real    plx, ply;

real    plxinc, plyinc;

real    pllwid;

short   plnpens;

short   plspdmax;

short   plltmax;

};

#define PLIPKTSIZE (sizeof(struct plipkt))

The driver receives this packet by reading PLIPKTSIZE bytes from its read pipe.

The packet fields are defined in the following table:

Field                           Meaning

code       Contains the command code BEGINPLOT (1)

level       UNIX ADI version level (currently 1).

plxmax, plymax  Maximum plot X and Y dimensions, in inches.

plx, ply     Plot X and Y dimensions, in inches, relative to

the user-supplied origin.  If the origin is (0,

0), plx and ply will be the same as plxmax and

plymax.

plxinc, plyinc  X and Y plotter steps per inch.

pllwid      Pen width (line width) in inches.

plnpens      Number of pens to be used.

plspdmax     Maximum pen speed in plotter device units.

plltmax      Maximum number of hardware line types supported

by the plotter device.

129

Autodesk Device Interface/ADI

If the driver accepts this initialization request, it should send the packet

back to AutoCAD by writing the packet (PLIPKTSIZE bytes) to the write pipe.  If

the driver cannot accept the initialization request it should set the code

field to zero, send the packet to AutoCAD, and exit.  The driver can change any

of the values in the packet; setting the code field to 2 will cause AutoCAD to

use the values supplied by the driver.

This feature has not been tested and is not

officially supported.  Little if any checking is

performed on the values returned, and unreason-

able values may cause AutoCAD to malfunction in

unpredictable ways.

Plotter Data Packets

After the driver has returned the initialization packet, AutoCAD sends plotter

data packets.  From this point on, AutoCAD sends only; it does no more reads.

The plotter data packets are all of the same size, but the contents of each

packet varies depending on the value in the code field.  Data packets are

defined as follows:

struct plpkt {

short          code;

unsigned short x, y;

};

#define PLPKTSIZE (sizeof(struct plpkt))

To receive a data packet, the driver should read PLPKTSIZE bytes from its read

pipe.  The fields are defined as follows:

code  Contains the command code,

described below.

X, Y  Contains either a single

argument or the X and Y

values.

The following table summarizes the plotter data packet codes.

Code    Value    Argument 1     Argument 2

ENDPLOT      2

MOVE         3     X coordinate   Y coordinate

DRAW         4     X coordinate   Y coordinate

NEWPEN       5      Pen number

NEWSPEED     6      Speed code

NEWLTYPE     7    Line type code

PENRAISE     8

ABORTPLOT    9

PLOTSIZE     10      X maximum      Y maximum

130

(8) Developing UNIX ADI Drivers

ENDPLOT is output at the end of a plot.  It has no arguments.  When this code

is received, the driver should perform its cleanup and exit.

The MOVE code is followed by two arguments, specifying the absolute coordinates

to which the pen should be moved with the pen up.  The coordinates (X and Y)

are specified as unsigned short numbers from 0 to 65535 inclusive.

The DRAW code is followed by two arguments, specifying the absolute coordinates

to which the pen should be moved with the pen down.  The coordinates (X and Y)

are specified as unsigned short numbers in the inclusive range from 0 to 65535.

NEWPEN selects a pen.  The X field specifies the pen number.

NEWSPEED chooses a drawing speed for a variable speed plotter.  The X field

contains the speed.  The speed argument is range checked and will not exceed

the maximum speed configured, but it is not otherwise checked.  For a plotter

with discrete speeds, a driver processing this command which encounters a speed

number the plotter cannot handle should send the next slower speed.

NEWLTYPE selects a plotter line font.  The X field contains the value for the

line font.  It is a universal convention within AutoCAD that zero selects a

continuous (solid) line.  The meaning of the other fonts is up to the driver.

The PENRAISE code is generated before every pen change at the start of a pass

over the vectors.  It allows the plotter to prepare for a possible manual pen

change (for example, raising the pen and moving it to the middle of the car-

riage).  It is not the actual change pen command, which is the NEWPEN code.

AutoCAD sends a PENRAISE command at the start of every plot.

ABORTPLOT is sent only when the plot has been terminated by the user by press-

ing the interrupt key (normally control C).  An installed driver will receive

the code to indicate that abnormal termination action should be taken (flushing

a buffer, for example).  However, the driver should not exit at this point.

The ABORTPLOT code will be followed by an ENDPLOT code, at which time the

driver should perform the usual end of plot functions (such as putting the pen

back, and exiting).

The PLOTSIZE code is sent at the beginning of the plot.  The X and Y arguments

contain the largest horizontal and vertical coordinates that AutoCAD will use

in generating the plot.  Values for X and Y coordinates in subsequent MOVE and

DRAW commands will vary from 0 to the X and Y maximums returned in this packet.

UNIX ADI Printer/Plotter Driver

Printer/Plotter Initialization Packet

After AutoCAD receives the heartbeat, it sends the following printer/plotter

initialization packet:

131

Autodesk Device Interface/ADI

struct ppipkt {

short          code;

short          level;

unsigned short xdots, ydots;

};

#define PPIPKTSIZE (sizeof(struct ppipkt))

To receive this initialization packet, the driver reads PPIPKTSIZE bytes from

its read pipe.  The packet fields are defined as follows:

Field                          Meaning

code      Contains the command code PPBEGINPLOT (1).

level     Identifies the format of the packets for upward

compatibility with future extensions.  The level

passed by this version is 1.

xdots, ydots  Specify the horizontal and vertical size in dots

of the printer.

color     color is a value returned to AutoCAD by the

driver.  0 indicates that the output is for a

monochrome plotter (one bit per pixel), and 1

indicates that the output is for a color plotter

(4 bits per pixel).

If the driver accepts this initialization request, it should set color appro-

priately and send the packet back to AutoCAD by writing the packet (PPIPKTSIZE

bytes) to the write pipe.  If the driver cannot accept this initialization

request it should set the code field to zero, send the packet to AutoCAD, and

exit.

Printer/Plotter Data Packets

After the initialization packet has been returned, AutoCAD sends

printer/plotter data packets.  From this point on, AutoCAD sends only; it does

no more reads.  The printer plotter data packets are all of the same size, but

the contents of each packet varies depending on the value in the code field.

Printer/plotter data packets are defined as follows:

struct pppkt {

short          code;

short          count;

unsigned short y;

};

#define PPPKTSIZE (sizeof(struct pppkt))

To receive a printer plotter data packet, the driver reads PPPKTSIZE bytes from

its read pipe.  The fields are defined as follows:

code   Contains the command code,

described below.

count  Contains the number of bytes

to follow in a data record.

continued ...

132

(8) Developing UNIX ADI Drivers

code   Contains the command code,

described below.

Y    Contains the vertical posi-

tion of the following data

on the page.

The following table summarizes the printer plotter data packet codes.

Code     Value       Count               Y

PPPLOTDATA     2    Data byte count  Vertical position

PPENDPLOT      3

PPABORTPLOT    4

PPPLOTDATA is sent when AutoCAD has a line of rasterized data for the plotter.

count indicates the number of data bytes contained in the rasterized line, and

Y contains the vertical position of the line on the page.  Blank lines (count =

0) are not sent to the driver.  After sending the PPPLOTDATA record, AutoCAD

sends the count bytes of rasterized data.  Consequently, following the receipt

of a PPPLOTDATA record, the driver should read count bytes from its read pipe.

PPENDPLOT is output at the end of a plot.  It has no arguments.  When this code

is received, the driver should perform its clean up and exit.

PPABORTPLOT is sent to indicate that the plot has been terminated by the user

by pressing the interrupt key (normally control C).  An installed driver will

receive the code to indicate that abnormal termination action should be taken

(flushing a buffer, for example).  However, the driver should not exit at this

point.  The PPABORTPLOT code will be followed by an PPENDPLOT code, which

should perform the usual end of plot functions (such as closing the output file

and exiting).

133

134

Appendix A

Standard Color Assignments

Whenever a color value is passed from an Autodesk software product to an ADI

graphics display, rendering, or printer/plotter driver, the following color

mapping should be used:

0     Background (erase)

1     red

2     yellow

3     green

4     cyan

5     blue

6     magenta

7     white

8-15  usually intensified versions of above (for RGBI type devices)

For devices that support 256 colors, the hue resulting from a color value in

the range 10-249 will be determined by its first two digits, and the saturation

and value from the last digit, as follows:

10    Red

50    Yellow

90    Green

130   Cyan

170   Blue

210   Magenta

Between each of these are three intermediate hues, e.g., between red and

yellow, we have:

20    reddish orange

30    orange

40    yellowish orange

To each hue number, 0, 2, 4, 6, or 8 can be added to give a different value, or

brightness, with 0 the brightest and 8 the weakest.

Finally, 1 can be added to produce a half-saturated, or pastel, color.  For

example, color 18 is the dimmest red and 10 the brightest red, 19 is the dim-

mest pink and 11 the brightest pink.  Colors 8 and 9 are up for grabs;

typically various grays or sometimes black.

If a display driver is requested to draw in a color that matches the background

color, then white should be used (unless the background color is white, in

which case black should be used).

Included in the ADI Driver Development Kit is the file CHROMA.DWG, an AutoCAD

drawing that shows all colors (1-255). This file can be found in the DSTEST

135

Autodesk Device Interface/ADI

directory on the ADI distribution disk. You should use this sample drawing for

debugging your device driver.

If a display driver supports fewer than 256 colors, it should honor all 256

color numbers and display the closest color to that requested.

136

Appendix B

Logical Display Color Assignments

The new Autodesk user interface uses pull-down menus and dialogue boxes as the

medium of communication between the software and the user.  The colors used for

the various elements of the new user interface are necessarily distinct from

those used for drawing and screen displays.  The concept of logical colors was

implemented to maintain this distinction.

A logical color is known to an Autodesk software product only as a number.

There is no direct connection between a physical color and a logical color

number.  It is the responsibility of the display driver to convert logical

color numbers to physical colors, and ensure that all colored elements are vis-

ible.

For example, border colors for user interface elements (pull-down menus, dia-

logue boxes, etc.) must be different from both the graphics area background

color and the background color of the element itself.

For monochrome display devices, all dialogue, menu, and pull-down background

colors are the same as the graphics area background color.

In addition, the foreground color of an element must differ from the background

color of the element.

The following table summarizes logical color assignments as currently defined

in the COLOURS.H file included in the ADI Driver Development Kit.

Mnemonic   Number               Usage

BGLCOLOR      -2    Graphics area background color

TBGLCOLOR     -3    Text area background color

TXTLCOLOR     -4    Color of text

BRDLCOLOR     -5    Color for border lines

ABGLCOLOR     -6    Alert box background color

AFGLCOLOR     -7    Alert box foreground color

ABRDLCOLOR    -8    Alert box border color

MBGLCOLOR     -9    Menu bar background color

MFGLCOLOR    -10    Menu bar foreground color

MBRDLCOLOR   -11    Menu bar border color

PBGLCOLOR    -12    Pull-down menu background color

PFGLCOLOR    -13    Pull-down menu foreground color

PBRDLCOLOR   -14    Pull-down menu border color

DBGLCOLOR    -15    Dialogue box background color

DFGLCOLOR    -16    Dialogue box foreground color

DBRDLCOLOR   -17    Dialogue box border color

LINELCOLOR   -18    Dialogue box line-drawing color

MINLCOLOR    -18    Most negative color number

To cause an alert box to appear (for testing) in AutoCAD, use the DDLMODES dia-

logue box and attempt to create a new layer named "0" (zero).

137

Autodesk Device Interface/ADI

When assigning actual colors to logical color values, you should keep the fol-

lowing in mind:

1. BGLCOLOR, TBGLCOLOR, TXTLCOLOR, and BRDLCOLOR describe colors used for

the corresponding areas of the display screen, and should be distinct

from user interface elements.

2. Alert boxes, dialogue boxes, the menu bar, and pull-down menus all

require three color elements: foreground, background, and border.  The

border color must be different from the graphics area background color

(BGLCOLOR).

3. LINELCOLOR must be different from DBGLCOLOR and ABGLCOLOR.  If conve-

nient, it should also be different from DFGLCOLOR and AFGLCOLOR.

4. Using too many colors may result in a distracting and unattractive

display.  Don't hesitate to use the same color or color combination

for multiple purposes where no color conflict is likely to arise.

5. If the display supports only two colors (e.g., black and white), then

the entire color scheme is determined by the color assigned to

BGLCOLOR.  In this case, all background colors are the same as

BGLCOLOR, and all other colors are different from BGLCOLOR.  Two

#defines in COLOURS.H, DECLBGARRAY and the ISBGLCOLOR macro, are pro-

vided to assist the driver in testing for background color.

138

(B) Logical Display Color Assignments

Sample color assignments for a 16-color display such as the IBM VGA are shown

below. This table shows the colors assigned to each physical color number

(using light-grey for the graphics background).

Physical      Color

Color #      Name

0      light grey

1          red

2        yellow

3         green

4         cyan

5         blue

6        magenta

7         black

8       dark grey

9       dark red

10      dark brown

11      dark green

12       dark cyan

13       dark blue

14     dark magenta

15         white

The table below shows how these colors might be used for the various logical

portions of the screen.

Logical           Used for            Sample Settings

Color #                             (Physical Color #)

-2       graphics background       0 (light grey)

-3         Text background         13 (dark blue)

-4               Text                15 (white)

-5           Border lines           8 (dark grey)

-6       Alert box background        15 (white)

-7       Alert box foreground          1 (red)

-8         Alert box border         8 (dark grey)

-9       Menu bar background         15 (white)

-10       Menu bar foreground       13 (dark blue)

-11         Menu bar border          8 (dark grey)

-12    Pull-down menu background      15 (white)

-13    Pull-down menu foreground    13 (dark blue)

-14      Pull-down menu border       8 (dark grey)

-15     Dialogue box background       2 (yellow)

-16     Dialogue box foreground        7 (black)

-17       Dialogue box border        8 (dark grey)

-18     Lines in dialogue boxes     13 (dark blue)

139

140

Appendix C

Revision History

ADI Display Version 4.0

(Interface level 6) This version of the ADI display interface adds packet mode

support for the new, multiple viewport capabilities of AutoCAD Release 10.

In addition, ADI 4.0 provides more direct support for vendors who choose to

write display list drivers.  In the past, it was necessary for such drivers to

"fool" AutoCAD with several tricks in order to manage a display list; now, a

driver can tell AutoCAD its actual physical screen size and its display list

logical space size.  A display list (or "big screen") driver can tell AutoCAD

when it wants to perform a pan or zoom and AutoCAD will ask the driver to do

it.  At this point, the driver can either perform the operation or tell AutoCAD

that it is unable to do so.  This solves several problems experienced by older

display list drivers (such as losing grid and other non-drawing entities).

Baroque digitizer data scaling is also no longer necessary.

Support has also been added for optional "smart" dynamic dragging, optional

transparent command parsing, and optional "slave" viewports.

The standard for packet exchange has been modified so that drivers will trun-

cate packets that are longer than expected and ignore unrecognized packets,

thus making compatability with future changes easier.  (See "Development Hints

for Display Drivers" on page 80 for a summary list of changes/additions to the

packet mode function calls.)

ADI Display Version 3.1 (November, 1987)

(Interface level 5).  This version of the ADI display driver resolved some

deficiencies reported in version 3.0, and a new DSGEN.DRV embodying it was sup-

plied as a driver update for AutoCAD Release 9.  The problems addressed in this

version are noted below.

o  Several AutoCAD display drivers have been developed for graphics

boards that have hardware pan/zoom capabilities.  The driver develop-

ers found that digitizer input was a problem, however, since AutoCAD's

coordinates didn't necessarily match the portion of the drawing cur-

rently on the screen.  Some of the manufacturers wrote their own

digitizer drivers to resolve this, but they'd much rather take advan-

tage of the wide range of digitizer drivers supplied with AutoCAD.

Therefore, a new ability to capture raw input from any configured dig-

itizer was added.  The display driver can now process the digitizer

input before sending it on to AutoCAD.

141

Autodesk Device Interface/ADI

o  Also for the benefit of displays possessing hardware pan/zoom capabil-

ities, a means was added to allow the driver to adjust the size of

AutoCAD's "pick" box and the object snap "aperture" as needed.

o  Version 3.0 of the ADI spec required a driver to cope with four dif-

ferent screen coordinate systems.  Version 3.1 makes this a bit sim-

pler by eliminating one of these systems.

ADI Display Version 3.0 (September, 1987)

(Interface level 4).  This version of the ADI display driver was included in

the initial shipments of AutoCAD Release 9 and in the initial release of

AutoShade.  The specifications were distributed to a limited number of develop-

ers.  Version 3.0 added the following new features:

o  extensions to the INT interface to support the advanced user interface

used by AutoCAD Release 9 ADE-3

o  a faster, more efficient Packet interface for AutoCAD graphics display

drivers (used also for AutoShade and AutoSketch)

o  an even speedier "fast vector" drawing facility

o  optional SHELL command notification

o  support for AutoShade's rendering device requirements

Extensions to the INT Interface

A number of extensions were made to the INT interface to aid developers who

wish to upgrade their existing drivers to implement the advanced user interface

requests.

In most cases, these extensions to the INT interface do not alter the display

driver requests specified in version 2.1 of the ADI Driver Development Kit.

One notable exception concerns the INIT4 request.  The advanced user interface

is enabled by setting the code bit EF_NEWI in the response returned by the

INIT4 request.  None of the advanced user interface requests (extended

requests) will work unless this bit is set.

The extended requests use a screen model defined as a bitmap subdivided into

fixed-width character cells.  The screen size is usually an even multiple of

the character cell size, but it need not be.  For this reason, your display

driver must independently specify pixel size, character size, and cell size.

Extra pixels outside the character frame are wasted.  The advanced interface

requests always refer to the physical screen, even for those drivers supporting

a larger logical screen.

142

(C) Revision History

Pixels and character cells are addressed in an unbiased coordinate space with

(0,0) at the upper left corner, X coordinates increasing to the right and Y

coordinates increasing toward the bottom of the screen.

The design of the advanced user interface dialogue boxes assumes that the dis-

play screen is at least 80 characters wide and 24 characters high.  Systems

with smaller displays should not enable the advanced user interface.

Systems that allow resizing of the screen may continue to use the advanced user

interface even if the screen is made smaller than the 80 x 24 character mini-

mum, as long as the display driver clips correctly and there is a way to escape

when a dialogue box is displayed whose oCANCELp and oOKp buttons are off-

screen.

The status line is assumed to occupy exactly one character row of vertical

space at the top of the screen.

A driver that supports the advanced user interface is presumed to be capable of

dealing with a DRAW request with both highlighting and XOR mode requested.

This is used by AutoCAD and AutoSketch to distinguish "crossing" selection

boxes from "window" selection boxes.

A new EF_SHELL bit was added to INIT4, and new BSHELL and ASHELL requests were

introduced to allow the driver to request notification of SHELL commands from

AutoCAD.

ADI Display Version 2.1 (December, 1986)

(Interface level 3).  This version was first supplied as a driver update to

AutoCAD Version 2.52, and remained in use through AutoCAD Version 2.62.

o  MOVE fixed to recognize vectors that are not part of a drawing entity

(e.g. blips) and should not be redrawn on REDRAW calls.

o  MODELINE fixed to pass correct current entity drawing color.

o  Fixed machine hang when ADI driver was not found at specified inter-

rupt, or when ADI driver returned 0 (failure) on INIT1 call.

ADI Display Version 2.0 (October, 1986)

(Interface level 2).  This version was shipped with AutoCAD Version 2.52.

o  Added INIT4 Extended Functions ED_SYNC and EF_REDRAW.

o  FLOOD function fixed to include color specification.

o  MOVE calls added prior to FLOOD calls to pass highlight flag.

143

Autodesk Device Interface/ADI

o  DSCOMAND and DSXPCOMD added to enable passing of transparent commands

to the display driver.

o  AutoCAD ADI display driver corrected to handle selection of interrupt

vectors other than the default, 7Ah.

144

Appendix D

ADI Graphics Display (INT Mode)

This appendix describes the INT interface for ADI

graphics display drivers.  This information is basi-

cally for maintenance of existing INT-mode AutoCAD

display drivers.  While AutoCAD supports both the INT

interface and the Packet interface, new Autodesk

products such as AutoSketch and AutoShade work only

with the Packet interface (described in Chapter 3).

The Packet interface is also faster and more

straightforward.  Therefore, all new drivers should

be written using the Packet interface. Release 10 of

AutoCAD will be the last version of AutoCAD to sup-

port the INT interface.

General information on INT-mode calling sequences can be found in Chapter 2.

The INT-mode requests, their purposes, function codes, parameters, and result

codes, are described in detail below.

Display list drivers before ADI 4.0

(The following section has been moved from its original location in Chapter 3.

The techniques it describes are no longer necessary, but are included here for

historical interest.)

If you are writing an ADI display driver and plan to "fool" AutoCAD by telling

it you have a much larger screen than you actually have (to allow for local

pans, zooms, etc.), carefully read the following notes:

1. Set ixdots and iydots to values relating to your large logical screen

size.

2. Set nuxdots, nuydots, nuxchars, nuychars, nuxbpc, and nuybpc to values

relating to your smaller physical screen size.  These will be used by

the advanced user interface.

3. Note that there's no nuxdotsm to tell AutoCAD about the actual graph-

ics area width (less any screen menu).  AutoCAD calculates a value for

this internally, using the following formula:

graphics width = (((ixdots + 1) * nuxdots) / (ixdotsm + 1)) - 1

You should send AutoCAD a value for ixdotsm that will force this cal-

culation to come out properly.  Otherwise, the menu bar at the top of

145

Autodesk Device Interface/ADI

the screen (part of the advanced user interface) might overlap the

menu area at the right of the screen.

INT-mode Display Driver Requests

This section describes the requests used by AutoCAD to communicate with an

installed ADI graphics display driver employing the INT interface.

In the following descriptions of the arguments passed to and from the display

driver, two items enclosed in parentheses represent a single 16 bit value, with

the first item contained in the most-significant 8 bits and the second item

contained in the least-significant 8 bits.

146

(D) ADI Display / INT Mode

--------------------  INIT1 (1)  --------------------

The INIT1 request is the initial request issued to the driver.

Parameters:  Code    (AX):   1

Arg1    (BX):   CONFIG

Arg2    (CX):   (reserved,INTLEVEL)

Return values:  OK   (AX):   0  =  unsuccessful initialization

1  =  successful initialization

Res1    (BX):   IXDOTS

Res2    (CX):   IYDOTS

Res3    (DX):   IXDOTSM

INTLEVEL (CL) identifies the version of the driver interface for upward compat-

ibility.  This version of the driver interface assigns the value 6 to INTLEVEL.

CONFIG (BL) is a 16-bit, bit-coded value that describes the the screen configu-

ration defined by the user during configuration.  Currently defined bits for

CONFIG are as follows:

Mnemonic   Bit  Value                 Meaning

CF_TEXT      0     1    set if text scrolling area defined

CF_MENU      1     2    set if screen menu area defined

CF_STATUS    2     4    set if status line area defined

CF_CURS      3     8    set if raster cursor required

CF_DIGTIZE   4     16   set if digitizer filtering supported

CF_ZOUT      5     32   set if initial display is zoomed out

CF_SYNC      6     64   set if SYNC (50) request supported

CF_SHADE     13   8192  set if product is AutoShade

CF_SKETCH    14  16384  set if product is AutoSketch

CF_ACAD      15  32768  set if product is AutoCAD

The CF_CURS bit, if set, indicates a requirement for raster cursors, which

cannot be supported by INT-mode drivers.  AutoCAD does not require raster cur-

sors, and does not set this bit.  If this bit is set, an INT-mode driver should

return with OK set to zero to indicate initizliation failure.

CF_ZOUT, CF_SYNC, and the product identification bits (CF_SHADE, CF_SKETCH, and

CF_ACAD) are only meaningful for INTLEVEL values 5 and greater.

OK (AX) is returned equal to 1 if the device was initialized correctly.  Other-

wise, it is returned equal to 0.

IXDOTS (BX) should be returned equal to one less than the horizontal size of

the graphics area, less the screen menu area.  (A special value of zero

returned in BX indicates that packet mode is to be used for all subsequent

requests.  See "Packet Interface for ADI Graphics Display Drivers" on page 31.)

IYDOTS (CX) is returned equal to one less than the vertical size of the graph-

ics area (less text scrolling area and status line).

147

Autodesk Device Interface/ADI

IXDOTSM (DX) is the maximum horizontal pixel number (including screen menu

area) beginning at zero.  The total number of pixels in the X direction is

IXDOTSM+1.

148

(D) ADI Display / INT Mode

--------------------  INIT2 (2)  --------------------

INIT2 instructs the driver to provide additional information.

Parameters:  Code    (AX):   2

Return values:  Res1 (BX):   (MODELINL,MNUCHARS)

Res2    (CX):   (MAXBOXES,MODES)

Res3    (DX):   (SPLIT,HWFILL)

MODELINL (BH) is the number of characters available for the status line.

MODELINL must be at least 37 to avoid an abbreviated status line.  The count

does not include characters available for the coordinate display.

MNUCHARS (BL) is the number of characters available for screen menu items.

MAXBOXES (CH) is the number of screen menu boxes on the screen.  It must be

zero if no screen menu is configured.  (See the CF_MENU bit of the INIT1

request.)

MODES (CL) is an 8-bit, bit-coded field that specifies modes pertaining to the

operation of this display device.  Currently defined bits for MODES are as fol-

lows:

Mnemonic  Bit  Value                 Meaning

DM_CCECH   0     1    0 = echo CTRL-C

1 = suppress echo of CTRL-C

DM_HLEN    1     2    set if display supports highlighting

DM_GROK    2     4    0 = dual-screen operation

1 = single-screen operation

DM_CCECH (MODES bit 0) indicates that echoing a CTRL-C to the screen while in

graphics mode will destroy the display, and that CTRL-C should therefore be

suppressed.  This bit should be set only for single-screen displays that lack

separate image planes for text and graphics.  Setting this bit will inhibit a

CTRL-C that is typed ahead (follows other as yet unprocessed keyboard input)

from taking effect immediately.

DM_HLEN (MODES bit 1) should be set if the display supports highlighting of

selected objects (preferably by drawing them with dashed lines).

DM_GROK (MODES bit 2) should be set for single-screen operation that requires

the driver to switch between a text screen and a graphics screen.  DM_GROK

should be cleared if a separate graphics screen will be used.  A driver that

supports both single- and dual-screen modes must find out for itself whether a

second screen is present (i.e., check equipment byte, etc.), and then set

DM_GROK accordingly.

If DM_GROK is set (specifying single-screen operation), SPLIT (DH) should

return the number of lines of text that appear in the text scrolling area (nor-

mally three).

149

Autodesk Device Interface/ADI

If DM_GROK is cleared (specifying dual-screen operation), SPLIT should return

the value 0.

HWFILL should return the value 0 for devices without hardware fill, or 2 for

devices with polygon fill (1 is not a valid return value).

150

(D) ADI Display / INT Mode

--------------------  INIT3 (3)  --------------------

INIT3 instructs the driver to provide additional information.

Parameters:  Code    (AX):   3

Return values:  Res1 (BX):   PIXWID

Res2    (CX):   PIXHGT

Res3    (DX):   (HIDLINES,HIDCOLS)

PIXWID is the horizontal physical spacing between pixels, and PIXHGT is the

vertical physical spacing between pixels.  The ratio of PIXWID to PIXHGT is

used to correct the aspect ratio so that the horizontal and vertical measure-

ments of objects drawn on the screen are equal.  On a device with square

pixels, PIXWID and PIXHGT should both return the value 1.  On devices with dif-

ferent X and Y scales, PIXWID and PIXHGT should return a value based on the

physical screen measurement in each direction divided by the number of pixels

on the appropriate axis.

Only the ratio of PIXWID to PIXHGT is significant, so any desired units may be

used.  However, the values must be greater than 0 and less than 32767, and

should be as accurate as possible, since changes in the second decimal place of

the ratio can result in visible changes on the screen.  See "PIXWID and PIXHGT"

on page 25 for more information and example calculations.

The values returned in HIDLINES and HIDCOLS (normally 25 and 80, respectively)

specify the number of lines and characters to be used for the hidden screen.

If the driver does not require AutoCAD to maintain a hidden screen, it should

return HIDLINES and HIDCOLS equal to 0.

See GOTEXTU (34) request for details.

151

Autodesk Device Interface/ADI

--------------------  INIT4 (4)  --------------------

INIT4 instructs the driver to provide additional information.

Parameters:  Code    (AX):   4

Return values:  Res1 (BX):   upper half of 32-bit field

Res2    (CX):   lower half of 32-bit field

Res1 and Res2 are the upper and lower halves, respectively, of a 32-bit options

field.  The status of certain bits in this field specify the presence (or

absence) in the driver of a handler for various extended requests.  The cur-

rently defined bits and the requests they represent are as follows:

Mnemonic   Bit  Value                      Meaning

EF_SYNC   0     1    Use SYNC (50) to synchronize the display with

the local display list, or to request special

functions from the Autodesk product.

EF_REDRAW 1     2    Use REDRAW (51) to redraw from local display

list

EF_CMD    2     4    Use DSCOMAND (52) to process unrecognized com-

mands

EF_XPCMD  3     8    Use DSXPCOMD (53) to process transparent com-

mands

EF_NEWI   4     16   Support advanced user interface requests

EF_SAVEB  5     32   AutoCAD provides screen-save buffers

(BOXSIZE - 62)

EF_FDRAW  6     64   Ignored in INT mode

EF_CURS   7    128   Ignored in INT mode

EF_SHELL  8    256   SHELL command notification desired

EF_DIGTIZE9    512   Use DIGTIZE (56) to filter digitizer input

The remaining bits in Res1 and Res2 are currently unused, and should be explic-

itly cleared by the driver to avoid incompatibilities with future versions of

ADI.

152

(D) ADI Display / INT Mode

--------------------  INIT5 (5)  --------------------

INIT5 instructs the driver to provide additional information.

Parameters:  Code    (AX):   5

Return values:  Res1 (BX):   YMENUMAX

Res2    (CX):   IYDOTSM

YMENUMAX is the maximum vertical pixel assigned to the screen menu area, and is

normally equal to the value assigned to IYDOTS on return from the INIT1

request, but may be assigned another value if the screen menu area has a height

different from the graphics portion of the screen.

IYDOTSM is the maximum vertical dot number of the screen in pixels, including

the text scrolling area and the status line.  The total number of pixels in Y

direction is IYDOTSM+1.

After the INIT5 request is processed, the screen should be in graphics mode,

cleared, any borders between logical screen areas drawn, and ready to accept

graphics-mode commands.

--------------------  INIT6 (6)  --------------------

INIT6 instructs the driver to provide additional information required by the

advanced user interface.  This request is issued only if the driver set the

EF_NEWI bit when responding to the INIT4 request.

Parameters:  Code    (AX):   6

Return values:  Res1 (BX):   NUXDOTS

Res2    (CX):   NUYDOTS

Res3    (CX):   NUXCHARS

NUXDOTS is the width of the physical screen in pixels.  Note that this is the

total width of the screen, not the highest-numbered pixel.

NUYDOTS is the height of the physical screen in pixels.

The screen is presumed to be divided into a matrix of fixed-size character

cells.  NUXCHARS is the width of the physical screen in characters.

153

Autodesk Device Interface/ADI

--------------------  INIT7 (7)  --------------------

INIT7 instructs the driver to provide additional information required by the

advanced user interface.  This request is issued only if the driver set the

EF_NEWI bit when responding to the INIT4 request.

Parameters:  Code    (AX):   7

Return values:  Res1 (BX):   NUYCHARS

Res2    (CX):   NUXBPC

Res3    (CX):   NUYBPC

NUYCHARS is the height of the physical screen in characters.

NUXBPC is the width of a character cell in pixels.

NUYBPC is the height of a character cell in pixels.

--------------------  TERM (9)  --------------------

The TERM request indicates that AutoCAD is exiting graphics mode and that the

driver should perform any required cleanup.

Parameters:  Code    (AX):   9

Return values:  None

The driver should not clear the text screen, since this would erase any error

messages displayed by the Autodesk product should it exit abnormally.

A single-screen display will always be in text mode when this request is

issued.

154

(D) ADI Display / INT Mode

--------------------  CLEAR (10)  --------------------

CLEAR instructs the driver to clear the graphics portion of the screen.  If

possible, the screen menu area, text scrolling area, and status line should not

be disturbed.

Parameters:  Code    (AX):   10

Return values:  Res1 (BX):   repaint

repaint is a bit-coded field specifying which screen areas, if any, should be

repainted following the CLEAR request.

If the driver is capable of clearing only the graphics portion of the screen

without disturbing the screen menu area, scrolling area, and status line,

repaint should be returned equal to 0.  Otherwise, the following bits in

repaint should be set, depending on which areas of the screen must be redrawn:

Mnemonic   Bit  Value                    Meaning

RP_STAT     0     1    set if status line should be redrawn

RP_MENU     1     2    set if screen menu area should be redrawn

RP_SCROLL   2     4    set if text scrolling area should be redrawn

RP_GRAPH    3     8    set if graphics area should be redrawn

If the driver draws borders on the screen, and they are destroyed by clearing

the screen, CLEAR should replace them.

--------------------  DOT (11)  --------------------

DOT instructs the driver to XOR the dot at the location given by (X,Y).

Parameters:  Code    (AX):   11

Arg1    (BX):   X

Arg2    (CX):   Y

Return values:  None

If the display device is a multi-plane color display, the dot should be XORed

in all planes.

This request is currently used for the GRID command in AutoCAD, and may be

tested by using that command.

155

Autodesk Device Interface/ADI

--------------------  MOVE (12)  --------------------

MOVE passes information to the driver prior to issuing a drawing request.

Parameters:  Code    (AX):   12

Arg1    (BX):   FROM_X

Arg2    (CX):   FROM_Y

Arg3    (DX):   (0,drawmodes)

Return values:  None

The MOVE request passes information to the driver associated with a DRAW or

FLOOD request.  MOVE instructs the driver to save the values passed in FROM_X,

FROM_Y, and drawmodes for use in processing the associated request.  Each DRAW

request is immediately preceded by a MOVE request.  There is one MOVE request

before a sequence of FLOOD requests, and another MOVE request with identical

parameters immediately following the FLOOD sequence.

If the following request is a DRAW request, FROM_X,FROM_Y represents the start-

ing point of the vector to be drawn.

If the associated request is a FLOOD request, FROM_X represents the number of

vertices in the polygon, and FROM_Y contains the value -2.

For both DRAW or FLOOD requests, drawmodes is a bit-coded field that specifies

certain drawing modes.  Currently defined bits for drawmodes are as follows:

Mnemonic   Bit  Value                  Meaning

DR_HILITE   0     1    0 = do not use highlighting

1 = draw using highlighting

DR_NORDRW   1     2    0 = vector is part of drawing entity

1 = vector is not part of drawing entity

DR_HILITE (drawmodes bit 0) should be saved to provide the highlighting status

for the following DRAW or the associated FLOOD request.

DR_NORDRW marks vectors that are not part of any drawing entity, but are simply

screen artifacts (for example, the two lines forming a marker blip).  This is

provided for drivers that use the REDRAW request.  If DR_NORDRW is set, the

vector should not be redrawn by the next REDRAW request.  Drivers that do not

use the REDRAW request can ignore DR_NORDRW.

156

(D) ADI Display / INT Mode

--------------------  DRAW (13)  --------------------

DRAW instructs the driver to draw a vector from the point specified by

(FROM_X,FROM_Y) (passed to the driver by an immediately preceding MOVE

request), to the point specified by TO_X,TO_Y.

Parameters:  Code    (AX):   13

Arg1    (BX):   TO_X

Arg2    (CX):   TO_Y

Arg3    (DX):   COLOR

Return values:  None

If the driver supports the advanced user interface, COLOR may be less than -1,

indicating a logical color number.  In this case, the driver should substitute

the appropriate physical color corresponding to the requested logical color.

If the (possibly adjusted) COLOR is neither zero nor -1, that color should be

used to draw the vector.  Refer to Appendices A and B for information on color

numbers.

If COLOR is zero, the vector should be erased (preserving the background color

if it is something other than black).  If COLOR is -1, the vector should be

XORed with the background color in all image planes.

If the driver supports highlighting, COLOR is greater than zero, and the cur-

rent highlighting status is 1, the vector should be drawn highlighted, prefera-

bly in a dashed-line font, but the driver may use other forms of highlighting

if they are more effective.

If COLOR is nonzero, but the driver does not support highlighting or the cur-

rent highlighting status is 0, a solid vector should be drawn.

If the driver supports the advanced user interface

(EF_NEWI was set in the result from the INIT4

request), it must be capable of drawing a highlighted

line in either normal drawing (pcolour > 0) or XOR

mode (pcolour = -1).

A MOVE request is always issued immediately before a DRAW request.

157

Autodesk Device Interface/ADI

NOTES

Highlighting support is determined by the value of DM_HLEN

(bit 1 of the MODES field) returned in response to the

INIT2 request.

Current highlighting status is determined by the value of

DR_HILITE (bit 0 of the drawmodes field) passed to the

driver by the preceding MOVE request.

It is not unusual for some graphics controllers to draw a

different set of pixels depending on the direction the

vector is drawn.  If garbage is left on the screen after

erasing a polyline or solid, you might try sorting the

coordinates before drawing the vector.

158

(D) ADI Display / INT Mode

--------------------  FLOOD (14)  --------------------

The FLOOD request is issued only for devices that support hardware fill.

Parameters:  Code    (AX):   14

Arg1    (BX):   X

Arg2    (CX):   Y

Arg3    (DX):   COLOR

Return values:  None

If the device supports hardware fill, the driver should return the value 2 in

the HWFILL field in response to an INIT2 request.

For devices that support hardware fill, a FLOOD request is issued for each

vertex of the polygon to be filled.  The sequence of FLOOD requests is both

preceded and followed by a special MOVE request with Y equal to -2 and X equal

to the number of vertices in the polygon.

If the driver supports the advanced user interface, COLOR may be less than -1,

indicating a logical color number.  In this case, the driver should substitute

the appropriate physical color corresponding to the requested logical color.

If the (possibly adjusted) COLOR is neither zero nor -1, that color should be

used to draw the area.  Refer to Appendices A and B for information on color

numbers.

If COLOR is zero, the area should be erased (preserving the background color if

it is something other than black).

If the driver supports highlighting, COLOR is greater than zero, and the cur-

rent highlighting status is 1, the area should be drawn highlighted (usually by

highlighting every fifth line of the fill).

If COLOR is nonzero, but the driver does not support highlighting or the cur-

rent highlighting status is 0, a solid area should be drawn.

--------------------  MARK (15)  --------------------

MARK instructs the driver to draw a cursor of type CURSEL at screen coordinates

(CUR_X,CUR_Y).

Parameters:  Code    (AX):   15

Arg1    (BX):   CUR_X

Arg2    (CX):   CUR_Y

Arg3    (DX):   CURSEL

Return values:  CAN_DO  (AX):0  =  cursor not drawn

1  =  cursor drawn

159

Autodesk Device Interface/ADI

For all values of CURSEL other than 0 or 3, the driver should simply return

CAN_DO set to zero, and the Autodesk product will draw the cursor itself.

CURSEL values of 0 and 3 specify simple orthogonal crosshairs, and the display

driver may choose to draw these with optimized code.  Autodesk products

normally draw a full-screen crosshair, but the driver can draw it smaller (or

allow the size to be configured by the user) if desired.

If the driver draws the cursor, it should return a value of 1 in CAN_DO.  Oth-

erwise, it should return with CAN_DO set to zero, instructing the Autodesk

product to draw the cursor itself.

160

(D) ADI Display / INT Mode

--------------------  CMARK (16)  --------------------

CMARK instructs the driver to clear a cursor of type CURSEL at screen coordi-

nates (CUR_X,CUR_Y).

Parameters:  Code    (AX):   16

Arg1    (BX):   CUR_X

Arg2    (CX):   CUR_Y

Arg3    (DX):   CURSEL

Return values:  CAN_DO  (AX):0  =  cursor not cleared

1  =  cursor cleared

For all values of CURSEL other than 0 or 3, the driver should simply return

with CAN_DO set to zero.

If the driver clears the cursor, it should return a value of 1 for CAN_DO.

Otherwise, it should return with CAN_DO set to zero, instructing the Autodesk

product to clear the cursor itself.

--------------------  CHAR (20)  --------------------

CHAR instructs the driver to draw the character specified by CHAR at the cur-

rent text position.  The current text position should then be moved one charac-

ter to the right.

Parameters:  Code    (AX):   20

Arg1    (BX):   CHAR

Return values:  None

A sequence of CHAR requests is always terminated by an ECHAR request, so driv-

ers can buffer the characters and process them when the ECHAR request is

received if they so choose.

See following requests that set the current text position.

--------------------  ECHAR (21)  --------------------

The ECHAR request signals the end of a string of characters sent by a sequence

of CHAR requests.

Parameters:  Code    (AX):   21

Return values:  None

161

Autodesk Device Interface/ADI

--------------------  MHLITE (22)  --------------------

MHLITE instructs the driver to highlight the menu box specified by the current

text position.

Parameters:  Code    (AX):   22

Arg1    (BX):   NCHARS

Return values:  Res1 (BX):   SEND_CHARS

NCHARS contains the number of characters in the menu box.

If the driver can highlight the box without knowing the characters within it,

it should do so, and return SEND_CHARS equal to 0.  Otherwise, the driver

should return SEND_CHARS equal to 1 and AutoCAD will pass the contents of the

box to the driver with NCHARS CHAR requests, terminated by an ECHAR request.

--------------------  MDHLITE (23)  --------------------

MDHLITE instructs the driver to dehighlight the menu box specified by the cur-

rent text position.

Parameters:  Code    (AX):   23

Arg1    (BX):   NCHARS

Return values:  Res1 (BX):   SEND_CHARS

NCHARS contains the number of characters in the menu box.

If the driver can dehighlight the box without knowing the characters within it,

it should do so, and return SEND_CHARS equal to 0.  Otherwise, it should return

SEND_CHARS equal to 1 and AutoCAD will send the contents of the box to the

driver with NCHARS CHAR requests, terminated by an ECHAR request.

--------------------  MNUCUR (24)  --------------------

MNUCUR instructs the driver to set the current text cursor to menu box number

BOXNO (0 is the top box).

Parameters:  Code    (AX):   24

Arg1    (BX):   BOXNO

Arg2    (CX):   NBOXES

Return values:  None

162

(D) ADI Display / INT Mode

NBOXES contains the number of boxes on the screen.  This request may be fol-

lowed by a sequence of CHAR requests to write to the box, or by an MHLITE or

MDHLITE request to highlight or dehighlight the box.

163

Autodesk Device Interface/ADI

--------------------  TPROMPT (30)  --------------------

The TPROMPT request is issued only on dual-screen displays (indicated by

DM_GROK (bit 2) cleared in the MODES field returned in response to the INIT2

request).

Parameters:  Code    (AX):   30

Return values:  None

TPROMPT instructs the driver to set the current text cursor to the prompt line

on the screen.  This request ia typically followed by one or more CHAR requests

to write to the prompt line.

Text output to the prompt line is terminated by an ECHAR request.

--------------------  QPLOT (31)  --------------------

QPLOT instructs the driver to perform a screen dump to printer.

Parameters:  Code    (AX):   31

Return values:  CAN_DO  (AX):0  =  unable to dump to printer

1  =  screen dumped to printer

If the driver performs the screen dump, it should return CAN_DO equal to 1.

Otherwise, the driver should do nothing, and return CAN_DO equal to 0.

164

(D) ADI Display / INT Mode

--------------------  RPEN (32)  --------------------

RPEN instructs the driver to sense the position of the screen pointing device

(e.g., light pen) and return its current position (in terms of pixels on the

graphics screen) in PEN_X and PEN_Y.

Parameters:  Code    (AX):   32

Return values:  Res1 (BX):   STATUS

Res2    (CX):   PEN_X

Res3    (DX):   PEN_Y

The driver should return STATUS, PEN_X, and PEN_Y as indicated in the following

table.

Response     STATUS   PEN_X     PEN_Y

No response       0

Tracking point    2     X-coord   Y-coord

Picked point      3     X-coord   Y-coord

Button pick       4     Button #

In AutoCAD, button picks returned by RPEN (STATUS = 4) access the "AUX1" menu

and can be used to support a multi-button function box that is not necessarily

associated with the light pen.  Button code 0 references the first item in the

"AUX1" menu. If the "AUX1" portion of the menu file contains items that submit

individual characters (as in "A\"), the driver can use them in sequence to

submit any command sequence it needs.  For example, see the DSHERCP.C sample

driver and its DSHERCP.MNU menu file.

165

Autodesk Device Interface/ADI

--------------------  GOTEXT (33)  --------------------

The GOTEXT request is issued only on single-screen systems (indicated by

DM_GROK (bit 2) set in the MODES field returned by the INIT2 request).

Parameters:  Code    (AX):   33

Return values:  None

GOTEXT instructs the driver to switch to the text screen and clear it.  The

cursor on the text screen should be left at the top of the screen.

Subsequent to this request, AutoCAD will use regular DOS I/O to write to the

text screen.

--------------------  GOTEXTU (34)  --------------------

The GOTEXTU request is issued only on single-screen systems (indicated by

DM_GROK (bit 2) set in the MODES field returned by the INIT2 request).

Parameters:  Code    (AX):   34

Return values:  None

GOTEXTU instructs the driver to switch to the text screen, displaying hidden

screen information.

If the driver maintains its own hidden screen (indicated by returning both

HIDLINES and HIDCOLS equal to 0 in response to the INIT3 request) it should

switch to the text screen, display the hidden-screen information, and restore

the cursor to its previous position on the text screen.

If the hidden screen is maintained by AutoCAD (indicated by a nonzero value

returned in HIDLINES and HIDCOLS in response to the INIT3 request) the driver

should switch to the text screen, clear it, and leave the cursor at the top.

AutoCAD will automatically rewrite the hidden screen.

Subsequent to this request, AutoCAD will use regular DOS I/O to write to the

text screen.

166

(D) ADI Display / INT Mode

--------------------  GOGRAPH (35)  --------------------

The GOGRAPH request is issued only on single-screen systems (indicated by

DM_GROK (bit 2) set in the MODES field returned by the INIT2 request).

Parameters:  Code    (AX):   35

Return values:  Res1 (BX):   repaint

GOGRAPH instructs the driver to switch to the graphics screen.  Ideally, this

operation should be nondestructive, displaying the picture, status line, and

screen menu area exactly as they were when the display was switched to the text

screen.  The text scrolling area should reflect the last few lines of the text

screen if possible.

repaint is a bit-coded field identifying the screen areas, if any, that should

be repainted by requests from AutoCAD following the GOGRAPH request.

If the driver can switch to the graphics screen nondestructively, it should

return repaint equal to 0.  Otherwise, the following bits in repaint should be

set, depending on which areas of the screen must be redrawn:

Mnemonic   Bit  Value                    Meaning

RP_STAT     0     1    set if status line should be redrawn

RP_MENU     1     2    set if screen menu area should be redrawn

RP_SCROLL   2     4    set if text scrolling area should be redrawn

RP_GRAPH    3     8    set if graphics area should be redrawn

If RP_GRAPH (repaint bit 3) is set, requiring AutoCAD to redraw the picture,

the redraw process will issue the CLEAR request.  If the CLEAR request in turn

uses repaint to restore the screen menu area, text scrolling area, or status

line, there is no need to request restoration of these screen areas in the

repaint field returned by the GOGRAPH request.  Note that AutoCAD cannot redraw

the text scrolling area if the driver maintains its own hidden text screen (as

indicated by zero values for HIDLINES and HIDCOLS returned by the INIT3

request).

If the driver draws borders (or other information not known to AutoCAD) that

are destroyed by switching to the text screen and back to the graphics screen,

it is the responsibility of GOGRAPH to replace this information.

167

Autodesk Device Interface/ADI

--------------------  WRSPLIT (36)  --------------------

WRSPLIT instructs the driver to write the character specified by CHAR to the

text scrolling area of the graphics screen at the current text scrolling area

position.  Following the WRSPLIT request, the current text scrolling area posi-

tion should be moved one character to the right.  The driver should maintain a

current text scrolling area position distinct from the text position used by

other requests.

Parameters:  Code    (AX):   36

Arg1    (BX):   CHAR

Return values:  None

If the driver maintains its own hidden screen (indicated by returning both

HIDLINES and HIDCOLS equal to 0 in response to the INIT3 request),CHAR should

also be saved on the hidden screen.

If the hidden screen is maintained by AutoCAD (indicated by a nonzero value

returned in HIDLINES and HIDCOLS in response to the INIT3 request), the driver

need only draw the character on the graphics screen.  AutoCAD will take care of

saving it.

--------------------  MODELINE (37)  --------------------

MODELINE instructs the driver to set the current text position to the start of

CHAR the status line.

Parameters:  Code    (AX):   37

Arg1    (BX):   LENGTH

Arg2    (CX):   COLOR

Return values:  None

The string to be displayed is written to the status line by LENGTH CHAR

requests, terminated by an ECHAR request.

The current drawing color (the color in which new entities will appear, however

derived) is passed to the driver in COLOR so that the driver can, if it

chooses, represent this on the mode status line.

--------------------  COORDLINE (38)  --------------------

COORDLINE instructs the driver to set the current text position to the start of

the coordinate-status line (usually MODELINL+1).

168

(D) ADI Display / INT Mode

Parameters:  Code    (AX):   38

Arg1    (BX):   LENGTH

Return values:  None

The value of MODELINL is returned in response to the INIT2 request.

The string to be displayed is written to the coordinate status line by LENGTH

CHAR requests, terminated by an ECHAR request.

169

Autodesk Device Interface/ADI

--------------------  SYNC (50)  --------------------

The SYNC request is issued only if the driver set the EF_SYNC bit in the

options field returned in response to the INIT4 request.

Parameters:  Code    (AX):   50

Arg1    (BX):   0

Arg2    (CX):   APERTURE

Return values:  Res1 (BX):   REQUEST

Res2    (CX):   APERTURE

For drivers that maintain local display buffers (vector lists), the SYNC

request forces the local display buffers (queued vectors) to be drawn, bringing

the picture into synchronization with AutoCAD's internal state.  For example,

this request is issued before digitizer samples are taken, to ensure that the

entire picture is on the screen for the user to pick from.

REQUEST is a bit-coded field that lets the ADI driver make special requests of

the Autodesk product.  Currently defined bits are:

Mnemonic   Bit  Value                   Meaning

SY_APERTURE   0     1    set to change aperture (and pickbox) size

SY_HILITE     1     2    set to redraw nondrawing vectors

SY_PROMPT     2     4    set to reissue current prompt

AutoCAD supplies the current size (in pixels) of the object snap aperture in

APERTURE.  If the driver wishes to change the aperture size, it should store

the new size in APERTURE and set the SY_APERTURE bit in REQUEST field before

returning.  AutoCAD will then set the object snap aperture to the specified

value, and will also set the object selection pickbox to one third of the aper-

ture size. The aperture setting is stored in AutoCAD's configuration file, so

it is normal for an aperture change to cause a disk access. In the future, the

aperture size may be used to govern the size of marker blips and other drawing

features as well.

The SY_HILITE bit in REQUEST, if set by the driver, instructs AutoCAD to redraw

any vectors that were not part of the drawing proper, such as highlighting.

Marker blips are never redrawn, however.

The driver can set the SY_PROMPT bit in REQUEST to instruct AutoCAD to reissue

the current text prompt.

170

(D) ADI Display / INT Mode

--------------------  REDRAW (51)  --------------------

The REDRAW request is issued only if EF_REDRAW (bit 0) is set in the options

field returned in response to the INIT4 request.

Parameters:  Code    (AX):   51

Arg1    (BX):   0  =  redraw picture

1  =  clear display buffer

Return values:  CAN_DO  (AX):0  =  unable to redraw complete picture

1  =  complete picture redrawn

When a redraw of the picture is necessary, AutoCAD issues the REDRAW request

with an argument of 0, instructing the driver to redraw all vectors.  AutoCAD

will still issue a CLEAR request prior to the REDRAW request, and will draw

grid dots, etc. afterwards.

If the driver is able to redraw the complete picture, it should return with

CAN_DO set to 1.  Otherwise, it should return CAN_DO equal to 0, in which case

AutoCAD will then redraw the image vector-by-vector as usual.

When the current contents of any display buffer are no longer valid, AutoCAD

will issue the REDRAW request with an argument of 1 to indicate that the vector

space should be cleared.  In this case, no return status is necessary.

171

Autodesk Device Interface/ADI

--------------------  DSCOMAND (52)  --------------------

The DSCOMAND request is issued only if EF_CMD (bit 3) is set in the options

field returned in response to the INIT4 request.

Parameters:  Code    (AX):   52

Arg1    (BX):   string length

Arg2    (CX):   string OFFSET

Arg3    (DX):   string SEGMENT

Return values:  CAN_DO  (AX):0  =  command not recognized

1  =  command recognized and processed

DSCOMAND allows the driver to examine all input commands that are not recog-

nized by AutoCAD.

The command string location is passed as a SEGMENT:OFFSET pair in DX:CX, with

the string length in BX.

The command string is terminated by a null character in the usual C fashion.

If the driver returns CAN_DO equal to 0, AutoCAD proceeds with its unknown-

command handling.  Otherwise, AutoCAD assumes that the string has been recog-

nized and processed by the driver, and behaves as if it were any valid AutoCAD

command (that is, a null line repeats the command).

--------------------  DSXPCOMD (53)  --------------------

The DSXPCOMD request is issued only if EF_XPCMD (bit 4) is set in the options

field returned in response to the INIT4 request.

Parameters:  Code    (AX):   53

Arg1    (BX):   command character

Return values:  CAN_DO  (AX):0  =  command not recognized

1  =  command recognized and processed

DSXPCOMD notifies the driver when a special transparent display-driver command

character is entered.  Currently, there is only one such character: oCTRLpoLp

(0C hexadecimal).

The command character is passed to the driver in BX.  If the driver returns

CAN_DO equal to 0, AutoCAD proceeds with its unknown-character handling.  Oth-

erwise, AutoCAD assumes that the character has been recognized and processed by

the driver.

The transparent command character may be entered at any time by the user, and

will be immediately passed to the driver.  This is the mechanism normally used

to invoke such functions as hardware pan and zoom.

172

(D) ADI Display / INT Mode

--------------------  BSHELL (54)  --------------------

The BSHELL request is issued when a SHELL command is about to begin, if

EF_SHELL (bit 8 in the INIT4 result) was set to let AutoCAD know that the

driver wants to be informed of SHELL commands.

Parameters:  Code    (AX):   54

Return values:  None

This request will not be issued unless EF_SHELL (bit 8 in the INIT4 result) was

set to let AutoCAD know that the driver wants to be informed of SHELL commands.

If the driver is to use LIM EMS expanded memory, it should request it before

terminating and remaining resident (TSR), mapping 4 logical->physical pages.

AutoCAD will then take the rest of expanded memory when it starts.  AutoCAD

will save and restore the driver's page maps, but the SHELL command lets other

programs trash them, so the BSHELL notification is issued to let drivers pro-

tect their page maps.

After BSHELL, the driver should not use expanded memory until an ASHELL is

issued.

--------------------  ASHELL (55)  --------------------

The ASHELL request is issued when a SHELL command is complete, if EF_SHELL (bit

8 in the INIT4 result) was set to let AutoCAD know that the driver wants to be

informed of SHELL commands.

Parameters:  Code    (AX):   55

Return values:  None

If the driver is to use LIM EMS expanded memory, it should request it before

terminating and remaining resident (TSR), mapping 4 logical->physical pages.

AutoCAD will then take the rest of expanded memory when it starts.  AutoCAD

will save and restore drivers page maps, but the SHELL command lets other pro-

grams trash them, so BSHELL is issued to protect drivers page maps.

ASHELL is issued when the SHELL command is complete so that driver may again

use expanded memory.

173

Autodesk Device Interface/ADI

--------------------  DIGTIZE (56)  --------------------

Parameters:  Code    (AX):   56

Arg1    (BX):   STATUS

Arg2    (CX):   X

Arg3    (DX):   Y

Return values:  Arg1 (BX):   STATUS

Arg2    (CX):   X

Arg3    (DX):   Y

The DIGTIZE request is issued whenever a digitizer sample is obtained from the

configured digitizer, and permits the display driver to intercept and/or alter

digitizer samples.  This request is only issued if the CF_DIGTIZE bit was set

in the CONFIG field of the INIT1 request, and the driver sets the EF_DIGTIZE

options bit when returning from the INIT1 request.

When a digitizer sample has been passed with this request, the ADI driver can

take several actions.  It can do nothing, change the sample, request another

sample, or request that some digitizer coordinates be mapped to screen coordi-

nates.  Finally, the filtered coordinates are returned to AutoCAD.

The interface provided by this request is similar to an ADI digitizer driver

SENSE function (although some of the encodings vary slightly).  Each DIGTIZE

request passes in a status, an X and a Y.  The ADI driver returns the same

fields (possibly having altered the values).  Except for requests with a STATUS

value of 1, the X and Y values are in digitizer space with 0,0 being the bottom

left corner of the digitizer and 20480,20480 being the upper right corner.

Note that not all of the digitizer surface is necessarily part of the screen

pointing area and that some digitizers may (inappropriately) return coordinates

all the way up to 32767.

Possible STATUS values sent to the ADI driver are:

Value                         Meaning

0       No sample available (X and X are meaningless).

1       X and Y are the requested screen equivalents of

the X and Y values most recently passed back

from the ADI driver.  (See status 1 returned by

the ADI driver, below.) X will be between 0 and

IDXOTS.  Y will be between 0 and YMENUMAX or

IYDOTS (whichever is greater).  This status

value is only returned immediately following a

previous DIGTIZE request to which the driver

replied with a status of 1.

2       X and Y are a tracking coordinate.

3       X and Y are a pick coordinate.

-1, -2, ...  Button number 1, 2, ...  was pressed.  X and Y

are the coordinates at which this occurred, or

are both -1 if no coordinates were returned by

the digitizer.

174

(D) ADI Display / INT Mode

The ADI driver can return any of the preceeding status values (except 1) with

the corresponding X and Y coordinates, and AutoCAD will perform the appropriate

action.  The ADI driver can return a status of 1 or 4 to request special

action.  For these two returned status values, the immediately following

request will be another DIGTIZE.  No user keyboard input (not even CTRL-C) is

permitted before the next request.

Value                      Meaning

1    Convert the returned X and Y values to screen

coordinates and call the ADI driver immediately

with the converted values of X and Y.

4    Do not use the returned X and Y values.

Instead, immediately request another digitizer

sample and return it in the next DIGTIZE

request.

Note that AutoCAD's digitizer interface does not permit tracking coordinates to

be returned while a pick button is depressed, nor does it distinguish a short

button click from a long one.

--------------------  BOX1 (60)  --------------------

The BOX1 request specifies the upper left corner of a rectangle, expressed in

screen pixels, on which several of the following requests operate.

Parameters:  Code    (AX):   60

Arg1    (BX):   XL

Arg2    (CX):   YL

Return values:  None

The driver should save the coordinates passed by the BOX1 request for use by

the following request.

The coordinates passed by the BOX1 request are guaranteed to be less than or

equal to the corresponding coordinates passed by the following request.

--------------------  BOXCLR (61)  --------------------

BOXCLR instructs the driver to clear, to the color specified by COLOR, the

rectangle inclusively delimited by the points (XL,YL), passed by the preceding

BOX1 request, and (XH,YH).

175

Autodesk Device Interface/ADI

Parameters:  Code    (AX):   61

Arg1    (BX):   XH

Arg2    (CX):   YH

Arg3    (DX):   COLOR

Return values:  None

176

(D) ADI Display / INT Mode

--------------------  BOXSIZE (62)  --------------------

To support the advanced user interface, the driver must be capable of saving a

screen rectangle and subsequently restoring it (see the BOXPUSH and BOXPOP

requests below).  If necessitated by space requirements, the rectangle can be

broken up into horizontal strips.  Each such strip will be the full width of

the rectangle and one or more pixels high.  Screen data may be saved in one of

the following places:

1. in local memory provided by the display adapter;

2. in memory obtained from AutoCAD; or

3. in a combination of (1) and (2), depending on the amount of screen

data to be saved.

When the driver receives a BOXSIZE request, it should calculate the amount of

memory (in bytes) required to save the screen rectangle inclusively delimited

by the points (XL,YL), passed to the driver by a preceding BOX1 request, and

(XH,YH), plus any control storage the driver needs, and return the result in

SIZE.

Parameters:  Code    (AX):   62

Arg1    (BX):   XH

Arg2    (CX):   YH

Return values:  Res1 (BX):   SIZE

The BOXSIZE request is issued only if EF_SAVEB (bit 5) is set in the options

field returned in response to the INIT4 request (indicating that the driver

requires AutoCAD to allocate memory for saving screen data).  In this case,

each screen save request, BOXPUSH, is preceded by a BOXSIZE request.

If more than 64K of memory is required, the driver should return SIZE equal to

0xFFFF.

Many drivers have some space available in otherwise unused portions of the dis-

play adapter's memory.  This local memory should be exhausted before requesting

memory from AutoCAD.

If enough local memory is available to save a horizontal strip (at least one

pixel high) all the way across the screen rectangle, then the driver should

prepare to use this local memory, and return SIZE equal to the total amount of

memory required, less the amount of local memory available.

If SIZE is returned as a nonzero value, AutoCAD will attempt to allocate the

requested amount of memory.  Presently, the maximum amount of memory AutoCAD

will allocate in a single request is 65535 bytes.  Drivers should not count on

being able to obtain all of the requested memory in one block, since AutoCAD

may allocate a buffer smaller than the requested size.  In this case, the

driver will be expected to save the largest horizontal strip of the screen

rectangle that will fit in the buffer, and AutoCAD will continue to supply

buffers to hold the rest of the screen rectangle.

177

Autodesk Device Interface/ADI

The address of the allocated buffer is passed to the driver by a BOXBUF request

immediately prior to a BOXPUSH or BOXPOP request.

If AutoCAD is requested to provide a buffer for a subsequent BOXPUSH request,

and is unable to obtain even a minimum size buffer (2K), then the BOXPUSH

request to save the screen rectangle will not be issued, and the area of the

screen will be lost.  This, however, should happen only very rarely.

See also:  BOXPUSH (63), BOXPOP (64), BOXBUF (69)

178

(D) ADI Display / INT Mode

--------------------  BOXPUSH (63)  --------------------

BOXPUSH instructs the driver to save the contents of a screen rectangle.

Parameters:  Code    (AX):   63

Arg1    (BX):   XH

Arg2    (CX):   YH

Return values:  Res1 (BX):   HEIGHT

Res2    (CX):   LOCATE

The rectangle to be saved is inclusively delimited by the points (XL,YL),

passed to the driver by a preceding BOX1 request, and (XH,YH).

If AutoCAD provides memory for the saved image (indicated by EF_SAVEB (bit 5)

set in the options field returned by the INIT4 request and a nonzero SIZE

returned by the BOXSIZE request), the address of the buffer allocated by

AutoCAD is passed to the driver by a BOXBUF request issued immediately prior to

the BOXPUSH request.

If the driver provides its own buffer, no BOXBUF request is issued prior to

BOXPUSH.

Upon return, HEIGHT must be set to the number of pixel rows the driver was able

to save in the space available.  If the entire rectangle was saved, this would

be YH - YL + 1.

The value LOCATE is for the driver's convenience, and may be used to identify

where in local memory the screen rectangle has been saved.  The value assigned

to LOCATE will be passed as an argument to the matching BOXPOP request.

See also:  BOXSIZE (62), BOXPOP (64), BOXBUF (69)

--------------------  BOXPOP (64)  --------------------

BOXPOP instructs the driver to restore the screen saved by the most recent

BOXPUSH request.

Parameters:  Code    (AX):   64

Arg1    (BX):   XH

Arg2    (CX):   YH

Arg3    (DX):   LOCATE

Return values:  None

If AutoCAD provides memory for the saved image (indicated by EF_SAVEB (bit 5)

set in the options field returned by the INIT4 request and a nonzero SIZE

returned by the BOXSIZE request), the address of the buffer allocated by

AutoCAD is passed to the driver by a BOXBUF request issued immediately prior to

the BOXPOP request.

179

Autodesk Device Interface/ADI

If the driver provides its own buffer, no BOXBUF request is issued prior to

BOXPOP.

The LOCATE parameter to BOXPOP is equal to the value LOCATE returned by the

matching BOXPUSH request.  This parameter is for the driver's convenience and

may be used to identify where in local memory the screen rectangle has been

saved.

In single-screen systems, everything that has been pushed will be popped prior

to a request to flip to AutoCAD's text screen.

See also:  BOXSIZE (62), BOXPUSH (63), BOXBUF (69)

180

(D) ADI Display / INT Mode

--------------------  TATTR (65)  --------------------

The TATTR request precedes a series of TEXT requests used to draw raster text

on the screen.

Parameters:  Code    (AX):   65

Arg1    (BX):   ATTR

Arg2    (CX):   BG

Arg3    (DX):   FG

Return values:  None

ATTR is a bit-coded field in which the bits have the following meanings:

Mnemonic  Bit  Value             Meaning

TANORM    all    0    Normal text (all bits clear)

TAINV      0     1    Reverse video text

TAGREY     1     2    Greyed out text

TANORM specifies normal text, whose background is the same color as the back-

ground of the screen and whose intensity is normal.

TAINV causes text to be written in reverse video, with the background and fore-

ground reversed or otherwise highlighted at the driver's discretion.

TAGREY specifies that the text should be displayed as greyed out, usually by

forcing every other pixel in a checkerboard pattern to the background color.

(but a driver is free to use other visual cues if they work better). The driver

must be able to handle TAGREY in combination with TANORM either set or cleared.

See "ADI Graphics Display Test Procedures" on page 83 for tips on testing

greyed-out text.

BG and FG are the colors (usually logical colors) used to draw the text.

--------------------  TEXT (66)  --------------------

The TEXT request is used to write raster text on the screen, using the fore-

ground and background colors and text attributes set by the most recent TATTR

request.

Parameters:  Code    (AX):   66

Arg1    (BX):   X

Arg2    (CX):   Y

Arg3    (DX):   CHAR

Return values:  None

TEXT instructs the driver to draw the text character, CHAR, in the character

cell specified by (X,Y).  The entire character cell should be filled with

pixels of either the background or foreground color.

181

Autodesk Device Interface/ADI

Character codes passed in CHAR will, for the U.S. domestic version of AutoCAD,

always be in the inclusive range from 32 to 128, with 128 signifying a check

mark.  Editions of AutoCAD in languages other than English may use additional

character codes beyond 128, and may use a different code for the check mark

character.

If it is impossible for the driver to display a check mark, a capital X is an

acceptable substitute.

--------------------  VMOVE (67)  --------------------

VMOVE instructs the driver to save the endpoint specified by (FROM_X,FROM_Y)

for use by a following VDRAW request.

Parameters:  Code    (AX):   67

Arg1    (BX):   FROM_X

Arg2    (CX):   FROM_Y

Arg3    (DX):   dmodes

Return values:  None

The coordinates passed to the driver by this request are in the unbiased device

coordinate system with (0,0) representing the upper left corner of the display.

For compatibility with the MOVE request (request 12), the following bits are

set in the dmodes field passed in Arg3:

Mnemonic   Bit  Value     Required Setting for VDRAW Request

DR_HILITE   0     1    0 = do not use highlighting

DR_NORDRW   1     2    1 = vector is not part of drawing entity

--------------------  VDRAW (68)  --------------------

VDRAW instructs the driver to draw a vector from (FROM_X,FROM_Y), specified by

the last VMOVE request, to (TO_X,TO_Y).

Parameters:  Code    (AX):   68

Arg1    (BX):   TO_X

Arg2    (CX):   TO_Y

Arg3    (DX):   COLOR

Return values:  None

The vector should be drawn in the color specified by COLOR.

If COLOR is -1, the vector should be XORed with the background color in all

image planes.

182

(D) ADI Display / INT Mode

A vector drawn with VMOVE and VDRAW will never be highlighted and will never be

part of a drawing entity, and therefore should never be saved in a local dis-

play list.

183

Autodesk Device Interface/ADI

--------------------  BOXBUF (69)  --------------------

If AutoCAD provides memory for saving screen data (indicated by EF_SAVEB (bit

5) set in the options field returned by the INIT4 request and a nonzero SIZE

returned by the BOXSIZE request), the address of the buffer allocated by

AutoCAD is passed to the driver by a BOXBUF request issued immediately prior to

a BOXPUSH or BOXPOP request.

Parameters:  Code    (AX):   69

Arg1    (BX):   SIZE

Arg2    (CX):   OFFSET

Arg3    (DX):   SEGMENT

Return values:  None

If the driver provides its own buffer, no BOXBUF request is issued prior to

BOXPOP.

See also:  BOXSIZE (62), BOXPUSH (63), BOXPOP (64)

184

Index

A                                  CMARK (16)  161

Coding practices  5

ADI digitizers  105                  UNIX drivers  125

UNIX  126                          Colors

ADI driver                           logical  137

calling sequences  5                 standard  135

coding practices  5                Compatibility  7

communication with  9              Configuration  6, 17

compatibility  7                   Coordinates

configuration  6                     natural and biased  23

debugging  16                      COORDLINE (38)  168

development aids  5

initialization  7

Interrupt vectors  6               D

UNIX development aids  125

ADI graphics displays  17          debugging  16

ADI plotters  111                  Default interrupt vectors  6

UNIX  129                            plotter  111

ADI printer/plotters  119            printer/plotter  120

UNIX  131                          Developer's responsibilities  2

ADI rendering devices  89          Development aids  5

ADI.H  105                         DGADI.H  105

Advanced User Interface  20        DGDEMO.C  105

ASHELL (55)  173                   Dialogue boxes  59, 175

AutoCAD GRID command  42, 155      Digitizer driver

AUX1 menu  50, 165                   test procedures  108

UNIX  126

Digitizer driver requests

B                                    INIT (1)  105

initialization  105

Biased and natural coordinates  23   SENSE (3)  106

BIOS calls  15                       TERM (2)  106

BOX1 (60)  175                     DIGTIZE (56)  174

BOXBUF (69)  184                   direct hardware programming  15

BOXCLR (61)  175                   Direction flag (DF)  6

BOXPOP (64)  179                   Display driver

BOXPUSH (63)  179                    specification  17

BOXSIZE (62)  177                    test procedures  83

BSHELL (54)  173                   DOT (11)  155

Byte ordering  12                  DRAW (13)  157

Driver coding practices  5

UNIX  125

C                                  Driver development aids  5

Driver initialization  7

Calling sequences  5               DSCOMAND (52)  172

INT mode  8                        DSEGA.C  77

packet mode  9                     DSEGA4.ASM  77

UNIX drivers  125                  DSEGA4.C  77

CHAR (20)  161                     DSEGAA.ASM  77

CLEAR (10)  155                    DSGEN.DRV  17

185

Autodesk Device Interface/ADI

DSHERCP.C  11                      INIT6 (6)  153

DSXPCOMD (53)  172                 INIT7 (7)  154

Initialization  7

INT interface extensions  142

E                                  INT-mode calling sequence  8

INT-mode display requests

ECHAR (21)  161                      ASHELL (55)  173

Expanded memory  12, 26              BOX1 (60)  175

Extended memory  13                  BOXBUF (69)  184

Extensions to INT interface  142     BOXCLR (61)  175

BOXPOP (64)  179

BOXPUSH (63)  179

F                                    BOXSIZE (62)  177

BSHELL (54)  173

Fast vector mode  77                 CHAR (20)  161

File level  7                        CLEAR (10)  155

Floating point operations  15        CMARK (16)  161

FLOOD (14)  159                      COORDLINE (38)  168

Function box  50, 165                DIGTIZE (56)  174

DOT (11)  155

DRAW (13)  157

G                                    DSCOMAND (52)  172

DSXPCOMD (53)  172

GOGRAPH (35)  167                    ECHAR (21)  161

GOTEXT (33)  166                     FLOOD (14)  159

GOTEXTU (34)  166                    GOGRAPH (35)  167

Graphics display driver              GOTEXT (33)  166

configuration  17                    GOTEXTU (34)  166

Development Hints  80                INIT1 (1)  147

fast vector mode  77                 INIT2 (2)  149

INT Interface  22, 145               INIT3 (3)  151

interrupt vector  145                INIT4 (4)  152

Packet Interface  22, 31             INIT5 (5)  153

specification  17                    INIT6 (6)  153

test procedures  83                  INIT7 (7)  154

Graphics display environment  19     Initialization  147

GRID command  42, 155                MARK (15)  159

MDHLITE (23)  162

MHLITE (22)  162

H                                    MNUCUR (24)  162

MODELINE (37)  168

Highlighting                         MOVE (12)  156

status  158                          QPLOT (31)  164

support  158                         REDRAW (51)  171

RPEN (32)  165

SYNC (50)  170

I                                    TATTR (65)  181

TERM (9)  154

INIT (1)  105                        TEXT (66)  181

INIT1 (1)  31, 147                   TPROMPT (30)  164

INIT2 (2)  149                       VDRAW (68)  182

INIT3 (3)  151                       VMOVE (67)  182

INIT4 (4)  152                       WRSPLIT (36)  168

INIT5 (5)  153                     Interface level  7

186

Index

Interrupt vectors  6                 PDRAG (43)  73

IWAIT.ASM  8, 105                    PECHAR (10)  48

PFILL (6)  44

PGOGRAPH (19)  52

L                                    PGOTEXT (17)  51

PGOTEXTU (18)  51

Level indicator  7                   PHLITE (11)  48

LIM EMS expanded memory  12, 26      PINIT (1)  32

Logical display colors  137          PLINE (32)  64

PLOPEN (44)  74

PMARK (7)  46

M                                    PMNUCURS (13)  49

PMODELINE (21)  53

MARK (15)  159                       POPENVP (38)  69

MDHLITE (23)  162                    PQPLOT (15)  50

Memory mapping  12                   PREDRAW (24)  56

MHLITE (22)  162                     PREVEC (46)  76

MNUCUR (24)  162                     PRPEN (16)  50

MODELINE (37)  168                   PSYNC (23)  55

MOVE (12)  156                       PTABCFG (42)  72

PTERM (2)  41

PTEXT (31)  63

N                                    PTPROMPT (14)  49

PVEC (5)  42

Natural and biased coordinates  23   PVIEWPORT (40)  71

PWRSPLIT (20)  53

PXPCOMD (26)  57

P                                  PASHELL (36)  66

PBOXCLR (27)  59

Packet fields                      PBOXPOP (30)  62

byte ordering  12                  PBOXPUSH (29)  60

Packet-mode calling sequence  9    PBOXSIZE (28)  59

Packet-mode display requests       PBSHELL (35)  66

INIT1 (1)  31                      PCALL.ASM  11

initialization  32                 PCCURS (34)  66

PASHELL (36)  66                   PCHAR (9)  48

PBOXCLR (27)  59                   PCLEAR (3)  41

PBOXPOP (30)  62                   PCLOSEVP (39)  70

PBOXPUSH (29)  60                  PCLVP (45)  76

PBOXSIZE (28)  59                  PCMARK (8)  46

PBSHELL (35)  66                   PCOMMAND (25)  57

PCCURS (34)  66                    PCONIN (41)  72

PCHAR (9)  48                      PCOORDLINE (22)  53

PCLEAR (3)  41                     PDCURS (33)  65

PCLOSEVP (39)  70                  PDHLITE (12)  49

PCLVP (45)  76                     PDIGTIZE (37)  68

PCMARK (8)  46                     PDOT (4)  42

PCOMMAND (25)  57                  PDRAG (43)  73

PCONIN (41)  72                    PECHAR (10)  48

PCOORDLINE (22)  53                PFILL (6)  44

PDCURS (33)  65                    PGOGRAPH (19)  52

PDHLITE (12)  49                   PGOTEXT (17)  51

PDIGTIZE (37)  68                  PGOTEXTU (18)  51

PDOT (4)  42                       PHLITE (11)  48

187

Autodesk Device Interface/ADI

PINIT (1)  32                      R

PIXHGT  25

PIXWID  25                         RDCLEAR (2006)  96

PLGEN.DRV  115                     RDCMAP (2008)  97

PLINE (32)  64                     RDCMAPB (2007)  96

PLOPEN (44)  74                    RDCMAPE (2009)  97

Plotter driver                     RDCPOLY (2013)  102

communication with  115            RDCRANGE (2012)  100

configuration  111                 RDEND (2003)  94

default interrupt vector  111      RDFNAME (2011)  99

file format  115                   RDINIT (2001)  91

installation  115                  RDLINKUP (2000)  89

interface specification  111       RDPGCP.C  11, 89

operation  113, 115                RDPOLY (2010)  98

output format  111, 113            RDRESIZE (2005)  95

test procedures  116               RDSTART (2002)  94

UNIX  129                          RDTERM (2004)  95

PMARK (7)  46                      REDRAW (51)  171

PMNUCURS (13)  49                  Releasing your driver  3

PMODELINE (21)  53                 Rendering device driver

POPENVP (38)  69                     specification  89

PPGEN.DRV  122                     Rendering requests

PQPLOT (15)  50                      RDCLEAR (2006)  96

PREDRAW (24)  56                     RDCMAP (2008)  97

PREVEC (46)  76                      RDCMAPB (2007)  96

Printer/plotter driver               RDCMAPE (2009)  97

color selection  122                 RDCPOLY (2013)  102

communication with  122              RDCRANGE (2012)  100

configuration  119                   RDEND (2003)  94

default interrupt vector  120        RDFNAME (2011)  99

installation  122                    RDINIT (2001)  91

interface specification  119         RDLINKUP (2000)  89

operation  121, 122                  RDPOLY (2010)  98

output format  120, 121              RDRESIZE (2005)  95

test procedures  123                 RDSTART (2002)  94

UNIX  131                            RDTERM (2004)  95

PRPEN (16)  50                     Revision history  141

PSYNC (23)  55                     RPEN (32)  165

PTABCFG (42)  72

PTERM (2)  41

PTEXT (31)  63                     S

PTPROMPT (14)  49

PVEC (5)  42                       Sample drivers  5

PVIEWPORT (40)  71                   UNIX  125

PWRSPLIT (20)  53                  SENSE (3)  106

PXPCOMD (26)  57                   SHELL command notification  26, 66,

173

single-screen vs. dual-screen  22

Q                                  Standard color assignments  135

SYNC (50)  170

QPLOT (31)  164

188

Index

T                                    coding practices  125

UNIX development aids  125

TATTR (65)  181                    UNIX digitizer driver  126

TERM (2)  106                      UNIX driver development aids  125

TERM (9)  154                      UNIX plotter driver  129

Test procedures

digitizer  108

graphics display  83               V

plotter  116

printer/plotter  123               VDRAW (68)  182

Testing your driver  3             VMOVE (67)  182

TEXT (66)  181

TPROMPT (30)  164

tracing  16                        W

WRSPLIT (36)  168

U

UNIX ADI driver

calling sequences  125

189

190

Table of Contents

Chapter 1:   The Autodesk Device Interface/ADI   . . . . . . . . . . . . . .  1

Overview  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  1

Developer's Responsibilities  . . . . . . . . . . . . . . . . . . . . . .  2

What You Need. . . . . . . . . . . . . . . . . . . . . . . . . . . . .  2

What The ADI Kit Includes. . . . . . . . . . . . . . . . . . . . . . .  2

Working with ADI  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  3

Testing. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  3

Releasing Your Driver. . . . . . . . . . . . . . . . . . . . . . . . .  3

Chapter 2:   ADI Calling Sequences and Coding Practices  . . . . . . . . . .  5

Sample Drivers and Development Aids. . . . . . . . . . . . . . . . . .  5

The Direction Flag (DF). . . . . . . . . . . . . . . . . . . . . . . .  6

Interrupt Vectors. . . . . . . . . . . . . . . . . . . . . . . . . . .  6

ADI Driver Initialization. . . . . . . . . . . . . . . . . . . . . . .  7

Driver Compatibility  . . . . . . . . . . . . . . . . . . . . . . . . . .  7

INT-mode Calling Sequence. . . . . . . . . . . . . . . . . . . . . . .  8

Packet-mode Calling Sequence  . . . . . . . . . . . . . . . . . . . . . .  9

Determining Maximum Packet Size   . . . . . . . . . . . . . . . . . . . 11

Use of Expanded Memory  . . . . . . . . . . . . . . . . . . . . . . . . . 12

Use of Extended Memory  . . . . . . . . . . . . . . . . . . . . . . . . . 13

Suggested Standards for Floating Point Operations. . . . . . . . . . . 15

Direct Hardware Programming vs. BIOS Calls  . . . . . . . . . . . . . . . 15

Debugging Your Driver. . . . . . . . . . . . . . . . . . . . . . . . . 16

Chapter 3:   ADI Graphics Display Specification  . . . . . . . . . . . . . . 17

Configuration. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17

For AutoCAD Version 2.18  . . . . . . . . . . . . . . . . . . . . . . . 17

For AutoCAD Versions 2.5x and 2.6x  . . . . . . . . . . . . . . . . . . 17

For AutoCAD Release 9   . . . . . . . . . . . . . . . . . . . . . . . . 18

For AutoCAD Release 10  . . . . . . . . . . . . . . . . . . . . . . . . 18

For Other Autodesk Products   . . . . . . . . . . . . . . . . . . . . . 18

The Graphics Display Environment  . . . . . . . . . . . . . . . . . . . . 19

AutoCAD   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 19

Advanced User Interface  . . . . . . . . . . . . . . . . . . . . . . 20

AutoSketch  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

AutoShade   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

Writing Display Drivers. . . . . . . . . . . . . . . . . . . . . . . . 22

Single-Screen vs. Dual-Screen AutoCAD Drivers. . . . . . . . . . . . . 22

Natural and Biased Coordinates  . . . . . . . . . . . . . . . . . . . . . 23

i

Autodesk Device Interface/ADI

PIXWID and PIXHGT   . . . . . . . . . . . . . . . . . . . . . . . . . . 25

Notification of SHELL Commands  . . . . . . . . . . . . . . . . . . . . . 26

Function Usage Table  . . . . . . . . . . . . . . . . . . . . . . . . . . 26

Packet Interface for ADI Graphics Display Drivers. . . . . . . . . . . 31

Packet-mode Display Driver Requests   . . . . . . . . . . . . . . . . . 31

INIT1 (1)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 31

PINIT (1)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 32

PTERM (2)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 41

PCLEAR (3)   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 41

PDOT (4)   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 42

PVEC (5)   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 42

PFILL (6)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 44

PMARK (7)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 46

PCMARK (8)   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 46

PCHAR (9)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 48

PECHAR (10)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 48

PHLITE (11)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 48

PDHLITE (12)   . . . . . . . . . . . . . . . . . . . . . . . . . . . 49

PMNUCURS (13)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 49

PTPROMPT (14)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 49

PQPLOT (15)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 50

PRPEN (16)   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 50

PGOTEXT (17)   . . . . . . . . . . . . . . . . . . . . . . . . . . . 51

PGOTEXTU (18)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 51

PGOGRAPH (19)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 52

PWRSPLIT (20)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 53

PMODELINE (21)   . . . . . . . . . . . . . . . . . . . . . . . . . . 53

PCOORDLINE (22)  . . . . . . . . . . . . . . . . . . . . . . . . . . 53

PSYNC (23)   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 55

PREDRAW (24)   . . . . . . . . . . . . . . . . . . . . . . . . . . . 56

PCOMMAND (25)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 57

PXPCOMD (26)   . . . . . . . . . . . . . . . . . . . . . . . . . . . 57

PBOXCLR (27)   . . . . . . . . . . . . . . . . . . . . . . . . . . . 59

PBOXSIZE (28)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 59

PBOXPUSH (29)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 60

PBOXPOP (30)   . . . . . . . . . . . . . . . . . . . . . . . . . . . 62

PTEXT (31)   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 63

PLINE (32)   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 64

PDCURS (33)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 65

PCCURS (34)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 66

PBSHELL (35)   . . . . . . . . . . . . . . . . . . . . . . . . . . . 66

PASHELL (36)   . . . . . . . . . . . . . . . . . . . . . . . . . . . 66

PDIGTIZE (37)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 68

POPENVP (38)   . . . . . . . . . . . . . . . . . . . . . . . . . . . 69

PCLOSEVP (39)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 70

PVIEWPORT (40)   . . . . . . . . . . . . . . . . . . . . . . . . . . 71

PCONIN (41)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 72

PTABCFG (42)   . . . . . . . . . . . . . . . . . . . . . . . . . . . 72

PDRAG (43)   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 73

PLOPEN (44)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 74

PCLVP (45)   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 76

ii

Table of Contents

PREVEC (46)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 76

Fast Vector Mode  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 77

Development Hints for Display Drivers. . . . . . . . . . . . . . . . . 80

Existing ADI 3.1 drivers  . . . . . . . . . . . . . . . . . . . . . . . 80

New ADI 4.0 non-display list drivers  . . . . . . . . . . . . . . . . . 80

ADI 4.0 Display list drivers  . . . . . . . . . . . . . . . . . . . . . 80

Summary of changes in ADI 4.0   . . . . . . . . . . . . . . . . . . . . 82

ADI Graphics Display Test Procedures  . . . . . . . . . . . . . . . . . . 83

Chapter 4:   ADI Rendering Driver Specification  . . . . . . . . . . . . . . 89

Rendering Driver Requests. . . . . . . . . . . . . . . . . . . . . . . 89

RDLINKUP (2000)  . . . . . . . . . . . . . . . . . . . . . . . . . . 89

RDINIT (2001)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 91

RDSTART (2002)   . . . . . . . . . . . . . . . . . . . . . . . . . . 94

RDEND (2003)   . . . . . . . . . . . . . . . . . . . . . . . . . . . 94

RDTERM (2004)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 95

RDRESIZE (2005)  . . . . . . . . . . . . . . . . . . . . . . . . . . 95

RDCLEAR (2006)   . . . . . . . . . . . . . . . . . . . . . . . . . . 96

RDCMAPB (2007)   . . . . . . . . . . . . . . . . . . . . . . . . . . 96

RDCMAP (2008)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 97

RDCMAPE (2009)   . . . . . . . . . . . . . . . . . . . . . . . . . . 97

RDPOLY (2010)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 98

RDFNAME (2011)   . . . . . . . . . . . . . . . . . . . . . . . . . . 99

RDCRANGE (2012)  . . . . . . . . . . . . . . . . . . . . . . . . .  100

RDCPOLY (2013)   . . . . . . . . . . . . . . . . . . . . . . . . .  102

Chapter 5:   ADI Digitizer Specification   . . . . . . . . . . . . . . . .  105

Digitizer Driver Requests   . . . . . . . . . . . . . . . . . . . . .  105

INIT (1)   . . . . . . . . . . . . . . . . . . . . . . . . . . . .  105

TERM (2)   . . . . . . . . . . . . . . . . . . . . . . . . . . . .  106

SENSE (3)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  106

ADI Digitizer Test Procedures. . . . . . . . . . . . . . . . . . . .  108

Chapter 6:   ADI Plotter Specification   . . . . . . . . . . . . . . . . .  111

Configuration. . . . . . . . . . . . . . . . . . . . . . . . . . . .  111

Using the Driver  . . . . . . . . . . . . . . . . . . . . . . . . . . .  113

Output Format. . . . . . . . . . . . . . . . . . . . . . . . . . . .  113

File Format. . . . . . . . . . . . . . . . . . . . . . . . . . . . .  115

Installed Driver Operation  . . . . . . . . . . . . . . . . . . . . . .  115

ADI Plotter Test Procedures. . . . . . . . . . . . . . . . . . . . .  116

Chapter 7:   ADI Printer/Plotter Specification   . . . . . . . . . . . . .  119

Configuration. . . . . . . . . . . . . . . . . . . . . . . . . . . .  119

Using the Driver  . . . . . . . . . . . . . . . . . . . . . . . . . . .  121

iii

Autodesk Device Interface/ADI

Output Format. . . . . . . . . . . . . . . . . . . . . . . . . . . .  121

Installed Driver Operation  . . . . . . . . . . . . . . . . . . . . . .  122

ADI Printer/Plotter Test Procedures. . . . . . . . . . . . . . . . .  123

Chapter 8:   Developing UNIX ADI Drivers   . . . . . . . . . . . . . . . .  125

UNIX Sample Drivers and Development Aids  . . . . . . . . . . . . . . .  125

UNIX ADI Digitizer Driver. . . . . . . . . . . . . . . . . . . . . .  126

UNIX ADI Plotter Driver. . . . . . . . . . . . . . . . . . . . . . .  129

UNIX ADI Printer/Plotter Driver. . . . . . . . . . . . . . . . . . .  131

Appendix A:   Standard Color Assignments   . . . . . . . . . . . . . . . .  135

Appendix B:   Logical Display Color Assignments  . . . . . . . . . . . . .  137

Appendix C:   Revision History   . . . . . . . . . . . . . . . . . . . . .  141

ADI Display Version 4.0. . . . . . . . . . . . . . . . . . . . . . .  141

ADI Display Version 3.1 (November, 1987)  . . . . . . . . . . . . . . .  141

ADI Display Version 3.0 (September, 1987). . . . . . . . . . . . . .  142

Extensions to the INT Interface   . . . . . . . . . . . . . . . . . .  142

ADI Display Version 2.1 (December, 1986)  . . . . . . . . . . . . . . .  143

ADI Display Version 2.0 (October, 1986). . . . . . . . . . . . . . .  143

Appendix D:   ADI Graphics Display (INT Mode)  . . . . . . . . . . . . . .  145

Display list drivers before ADI 4.0. . . . . . . . . . . . . . . . .  145

INT-mode Display Driver Requests  . . . . . . . . . . . . . . . . . . .  146

INIT1 (1)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  147

INIT2 (2)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  149

INIT3 (3)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  151

INIT4 (4)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  152

INIT5 (5)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  153

INIT6 (6)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  153

INIT7 (7)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  154

TERM (9)   . . . . . . . . . . . . . . . . . . . . . . . . . . . .  154

CLEAR (10)   . . . . . . . . . . . . . . . . . . . . . . . . . . .  155

DOT (11)   . . . . . . . . . . . . . . . . . . . . . . . . . . . .  155

MOVE (12)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  156

DRAW (13)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  157

FLOOD (14)   . . . . . . . . . . . . . . . . . . . . . . . . . . .  159

MARK (15)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  159

CMARK (16)   . . . . . . . . . . . . . . . . . . . . . . . . . . .  161

CHAR (20)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  161

ECHAR (21)   . . . . . . . . . . . . . . . . . . . . . . . . . . .  161

MHLITE (22)  . . . . . . . . . . . . . . . . . . . . . . . . . . .  162

MDHLITE (23)   . . . . . . . . . . . . . . . . . . . . . . . . . .  162

MNUCUR (24)  . . . . . . . . . . . . . . . . . . . . . . . . . . .  162

TPROMPT (30)   . . . . . . . . . . . . . . . . . . . . . . . . . .  164

QPLOT (31)   . . . . . . . . . . . . . . . . . . . . . . . . . . .  164

RPEN (32)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  165

iv

Table of Contents

GOTEXT (33)  . . . . . . . . . . . . . . . . . . . . . . . . . . .  166

GOTEXTU (34)   . . . . . . . . . . . . . . . . . . . . . . . . . .  166

GOGRAPH (35)   . . . . . . . . . . . . . . . . . . . . . . . . . .  167

WRSPLIT (36)   . . . . . . . . . . . . . . . . . . . . . . . . . .  168

MODELINE (37)  . . . . . . . . . . . . . . . . . . . . . . . . . .  168

COORDLINE (38)   . . . . . . . . . . . . . . . . . . . . . . . . .  168

SYNC (50)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  170

REDRAW (51)  . . . . . . . . . . . . . . . . . . . . . . . . . . .  171

DSCOMAND (52)  . . . . . . . . . . . . . . . . . . . . . . . . . .  172

DSXPCOMD (53)  . . . . . . . . . . . . . . . . . . . . . . . . . .  172

BSHELL (54)  . . . . . . . . . . . . . . . . . . . . . . . . . . .  173

ASHELL (55)  . . . . . . . . . . . . . . . . . . . . . . . . . . .  173

DIGTIZE (56)   . . . . . . . . . . . . . . . . . . . . . . . . . .  174

BOX1 (60)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  175

BOXCLR (61)  . . . . . . . . . . . . . . . . . . . . . . . . . . .  175

BOXSIZE (62)   . . . . . . . . . . . . . . . . . . . . . . . . . .  177

BOXPUSH (63)   . . . . . . . . . . . . . . . . . . . . . . . . . .  179

BOXPOP (64)  . . . . . . . . . . . . . . . . . . . . . . . . . . .  179

TATTR (65)   . . . . . . . . . . . . . . . . . . . . . . . . . . .  181

TEXT (66)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  181

VMOVE (67)   . . . . . . . . . . . . . . . . . . . . . . . . . . .  182

VDRAW (68)   . . . . . . . . . . . . . . . . . . . . . . . . . . .  182

BOXBUF (69)  . . . . . . . . . . . . . . . . . . . . . . . . . . .  184

v

Autodesk Device Interface/ADI

vi