README.DOC for SVADI.COM - Super VGA ADI

DOS Real Mode ADI 4.0 Driver for AutoCAD/Sketch/Shade

Copyright (C) 1990-1991 Davis Augustine

Last Updated: January 25, 1991

FILES:

svadi.com	- The Super VGA ADI driver.

svconfig.exe	- The configuration utility for svadi.com.

readme.doc	- This documentation file.

Readme.Doc Contents:

Introduction

1 Installation and Configuration

1.1 Installation

1.2 Using with AutoCAD

1.3 Using with AutoShade

1.4 Using with AutoSketch

1.5 Configuration

2 Screen Text

2.1 Text Fonts

2.2 Configuring Screen Text Areas

2.3 Dual Screen Operation

2.4 Flipscreen Save/Restore

3 Colors

3.1 320x200 x 256 colors mode

3.2 Logical and Physical Colors

3.3 AutoShade Logical Colors

3.4 Drawing Colors in 16 Color Modes

3.5 Drawing Colors in 256 Color Modes

4 VGA Cards

4.1 VGA Display Memory Requirements

4.2 VGA Modes and Monitor Frequencies

4.3 VGA Cardiology

4.4 VESA Modes

5 Known Problems

5.1 AutoShade 386 1.1 Pull-Downs and Dialogue Boxes not Restoring Properly

5.2 AutoShade 1.1 Text Colors not Configurable

5.3 AutoShade Rendering in 16 Color Modes

5.4 Rendering Colors not Matching Wireframe Colors

5.5 Crosshairs Hidden By Some Colors in 256 Color Modes

5.6 Pull-downs not Correctly Replaced When Side Menu Width > 10

6 Troubleshooting

6.1 Driver Not Installed

6.2 Flipscreen (F1 function key) Not Restoring Graphics Screen

6.3 Loss of Graphics Mode or Failure to Enter It

6.4 VGA Screen Goes Dim or Blank During Dual Screen Mode

INTRODUCTION

------------

Svadi is a Super VGA ADI 4.0 display driver for AutoCAD,

AutoSketch and AutoShade.  It is used in place of the built-in

display driver for the IBM Video Graphics Array (VGA).	Its advantages

over the built-in VGA driver include:

o Super VGA resolutions.  The built-in driver operates at 640x480

dots in 16 colors.  Svadi supports 640x480, 800x600 and 1024x768

resolutions, in both 16 and 256 colors, or whatever subset of

those is allowed by your particular VGA card.

o More flexible configuration.  Svadi allows you to change the size

and color of text areas, the text font size, flipscreen behavior,

the red-green-blue shades of display colors, and more.

o Dual screen operation.  If you have both VGA and monochrome

adapters in your system, Svadi can display text on the monochrome

and graphics on the VGA.

o Performance.  Svadi screen redraw is approximately 15 percent

faster than the built-in VGA driver.

1.1 INSTALLATION

----------------

1. Create a subdirectory on your hard disk which will contain

the driver files.  Eg:  mkdir C:\SVADI

2. Copy the files from the product diskette to the hard disk

directory.  Eg:  copy /v A:\*.* C:\SVADI

1.2 USING WITH AUTOCAD

----------------------

Svadi is a "TSR".  That means it loads itself into memory and

stays there until you unload it or until the PC is rebooted.  To load

Svadi, just type SVADI and it will load itself.  If it finds that it has

already been loaded, then it will show a warning message and refuse to

reload itself.	To unload Svadi, just type SVADI /U and it will remove

itself from memory, or show a warning message saying it was not found

in memory.  For a brief help message, type SVADI /H.  (NOTE: if you get

"Command not found" errors when typing SVADI, you need to CD to the

directory that contains Svadi, or put that directory in your PATH.)

The only other thing to be done is to configure AutoCAD to use

ADI 4.0 as its display driver.	From the main AutoCAD menu, go into

"Configure AutoCAD" and then into "Configure Video Display".  Follow the

menu's questions and tell it to use ADI 4.0.  We recommend the default

interrupt vector of 7a.

1.3 USING WITH AUTOSHADE

------------------------

First, configure Svadi for the screen resolution and colors

desired, using Svconfig.  Note that the "Resolution/Modes" menu in Svconfig

has separate displays for Display and Rendering.  The driver is then loaded

and unloaded the same way as for AutoCAD (see above).

You also need to configure AutoShade for ADI.  Type SHADE -r to

enter AutoShade's configuration menus.  For both the display device and

the rendering display device, select "Autodesk Device Interface driver"

(usually it is pick #1).  We also recommend that you stay with the default

interrupt vector 7a.  Answer YES to the questions "do the devices share

a single screen?" and "is a redraw required on flipscreen".  IF YOU DO NOT

ANSWER "YES" THEN THE DRIVER WILL FAIL TO INITIALIZE AND AUTOSHADE WILL

REFUSE TO START UP.  After the last configuration menu, you should be

launched into AutoShade.

Once Svadi is loaded, you can go into and out of AutoCAD and

AutoShade as often as desired.	It is not necessary to reload or

reconfigure the driver between sessions.

Note also that it is possible to use Svadi as an AutoShade driver

and someone else's (or even another copy of Svadi!) as the AutoCAD driver.

You simply configure AutoCAD and driver A for one interrupt (eg 7ah)

and AutoShade and driver B for another (eg 7b) and they will peacefully

coexist.  It is not possible, however, to split AutoShade support between

two different drivers, ie. to use driver A for AutoShade's display

mode and driver B for AutoShade's rendering mode.

1.4 USING WITH AUTOSKETCH

-------------------------

To use SVADI with AutoSketch, configure and load SVADI the same

way you do for AutoCAD.  Then set AutoSketch for ADI 4.0, by typing the

command "sketch -r".  When it gets to the "Configure Display Device" menu,

select "Autodesk Device Interface Display".

1.5 CONFIGURATION

-----------------

The Svconfig utility allows you to modify various characteristics

of the Svadi driver.  The most important of these are the screen resolution

and the logical colors.  The utility is fully menu based and self

explanatory.  Each menu pick displays a help message at the bottom of

the screen.

Note that Svconfig operates on the SVADI.COM driver file.  It does

not affect the driver already loaded in memory.  You must unload the

driver and then reload it in order to get the new configuration you set.

You may want to make a few copies of the driver file and configure

them differently so they are ready when you need them.	For example, you

might have SV640.COM, SV800.COM and SV1024.COM for the different resolutions.

Also, please make sure to keep one original copy of the driver unmodified.

The original distribution diskette will do as long as you don't run Svconfig

on that diskette.

2.1 TEXT FONTS

--------------

The text font is a table of data in the VGA card's ROM which is used

to create the driver's message text characters on the screen.  Note that

this "message text" has nothing to do with "drawing text".  Message text

includes things like pull-down menus and command prompts (see section 2.2:

"Configuring Screen Text Areas" below), while drawing text is that which

you actually include in your drawing, using AutoCAD's TEXT command.

Svconfig allows you to select which font is used for Svadi's message

text.  The default is the 8 by 16 (8 pixels wide, 16 pixels high) font.

There are two smaller ones: 8x10 and 8x14, which probably don't have any

advantages over the 8x16, other than allowing a few more scan lines in the

drawing screen.  Then there is a larger one, 12x24, which is actually

created by stretching, or scaling, the 8x16 font.  It is intended for use

at screen resolutions of 1024x768 and above, when the smaller fonts become

relatively tiny and hard to read.

With the 12x24 font, however, there is the following "fine print":

First, because it is a scaled version of a lower resolution font, it will

not be as smooth and pleasing to the eye as might be expected.	Second, if

it is used at VGA resolutions lower than 1024x768 it will cause some

AutoCAD dialogue boxes and icon menus to not fit on the screen.

When you use Svconfig to change the resolution of Svadi's display

screen, the font size is adjusted if necessary to fit the new resolution.

In particular, if the font is set to 12x24 and you change to a display

resolution below 1024x768, then the font is automatically set to 8x16.

If the current font is 8x16 and you change to a display resolution of

640x350, then the font is automatically adjusted to 8x14 (thus using

the EGA font at the EGA resolution).

2.2 CONFIGURING SCREEN TEXT AREAS

---------------------------------

The three text areas in graphics mode are: the top status line, the

side menu bar, and the bottom scroll area.  The color of the characters

themselves is called the "foreground color".  The "background color" is of

course the color of the screen behind the characters.  Svconfig allows you

to change these logical colors.

These areas are turned on and off by configuring AutoCAD, but their

height and width is determined by Svconfig.  This is done in the "General"

menu.  You can pretty much set them to any value that fits on the screen,

but there are some special considerations regarding the status line width

and the side menu width.

If the status line is set to less than 37 characters wide, then

AutoCAD will abbreviate the status messages.  You may get "L:PLUMBING"

instead of "LAYER:PLUMBING".  Also, if you set it too wide, then the

coordinates may be truncated at the right edge of the screen.  The default

width of the status line is half the screen width, which works pretty well.

Changing the side menu width requires you to recompile AutoCAD's

menu files.  Menu compilation does, among other things, the formatting of

the menu entries for the given menu width.  To cause recompilation of a

menu, you simply delete its ".MNX" file.  AutoCAD will then recompile the

".MNU" file the next time you enter the drawing editor.  Since most menus

are probably designed for the standard 8 character width, it may not be

useful to go fooling with this value.  In addition, icon menus tend to

misbehave at menu widths smaller than 8.

2.3 DUAL SCREEN OPERATION

-------------------------

If your system has a monochrome controller and monitor (MDA), you

can run AutoCAD in "dual screen" mode.  In this mode, the AutoCAD prompts

and text messages appear in text mode on the monochrome screen and the

drawing in graphics mode on the VGA screen.  There is no F1 flipscreen

necessary.

SVADI automatically runs in dual screen mode if AutoCAD is started

from the MDA screen.  To change the current screen to the MDA, use the

DOS command "mode mono".  To change the current screen to the VGA, use the

DOS command "mode co80".

SVADI runs in single screen mode if AutoCAD is started from the VGA

screen.  You can also disable dual screen mode using SVCONFIG (in the

General Configuration Menu), and in that case it will run in single screen

mode even if started from the MDA.

2.4 FLIPSCREEN SAVE/RESTORE

---------------------------

In single screen mode, AutoCAD uses the F1 function key to toggle

between text and graphics mode.  There are also the TEXTSCR and GRAPHSCR

commands, and several others which cause AutoCAD to change to text or

graphics mode.	SVADI can save the graphics screen's contents when you

enter text mode, and then restore them when you go back to the graphics

screen.  The alternative is to have AutoCAD redraw the graphics screen

when you return to graphics mode, but this is usually a much slower

operation.  You can use SVCONFIG (see the General Configuration Menu) to

enable and disable the flipscreen save/restore capability.

When you do an AutoCAD shell, the graphics screen is always redrawn

upon returning to graphics mode.  This is because shell programs often modify

the VGA controller memory, thus destroying the saved graphics image.

AutoShade also allows a flipscreen between the display and rendering

screens.  Svadi does NOT restore the rendering image when you flip back to

the rendering screen.  The rendering screen is left blank.

3.1 320x200 x 256 colors mode

-----------------------------

This mode is the only 256 color mode available in the standard

IBM VGA.  It is allowed in Svadi for the rendering display only.

The resolution is too low to support AutoCAD/Sketch/Shade menus.

If you want to work in 256 colors then you ought to get a super vga card

with 512k of RAM that can do 640x480x256 and 800x600x256.

3.2 LOGICAL AND PHYSICAL COLORS

-------------------------------

AutoCAD currently supports 256 drawing colors and thirty or so

non-drawing colors.  The non-drawing colors are those used by various

elements of the user interface, such as the menus, crosshair cursor

and graphics background.  These 256 drawing and thirty or so non-drawing

colors comprise the set of "logical" colors.

A VGA card can display 16 or 256 colors at a time, depending on the

mode you select.  These are referred to as "physical colors".  Svadi

translates logical colors from AutoCAD into physical colors on the VGA

display.  Svconfig allows you to define this mapping, so that basically

each logical color can be mapped to any of the physical colors.

To further complicate matters, each physical color is translated

by the VGA DAC (Digital to Analog Converter) also known as a LUT (Look-up

Table) into actual RGB (Red, Green, Blue) values.  RGB values are triplets

with each value between 0 and 63.  For example, (63,0,0) is the most intense

shade of red, (0,0,0) is black, and (0,40,40) is a cyan.  Svconfig allows

you to specify RGB values for all of the physical colors.

+---+	 AutoCAD Drawing    +---+				     +---+

| A |	 or Non-Drawing     | S |   Physical	+---+	  RGB	     | V |

| C |	 Logical Color	    | V |    Color	| D |	Triplets     | I |

| A |	------------------> | A |  ---------->	| A | -------------> | D |

| D |	   (-33..256)	    | D |   (0..15 or	| C |  (0..63, 0..   | E |

+---+			    | I |    0..255)	+---+	63, 0..63)   | O |

+---+				     +---+

3.3 AUTOSHADE LOGICAL COLORS

----------------------------

The logical color settings do not work on versions of AutoShade

through 1.1.  Text will be drawn in color 1 (red) on a background that

matches the graphics screen background.

In version 2.0 of AutoShade the Svadi logical colors do work.

You can configure the colors for the pulldown menus, the dialogue boxes

and the top and bottom command lines.  Use the "Cmd Scroll Area" pick

to change the color of the top and bottom command lines.

3.4 DRAWING COLORS IN 16 COLOR MODES

------------------------------------

When the VGA is used in 16 color mode, it is normally set up to

provide 2 shades each of red, yellow, green, cyan, blue, magenta and

gray plus 1 shade each of white and black.  Since AutoCAD's drawing color

numbers range from 1 to 255 and AutoSketch's from 1 to 127, the obvious

question is: what do we do with all the extra drawing colors?

The lower drawing colors are firmly standardized (see

AutoCAD Reference Manual) as:

Background    0      1    2	3    4	  5    6    7	    8

Light    lt gray  red  yel  grn  cyn  blu  mag black  dk gray

Dark      black   red  yel  grn  cyn  blu  mag  wht	 dk gray

In Svconfig's "Logical Colors" menu there is a selection

called "Drawing Logical Colors" which allows you to specify the way

in which drawing colors above 8 are laid out.  The selections are

1: Approximate 256 Mapping.  This is the default scheme.  Color 9 is white

(if light background) or light gray (if dark background).  Colors 10

through 249 are mapped to the "nearest available" color.  For example,

the 230's (maroon), 10's (red), 20's (reddish orange) and 30's (orange)

are all mapped to red or dark red.	The dimmest colors, 18, 19, 28, 29,

etc. are mapped to dark gray.  Colors 250 through 255 are shades of

white, gray and black.

2: Approximate 128 Mapping.  This is similar to the above, but the range

is squeezed to fit into 128 drawing colors for AutoSketch.	Colors

128 through 255 are set to white.  They should not be used.

3: Set all to Gray.  In this scheme we admit that it is a 16 color device

and don't bother with the higher drawing colors.  Colors 9 through 14

are dark versions of 1 through 6.  Color 15 is white if the background

is light and light gray if it is dark.  All higher colors are mapped

to gray.  This scheme is used by AutoCAD's internal VGA driver (with

higher colors mapped to white).

4: Repeat 0..15.  In this scheme we also admit that it is a 16 color device

but we are too lazy to even turn off the higher colors.  Drawing

colors 0 through 15 are mapped as in scheme 3 above, and the same

16 color pattern is repeated all the way through 255.  For bits and

bytes aficionados, Color = Color and 0fh.  This scheme is not

recommended.

Finally, it is possible to make up your own custom palette through

the "Detailed Logical Colors" menu.  You can even change the standard

drawing colors below 9 if you wish.  Of course, you will then have non-

standard drawings and might have difficulty interfacing with other drawings,

documentation, software and CAD operators.

3.5 DRAWING COLORS IN 256 COLOR MODES

-------------------------------------

When the VGA is used in 256 color mode we are able to assign a

unique shade to each drawing color.  In the standard layout, each color

consists of 3 components: its hue, brightness and saturation.  The hue

changes every 10 drawing colors, like this:

10: red, 20: reddish orange, 30: orange, 40: yellowish orange,

50: yellow, ... 90: green, ... 130: cyan, ... 210: magenta.

The brightness changes every 2 drawing colors, so 10, 12, 14, 16 and 18

are the brightest to darkest reds.  The saturation changes every drawing

color.	Odd colors are half saturated, or pastel.  Thus, 11, 13, 15, 17

and 19 are the brightest to darkest pastel reds.  Colors 250 through

255 are shades of gray.

The AutoCAD sample drawings CHROMA and COLORWH are useful for

understanding the 256 color palettes.  There is also a discussion similar

to this one in the AutoCAD Reference Manual.

AutoSketch only supports 128 drawing colors.  We cannot use the

above palette because the highest color, 127, only gets us into the

aquamarines!  The cyans, blues, purples and magentas would be inaccessible.

Therefore, a 128 color palette is provided for use with AutoSketch.

The 128 color palette uses 18 hues, 3 intensities and 2 saturations,

(We have seen above that the 256 color palette uses 24 hues, 5 intensities

and 2 saturations.)  Thus, 10, 12 and 14 are bright to dark red and

11, 13 and 15 are bright to dark pastel red.  Drawing colors 118 through

127 are shades of gray.

The 128 and 256 color palettes are selected by going into the

"Physical Colors" menu and then selecting "Reset all DAC Values".  The

128 color distribution should only be used with AutoSketch.  Note also

that if you import and export drawings from AutoCAD and AutoSketch, the

higher drawing colors will be perceived differently.  There is not really

a solution to this problem, other than to not use the higher drawing

colors if you are going to view the drawing in both products.

4.1 VGA DISPLAY MEMORY REQUIREMENTS

-----------------------------------

VGA cards store the raster display data in on-board memory.  The

maximum screen resolution is thus limited by how much memory is on the

particular VGA card.  Here are the minimum memory requirements for the

various VGA resolutions:

16 Colors		256 Colors

---------		----------

640x480  256k		640x400  256k

800x600  256k		640x480  512k

1024x768  512k		800x600  512k

1024x768  1Meg

4.2 VGA MODES AND MONITOR FREQUENCIES

-------------------------------------

You will most likely be using your VGA card with a "multi-scan"

or "multi-sync" monitor.  Unlike "fixed frequency" monitors, the multi-scan

monitors are capable of operating in a range of frequencies.  Following is

a table of typical refresh rates for various VGA modes.  You should check

with your hardware supplier to be sure that your monitor will handle all

of the resolutions of your super VGA card.

Resolution	Vertical	Horizontal

----------	--------	----------

640x350 	 70 Hz		 31.50 kHz

640x480 	 60 Hz		 31.50 kHz

800x600 	55/60 Hz	35.20/37.88 kHz

1024x768 	 60 Hz		 48.00 kHz

1024x768 	43.48 Hz	 35.52 kHz (interlaced)

4.3 VGA CARDIOLOGY

------------------

"Therefore is the name of it called Babel; because the Lord did

there confound the language of all the earth."  - Genesis 11:9

The heart of a VGA card is the VGA controller chip.  There are

several VGA chip vendors.  They are all different from and incompatible

with each other.  They may also have multiple versions of their own chips

(eg Trident 8800 and 8900) and thus be incompatible even with themselves.

The same VGA controller chip may be used by several board

manufacturers.	In addition, board manufacturers may put out different

products based on different controllers.  Generally speaking, boards

which come from different board vendors but which use the same controller

chip are compatible with each other.  For example, you can configure the

ADI driver for Orchid Prodesigner Plus and it will work ok on an STB EM16,

because both cards incorporate the Tseng Labs ET3000 controller chip.

VGA circuitry may also be built into the PC's mother board, as

in the case of IBM's, Compaq's and a numerous imported clone PCs.  These

built in controllers are often "standard" VGA (no extended modes),

but not always.

There are several companies which manufacture both the controller

chips and the boards (eg. Headland and ATI).  They generally do not resell

or OEM the chips to other board makers.

And then there are some companies which manufacture neither

chip nor board, simply buying ready-made boards and putting their own

name on them for reselling.

Finally, there is the Video Electronics Standards Association.

VESA is a consortium of chip, card and software companies which is trying

to restore order to the VGA anarchy.  They have defined a standard

software interface which appears the same on all vendors' VGA cards.

See below for more information about VESA modes.

Following is a list of VGA cards supported by Svadi.  The ones

with a "*" next to them have not been tested by Autodesk yet (due to

unavailability of boards), but are believed to work because of their

compatibility with cards which have been tested.

Controller			Card

----------			----

Chips and Technologies	       *Chips and Technologies

*Boca Research 1024

Metheus Premier

Compaq				Compaq VGA adapters and

SystemPro motherboards

Genoa 6400			Genoa

Headland (Video 7)		Headland (Video 7) VEGA,

FastWrite, VRAM

IBM				Standard VGA on IBM adapters

and motherboards

Tseng Labs ET3000	       *Orchid Prodesigner Plus

SOTA VGA/16

*STB EM16

*Tecmar

*Tseng Labs ET3000 (Generic)

Tseng Labs ET4000		Orchid Prodesigner II

Sigma Legend

STB PowerGraph

*Tseng Labs ET4000 (Generic)

VESA Standard Interface        *Various Vendors

Western Digital (Paradise)	Western Digital (Paradise)

4.4 VESA MODES

--------------

If you think of all the various super VGA cards each speaking

their own unique languages, then VESA is the universal language.

Applications and their drivers will only need to speak VESA, and

the board vendors will either build boards that speak VESA natively

or provide translators that mediate between VESA and their native

tongue.  As with all standards, this only works if and when it is

believed and participated in by enough people.	Although VESA may not

yet have achieved "critical mass", it does appear to have pretty good

momentum at this time.

In theory, all VGA cards, old and new, are capable of supporting

VESA.  If the card's ROM BIOS does not support it directly, then a small

DOS TSR program can be provided which does the translation from VESA to

the card's BIOS.  These TSR's might be found on the card's utility and

driver diskettes, or on the electronic bulletin boards that some vendors

provide for customer updates.  You might also call the vendor's support

line or your local dealer.

Svadi supports VESA versions 1.0 and up.  Note that with many

cards you can choose between two ways of using Svadi: it can be

configured for that card specifically or for VESA.  There is not much

difference between the two methods.  The VESA configuration may use a

little more memory (because the vendor's VESA TSR interface is loaded)

and may be a little slower in some cases.

5. KNOWN PROBLEMS

5.1 AUTOSHADE 386 1.1 PULL-DOWNS AND DIALOGUE BOXES NOT RESTORING PROPERLY

--------------------------------------------------------------------------

The screen area underneath some pull-down menus and dialogue boxes

are not restored after the menu or dialogue box is exited.  It is more

likely to happen with the larger menus and dialogue boxes.  For example,

try SHADING MODEL under the SETTINGS menu.  Then pick BACKGROUND COLOR,

and pick CANCEL.  Pieces of the last dialogue box will remain on the screen.

This only happens with version 1.1 of AutoShade 386, and with

Svadi configured to either: (a) a display mode other than 640x480 and 1024x768

(in either 16 or 256 colors); or (b) any display mode with the text font

configured for the 12x24 (large) size.

The problem is not correctable.  If you have this version of

AutoShade 386 then you should configure the display resolution to 640x480

or 1024x768 in either 16 or 256 colors, and do not use the 12x24 text font.

5.2 AUTOSHADE 1.1 TEXT COLORS NOT CONFIGURABLE

----------------------------------------------

The logical color settings do not work on versions of AutoShade

through 1.1.  See section 3.3 above.

5.3 AUTOSHADE RENDERING IN 16 COLOR MODES

-----------------------------------------

Some drawings will not properly render if Svadi is set to a

16 color rendering mode.  You may get an alert message saying "Too

many shades generated, some lost.", after which it will still produce

a rendering.  In the worst case it will hang up while displaying the

screen message "Calculate shading and assign colors.".  You can control-C

out at this point, and then use F1 to flip back to the menu screen.

This is known to happen with the sample CHROMA file.  It is recommended

that you do rendering in 256 color modes.

5.4 RENDERING COLORS NOT MATCHING WIREFRAME COLORS

--------------------------------------------------

Colors 7, 8 and 9 are shades of white, gray and black (depending

on the graphics background setting) in the wireframe, while they are all

a light gray in the rendering.	This can be observed in the sample CHROMA

rendering file.  This is normal and is a result of the difference between

the rendering algorithms and the default Svadi drawing colors.

5.5 CROSSHAIRS HIDDEN BY SOME COLORS IN 256 COLOR MODES

-------------------------------------------------------

In 256 color modes, the crosshairs do not show up on objects of

some colors.  They do show up against the background and against the

basic drawing colors 1 through 15.  The easiest way to see this is to

bring up the drawing "CHROMA", which shows all of the 255 drawing colors,

and move the crosshairs across the different boxes.  The crosshairs will

appear on some boxes and not on others.  On some it will be very dim.

This problem will be fixed in a later release.

5.6 PULL-DOWNS NOT CORRECTLY REPLACED WHEN SIDE MENU WIDTH > 10

---------------------------------------------------------------

This only occurs in AutoCAD Release 11, when Svadi's side menu

width is configured to 11 or more characters.  It occurs with the pull-

down menu fields that have submenus.  These fields have a ">" (greater

than symbol) next to them.  The problem is that the submenu appears but

the bottom of the parent menu extends below it.

For example, select the OPTIONS menu from the top status line.

Then select HATCH.  The HATCH menu will appear correctly, but instead

of ending at the "OPTIONS >" field, there will be several more fields

below it which are inherited from the previous OPTIONS menu.

As long as you do not select one of these additional fields,

this problem should not affect anything.  If you want to avoid it

completely, then you would have to set the side menu width to less

than 11 characters (8 is the normal width).

6 TROUBLESHOOTING

6.1 DRIVER NOT INSTALLED

------------------------

If, when entering an AutoCAD drawing, you get a message like:

"ERROR: No display driver is installed at interrupt vector xx" then

either Svadi is not installed or it is configured for a different

interrupt vector from AutoCAD's.  You should double check that Svadi

and AutoCAD are both configured for the same vector (eg 7a) and that

Svadi has been loaded into memory.

AutoShade will give a slightly different error message, such

as "Fatal Error: Rendering driver not installed at interrupt ...".

This indicates the same problem: either Svadi is not installed or it

is not configured for the same interrupt vector as AutoShade.

Note that AutoShade uses two interrupts, one for displaying and

one for rendering, and they must both be matched to Svadi's.  If you get

the message "Error status returned by ADI driver" then AutoShade has found

a driver at the interrupt but it is not the correct type (display or

rendering).  In this case, Svadi is probably installed but is not

configured with the same display and/or rendering interrupt as AutoShade.

One final caution about interrupt vectors: you need to watch out

for conflicts with other software packages that use them.  One common

example is the driver software used by many LANs.  There are some which

are shipped with a default setting to use interrupt 7a, which is also

the default for Svadi and AutoCAD.  In that case, you must change either

the network driver or Svadi and AutoCAD to use a different vector (7b,

for example).  Svadi will display a warning message if there is already

somebody using its interrupt vector when it is loaded.

6.2 FLIPSCREEN (F1 FUNCTION KEY) NOT RESTORING GRAPHICS SCREEN

--------------------------------------------------------------

See the section above on Flipscreen Save/Restore for a description

of this subject.  If the save/restore is not working, then you may see

streaks or blotches at the top of the graphics screen.	The part that is

in the drawing will be cleared up with a REDRAW, but any garbage in the

status bar and menu areas will remain there.  A SHELL command will cause

the entire screen to be redrawn and the garbage to disappear.

This behavior indicates that Svadi does not know how to save

and restore the graphics screen for this particular card.  If this happens,

then you should go into Svconfig's GENERAL menu, then select FLIPSCREEN

SAVE/RESTORE, and then DISABLE.  This will cause Svadi to redraw the entire

screen whenever you change from text back to graphics.

6.3 LOSS OF GRAPHICS MODE OR FAILURE TO ENTER IT

------------------------------------------------

Because there is such a variety of species of PC's and of VGA

cards, there may be cases where a particular card does not work properly

in your PC at a certain display resolution.  The types of problems you

may get are:

o AutoCAD (AutoShade, etc) may return an error message when going

into graphics mode (the drawing editor) saying it cannot initialize

the display adapter.

o AutoCAD (etc) may proceed into graphics mode, thinking the VGA

is functioning properly, while you see some form of scrambled

graphics, blank screen or other unexpected display.  Or, the

trouble may occur after AutoCAD has been running for a while,

although this is less likely.

There are many ways for these problems to occur, and diagnosing

them can be difficult and frustrating.	You should seek help from your

dealer or the VGA vendor's technical support.  You should not open

your machine or fiddle with card slots, dip switches and jumpers

unless you are familiar with such practices.  Following is a list of

possible things that you can check out, but it is not meant to be

comprehensive or to take the place of competent technical support:

o Check the monitor and VGA card frequencies.  Make sure that the

mode displays at a scanning frequency that the monitor can handle.

o Check the display memory on the card.  Make sure that there is

enough display RAM on board (see table above).

o Make sure jumpers and switch settings are correct.  Most cards

have a switch for 8-bit and 16-bit BIOS.	Try it both ways.

o Try the card in both 8-bit and 16-bit slots.  Some cards will only

work in one or the other in certain modes on certain machines.

o Run any diagnostic programs that came with the card.  Try to

bring up the graphics mode that you are having trouble with.

This can help determine whether it is a hardware or software

problem.

o Try a different VGA card.  If it works then you may have a

bad VGA card or one that is incompatible with your PC.

o Try the VGA card in a different PC.  If it fails then you may

have a bad card.

o Some PC's have more than one speed setting, perhaps "turbo" and

"normal" modes.  See if the card works at any of the speeds.  If

so then it is probably a compatibility problem between card and PC.

o If you have other application software that uses VGA graphics,

try to run it.  This will also help distinguish between software

and hardware problems.  You may get different results even if

the problem is in the hardware, however, since different software

may access the VGA in different ways.

o Try it with or without a monochrome card in the system.  Some VGA

cards are affected by this.

6.4 VGA SCREEN GOES DIM OR BLANK DURING DUAL SCREEN MODE

--------------------------------------------------------

Before you even enter AutoCAD, when you change the current

screen from the VGA to the monochrome (by typing "mode mono"), the

VGA screen goes dim or blank.  This is caused by an incompatibility

between the VGA card and the PC.  It may be fixable or not.  See if

the card has any jumpers relating to companion monochrome cards.  See

if the PC's user manual or setup program discuss dual video cards.

Try changing the VGA card from an 8-bit to 16-bit slot, or vice versa.

Try changing the jumper on the card from 8-bit BIOS to 16-bit BIOS.

If nothing works, then you will have to use single screen mode.