Autodesk, Inc.

Autodesk Device Interface/ADI

Driver Development Kit

Version 2.1

_______

Graphics Displays

Digitizers

Plotters

Printer/Plotters

Publication TDADIDOC01.00                      December 1, 1986

Product of Autodesk, Inc.

This publication  is distributed  by Autodesk  so that independent

parties   can   develop   graphics   peripheral   device   drivers

independently of  Autodesk,  Inc.    This  publication,  or  parts

thereof, as  well as associated materials distributed on diskette,

and software  developed using  these materials,  i.e.  ADI  device

drivers  intended   to  function   with  Autodesk,  Inc.  software

products, may  be distributed  freely without royalty to Autodesk,

Inc., subject to the following conditions.

Users of  the ADI  Driver Development  Kit are  granted a  limited

license to  use and/or  make copies  of these materials for use by

others in  order to develop graphics peripheral device drivers for

Autodesk products.  This limited license does not include:

1. Distributing the  ADI Kit  in connection  with any  other

product.

2. Making the  ADI Kit  available for  any consideration  or

"disk fee."

3. Distributing the ADI Kit in modified form.

Autodesk, Inc.  makes no  warranty, either  expressed or  implied,

including  but   not  limited   to  any   implied  warranties   of

merchantability or  fitness for  a particular  purpose,  regarding

these materials  and makes  such materials available on an "as-is"

basis.

In no  event shall Autodesk, Inc. be liable to anyone for special,

collateral, incidental,  or consequential  damages  in  connection

with or  arising out  of purchase  or use of these materials.  The

sole and  exclusive liability to Autodesk, Inc., regardless of the

form of  action, shall  not  exceed  the  purchase  price  of  the

materials described herein.

Autodesk, Inc.  reserves the  right to  revise and  improve  these

materials as it sees fit.  This publication describes the state of

the ADI  Kit at  the time  of its publication, and may not reflect

the ADI Kit at all times in the future.

The latest  version of  the ADI  Driver Development Kit, including

printed documentation,  may be  obtained by sending $100 in check,

VISA, or MasterCard to:

ADI Program

Autodesk, Inc.

2320 Marinship Way

Sausalito, California 94965

Add $6 sales tax if you are located in California.

Comments on  these materials are welcome.  Correspondence may also

be exchanged  on the  Autodesk, Inc. Forum on CompuServe.  Type GO

ADESK when you log on.

IBM and PC-DOS are registered trademarks of International Business

Machines  Corporation.     MS-DOS  is  a  trademark  of  Microsoft

Corporation.     Intel  is   a  trademark  of  Intel  Corporation.

CompuServe is a registered trademark of CompuServe Incorporated.

AutoCAD is  a registered  trademark of  Autodesk, Inc.    Autodesk

Device Interface/ADI,  ADE,  3D  Level  1,  CAD/camera,  AutoLISP,

AutoSHADE, AutoSketch, and AutoCAD AEC are trademarks of Autodesk,

Inc.

Table of Contents

Chapter 1  THE AUTODESK DEVICE INTERFACE/ADI ................ 1

1.1 Overview ............................................ 1

1.2 Developer's Responsibilities ........................ 1

1.3 What You Need ....................................... 1

1.4 What The ADI Kit Includes ........................... 1

1.5 Working with ADI .................................... 1

1.6 Testing ............................................. 2

1.7 Releasing the Driver ................................ 2

Chapter 2  THE ADI GRAPHICS DISPLAY SPECIFICATION ........... 3

2.1 The ADI Graphics Display ............................ 3

2.2 Configuration ....................................... 3

2.3 The ADI Graphics Display Interface .................. 4

2.4 Writing Display Drivers ............................. 5

2.5 Display Driver Functions ............................ 6

2.6 AutoCADR Standard Colors ........................... 15

2.7 ADI Graphics Display Test Procedures ............... 16

Chapter 3  THE ADI DIGITIZER SPECIFICATION ................. 21

3.1 The ADI Digitizer .................................. 21

3.2 Configuration ...................................... 21

3.3 The ADI Digitizer Interface ........................ 21

3.4 ADI Digitizer Test Procedures ...................... 25

Chapter 4  THE ADI PLOTTER SPECIFICATION ................... 29

4.1 The ADI Plotter .................................... 29

4.2 Configuration ...................................... 29

4.3 Using the Driver ................................... 31

4.4 Output Format ...................................... 31

4.4.1 File Format .................................. 33

4.5 Installed Driver Operation ......................... 34

4.6 ADI Plotter Test Procedures ........................ 36

Chapter 5  THE ADI PRINTER/PLOTTER SPECIFICATION ........... 37

5.1 The ADI Printer/Plotter ............................ 37

5.2 Configuration ...................................... 37

5.3 Using the Driver ................................... 39

5.4 Output Format ...................................... 39

5.5 Installed Driver Operation ......................... 40

5.6 ADI Printer/Plotter Test Procedures ................ 42

Appendix A  REVISION HISTORY ............................... 43

A.1 ADI Display Interface Version 2.0 (October, 1986) .. 43

A.2 ADI Display Interface Version 2.1 (December, 1986) . 43

__________________________________________________________________

Chapter 1

THE AUTODESK DEVICE INTERFACE/ADI

__________________________________________________________________

1.1 Overview

The Autodesk  Device Interface  is an interface specification that

allows  manufacturers,  dealers  and  users  to  develop  graphics

peripheral drivers  that work  with MS-DOS  versions of  AutoCADR,

AutoSketchtm, and  other Autodesk  products.   By using the ADI, a

developer can  write, test,  distribute and  support a  peripheral

driver that  is compatible with Autodesk software products without

having to wait for Autodesk, Inc. to implement the driver.

1.2 Developer's Responsibilities

ADI drivers  are distributed and supported by the developer (you),

not by  Autodesk, Inc.   It is your responsibility to inform users

how to  install and  configure your driver and peripheral.  If you

are a peripheral manufacturer, and wish to use the ADI Development

Kit, you must supply Autodesk with the name of a technical support

contact person  in your  organization.  Autodesk's product support

staff will  be supplied  with a list of these names, and questions

from users  about your  driver will  be referred to that person in

your organization.

1.3 What You Need

To implement an ADI driver, you should be familiar with AutoCAD or

the Autodesk product you intend to support and its operation.  You

must  purchase   the  Autodesk   product  to  test  your  finished

interface.   An ADI driver may be written in any language that can

utilize MS-DOS  software interrupts; however, the examples in this

documentation are  in Lattice  C, version  2.14 or  2.151,  or  in

assembly language.

1.4 What The ADI Kit Includes

The ADI  Kit contains  C and  assembly language sample source code

and this  documentation.   The sample  source code, or any portion

thereof, can  be used  in your  ADI driver implementation, without

any royalties or additional fees paid to Autodesk, Inc.

____________________

1The examples in the ADI Development Kit are intended for use with

Lattice C  version 2.14 or 2.15.  If you are using a later version

of this compiler, or another compiler, you will need to modify the

examples accordingly.

1

The Autodesk Device Interface - Chapter 1

The disk  files also  include  batch  files,  link  files,  sample

installation guide text, and debugging aids.

1.5 Working with ADI

Because of resource limitations, Autodesk cannot provide technical

assistance to  ADI Kit  developers.  ADI Kit users are expected to

have  sufficient  technical  skills  to  implement  their  drivers

without assistance  from Autodesk.   If  a bug  or  deficiency  is

discovered in the ADI Kit, developers will be notified through the

Autodesk Forum on CompuServe.

ADI Kit  users  may  wish  to  exchange  information  through  the

Developers' Corner  in the  Autodesk Forum on CompuServe.  Type GO

ADESK when you log on to CompuServe.

1.6 Testing

The importance  of testing  your finished  ADI  driver  cannot  be

overemphasized.   This  documentation  describes  the  tests  that

Autodesk's Quality  Assurance Department performs AS A MINIMUM for

every device  driver supported.  Since you are responsible for the

support of  the finished  ADI driver,  this testing is critical to

the success  of your  driver and also to reduce the volume of user

support calls  that may  be generated and directed to you due to a

malfunctioning driver.

1.7 Releasing the Driver

How you  distribute your ADI driver is up to you.  You may want to

ship it on a disk included with your product, or make it available

to  your   dealers.    The  AutoCAD  files  DSGEN.DRV,  DGGEN.DRV,

PLGEN.DRV, and  PPGEN.DRV may  not be  distributed by  you.  These

drivers  are   proprietary  Autodesk  software  products  and  are

specific to individual AutoCAD releases.  AutoCAD users may obtain

updates of  these drivers  (if necessary)  from  their  authorized

AutoCAD dealer.   Versions  of these  drivers are included in each

AutoCAD MS-DOS  release (along  with the  other peripheral  device

drivers).   Your driver  should continue to work with new releases

of Autodesk  software products  without any changes required.  New

features added  to the  ADI interface  may require that you update

your ADI driver in order to take advantage of them.

Autodesk,  Inc.   may  choose   to  publish  a  list  of  hardware

manufacturers and  other developers  known to  have completed  ADI

device drivers.   These  lists may  be distributed  to dealers and

posted in  the Autodesk  Forum on  CompuServe.  To be eligible for

this list,  you must  send Autodesk, Inc. a copy, on disk, of your

completed ADI  driver, along with all user documentation.  Send it

to the ADI Program, Autodesk, Inc., 2320 Marinship Way, Sausalito,

CA 94965.

2

___________________________________________________________________

Chapter 2

THE ADI GRAPHICS DISPLAY SPECIFICATION

__________________________________________________________________

The following  describes, in  detail, the  interface specification

for ADI graphics display drivers.

2.1 The ADI Graphics Display

The ADI Driver Development Kit disk for graphics displays contains

source and  executable code  for an  ADI  Hercules  Graphics  Card

driver (DSHERC.C).   The  IWAIT.ASM routine  is also  on this disk

(more on  this later).   DSHERC.C  has numerous debugging features

and can  emulate a  dumb display  that clears  everything on  flip

screen, or  a  smart  one  with  higher  level  features  just  by

compiling with  DUMB or  SMART defined.   The  disk also  contains

source and  executable for  the DSDEMO  debugging aid,  as well as

ADI.H  (functions   common  to   all  ADI  drivers),  and  DSADI.H

(functions for graphics display drivers).

2.2 Configuration

The driver,  DSGEN.DRV, is  integrated into  selected versions  of

AutoCADR.  This driver appears on the AutoCAD Display options menu

as ADI display Vxx.1

DSGEN.DRV performs  its functions  by calling a DOS "terminate and

stay resident"  display driver  (the  one  you  create)  installed

before AutoCAD  was executed.   When  AutoCAD is  configured  with

DSGEN.DRV,  one  of  the  configuration  questions  asked  is  the

interrupt vector  number used  to communicate  with your installed

driver.

____________________

1DSGEN.DRV was  first distributed  in the  IBM PC/XT/AT version of

AutoCAD 2.18 shipped in March of 1986.  This driver appears as ADI

display v2.18 on the menu selection.  AutoCAD 2.18 users must have

this driver  for the  ADI graphics display to work--DSGEN.DRV with

an earlier  file date-stamp  will not  function correctly.    This

driver is  available to  users as  an update  from their  dealers.

AutoCAD 2.5  (or later  version) users  must  have  the  DSGEN.DRV

distributed with their AutoCAD.  See the appendix to this document

for a  list of which versions of DSGEN.DRV support which features.

To use  all of the extended functions of the ADI display interface

introduced in  this version  of the  kit (version 2.1), users must

have AutoCAD  2.52 and  the DSGEN.DRV  that appears in the AutoCAD

configuration menu as ADI display V2.1 (or later).

3

ADI Graphics Display Version 2.1 - Chapter 2

The generic  display driver communicates with the installed driver

through the  registers AX,  BX, CX,  and DX.  Arguments are loaded

into these  registers and the driver is invoked with an INT to the

vector used  to call  the driver.   AX  is always  loaded  with  a

function code that selects the task the driver is to perform.  The

other registers have meanings that vary from function to function.

The display  driver returns its status in the same four registers.

The meaning of the results returned depends upon the function.

2.3 The AutoCAD Graphics Display Interface

It is  necessary to  understand what the AutoCAD display interface

looks like  and how it works.  To accomplish this, it is suggested

that you  experiment with  AutoCAD and its features as they relate

to  the  display  (virtually  all  of  them  do).    Conceptually,

AutoCAD's display  interface uses two screens - a text screen of X

rows by  Y columns (the "hidden" screen), and a graphics screen of

X pixels  by Y pixels (see illustration in the Introduction to the

AutoCAD Drafting  Package Reference Manual).  The AutoCAD user may

flip between  these screens  at any time by the push of a function

key.   The text  screen contains  the last  25 lines of text.  The

graphics screen  is  logically  divided  into  four  windows:  the

graphics area,  the text  scrolling area, the status line, and the

screen menu area.  The last three of these areas can be configured

on or  off at the user's option, in order to maximize the graphics

area of  the screen.   Since  not all  display  hardware  has  the

ability to  store and  manipulate a complete graphics bitmap AND a

full screen  of text,  AutoCAD will take care of certain functions

that the  display device  cannot handle.  The ADI driver must tell

AutoCAD  what   capabilities  it   has   through   a   series   of

initialization calls.   For  example, if  the display  device  and

driver are  not capable  of storing  and  scrolling  an  80  x  25

character  text  screen,  AutoCAD  will  keep  a  buffer  of  2000

characters and scroll it as needed. There are two forms of display

devices: those  that can operate in a single screen mode and those

that operate  in a  dual screen mode.  Dual screen devices use one

monitor for  the graphics display and another monitor for the text

screen.   Examples of currently supported dual screen displays are

the Control Systems Artist I and the Number Nine Revolution board.

Single screen  displays use  only one monitor.  Examples of single

screen displays  are  the  Hercules  Graphics  Card  and  the  IBM

Enhanced Graphics Adapter.

A user  configuring AutoCAD  for  use  with  an  ADI  driver  will

configure  AutoCAD  according  to  the  AutoCAD  Drafting  Package

Installation/Performance  Guide   for  an   ADI  display  and  the

instructions you provide.  You should fully acquaint yourself with

this process  in order to provide clear documentation for users of

your ADI driver.

4

ADI Graphics Display Version 2.1 - Chapter 2

2.4 Writing Display Drivers

Thanks to  a magical  routine called  IWAIT, display  drivers  may

easily be  written in  C.   A display  driver  should  be  written

essentially  as   a  large   loop.    After  doing  any  once-only

initialization, the driver should fall into a loop as follows:

while (1) {

iwait(VECTOR, &iregs, &oregs);

... processing ...

}

IREGS  and   OREGS  are   structures  consisting  of  four  SHORTs

corresponding to the registers AX, BX, CX, and DX.  For example:

struct cregs {

short ax, bx, cx, dx;

};

static struct cregs iregs, oregs;

On the  first call  to IWAIT, the program will be installed at the

interrupt vector  given by  VECTOR, and  will terminate but remain

resident.   When a  user makes an INT to that vector, the contents

of his  AX through  DX will  be stored in IREGS and the IWAIT will

"return" to  the resident  program.   It may do whatever it likes,

then load its results in OREGS, and re-execute the IWAIT.  You may

notice that  IWAIT is a complement of the Computer Innovations C86

library function,  SYSINT (except  that it  uses fewer registers).

In fact,  a pair  of programs  doing SYSINTs  and  IWAITs  form  a

coroutine linkage.   IREGS and OREGS may (and usually will) be the

same structure.

For  more  information  on  this  linkage  (which  makes  lots  of

difficult things easier to do), look at the demo program DSDEMO.C,

as well  as DSHERC.C.   The  ultimate reference, of course, is the

code in IWAIT.ASM.

To aid in identifying the ADI driver, the developer should include

a message,  displayed at  driver load time, indicating the version

number of  the driver, the name of the developer, and the date the

driver was created.

WARNING !!

Both MS-DOS  and the Lattice C compiler rely on the decrement flag

being CLEAR.  If your driver sets the decrement flag (i.e. doesn't

perform a  CLD instruction  before returning to AutoCAD) horrible,

horrible things will probably happen.

5

ADI Graphics Display Version 2.1 - Chapter 2

2.5 Display Driver Functions

In the  following descriptions of the arguments passed to and from

the driver,  two items enclosed in parentheses means that they are

stored as  a single  16 bit value, with the first item in the most

significant 8  bits and the second item in the least significant 8

bits.

INIT1       Code (AX): 1   Arg1 (BX)  =   (0, config)

Arg2 (CX)  =   (0, intlevel)

Result      Code (AX): ok  Res1 (BX)  =   ixdots

Res2 (CX)  =   iydots

Res3 (DX)  =   ixdotsm

The INIT1  is the initial call to the driver.  The CONFIG argument

specifies the  screen configuration requested by the user when the

driver was  installed.   It is  a bit  coded value, with the 1 bit

indicating that  the text  scrolling area  is desired,  the 2  bit

indicating that  the screen  menu is  to be  drawn, and  the 4 bit

indicating  that  the  status  line  is  to  be  used.    INTLEVEL

identifies  the   version  of  the  driver  interface  for  upward

compatibility.   This version of the driver passes 3 for INTLEVEL.

The driver  returns IXDOTS  and IYDOTS  equal to one less than the

horizontal and  vertical size,  respectively, of the graphics area

of the  screen (less  menu,  scrolling  area,  and  status  line).

IXDOTSM is  the maximum  horizontal dot  number, including  screen

menu, beginning at zero (total dots in x direction = IXDOTSM + 1).

If the  device was  correctly initialized,  OK should  be returned

equal to 1, otherwise it should be set to 0.

INIT2       Code (AX): 2

Result         Res1 (BX)  =   (modelinl, mnuchars)

Res2 (CX)  =   (maxboxes, modes)

Res3 (DX)  =   (split, hwfill)

The INIT2 call causes the driver to furnish additional information

to AutoCAD.   MODELINL  is the  number of characters available for

the mode  status line.   MNUCHARS  is  the  number  of  characters

available for screen menu items.  MAXBOXES is the number of screen

menu boxes  on the  screen.   MODES is  a  bit  coded  field  that

specifies modes  pertaining  to  the  operation  of  this  display

device.   The 1  bit indicates  that echoing  a Control  C to  the

screen while  in graphics  mode  will  destroy  the  display,  and

therefore that  Control C  should be suppressed.  This mode should

be set  only on  single screen  displays that  lack separate image

planes for  text and  graphics.   The 2  bit should be set if this

display implements  highlighting for  selected objects (preferably

by drawing them with dashed lines).  The 4 bit is set if this is a

single screen  that switches  between a  text screen  and graphics

screen, and  should be  clear if  the driver  supports a  separate

graphics screen.   If  this is  a  single  screen  display,  SPLIT

specifies the  number of  lines of  text that  appear in  the text

6

ADI Graphics Display Version 2.1 - Chapter 2

scrolling region  (normally three).   If  this is  a  dual  screen

display (bit  4 clear  in MODES),  SPLIT should  be set  to  zero.

HWFILL is  set to  0 for  devices without hardware fill, and 2 for

those with polygon fill (1 is not used).

INIT3       Code (AX): 3

Result                     Res1 (BX)  =   pixwid

Res2 (CX)  =   pixhgt

Res3 (DX)  =   (hidlines, hidcols)

The INIT3  call requests  further  information  from  the  driver.

PIXWID is  the horizontal  physical spacing  between  pixels,  and

PIXHGT is the vertical physical spacing between pixels.  The ratio

of PIXWID  and PIXHGT  is used  to correct  the "aspect  ratio" of

objects drawn  on the  screen  so  that  horizontal  and  vertical

measurements are  equal on  the screen.   On a device with "square

pixels", PIXWID  and PIXHGT  should both  be set to 1.  On devices

with different  X and  Y scales,  PIXWID and  PIXHGT should be set

based on the physical screen measurement in each direction divided

by the  number of  pixels on  that axis.  Only the ratio of PIXWID

and PIXHGT  is significant,  so any  desired units  may  be  used.

PIXWID and  PIXHGT should be made as accurate as possible; changes

in the  second decimal  place in the ratio make visible changes on

the screen.   PIXWID  and PIXHGT  must be  greater than 0 and less

than 32767.   The values HIDLINES and HIDCOLS (normally 25 and 80,

respectively) give  the number  of lines and characters to be used

for the  "hidden" screen.   If the driver does not require AutoCAD

to maintain  a hidden  screen, both  of  these  values  should  be

returned as 0.

INIT4       Code (AX): 4

Result                     Res1 (BX)

Res2 (CX)

The INIT4  function requests  further information from the driver.

Res1 and  Res2 are  the upper and lower halves (respectively) of a

32-bit flag  field, i.e.  [(Res1)(Res2)].   Each bit,  when set ON

(1), specifies the presence in the driver of a handler for various

extended functions.  The currently available functions are:

Code        Function       Use

EF_SYNC     SYNC           Flush local display buffers

EF_REDRAW   REDRAW         Redraw from local display buffers

(or) clear local display buffers

EF_CMD      DSCOMAND       Register to receive commands

EF_XPCMD    DSXPCOMD       Register to receive transparent commands

INIT5       Code (AX): 5

Result                     Res1 (BX)  =   ymenumax

Res2 (CX)  =   iydotsm

The INIT5  call requests  further  information  from  the  driver.

YMENUMAX specifies  the number  of pixels  assigned to  the screen

menu area.   This  normally equals the value returned by the INIT1

7

ADI Graphics Display Version 2.1 - Chapter 2

function for  IYDOTS.   If the  screen menu has a height different

from the graphics portion of the screen, YMENUMAX should be set to

whatever that  height is.   IYDOTSM  is the  maximum vertical  dot

number of  the screen in pixels, including the text scrolling area

and the status line (total dots in y direction = IYDOTSM + 1).

After the  INIT5 call  is  processed,  the  screen  should  be  in

graphics mode,  cleared, any  borders between logical screen areas

drawn, and ready to accept graphics mode commands.

TERM        Code (AX): 9

The TERM call indicates that AutoCAD is leaving the drawing editor

and that  the driver should perform any cleanup desired.  A single

screen display will always be in text mode when this call is made.

CLEAR       Code (AX): 10

Result                     Res1 (BX)  =   repaint

The graphics  portion of  the screen should be cleared.  The menu,

scrolling area,  and status line should not be disturbed.  If this

is not possible, set REPAINT as the sum of: 1 to redraw the status

line, 2  to redraw  the menu,  and 4  to redraw the text scrolling

area.   Otherwise, repaint  should be  0.   If  the  driver  draws

borders on  the screen  and they  are destroyed  by  clearing  the

screen, CLEAR should replace them.

DOT         Code (AX): 11  Arg1 (BX)  =   x

Arg2 (CX)  =   y

The driver  should XOR  the dot  at the location given by X and Y.

If a  multi-plane color  display, the  dot should  be XORed in all

planes.   This function  is currently used for the GRID command in

AutoCAD and may be tested by using that command.

MOVE        Code (AX): 12  Arg1 (BX)  =   from_x

Arg2 (CX)  =   from_y

Arg3 (DX)  =   (0,drawmodes)

The driver  should save from_x, from_y, and drawmodes for use when

drawing the next vector or filled polygon.  Each MOVE call will be

immediately followed  by a  DRAW or  FLOOD call.  If the following

call is  a DRAW,  from_x and  from_y are  the start  point of  the

vector to be drawn.  If the following call is a FLOOD, from_x will

be the  number of  vertices in the polygon, and from_y will be -2.

For either  DRAW or  FLOOD, drawmodes  is a  bit-coded field  that

specifies certain drawing modes.  Currently defined bits are:

8

ADI Graphics Display Version 2.1 - Chapter 2

Name             Value   Meaning

DR_HILITE         1      The vector/area should be highlighted.

DR_NORDRW         2      The vector  is not  part of  a drawing

entity and  should not be displayed on

REDRAW calls.

The DR_HILITE bit should be saved as the "highlighting status" for

the following DRAW or FLOOD call.  The DR_NORDRW bit marks vectors

that are  not part  of any  drawing entity  (for example,  the two

lines forming  a marker  blip), but  are simply  screen artifacts.

This is  provided for drivers that use the REDRAW entry, and marks

vectors that should NOT be redrawn on the next REDRAW request.

DRAW        Code (AX): 13  Arg1 (BX)  =   to_x

Arg2 (CX)  =   to_y

Arg3 (DX)  =   color

The driver  should draw  a vector  from the  saved FROM_X,  FROM_Y

point to  TO_X, TO_Y.   If COLOR is positive, that color should be

used (see  AutoCAD Standard Color Assignments).  If COLOR is zero,

the vector should be erased (preserving the background color if it

is something other than black).  If COLOR is -1, the vector should

be XORed  with the  background in all image planes.  If the driver

implements  highlighting,  COLOR  is  positive,  and  the  current

highlighting status  is 1 (see the MOVE command above), the vector

should be  drawn highlighted  (preferably dashed).    Otherwise  a

solid vector  should  be  drawn.    MOVE  will  always  be  called

immediately before DRAW.2

FLOOD       Code (AX): 14  Arg1 (BX)  =   x

Arg2 (CX)  =   y

Arg3 (DX)  =   color

The driver  should handle  the X,  Y information  depending on the

type of  hardware fill  it implements.   If the HWFILL result from

the INIT2  function call  was 0,  the FLOOD function will never be

called.   If HWFILL  was 2,  this function will be called once per

vertex of  the polygon  to be   filled, preceded and followed by a

special call  with Y  equal to  -2 and  X equal  to the  number of

vertices in  the polygon.   If  COLOR is positive, the filled area

should be that color (see AutoCAD Standard Color Assignments).  If

COLOR  is   zero,  the  area  should  be  erased  (preserving  the

background color  if it  is something  other than  black).  If the

____________________

2It is  not unusual  for  some  graphics  controllers  to  draw  a

different set  of pixels  depending on the direction the vector is

drawn.   If garbage is left on the screen after erasing a polyline

or solid, you might try sorting the coordinates before drawing the

vector.

9

ADI Graphics Display Version 2.1 - Chapter 2

driver implements highlighting, COLOR is positive, and the current

highlighting status  is 1 (see the MOVE command above), the filled

area should be highlighted (usually by highlighting every 5th line

of the  fill).   MOVE will always be called immediately before the

first FLOOD call for a given polygon.

MARK        Code (AX): 15  Arg1 (BX)  =   cur_x

Arg2 (CX)  =   cur_y

Arg3 (DX)  =   cursel

Result      Code (AX): can_do

The device  should draw a cursor of type CURSEL.  CURSEL values of

0 and  3 specify simple full screen orthogonal crosshairs, and the

display driver  may choose to draw these with optimized code.  The

cursor should be drawn at screen coordinates CUR_X, CUR_Y.  If the

device does not implement a cursor, or cannot a draw a cursor of a

certain type,  it should  do nothing  and  return  a  code  of  0.

AutoCAD will  then draw  the cursor  itself.  If the driver itself

draws the  cursor, a  code of  1 should  be returned.   The driver

should return a code of 0 for all values of CURSEL other than 0 or

3.

CMARK       Code (AX): 16  Arg1 (BX)  =   cur_x

Arg2 (CX)  =   cur_y

Arg3 (DX)  =   cursel

Result      Code (AX): can_do

The device  should clear  a cursor  of type  CURSEL (see above) at

screen coordinates CUR_X, CUR_Y.  If the device does not implement

a cursor, or cannot clear a cursor of a certain type, it should do

nothing and  return a  code of  0.   AutoCAD will  then clear  the

cursor itself.   If the driver itself clears the cursor, a code of

1 should  be returned.   The  driver should return a code of 0 for

all values of CURSEL other than 0 or 3.

CHAR        Code (AX): 20  Arg1 (BX)  =   char

The character  specified by  CHAR should  be drawn on the graphics

screen at the current text position.  (See commands below that set

the current  text position.)   The current text position should be

moved one character to the right.

ECHAR       Code (AX): 21

This function  indicates the  end of  a text  string sent  by CHAR

functions.

MHLITE      Code (AX): 22  Arg1 (BX)  =   nchars

Result                     Res1 (BX)  =   send_chars

The driver  should highlight the menu box specified by the current

text position.   NCHARS  is the  number of  characters in the menu

10

ADI Graphics Display Version 2.1 - Chapter 2

box.   If the  driver can  highlight the  box without  knowing the

characters within  it, it  should return  SEND_CHARS equal  to  0.

Otherwise, it should return SEND_CHARS equal to 1 and AutoCAD will

send the  contents of  the box  to the driver with NCHARS calls on

the CHAR function, followed by an ECHAR function.

MDHLITE     Code (AX): 23  Arg1 (BX)  =   nchars

Result                     Res1 (BX)  =   send_chars

The driver  should de-highlight  the menu  box  specified  by  the

current text  position.  NCHARS is the number of characters in the

menu box.   If the driver can de-highlight the box without knowing

the characters  within it, it should return SEND_CHARS equal to 0.

Otherwise, it should return SEND_CHARS equal to 1 and AutoCAD will

send the  contents of  the box  to the driver with NCHARS calls on

the CHAR function, followed by an ECHAR function.

MNUCUR      Code (AX): 24  Arg1 (BX)  =   boxno

Arg2 (CX)  =   nboxes

The current text cursor should be set to BOXNO on the screen (0 is

the top  box).  There are NBOXES on the screen.  This function may

be followed by CHAR functions to write into the box, or MHLITE and

MDHLITE functions to highlight or de-highlight the box.

TPROMPT     Code (AX): 30

The current  text cursor  should be  set to the prompt line on the

screen.  This function will be followed by CHAR functions to write

into the  prompt line.   This  function will  be used only on dual

screen displays (4 bit clear in MODES result of INIT2).

QPLOT       Code (AX): 31

Result      Code (AX): can_do

The driver should perform a screen dump to printer.  If the driver

cannot do this, it should do nothing and return CAN_DO equal to 0.

If the  driver did  do the  screen dump, CAN_DO should be returned

equal to 1.

RPEN        Code (AX): 32

Result                     Res1 (BX)  =   status

Res2 (CX)  =   pen_x

Res3 (DX)  =   pen_y

The driver should sense the position of the screen pointing device

(e.g. light  pen) and  return its  current position  in PEN_X  and

PEN_Y in terms of pixels on the graphics screen.  STATUS should be

set to  2 for  a tracking  coordinate and  3 for  a pick.   If the

driver supports  a "function  box" picks  from it  may be returned

with STATUS  equal to  4 and  PEN_X set  to the button number (the

first button  is number  0).  Function box picks will select items

11

ADI Graphics Display Version 2.1 - Chapter 2

from the AUX1 menu.  If there is no screen pointing device, or the

device is not responding, STATUS should be returned as 0.

GOTEXT      Code (AX): 33

The driver  should switch  to the  text screen, and clear it.  The

cursor on the text screen should be left at the top of the screen.

Subsequent to this call, AutoCAD will use regular DOS I/O to write

to the  text screen.  This call will occur only on a single screen

system (bit 4 set in MODE result from INIT2 function).

GOTEXTU     Code (AX): 34

The driver  should switch  to the  text screen,  displaying hidden

screen information.   If  the  driver  maintains  its  own  hidden

screen, it  should switch  to the  text screen  and see  that  the

hidden information appears with the cursor on the text screen left

after the  hidden information.   If  the driver  asked AutoCAD  to

maintain the hidden screen (by setting HIDLINES and HIDCOLS in the

result to the INIT3 function), it should simply switch to the text

screen, clear  it, and  leave the cursor at the top.  AutoCAD will

automatically rewrite the hidden screen.  Subsequent to this call,

AutoCAD will  use regular  DOS I/O  to write  to the  text screen.

This call  will occur only on a single screen system (bit 4 set in

MODE result from INIT2 function).

GOGRAPH     Code (AX): 35

Result                     Res1 (BX)  =   repaint

The driver  should switch  to the  graphics screen.  Ideally, this

operation should  be non  destructive, leaving  the picture,  text

scrolling area,  status line, and screen menu exactly as they were

when the  screen was  switched to  the text screen.  If the driver

can switch  to the  graphics screen  non-destructively, it  should

return REPAINT equal to 0.  If it cannot, it should return REPAINT

as the  sum of the following codes that instruct AutoCAD to redraw

various information  on the screen: 1 to redraw the status line, 2

to redraw  the menu, 4 to redraw the text scrolling area, and 8 to

redraw the  picture in  the graphics area.  Note that if the 8 bit

is set,  causing AutoCAD to redraw the picture, the redraw process

will call  the CLEAR function.  If the CLEAR function in turn uses

REPAINT to restore the status, menu, or text scrolling area, there

is no  need to  request restoration  of these items in the REPAINT

result returned  by the GOGRAPH function.  If the driver has drawn

borders or  other information  on the screen not known to AutoCAD,

and this information was destroyed by switching to the text screen

and back, it is the responsibility of the driver to redraw it on a

GOGRAPH function.   This  function will only be called on a single

screen system (bit 4 set in MODE result from INIT2 function).

12

ADI Graphics Display Version 2.1 - Chapter 2

WRSPLIT     Code (AX): 36  Arg1 (BX)  =   char

The driver  should write the character CHAR to the graphics screen

at the current text position.  If the driver implements the hidden

screen itself,  the character  should also  be saved on the hidden

screen.   If the  driver is  letting AutoCAD  maintain the  hidden

screen (by setting HIDLINES and HIDCOLS in the result of the INIT3

function), the driver need only draw the character on the graphics

screen; AutoCAD  will take  care of  saving it.   The current text

position should be moved one character to the right.

MODELINE    Code (AX): 37  Arg1 (BX)  =   length

Arg2 (CX)  =   color

The driver  should set  the current  text position to the start of

the mode  status line.   LENGTH specifies the number of characters

that will  be sent  via the CHAR function following this function.

An ECHAR  function will terminate the string.  The current drawing

color (color  new entities  will appear  in, however  derived)  is

passed in  COLOR so  that the device can, if it chooses, represent

this on the mode status line.

COORDLINE   Code (AX): 38  Arg1 (BX)  =   length

The driver  should set  the current  text position to the start of

the coordinate  status line  (usually  one  more  than  MODELINL).

LENGTH specifies  the number  of characters  that will be sent via

the CHAR function following this function.  An ECHAR function will

terminate the string.

SYNC        Code (AX): 50

Informs drivers  with local display buffers to force those buffers

to be  drawn,  bringing  the  picture  into  sync  with  AutoCAD's

internal state.   For example, this can be called before digitizer

samples are  taken to  ensure that  the entire  picture is  on the

screen for the user to pick from.

REDRAW      Code (AX): 51  Arg1 (BX)  =   clear flag

Result      Code (AX):     0 - Unable to redraw complete picture

1 - Complete picture redrawn

Arg1 = 0 - Perform a REDRAW

When a  redraw of the picture is necessary, AutoCAD will call this

entry with  an argument  of 0  to get  it to  attempt  to  do  the

redrawing of  vectors.   AutoCAD will  still call CLEAR before and

will draw  grid dots,  etc. afterwards.   If  this returns  a ZERO

value, AutoCAD  will assume that the driver has declined to do the

job, and will redraw the image vector by vector as usual.

13

ADI Graphics Display Version 2.1 - Chapter 2

Arg1 = 1 - Flush display buffer

When the  current contents  of any  display buffer  are no  longer

valid, AutoCAD  will call  this entry  with argument 1 to indicate

that the  vector space  should be  cleared.   No return  status is

necessary.

DSCOMAND    Code (AX): 52  Arg1 (BX)  =   string length

Arg2 (CX)  =   string OFFSET

Arg3 (DX)  =   string SEGMENT

Result      Code (AX):     0 - Command not recognized

1 - Command recognized and processed

The driver  has (in  the INIT4  function)  requested  that  it  be

allowed to  examine all  input commands that are not recognized by

AutoCAD.     The  command   string  location   is  passed   as   a

SEGMENT/OFFSET pair  in DX/CX,  with the string length in BX.  The

command string  is terminated  by a  zero  byte  in  the  usual  C

fashion.  If the driver returns 0 in AX, AutoCAD will proceed with

its unknown  command handling;  if the driver returns a 1, AutoCAD

will assume  that the  string has been recognized and processed by

the driver,  and will  behave as  if it  were  any  valid  AutoCAD

command (e.g. a null line will repeat the command).

DSXPCOMD    Code (AX) 53   Arg1 (BX): command character

Result      Code (AX)      0 - Command not recognized

1 - Command recognized and processed

The driver  has (in  the INIT4  function)  requested  that  it  be

notified  when   a  special  transparent  display  driver  command

character is entered.  Currently, there is only one such character

- Ctrl-L  (0C hexadecimal).   The character is supplied in BX.  If

the driver  returns 0 in AX, AutoCAD will proceed with its unknown

character handling; if the driver returns a 1, AutoCAD will assume

that the  character has  been  recognized  and  processed  by  the

driver.   The transparent  control character may be entered AT ANY

TIME by the user, and will be immediately passed to the driver.

14

ADI Graphics Display Version 2.1 - Chapter 2

2.6 AutoCAD Standard Colors

Whenever a  color value is passed from AutoCAD to the ADI Graphics

Display driver, the following mapping should be used:

0     Background (erase)

1     red

2     yellow

3     green

4     cyan

5     blue

6     magenta

7     white

8-15     usually intensified version of above

(for RGBI type devices)

For displays  that use  256 colors,  the  hue  resulting  from  an

AutoCAD color  10-249 will  be determined by its first two digits,

and the saturation and value from the last digit, as follows:

Hues:

10     Red

50     Yellow

90     Green

130     Cyan

170     Blue

210     Magenta

Between each  of these  are three intermediate hues, e.g., between

red and yellow, we have:

20     reddish orange

30     orange

40     yellowish orange

To each  hue number,  0, 2,  4, 6,  or 8  can be  added to  give a

different "value",  or brightness,  with 0 the brightest and 8 the

weakest.   Finally, 1  can be added to produce a "half-saturated",

or pastel, color.  For example, color 18 is the dimmest red and 10

the brightest  red, 19  is the  dimmest pink  and 11 the brightest

pink.  Colors 8 and 9 are up for grabs; typically various greys or

sometimes black.

When AutoCAD  asks the  driver to draw in a color that matches the

background color, then white should be used (unless the background

color is white, in which case black should be used).

Included on  the ADI  diskette is  the file CHROMA.DWG, an AutoCAD

drawing that  shows all  colors 1-255.  You should use this sample

drawing for debugging your driver.

15

ADI Graphics Display Version 2.1 - Chapter 2

2.7 ADI Graphics Display Test Procedures

Included in  the ADI Development Kit is a set of files for testing

graphics display  drivers.   These files  use  the  AutoLISP  LISP

interpreter built into AutoCAD to automate the testing of graphics

display drivers.   Type  (load"DSTEST") at  the  AutoCAD  COMMAND:

prompt, or  rename DSTEST.LSP  to ACAD.LSP  to  have  the  program

loaded automatically  when AutoCAD  is started.   Type "single" or

"dual" at the AutoCAD prompt to test those configurations.  If you

wish, you  may also  wish to  perform the  manual test  procedures

outlined below.

Manual test procedures follow:

1) Set up  the machine  to be  tested with  the display  board, an

appropriate monitor, and a digitizer.

2) Copy the  AutoCAD release  into a  test directory.    Configure

AutoCAD for the board, monitor, and digitizer you are using.

3) Start a new drawing and perform the following tests.

a) Draw a  circle.   DRAG the  placement of  the circle  on the

screen.   Garbage from  circles drawn by dragging should not

appear.   ERASE the  circle.  For ADE3, the circle should be

highlighted.   Toggle the  screen from graphics to text, and

back to  graphics, with the flipscreen key.  Garbage (or the

circle)  should   NOT  appear  and  the  border  should  not

disappear.

b) Draw a box around the edges of the graphics area, overlaying

the borders.   (This  is most easily done by starting a line

at a  corner and  then continuing the "to points" at each of

the remaining  corners.)  ERASE the box.  The borders of the

graphics area should not disappear.

c) Configure the  display with  MENU on and COMMAND PROMPT AREA

off.   Scroll through the menu with the pointing device from

top to  bottom and  back to  the top.  The Menu items should

not scroll  (move).   Scroll through the menu from bottom to

top and  back to  the bottom.   Again, the menu items should

not scroll.

d) Configure the  display with  COMMAND PROMPT  AREA on.  Enter

the drawing  editor using  a large  drawing (e.g.   CHROMA).

Point to  the last  item in  the menu area with the pointing

device.   Type in REGEN.  Enter CONTROL C while the regen is

in progress.   After  the REGEN  is canceled,  enter several

more commands.  The command text area should not scroll into

the drawing area.

16

ADI Graphics Display Version 2.1 - Chapter 2

e) For  COLOR  display  drivers,  check  that  the  colors  are

correct.  Also, if colors for graphics background, menu text

and prompt  text are  configurable,  check  that  the  color

assignments really work.

f) For COLOR display drivers, turn AXIS on, draw something with

any color  other than  that used  for the  axis,  erase  the

object you  drew and  flip screen  from graphics to text and

back again.  The axis should NOT be drawn with the color you

drew the object in.  Also, the border should NOT disappear.

g) Draw a  wide polyline.   MOVE  the polyline.  Garbage should

not appear  or be  left behind  on the  screen.   ERASE  the

polyline.   There should  be no  garbage left on the screen.

Perform the same steps with a solid.3

4) The next  group of  procedures involve  configuring the  STATUS

LINE, COMMAND PROMPT AREA, and MENU AREA in all possible on/off

combinations and performing tests with each configuration.  Use

the drawing  DSTEST.DWG in the remainder of the tests.  Use the

accompanying chart  to track your progress.

a) Go to  the AutoCAD  configuration  menu  and  configure  the

STATUS LINE, COMMAND PROMPT AREA, and MENU AREA according to

the appropriate step in the chart.

b) Enter the drawing editor using DSTEST.DWG.

c) Draw a  box around  the limits  of the  display  (as  in  3b

above).   Make sure  that  the  edges  of  the  display  are

accessible.

d) With the  STATUS LINE  configured off,  there should  be  no

STATUS LINE  at the  top of  the screen.  Conversely, if the

STATUS LINE   was  configured on,  it should  be  there  and

should contain  the correct  information on layer, modes and

coordinates.

e) With the COMMAND PROMPT AREA configured off, there should be

no command  prompt at  the bottom  of the  screen.   If  the

COMMAND PROMPT AREA  was configured on, there should be text

lines usable at the bottom of the screen (3 lines for single

screen, 1  line for  dual screen)  for entering  commands or

data.

f) With the  MENU AREA  configured off,  there should  not be a

MENU AREA  on the  right  hand  side  of  the  screen.    If

configured on,  the complete menu area should be accessible.

Scroll through  the MENU  AREA from  top to  bottom and from

____________________

3See footnote to DRAW function above.

17

ADI Graphics Display Version 2.1 - Chapter 2

bottom to top.  You should be able to access from the top of

the menu  to the  bottom.   Make sure  that picking an entry

works.

g) Flip screen  from graphics  to text  and back.   None of the

lines drawn around the display should disappear.

h) Draw a  unit circle  in the  middle of  the  screen.    Zoom

extents.   The border  should not clip the circle at the top

of the  screen.   Erase the  circle.  The borders at the top

and bottom  edges of  the screen  should not be erased.  Now

zoom out  and draw  another unit  circle to the right of the

first one  such that  the distance  between the  centers  is

greater than  the diameter.   Zoom  extents.     The  border

should not  clip the  circles at  the sides  of the  screen.

Erase both  circles.   This should  not erase the borders at

the sides of the screen.

i) Go to  step 4a and repeat the above procedures with the next

configuration.

18

ADI Graphics Display Version 2.1 - Chapter 2

ADI GRAPHICS DISPLAY

TEST CHART

STATUS    PROMPT     MENU        PASS/FAIL

LINE      AREA      AREA

___________________________________________________________

Config 1    OFF       OFF       OFF           ______

Config 2    OFF       OFF        ON           ______

Config 3    OFF        ON       OFF           ______

Config 4    OFF        ON        ON           ______

Config 5     ON       OFF       OFF           ______

Config 6     ON       OFF        ON           ______

Config 7     ON        ON       OFF           ______

Config 8     ON        ON        ON           ______

19

ADI Graphics Display Version 2.1 - Chapter 2

20

__________________________________________________________________

Chapter 3

THE ADI DIGITIZER SPECIFICATION

__________________________________________________________________

The following  describes, in  detail, the  interface specification

for ADI digitizer drivers.

3.1 The ADI Digitizer

The ADI  Driver Development Kit contains the information needed to

create an  ADI digitizer  driver for  use with  Autodesk  software

products such  as AutoCAD  and AutoSketch.  A demonstration driver

for the  Microsoft Mouse,  DGDEMO.C, is  included.    This  driver

requires IWAIT.ASM,  ADI.H (functions  common to all ADI drivers),

and DGADI.H (functions for ADI digitizer drivers).

3.2 Configuration

ADI  Digitizer   drivers  may   be  used  with  both  AutoCAD  and

AutoSketch.

When the  AutoCAD user specifies the ADI Digitizer driver from the

AutoCAD Configuration  Menu, the only configuration question asked

is:

Hexadecimal interrupt code (INT 0XXh) <79>:

The user  should respond  with the interrupt location at which the

digitizer driver  is installed.   You  need to  inform the user in

your documentation  which interrupt  to use.   The  default on new

installations is 79h.    AutoCAD  may  be  configured  to   use  a

different interrupt vector.  AutoSketch assumes the  ADI digitizer

is installed at interrupt 79 and may not be changed.

No other  configuration questions are required.  The developer may

choose to  include any  desired configuration  questions when that

driver is installed, or may supply them in AUTOEXEC.BAT, etc.

3.3 The ADI Digitizer Interface

The generic  driver code simply makes calls to an installed driver

and passes its responses back to the program.  The driver is not a

true MS-DOS device driver (although it may be implemented as one),

but rather  a program that "terminates and remains resident" after

patching a  call to  itself into  an interrupt vector by which the

program calls it.

21

ADI Digitizer Version 2.1 - Chapter 3

Information is  passed back  and forth between the program and the

driver through  the registers AX, BX, CX, and DX.  The driver code

must preserve all segment registers and the registers SI and DI.

Commands to driver         Code (AX)

Initialize                 1        BX = Interface version (1)

Terminate                  2

Probe                      3

When the  drawing editor is initialized, the driver will be called

with AX  equal to  1 (initialize).   BX contains a code indicating

the revision  number of  the driver  interface  (to  allow  upward

compatibility).  The current version of the driver passes 1 in BX.

The installed  driver should  perform whatever  initialization  it

wishes and  return with  a status  in AX.   If  the initialization

failed,  the  driver  should  return  0  in  AX,  otherwise  1  if

everything went  well.    If  1  is  returned  in  AX,  indicating

successful initialization,  BX  is  expected  to  contain  a  code

indicating the  type of  the pointing  device.   If BX  is 0,  the

device is  configured as  a relative  pointing device  such  as  a

mouse; tablet menus and tablet mode will be disabled.  If BX is 1,

the device  is configured  as  a  tablet  and  all  tablet-related

commands will  be turned  on.   In addition,  CX  is  expected  to

contain a  code indicating  whether the  pointer sends  continuous

samples or pauses between samples.  If the pointer always responds

with a  sample, or  if pauses  are meaningful  (for example,  they

indicate the  cursor has been removed from the tablet surface), CX

should be  set to 0.  If the device occasionally returns no sample

because of  its inherent  properties (for  example, a device which

only reports  changes in  coordinates), CX  should be  set  to  1.

Setting CX incorrectly can cause the crosshairs to blink.

If the driver returns zero, the message:

Could not initialize pointer driver.

will be issued and the device disabled.  Before calling the driver

to initialize  it, the program driver tests whether the configured

interrupt vector  is nonzero.  If it is zero, indicating no driver

is installed,  and hence  that trying  to call  it would crash the

machine, the message:

** No driver is installed for interrupt vector 0XX **

will be issued and the driver disabled.

As the  drawing editor  runs, whenever  the program asks for input

from the  digitizer, the installed driver will be called with 3 in

AX (Probe).   The  driver should respond with one of the following

status codes in AX, with additional arguments as indicated.

22

ADI Digitizer Version 2.1 - Chapter 3

Response         AX        BX        CX        DX

Nothing          0

Tracking point   2         X-coord   Y-coord

Picked point     3         X-coord   Y-coord

Button pick      4         Button #

Button + coords  5         Button #  X-coord   Y-coord

AX =  0 indicates the pointer had nothing to report.  No action is

taken by  the program.  This code may be returned when waiting for

a button  to be  released following  a selection.   (Note that the

program  driver   does  not   perform  this  function--it  is  the

responsibility of  the installed  driver, as  is any debouncing of

buttons which may be required).

AX =  2 indicates  a "tracking"  coordinate pair is returned in BX

and CX.   The X coordinate, in the range from 0 to 20480 inclusive

is returned  in BX,  and the Y coordinate in the same range in CX.

This is  how coordinates  are returned when the pick button is not

pressed.  The program will track with the screen or menu cursor.

AX =  3 indicates  a picked  coordinate pair is returned in BX and

CX.   The X  coordinate, in the range from 0 to 20480 inclusive is

returned in  BX, and  the Y  coordinate in  the same  range in CX.

This code is used to return the coordinates when the "pick" button

is pressed.

AX =  4 indicates  a button  has been  pressed, which  generates a

button menu  pick.   The button number is returned in BX, with the

first button indicated by 0, the second by 1, and so on.

AX =  5 indicates  a button  has been  pressed, which  generates a

button menu  pick, and  that the driver can return the coordinates

of the  pointer at the time of the button pick.  The button number

is returned  in BX, the X coordinate (0 to 20480) in CX, and the Y

coordinate in  DX.   Devices  which  return  the  current  pointer

position with  button menu picks should call function 5 and return

the coordinates.   Devices  which lack  this capability should use

function 4 described above.

If any  value other  than those given above is returned in AX, the

message:

** Pointer driver returned illegal code X.  Pointer disabled **

will be issued and the pointer disabled.

When the  drawing editor  returns to  the main menu, the installed

driver will  be called with 2 in AX.  This instructs the driver to

terminate.

23

ADI Digitizer Version 2.1 - Chapter 3

Note:

The point  coordinates returned by the digitizer driver should lie

in the range 0 to 20480 with (0,0) at the lower left corner of the

tablet and  (20480,20480) at  the  upper  right.    Even  relative

pointing devices,  such as  mice, must return absolute coordinates

in the  range 0  to 20480.   If  a digitizer  tablet has differing

height and  width, the  full range of the SMALLER dimension should

be scaled  to 0  to 20480, with the larger dimension's coordinates

using the  same scale.   This  scheme will  result in  the  larger

dimension's coordinates  exceeding 20480,  though they must remain

less than 32768.  If Tablet Mode is to function properly, vertical

and horizontal digitizer coordinates must represent equal physical

distances.

24

ADI Digitizer Version 2.1 - Chapter 3

3.4 ADI Digitizer Test Procedures

These test  procedures are for use with AutoCAD.  If you intend to

use your  ADI digitizer  driver with  AutoSketch, we  suggest that

create a  simple drawing with AutoSketch and edit it using all the

options in  the Change  menu.  Be sure that you are able to access

the entire drawing area and that picking works correctly.

1) Set-up machine  for the  digitizer you  are testing.  Cable the

digitizer  as   documented  in  the  AutoCAD  Drafting  Package

Installation/Performance Guide.

2) Configure AutoCAD for the digitizer.

3) Start a  new drawing.   Cross  hairs should  appear  on  screen

immediately after  bringing up  a the  drawing.  Check that the

pointing device  moves cross-hairs  smoothly across  the screen

and that  the cross-hairs  can be moved over the entire drawing

screen.   Moving the  pointing device  from the  bottom of  the

digitizer pointing  area to the top should move the cross-hairs

from the  bottom to the top of the screen.  Moving the pointing

device from  the extreme left of the digitizer pointing area to

the right  should move  the cross-hairs  from the  left of  the

screen to  the right  (into the  menu area  if the menu area is

configured on).

4) Check that  the "button" options for each supported puck, mouse

or stylus work as documented in the Installation Guide.

The following  table gives  the function  representation of the

Menu item buttons in the standard AutoCAD menu:

Menu item button                Function

------------------------------------------------------------

1 = ; (Semicolon)               <Return>

2 = Ctrl C;Redraw               Redraw drawing in screen area

3 = Ctrl C                      *Cancel*

4 = Ctrl B                      Snap mode on/off

5 = Ctrl O                      Ortho mode on/off

6 = Ctrl G                      Grid on/off

7 = Ctrl D                      Coordinate Display

8 = Ctrl E                      Isometric Plane

9 = Ctrl T                      Tablet mode on/off

10-16                           Not set

Note:   On the  12 and 16-button cursors, the "*" button can be

used to  select menu  items, but  only if  the tablet's  Cursor

Function Disable sequence "**0" has been executed.  See page 4-

6 of the Calcomp 9000 Series Digitizer Operator's Manual 50090-

1.

25

ADI Digitizer Version 2.1 - Chapter 3

Note:   The black  button on  the back of the Kraft Joystick or

the right  button on  the pad may be used to select slow motion

mode, or to invoke the first item from the button menu.

The following  procedure steps  5 and  6 test  commands with modes

that operate outside the "normal" operating environment of AutoCAD

commands and  are specific  to the digitizer and therefore singled

out for  testing.  For definitions of these commands, consult your

AutoCAD Drafting Package Reference Manual.

5) Test the  pointing device  in SKETCH mode.  The normal "button"

menu cannot be used while a SKETCH command is in progress.

26

ADI Digitizer Version 2.1 - Chapter 3

The following  is a list of SKETCH subcommands, and the pointer

buttons which correspond to them:

Command Character  Pointer Buttons        Function

--------------------------------------------------------------

P                       "Pick"            Raise/Lower pen

. (period)                1               Line to point

R                         2               Record lines

X,Space,<Return>          3               Record lines and exit

Q,Ctrl C                  4               Discard lines and exit

E                         5               Erase

C                         6               Connect

6) If testing a Digitizer Tablet:

a) Configure the tablet for four Menu areas (Maximum supported)

with the  standard TABLET  TEST SHEET  and check to see that

all menus  are accessible with pointing device by calling up

the digitizer test menu "DGTEST".

a) Calibrate tablet  to area  with known  dimensions and  check

various  points   (Tablet  mode  on)  for  accuracy.    When

calibrating the  Tablet, choose  two points that require the

command to  perform coordinate  calculations more  difficult

than those  required by picking the origin and another point

on either the X or Y axis.

27

ADI Digitizer Version 2.1 - Chapter 3

28

__________________________________________________________________

Chapter 4

THE ADI PLOTTER SPECIFICATION

__________________________________________________________________

The following  describes, in  detail, the  interface specification

for ADI Plotter drivers.

4.1 The ADI Plotter Kit

The ADI  Development Kit  allows users  to  interface  AutoCAD  to

plotters and  other vector-oriented devices that Autodesk does not

directly support.   The  driver is fully configurable and offers a

variety of different output formats.

The output  from the  AutoCAD ADI  driver may  be sent to an ASCII

file,  a  binary  file,  or  configured  to  communicate  with  an

installed INT driver.

4.2 Configuration

Because the  driver is  indeed generic, the process of configuring

it is more complex than that required by most other plotters.  All

the hardware information provided by a conventional driver must be

supplied by  the user  when configuring this driver.  As a result,

the dialogue  is extended.   First,  the driver's output format is

selected.  The following prompt is issued:

Select output format:

0. ASCII file

1. Binary file

2. AutoCAD DXB file

3. Installed INT driver

Output format, 0 to 3 <0>:

The ASCII  and binary  file formats  will be described later.  The

DXB file  format is  described in  the  AutoCAD  Drafting  Package

Reference Manual.   The generic driver can also send commands to a

resident driver  via an  INT call.  If this option is selected via

option 3, the additional question:

Hexadecimal interrupt code (INT 0XXh) <78>:

will be  issued.   The response selects the interrupt vector to be

used to  communicate with the plotter driver.  The default is 78h.

Any vector  from 1 to 0FFh is permissible.  If an invalid reply is

entered, the message:

29

ADI Plotter Version 2.1 - Chapter 4

** Interrupt number must be between 1 and 0FF

is given and the prompt reissued.

Following the  output format dialog, the user is asked whether the

plotter is multi-pen:

Does the plotter have multiple pens? <N>

If this is answered yes, the user is asked:

How many pens does it have, 2 to 127 <2>:

Next, information  on plotter-implemented line styles is requested

with the questions:

Does the plotter have multiple dashed line styles? <N>, and,

How many line styles are there, 2 to 127 <5>:

If the  plotter provides  multiple pen speeds, the user may define

them with the questions:

Is the plotter variable speed? <N>, and,

What is the numeric code for the fastest speed, 1 to 127 <10>

These questions  supply information  which is normally provided by

the hardware  definition of a plotter within a driver.  It is used

to enable  features in  the plot  setup  process  and  to  provide

rudimentary range checking of user specifications.

Next the  maximum plot  size and  plotter step size are specified.

These arguments  may be specified either in inches or millimeters.

The units are selected by the prompt:

Next you  will specify  the maximum  plot size  the plotter can

make, and the step size of  the plotter.  You may specify these

either in millimeters or inches.  The next question selects the

units.

Specify plot size in millimeters? <N>

If the  answer is  the default,  "no," the prompts that follow and

their responses  will refer to inches, otherwise millimeters.  The

examples given below assume inches were selected.

30

ADI Plotter Version 2.1 - Chapter 4

Maximum horizontal (X) plot size in inches? <11.0>

Plotter  steps  per  inch  in  the  horizontal  (X)  direction:

<1000.0>

Maximum vertical (Y) plot size in inches? <8.5>

Plotter steps per inch in the vertical (Y) direction: <1000.0>

These parameters  are floating  point numbers  that  are  used  to

compute the  plotter size  in plotter  steps.  The calculated step

count may  not exceed  65535.   If the specifications given exceed

the limit,  the following message will appear (after the steps per

inch are specified):

** Error:  Plot size multiplied by steps per inch cannot exceed

65535.  Specifications resulted in ?????.

The question  marks in  the prompt  are replaced by the step count

the user  tried to  configure.  After this error message, the pair

of prompts for plot size and steps per unit are reissued.

Once the  driver-specific configuration  is completed, the regular

plotter configuration  is performed.   The  generic plotter driver

remembers all  its configuration  options  and  supplies  them  as

defaults on  subsequent configurations.  The defaults given in the

prompts above  are those  assumed on  a new  configuration of  the

driver.

4.3 Using the Driver

If ASCII,  binary, or  DXB output  format has  been selected,  the

program will prompt for a file name following all the runtime plot

configuration dialogue:

Enter file name for plot:

The user  should reply  with a  file name  for the plot data.  The

name may  be entered  with a  file type,  if desired.   If none is

specified, ".plt"  (or ".dxb"  in the case of output format option

2) will be used.  If the file cannot be opened, the message:

Cannot open this file.  Try another name? <N>

will appear.  If answered affirmatively, the file name prompt will

reappear; otherwise the plot will be canceled.

4.4 Output Format

Regardless of the output format and method, plot data is expressed

in a  single representation.  Each plot command consists of a type

code, possibly  followed by  arguments.  The following table gives

the plot codes and their arguments:

31

ADI Plotter Version 2.1 - Chapter 4

Function       Code      Type      Argument 1    Argument 2

Begin plot     1         B         File level

End plot       2         A

Move           3         C         X coordinate  Y coordinate

Draw           4         C         X coordinate  Y coordinate

New pen        5         B         Pen number

Select speed   6         B         Speed code

Set line type  7         B         Line type

Pen change     8         A

Abort plot     9         A

The "type"  field in the above table specifies how the information

is represented  in the  file or  passed to  the  driver.    Before

discussing the  representation, let  us examine  the various  plot

commands and the meaning of their arguments.

The first code generated for any plot is the BEGIN PLOT code.  Its

argument is  a number  which specifies  the version  of  the  plot

command file  which follows.   Plots  generated by this version of

the driver will have a file level of 1.

The PEN  CHANGE code  is generated  before every pen change at the

start of  a pass  over the  vectors.   It allows  the  plotter  to

prepare for a possible manual pen change (for example, raising the

pen and  moving it  to the middle of the carriage).  It is not the

actual change pen command, which is the NEW PEN code.  Note that a

PEN CHANGE  command will  always be  generated at  the start  of a

plot.

The NEW  PEN code  is the actual command that selects a pen.  This

code is  followed by  an argument  which specifies the pen number.

See the Plotting chapter of the AutoCAD Drafting Package Reference

Manual for  information on  how AutoCAD  handles the assignment of

plotter pens to drawing colors.

The SELECT SPEED code chooses a drawing speed for a variable speed

plotter.   The code  is followed  by an  argument that selects the

speed.  Note that the speed argument is range checked and will not

exceed the maximum speed configured, but is not otherwise checked.

For a  plotter with  discrete speeds,  a  driver  processing  this

command which  encounters a speed number the plotter cannot handle

should send the next SLOWER speed.

The SET  LINE TYPE code selects a plotter line font.  This code is

followed by  an argument  which selects  the line  font.   It is a

universal convention within AutoCAD that zero selects a continuous

(solid line).  The meaning of the other fonts is up to the driver.

The MOVE  code  is  followed  by  two  arguments,  specifying  the

absolute coordinates to which the pen should be moved with the pen

32

ADI Plotter Version 2.1 - Chapter 4

up.   The coordinates  are specified  X first,  Y second,  both as

unsigned numbers in the inclusive range from 0 to 65535.

The DRAW  code  is  followed  by  two  arguments,  specifying  the

absolute coordinates to which the pen should be moved with the pen

down.   The coordinates  are specified  X first, Y second, both as

unsigned numbers in the inclusive range from 0 to 65535.

The END  PLOT code  is output  at the  end of  a plot.   It has no

arguments.

The ABORT PLOT is relevant only for installed drivers.  It is sent

to indicate  that the  plot has  been terminated  by the  user  by

pressing the  interrupt key.  If the plot is being sent to a file,

this action causes the file not to be generated, so the ABORT PLOT

code will  never be  seen by a program processing a plot file.  An

installed driver  will receive  the code to indicate that abnormal

termination  action  should  be  taken  (flushing  a  buffer,  for

example).   This code  will be followed by an END PLOT code, which

should perform  the usual  end of  plot functions (such as putting

the pen back).

4.4.1     File Format

If the  output format  is an  ASCII file,  each plot  command will

appear on  a new  line, starting  in the  first text  column.  All

codes and  arguments  appear  as  decimal  numbers,  separated  by

commas.   Type A codes appear by themselves.  Type B codes consist

of the  command code  followed by  the argument.    Type  C  codes

consist of  the command  code, followed  by the X coordinate and Y

coordinate, comma delimited.  All numbers appearing in a plot file

are unsigned.   Arguments  to type B codes are in the range from 0

to 127,  and arguments to type C codes range from 0 to 65535.  The

following is  an actual  plot file generated by plotting a square.

The annotations  did not  appear in  the original  file, but  were

added to explain the commands.

1,1                        Begin plot, file level 1

8                          Pen change (start of pass)

5,1                        Select new pen 1

7,0                        Select line type 0 (solid)

6,1                        Select speed 1

3,0,6953                   Move to 0, 6953 - pen up

4,6954,6953                Draw to 6954, 6953 - top of square

4,6954,0                   Draw to 6954, 0 - right side

4,0,0                      Draw to 0,0 - bottom

4,0,6953                   Draw to 0, 6953 - left side

2                          End of plot

In a binary file, the codes and arguments appear precisely as they

do in  an ASCII  file, but  are stored  as binary numbers.  Type A

codes are  written as  single bytes.   Type  B codes  consist of a

33

ADI Plotter Version 2.1 - Chapter 4

single byte  code, followed  by a one byte argument.  Type C codes

are written  as a  single  byte  code,  followed  by  two  16  bit

arguments representing  the X  and Y  coordinates.   Each  16  bit

argument is written least significant byte first, most significant

byte last,  the standard  8086 representation.  The following is a

hexadecimal dump  of a  binary file  generated by the same drawing

which created the ASCII file above.  Spacing has been added to the

dump to aid readability.

01 01

08

05 01

07 00

06 01

03 00 00 29 1B

04 2A 1B 29 1B

04 2A 1B 00 00

04 00 00 00 00

04 00 00 29 1B

02

4.5 Installed Driver Operation

The generic  plotter driver  can  communicate  with  an  installed

driver.   This allows  the development  of a  driver  which,  once

loaded in  memory, acts  exactly like  a driver implemented within

AutoCAD itself:   plots  are performed  in real  time, without the

need for  an intermediate file.  Installed drivers are implemented

as MS-DOS  programs that "terminate and remain resident"; they are

not true  DOS device drivers (although an alternate approach is to

use a DOS device driver and send the file output to it).

The installed  driver should  patch itself into a vacant interrupt

location  then   exit  to   DOS,  remaining  resident  in  memory.

Installation of  the driver must be accomplished before AutoCAD is

executed.

When the  generic driver  is configured,  the installed  driver is

selected by  the "3"  reply to  the  output  format  query.    The

interrupt vector  at which  the driver  installed itself  must  be

specified at configuration time.

When a  plot is made, the generic plotter driver will first make a

rudimentary check to see if the driver is installed:  it will test

the interrupt  vector and  only proceed  if it is nonzero.  If the

interrupt vector is zero, the message:

** No driver is installed for interrupt vector 0XX **

will be  issued, specifying  the interrupt  vector in hexadecimal,

and the plot will be canceled.

34

ADI Plotter Version 2.1 - Chapter 4

If the  driver is  properly installed,  each plot  command will be

sent to  it  by  loading  the  command  code  and  arguments  into

registers and  then calling  the driver with an INT instruction to

the configured  vector.   The command code is always passed in AX.

For type B commands, the first argument is passed in BX.  For type

C commands,  the X coordinate is passed in BX and the Y coordinate

in CX.

The driver  should return  with AX  zero.   (This is not currently

checked, but  may be used in the future for a driver status).  The

driver must  preserve the  contents of  all segment registers, and

the registers SI and DI.

35

ADI Plotter Version 2.1 - Chapter 4

4.6 ADI Plotter Test Procedures

1) Configure AutoCAD  for the  plotter to  be tested.   Cable  the

plotter  as   documented  in   the  AutoCAD   Drafting  Package

Installation/Performance Guide.

2) Use the PLTST2.DWG to test:

a) Scaling -Plot  the view  named "SCALE"  to a specified scale

(e.g. 1=1).   Measure the  dimensioned parts  of the drawing

checking that they are plotted to the scale specified.

b) 90 Degree  Rotation -  Plot the drawing specifying 90 degree

rotation, MAX  size paper,  FIT.  The plot should be rotated

90 degrees.  The plot should not be truncated; if it is, the

paper size is incorrect.

c) Linetypes, Pen  changes, and  Pen speeds - Change the layers

in the drawing to use the linetypes supported on the plotter

(as documented in the Installation Guide).  Also, change the

layers to  different pen numbers and pen speeds supported on

the plotter.   Plot  the drawing.   Check that the different

line types  were plotted.   Check that the correct pens were

used.   If the  plotter only  supports one  pen, time should

have been allowed between plot layers to change the pen.  If

multiple pen  speeds are  allowed, the  different pen speeds

should have been used during the plot.

d) Origin Offset  - Plot the drawing, specifying an offset from

the origin.   The  plot should  move the specified number of

units from the origin.

e) MAX Size  - Plot  EXTENTS, MAX  size, FIT, no rotation.  The

drawing should not be truncated.

36

___________________________________________________________________

Chapter 5

THE ADI PRINTER/PLOTTER SPECIFICATION

__________________________________________________________________

The following  describes, in  detail, the  interface specification

for ADI Printer/Plotter drivers.

5.1 The ADI Printer/Plotter

The  ADI   Development  Kit  allows  manufacturers  and  users  to

interface AutoCAD  to printers  that Autodesk  does  not  directly

support.   The sample  included in  the  ADI  Kit  demonstrates  a

printer/plotter that simply uses asterisks and spaces for output.

5.2 Configuration

Printer/plotters come  in a  variety of resolutions and sizes, and

some support  color output.   AutoCAD  must know  these parameters

before it  can begin  a plot.   These  parameters are  set  during

configuration of the ADI printer/plotter.

First the  maximum paper size and resolution are specified.  These

arguments may  be specified  either in inches or millimeters.  The

units are selected by the prompt:

Next you  will specify  the maximum  plot size the printer  can

make, and  its resolution  (dots per  unit).   You may  specify

these either  in millimeters  or inches.   The   next  question

selects the units.

Specify plot size in millimeters? <N>

If the  answer is  the default, "no," the prompts which follow and

their responses  will refer to inches, otherwise millimeters.  The

examples given below assume inches were selected.

Maximum horizontal (X) plot size in inches? <11.0>

Printer dots per inch in the horizontal (X) direction: <100.0>

Maximum vertical (Y) plot size in inches? <8.5>

Plotter dots per inch in the vertical (Y) direction: <100.0>

These parameters  are floating  point numbers  which are  used  to

compute the  printer X  and Y  size in dots.  The calculated sizes

may not  exceed 65535.   If  the specifications  given exceed  the

limit, the  following message will appear (after the dots per unit

are specified):

37

ADI Printer/Plotter Version 2.1 - Chapter 5

** Error:  Plot size  multiplied by dots per inch cannot exceed

65535.  Specifications resulted in ?????.

The question marks in the prompt are replaced by the size the user

tried to configure.  After this error message, the pair of prompts

for plot size and dots per unit are reissued.

Finally, the  driver's output  format is  selected.  The following

prompt is issued:

Select output format:

0. Binary file

1. CAD/camera image file

2. Installed INT driver

Output format, 0 to 2 <0>:

The binary  file format  will be  described later.  The CAD/camera

image file  format is  described in  the AutoCAD  Drafting Package

Installation/Performance Guide  under ADI  Printer/Plotter.    The

generic driver  can also send commands to a resident driver via an

INT call.  If this option is selected via option 2, the additional

question:

Hexadecimal interrupt code (INT 0XXh) <7B>:

will be  issued.   The response selects the interrupt vector to be

used to  communicate with the printer/plotter driver.  The default

is 7Bh.   Any vector from 1 to 0FFh is permissible.  If an invalid

reply is entered, the message:

** Interrupt number must be between 1 and 0FF

is given and the prompt reissued.

If binary  file format  is selected, the user must specify whether

color information  is to  be written  to the  file  (an  installed

driver provides  this information  to  AutoCAD  at  initialization

time, so  this question  does not appear if an installed driver is

selected).  The prompt is:

Does the printer/plotter support color? <N>

If answered  with the default of "No", the output file will be one

bit per pixel.  If color is selected, the output will be four bits

per pixel,  and will  contain color  information.  See the AutoCAD

Drafting Package  Installation/Performance Guide  for examples  of

how various printers handle color information.

Once the  driver-specific configuration  is completed, the regular

printer/plotter  configuration   is  performed.      The   generic

38

ADI Printer/Plotter Version 2.1 - Chapter 5

printer/plotter driver remembers all its configuration options and

supplies them  as defaults  on  subsequent  configurations.    The

defaults given  in the  prompts above  are those  assumed on a new

configuration of the driver.

5.3 Using the Driver

If binary  file output  format has  been selected,  when a plot is

made with  the driver,  following all  the  runtime  printer  plot

configuration dialogue, the prompt:

Enter file name for plot:

will appear.   The  user should  reply with  the file in which the

printer plot data should be written.  The name may be entered with

a file  type, if  desired.   If none  is specified, ".PRP" will be

used (or  "IMG" for  CAD/camera image  file output).   If the file

cannot be opened, the message:

Cannot open this file.  Try another name? <N>

will appear.  If answered affirmatively, the file name prompt will

reappear; otherwise the plot will be canceled.

5.4 Output Format

Regardless of whether the output is being written to a binary file

or passed  directly to  an installed  driver, printer plot data is

represented in  a uniform  fashion.   Each record  in the  file or

message to the installed driver begins with a single 16 bit value.

If the  sign bit  is set  in this value, it is a control function,

otherwise it  is a  count specifying  the number  of bytes of plot

data that follow.  The function codes are given below.

Function    Code                Arguments

Begin plot  8001h          File_level, Xdots, Ydots, Color

End plot    8002h

Abort plot  8003h

A binary  file will  begin with  the BEGIN  PLOT code  (all 16 bit

values in  the file  are written  in standard 8086 fashion:  least

significant byte  first, most  significant byte last), followed by

the 16 bit values FILE LEVEL, XDOTS, YDOTS, and COLOR.  FILE LEVEL

identifies the  format of  the file  for upward compatibility with

future extensions.   Files  written by  this version of the driver

will have  a file  level of  1.    XDOTS  and  YDOTS  specify  the

horizontal and  vertical size,  respectively, of  the printer  for

which this  file was  generated.   COLOR will  be 0 if the file is

monochrome format  (one bit  per pixel) and 1 if the file is color

format (4 bits per pixel).

39

ADI Printer/Plotter Version 2.1 - Chapter 5

Following the  BEGIN PLOT  record are  zero or  more line records.

Each line  record consists of a 16 bit byte count followed by that

number of  bytes of  raster data.    If  the  printer/plotter  was

configured as supporting color, each byte will contain two pixels,

with 4  bits per  pixel indicating  color.  If the printer/plotter

was configured  as monochrome, 8 one bit pixels will be stored per

byte.   In either  case, pixels  are arranged  on the page left to

right starting  with the  most significant  pixel  of  each  byte,

proceeding to  the least  significant, then continuing to the next

byte.   Records in  the file  are variable  length; trailing  zero

bytes are  not written.   A totally blank line on the printer will

be written  with a  byte count  of 0  and no following data bytes.

Totally blank  lines at the end of the plot are not written to the

file.   Consequently there  may be  fewer line records in the file

than the YDOTS count given in the BEGIN PLOT record.

The end  of the plot is indicated by an END PLOT record instead of

a byte  count word.  The ABORT PLOT item will never be encountered

in a printer plot file because aborting the plot causes the output

file to  be deleted.   It  is passed  to an  installed  driver  to

apprise it of the abnormal termination of the printer plot.

5.5 Installed Driver Operation

The  generic   printer/plotter  driver  can  communicate  with  an

installed driver.   This allows the development of a driver which,

once loaded  in memory,  acts exactly  like a  driver  implemented

within AutoCAD  itself:  printer plots are performed in real time,

without the  need for an intermediate file.  Installed drivers are

implemented  as   MS-DOS  programs   that  "terminate  and  remain

resident"; they  are not  true DOS  device drivers.  The installed

driver should  patch itself  into a vacant interrupt location then

exit to  DOS, remaining  resident in  memory.  Installation of the

driver must  be accomplished  before AutoCAD  is  executed.    The

standard interrupt  vector  for  printer/plotter  drivers  is  07B

hexadecimal.

When the  generic driver  is configured,  the installed  driver is

selected by  the "2"  reply to  the  output  format  query.    The

interrupt vector  at which  the driver  installed itself  must  be

specified at configuration time.

When a  printer plot  is made,  the generic printer/plotter driver

first makes a rudimentary check to see if the driver is installed:

it will  test the  interrupt vector  and only  proceed  if  it  is

nonzero.  If the interrupt vector is zero, the message:

** No printer driver is installed at interrupt vector 0XX **

will be  issued, specifying  the interrupt  vector in hexadecimal,

and the plot will be canceled.

40

ADI Printer/Plotter Version 2.1 - Chapter 5

Commands are passed to the installed driver through the registers.

The function  code or  length in  bytes of  the data  is passed in

register  AX.  The driver should examine AX and react accordingly.

The BEGIN  PLOT code  is passed  with the  FILE LEVEL  in BX.  The

configured size  of the printer/plotter in dots is passed with the

horizontal size  in CX  and the  vertical size  in DX.  The driver

must return with AX containing a status code: 1 if the printer was

successfully initialized  and 0  otherwise.   BX should  contain a

code which  selects monochrome  or  color  output  format,  0  for

monochrome (1bit per pixel), 1 for color (4 bits per pixel).

Successive lines  of plot data are passed with the length in bytes

of each line in AX.  BX contains the offset address of the line of

data,and CX  contains the  segment address  of  the  printer  data

(thus, the  full 32  bit address  of the  line is  in CX:BX).   DX

contains the  vertical position  of the  line on  the page  (since

blank lines  within the plot are output, most driver will not need

this value and may ignore it).  If the line is blank (AX is zero),

CX and BX are meaningless.

The end  of the  plot is signaled by the END PLOT code in AX.  The

driver should eject the page and close out the printer plot.

If the  printer plot  is aborted  by the  user,  the  driver  will

receive the ABORT PLOT code in AX.  If possible, the driver should

discard information  buffered for  the printer.   The  driver will

receive an ENDPLOT code after the ABORT PLOT code.

A sample  driver, intended  to illustrate  how an installed driver

operates, and to exercise the facility for testing, is provided on

the disk as PPDRV.C and PPDRV.EXE.

This driver  sends its  data to  the  printer  in  "ultimate  dumb

printer/plotter mode":   zero  bits are  printed as spaces and one

bits as asterisks.  To configure a 132 column printer for use with

this driver, answer the configuration questions:

No          <use inches>

13.2        <paper width>

10          <characters per inch horizontal>

10          <usable paper height>

6           <characters per inch vertical>

This driver  supports only  monochrome format (one bit per pixel).

The driver  will read  a binary  output file  and send  it to  the

printer when called with the command:

41

ADI Printer/Plotter Version 2.1 - Chapter 5

PPDRV <file>

The extension  of .PRP  is supplied automatically.  The driver may

be installed  into interrupt  vector  07Bh  by  calling  with  the

command:

PPDRV -I

Once installed,  printer/plotter output may be sent directly to it

by using the installed driver output option.

5.6 ADI Printer/Plotter Test Procedures

Use the test procedures outlined under ADI Plotter Test Procedures

to test your completed ADI Printer/Plotter driver.

42

__________________________________________________________________

Appendix A

REVISION HISTORY

__________________________________________________________________

This appendix describes changes made in the ADI specification from

previous versions.

A.1 ADI Display Interface Version 2.0 (October, 1986)

These additions  are supported in ADI display Vers. 2.0, available

in AutoCAD Version 2.52, November, 1986.

o  Added INIT4 Extended Functions ED_SYNC and EF_REDRAW.

o  FLOOD function fixed to include color specification.

o  MOVE calls added prior to FLOOD calls to pass highlight flag.

o  DSCOMAND and  DSXPCOMD added  to enable  passing of transparent

commands to the display driver.

o  AutoCAD ADI  display driver  corrected to  handle selection  of

interrupt vectors other than the default, 7Ah.

A.2 ADI Display Interface Version 2.1 (December, 1986)

These additions  are supported in ADI display Vers. 2.1, available

as a driver update to AutoCAD Version 2.52, December, 1986.

o  MOVE fixed  to recognize vectors that are not part of a drawing

entity (e.g.  blips) and  which should not be redrawn on REDRAW

calls.

o  MODLINE call  fixed to  pass  correct  current  entity  drawing

color.

o  Fixed machine  hang when  ADI driver was not found at specified

interrupt, or  when ADI  driver returned  0 (failure)  on INIT1

call.

43

--END OF ADIKIT.DOC--