Written by  Kurt Rauchenberger      73477,2260                     11/27/88

Acknowledgment:

Some of the Ideas for building the 3DFACEs and moving around the

PLINEs came from looking at the R-SURF.LSP function on the Bonus

Disk. I hope that the folks who wrote that function don't mind that

I borrowed one of their subfunctions (vernext). Thanks.

This function will allow you to create a 3D object in by defining the

cross sections (C-SECT) that make up the object.

Imagine taking a C.A.T. scan of a 3D object and connecting the outline

contours of each of the slices with 3DFACEs. These 3DFACEs would define

the external surface of the object.

For this function a cross section is a PLINE that defines the surface

contour at a specific Z-AXIS elevation. The PLINEs can be open or closed,

decurved or splined.

The only qualification is that ALL of the PLINEs (cross sections) MUST

have the same number of vertices. At first this would appear to be a

severly limiting factor but because the function can be used on different

groups of PLINEs it is not as bad as it first appears.

Some thoughts about using this function:

I think that the most effective way to develope 3D objects with

this function would be to create a master PLINE that has as many

vertices as you think are needed to define the most complex contour

of your object. Then make copies of this PLINE and use CHANGE to

adjust the elevation of each copy. Once the basic elevations and

PLINEs have been established use PEDIT to adjust the position of

each of the vertices on each of the PLINEs at each elevation. You

can also use PEDIT to SPLINE the PLINEs (remember that if you

"spline" one PLINE you have to "spline" the rest of the PLINEs in

that group so that the vertices count remains the same).

By working with different groups of PLINEs you should be able to

define some relatively complex objects.

I'm sure that the folks who have recieved RELEASE 10 will probably

be able to do this sort of thing in a more direct fashion but for

the rest of us who are still waiting this function may make working

in RELEASE 9's  2-1/2 Dimesions a little more exciting.

FILES INCLUDED IN C-SECT.ARC

C-SECT.LSP  AutoLisp source code

C-SECT.DOC  This file

BOAT.DWG    An example drawing