DOLISP.DOC

Re: DOLISP.LSP

Developed by: Perry H. Mason

Industrial TurnAround Corp.

922 E. Randolph Rd.

Hopewell, VA. 23960

(804) 541-8714

(804) 541-0751  (FAX)

DOLISP is an AutoCAD routine that creates AutoLISP routines from

entities

drawn by the CAD Operator.  DOLISP contains several function

programs within

it which are:

1.  READYGO - Checks to see if the current drawing contains an

entity or not.  If the drawing does not contain an entity

this sub-routine will place a line at drawing coordinates 0,0

to 1,0.  This entity or the last entity in the drawing will

then be used as the start point for locating all entities to

be used to create the macro.

2.  DISTANG - This sub-routine sets the distance and angle

variables used by MAKELISP when creating the entity points.

3.  CKCOLLT - Verifies the working entity color and linetype used

in the drawing and prepares variables to write lines to the

macro as required to handle the changes.

4.  WORKING - Displays to the CAD Operator the entity type while

MAKELISP is running.

5.  COLRLTYPE - Generates the macro commands to change the entity

color or linetype per the information obtained using the

CKCOLLT sub-routine.

6.  ORIGNPT - This sub-routine is used by the MAKELISP routine to

generate the next entity point command.

7.  CKNUMBER - A sub-routine to convert a whole number into the

integer and decimal parts.  Then changed into a string, which

can be written to the created macro.

8.  MAKELISP - The main program for creating an AutoLISP macro.

DOLISP looks for the following drawing entities: LINES, TEXT,

CIRCLES, ARCS, and SOLIDS.  If any of these entities are found in

the drawing database the entity is then processed.

DOLISP.DOC

Re: DOLISP.LSP

Developed by: Perry H. Mason

Industrial TurnAround Corp.

922 E. Randolph Rd.

Hopewell, VA. 23960

(804) 541-8714

(804) 541-0751  (FAX)

About layers, color and linetypes:

The routine checks for the entities layer.  If the entity is on a

different layer than the current layer this is written to the

created macro.  If thelayer contains color and linetype information

this is ignored by MAKELISP.  Although if the entity has been given

a color or an linetype then this is written to the created macro.

Other Information:

Because solids are recognized by MAKELISP, this means that

dimensioning can also been done in the original drawing.  There is

a draw back about dimensioning though and that is the quote marks

(").  If quote marks are placed in the string for inches the quote

marks are removed from the text string.  If a quote mark is found

at the end of a dimension text string then the CAD Operator is

informed of this during the run time of MAKELISP and informed about

how to fix this when the created macro is run later.

Also quote marks can not be placed in any portion of a string of

text.

MAKELISP:

The CAD Operator is prompted for the name of the created macro that

is to be generated.  If the name is longer than eight (8)

characters then the CAD Operator is prompted to try again.  The

file extension is added by this routine and can not be done by the

CAD Operator.  If an existing file name is found in the current

directory the CAD Operator is informed of this and given the option

to enter a new file name or over write the existing file with the

new file.