AFONT1.LSP TM, AFONT2.LSP TM, AutoFont TM

Copyright 1990 RobLand Corporation

Licensed Material, All Rights Reserved

The Dr.Lisp Utilities TM SM, Version 3.0

Copyright 1990 RobLand Corporation

Licensed Material, All Rights Reserved

--------------------------------------------------------------

W A R N I N G

THIS SOFTWARE AND MANUAL ARE BOTH PROTECTED BY U.S. COPYRIGHT

LAW (TITLE 17 UNITED STATES CODE). UNAUTHORIZED REPRODUCTION

AND/OR SALES MAY RESULT IN IMPRISONMENT OF UP TO ONE YEAR AND

FINES OF UP TO $10,000 (17 USC 506). COPYRIGHT INFRINGERS MAY

ALSO BE SUBJECT TO CIVIL LIABILITY.

--------------------------------------------------------------

AutoFont

The AutoFont font generator allows you to create an AutoCAD

shape file with a file extension of Shp. A shape file may

contain definitions for either text characters or shapes. The

shape file can then be compiled into an Shx file which may be

used in AutoCAD's style command if it contains text, or

AutoCAD's load command if it contains shapes.

AutoFont maximizes the stroke paths of the character to help

keep the pen from jumping back and forth unnecessarily during

plotting. If you are creating a filled letter, you can keep the

size of the shape description smaller by using arcs to fill a

letter rather than a bunch of small lines, making the characters

regen and plot much more quickly.

Prior to using AutoFont, you may have to make some adjustments to

your LispHeap and/or LispStack environment settings in order to

create room for AutoLISP programs and variables, etc.  AutoLISP

programs will not work unless there is enough room in memory.

Autodesk has heavily documented this procedure in AutoCAD's

support materials.  You should set the environment variable

LispStack to 5000.  If you are using extended AutoLISP, that is,

if you have extra ram loaded into your machine, you have defined

the environment variable AcadXMem correctly, and you have enabled

extended memory in Configure AutoCAD from AutoCAD's main menu,

you do not need to set an environment variable for LispHeap.  If

you are not using extended AutoLISP or you are not using AutoCAD

386, you must set the environment variable LispHeap.  A

suggestion is LispHeap=40000.  If you have made changes to any of

these settings while you are shelled out of AutoCAD, you must

exit and reenter AutoCAD before the new settings will take

effect.  Consult the AutoCAD documentation or your dealer for

more information on how to accomplish this.

If you are currently using another AutoLISP application such as

AutoCAD AEC(R), and you are not using extended AutoLISP, that

application may use up all or most of the LispHeap ram, and The

AutoFont will not install or run.  In this case, it is a

requirement that you install more ram in your machine and

configure AutoCAD to use extended AutoLISP.  Consult the AutoCAD

documentation or your dealer for more information on how to

accomplish this.

You must be first be sure that you have enabled AutoLISP in

Configure AutoCAD from AutoCAD's main menu.  If you are unsure,

from AutoCAD's main menu select 5.  Configure AutoCAD.  Then when

the current configuration screen appears, press the enter key.

Select 8.  Configure operating parameters.  Then select 7.

AutoLISP feature.  At the prompt

Do you want AutoLISP enabled?

Type Y and press the enter key.

Before installing AutoFont, you must be sure that you have an

ACAD.PGP file with an Sh definition.  Your Sh ram allocation

should be at least large enough to allow you to perform the

following procedure:

At the AutoCAD Command: prompt, type SH and press the enter key

TWICE.  If you see the DOS prompt, the Sh definition in ACAD.PGP

is set correctly, and you should type exit and press the enter

key.  If you do not see the DOS prompt, you must edit the file

Acad.Pgp using any word processing software in non-document mode

to change the amount of ram allocated for Sh.  If you have made

changes to the Sh definition while you are shelled out of

AutoCAD, you must exit and reenter the drawing before the new

setting will take effect.  Read the AutoCAD documentation or

consult with your dealer for instructions on how to accomplish

this.

Sometimes even though the Sh definition is set correctly, you

still may not be able to get to the DOS command prompt, and you

may get an error message reading

Shell error in (Exec) function: Insufficient memory

or some other insufficient memory error.  If this happens, you

will not be able to use AutoFont until the problem has been

corrected.  Read the AutoCAD documentation or consult with your

dealer for instructions on how to accomplish this.

There are several utilities on the AutoFont pull-down menu which

aid in drafting of the characters. The AutoFont menu is supplied

along with the files which you received in the AutoFont package.

The utility PEdit and Join can only be used after choosing one of

the Pre Processing menu  utilities. When these utilities are used

in conjunction with a character "template", characters can be

relatively easily entered into the system. The template can be a

clear, plastic photocopy of characters blown up on a photocopy

machine and taped to the screen, or you can use a product such as

Cad Overlay(R) which will allow you to scan a page of characters,

convert the scan file into a raster image, and place that image

onscreen in the drawing AutoFont to draw over. When you use Cad

Overlay, you  can pan and zoom as required and the raster image

pans and zooms along with the drawing. Another option is to

insert a block or some text on some new layer that you create,

and draw the character over it using the block or text as a

guide. Remember to erase the block or text before creating

another character.

You can only use AutoFont in the drawing AutoFont, which is

installed on the DRL directory. At the AutoCAD main menu, choose

option 2,

2. Edit an EXISTING drawing

and at the prompt

Enter selection:

type DRL\AutoFont and press the enter key.

YOU SHOULD NOT SAVE OR END THE DRAWING AutoFont! To prevent

this from happening, AutoFont undefines the AutoCAD commands

save and end, but if you enter the drawing, don't use any

AutoFont utilities, and change the drawing in any way, you can

use the save command. DON'T DO THIS. If you wish to exit the

drawing without executing any of AutoFont's utilities, use the

AutoCAD command Quit.

If you are using some other application such as AutoCAD AEC(R)

which displays its own menu, use the AutoCAD menu command and

specify \DRL\AutoFont as the menu to use.

If you wish to use another menu while creating a character, use

the AutoCAD menu command and specify the menu.  When you wish to

use AutoFont, if you have already done some pre-processing of

characters, you may type AutoFont at the AutoCAD Command: prompt,

otherwise use the menu command and specify \DRL\AutoFont as the

menu.

The ---><--- menu selection displays the object snap screen menu,

and the -------- is a transparent redraw, meaning that it can be

performed while in the midst of executing an AutoCAD command.

The selections for the other Dr.Lisp Utilities will prompt

Unavailable in Sample Utility package.

Check out Headline on the Misc menu! Loads 'o laffs!

While you are using AutoFont, it will be MUCH better for you if

you leave the snap on. When AutoFont cleans up the image which

you want to use to define a character you are creating, one of

the things it does is to shift the objects' endpoints to the

nearest 1,1 snap points. This has very little effect on the look

of the image, but AutoFont must do it because it requires

integers as the X and Y coordinates of the endpoints. Since

AutoFont will do this regardless of whether you have the snap on

or not when you are drawing a character, you might as well draw

the character where the endpoints won't be shifted from. Don't

worry yourself needlessly when you are drawing a character if you

would prefer to correct misplaced endpoints subsequently to using

the Cleanup utility, as explained below in the documentation.

When creating an ascii character, there is a template for sizing

the characters.  When you specify that you wish to create an ascii

character, some displays may not completely show the template.

If you do not see a vertical line displayed, control-c.  Then you

should zoom A, and then zoom W back into the drawing to display

enough of the template to create the ascii character.  If you don't

know what this means yet, don't worry about it, just do the zooms

and you will get the idea.  If you must zoom to change the display,

use the AutoCAD view command to redefine the view Normal, as

described in the following paragraph.

You may change the display by using the AutoCAD commands Zoom

and Pan as required, as often as you wish. To get the original

display back at any time, you may use the AutoCAD command

View to restore the view named Normal. You may redefine the view

Normal if you want the view to show more on the screen, but each

time you go into the drawing AutoFont the view normal will again

be the original view definition. These commands are accessible

from the AutoFont pull-down menu.

Creating shape files with AutoFont is a two stage process. The

AutoFont menu consists of two screen menus, the Pre-Processing

menu and the Post-Processing menu, reflecting the two major steps

in the process of creating a font.

Pre-Processing allows you to create the graphics which comprise

each of the characters in a font. You should draw each character

using AutoCAD's line, arc, circle, insert and polyline commands

and edit these objects using any of AutoCAD's editing commands.

ALL OBJECTS SHOULD BE DRAWN ON LAYER CHARACTR!

AutoFont presents a structured method for using any character

previously created with AutoFont as a starting point for creating

a new character, explained later in the documentation.

Post-Processing allows you to walk away from the machine once

the graphics have already been created in the Pre-Processor, and

converts each of the graphics in the specified font into an entry

in a shape file. This shape file is an Ascii file and may be

viewed in any word processing software. Unless you know what

you are doing, DON'T EDIT THIS FILE.

FontTest lets you view each of the characters that can be in a

text font in conjunction with several other characters following

each one to verify that the spacing of other characters following

each character which you create is correct. FontTest presents

these "character couplets" in the font Txt, changes them to the

font which you specify, and allows you to zoom around the

drawing by specifying a character to view. Changing the font

takes some time, so do it only when there are lots of characters

which you want to view. If you use any pre-processing or post-

processing utilities other than Compile Font after using

FontTest, keep in mind that if you want to use FontTest again,

you will have to wait for the character couplets to change their

font again.

The prompts in this documentation are for creating shape files

to be used as text in an AutoCAD drawing, but with minor

differences the prompts for creating shape files for use with

the Shape command in AutoCAD are virtually the same. Since there

are no "next characters" in a shape for use with the shape

command, there will be no prompts for start point of next

character, etc. when creating such shapes. DO NOT USE NAMES FOR

SHAPES THAT ARE LONGER THAN 8 CHARACTERS!

Shapes created in AutoFont with a height of 100 units will

appear at a height of 1 when used in AutoCAD's shape command.

You may elect to create only a few characters in one editing

session, adding a few more at another time. When the Pre-

Processing menu appears in the new editing session, you may pick

up where you left off the last time you used AutoFont. If you

have since erased the directory which contained the font files,

create a directory with the name of the font beneath the DRL

directory and copy the backed up files to that directory. For

example, if you created a font named SanSerif two months ago and

want to add the @ character, and you have since erased the font

files from your hard disk, make a directory named

DRL\SanSerif and copy the files on your backup disk to that

directory.

If you want to redefine characters in a font for which you have

an .Shx file but no .Shp file, there is a public domain program

written by Schreiber Instruments named Shx2Shp.Exe which has been

included on the program diskette and which is on the DRL

directory, which will convert an Shx file to an Shp file. At the

DOS prompt, type

DRL\SHX2SHP [Directory Name]\[FontName]

and press the enter key, and Shx2Shp will create a file named

[FontName].Shp on the same directory where the file

[FontName].Shx resides.

The first thing you must do to create a character is to select

Define Character from the Pre-Processing menu. Define Character

tells AutoFont which character of which font that all subsequent

pre-processing operations will be performing their actions on.

If there are any objects drawn on the screen beside the guide

box, you are prompted

Do you wish to erase the objects drawn on the screen? Yes/<No>:

If you type N or No or press the enter key to accept the

default, the objects will remain on the screen for use in

creating the character. If you type Yes or Y and press the

enter key, the objects will be erased.

Whether or not you were prompted to erase objects, next you are

prompted

Is character an Ascii or Shape character? <Ascii>:

If you type an A, Ascii or press the enter key to accept the

default, you are prompted

Ascii character to create:

and you must type in a character. The character is taken

literally, i.e. if you wish to create an upper case A, type A,

and if you wish to create a lower case a, type a. Type in only

one character. If you type in more than one character, the

first character you type will be used. Press the enter key after

you have entered the character.

If you enter either an opening parenthesis ( or an exclamation

point ! you will be prompted

Can't reenter AutoLISP.

Ignore this prompt. AutoCAD thinks you have tried to enter a

lisp expression, but will still accept the character as the

character to create.

To verify that the character which you will be creating is the

correct one, you are then prompted

Ascii character to create is [character] No/<Yes>:

If it is the correct character, type Y or Yes or press the enter

key to accept the default. If you type an N or No and press the

enter key, you will be prompted again for an Ascii character to

create.

Once you have selected a character, AutoFont needs to know the

name of the font that the character will be a part of. At the

prompt

Name of font for Ascii character [character]:

type in a font name. The font may or may not already exist, and

if it does exist the character which you are now defining may or

may not already exist. If the font and the character already

exist, you will be overwriting the existing shape description for

that font. If the font does not exist, you will be adding the

character you are creating to the new shape file. You can

create any number of characters in a text shape file up to the

total character set, even only one character if you want.

To verify that the font which you will be creating the character

in is the correct one, you are then prompted

Name of font for Ascii character [character] is [FONTNAME]

No/<Yes>:

If it is the correct font, type Y or Yes or press the enter key

to accept the default. If you type an N or No and press the

enter key, you will be prompted again for the name of a font.

If the font does not exist, AutoFont will create a directory

by the fonts name just below the DRL directory.

Next you are asked

Adapt Ascii [character] from an existing Ascii character

or shape? Yes/<No>:

and you have the opportunity to use an already defined character

as a starting point for creating the character you will be

defining. You could choose a character which has a similar look,

such as a P to create the character R, or you could use an R from

a different font. If you answer yes to this prompt, you are

prompted

Character to adapt from <current character>:

If you press the enter key, you will use the definition of the

current character you are defining. Then you are prompted

Name of font for existing [character] <current font>:

If you press the enter key, you will use the definition of the

character which you specified at the last prompt from the current

font you are defining characters in, otherwise you will use the

definition from the font which you specify at this prompt.

If the definition of the character which you specify does not

exist, you will be prompted

C:\DRL\[FONTNAME]\[CHARACTER FILE NAME].DWG does not exist.

The character file name is the name of that character's drawing

file. If the character you are specifying is an upper case A,

the character file name would be UCA.

If the specified character exists, it is inserted exploded. You

may want to explode polylines rather than use the AutoCAD command

PEdit.

Next, if you have previously defined any characters for the font

name that you are now defining a character in, and you have not

been asked the question yet, you are prompted

Overwrite post-processing list for font [FONTNAME]? Yes/<No>:

READ THIS ENTIRE PARAGRAPH BEFORE TYPING A RESPONSE TO THIS

PROMPT. If you have already used the Post Process utility on a

large number of characters and just want to add a few more, and

you want to avoid waiting for AutoFont to re-post process these

characters, type Yes or Y and press the enter key. If you want

to include previously defined characters of this font in the

list of characters to post process because they have not been

post processed yet, TYPE NO OR N AND PRESS THE ENTER KEY, OR

SIMPLY PRESS THE ENTER KEY TO ACCEPT THE DEFAULT OF NO. The

file in question which contains this list, DRL\[FontName]\

PostProc.Lst, contains information about where the start point of

any character following this character in inserted text in a

drawing is located, as well as the vertical spacing information.

If you overwrite the post-processing list, this information will

disappear. If for some reason you only want to post process the

characters you are creating now, but you haven't post processed

the characters currently in PostProc.Lst, you can save this file

for later use by copying it to some other file name, and answer

Yes to the above prompt. When you post process the font, only

those characters which you have defined since the post processing

list was overwritten will be post processed. Then at some future

time should you decide to post process the characters which were

previously in the old PostProc.Lst (now named YourFile.Lst), you

can copy YourFile.Lst to PostProc.Lst and run the Post Process

utility. Regardless of what characters are in the file

PostProc.Lst, you will never lose any character definitions once

they have been post processed, and they will be included in the

shape file created by Compile Font regardless of how you answer

this prompt.

Finally the prompt

Definition of character complete. Ready to create Ascii

character [character] DRL\[CHARACTER NAME].

appears and if there are objects on the screen you may edit them

to create the image you want for the character, or draw objects

from scratch. All objects which have been placed onscreen by

using a previously created character are polylines and may be

edited with the AutoCAD command pedit.

You may use the commands on the AutoFont pull-down menu to aid

in drafting on editing the characters. The "box" which appears

if you are creating an Ascii character is the space envelope

which defines the height of a character. If you are creating

the lower case letter l and draw a line from point 0,0 to the

top of the box, when the lower case l is used in a drawing with

AutoCAD's Text command it will appear at exactly the height which

you specify in the text command. If you are creating a shape

which appears as a circle and you draw a circle with a center

point of 50,50 and a radius of 50, the total height of the shape

will appear in a drawing with a height of 1 if a height of one is

specified for that character during the use of AutoCAD's shape

command in a drawing.

Tangent Arc lets you draw lots of arcs in a row which continue

tangent from the last arc drawn. For the first arc drawn, you

pick the start point of the arc, the end point of the arc, and

the direction from the start point. Then for every arc drawn

after the first one, you select the endpoint of the next arc and

AutoCAD places an arc to that point which is tangent from the

last arc drawn. The utility continues 20 times or until you

control-c it. This utility is useful to draw an outline letter.

If you are having trouble keeping the arcs "on track", that is,

conforming to the shape of the letter's "template", you may want

to use Direction Arc instead. It is okay if the arcs are not

exactly, exactly, exactly tangent to one another if the

character looks correct. This will not be detectable unless you

are plotting the letter at a very large size.

Direction Arc lets you draw lots of arcs in a row using

AutoCAD's start point, end point, and direction method. Each arc

that is drawn after the first one assumes that the start point

for that arc is the endpoint of the last one. Then you pick the

direction of the arc. The utility continues 20 times or until

you control-c it. This is useful for creating filled letters

using arcs rather than a whole bunch of short straight lines

which cause the shape file to be larger and cause regens and

plots to take longer.

Point 0,0 can be used when prompted for a point to enter point

0,0 as a response. Point 0,0 is the starting point for all

characters you define. This means that when you use AutoCAD's

text command in a drawing and you include the character in some

text, the character will be placed as close to the ending point

of the previous letter in the text as you place the objects in

relation to point 0,0 as you are defining it in AutoFont.

Last Point can be used when prompted for a point to enter the

last point as a response.

Change Point is AutoCAD's Change command, written so that you can

change the endpoint of a line. It will not work on arcs or

polylines. Change Point follows the rules of AutoCAD's change

command based upon the current state of ortho. Change point

repeats until you control-c it.

Stretch Crossing is AutoCAD's Stretch command using a crossing

window. The command continues until you control-c it. If you

included point 0,0 in the crossing window, it will include the

guide box to be stretched. Obviously, you should not do this. In

this case, use the generic AutoCAD command stretch rather than

AutoFont's Stretch Crossing utility, and use the R option in

Select objects: to remove the guide box from the selection set.

Stretch Crossing repeats until you control-c it.

Each use of Snap Up changes the snap to the next higher snap

from the snaps 1,1 2,2 4,4 and 7,7. If the snap is at 14,14

when Snap Up is used, you are prompted

Current snap value 14,14 is maximum allowed in AutoFont.

Each use of Snap Down changes the snap to the next lower snap

from the snaps 2,2 4,4 7,7 and 14,14. If the snap is at 1,1

when Snap Down is used, you are prompted

Current snap value 1,1 is minimum allowed in AutoFont.

Explode is AutoCAD's Explode command. Explode repeats until you

control-c it.

Pan, Zoom Window, Zoom Previous, Zoom Dynamic, Zoom .75x and

Zoom 1.25x are transparent and may be used while in the midst of

executing another command or utility. Zoom .75x displays more

of the drawing than was showing onscreen each time it is used.

Zoom 1.25x displays less of the drawing than was showing

onscreen each time it is used.

Restore View Normal restores the view that was onscreen when you

entered the drawing AutoFont. If you are creating larger

characters than will fit onscreen in this view, you can manually

change the definition of the view named Normal using AutoCAD's

view command. This view will not be saved for use in another

editing session of the drawing AutoFont because you cannot save

the drawing. If you know what you are doing, it is possible to

save the drawing with a different definition of the view normal,

but I am not documenting this here to avoid causing you

difficulty if you are not an expert in AutoCAD.

The buttons in the AutoFont menu are currently defined as

follows: Button 1 = Pick, Button 2 = Enter, Button 3 = Object

snaps on the screen menu, and Button 4 = Cancel and Graphics

Screen. These may be changed if you wish by editing the menu

file DRL\AutoFont.Mnu with any word processor in non-document

mode. Be sure that your word processor does not add an end-of-

file marker to the file.

When you have configured the objects on the screen to appear as

the character which you are creating, you are ready to go on to

the next step, Cleanup. You MUST use Cleanup if you are creating

a new character. Cleanup checks through all of the objects you

currently have onscreen and changes them to a form in which

AutoFont needs them to create the shape file definition. You do

not have to use Cleanup if you are only changing the spacing of

an EXISTING character. If this is the case, go directly from

Define Character to Save Character. If you are creating a

character which is a mirror image of an EXISTING character, such

as / and \, assuming you have the snap on when you mirror the

objects, you don't have to do either Cleanup or Spacing, and can

go directly to Save Character. You also don't have to do either

Cleanup or Spacing if you are only using Stretch Crossing on an

EXISTING character.

You may use Cleanup from the Command: prompt at any time, as

long as you have first used Define Character. Anytime you select

Cleanup, the last character which you created with Define

Character will be used as the character to cleanup and space,

regardless of which objects are on the screen. If you select

Cleanup from the Pre-Processing menu before you have used Define

Character, you will be prompted

You must use Define Character before using Cleanup.

The prompt

Cleaning up Ascii character [character]...

appears and as AutoFont cleans up the image on the screen it

prompts you each step of the way with the procedure that is being

executed and a percentage complete. First, just in case you have

any unexploded blocks in the drawing on layer Charactr, AutoFont

prompts

Exploding blocks...

Exploded [X] blocks. [XX.XX]% complete.

AutoFont performs this procedure as many times as it takes to

verify that there are no blocks inserted on the Charactr layer.

Each time after the first pass you are prompted

Exploding blocks, Pass [pass number]...

Exploded [X] blocks. [XX.XX]% complete.

and when there are no blocks found, AutoFont continues on and

erases any points which you may have placed in the drawing which

are useless in creating the shape file for any character and

prompts

Erasing points...

Erased [X] points. [XX.XX]% complete.

Then AutoFont explodes all of the polylines on layer Charactr

and prompts

Exploding polylines...

Exploded [X] polyline. [XX.XX]% complete.

Because of rules in the creation of shape files, circles must be

broken up into two arcs, and you are prompted

Converting circles to arcs...

Converted [X] circles to arcs. [XX.XX]% complete.

Also because of rules in the creation of shape files, arcs with

angles of over Pi radians must be broken up into two arcs and

you are prompted

Breaking large angle arcs...

[X] large angle arcs broken. [XX.XX]% complete.

Because of other rules in the creation of shape files, arcs with

start and end points over 127 units apart must be broken up into

two arcs and you are prompted

Breaking up long arcs...

[X] long arcs broken. [XX.XX]% complete.

Other rules in the creation of shape files cause lines with

start and end points over 127 units apart to be broken up into

two lines and you are prompted

Breaking up long lines...

[X] long lines broken. [XX.XX]% complete.

After specifying the spacing, AutoFont prepares the character

for use later when it is post processed. First it converts all

of the objects, by this time only arcs and lines, to polylines,

and prompts

Converting arcs and lines to polylines...

Converted [X] objects to polylines. [XX.XX]% complete.

Then it shifts all endpoints (vertices) in each polyline to its

nearest 1,1 snap point and prompts

Changing polyline vertices to 1,1 snap points...

Changed [X] polylines. [XX.XX]% complete.

Finally, all of the polylines that comprise the character are

joined so that there is less jumping around of the pen during

plotting of the character, and also to make the shape file

smaller. You are prompted

Joining polylines...

Joined [X] polyline. [XX.XX]% complete.

When Cleanup is complete, you are prompted

Cleanup of Ascii character [character] DRL\[FONTNAME]\

[CHARACTER FILE NAME] complete.

Occasionally, because the cleanup process shifts endpoints of

objects, and in some cases creates new objects with new

endpoints, some endpoints in some objects may be shifted one

unit away from where you would like it to be. If this happens,

you must manually correct the vertices of the polylines with the

AutoCAD stretch command, available from the AutoFont pull-down

menu as Stretch Crossing. YOU MUST DO THIS WITH THE SNAP ON,

WITH THE SNAP SET TO INTEGERS, I.E. 1,1!! You do not have to

use Cleanup again after using Stretch Crossing with the snap on.

If you have already used Cleanup and then use Stretch Crossing,

continue on to Spacing.

Spacing allows you define the spacing between the character and

characters following the character in text when using AutoCAD's

text command in a drawing. You MUST use Spacing if you are

creating a new character. You do not have to use Spacing if you

are only using Cleanup on an EXISTING character. If this is the

case, go directly from Cleanup to Save Character. If you are

creating a character which is a mirror image of an EXISTING

character, such as / and \, assuming you have the snap on when

you mirror the objects, you don't have to do either Cleanup or

Spacing, and can go directly to Save Character. You also don't

have to do either Cleanup or Spacing if you are only using

Stretch Crossing on an EXISTING character.

If you are unsure of whether you should use the Spacing utility,

you can use ViewSpac to view the spacing guide lines, if any,

described below. Selecting ViewSpac from the Pre-Processing

menu turns on the layers on which the spacing guide lines are

drawn, and prompts

If there are spacing guide lines, they should be displayed

now.

Press any key or digitizer button, or pick point or menu

item to continue...

If there are no green spacing guide lines, you must use the

Spacing utility. After you press any key or digitizer button,

or pick a point or menu item, the layers with the spacing guide

lines are turned off.

You may use Spacing from the Command: prompt at any time, as

long as you have first used Define Character. Anytime you select

Spacing, the last character which you created with Define

Character will be used as the character to space, regardless of

which objects are on the screen. If you select Spacing from the

Pre-Processing menu before you have used Define Character, you

will be prompted

You must use Define Character before using Spacing.

When you select Spacing from the Pre-Processing menu you are

prompted

Pick horizontal start point of NEXT character:

and you must pick a point to the right of the character's image

where you want the next character to start when using this

character in a drawing during AutoCAD's text command. This

point does not have to be on the baseline of the character.

Only the X coordinate of the selected point is used. If you

previously specified that you wanted to use a character that had

already been defined as a starting point for the creation of

this character, a green guide line will appear which was the

horizontal start point of that character when it was created.

This guide line is only present as a guide, and is not included

in the shape file. It is useful if you are redefining a

character which had poor horizontal spacing to the next character

the last time it was created. If this guide line is present and

its position is acceptable as the starting point of the next

character, the above prompt will read

Pick horizontal start point of NEXT character <Current>:

If you type a C and press the enter key, or simply press the

enter key to accept the default, the position of the guide line

will be used as the starting point of the next character.

After you select the point, if there was a guide line present

for indicating the old horizontal start point for the next

character, it is erased. Then whether there was a guide line

present or not, a guide line is drawn from the baseline to the

top of the screen to indicate the horizontal start point of the

character.

Next you must indicate the vertical start point of this

character so that if you use the character in AutoCAD's text

command with vertical text on, the character will be placed in

the correct position in relation to the character before it.

When prompted

Pick vertical start point of THIS character:

you must pick a point above the character's image where you want

the character to start when using it in a drawing during

AutoCAD's text command. This point can be anywhere, usually

above the character. Only the Y coordinate of the selected point

is used. If you previously specified that you wanted to use a

character that had already been defined as a starting point for

the creation of this character, a green guide line will appear

which indicates the vertical start point and end points of that

character when it was created. This guide line is only present

as a guide, and is useful if you are redefining a character

which had poor vertical spacing from the last character the last

time it was created. If this guide line is present and the

point at the top of it is acceptable as the vertical start point

of this character, the above prompt will read

Pick vertical start point of THIS character <Current>:

If you type a C and press the enter key, or simply press the

enter key to accept the default, the top point of the guide line

will be used as the vertical start point of this character.

Then you are prompted

Pick vertical start point of NEXT character:

and you must indicate the vertical start point of the next

character so that if you use the character in AutoCAD's text

command with vertical text on, the next character after this one

in the text will be placed in the correct position in relation

to this character. This point can be anywhere, usually below

the character. Only the Y coordinate of the selected point is

used. If the guide line is present and the point at the bottom

of it is acceptable as the vertical start point of the next

character, the above prompt will read

Pick vertical start point of NEXT character <Current>:

If you type a C and press the enter key, or simply press the

enter key to accept the default, the bottom point of the guide

line will be used as the vertical start point of the next

character.

After you select the two vertical points, if there was a guide

line present for indicating the old vertical start and end

points, it is erased. Then whether there was a guide line present

or not, a guide line is drawn from the point you selected for the

vertical start point for this character to the vertical start

point of the next character.

When Spacing is complete, you are prompted

Spacing of Ascii character [character] DRL\[FONTNAME]\

[CHARACTER FILE NAME] complete.

To complete the pre-processing of a character use Save Image.

Save Image wblocks the objects which comprise a character and

the spacing lines to DRL\[FontName]\[CharacterName].Dwg, and

adds the character to the post-processing list.

You may use Save Image from the Command: prompt at any time, as

long as you have first used Define Character, Cleanup and

Spacing. Anytime you use Save Image, the last character which you

created with Define Character will be used as the character of

which the image is being saved, regardless of which objects are

on the screen. If you select Save Image from the Pre-Processing

menu before you have used Define Character and Cleanup &

Spacing, you will be prompted

You must use Define Character, Cleanup and Spacing

before using Save Image.

You are prompted

Processing Ascii character [character]...

and the drawn image is saved to a drawing file named

DRL\[FONTNAME]\[CHARACTER NAME].Dwg. If there was already a

file with that name, it is renamed DRL\[FONTNAME]\[CHARACTER

NAME].Bak. If there was already a file with the .Bak extension,

it is overwritten. The spacing information is saved to the file

DRL\ [FontName]\PostProc.Lst which is described below, and

you are prompted

Pre-processing of Ascii character [character]

DRL\[FONTNAME]\[CHARACTER FILE NAME] complete.

When you are done with defining characters, you are ready to

continue on to post-processing of the characters in the font.

Before you do this, if you were prompted earlier if you wanted

to overwrite the post- processing list and you answered no, if

there are duplicate entries for any character in the post-

processing list, you may elect to word process the file if there

are duplicate characters. You do not have to do this, but when

you post-process the list, duplicates will be post-processed the

number of times that there are duplicates, with the last entry

being the version of the character that is used in the .Shp

file.

The name of the file to edit is DRL\[FontName]\PostProc.Lst

and you must edit it in any non-document word processor in non-

document mode to remove the duplicate references to that

character. Remove the whole line of each of the duplicates in

question. The last entry for the character is the last one that

was defined. Be sure that your word processor does not add an

end-of-file marker to the file, or AutoFont will stop there when

it is using the file to post process the font, ignoring entries

that come after the end-of-file marker.

In order to avoid having to quit AutoCad, word process the file,

and reenter AutoCAD, you can do this by shelling out of AutoCAD.

Consult the AutoCad manual for instructions on how to configure

your Acad.Pgp file to allow you to shell out of AutoCAD directly

into your word processor. If you do not want to deal with

configuring Acad.Pgp, at the AutoCAD Command: prompt type Shell

and press the enter key, and then press the enter key again to

shell out of AutoCAD. Use your word processor, and when you are

done, at the DOS prompt type exit and press the enter key.

When you are ready to post-process a font, select Post Process

from the Pre-Processing menu and the Post-Processing menu will

appear.

In addition to the selection Pre Process, there are four

selections you can choose from on this menu. The reason that

AutoFont divides the process into discrete operations is that if

you have already performed Post Process for a font and have not

added any new characters, you do not have to perform the

operation again. If you have already performed Post Process and

Save Font on a font, you can perform Compile Font without

performing Post Process and Save Font.

If you have not performed any post processing operations on a

font yet, you can perform All PostProc and AutoFont will perform

Post Process, Save Font and Compile Font on the font. If you

have not specified a font name yet, you are prompted

Name of font to post process:

Type the name of the font that you wish to post process and

press the enter key, and the entire process will be performed.

You will not be prompted during Compile Font about whether you

would like to terminate this session.

To execute these utilities separately, the first step in post

processing a font is Post Process. In this step, every character

in the font, whether Post Process has been performed before on

any character or not, has a shape description created for it on

directory DRL\[FONTNAME]. If you have not specified a font name

yet, you are prompted

Name of font to post process:

Type the name of the font that you wish to post process and

press the enter key. The prompt

Post-processing font DRL\[FONTNAME]...

appears, and Post Process then walks through all of the

characters on the DRL\[FONTNAME] directory, inserts each drawing

into the drawing AutoFont, analyzes the polylines which comprise

the graphic representation of that character and creates a shape

description file named DRL\[FONTNAME]\[CHARACTER NUMBER].Fnt for

each one. If there was already a file with that name, it is

renamed DRL\[FONTNAME]\ [CHARACTER NUMBER].Old. If there was

already a file with the .Old extension, it is overwritten. After

a character is inserted into the drawing, the prompt

Post-processing Ascii character

DRL\[FONTNAME]\[CHARACTER NAME]...

appears and stars are printed along the screen to indicate that

activity is occurring. When all of the character have been post

processed, you are prompted

Post-processing of font DRL\[FONTNAME] complete.

You can now use Save Font to combine the files DRL\

[FONTNAME]\[CHARACTER NUMBER].Fnt into the shape file

DRL\[FONTNAME]. Shp. If you would like the shape

descriptions to appear in numerical order in the shape file, use

a directory sorting program such as The Norton Utilities(R)

Directory Sort with the sort by file name option on directory

DRL\[FONTNAME] before performing Save Font.

Finally, you can select Compile Font to complete the process.

Compile Font will quit the drawing, go to AutoCAD's main menu,

and run option 7, Compile shape/font description file.

If you have not specified a font name yet, you are prompted

Name of font to post process:

Type the name of the font that you wish to post process

and press the enter key. Then you are prompted

Do you wish to terminate this editing session and compile

font DRL\[FONTNAME]? Yes/<No>:

and you must tell AutoFont whether you would like to terminate

the editing session continue with Compile Font. If you compile

the font, the drawing AutoFont will be quit. To compile the

font, type Yes or Y and press the enter key to compile the font.

To stop execution of Compile Font, type No or N and press the

enter key, or simply press the enter key to accept the default.

You can only compile one font at a time with the utility Compile

Font. If you want to compile more than one font at a time, you

may do this manually from AutoCAD's main menu.

If you choose to compile the font, the prompt

Post-processing font DRL\[FONTNAME]...

appears, the drawing AutoFont is quit, and the font DRL\

[FONTNAME] is compiled from AutoCAD's main menu. If the compile

is successful, you are prompted

Compiling shape/font description file

Compilation successful. Output file DRL\[FONTNAME]\

[FONTNAME].shx contains [XXX] bytes.

Press RETURN to continue:

If the compile is unsuccessful, you are prompted

Compiling shape/font description file

[Brief error description] at line [line number] of DRL\

[FONTNAME]\[FONTNAME].shp:

[Expanded error description]

Press RETURN to continue:

If this happens, it is a good possibility that one of the

following caused the problem. If one of the shape descriptions

was too big for the shape file compiler to convert, you may not

be able to create the character without removing some of the

objects that comprise the graphic image of the character. If

there is any number in a shape description which is higher than

127, it could mean that the endpoints of some line or arc were

farther apart than 254 units in length when the Cleanup &

Spacing utility was used. If this is the case recreate the

character with the offending object broken. Having a number in a

shape description which is higher than 127 could also mean that

the third number in a triple, i.e. (12,46,-184), is a tiny arc

in a character. If this happens, change the number to 127 or -

127 and compile the font from AutoCAD's main menu.

To check the spacing and appearance of characters in a text

font, you may use the utility FontTest, available from the post

processing menu. FontTest initially displays a table of

character couplets in the font Txt. These couplets consist of

every character that can be in a text shape file juxtaposed with

the characters A, B, C, X, a, b, c and x to their right.

FontTest will change the style and font of these couplets to the

font that you would like to test, allowing you to see how the

spacing you used looks in a variety of combinations. You may

specify any font name, even ones that you didn't create

yourself, as long as the font's .SHX file is either on the DRL

directory or on its own directory beneath the DRL directory,

where AutoFont places new fonts. After selecting FontTest from

the post processing menu, if it is the first time you are using

FontTest in the drawing or if you already used FontTest in the

drawing and pre- processed characters afterwards, the prompt

Are you sure you would like to use FontTest? Yes/<No>:

appears. Next, if you have not used any pre-processing or post-

processing utilities, you are prompted

Font name:

and you must enter a font name. If the font is not on either of

the directories that it must be on, you are prompted

Shape file [FONTNAME] does not exist. *Invalid*

If the shape file is on both directories, AutoFont uses the file

DRL\[FontName]\[FontName], so if for some reason you want to use

the font on the DRL directory and there is a different file of

the same name on the font's own directory, rename the file on the

DRL directory.

If you have used any of AutoFont's utilities prior to using

FontTest you will not be asked for a font name and FontTest will

assume that you want to use the font that you specified the last

time you were asked for a font name. If you have not yet

compiled the font which you are creating, you are prompted

Shape file [FONTNAME].SHX does not exist yet.

If the font file is on either of the directories, searched for

in the order described above, you are prompted

Inserting FONTTEST.DWG...

and FontTest displays the character couplets with font Txt, the

font used by the AutoCAD default style Standard. Then you are

prompted

Changing character couplets from font TXT to font [FONTNAME]...

Each character couplet is changed to style [FONTNAME] and a

prompt is displayed which informs you of the number of character

couplets that have been changed and the percent complete.

Changed [X] character couplets. [XX.XX]% complete.

When all of the character couplets have been changed, FontTest

goes into character inspection mode. Once you have changed the

font of the character couplets from font Txt to the font which

you want to test, every subsequent time you select FontTest from

the menu you will go directly into character inspection mode.

In character inspection mode you are prompted

Next [FONTNAME] character to view (Space Bar and Enter when

done) <!>:

To inspect the character !, press the enter key. To inspect

some other character, type the character and the enter key. The

character you type is case sensitive, that is FontTest recognizes

an A as being a different character than an a.

FontTest will then zoom in on the eight character couplets which

contain the character you specified as the first character in

the character couplet, and you are prompted

Viewing character [character]

A dashed box appears around the eight character couplets. The

appearance of a question mark instead of the character as the

first character of the character couplet means that there has

been no definition made of that character in the shape file yet.

When you are done inspecting character couplets, press the space

bar and the enter key and FontTest will zoom back to display all

of the character couplets. If you would like to plot or printer

plot the characters, it is suggested that you plot or printer

plot this display to fit.

If you use any pre-processing or post-processing utilities after

using FontTest, keep in mind that if you want to use FontTest

again, you will have to wait for the character couplets to change

their font again.

After you are finished creating characters with AutoFont and

have quit the drawing, BACKUP THE DIRECTORY DRL\[FONTNAME]

or later on if you want to edit the appearance of characters

which you have already created you will have to start from

scratch with those characters.

Should you feel that a font generated by AutoFont does not have

the correct distance for a space character in a particular font,

or when you use repeated AutoCAD text commands and the space

between the lines is incorrect for a particular font, you can

change them. Using a word processor in non-document mode, edit

the .Shp file for the font you want to change. On line 4 of the

file, you can change the line spacing by replacing the two 36s

with the same other number, keeping the first 36 a negative

number and the second 36 a positive number. For example, the

line may start out looking like this

2,8,(0,-36),14,8,(30,36),0

and could be changed to

2,8,(0,-28),14,8,(30,28),0

if you wanted the line spacing to be smaller in horizontal mode.

The line

2,8,(0,-36),14,8,(30,36),0

and could be changed to

2,8,(0,-36),14,8,(24,36),0

if you wanted the line spacing to be smaller when the style is

in vertical mode.

If you want to change the size of the space character, edit line

6 of the file. The line

2,8,(19,0),14,8,(-19,-30),0

could be changed to

2,8,(12,0),14,8,(-12,-30),0

if you wanted to have a smaller space character in horizontal

mode, and the line

2,8,(19,0),14,8,(-19,-30),0

could be changed to

2,8,(19,0),14,8,(-19,-18),0

if you wanted to have a smaller space character in vertical

mode.

When you are done editing the file, you MUST compile the shape

file, which has an extension of .Shp, to a compiled shape file

with an extension of .Shx. Go to AutoCAD's main menu and select

option 7, Compile shape/font description file. When prompted

Enter NAME of shape file:

enter the name of the file with the path, i.e.

DRL\[FontName]\[FontName]

and press the enter key. If the font compiles you are prompted

Compiling shape/font description file

Compilation successful. Output file DRL\[FontName]\

[FontName].shx contains [number] bytes.

Press RETURN to continue: