OUT OF RAM

Try placing the following line into your Config.sys file.

"SHELL=COMMAND.COM /P /E:1024".  Be sure to include the

spaces.  Sometimes ACADFREERAM is not set right but if

it always happens with the same drawing it means that

in the file AutoCAD is instructed to set up a table which

is incredibly big. Like an array with 1.000.000 copies of a block.

INTERNAL ERROR

"Internal error: FREE" indicates that AutoCAD is attempting to free

a temporary work buffer of some sort, but the memory allocation

routine doesn't recognize the address as that of an allocated

buffer.  This could point to a software bug (losing track of the

buffer address, or overwriting the housekeeping info in the

buffer).

It could also be caused by faulty hardware (dropped bit in memory

or on disk).

FMTOUT 2

This error is typical of hardware where the switches for the co-

proc. are not set correctly.  Check the switches on the motherboard

and also check to make sure the co-processor is seated correctly.

FMTOUT 1

"FMTOUT 1" is a new error message in Release 10. It is issued by

some initialization code that's trying to compute various constants

based on the mathematical precision of the particular computer, and

the error tends to indicate a problem with the math coprocessor.

SMIO 1

An "SMIO 1" error would indicate a an I/O error when reading one

of the drawing's symbol (layer, linetype, text style, view, etc.)

tables. Depending on when this happened (while editing, during a

PLOT, etc.), it could point to corruption of the drawing file or

of one of the temporary files maintained in I/O page space by the

drawing editor.

MWRITE

An "MWRITE failed m,n" error indicates that AutoCAD was trying to

write the header portion of the drawing file to disk (as for SAVE,

END, or WBLOCK), but the operation failed for some reason. The

numbers indicate the expected file position following the write

(the length of the drawing file header) and the ac

"Internal error: FREE" indicates that AutoCAD is attempting to free

a temporary work buffer of some sort, but the memory allocation

routine doesn't recognize the address as that of an allocated

buffer.  This could point to a software bug (losing track of the

buffer address, or overwriting the housekeeping info in the

buffer).

It could also be caused by faulty hardware (dropped bit in memory

or on disk).

DISK FULL

The "disk full" error message and the problems with saving blocks

are most likely related to a problem that AutoCAD had in addressing

hard disk partitions larger than 32 Meg (Actually this size may

vary. The number of cluster used on a harddisk seems to be the real

culprit. This may vary with different harddisk formatting schemes.)

Although this problem was corrected in AutoCAD Release 9.04.

EREAD

EREAD errors occur when AutoCAD tries to read the drawing file and

encounters some type of file or data error when reading entities

from the file. In other words, the file was stored with unreadable

data or data that AutoCAD does not know how to interpret. The data

got garbled somehow, usually due to extended memory handling

problems, hard disk error, power surge, damaged floppy

disk, or some other cause (act of God, etc....)

TCODE

Indicates a bad header record.  Possible work around is to examine

the header fields in the DXF file if the file is salvageable in

that

fashion.

SCANDR

An acronym for Scan Database Record.  Indicates a corrupted file

in the same fashion as Eread.

GETSM

Similar to SMIO

LTINIT

The linetype table section of the database contains erroneous

definitions.

RFSubs

This (probably) is the subroutine handling image refreshing on the

screen.  If this information is stored off screen in a file then

it indicates a hardware problem in the storage of that tempory

file.

Misbehaving memory boards and hard disk controllers could cause

this message to or

display purposes.

HMATH

Indicates that the database contains an inappropriate number.

Negative radii, co-ordinates of the order of le+98, can cause this

to occur.  Editing a DXF file and scanning for abnormal numbers

generally identifies the problem.

SHELL: "Insufficient memory to continue"

This occurs if AutoCad detects an installed INT in RAM while paging

to disk on execution of the shell command.  Shell should not be run

with TSRs resident.

SHELL: "Error swapping temp file"

Indicates a hardware error encountered while paging to extended

memory or hard disk during execution of the Shell command.#####################################################################################################################################################################################################################################################################################################################################################################################################