GENSURF

General Surface Modeling Program

Version 2.10

Graphics Software Technologies

P.O. Box 3064

Orange, CA  92665

(714) 998-2900

Copyright (c) 1992-1995 by Graphics Software Technologies

All Rights Reserved

REGISTRATION

Shareware is a term for software that can be freely copied and

shared. The term describes copyrighted software which the author

supports and encourages people to copy and share.

The concept is based upon these principles:

People need to try programs to see if they are useful.

Software authors can be supported directly by users.

Copying and networking of programs can be encouraged.

The 150 point version may be used with charge, however, we

encourage you to register this version by simply sending us a

note either by mail or to this CompuServe address: 73417,1055.

If you want the 25,000 point version you may ordering it according

to the instructions at the end of this document. The price for

the 25,000 point version is only $89.00.

You can test GENSURF on point files larger than 150 points but you

will not be able to permanently save the plot in an AutoCAD drawing.

Registration has a number of benefits to you:

You   will  receive the most current version of the program

if the one you register is not the most current.

You will receive a current manual if the one you registered

is not the most current.

You will encourage us to develop more useful programs.

1.   Introduction

GENSURF is a complete GENeral SURFace modeling and contouring

package that allows you to input contour data from the keyboard

into a text file. The program creates triangulated irregular

networks (TIN), contours and 3D meshes directly in the AutoCAD

drawing.

Shareware Version 2.10

150 points maximum

3D TIN as AutoCAD faces

Processes 200 TIN's in 1 second (486/50)

Straight contour lines

Rounded contour lines

Contour lines which honor the data points

40 x 40 mesh maximum

Mesh X lines, Y lines, X&Y lines and 3D faces

Documentation on diskette

Registered Version 2.10

25,000 points maximum

3D TIN as AutoCAD faces

Processes 20,000 TIN's in 40 seconds (486/50)

Straight contour lines

Rounded contour lines

Contour lines which honor the data points

400 x 400 mesh maximum

Mesh X lines, Y lines, X&Y lines and 3D faces

Documentation on diskette

In order to keep costs to a minimum we do not provide direct

technical support.

2.   Installation

Make a directory named GENSURF. Copy all the files from the

GENSURF diskette to the GENSURF directory by using the DOS

COPY command.

The files you should have are:

GENSURF.EXP

GENSURF.LSP

GENSURF.MNU

GENSURF.DCL

GSCOM.EXP

POINTS1.DAT

POINTS1.PAR

POINTS2.DAT

POINTS2.PAR

POINTS3.DAT

POINTS3.PAR

POINTS4.DAT

POINTS4.PAR

GENSURF.DOC

3.  Starting the Program

GENSURF is only for the DOS Release 12 of AutoCAD.

Start AutoCAD as you would for any drawing. Use any drawing file

name you wish. Then load the special menu called GENSURF.MNU. To start

GENSURF just pick the SETUP choice in the GENSURF menu. This will

load the program modules and initialize them. You can now load a data

file, edit the points, set the processing parameters, process the points,

make triangles, make contours, make meshes, and plot any of them. There

is no need to keep the drawing file name and the points data file the

same in Version 2.10 as it was in Version 1.00

The first step is to read in a point data file. GENSURF will create a

parameter file if none exists from a previous run. Once a point data

file has been read in, it may be edited or you may change the point

parameters in the parameter file.

GENSURF makes an internal point file based upon the original data file

name you gave it and any parameters which would affect the data points,

such as limiting the X values to not be less that a specific value. This

is done each time you read a point file. Therefore, if you have edited

the data points file or if you have changed any of the point parameters,

you should re-read the point data file.

The points parameter file will always remain even if you exit the

drawing. It is deleted if you select New Project from the GENSURF menu.

If you plot one data set and in the same drawing want to plot another

data set, you will probably want to execute the New Project command so

that a new parameter file will be computed for the new data set. If the

second data set has the same X and Y limits, this will not be necessary.

GENSURF creates several work files. These files are:

dwgname.BIN

dwgname.PAR

dwgname.TIN

dwgname.MSH

GENSURF creates the following layers:

gs-point

gs-tri

gs-cont

gs-index

gs-mesh

4.   PROGRAM FLOW

�������������������͸

�       SETUP       �

�                   �

�   [NEW PROJECT]   �

�������������������;

�

�

�<�������������������������<��������Ŀ

�                    ����������Ŀ    �

�                    �Edit Point�    �

�������������������͸      ���>�Data File ��>�Ĵ

�                   �      �   ������������    �

�    Read Points    ������Ĵ                   �

�                   �      �   ����������Ŀ    �

�������������������;      ���>�Edit Point��>���

�                    �Parameters�

�                    ������������

�������������������͸

�                   �       ����������������͸

�  Process Points   �������>�  Plot Points   �

�                   �       ����������������;

�������������������;

�

�������������������͸

�                   �       ����������������͸

� Process Triangles �������>� Plot Triangles �

�                   �       ����������������;

�������������������;

�

��������>�������Ŀ      ���<������>�Ŀ        ��������<������Ŀ

�                �      �            �        �               �

������������Ŀ   ������������������͸ ������������������͸   ��������������Ŀ

� Edit Mesh  �<�Ĵ                  � �                  ���>� Edit Contour �

� Parameters �   � Process Contours � �   Process Mesh   �   � Parameters   �

��������������   �                  � �                  �   ����������������

������������������; ������������������;

�                    �

������������������͸ ������������������͸

�                  � �                  �

�   Plot Contours  � �    Plot Mesh     �

�                  � �                  �

������������������; ������������������;

5.   AutoCAD Menu

-------------------

Setup                     this installs the Contour program and

lisp files

New Project               resets program parameters and deletes

the current work files

-------------------

Read Points XYZ           this will prompt for the point data

file name in an ASCII X, Y, Z format

Read Point# NEZ           this will prompt for the point data

file name in an ASCII point number,

Northing, Easting and elevation format

-------------------

Edit Point File           this will place the points data

file in the text editor for editing

Edit Point Parameters     allows editing of the points parameters

file through a dialog box

Edit Contour Parameters   allows editing of the contour parameters

file through a dialog box

Edit Mesh Parameters      allows editing of the mesh parameters

file through a dialog box

-------------------

Process Points            this processes the loaded point file

Plot Points               this plots the points in the drawing

Plot Points Fast          this plots ghosted point on the screen

-------------------

Process Triangles         this process the TIN

Plot Triangles            this will plot the triangles in the

drawing

Plot Triangles Fast       this will plot a ghosted TIN

-------------------

Process Contours          this process the contour lines

Plot Quick Contours       this plots the contour polygons as

straight lines

Plot Contours Fast        this plots ghosted contour lines

-------------------

Process Mesh              this will make a 3D mesh

Process Mesh  X           plots the terrain as a series of X lines

Process Mesh  Y           plots the terrain as a series of Y lines

Process Mesh  XY          plots the terrain as a series of X and Y lines

3D Mesh Faces             this will plot the terrain as colored 3D mesh faces

-------------------

Erase Points              this erases any points on the points layer

Erase Triangles           this erases any points on the triangle layer

Erase Contours            this erases any points on the contour layer

Erase Mesh                this erases any points on the mesh layer

Erase All                 this erases all the points, triangles,

contours and meshes from their respective

layers.

-------------------

ACAD Menu                 this places you back in the regular

AutoCAD menu

-------------------

SETUP

The setup menu item runs a shall lisp program which installs

the GENSURF ADS program.

NEW PROJECT

This option resets the program parameters and deletes the current

working files. It is only necessary to use this command if you

are going to work on an entirely new data set within the same

drawing.

READ POINTS XYZ

This menu will display the copyright notice and request the points

file name. It then reads all the data for further processing. The

points are in an ASCII file with one X, Y, and Z point triplet per

line.

READ POINTS # NEZ

This menu will display the copyright notice and request the points

file name. It then reads all the data for further processing. The

points are in an ASCII file with one point number, Northing, Easting,

and elevation point triplet per data line.

EDIT POINT FILE

This menu item causes the program to place you in the text

editor with the points file. You can them edit any

coordinate.

EDIT PARAMETER FILE

This menu item places you in a dialog box so you can modify any of

the data parameter. For AutoCAD Release 11 uses a text editor is

is used to edit the parameter file.

PROCESS POINTS

This will process the points so that GENSURF can begin to make

triangles, contours and meshes.

PLOT POINTS

This will plot the points in the drawing. The points are on

a layer named

PLOT POINTS FAST

This plots the points using an AutoCAD GRDRAW command. It is

very fast since the points do not become a part of the

drawing. Any command which causes a redraw will erase this

image.

PROCESS TRIANGLES

This create the TIN.

PLOT TRIANGLES

This will plot the triangles in the drawing. The triangles

are on a layer named

PLOT TRIANGLES FAST

This plots the triangles using an AutoCAD GRDRAW command. It

is very fast since the triangles do not become a part of the

drawing. Any command which causes a redraw will erase this

image.

PROCESS CONTOURS

This will create the connected contour lines. The lines are

stored as polygons.

PLOT QUICK CONTOURS

This will plot the contour polylines in the AutoCAD drawing.

They are drawn as straight lines with no rounding. The lines

honor all data points. The polylines could be smoothed using

an AutoCAD command.

PLOT ROUND CONTOURS

This will plot the contour polylines in the AutoCAD drawing.

They are drawn as rounded lines. The rounding process does not

allow the lines to honor the data points but they look pleasing

to the eye. This is the same process that nearly all contouring

program use.

PLOT HONOR CONTOURS

This will plot the contour polylines in the AutoCAD drawing.

They are drawn as so as to honor the data points as best as

possible. Wherease in the rounding method the curves fall short

of data points which carry the same elevation as the contour line

this method reforms the lines to come exactly to or very nearly

to the data points that are the same elevation as the contour

line.

PLOT CONTOURS FAST

This plots the contour polygons using an AutoCAD GRDRAW

command. It is very fast since the are not part of the

drawing. Any command which causes a redraw will erase this

image.

PROCESS MESH

This creates the mesh

PLOT MESH X

This plots a mesh in the AutoCAD drawing. Each line in the X

direction is drawn as a single polygon.

PLOT MESH Y

This plots a mesh in the AutoCAD drawing. Each line in the Y

direction is drawn as a single polygon.

PLOT MESH XY

This plots a mesh in the AutoCAD drawing. Each line in the X

direction is a single polygon as is each line in the Y

direction.

PLOT 3D MESH

This plots a mesh in the AutoCAD drawing. Each rectangle is drawn

as an AutoCAD 3D face. The color is a function of the number of

the color step defined in the parameter file.

ERASE POINTS/TRIANGLES/CONTOURS/MESHES/ALL

These series of commands will erase the corresponding entities

from their respective layers. This allows the use to quickly

change the parameters and re-plot the drawing.

6.   Data File

The points data file is a file containing the X, Y, Z

information for each point of data to be used in the

calculations of the contour map. The program will prompt for

the data file name. The name may end with any extension.

GENSURF expects the data to be stored in an ASCII text file.

This file may be created from a text editor, spreadsheet,

output from a data collector, or entered directly from the

DOS editor.

The X, Y, Z data are entered in free format. That means they

can be located anywhere on the line as long as they are

separated by at least one space.

460.    0.    150.

540.    0.    150.

480.  300.    152.

560.  280.    152.

620.  700.    155.

705.  700.    155.

465.  100.    150.5

545.  100.    150.5

470.  160.    151.

550.  160.    151.

475.  240.    151.5

555.  240.    151.5

530.  440.    153.

615.  440.    153.

570.  560.    154.

655.  560.    154.

590.  610.    154.5

675.  610.    154.5

510.  380.    152.5

595.  380.    152.5

GENSURF can also read data files from programs such as COGO

programs which carry the data points in a format which consists

of a point number, Northing, Easting and elevation. GENSURF

ignores the point number.

The point number, Northing, Easting and elevation data are entered

in free format. That means they can be located anywhere on the line

as long as they are separated by at least one space.

1       0.  460.    150.

2       0.  540.    150.

3     300.  480.    152.

4     280.  560.    152.

5     700.  620.    155.

6     700.  705.    155.

7     100.  465.    150.5

8     100.  545.    150.5

9     160.  470.    151.

10     160.  550.    151.

11     240.  475.    151.5

12     240.  555.    151.5

13     440.  530.    153.

14     440.  615.    153.

15     560.  570.    154.

16     560.  655.    154.

17     610.  590.    154.5

18     610.  675.    154.5

19     380.  510.    152.5

20     380.  595.    152.5

7.   Parameter File Format

Contour Parameter File .PAR

The Parameter File contains the program parameters that determine

ranges of data for calculations, plotting conditions, and mesh

information. There is no need to manually make any changes to this

file. GENSURF makes these changes as you edit the parameters through

a dialog box.

Each line contains a text description of the parameter followed

by an equal sign and then followed by the corresponding data.

Changes can only be made to the values to the right of the equal

sign (=). You cannot add, delete, or move any lines in this file.

A minimum of one space must precede and follow the equal sign.

Not all values are checked by GENSURF for validity, therefore,

incorrect entries may cause unusual behavior.

Listed below is the information file, with example data and an

explanation of the use of each parameter.

0.00  =    easting (x) minimum

1000.00  =    easting (x) maximum

0.00  =    northing (y) minimum

1000.00  =    northing (y) maximum

0.00  =    minimum elevation

250.00  =    maximum elevation

5.0   =    contour interval

50.0   =    heavy contour interval

1.50  =    elevation exaggeration

20.00  =    number of mesh lines in the X direction

20.00  =    number of mesh lines in the Y direction

10.00  =    number of color steps in the 3D mesh

easting (x) minimum

This value determines the left most boundary of data

that will be read in and used for calculations.

easting (x) maximum

This value determines the right most boundary of data

that will be read in and used for calculations.

northing (y) minimum

This value determines the bottom boundary of data that

will be read in and used for calculations.

northing (y) maximum

This value determines the upper boundary of data that

will be read in and used for calculations.

minimum elevation

This value determines the lowest elevation of data that

will be read in and used for calculations. This number

is usually an integer and used as the starting

elevation for plotting contour lines.

maximum elevation

This value determines the highest elevation of data

that will be read in and used for calculations.

20.00  =    number of mesh lines in the X direction

20.00  =    number of mesh lines in the Y direction

10.00  =    number of color steps in the 3D mesh

contour interval

This value determines the elevation interval of

calculated and plotted contour lines. In the example, a

contour line will be plotted every 2 feet.

heavy contour interval

This value is the annotated contour line interval. For

example, for a contour interval of 2 feet, a heavy line

will be drawn every 10 feet or each fifth interval when

cont heavy int is set to 10.0. A value of 0.0 indicates

no heavy contour will be drawn.

elevation exaggeration

When plotting in 3D, it is often necessary to

exaggerate the elevations in order make them look

appropriate. The plot z scale factor allows you to

exaggerate the elevation scale. There are three values

available for selection:

< 0.0  automatic z factoring

= 0.0  uses actual elevation

> 0.0  uses the factor as a multiplication factor

number of mesh lines in the X direction

This value determines the number of mesh intervals in the

X direction. It can base the number of units on an integer

number of intervals or a specific distance depending upon

the sign of the value. The minimum number of intervals is 10.

> 0.0  uses the value as the number of integer intervals

= 0.0  uses 10 intervals

< 0.0  uses the value as the true distance between intervals

number of mesh lines in the Y direction

This value determines the number of mesh intervals in the

Y direction. It can base the number of units on an integer

number of intervals or a specific distance depending upon

the sign of the value. The minimum number of intervals is 10.

> 0.0  uses the value as the number of integer intervals

= 0.0  uses 10 intervals

< 0.0  uses the value as the true distance between intervals

number of color steps in the 3D mesh

This value determines the number of mesh colors to be used in

displaying the 3D mesh. For example, if the number is 6, then

the 3D mesh will be displayed in six arbitrary colors from the

lowest to highest elevation.

8.   EDITOR

GENSURF makes use of the DOS editor EDIT. The purpose of the

editor is to quickly and easily edit GENSURF data files.

You can use another editor. GENSURF uses a program named

EDIT.EXE as the editor. This is the DOS editor by default, but

if you name another editor to have the same name, and it is

in the GENSURF or ACAD diretory, then it will be used instead.

When you have completed your editing by either quitting or

saving a file and exiting, you will be back in AutoCAD. If

you save the file before returning to the program, the

editor automatically saves a backup file with the extension

".BAK".

If you have made changes to your data file then you will

need to load the points file again by chosing that option in

the menu.

9.   THEORY AND CONCEPTS

Data Selection

The GENSURF program reads all of the data to create a

contour map. It is possible to use only a subset of the data

for creating a map. There are two situations where this may

be required:

1.   There is more data in the file than you want to use to

make your map.

2.   You may want a map containing only certain contour

intervals.

There are two methods of data selection. One method is to

define the window of included data by entering the

appropriate values in the .PAR file for:

x min calc

x max calc

y min calc

y max calc

The second method is to define the range of acceptable

elevations. If a point contains an elevation outside this

range, it will not be included in the calculations.

Creating the TIN

GENSURF is based upon the method and algorithms for

Triangulated Irregular Network or TIN's. GENSURF generates

true Delauney triangulation.

The program determines the best way to connect the points to

form Delauney triangles. It analyzes these triangles in a

fast, single pass procedure.

Interpolated Contour Strings

Once the triangles are established, the program locates the

point of crossing of the specified contour interval for each

leg of the triangle.

The program will then connect all of the located points of

the same contour values for each triangle. These lines are

referred to as elements.

Next, the program will sort all of the elements by

elevation, taking into account the adjoining of the points

creating an AutoCAD polyline.

The polylines may then be displayed by by using the Quick

method.

Display Methods

There is one method for displaying the contour lines that

have been calculated.

The QUICK method displays the contour lines as straight

lines between the interpolated points along each triangle.

This method is useful for quickly checking changes made in

the map, as well as producing final maps which have a lot of

data points.

The FAST method will display the line using GRDRAW. This

method allows you to see the lines without having them

actually become part of the drawing.

10.  TESTING LARGER POINT FILES

You can test point files larger than 150 points, however only the

"fast plot" options will be available. You can see either the points

plotted, the triangles plotted or the contours plotted. You cannot

see them plotted at the same time and you cannot plot meshes. This

mode allows a maximum of 2000 points.

GENSURF REGISTRATION FORM

Name________________________________________________

Company_____________________________________________

Address_____________________________________________

City, State, Zip____________________________________

Daytime telephone___________________________________

To  register for the 25,000 point version send $89.00 (money order,

or personal check  -  California residents add 7.75% sales tax) to:

Graphics Software Technologies

P.O. Box 3064

Orange, CA 92665

There is no charge for using the 150 point version, but you are

requested to let us know by that you are using the product by leaving

us a simple note on Compuserve (73417,1055) or by mail.

Likes, dislikes, features wanted, problems?  List below:

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________