LaserJet series AutoCAD driver version 1.00.

Copyright (C)1994 by Walter Fredrick Associates, Inc. All Rights Reserved

Programmed by Sterling Blake.  Release 1.00 on 1/24/94.

Walter Fredrick Associates, Inc.

6 Summit Industrial Park

Peabody, MA 01960

Phone: 800-343-8296 and 508-531-3999

Fax:508-531-4664 and 800-269-1071

BBS:508-531-3883 300..14400,8,n,1

Compuserve: 76310,212

GEnie: SMBLAKE

InterNet: smblake@genie.geis.com

***Installation***

This driver works with ACAD release 12 when copied to the directory in which

your drivers are installed (C:\ACAD\DRV is default). To use this driver you

must:

1. Copy the PLPLJWLW.EXP file to your drivers directory.

2. Start AutoCAD.

3. Issue "CONFIG" command via prompt or menu.

4. Select "Configure ACAD."

5. Select "Configure plotter."

6. Select "Add a plotter configuration."

7. Select driver description "HP LaserJet ADI 4.2 version 1.0 - by Walter

W. Fredick Assoc, Inc."

8. Select the appropriate model.

9. Select port type (serial or parallel) however your printer is connected.

10. Select resolution (75, 150, or 300), 300 is nicest and slowest.

11. Select number of copies (1 to 99).

12. Select paper size, letter or legal.

13. Select media source.

14. Opt to delete downloaded fonts or preserve.

15. Answer registration information.

16. Select port.

17. Change plot-time options. (All of these options may be altered from the

plot dialog box.)

18. Enter a description for this plotter. You can use anything you like here.

19. Exit to configuration menu.

20. Exit out to drawing editor (keep configuration changes).

21. When you are ready to print, issue a plot command and use the Device and

Default Selections button to pick the description you just gave at

configuration.

22. I strongly recommend that you turn off optimization.

***Configuration Options***

Resolution:

The default resolution for each printer model is the highest resolution that

your printer is likely to have sufficient memory to print. If you generate a

printer error #20, memory overflow, then you should reduce the resolution in

your configuration (or delete downloaded fonts, see below).

Copies:

Additional copies of a plot are handled by the printer; AutoCAD will not re-

transmit your drawing.

Paper source:

The four options available in this driver are current, main, manual, and

alternate.  Not all of these sources may be available on your printer, so you

should refer to your user's manual for a description of your printer's tray

types. The PCL command associated with each of these options is "[ESC]&l#H".

The value of # is as follows: current=0, main=1, manual=2, and alternate=4.

If in doubt of which media source command corresponds to which tray on your

printer (or if another # describes the tray you wish to use), set the source

from your control panel and leave the driver setting at current.

Delete downloaded fonts:

A "Y" respose will cause the driver to issue a printer reset command before

and after your plot. This will make all of the printer's memory available

for your plot, and eject any partial pages left in the printer. A "N"

response to this option will force the driver to skip the resetting commands

at both the start and end of your plot.

Registration:

This driver must be registered or removed from your system within 30 days.

At new plotter configuration time you will be asked if this driver is

registered. If you have not purchased a license answer 'N' for no. If you

have you may answer 'Y' for yes. After answering 'Y' you will be asked for

the serial number and password provided to you by Walter Fredrick Associates.

You will have to re-enter these numbers anytime you select a new device as

being "! HP DeskJet..." After entering the serial number and password you

will be asked if you would like single answer trivia activated. A 'N'

response will make this driver behave as any other plotter driver in ACAD,

and a 'Y' response will provide you with trivia in the form of an ACAD alert

after each plot.

***Known Problems***

Digitizer:

When the ACAD alert box is displayed any digitizer is disabled until the box

is cleared (by hitting the RETURN key).  This is related to the AutoCAD ADI

Kit, and does not happen in the registered version, where there is no alert

displayed after every plot.

Plot optimization:

The default plot optimization is set to level 1 (end-point swap).

Optimization is useless on a raster device such as the DeskJet so please turn

it off (after you've made one print with it turned off it will remain in that

state until you specifically turn it on or reconfigure).  Turning off plot

optimization actually makes the driver run more quickly.  Again, this is an

AutoCAD limitation.

***Registration***

This driver is provided as is. There are no warranties expressed or implied.

This driver is distributed as shareware. You may use it without charge on an

evaluation basis for a period of not longer than 30 days. If you wish to use

it after the evaluation period, you must purchase a registration. That

registration allows use by one copy of AutoCAD. There is no cost to register

a later version in the same series after registering any version in that

series (i.e., if you register v1.20, then registration of v1.31 is at no

additional charge, however v2.10 would require a registration fee. Please

note that although an upgrade within a series is without charge, it is still

good for only one copy, therefore if you had registered v1.20 and then v1.31,

the registration on v1.20 expires.)

For network and site licensing please contact us.

Unregistered versions of this driver may be distributed freely, provided that

all accompanying files are included and unmodified, and there is no charge

for it.

To register you may print and fill out the REGISTER.TXT file and then mail to

Walter Fredrick Associates, 6 Summit Industrial Park, Peabody, MA 01960, or

fax it to 508-531-4664.  Alternately, you may call 508-531-3999 or

800-343-8296 and order with Visa, MasterCard, or Discover.

The fee to register is $25 per license. Please add sales tax in CT, MA, ME,

RI, and VT.

The registration fee gets you the most recent version of this driver (which

won't tell you to register everytime it's used) as well as technical support

in using it. This will also ensure that the driver is updated in a timely

manner.