AutoSTAIR

---------

STAIR STRINGER DRAWING PROGRAM

------------------------------

copywrited  (C) 1993 by

John Williams Detailing

6012 stellhorn road suite 212

Fort Wayne, IN 46815

Phone/fax (219) 486-0513

AutoCAD is a registered trade mark of Autodesk inc.

Highlighs of AutoSTAIR

----------------------

Will draw stair stringers and dimension them with a special fraction font

standard to steel industry.

Included with the stair program is a horiz. & ver. dimension program that

dimensions with the same font.

Allows detailer to choose the stair pan style. Encludes bolted treads and

checked plate.

Wblocks stair pans for later use in preparing stair pan fabrication drawing.

Getting started:

----------------

Copy all Files on the floppy disk to your hard drive.

The files must be located in a directory that AutoCAD will find.

AutoCAD looks for files in the current directory, the directory

that ACAD.EXE file is located or a directory set by DOS before

entering AutoCAD.  Example:  set acad=c:\acad\lisp

Example  :  copy a:\*.* c:\acad\lisp

Replace c:\acad\lisp with directory you want files to reside.

Note, Directory must exist for files to be copyed to it.

Enter AutoCad as normnal.

To start stair, at the command prompt type.    (load"stair")

to run the stair program, at the command prompt type.   stair

Page 1

Answering the prompts

----------------------

To except the defaults just hit the enter key.

Note: After the program runs once, the Defaults will be remembered

and used the next time program is ran in this drawing session.

If you exit the drawing and return, the previous DEFAULTS are lost.

Type of stair 1 or 2 :

1 for stringer style that sets on the floor.

2 for stringer style that has a dog leg top and bot.

Enter stringer size <DEFAULT> : example : MC12x10.6

Enter run in FEET <DEAULT> : Enter number FEET only in run

example : if run is 8'-4  Enter 8

Enter run in INCHES <DEFAULT> :Enter numher INCHES in run

example : if run is 8'-4  Enter 4

Enter floor to floor in FEET <DEFAULT> :  Enter FEET only

Enter floor to floor in INCHES <DEFAULT> : Enter INCHES only

Enter nosing to stringer edge <DEFAULT> : Measured from stair nosing

perpendicular to stringer edge.

Enter dimension of stringer above TOP landing <DFAULT> :

Note: dimension is above finish floor. (enter a minus dimension

if top is below finish flooor or landing.)

Enter dimension of stringer above BOT. landing <DEFAULT> :  As above

Enter number of treads  <DEFAULT> :  Note : program defaults to number

of treads that will divide by an interger betwwen 6" and 13",

if tread dimension is not whole number, enter number of treads.

Dimension stringer [Y]es or [N]o ? <Y> : dimension stringer for

fabrication drawings, if stringer is used in an erection

drawing, you might not want dimensions.

Select stair pan style : Enter numer of pan style reqd.

First riser dimension <DEFAULT> : program divides number of risers into

floor to floor, If for some reason you want a different first

riser enter it.

Pick point point of top nosing :

Stringer will be drawn.

Do you want to wblock stair pan ? : If you want to wblock pan to be used

later  for a pan fabrication drawing, enter Y

and answer the prompts.

Lnading depth :

Pan depth :

Stringer is drawn with pans and dimensions. Detailer can finish horz.

details. Add support angle requirments, closer plates, if reqd. etc.

========================================================================

==== Demo ver. will draw stair type 1 only and stair pan type 1 only==

========================================================================

7

page 2

The stair program uses a special fraction font file, addwg.shx

to get the fractions with the text commnd or in the dimensioning

command enter the following :

~1  for 1/16

~2  for 1/8

~3  for 3/16

~4  for 1/4

~5  for 5/16

~6  for 3/8

~7  for 7/16

~8  for 1/2

~9  for 9/16

~A  for 5/8  note must be capital A

~B  for 11/16  must be caps

~C  for 3/4    must be caps

~D  for 13/16  must be caps

~E  for 7/8    must be caps

~F  for 15/16  must be caps

======  These programs not included with demo ver.=====

Also included with the stair program is a program that replaces AutoCAD,s

horz. and vertical dimensioning command.

The dimensioning program save you from having to type in the fraction

characters above.

To use (LOAD"HVDIM") at the command line type HVDIM

To include in you menu

^C^C^P(progn (if (not c:hvdim)(load"hvdim"))(setq DIMV "H")(C:HVDIM)(PRINC))

This will cause HVDIM to load and run and will automatically be in the

horiz. dimensioning mode.

^C^C^P(progn (if (not c:hvdim)(load"hvdim"))(setq DIMV "V")(C:HVDIM)(PRINC))

This will cause HVDIM to load and run and will automatically be in the

vrt. dimensioning mode.

You might note these programs set snap mode to end point or intersection

so that you can just pick near the line or intersection of lines you want

to dimension.

There is a small program to draw witness lines. To use it (load"wtlne")

at the command line type wtlne

Another usefull dimensioning routine is CSDH.LSP & CSDV.LSP. These programs

are used for continues dimensioning. It doesn't draw witness

lines however, wtlne routine can be used for this if line or lines you need

to dimension to aren't already in place.

Note: these dimensioning programs only dimension horz. and vertical modes.

(for now), You will have to use AutoCAD's dimensioning command for diagonal

dimensioning.

To draw lines, move, stretch, copy etc. perpendicular or parallel to

stringer load angpic program. It will change the cross hair angle to

the angle of line that you pick. (must pick a line, not pline)

When dtext command is used the angle of text can be set with angpic.

Pick starting point and drag direction of text, text angle is set.

page 3

If stairpans were wblocked and are inserted into a drawing for

dimensioning you will note the text has been scaled down by 1/3.

It is assumed stringer was to be plotted 1"=12"  and stairpans are

to be plotted at 3"-12".

Stair pans do not have sanity cove, if it is reqd, pan will have to

be edited and length of material incressed.

The length of material to make the stair pans has been calculated by the

program. However, the bottom pan for stringer type 2 has been calculated

with a 3" horz. dimension. The material length will have to be changed.

A useful program to do this is included on the bonus disk from AutoCAD.

It's called CHGTXT.Lsp  . Load it, at the command line type chgtxt  .

The stair program sets layers Steel, dim, and dimtxt and sets colors green,

red and white.  If you want different colors set these layers before running

stair program.  Better yet have these layers set in your prototype drawing.

To set text style to addwg, at the command prompt type style

text style name (or ?) <text>: addwg

Existing style.

font file <text>: simplex,addwg

Height <0'-1 1/2>: 1.5

Width factor <1.00>: 1

Obliquing angle <0d0'0">

backwards <N> N

Uside-down <N> N

ADDWG is now the current text style.  <== you should see this

This can also be entered on your protype drawing.

It is sugested limits to be set to 36',24' if plotting at 1"=12"

when plotting stringers it is suggested the steel layer be plotted with a

heavier pin than the other layers. I use 0.70mm for steel and title layers

and 0.35mm for  all others.

To have the programs to load and run automatically, they can be called

from your menu.  to be ran from your menu, edit AutoCAD's menu with a

text editer or edlin.   To call it from the tablet menu, under

***tablet1   <<<< number of tablet menu reqd.

^c^c^(progn (if (not c:stair)(load"stair"))(stair)(princ))

Write the name of the command on your menu at the proper place.

to call from screen or pop down, under

***pop1     <<<< number pop menu reqd.

^c^c^p[stair](progn (if (not c:stair)(load"stair"))(stair)(princ))

Mirror stringer.

----------------

If stringer needs to be detailed opp. hand, mirror stringer and move

vertical text. AutoCAD system variable MIRRTEXT must be set to 0.

To do this at the command prompt

Command: SETVAR

Variable name or ?:MIRRTEXT

New value for MIRRTEXT <1>: 0

page 4

--------------------------------------------------------------------------

ORDER FORM

Name    _______________________________________________________________

Address _______________________________________________________________

_______________________________________________________________

City    _________________________________ State ______ Zip ____________

Phone   (______) ___________________

Floppy disk size required:    [  ] 5 1/4"     [  ] 3 1/2"

Your purchase includes unlimited telephone support.

NOTICE:  The software license for AutoStair is for the EXCLUSIVE use

of the ORIGINAL purchaser on ONE computer only.  AutoStair is

protected by U.S. copyright law.  For discounts on multiple copies of

AutoStair contact John Williams Inc.

Extended

Quantity    Item                Price         Price

________    AutoStair          $500.00      $__________

For Indiana orders add 5% :   __________

TOTAL:  $__________

Mail order form with payment to:

John Williams

6012 Stellhorn Road, Suite 212

Fort Wayne, IN  46815

Phone/fax:  (219) 486-0513