SOMETIMES LESS IS BETTER

We use some big Lisp routines, and we have a big CAD program

that runs them, but on an every day basis, I never forget that most

of my time is spent just drawing. Lines, circles, and blocks. Over

and over. There are some small utility routines that make it easier.

Simon Jones' Lisp routine "Relative.lsp", which comes with ACAD, is

a congenial way to go up an "angled" line a specific distance and

then start one of the draw commands. Here's how it works.(I'll assume

that Relative.lsp is loaded into ACAD): You've got to start a line

3.5 inches from the end of a line that's on a 33 degree angle:

WHAT TO DO COMMAND or ENTRY the line command:.............LINE 2)

Type:.................................!(relative)   (Or better still,

put the    following in your menu:

[RELATIVE]!(RELATIVE)$S=  ) 3) Pick the end of the 33 deg.line when

prompted with "Relative to: .(use snap "END") 4)   Tell the routine

how far up the line you want to go: 3.5 5)  Tell the routine the angle:

33 6) The line will start 3.5" along the 33 deg. line: and, at 3.5",

it will begin.

This is what the routine looks like:

(defun RELATIVE () (setq pts (getpoint "\nRelative to: ")) (setq dist

(getdist pts "\nRelative distance: ")) (setq ang (getangle pts "\nRelative

angle: ")) (setq pt (polar pts ang dist)) )  I don't use this routine

often because I seldom have to go up angled lines. But what I need

to do often is draw something a known X and Y distance away from a

point on a drawing without having to draw construction lines or figure

out the absolute coordinates. An example would be: "Oops, I left a

line out. It is 6" over and 2" down from the corner of the part".

To do this easily, I wrote "Newpoint.Lsp"; and this is how it works:

WHAT TO DO COMMAND or ENTRY the line command: .LINE 2. When AutoCad

asks for a point,  enter !(NEWPOINT) (Or better still, put the following

in your menu: [NEWPOINT]!(NEWPOINT)$S=  ) 3. Then select the point

of reference,  (Relative to) .pick the corner4. Last, type in the

X and Y distances from the above point 6,-2 5) A line will start from

the relative    coordinates that you just picked: look, Jane - see

the line.

This is what the routine looks like:

(defun NEWPOINT (/ pts dist lst lim)  (setq lim(getvar "limcheck"))  (setvar

"limcheck" 0)  (setq pts (getpoint "\nRelative to ?:(pick a point):  "))  (setq

dist (getpoint "\nEnter the Relative Distance From the Point (X,Y)

: "))  (setvar "limcheck" lim)  (setq lst(list(+ (car pts)(car dist))(+

(cadr pts)(cadr dist)))) )

What would you do if you had to start a line 3.125 + 5.062 +

10.875 - 4.187  away from the corner of a porch in the X axis, and

something even worse in the Y axis? Well, that is what Calc.lsp does.

It is a full algebraic calculator that can accomplish the above task;

in fact it was written specifically to solve for X and Y coordinates

and then allow draw commands to take place at the calculated coordinates.

(It adds, subtracts, multiplies, and divides too.) I will describe

this program in the next TAG journal.

Thanks, Simon.

Bob De Vault

#