The purpose of this program is to allow the addition of an extra

communications port to IBM-family machines.  This is a

programmable port addressed as com3: and located at address

3e8h.  It may be configured using the TERM.EXE program.   Type

TERM HELP for details.

The existing IBM-supplied single communications port may be used

in this application if installed multi-function boards are

presently supplying COM1: and COM2:

MODS to ASYNC BOARD to SUPPORT COM3:

1.      Break trace A27 to U2-5

2.      Connect A27 to U6-12

3.      Connect U6-11 to U2-5

4.      Connect U6-13 to +5Vdc. (U6-14)

5.      Break trace from U15-6 to B25

6.      Connect U15-6 to B4.

Refer to IBM Technical Reference Personal Computer XT for above designations.

These wiring changes should only be undertaken by personnel experienced and

equipped with PC board tools and the techniques involved.

Install async3.sys in the config.sys file as DEVICE=ASYNC3.SYS

The new serial port will not appear in the port table located at Segment40:0

This modification may be used with any software that allows you to specify

either COM3: or port 3E8h.

Direct inquiries to:

Art Shulman,

Box 850

Waterford, Ontario, Canada.

N0E 1Y0

Telephone (519) 443-4321 after 6:00 Pm. EST.

Files included:         This file you are reading.

ASYNC3.SYS

TERM.EXE

--------------------------------------------------------------------------

Notes added 6/3/86, Edward Vielmetti, University of Michigan.

TERM HELP yields the following output.

This command accepts many options to control the

behavior of the ASYNC drivers.

Any option terminated with : is assumed to be the

name of the ASYNC drive to operate on.   COM3: is default.

The option SPEED=nnn will set the baud rate of the driver

to nnn.  Illegal baud rates are ignored.

All the following ASYNC driver options can be set by

typing the first few letters of the option.  To turn

an option off, prefix it with NO.

dsr is output data throttle signal

cts is output data throttle signal

xonxoff throttles output data

offcts -- cts is ofline signal

offcd -- carrier detect is offline signal

offdsr -- dsr is offline signal

outstrip -- strip parity on all output characters

outupper -- convert output chars to upper case

tabs are converted to spaces on output

dtr is input data throttle signal

rts is input data throttle signal

inxon -- xon/xoff is used to throttle input data

bell is echoed on input buffer full

half duplex -- input chars are echoed when read

flush sequence is recognized (control-x)

redraw sequence is recognized (control-r)

delete line sequence is recognized (control-u)

skip output sequence is recognized (contril-o)

errstat -- line errors produce MSDOS error status returns

errcodes -- line errors return codes with parity bit on

strip parity on input characters

upper case all input characters

control-c passes through input buffer on non-destructive read

reboot sequence is recognized (control-c control-r control-b

status of device COM3  :        Baud rate: 0

>>>>Special treatment on input:

>>>>Special treatment on output: