�˻�ͻ�˻

����ͼ��� - print

�

(C) 1985

�˻�ͻ�˻

����ͼ��� enterprises, inc.

�

306 Park Avenue

Wilmette, Illinois  60091

�˻�ͻ�˻

����ͼ��� - print  is a utility which allows any Epson

�               printer to print IBM graphics characters

in a way that is completely transparent to

you (the user) or any program or operation

on the computer (including PrintScreen).

When it is installed, MPM-print becomes

memory-resident, that is, it stays active

in a small portion of your computer's

memory until your computer is turned off

or "rebooted".

After it is installed, MPM-print allows an

Epson printer to print 240 of the 255

graphics characters that IBM PC monitors

can display (which is more IBM characters

than even the IBM Graphics Printer can

print).

MPM-print is an ideal addition to your

"AUTOEXEC.BAT" file; if it is included, it

will be installed in memory whenever you

restart your computer, and will then

always allow IBM's special characters to

print on your printer.

The system will work with Epson FX, MX

(with Graftrax Plus), and RX series

printers, AND with the IBM Graphics

Printer (which is a version of the Epson

MX printer).  MPM-print uses less than

3000 bytes of system memory when it is

installed, and will work with PC-DOS

versions 1.1 and above.

�˻�ͻ�˻

����ͼ��� - print                                      Page 2

�

INSTALLATION       MPM-print is installed simply by typing

������������       its name at the DOS prompt, i.e.

DOS prompt

����������

�

A>MPMprint <enter>

�         �

����������Ŀ   �� carriage return or

program name      enter key

MPM-print will then "attach" itself to

DOS, and will print the following message

to the screen:

�˻�ͻ�˻

����ͼ��� - print

�

(C) 1985

�˻�ͻ�˻

����ͼ��� enterprises, inc.

�

306 Park Avenue

Wilmette, Illinois  60091

After it has been installed, MPM-print

cannot be re-installed (except in unusual

circumstances, see TECHNICAL NOTES below).

If you try to reinstall the system MPM-

print will give you the following message:

�˻�ͻ�˻

����ͼ��� - print

�

is already installed

�˻�ͻ�˻

����ͼ��� - print                                      Page 3

�

USE                After it has been installed, MPM-print is

���                completely transparent both to you (the

user) and to programs running on your

computer.

MPM-print "reviews" each character that is

sent to the printer, and takes one of four

actions:

1.  If the character is "shared" by

Epson and by the IBM monitors, it will

be printed normally.  Only the

"standard" ASCII characters, ranging

from the space (ASCII 32) to the ~

(ASCII 126) are shared.

2.  If the character is an Epson

control character, it will be passed

without change to the printer (see

Appendix B for a listing of all Epson

control codes).

3.  If the character is an Escape

(ASCII 27), MPM-print will send the

Escape character AND the appropriate

number of following characters to the

printer without change (Epson printers

use "Escape sequences" to control a

wide variety of printing functions.

See Appendix B for a listing of Epson

escape sequences).

4.  Finally, if the character is one of

the 145 characters that are neither

"shared" nor printer control

characters, then MPM-print will

"translate" the character and print the

IBM monitor's version of that character

using the Epson printer's graphics

capability.

�˻�ͻ�˻

����ͼ��� - print                                      Page 4

�

TESTING            Included with MPM-print are two versions

�������            of a testing program, TEST.COM and

TEST.PAS.  Written in TURBO Pascal, the

testing programs will "exercise" most of

the print commands available for the Epson

printers.

For each control code and escape sequence

(see Appendix B), the program will set the

printer's mode, then (if appropriate) it

will print the 240 non-control characters,

and finally will restore the printer to

its original mode.

The purpose of the test is to verify that

MPM-print will properly transmit the

control codes/escape sequences to the

printer, and that it will properly print

the full 240 MPM-print character set after

setting the printer's mode.

TEST.COM was compiled using TURBO Pascal

version 3.0, and will only run with DOS

2.0 or higher.

The source code (TEST.PAS) is provided to

allow the user to study the use of the

Epson control codes and escape sequences,

and to convert the testing program to

other languages.

The compiled version of TEST will take 20-

25 minutes to run (it is limited by the

speed of the Epson printers), and will

print about seven pages.

�˻�ͻ�˻

����ͼ��� - print                                      Page 5

�

DISTRIBUTION       MPM-print is being distributed as

������������       "user-supported software", with the

following conditions:

MPM-print and its associated

documentation are the copyrighted

property of MPM Enterprises, Inc.

We grant NON-COMMERCIAL users the right

to use, copy and share MPM-print.

Any distribution of MPM-print must

contain the unaltered versions of the

program itself and all associated

documentation:

MPMprint.com

MPMprint.doc

MPMprint.prt

READ    .ME

TEST    .PAS

TEST    .COM

MPM-print may NOT be sold or

distributed for profit.

Users are encouraged to register their

use with MPM Enterprises, Inc.  A $10

contribution to encourage our future

developments is suggested.  Registered

users will be notified of updates,

enhancements, and new products from

MPM Enterprises, Inc.

Commercial use WITHOUT a $10 per copy

registration fee is prohibited.  Site

licenses are available.

MPM-print is provided on an "as-is" basis,

and no warranty is expressed or implied.

In no event shall MPM Enterprises, Inc., be

liable for any commercial or non-

commercial damage arising from the use of

MPM-print.

�˻�ͻ�˻

����ͼ��� - print                                      Page 6

�

TECHNICAL NOTES    SPEED

���������������    �����

MPM-print uses the graphics printing

capabilities of the Epson printers to

produce the 145 IBM characters that the

Epson printers normally don't print.

Printing using the graphics mode requires

more information and effort on the part of

the printer, so the 145 non-Epson

characters take longer to print.  (This is

why the 'TEST' program takes so long to

print 7 pages of output).  "Normal" text

is printed at the normal rate of speed.

If your text makes use of a large number

of the IBM's "special" characters, the

text printing will be significantly slower

than it would have been if you wanted to

look at the funny characters that the

Epson uses in place of the IBM characters.

If, however, your text contains relatively

few "special" characters, then printing

will be virtually as fast as it would be

WITHOUT MPM-print installed.

LINE SPACING

������������

Because of limitations with certain Epson

printers, the graphics characters printed

by MPM-print are 8 dots high.  Therefore,

to get any of the vertical bar characters

(used for drawing boxes, or for the MPM

logo, for example) to print contiguously

(without any vertical white space between

them), the printer must be set to print 8

dots per line.  This can be done by

sending the escape sequence ESCAPE 65 8 to

the printer (see the listing of escape

code sequences in Appendix B).

CHARACTER SIZES

���������������

The IBM characters that MPM-print produces

are printed in the normal (pica) mode.

Although MPM-print will allow a program to

set the printer to any mode (expanded,

compressed, etc.), the IBM characters will

still be printed in the pica mode.

�˻�ͻ�˻

����ͼ��� - print                                      Page 7

�

TECHNICAL NOTES    EPSON CONTROL CODES

���������������    �������������������

Epson printers use certain one-byte codes

(ranging from ASCII 7 to 27) to control

some of the printers' features (see

Appendix B).  Each feature can be

activated by sending EITHER the code OR

the code+128 to the printer (for example,

both ASCII 7 and ASCII 135 would "beep"

the printer's speaker).  We have chosen to

print IBM characters for all ASCII codes

above 127, since printer control codes are

already provided by ASCII characters 7-27.

We have also chosen to print the IBM

character  (ASCII 127) instead of the

"delete" code for the Epson printers.  The

"delete" code deletes the last character

in the Epson printer's line buffer; since

it is rarely used, we chose to print the

IBM character instead.

All other control codes and escape

sequences (see Appendix B) are passed

directly to the printer.

REMOVING (de-installing?) MPM-print

�����������������������������������

Reboot your computer (using the Ctrl-Alt-

Del keys simultaneously), and DON'T run

MPM-print after rebooting.

REINSTALLING MPM-print (or trying to)

�������������������������������������

MPM-print works by taking control of

IBM's ROM BIOS interrupt 17h (Printer

I/O). When you try to re-run MPM-print,

it checks the ROM BIOS interrupt to see

if it has already been installed.  If it

has, MPM-print will notify you of that

fact.

A problem can occur, however, if you

install some other piece of memory-

resident software (such as a print

spooler; please read the following

section) that also takes control of

interrupt 17h AFTER MPM-print has been

installed.  If this is the case, and IF

you try to re-run MPM-print, it will NOT

find a copy of itself when it checks, and

it WILL proceed to install itself again.

�˻�ͻ�˻

����ͼ��� - print                                      Page 8

�

TECHNICAL NOTES    PRINT SPOOLERS

���������������    ��������������

Print spoolers are memory-resident

programs that "reserve" a portion of

memory to buffer information that is sent

to the printer.

Ideally, MPM-print should be installed

BEFORE a print spooler.  This would allow

your print spooler to hold characters as

single bytes, and would let MPM-print

handle sending the special characters to

the printer in graphics mode.

HOWEVER, some print spoolers (notably

AST's SUPERSPL) will ignore the existence

of MPM-print IF they are installed after

MPM-print.  If you are using one of those

spoolers, then please install MPM-print

AFTER installing your print spooler.

COMPATIBILITY

�������������

We have not been able to test MPM-print

with every available piece of software.

We HAVE, however, tested it with:

IBM PC-DOS versions 1.1,2.1,3.0,3.1

PrintGraph/Lotus 1-2-3

Graphics screen dumps (as long as the

DOS utility GRAPHICS.COM is also

installed)

Sidekick (from Borland International)

PC-WINDOW (a user supported, memory-

resident system utility from:

Cannon/Mill Software, PO Box 10047,

Columbia, MO  65205)

The following user supported "quality

printing" systems:

LQ-PRINT (from:

Centar Technology, PO Box 60577,

Sunnyvale, CA  94088)

BRADFORD (from:

Aaron Contorer, 1521 Central Ave.,

Deerfield, IL  60015)

PC-FONT (v 2.04, from:

SJL, 4473 Marlborough, #5

San Diego, CA 92116)

If you have compatibility problems with

other memory-resident programs, the first

thing to try is changing the order in

which the programs are loaded into memory.

If you discover compatibility problems

that you are unable to solve, please let

us know about them.

�˻�ͻ�˻

����ͼ��� - print                                      Page 9

�

���������������������������������������������������������������

�˻�ͻ�˻

MPM-print, ����ͼ��� - print, and its associated documentation

�

are copyrighted (C), 1985, MPM Enterprises, Inc.

Please direct all comments, registrations and contributions

to:

�˻�ͻ�˻

����ͼ��� enterprises, inc.

�

306 Park Avenue

Wilmette, Illinois  60091

���������������������������������������������������������������

�˻�ͻ�˻

����ͼ��� - print                               APPENDIX A-1

�

APPENDIX A

����������

ASCII CODES

�����������

Decimal

� Hexidecimal

�   �   Character or function

�   �   �

0  00           32  20           64  40   @       96  60   `

1  01   #       33  21   !       65  41   A       97  61   a

2  02   #       34  22   "       66  42   B       98  62   b

3  03   #       35  23   #       67  43   C       99  63   c

4  04   #       36  24   $       68  44   D      100  64   d

5  05   #       37  25   %       69  45   E      101  65   e

6  06   #       38  26   &       70  46   F      102  66   f

7  07 BEL       39  27   '       71  47   G      103  67   g

8  08  BS       40  28   (       72  48   H      104  68   h

9  09  HT       41  29   )       73  49   I      105  69   i

10  0A  LF       42  2A   *       74  4A   J      106  6A   j

11  0B  VT       43  2B   +       75  4B   K      107  6B   k

12  0C  FF       44  2C   ,       76  4C   L      108  6C   l

13  0D  CR       45  2D   -       77  4D   M      109  6D   m

14  0E  SO       46  2E   .       78  4E   N      110  6E   n

15  0F  SI       47  2F   /       79  4F   O      111  6F   o

16  10   #       48  30   0       80  50   P      112  70   p

17  11 DC1       49  31   1       81  51   Q      113  71   q

18  12 DC2       50  32   2       82  52   R      114  72   r

19  13 DC3       51  33   3       83  53   S      115  73   s

20  14 DC4       52  34   4       84  54   T      116  74   t

21  15   #       53  35   5       85  55   U      117  75   u

22  16   #       54  36   6       86  56   V      118  76   v

23  17   #       55  37   7       87  57   W      119  77   w

24  18 CAN       56  38   8       88  58   X      120  78   x

25  19   #       57  39   9       89  59   Y      121  79   y

26  1A           58  3A   :       90  5A   Z      122  7A   z

27  1B ESC       59  3B   ;       91  5B   [      123  7B   {

28  1C   #       60  3C   <       92  5C   \      124  7C   |

29  1D   #       61  3D   =       93  5D   ]      125  7D   }

30  1E   #       62  3E   >       94  5E   ^      126  7E   ~

31  1F   #       63  3F   ?       95  5F   _      127  7F   

�˻�ͻ�˻

����ͼ��� - print                               APPENDIX A-2

�

APPENDIX A (continued)

����������������������

ASCII CODES

�����������

Decimal

� Hexidecimal

�   �   Character or function

�   �   �

128  80   �      160  A0   �      192  C0   �      224  E0   �

129  81   �      161  A1   �      193  C1   �      225  E1   �

130  82   �      162  A2   �      194  C2   �      226  E2   �

131  83   �      163  A3   �      195  C3   �      227  E3   �

132  84   �      164  A4   �      196  C4   �      228  E4   �

133  85   �      165  A5   �      197  C5   �      229  E5   �

134  86   �      166  A6   �      198  C6   �      230  E6   �

135  87   �      167  A7   �      199  C7   �      231  E7   �

136  88   �      168  A8   �      200  C8   �      232  E8   �

137  89   �      169  A9   �      201  C9   �      233  E9   �

138  8A   �      170  AA   �      202  CA   �      234  EA   �

139  8B   �      171  AB   �      203  CB   �      235  EB   �

140  8C   �      172  AC   �      204  CC   �      236  EC   �

141  8D   �      173  AD   �      205  CD   �      237  ED   �

142  8E   �      174  AE   �      206  CE   �      238  EE   �

143  8F   �      175  AF   �      207  CF   �      239  EF   �

144  90   �      176  B0   �      208  D0   �      240  F0   �

145  91   �      177  B1   �      209  D1   �      241  F1   �

146  92   �      178  B2   �      210  D2   �      242  F2   �

147  93   �      179  B3   �      211  D3   �      243  F3   �

148  94   �      180  B4   �      212  D4   �      244  F4   �

149  95   �      181  B5   �      213  D5   �      245  F5   �

150  96   �      182  B6   �      214  D6   �      246  F6   �

151  97   �      183  B7   �      215  D7   �      247  F7   �

152  98   �      184  B8   �      216  D8   �      248  F8   �

153  99   �      185  B9   �      217  D9   �      249  F9   �

154  9A   �      186  BA   �      218  DA   �      250  FA   �

155  9B   �      187  BB   �      219  DB   �      251  FB   �

156  9C   �      188  BC   �      220  DC   �      252  FC   �

157  9D   �      189  BD   �      221  DD   �      253  FD   �

158  9E   �      190  BE   �      222  DE   �      254  FE   �

159  9F   �      191  BF   �      223  DF   �      255  FF   �

�˻�ͻ�˻

����ͼ��� - print                               APPENDIX B-1

�

APPENDIX B

����������

EPSON CONTROL CODES AND ESCAPE SEQUENCES

����������������������������������������

EPSON control codes

The following codes, when sent individually to the

appropriate printer, will cause the described action to be

taken.

ASCII     chars

code use after   action

���������������������������������������������������������������

7  fmr   0     bell

8  fmr   0     backspace

9  fmr   0     horizontal tab

10  fmr   0     line feed

11  fmr   0     vertical tab

12  fmr   0     form feed

13  fmr   0     carriage return

14  fmr   0     shift out (expanded mode on)

15  fmr   0     shift in (compressed mode on)

17  f     0     printer in active condition

18  fmr   0     turns compressed mode off

19  f     0     printer inactive until char(17) sent

20  fmr   0     turns expanded mode off

24  f     0     cancels all text in the printer's buffer

27  fmr   -     escape

127  fmr   0     deletes the last character in the printer's

line buffer

NOTES:

use   : designates the printers that make use of the

particular code

f : FX series

m : MX with Graftrax Plus, IBM Graphics printer

r : RX series

chars

after : number of characters the Epson printer expects

AFTER the escape + character sequence is started.

An asterisk (*) means that the number of

characters is context-specific.

�˻�ͻ�˻

����ͼ��� - print                               APPENDIX B-2

�

EPSON escape codes

The escape char (ASCII 27) followed by one of the below ASCII

codes followed by the appropriate number of characters will

cause the described action to be taken.

ASCII     chars

code use after   action

���������������������������������������������������������������

14  f     0     shift out (expanded mode on)

15  f     0     shift in (compressed mode on)

33  f     1     master print mode select (selects 16 unique

print mode combinations)

35  fm    0     accepts 8th bit 'as is' from the computer

37  f     2     activates either ROM or download character set

38  f     *     downloads one or more characters

42  f r   *     turns graphic mode on; followed by three

numbers, m,n1,n2 and n data codes where

n=n1+256*n2

45  fmr   1     turns underline mode on and off

47  f     1     selects a vertical tab channel

48  fmr   0     sets line spacing to 1/8 inch (9 dots)

49  fmr   0     sets line spacing to 7/72 inch (7 dots)

50  fmr   0     sets line spacing to default 1/6 inch

(12 dots)

51  fmr   1     sets line spacing to n/216 inch

(1/216 inches = 1/3 dot)

52  fmr   0     turns the italic character set on

53  fmr   0     turns the italic character set off

54  f     0     enables printing the symbols stored in

ASCII codes 128 - 159

55  f     0     causes ASCII codes 128 - 159 to print as

control codes

56  fmr   0     disables 'paper out' sensor

57  fmr   0     enables 'paper out' sensor

58  f     3     copies character set from ROM to the

download RAM

60  fmr   0     turns unidirectional mode for only one line

61  fm    0     sets the 8th bit to 0 (limits printing to

characters in the range of ASCII 0 - 127)

62  fm    0     sets the 8th bit to 1 (limits printing to

characters in the range of ASCII 128 - 255)

64  fmr   0     master reset code

65  fmr   1     sets line spacing to n/72 inch (n dots)

66  f     *     sets up to 16 vertical tabs, is followed

by n numbers where the last number is less

than its predecessor

�˻�ͻ�˻

����ͼ��� - print                               APPENDIX B-3

�

EPSON escape codes (continued)

ASCII     chars

code use after   action

���������������������������������������������������������������

67  fmr   *     if the next number is nonzero, sets form

length to n lines

if the next number is zero, then takes the

following number and sets the form length

to n inches

68  fm    *     sets up to 32 horizontal tabs, is followed

by n numbers where the last number is less

than its predecessor

69  fmr   0     turns emphasized mode on

70  fmr   0     turns emphasized mode off

71  fmr   0     turns double strike mode on

72  fmr   0     turns double strike mode off

73  f     1     enables/disables printing of non-control

characters stored in ASCII 0 - 31

74  fmr   1     forces an immediate line feed of n/216 inches

75  fmr   *     normal graphics mode, followed by n1 & n2,

and then n data numbers where n=n1+256*n2

76  fmr   *     low-speed double-density graphic mode, same

logic as for code #75

77  f r   0     turns elite mode on

78  fmr   1     sets skip-over-perforation to n lines

79  fmr   0     turns skip-over-perforation off

80  f r   0     turns elite mode off

81  fmr   1     sets the column width  (right margin)

82  f r   1     selects an international character set

83  fmr   1     turns script mode on (to either

super- or subscript)

84  fmr   0     turns script mode off

85  fmr   1     turns unidirectional mode on/off

87  fmr   1     turns expanded mode on/off

89  f r   *     high-speed double-density graphics, same

logic as for code #75

90  f r   *     quadruple density graphics mode; followed by

n1 & n2 and n data numbers where n=n1+256*n2

94  f     *     9-pin graphics mode, followed by d, n1 & n2 and

2n data numbers where n=n1+256*n2

98  f     1     sets vertical tab for channel n

101    r   2     sets horizontal and vertical tab increments

102    r   2     prints spaces or line feeds without carriage

returns

105  f     1     turns immediate mode on/off

106  f     1     causes immediate reverse line feed of n/216 inch

108  f r   1     sets left margin to n characters

109    r   1     allows graphics characters at ASCII 128-159

to print

112  f     1     turns proportional mode off/on

115  f r   1     turns half-speed mode on/off