****************

********************* FLANGES v3.1 ******************************

****************

I. Introduction

FLANGES v3.1 is a pipe flange analysis program developed by

Carlos S. Perez-Ramos, Aerospace Technologist, at NASA Langley

Research Center in Hampton, Virginia. It is partly written in

Microsoft Quick C version 2.5. Parts of the program are written

in 8086 assembly language and compiled with Microsoft Quick

Assembler version 2.5. The implementation of pulldown menus,

popup menus, and dialog boxes comes from Mark Goodwin's "User

Interfaces in C", MIS Press, Portland, OR, 1989. Microsoft mouse

support was added by Carlos Perez-Ramos.

II. Description

This program analyzes circular pipe flanges in accordance with

the rules in the ASME Boiler and Pressure Vessel Code, Section

VIII, Division 1. The following is a list of the kinds of

flanges which this program will handle:

a. Flat face flanges (Non mandatory Appendix Y)

- integral

- loose with hub

- loose without hub

b. Raised face flanges (Mandatory Appendix 2)

- integral

- loose with hub

- loose without hub

c. Blind flanges (Section UG-34)

FLANGES v3.1 supports Class 1 assemblies of flanges, that is,

assemblies with two identical flanges. The analysis of Class 2

assemblies can be performed by assuming that each flange in the

Class 2 assembly is one flange in a Class 1 assembly. This

method MAY require more bolting to be used.

II. System requirements

The minimum system requirements to run this program are:

IBM PC, XT, AT or compatible computer

256 K of available RAM

EGA or VGA graphics card and monitor

Microsoft-compatible mouse (recommended)

****************

********************* FLANGES v3.1 ******************************

****************

III. Pull-Down Menu System

The user interface in FLANGES version 3.1 uses a pulldown menu

system. The easiest way to work with pulldown menus is to use

a pointing device, such as a mouse. With a mouse, you can

point at a menu item and click the left button to activate the

menu. A small window showing all the available options under

that menu will be shown. Once you have pulled down a menu, you

can select any of the menu options by pointing to it with the

mouse pointer and clicking the left button.

If a mouse is not available (the program assumes you have a

Microsoft-compatible mouse), the keyboard can be used to access

the pulldown menus. To pull down a menu with the keyboard,

notice that every menu has a letter that is highlighted. Hold

down the "Alt" key, then tap the key corresponding to the

highlighted letter in the menu. Once the menu is down, notice

that each menu option has a letter highlighted. To select a

particular menu option, simply tap the key corresponding to the

highlighted letter. As an alternative, once a menu has been

pulled down, you can use the cursor arrow keys and move the

highlighted bar to the desired menu option. Once this is done,

hitting the ENTER (or RETURN) key will select the menu item.

To safely back out of ALMOST any menu, simply hit the "Esc" key.

****************

********************* FLANGES v3.1 ******************************

****************

IV. Program Validation

Included with this copy of FLANGES v3.1 is a set of four sample

data files. Three of these data files (jpvt1.flg, jpvt2.flg &

jpvt3.flg) originate from 3 example problems published in the

Journal of Pressure Vessel Technology (JPVT), Volume 101,

February 1979. These three examples are analyses of flat faced

flanges. The fourth sample data file (ansi3900.flg) is an

example of a 3" NPS, 900 pound class, raised face flange with

dimensions in accordance with the ANSI B16.5 standard.

Here is a comparison chart between the results published by the

JPVT and the results obtained with FLANGES v3.1:

------------------------------------------------------

| Longit | Tangen | Radial | Radial | Bolt   | Bolt   |

| @ ID   | @ ID   | @ ID   | @ BC   | Stress | Prest  |

|--------|--------|--------|--------|--------|--------|

JPVT Examp 1 | 12,612 |  3,747 |  2,388 | 18,466 | 19,463 | 17,709 |

-------------|--------|--------|--------|--------|--------|--------|

FLANGES v3.1 | 12,611 |  3,747 |  2,388 | 18,461 | 19,460 | 17,706 |

-------------|--------|--------|--------|--------|--------|--------|

Error (%)    |  0.008 |  0.000 |  0.000 |  0.027 |  0.015 |  0.017 |

--------------------------------------------------------------------

-------------------------------------------------------

| Longit | Tangen | Radial | Radial | Bolt   | Bolt   |

JPVT Examp 2 | @ ID   | @ ID   | @ ID   | @ BC   | Stress | Prest  |

-------------|--------|--------|--------|--------|--------|--------|

FLANGE I     |  6,351 |  1,330 |  6,006 |  8,874 | 17,510 | 13,300 |

-------------|--------|--------|--------|--------|--------|--------|

FLANGES v3.1 |  6,351 |  1,330 |  6,006 |  8,874 | 17,514 | 13,297 |

-------------|--------|--------|--------|--------|--------|--------|

Error (%)    |  0.000 |  0.000 |  0.000 |  0.000 | 0.023  |  0.023 |

--------------------------------------------------------------------

-------------------------------------------------------

| Longit | Tangen | Radial | Radial | Bolt   | Bolt   |

JPVT Examp 2 | @ ID   | @ ID   | @ ID   | @ BC   | Stress | Prest  |

-------------|--------|--------|--------|--------|--------|--------|

FLANGE II    | 19,410 |  5,019 |  8,703 | 11,540 | 35,240 | 32,240 |

-------------|--------|--------|--------|--------|--------|--------|

FLANGES v3.1 | 19,415 |  5,019 |  8,703 | 11,538 | 35,240 | 32,621 |

-------------|--------|--------|--------|--------|--------|--------|

Error (%)    |  0.025 |  0.000 |  0.000 |  0.017 |  0.000 |  1.182 |

--------------------------------------------------------------------

****************

********************* FLANGES v3.1 ******************************

���������****************

The raised face flange routine in FLANGES v3.1 has not been

validated with results published in the technical literature.

Nevertheless, it has been used many times at NASA-Langley

in the successful design of flanged joints in the six years

since the creation of version 1.0 of the program. So far,

so good. (Results are always hand-checked).

V. Menu Reference

FILE MENU

FILE - New

----------

The File-New menu initiates a new flange analysis. The user

is presented with all the data entry screens required for a

flange analysis.

FILE - Open

-----------

The File-Open menu presents a requester window where the

user may choose to load an already saved data file. FLANGES

v3.1 can store a flange's description to a file for later

recall. Earlier versions of the program required the flange

data file to use a ".flg" extension in its name, e.g.,

"myflange.flg". This requirement has been dropped in FLANGES

v3.1. Nevertheless, the program will still check whether the

selected file is a valid flange data file.

FILE - Save

-----------

The File-Save menu saves the current flange data to a disk

file. The currently defined file name is used by default. If

no file name has been defined, the program will call the

"FILE - Save As" menu option.

FILE - Save As

--------------

The File-Save_As menu performs the same function as the

File-Save menu except that it always requests a file name.

Thus, it is primarily intended to be used to save the data

to a different file name than the one currently defined.

FILE - Print / View Report

--------------------------

The File-Print/View_Report menu allows the user to print a

comprehensive report of the flange analysis to any of the

devices supported by FLANGES v3.1. The default output device

is LPT1:. Refer to the "Change printer" menu for information

****************

********************* FLANGES v3.1 ******************************

****************

FILE - Print / View Report (CONT'D)

--------------------------

on the available devices. The output includes a summary of all

the input data plus a summary of calculated loads, moments,

design factors, as well as stresses and ASME B&PV Code

allowables.

FILE - Change Output Device

---------------------------

The File-Change_Printer menu routes the program's output to

any one of the devices supported, i.e., LPT1:, DISK FILE, or

SCREEN.

FILE - Exit

-----------

The File-Exit menu is self explanatory (I hope). A dialog

menu will appear to confirm that the user really intends

to exit the program.

FILE - About

------------

The File-About menu shows the program's credits.

FLANGE TYPE MENU

The flange_Type menu allows the user to switch from one type

of flange configuration to another (e.g., flat face, raised

face, & blind). This menu is useful in evaluating the

relative performance of different types of flanges having

the same basic dimensions.

CATEGORY MENU

The Category menu allows the selection of one of three types

of flange categories, i.e., integral flange, loose flange

with a hub, and loose flange without a hub.

RUN MENU

RUN - Analyze flange

--------------------

The Run-Analyze_flange menu will perform a flange analysis

based on the current geometry information in memory. Prior

to performing an analysis, the program scans the data for

possible errors. If any errors are found, a warning message

is displayed. If no errors are detected, the analysis is

performed and the program asks the user whether the results

should be displayed on the screen.

****************

********************* FLANGES v3.1 ******************************

****************

RUN - Preview flange

--------------------

The Run-Preview_flange menu draws a flange using the data

currently in memory. Prior to this, the program scans the

data for possible errors. Whether or not the program detects

errors in the flange data, it will make an attempt to draw

it. This allows visualizing some common dimensional errors.

RUN - Help

----------

The Run-Help menu displays help information on the screen.

The file "flanges.hlp" must be in the current directory

for this menu to work.

MODIFY MENU

MODIFY - Bolts

------------------

The Modify-Bolt_data menu allows the user to input data

relating to the bolts in the flange, such as bolt diameter,

stress area, bolt length, etcetera.

MODIFY - Flange dimensions

--------------------------

The Modify-Flange_dimensions menu allows the input of flange

dimensions, such as OD, ID, hub length, flange thickness,

etc.

MODIFY - Gasket

---------------

The Modify-Gasket menu allows the input of gasket data,

such as gasket dimensions and design parameters.

MODIFY - Material Props

-----------------------

The Modify-Material_props menu allows the input of material

data, such as modulus of elasticity and ASME B & PV Code

allowable stresses.

MODIFY - Working Pressure

-------------------------

The Modify-Working_pressure menu allows the input of the

hydrostatic pressure load acting on the flange.

****************

********************* FLANGES v3.1 ******************************

****************

MODIFY - Add External Moment

----------------------------

The Modify-External_moment menu allows the user to enter a

value for external moment acting on the flange. This value

is added directly to the total moment due to the hydrostatic

pressure loads and the gasket seating load, i.e.,

M = Mt + Md + Mg + Me;

|    |    |    |

|    |    |    +-- external moment

|    |    |

|    |    +-- gasket reaction moment

|    |

|    +-- hydrostatic end force moment

|

+-- difference between Md and Mh

(Mh is the total moment produced by the hydrostatic force

acting on the flange up to the mean gasket diameter.)

****************

********************* FLANGES v3.1 ******************************

****************

VI. Program History

Version 1.0 - Circa early 1984.

This version of FLANGES was written in GW-BASIC. It was

limited to the analysis of flat-face flanges.

Version 2.0 - Early 1988.

This version was a complete rewrite of the source code with

the exception of the subroutine to calculate the flange

design factors f, F, and V. Version 2.0 was totally written

in Quick BASIC 4.0. For the first time, the program had

acquired a "user friendly" interface (that is, according to

me, of course). Raised face flange analysis was added in

version 2.0. In addition, the ability to modify the design

parameters if the calculated flange stresses exceeded the

code allowables was added.

Version 3.0 - Early 1990.

Upon purchasing Mark Goodwin's "User Interfaces in C", I

decided it was time to give FLANGES a prettier look and an

even friendlier interface (again, this is according to me).

The program code was rewritten mostly in Quick C, with some

parts (like the screen handling and mouse support routines)

written in 8086 assembly language. The user interface now

includes pulldown menus, dialog boxes, and better designed

data entry screens. In addition, the program can now save

the flange data to disk, to avoid having to re-enter all the

data every time the program is run. With version 3.0  the

user can preview the flange geometry on the screen.

Version 3.1 - December 1990

This is a "minor" revision of the program, I guess. New

things include improved print reports (screen and printer

output are now identical in version 3.1), and close up

preview of the flange cross-section. I played around with

the pulldown menus a little bit, got rid of the Characterize

menu, splitting it into two new menus, the "flange Type"

menu and the "Category" menu. The help function has been

improved somewhat.

Version 4.0 - ???? 1991

PostScript support, LaserJet support, and... who knows?

(I thought I should throw in a little vaporware just like

the big software developers do!).