FMSIM

FM Stereo Simulation and Analysis Program

Demonstration Version

Version 1.2

Copyright (c) 1990 - 1991

All Rights Reserved

by

QUANTICS

P. O. Box 2163

Nevada City, California 95959-2163

Developed in cooperation with:

BROADCAST ELECTRONICS

4100 North 24th Street

P. O. Box 3606

Quincy, Illinois  62305-3606

(217) 224-9600

FMSIM

FM Stereo Simulation and Analysis Program

System requirements:

MS-DOS 2.0 or better

384k RAM

CGA, EGA, or VGA color graphics or AT&T 640x400 graphics

Optional (for hard copy):

EPSON MX, FX or compatible dot matrix printer

8087, 80287, or 80387 math coprocessor recommended

I. INTRODUCTION

FMSIM  is an analysis tool for those working with FM  stereo

transmission  and  reception  systems.  FMSIM  calculates  signal

degradation  due  to interference,  signal  filtering,  nonlinear

processing, and multipath propagation. It may be used to  predict

performance  of  many  system components  such  as  transmitters,

multiplexers, antennas, and receiver IF bandpass filters.

It  is an incorrect assumption that FM is  almost  perfectly

"bulletproof" as a transmission mode. If this were the case  then

there would no such thing as multipath distortion, familiar to FM

car radio listeners in hilly areas. The popular press points  out

that  FM  enjoys  the  capture  effect;  when  two  FM   stations

interfere,  only the stronger signal's modulation will be  heard.

Also  commonly  assumed  is  that  FM  is  immune  to   amplitude

variations  because  they  are removed prior to  detection  by  a

limiter.   These   common   assumptions   are   only   approached

asymptotically  as  the  interference  becomes  small  and   more

importantly, as the modulation index becomes large. (In the  case

of  FM  stereo,  modulation index is small  for  the  L-R  stereo

difference signal).

In  the  case  of FM capture, the  interfering  signal  will

create a resultant phase modulation product at the  instantaneous

beat  note frequency. Since PM and FM are the same  thing,  there

will always be a detected interference component. The question is

whether it is significant.

Similarly, interference (e. g. impulse noise) which  creates

amplitude variations will simultaneously produce phase modulation

interference which will be detected as FM along with the  desired

modulation.

FMSIM's  multipath  and interference models  illustrate  and

quantify these distortions.

Another  popular  misconception  is  that  an  FM   signal's

bandwidth is simply twice the deviation. However, analysis  shows

that  an  FM  signal actually occupies  infinite  bandwidth.  The

pertinent  question is, "how many FM sidebands are  significant?"

Carson's  rule  (a  rule of thumb) states  that  an  FM  system's

bandwidth  is  equal to twice the sum of the deviation  plus  the

maximum  modulating  frequency. FMSIM gives  much  more  accurate

information  on  FM  system  bandwidth, and  on  the  effects  of

spectrum truncation.

II. DEMO VERSION DESCRIPTION and INSTALLATION

To  install the demonstration version of FMSIM, simply  copy

the files contained in the archive file (FMDEMO.ZIP) to your hard

disk  or  working  floppy. It is recommended that  you  create  a

directory  on  your hard disk called "FMSIM" and copy  the  files

into  it. The demonstration version of FMSIM, called  FMDEMO.EXE,

will use an 80x87 math coprocessor if one is found; otherwise  it

will emulate the math coprocessor.

Note:  the demonstration version of FMSIM (FMDEMO)  performs

only  some of the calculations described; timing  loops  simulate

normal  program operation in some areas. Although FMSIM  executes

approximately as fast as the demonstration version depicts, there

will  be some variation from computer to computer.  Estimates  of

execution speed should not be based on the demonstration  version

of FMSIM.

When  you  invoke FMDEMO, you will see  an  opening  screen.

Entering <CR> will begin the "auto-pilot" entry of FM  modulation

parameters.  The  demo  version of FMSIM is limited  to  the  one

particular  set  of modulation conditions entered by  the  "auto-

pilot." However, the CGA/EGA/VGA/AT&T graphics analysis modes and

screen dumps operate the same way that they do in the  production

vrsion.  You  can  display and obtain hard copy  of  any  of  the

analyses from FMDEMO.

The production version of FMSIM is available from:

QUANTICS

P. O. Box 2163

Nevada City, California 95959-2163

-or-

BROADCAST ELECTRONICS

4100 North 24th Street

P. O. Box 3606

Quincy, Illinois  62305-3606

(217) 224-9600

III. PROGRAM DESCRIPTION

FMSIM   simulates   FM  stereo   modulation,   interference,

nonlinear effects, filtering, transmission, and demodulation.  It

is  useful for qualitatively and quantitatively  determining  the

effects  of  bandpass  filters,  multipath  transmission,   tuned

amplifiers,  multiplexers, etc. on FM stereo signals,  both  with

and  without SCA subcarriers. Although many reference  texts  and

some  computer software analyze single tone and multiple tone  FM

bandlimited   systems,  such  simple  cases  cannot   easily   or

accurately  be  extrapolated  to analyze the  performance  of  FM

stereo  broadcast  systems.  This program  accurately  models  FM

broadcast   systems,  and  evaluates  system  performance   using

standard   and   relevant   measurements:   stereo    separation,

distortion, crosstalk, etc.

FMSIM models an FM stereo (or mono) signal generator with or

without SCA(s), a filtering process, and an FM demodulator  which

is   followed  by  de-emphasis,  stereo  demodulation   and   SCA

demodulation if needed.

The  program  operates in the time  and  frequency  domains,

using  a complex 1024 point FFT to transform between the two.  FM

modulation and demodulation are performed at a carrier  frequency

of  zero,  using a complex time domain  representation  to  allow

unequal  positive  and  negative  frequency  components.  A  zero

carrier frequency increases numerical accuracy and greatly speeds

execution  time.  Although  zero  carrier  frequency  cannot   be

accomplished in the real world, it can be shown to be  equivalent

to  using  a nonzero carrier frequency by means of  the  shifting

theorem.

Certain   parameters  are  restricted  to  integer   values.

Modulation  frequencies  and  SCA  carrier  frequencies  must  be

specified  as an integer number of kHz. This  restriction  allows

the  highest  simulation accuracy to be maintained while  at  the

same   time  reducing  computation  time.   Although   modulation

frequencies  must  be  at  least  1 kHz,  it  is  at  the  higher

frequencies  where  the distortion effects of interest  are  most

severe.   Bessel  nulls  still  can  be  verified  by   adjusting

modulation levels rather than modulating frequency.

To run the program, enter FMSIM at the DOS prompt and follow

the  menus. You will be asked for a set of modulation  conditions

(mono,  stereo,  modulating  frequencies  and  percentages,   SCA

parameters,  etc.).

FMSIM   can   also  simulate  the  effects   of   co-channel

interference;  you  will  be  given  the  option  of  adding   an

interfering  signal  (specified  by an  external  file).  If  you

specify  an interfering signal, it will be added to  the  desired

signal  before any bandpass filtering or multipath  operation  is

performed.  In other words, the filtering affects the sum of  the

desired and interfering signals.

The  interfering  signal is read in from a  file  containing

frequency domain (spectrum) data. The file format is the same  as

the  FM  spectrum  output file from FMSIM.  The  easiest  way  to

generate an interfering signal file is to use FMSIM to generate a

signal  file  (SPECTRUM.FM).  Rename  the  file  to  some   other

convenient file name (e. g. SPECTRUM.QRM) then restart FMSIM  and

when it asks for the file containing the interfering signal, give

it  the file name you created previously. You must either  rename

the  file SPECTRUM.FM or use the /E or /NF switch on the  command

line,  otherwise FMSIM will overwrite the file  SPECTRUM.FM  with

the desired spectrum file.

If you need to create your own filter file, the file  format

for the interference data is as follows: the data format for  the

interference  consists of 1024 lines of data, from -512 kHz  from

carrier  to  +511 kHz  from  carrier.  Each  line  contains   the

frequency offset from carrier in kHz (as a check for errors), the

relative magnitude, and the phase in degrees. The three values on

each  line may be separated by spaces or commas. For example,  an

interference file might look like this:

;Transmitted RF (kHz from carrier; amplitude; phase (degrees)

-512     .465661E-08   -36.8699

-511     .161729E-07  -112.8737

-510     .186265E-07    90.0000

.

.

.

-3     .745507E-01  -121.3878

-2     .142619      -167.4025

-1     .413281E-01   127.3817

0     .326305E-01    48.0013

1     .652068E-01   -94.4402

2     .961138E-01  -173.4791

3     .606690E-01    33.6253

.

.

.

509     .217220E-07    30.9638

510     .232831E-07   -90.0000

511     .171727E-07  -139.3987

All  1024  frequencies  must be  specified;  otherwise  an  error

message will be generated. Comments may be inserted at any  point

in  the file. A comment line has a semicolon (;) in  column  one,

and must not have a numerical value at the end of the line unless

it is immediately preceded or followed by non-numeric characters.

Next  is  the filter specification. You may either  use  the

internal filter design program, which produces transfer functions

for    Butterworth,   Chebychev,   elliptic   (Cauer),    Bessel,

synchronously  tuned, or ideal filters, or you can  specify  your

own filter transfer function, to be read in from a file by FMSIM.

For example, your externally specified transfer function might be

taken  from  SPICE  analysis  or  empirically  measured   network

analyzer data of a multiplexer, an antenna system, a  transmitter

cavity, or a receiver IF filter.

FMSIM  also  includes  a  multipath  model  with  up  to  64

reflections.  Each reflection is characterized by a dB  magnitude

value (usually negative), a time delay value, and a carrier phase

value, all with respect to the direct path.

The ideal filter allows you to compare distortions resulting

from ideal (linear phase) sideband truncation versus the  effects

of realizable filters with group delay variations.

The  effects of group delay and amplitude may be  separated.

For  the purposes of analysis, it is possible to simulate  either

just  the  amplitude  characteristic  or  just  the  group  delay

characteristic  of  a  filter, either  internally  or  externally

specified.  Although such a transfer function is not  realizable,

this  capability  provides the user with  insight  to  understand

which  filter characteristic is causing the greatest  distortion,

spectrum truncation or nonlinear phase. (As FMSIM will verify for

you, in most cases it will be the nonlinear phase  characteristic

which causes the greatest distortions.)

The  data format for the externally specified filter is  the

same  as the format for the interfering signal  described  above.

The  filter  file consists of 1024 lines of data,  from  -512 kHz

from  carrier  to +511 kHz from carrier. Each line  contains  the

frequency offset from carrier in kHz (as a check for errors), the

relative magnitude, and the phase in degrees. The three values on

each  line may be separated by spaces or commas. For  example,  a

filter file might look like this:

;COMMENT - this is a sample filter file

-512     .496112E-01  -133.4725

-511     .500874E-01  -133.5731

-510     .505680E-01  -133.6742

.

.

.

-3     .988528         1.4043

-2     .988542          .9362

-1     .988550          .4681

0     .988553          .0000

1     .988550         -.4681

2     .988542         -.9362

3     .988528        -1.4043

.

.

.

509     .510530E-01   133.7758

510     .505680E-01   133.6742

511     .500874E-01   133.5731

All  1024 frequencies must be specified. Rather than  have  FMSIM

guess on how to best interpolate the data, that judgment call  is

left to the user. (You may wish to write your own simple  program

to  interpolate  your  data  if  you  only  have  a  few   points

characterized.)  Comments  may be inserted at any  point  in  the

file. A comment line has a semicolon (;) in column one, and  must

not  have a numerical value at the end of the line unless  it  is

immediately preceded or followed by non-numeric characters.

If you have FMSIM design a filter for you, it will write the

transfer  function to a file (FILTER.FM) in the format  specified

above. This file can also serve as an example of how to specify a

filter transfer function for FMSIM, or it can be used as data  to

be read by another program.

There  are  two  ways to calculate  the  effect  of  several

cascaded filters. You can either create a filter file or use NFDL

(Nonlinear Filter Description Language, specified later).

To create a filter file representing the cascade of  several

filters,  use  the  program  FILTMULT. You  can  either  use  the

compiled  .EXE version or the .BAS version which will  run  under

most  BASIC interpreters and compilers. FILTMULT  multiplies  two

filter  transfer functions from two files stored with the  format

described  above, and writes to a third file in the same  format.

To obtain the input files, you can run FMSIM to design a  filter;

you will then have to rename at least one of the files (or  store

it  on  a  different  drive/directory) so that  it  will  not  be

overwritten by FMSIM.

Nonlinear filtering operations are simulated through the use

of  Nonlinear  Filter Description Language  (NFDL).  NFDL  allows

modeling of cascaded linear and nonlinear functions. For example,

if  you want to simulate a tuned class C amplifier, it  could  be

modeled as the cascade of three functions: a singly tuned  filter

(grid or cathode circuit), a limiting device, and a second  order

bandpass filter (doubly tuned output matching network).

NFDL  includes  the  following  nonlinear  functions:   hard

limiting, clipping, AGC, simple gain blocks, and piecewise linear

amplitude  transfer  functions. In addition, all  of  the  linear

filtering   functions  are  available:  Butterworth,   Chebychev,

elliptic,   Bessel,  synchronously  tuned,  and  ideal   bandpass

filters, multipath propagation, and filters or arbitrary transfer

functions  specified  by external files. A  description  of  NFDL

syntax and capabilities is given in a separate section  following

the Reference section of this manual.

After  FMSIM completes simulation of modulation,  filtering,

and   demodulation,  an  ASCII  numerical  file  containing   the

simulation  results  is  written  to the  default  drive  and  an

analysis menu is presented. Do not overlook the file ANALYSIS.FM;

it  contains  most  of  the  important  "bottom  line"  numerical

analysis   results.  The  analysis  menu  allows  the   user   to

graphically  look at any of a variety of the signals produced  by

the  simulation.  For example, the transmitted RF  spectrum,  the

filter  bandpass characteristic, or the left channel time  domain

response can be viewed.

The  analysis is viewed on an oscilloscope-like screen.  The

function  keys  control  vertical and  horizontal  position,  and

vertical and horizontal gain in a 1-2-5 sequence. In addition,  a

screen  graphics  dump  can be sent to a printer  in  Epson  (FX)

format when hard copy is required.

Frequency   domain  signals  are  presented  as   either   a

continuous curve which connects all spectral data points, or as a

bargraph  whenever the number of spectral lines is less than  the

available horizontal pixels.

For  time domain signals, the graphical display uses  spline

interpolation whenever the number of horizontal pixels  available

exceeds the number of data points to be plotted. This produces  a

visually smooth display rather than the piecewise linear  display

that would be obtained with linear interpolation. When the number

of  plotted points exceeds the number of pixels, the  outline  of

the  oscilloscope  display is blue (on  EGA/VGA  displays).  When

spline  interpolation  is  enabled, the oscilloscope  outline  is

green (on EGA/VGA displays).

When  the  number of pixels is greater than  the  number  of

points  plotted by at least a factor of four, then  the  original

data  points  are plotted in a different color. Function  key  F9

toggles the original data points on and off.

IV. PROGRAM REFERENCE

The command line option switches for FMSIM are:

/ATT or /AT&T - use AT&T graphics (640x400)

/FX           - use Epson FX graphics (default is MX)

/50           - use 50 microsecond de-emphasis

/NF           - suppress output files except ANALYSIS.FM

/Exxx         - use "xxx" as output file extension

/H or /?      - help (this info)

FMSIM is case insensitive to command line arguments. Command line

options  may be specified with the forward-slash ("/")  character

as shown above, the hyphen ("-"), or with no delimiter other than

a  space.  Multiple command line arguments may  be  specified  if

separated  by spaces. I. e. to specify Epson FX graphics with  50

microsecond  de-emphasis,  FMSIM  may be invoked in  any  of  the

following ways:

FMSIM /FX /50

fmsim -fx 50

FmSiM -50 fX

FMSIM 50 /FX

fmsim fx 50

etc...

Entering  /ATT  will  invoke  the  high  resolution  640x400

monochrome  graphics mode of AT&T computers. You do not  need  to

use  this switch if you have an AT&T computer; without  the  /ATT

switch,  CGA  graphics will be invoked (unless  the  computer  is

equipped with EGA or VGA).

The  default  hard  copy  output  mode  is  Epson  MX  mode.

Depending  on the screen resolution, Epson MX 480 horizontal  dot

mode is selected for CGA resolution and 960 dot (double  density)

is  selected for EGA, VGA, and AT&T modes. Since there  are  more

dots  available  than  horizontal  pixels,  the  screen  dump  is

centered but does not occupy the entire page width.

Epson  FX  printers  (FX85, FX286, etc.)  or  FX  compatible

printers have more graphics modes available and have near  letter

quality  ("NLQ") text capability. FMSIM uses NLQ for  text  which

appears  with screen dump hard copy, and the "CRT I"  mode  which

uses  the entire paper width for 640 pixel horizontal  resolution

(EGA,  VGA,  and  AT&T  modes). The  Epson  FX  mode  switch  is,

predictably, "/FX."

For FM analysis applicable to countries using 50 microsecond

pre-emphasis and de-emphasis (e. g. Europe), use the /50 switch.

If  you find yourself regularly invoking one or more of  the

command  line options, you might want to create a batch  file  to

invoke  FMSIM. For example if you use an AT&T computer  with  its

640x400  graphics and an Epson FX85 printer, rather  than  typing

"FMSIM  /ATT  /FX" every time, you could instead create  a  batch

file  called "FM.BAT" or some other convenient name  which  would

contain the single line:

FMSIM /ATT /FX

/NF (for No Files) suppresses all of the output files except

for  ANALYSIS.FM. This speeds program execution when you  do  not

need the output files, especially if you are using a floppy disk.

Since the file ANALYSIS.FM contains important numerical  results,

its creation is not suppressed by this switch.

/Exxx  substitutes  a different filename extension  for  the

output  files. The default is ".FM" but it can be changed to  any

zero  to three character string by using the /Exxx  switch.  This

switch may be used when you do not want to overwrite earlier  .FM

files,  or  if  you want to create output  files  with  a  unique

extension  that will not be overwritten with default  .FM  files.

For example, to make the output extension ".DAT" you would invoke

FMSIM  with the switch /EDAT. To suppress the extension  entirely

(null string extension), you can use the switch /E (with  nothing

following).   Anything  beyond  three  characters   is   ignored,

consistent with the MS-DOS file name convention.

FMSIM produces several output files for each run. These are:

MODULATE.FM    Generated  time  domain  composite  baseband.  The

units are microseconds and normalized  modulation.

+1.0   represents   +100%  modulation   and   -1.0

corresponds to -100% modulation.

SPECTRUM.FM    Transmitted  RF spectrum. The units are  kHz  from

carrier   (-512  to  +511),   absolute   amplitude

relative  to  an unmodulated  carrier  (1.0),  and

phase in degrees of each sideband.

FILTER  .FM    Filter  transfer function. The units are kHz  from

carrier  (-512 to +511), absolute  amplitude,  and

phase shift in degrees for each 1 kHz point.

COMPSPEC.FM    Detected composite spectrum from DC to 200 kHz  in

1 kHz increments, specified in dB.

DETECTED.FM    Detected time domain composite baseband. The units

are  microseconds and normalized modulation.  +1.0

represents  +100% modulation and -1.0  corresponds

to -100% modulation.

ANALYSIS.FM    A  comprehensive  analysis of the  FM  simulation.

Depending  on  the  modulation  conditions  (mono,

stereo,   and   number  of  SCAs)   an   extensive

evaluation of the simulation is presented.  Stereo

separation,   THD  values,   nonlinear   crosstalk

analysis,  incidental  AM,  recovered  pilot   and

subcarrier injection levels, and detected  spectra

before and after de-emphasis are included.

At the opening menu, the options are:

S - simulate stereo

M - simulate mono

F - design filter prior to FM simulation

Q - quit

FMSIM  will respond to any other (invalid) reply with  (1) a

list of valid command line options, and (2) the serial number  of

FMSIM.

When  "S" is specified (stereo) FMSIM will ask for left  and

right  channel  frequencies and modulation  percentages.  If  you

specify  0 for a modulation frequency FMSIM will assume you  want

that  channel  unmodulated  and will not  ask  for  a  modulation

percentage. FMSIM also asks for pilot injection in case you  want

to simulate at something other than the standard 9% injection. If

you  enter a zero, FMSIM assumes you want 9%. If you really  want

0% pilot (for instance, to check baseline flatness), enter a very

small value such as 0.000001%.

Similarly,  if  mono  is  specified,  FMSIM  will  ask   for

modulation frequency and percentage modulation. As in the  stereo

case,  entering "0" for modulation frequency will be  interpreted

as an unmodulated channel and FMSIM will not ask for a modulation

percentage.

"F"  for filter design allows you to experiment with  filter

design,  "tweaking"  the  response,  changing  filter  type   and

bandwidth, etc., before proceeding with the FM simulation itself.

If  you  do  not specify "F" first, you will be  asked  later  to

specify your filter.

"Q" (quit) permits a convenient exit point from the program.

It is possible to exit from the program at any of the prompts  by

hitting a <control-C>.

Incorrect or out-of-range entries at some points may  invoke

a   reference   to  Charles  Babbage   (1792-1871),   a   British

mathematician and designer of large mechanical computing systems,

most of which were impractical. Attempts at building anything but

Babbage's  most modest computing engines faltered and were  never

completed.  Some  of the partially completed  Babbage  mechanical

computer  assemblies  are  on display at the  Science  Museum  in

London.

FMSIM's  filter  design facility calculates  pole  locations

and/or polynomials for the following filter types:

Butterworth (maximally flat)

Chebychev (equiripple)

Elliptic (Cauer)

Bessel (maximally flat delay)

Ideal (not realizable)

Multipath (up to 64 reflections)

In   addition,   FMSIM   accepts   arbitrary   filter    transfer

characteristics which are read in from an ASCII file.

FMSIM includes the capability to modify any of these filters

(except the ideal filter) to set either the magnitude response to

unity gain and evaluate the group delay function only, or to  set

the group delay to zero and evaluate the magnitude function only.

Although  the resulting filters are not realizable,  this  allows

the user to gain an understanding of how much system  degradation

comes from sideband truncation (filter magnitude response) versus

sideband differential delay (filter group delay function).  While

insight  can be gained using this capability, there is a  caveat:

do  not assume that superposition applies; e. g.  the  distortion

caused  by the complete filter in general will not be the sum  of

the  distortion from group delay alone plus the  distortion  from

magnitude response alone. FM is a nonlinear (exponential)  system

and superposition does not apply.

As   FMSIM  runs,  it  gives  you  a  play-by-play   running

commentary on what it is doing. Messages will appear such as:

Generating stereo baseband

Modulating FM carrier

Filtering RF signal

Demodulating main FM carrier

Performing FFT on received baseband and computing dB

etc...

If  you  have  a slow computer and/or no  math  coprocessor  this

information will assure you that the program is actually  running

and  your computer is not "crashed." In any event,  the  messages

may satisfy your curiosity as to just what the program is doing.

After  the number crunching is complete, FMSIM presents  you

with  an  analysis menu which looks like  this  (with  variations

depending on stereo/mono, SCAs, etc.):

ANALYSIS MENU:

0 - exit; QUIT PROGRAM

1 - view transmit baseband time domain data

2 - view transmit baseband spectrum data

3 - view transmit RF spectrum data

4 - view filter dB amplitude characteristic

5 - view filter group delay characteristic

6 - view received RF spectrum data

7 - view detected baseband time domain data

8 - view detected baseband spectrum data

9 - view detected left channel time domain data

10 - view detected left channel spectrum data

11 - view detected right channel time domain data

12 - view detected right channel spectrum data

13 - view detected SCA channel time domain data

14 - view detected SCA channel spectrum data

15 - RESTART PROGRAM with same modulation conditions

16 - DISCARD ANALYSIS; RESTART PROGRAM

17 - DOS shell

Your selection: >

Simply  enter the number corresponding to the analysis  you  want

followed  by  a carriage return. The selections are  arranged  in

sequential order, starting with signal generation and ending with

demodulated outputs.

For  some of the selections (demodulated audio signals)  you

will  be asked whether you want to view the output flat ("F")  or

de-emphasized  ("D").  Whatever you enter (a "F" or a  "D")  will

become  the  default answer the next time you  view  the  current

analysis.  So  if  for example you are  only  interested  in  de-

emphasized  analysis, you can enter a <CR> instead of "D" at  the

prompt. The default at the beginning is de-emphasis ("D"), and is

reset to this same default at the beginning of each analysis.

After you make your selection, you will be presented with  a

graphics screen which "drives" like an oscilloscope (time  domain

analyses) or spectrum analyzer (frequency domain analyses). Using

the  function  keys  you can zoom in or  out,  vertically  and/or

horizontally.

There  are  three basic display modes to be aware  of.  They

are:

1. Linear interpolation, continuous curve

2. Spline interpolation, continuous curve

3. Bargraph

If  the number of points to be plotted is greater  than  the

number of available horizontal pixels, all points are plotted and

connected  with straight lines. This mode is used for  both  time

domain and frequency domain displays.

If  the  number  of points to be plotted is  less  than  the

number   of  available  horizontal  pixels,  then  cubic   spline

interpolation  (rather than straight line interpolation) is  used

to draw a smooth curve through all points. If the density of data

points is less than 25% of the available horizontal pixels,  then

a  small  square box is drawn at each data point.  This  mode  is

intended  for  use with time domain signals, but it can  also  be

selected for frequency domain signals.

Bargraph mode is used for frequency domain signals when  the

number  of  points  to  be plotted is less  than  the  number  of

available  horizontal pixels. It is possible to toggle to  spline

mode and back to bargraph mode by hitting "B".

When spline interpolation is enabled, the outside border  of

the  graticule  is the same color as the rest of  the  graticule.

When spline interpolation is not being used, the graticule border

is  a  different  color from the rest of  the  graticule.  (Color

comments  are not applicable to AT&T 640x400 mode.) In  addition,

the  "O:Overshoot" toggle option shows up on the  graphics  menu.

(This  is  the way to tell if interpolation is  straight-line  or

spline in AT&T 640x400 mode.)

While  the spline curve produces a visually pleasing  smooth

curve,  it  does this at the expense of overshoot.  Overshoot  on

frequency domain plots can be misleading if you do not know where

the original data points are. For example, if you are  modulating

in  stereo  with  a  10 kHz tone in left  and  11 kHz  in  right,

composite L-R sidebands will appear at 27, 28, 48, and 49 kHz. As

the  spline  curve  draws  through  these  immediately   adjacent

sidebands,  it  will  overshoot  to  false  maxima  at  27.5  and

48.5 kHz. Although a real-world spectrum analyzer may do the same

thing if set for bandwidth too wide to resolve the 1 kHz sideband

spacing,  the  results  can  be  misinterpreted.  To  avoid  this

problem, for spectrum analysis FMSIM plots overshoot intervals in

a  different  color and/or intensity than the  spline  curve.  In

intervals where there is overshoot, you will see two curves:  the

smooth  spline, and a straight-line linear interpolation  segment

which  will be "not so smooth" (discontinuous  derivatives).  The

straight line segments can be toggled off and on by pressing  "o"

(the letter, not the zero) for "overshoot."

The  file  "ANALYSIS.FM" contains most  of  the  bottom-line

numerical data. It includes tabulated demodulated audio  spectrum

data,  both  before  and after de-emphasis. In  addition  to  the

conventional  distortion,  separation, and  subcarrier  injection

levels etc., an unconventional value may be calculated  depending

on  the  modulation  conditions: "distortion in L due  to  R"  or

"distortion  in R due to L." If one channel is unmodulated,  then

all  of  the nonlinear crosstalk terms  are  root-sum-squared  to

calculate  a  cross-distortion  value. For example,  if  left  is

modulated  at  1 kHz  and  right  is  unmodulated,  then  the  or

"distortion  in R due to L" is taken as 100% times the  root  sum

square  (rss)  right  channel data from  2  to  15 kHz  inclusive

divided  by  the root sum square of all left  channel  terms.  In

other words, it is like a conventional rms THD calculation except

that the distortion terms all come from the unmodulated channel.

V. Nonlinear Filter Description Language (NFDL)

NFDL is a simple script file which allows modeling of  mixed

cascaded  linear and nonlinear transfer functions. NFDL  is  line

oriented  with three types of lines:  (1) comments,  (2) function

specifications,  and (3) parameters (numerical values).  Comments

are lines which begin with a semicolon. Comments are only allowed

to appear directly before specification statements in NFDL files.

Comments may not appear immediately before or amongst parameters.

Specification  statements  are ASCII strings  which  specify  the

function  to be performed, e. g. Butterworth bandpass  filtering,

clipping, etc. Parameter statements contain the numerical  values

pertaining  to  the specified function, e. g.  filter  bandwidth,

filter offset, clipping level, piecewise linear data points, etc.

Specification  statements are case insensitive.  Upper  case

and lower case may be used interchangeably.

Parameter statements are free form; the values on each  line

may be separated by commas or spaces. Values may be expressed  as

integers, floating point, or in exponential notation.

An example of a comment is:

; doubly tuned output circuit

An example of a specification statement is:

BUTTERWORTH

An example of a parameter statement is:

2,800,1.5

The  functions included in NFDL and the correct  syntax  for

each specification are as follows:

1. Butterworth Bandpass Filter

The specification string must contain the substring "BUTTER"

to   be  recognized.  The  parameters  are:

(1) filter order,

(2) bandwidth in kHz, and

(3) offset in kHz.

Example:

BUTTERWORTH

7,4E3,25

This specifies a seventh order Butterworth bandpass filter with a

bandwidth of 400 kHz offset 25 kHz from carrier.

2. Chebychev Bandpass Filter

The  specification string must contain the substring  "CHEB"

or "EQUIRIPPLE" to be recognized. The parameters are:

(1) filter order,

(2) filter passband ripple in dB

(3) bandwidth in kHz, and

(4) offset in kHz.

Example:

; equiripple filter

CHEBYCHEV

4,0.1,500,0

This  specifies a fourth order 0.1 dB ripple  Chebychev  bandpass

filter  with a bandwidth of 500 kHz centered symmetrically  about

the  carrier  (zero offset). Note that the comment  is  correctly

placed just before the specification statement.

3. Elliptic Bandpass Filter

The specification string must contain the substring  "ELLIP"

or "CAUER" to be recognized. The parameters are:

(1) filter order,

(2) normalized cutoff frequency (>1.0)

(3) stopband attenuation in dB

(4) passband ripple in dB

(5) bandwidth in kHz, and

(6) offset in kHz.

One  and  only one of the first four parameters must be  0.  Only

three of the four parameters are necessary to completely  specify

the  normalized  poles  and  zeros of  an  elliptic  filter.  The

elliptic filter design capability built into FMSIM calculates the

missing parameter from the three nonzero values supplied. If none

of the first four parameters is zero, an error will be  reported.

Similarly,  if  more than one of the first  four  parameters  are

zero, an error will also be reported.

Example:

ELLIPTIC

5,0,60,0.1,550,10

This  specifies  a  fifth order  elliptic  bandpass  filter  with

passband  ripple of 0.1 dB, stopband attenuation of 60 dB, and  a

bandwidth  of  550 kHz offset 10 kHz from carrier.  The  stopband

width (entered as a 0 in this example) is calculated by FMSIM.

4. Bessel Bandpass Filter

The specification string must contain the substring "BESSEL"

or "LINEAR PHASE" to be recognized. The parameters are:

(1) filter order,

(2) bandwidth in kHz, and

(3) offset in kHz.

Example:

BESSEL

3,300,5

This  specifies  a  third order Bessel  bandpass  filter  with  a

bandwidth of 300 kHz offset 5 kHz from carrier. NOTE: the  Bessel

filter  bandwidth is in canonical form relating to  group  delay,

which yields a 3 dB (half power) bandwidth which is  considerably

wider than the specified "bandwidth." It is recommended that  you

experiment  with  the  filter design  utility  ("F"  option  from

beginning menu) to determine the proper Bessel "delay"  bandwidth

to specify for a desired half power bandwidth.

5. Synchronously Tuned Bandpass Filter

The  specification string must contain the substring  "SYNC"

to be recognized. The parameters are:

(1) filter order,

(2) bandwidth in kHz, and

(3) offset in kHz.

Example:

SYNCHRONOUSLY TUNED

2,6E3,-10

This specifies a second order synchronously tuned bandpass filter

with  a  bandwidth (per section) of 600 kHz offset  -10 kHz  from

carrier.  NOTE:  the bandwidth specified  for  the  synchronously

tuned  filter is the half power bandwidth for each  section.  The

overall  half power bandwidth (for filters of order greater  than

one) will be less than the specified per-section bandwidth. It is

recommended  that you experiment with the filter  design  utility

("F"  option  from beginning menu) to determine the  proper  per-

section  bandwidth  to specify for a desired overall  half  power

bandwidth.

6. Multipath Model

The   specification  string  must  contain   the   substring

"MULTIPATH" or "REFLECT" to be recognized. Two types of parameter

lines  must  be entered. The first line contains a  single  value

specifying:

(1) the number of reflections

Following  this  line  there  must  be  a  number  of  reflection

parameter   lines  equal  to  the  number  of  reflections.   The

reflection parameters are:

(1) reflection amplitude in dB (usually negative)

(2) time delay in microseconds

(3) reflection phase at carrier frequency in degrees

Example:

MULTIPATH

3

-45,40,90

-50,700,135

-55,10,-45

This  example specifies a multipath model with three  reflections

at  -45, -50, and -55 dB from the direct signal. The  reflections

are  delayed  40,  700, and  10  microseconds  respectively.  The

carrier  phases at the point of reception are +90, +135, and  -45

degrees respectively.

7. Filter Data File

The  specification string must contain the substring  "DATA"

or  "FILE" to be recognized. The parameter statement consists  of

an ASCII string:

(1) filter file name

The  file  name  must be for a filter file  in  FMSIM  format  as

described elsewhere in this manual.

Example:

DATA

C:\FMSIM\ANTENNA.FM

This  set of commands would read in filter data from an  external

filter file called "C:\FMSIM\ANTENNA.FM" (note that the full path

name does not need to be given if the data file is in the default

directory).

8. Ideal Filter

The specification string must contain the substring  "IDEAL"

to be recognized. The parameters are:

(1) bandwidth in kHz, and

(2) offset in kHz.

Example:

IDEAL FILTER

360,-50

This  set  of  commands specifies an  ideal  filter  (rectangular

bandpass,  zero group delay) of 360 kHz bandwidth offset  -50 kHz

from carrier.

9. Hard Limiter

The specification string must contain the substring  "LIMIT"

to be recognized. There is no parameter statement associated with

this  function (because there are no parameters to specify).  The

limiter function performs the same function as a hard limiter  in

hardware:   it   has  a  gain  inversely  proportional   to   the

instantaneous  envelope amplitude. As the envelope approaches  0,

the  gain  approaches infinity. The limiter function  produces  a

constant  amplitude  envelope of amplitude 1  (nominal  level  in

FMSIM).

10. Clipper

The  specification string must contain the substring  "CLIP"

to  be  recognized.  The parameter statement  contains  just  one

value:

(1) Clipping threshold

The  clipper  function is similar to the hard  limiter  function.

Above the clipping threshold, the function limits: it has a  gain

inversely  proportional to the instantaneous  envelope  amplitude

and  equal  to  the clipping threshold divided  by  the  envelope

amplitude. But below clipping threshold it is a linear  amplifier

with  a  gain of 1. After the clipping function takes  place,  it

reports to you the percentage of time the signal was in clipping.

When  specifying a clipping threshold, remember that  the  normal

operating level in FMSIM is 1. So if you specify a clipping level

of  10.0, you may not do any clipping. If you specify a  clipping

level of 0.1, you will probably be doing lots of clipping and you

will also end up with a small signal of about 0.1 amplitude.

Example:

CLIP

0.8

This  set of statements provides clipping at a level of 0.8  (80%

of nominal operating level).

11. AGC

The specification string must contain the substring "AGC" to

be recognized. The parameter statement contains just one value:

(1) AGC level

The  AGC function is actually a linear function which  first

calculates  the average envelope value then multiplies  the  time

domain  signal  by the desired AGC level divided by  the  average

envelope  value.  So as in the previous example, if you  want  to

clip  the FM signal at an amplitude of 0.1 but then you want  its

average  amplitude set back to 1.0, use the AGC function with  an

AGC  level  parameter  of  1.0. (You could  also  use  the  LIMIT

function  but  all envelope modulation would be  removed  by  the

limiter.)

Example:

AGC

1.05

This set of statements adjusts the signal level to of 1.05  (105%

of nominal operating level).

12. Gain

The  specification string must contain the substring  "GAIN"

to  be  recognized.  The parameter statement  contains  just  one

value:

(1) GAIN value

The  Gain  function is also a linear function  which  simply

multiplies  the time domain signal by the desired gain value.  So

as in the previous example, if you want to clip the FM signal  at

an  amplitude of 0.1 but then you want its average amplitude  set

back  to  1.0, you could also use the GAIN function with  a  GAIN

value  parameter of 10 to bring the signal back to  approximately

the right value.

Example:

GAIN

2.0

This set of statements provides a gain of two.

13. PWL (Piecewise Linear) Amplitude Transfer Function

The specification string must contain the substring "PWL" or

"PIECEWISE  LINEAR"  to  be  recognized.  The  Piecewise   Linear

function  of  NFDL  is  one of  the  most  powerful  and  general

nonlinear functions. It can be used to characterize many kinds of

nonlinear   devices   including  class  C   amplifiers,   "center

clippers,"  semi-linear amplifiers, etc. Two types  of  parameter

lines  must  be entered. The first line contains a  single  value

specifying:

(1) the number of points which follow

Following  this  line there must be a number of PWL  (x,y)  point

parameter  lines  equal to the number of points.  The  PWL  point

parameters are:

(1) X value

(2) Y value

The  entry (0,0) is automatically assumed by FMSIM and  does  not

need to be entered. Each pair of values describes an (x,y)  point

on  an amplitude transfer function. FMSIM  linearly  interpolates

between the points to calculate an envelope transfer function.

Example 1:

PIECEWISE LINEAR

4

0.1,0.5

0.2,0.75

0.4,0.9

1.0,1.0

This  example  specifies a transfer function  which  resembles  a

"soft  clipping"  saturating  amplifier. For  small  signals  the

amplifier has a voltage gain of 5 (0.1,0.5) but the amplifier  is

well into clipping at normal signal levels (1.0,1.0).

Example 2:

PIECEWISE LINEAR

4

0.1,0.0

0.5,0.8

1.0,1.0

1.2,1.05

This example simulates a class C amplifier with a bias  threshold

of  0.1. Up to a signal level of 0.1, there is no output at  all.

At  signal levels above 1.0 the amplifier is well  into  clipping

(1.2,1.05).

Example 3:

PIECEWISE LINEAR

3

0.2,0.0

0.3,0.1

1.0,0.8

This  example simulates a "center clipping" function where  there

is  a  range of small signals (up to a level of  0.2)  for  which

there  is no output at all (similar to a class C  amplifier).  At

signal  levels  above  0.2 the amplifier  performs  as  a  linear

amplifier.